3GPP TSG-RAN WG4 Meeting #114bis	R4-2503297

WuHan, China, 07 April – 11 April, 2025

Source: 	vivo

Title: 	Simulation results for LP-WUR

Agenda Item:	7.26.4.1

Document for:	Discussion

Introduction

In RAN Plenary #102 meeting, WI for “Low-power wake-up signal and receiver for NR” was agreed [1]. The WI was further updated at RAN Plenary #103 meeting as [2]. The initial discussion on simulation assumptions was triggered at RAN4 111 meeting and the simulation assumptions and results were further discussed at following RAN4 meetings. In this contribution we provide our further consideration on simulation assumptions and also provide further simulation results from our side.

Discussion

The objectives of WI are duplicated as below:

The objectives of the work item are the following:

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4)

  - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol

    - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information.

  - At least duty-cycled monitoring of LP-WUS is supported

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

  - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4)

    - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI.

    - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS.

    - Y will be decided within WI. 320ms is the start point.

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1)

  - Check in RAN#105 for potential TU adjustment in RAN2

  - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

- Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3.

- Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode.

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance

    - Define guard RBs for ACS and ASCS cases

    - Study testability of above requirements

    - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

  - Specify necessary RRM requirements

The WF and contribution summary for this WI at RAN4 114 meeting are [3] and [4]. In this contribution, we provide our simulation results for LP-WUR.

Simulation results

The simulation results are provided in the Appendix.

Based on these simulation results, we have the following observation:

- For LP-RSRP, under all alternatives and combinations including different LP-SS sequence, different interference power setting, different synchronous conditions and different channel conditions (AWGN or TDL-C):

- The target RSRP measurement accuracy can be achieved by 1 LP-SS sample SINR = -3dB based on 320ms periodicity of LP-SS if the ideal RSRP is pass the fading channel.

- If the ideal RSRP calculation is not passed the fading channel, 3 samples are required under TDL-C channel to achieve the target accuracy.

- For LP-RSRQ, under all alternatives and combinations including different LP-SS sequence, different interference power setting, different synchronous conditions and different channel conditions (AWGN or TDL-C):

- The target RSRP measurement accuracy can be achieved by 1 and 3 samples for AWGN and TDL-C channel at SINR = -3dB based on 320ms periodicity of LP-SS.  Not matter the ideal RSRQ is pass the fading channel or not.

- For SSB based, LR can satisfy RSRP/RSRQ measurement accuracy based on 1 sample at SINR = -3dB under AWGN channel and 1 sample at SINR = -3dB under TDL-C channel, no matter the ideal RSRP/RSRQ is passed the fading channel or not.

Conclusion

In this contribution, we provide our considerations on the LP-WUS/WUR WI and have the following observations and proposals.

- For LP-RSRP, under all alternatives and combinations including different LP-SS sequence, different interference power setting, different synchronous conditions and different channel conditions (AWGN or TDL-C):

- The target RSRP measurement accuracy can be achieved by 1 LP-SS sample SINR = -3dB based on 320ms periodicity of LP-SS if the ideal RSRP is pass the fading channel.

- If the ideal RSRP calculation is not passed the fading channel, 3 samples are required under TDL-C channel to achieve the target accuracy.

- For LP-RSRQ, under all alternatives and combinations including different LP-SS sequence, different interference power setting, different synchronous conditions and different channel conditions (AWGN or TDL-C):

- The target RSRP measurement accuracy can be achieved by 1 and 3 samples for AWGN and TDL-C channel at SINR = -3dB based on 320ms periodicity of LP-SS.  Not matter the ideal RSRQ is pass the fading channel or not.

- For SSB based, LR can satisfy RSRP/RSRQ measurement accuracy based on 1 sample at SINR = -3dB under AWGN channel and 1 sample at SINR = -3dB under TDL-C channel, no matter the ideal RSRP/RSRQ is passed the fading channel or not.

- References

# [1] RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), RAN 102

# [2] RP-240801, Revised WID: Low-power wake-up signal and receiver for NR ((LP-WUS/WUR), RAN 103

# [3] R4-2500539, Topic summary for [114][225] NR_LPWUS, RAN4 114, vivo

# [4] R4-2502600, WF on RRM requirements for NR_LPWUS, RAN4 114, vivo

# Appendix Simulation results

- Table 1:OOK,RSRP,Ideal signal pass the fading channel

| {M,L} | LP-SS Binary Sequence | RSRP Measurement Accuracy [dB] vs Number of Samples X |  |  |  |  |  | SINR(dB) | channel | Interference Source | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | X = 1 |  | X = 2 |  | X = 3 |  |  |  |  |  |
|  |  | mean | std | mean | std | mean | std |  |  |  |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.30 |  |  |  |  | -3 | AWGN | w/o interference |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 1.00 |  |  |  |  | -3 | AWGN | w/o interference |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.66 |  |  |  |  | -3 | AWGN | w/o interference |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.98 |  |  |  |  | -3 | AWGN | w/o interference |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.22 |  |  |  |  | -3 | AWGN | NR signal interference with 3msdelay Set1 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.91 |  |  |  |  | -3 | AWGN | NR signal interference with 3msdelay Set1 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.62 |  |  |  |  | -3 | AWGN | NR signal interference with 3msdelay Set1 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.92 |  |  |  |  | -3 | AWGN | NR signal interference with 3msdelay Set1 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.25 |  |  |  |  | -3 | AWGN | LP-SS Interference With 3ms delay Set1 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.84 |  |  |  |  | -3 | AWGN | LP-SS Interference With 3ms delay Set1 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.54 |  |  |  |  | -3 | AWGN | LP-SS Interference With 3ms delay Set1 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.81 |  |  |  |  | -3 | AWGN | LP-SS Interference With 3ms delay Set1 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.18 |  |  |  |  | -3 | AWGN | NR signal interference with 3ms delay set2 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.90 |  |  |  |  | -3 | AWGN | NR signal interference with 3ms delay set2 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.61 |  |  |  |  | -3 | AWGN | NR signal interference with 3ms delay set2 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.91 |  |  |  |  | -3 | AWGN | NR signal interference with 3ms delay set2 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.24 |  |  |  |  | -3 | AWGN | LP-SS Interference With 3ms delay set2 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.74 |  |  |  |  | -3 | AWGN | LP-SS Interference With 3ms delay set2 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.43 |  |  |  |  | -3 | AWGN | LP-SS Interference With 3ms delay set2 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.70 |  |  |  |  | -3 | AWGN | LP-SS Interference With 3ms delay set2 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.22 |  |  |  |  | -3 | AWGN | NR signal interference with CP/2 delay Set1 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.92 |  |  |  |  | -3 | AWGN | NR signal interference with CP/2 delay Set1 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.63 |  |  |  |  | -3 | AWGN | NR signal interference with CP/2 delay Set1 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.94 |  |  |  |  | -3 | AWGN | NR signal interference with CP/2 delay Set1 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.24 |  |  |  |  | -3 | AWGN | LP-SS Interference With CP/2 delay Set1 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.91 |  |  |  |  | -3 | AWGN | LP-SS Interference With CP/2 delay Set1 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.63 |  |  |  |  | -3 | AWGN | LP-SS Interference With CP/2 delay Set1 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.92 |  |  |  |  | -3 | AWGN | LP-SS Interference With CP/2 delay Set1 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.17 |  |  |  |  | -3 | AWGN | NR signal interference with CP/2 delay set2 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.89 |  |  |  |  | -3 | AWGN | NR signal interference with CP/2 delay set2 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.62 |  |  |  |  | -3 | AWGN | NR signal interference with CP/2 delay set2 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.91 |  |  |  |  | -3 | AWGN | NR signal interference with CP/2 delay set2 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.24 |  |  |  |  | -3 | AWGN | LP-SS Interference With CP/2 delay set2 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.88 |  |  |  |  | -3 | AWGN | LP-SS Interference With CP/2 delay set2 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.58 |  |  |  |  | -3 | AWGN | LP-SS Interference With CP/2 delay set2 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.88 |  |  |  |  | -3 | AWGN | LP-SS Interference With CP/2 delay set2 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.76 |  |  |  |  | -3 | TDLC | w/o interference |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 1.27 |  |  |  |  | -3 | TDLC | w/o interference |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.90 |  |  |  |  | -3 | TDLC | w/o interference |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 1.29 |  |  |  |  | -3 | TDLC | w/o interference |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.71 |  |  |  |  | -3 | TDLC | NR signal interference with 3msdelay Set1 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 1.24 |  |  |  |  | -3 | TDLC | NR signal interference with 3msdelay Set1 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.88 |  |  |  |  | -3 | TDLC | NR signal interference with 3msdelay Set1 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 1.25 |  |  |  |  | -3 | TDLC | NR signal interference with 3msdelay Set1 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.76 |  |  |  |  | -3 | TDLC | LP-SS Interference With 3ms delay Set1 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 1.36 |  |  |  |  | -3 | TDLC | LP-SS Interference With 3ms delay Set1 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 1.24 |  |  |  |  | -3 | TDLC | LP-SS Interference With 3ms delay Set1 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 1.47 |  |  |  |  | -3 | TDLC | LP-SS Interference With 3ms delay Set1 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.62 |  |  |  |  | -3 | TDLC | NR signal interference with 3ms delay set2 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 1.22 |  |  |  |  | -3 | TDLC | NR signal interference with 3ms delay set2 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.83 |  |  |  |  | -3 | TDLC | NR signal interference with 3ms delay set2 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 1.22 |  |  |  |  | -3 | TDLC | NR signal interference with 3ms delay set2 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 2.02 |  |  |  |  | -3 | TDLC | LP-SS Interference With 3ms delay set2 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 1.56 |  |  |  |  | -3 | TDLC | LP-SS Interference With 3ms delay set2 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 1.96 |  |  |  |  | -3 | TDLC | LP-SS Interference With 3ms delay set2 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 1.86 |  |  |  |  | -3 | TDLC | LP-SS Interference With 3ms delay set2 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.67 |  |  |  |  | -3 | TDLC | NR signal interference with CP/2 delay Set1 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 1.22 |  |  |  |  | -3 | TDLC | NR signal interference with CP/2 delay Set1 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.86 |  |  |  |  | -3 | TDLC | NR signal interference with CP/2 delay Set1 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 1.27 |  |  |  |  | -3 | TDLC | NR signal interference with CP/2 delay Set1 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.77 |  |  |  |  | -3 | TDLC | LP-SS Interference With CP/2 delay Set1 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 1.27 |  |  |  |  | -3 | TDLC | LP-SS Interference With CP/2 delay Set1 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.93 |  |  |  |  | -3 | TDLC | LP-SS Interference With CP/2 delay Set1 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 1.26 |  |  |  |  | -3 | TDLC | LP-SS Interference With CP/2 delay Set1 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.61 |  |  |  |  | -3 | TDLC | NR signal interference with CP/2 delay set2 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 1.18 |  |  |  |  | -3 | TDLC | NR signal interference with CP/2 delay set2 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.84 |  |  |  |  | -3 | TDLC | NR signal interference with CP/2 delay set2 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 1.23 |  |  |  |  | -3 | TDLC | NR signal interference with CP/2 delay set2 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 2.10 |  |  |  |  | -3 | TDLC | LP-SS Interference With CP/2 delay set2 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 1.21 |  |  |  |  | -3 | TDLC | LP-SS Interference With CP/2 delay set2 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 1.08 |  |  |  |  | -3 | TDLC | LP-SS Interference With CP/2 delay set2 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 1.21 |  |  |  |  | -3 | TDLC | LP-SS Interference With CP/2 delay set2 |  |

- Table 2:OOK,RSRP, ideal signal not pass the fading channel

| {M,L} | LP-SS Binary Sequence | RSRP Measurement Accuracy [dB] vs Number of Samples X |  |  |  |  |  | SINR(dB) | channel | Interference Source | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | X = 1 |  | X = 2 |  | X = 3 |  |  |  |  |  |
|  |  | mean | std | mean | std | mean | std |  |  |  |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.33 |  |  |  |  | -3 | AWGN | w/o interference |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.98 |  |  |  |  | -3 | AWGN | w/o interference |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.67 |  |  |  |  | -3 | AWGN | w/o interference |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 1.00 |  |  |  |  | -3 | AWGN | w/o interference |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.22 |  |  |  |  | -3 | AWGN | NR signal interference with 3msdelay Set1 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.91 |  |  |  |  | -3 | AWGN | NR signal interference with 3msdelay Set1 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.62 |  |  |  |  | -3 | AWGN | NR signal interference with 3msdelay Set1 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.92 |  |  |  |  | -3 | AWGN | NR signal interference with 3msdelay Set1 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.25 |  |  |  |  | -3 | AWGN | LP-SS Interference With 3ms delay Set1 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.84 |  |  |  |  | -3 | AWGN | LP-SS Interference With 3ms delay Set1 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.54 |  |  |  |  | -3 | AWGN | LP-SS Interference With 3ms delay Set1 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.81 |  |  |  |  | -3 | AWGN | LP-SS Interference With 3ms delay Set1 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.18 |  |  |  |  | -3 | AWGN | NR signal interference with 3ms delay set2 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.90 |  |  |  |  | -3 | AWGN | NR signal interference with 3ms delay set2 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.61 |  |  |  |  | -3 | AWGN | NR signal interference with 3ms delay set2 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.91 |  |  |  |  | -3 | AWGN | NR signal interference with 3ms delay set2 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.25 |  |  |  |  | -3 | AWGN | LP-SS Interference With 3ms delay set2 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.74 |  |  |  |  | -3 | AWGN | LP-SS Interference With 3ms delay set2 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.43 |  |  |  |  | -3 | AWGN | LP-SS Interference With 3ms delay set2 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.70 |  |  |  |  | -3 | AWGN | LP-SS Interference With 3ms delay set2 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.22 |  |  |  |  | -3 | AWGN | NR signal interference with CP/2 delay Set1 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.92 |  |  |  |  | -3 | AWGN | NR signal interference with CP/2 delay Set1 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.63 |  |  |  |  | -3 | AWGN | NR signal interference with CP/2 delay Set1 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.95 |  |  |  |  | -3 | AWGN | NR signal interference with CP/2 delay Set1 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.24 |  |  |  |  | -3 | AWGN | LP-SS Interference With CP/2 delay Set1 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.92 |  |  |  |  | -3 | AWGN | LP-SS Interference With CP/2 delay Set1 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.63 |  |  |  |  | -3 | AWGN | LP-SS Interference With CP/2 delay Set1 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.92 |  |  |  |  | -3 | AWGN | LP-SS Interference With CP/2 delay Set1 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.17 |  |  |  |  | -3 | AWGN | NR signal interference with CP/2 delay set2 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.89 |  |  |  |  | -3 | AWGN | NR signal interference with CP/2 delay set2 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.62 |  |  |  |  | -3 | AWGN | NR signal interference with CP/2 delay set2 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.91 |  |  |  |  | -3 | AWGN | NR signal interference with CP/2 delay set2 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.23 |  |  |  |  | -3 | AWGN | LP-SS Interference With CP/2 delay set2 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.88 |  |  |  |  | -3 | AWGN | LP-SS Interference With CP/2 delay set2 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.59 |  |  |  |  | -3 | AWGN | LP-SS Interference With CP/2 delay set2 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.88 |  |  |  |  | -3 | AWGN | LP-SS Interference With CP/2 delay set2 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 5.05 |  | 3.38 |  | 2.53 | -3 | TDLC | w/o interference |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 4.98 |  | 3.42 |  | 2.61 | -3 | TDLC | w/o interference |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 4.91 |  | 3.42 |  | 2.52 | -3 | TDLC | w/o interference |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 4.96 |  | 3.40 |  | 2.49 | -3 | TDLC | w/o interference |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 4.98 |  | 3.56 |  | 2.91 | -3 | TDLC | NR signal interference with 3msdelay Set1 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 5.01 |  | 3.53 |  | 2.83 | -3 | TDLC | NR signal interference with 3msdelay Set1 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 4.83 |  | 3.53 |  | 2.91 | -3 | TDLC | NR signal interference with 3msdelay Set1 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 4.96 |  | 3.66 |  | 2.95 | -3 | TDLC | NR signal interference with 3msdelay Set1 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 5.03 |  | 3.67 |  | 2.86 | -3 | TDLC | LP-SS Interference With 3ms delay Set1 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 5.01 |  | 3.53 |  | 2.88 | -3 | TDLC | LP-SS Interference With 3ms delay Set1 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 4.93 |  | 3.53 |  | 2.91 | -3 | TDLC | LP-SS Interference With 3ms delay Set1 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 5.09 |  | 3.51 |  | 2.96 | -3 | TDLC | LP-SS Interference With 3ms delay Set1 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 5.01 |  | 3.57 |  | 2.90 | -3 | TDLC | NR signal interference with 3ms delay set2 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 5.00 |  | 3.56 |  | 2.88 | -3 | TDLC | NR signal interference with 3ms delay set2 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 4.84 |  | 3.53 |  | 2.92 | -3 | TDLC | NR signal interference with 3ms delay set2 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 4.93 |  | 3.59 |  | 2.96 | -3 | TDLC | NR signal interference with 3ms delay set2 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 5.41 |  | 3.93 |  | 3.08 | -3 | TDLC | LP-SS Interference With 3ms delay set2 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 5.20 |  | 3.63 |  | 2.91 | -3 | TDLC | LP-SS Interference With 3ms delay set2 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 5.15 |  | 3.64 |  | 3.03 | -3 | TDLC | LP-SS Interference With 3ms delay set2 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 5.14 |  | 3.60 |  | 3.00 | -3 | TDLC | LP-SS Interference With 3ms delay set2 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 4.96 |  | 3.61 |  | 2.88 | -3 | TDLC | NR signal interference with CP/2 delay Set1 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 5.02 |  | 3.53 |  | 2.88 | -3 | TDLC | NR signal interference with CP/2 delay Set1 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 4.82 |  | 3.54 |  | 2.90 | -3 | TDLC | NR signal interference with CP/2 delay Set1 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 5.01 |  | 3.66 |  | 2.96 | -3 | TDLC | NR signal interference with CP/2 delay Set1 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 5.06 |  | 3.72 |  | 2.89 | -3 | TDLC | LP-SS Interference With CP/2 delay Set1 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 5.01 |  | 3.56 |  | 2.86 | -3 | TDLC | LP-SS Interference With CP/2 delay Set1 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 4.78 |  | 3.50 |  | 2.90 | -3 | TDLC | LP-SS Interference With CP/2 delay Set1 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 5.03 |  | 3.62 |  | 2.95 | -3 | TDLC | LP-SS Interference With CP/2 delay Set1 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 4.95 |  | 3.58 |  | 2.91 | -3 | TDLC | NR signal interference with CP/2 delay set2 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 4.96 |  | 3.58 |  | 2.88 | -3 | TDLC | NR signal interference with CP/2 delay set2 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 4.82 |  | 3.54 |  | 2.91 | -3 | TDLC | NR signal interference with CP/2 delay set2 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 4.98 |  | 3.62 |  | 2.97 | -3 | TDLC | NR signal interference with CP/2 delay set2 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 5.48 |  | 4.01 |  | 2.97 | -3 | TDLC | LP-SS Interference With CP/2 delay set2 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 5.02 |  | 3.62 |  | 2.87 | -3 | TDLC | LP-SS Interference With CP/2 delay set2 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 4.59 |  | 3.28 |  | 2.80 | -3 | TDLC | LP-SS Interference With CP/2 delay set2 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 4.86 |  | 3.43 |  | 2.88 | -3 | TDLC | LP-SS Interference With CP/2 delay set2 |  |

- Table 3:OOK,RSRQ, Ideal signal pass the fading channel

| {M,L} | LP-SS Binary Sequence | RSRQ Measurement Accuracy [dB] vs Number of Samples X |  |  |  |  |  | SINR(dB) | channel | Interference Source | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | X = 1 |  | X = 2 |  | X = 3 |  |  |  |  |  |
|  |  | mean | std | mean | std | mean | std |  |  |  |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.28 |  |  |  |  | -3 | AWGN | w/o interference |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.90 |  |  |  |  | -3 | AWGN | w/o interference |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.61 |  |  |  |  | -3 | AWGN | w/o interference |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.90 |  |  |  |  | -3 | AWGN | w/o interference |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.18 |  |  |  |  | -3 | AWGN | NR signal interference with 3msdelay Set1 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.85 |  |  |  |  | -3 | AWGN | NR signal interference with 3msdelay Set1 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.57 |  |  |  |  | -3 | AWGN | NR signal interference with 3msdelay Set1 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.84 |  |  |  |  | -3 | AWGN | NR signal interference with 3msdelay Set1 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.19 |  |  |  |  | -3 | AWGN | LP-SS Interference With 3ms delay Set1 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.77 |  |  |  |  | -3 | AWGN | LP-SS Interference With 3ms delay Set1 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.46 |  |  |  |  | -3 | AWGN | LP-SS Interference With 3ms delay Set1 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.72 |  |  |  |  | -3 | AWGN | LP-SS Interference With 3ms delay Set1 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.15 |  |  |  |  | -3 | AWGN | NR signal interference with 3ms delay set2 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.82 |  |  |  |  | -3 | AWGN | NR signal interference with 3ms delay set2 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.56 |  |  |  |  | -3 | AWGN | NR signal interference with 3ms delay set2 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.83 |  |  |  |  | -3 | AWGN | NR signal interference with 3ms delay set2 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.19 |  |  |  |  | -3 | AWGN | LP-SS Interference With 3ms delay set2 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.66 |  |  |  |  | -3 | AWGN | LP-SS Interference With 3ms delay set2 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.35 |  |  |  |  | -3 | AWGN | LP-SS Interference With 3ms delay set2 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.59 |  |  |  |  | -3 | AWGN | LP-SS Interference With 3ms delay set2 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.16 |  |  |  |  | -3 | AWGN | NR signal interference with CP/2 delay Set1 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.85 |  |  |  |  | -3 | AWGN | NR signal interference with CP/2 delay Set1 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.57 |  |  |  |  | -3 | AWGN | NR signal interference with CP/2 delay Set1 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.84 |  |  |  |  | -3 | AWGN | NR signal interference with CP/2 delay Set1 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.18 |  |  |  |  | -3 | AWGN | LP-SS Interference With CP/2 delay Set1 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.84 |  |  |  |  | -3 | AWGN | LP-SS Interference With CP/2 delay Set1 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.57 |  |  |  |  | -3 | AWGN | LP-SS Interference With CP/2 delay Set1 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.83 |  |  |  |  | -3 | AWGN | LP-SS Interference With CP/2 delay Set1 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.11 |  |  |  |  | -3 | AWGN | NR signal interference with CP/2 delay set2 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.82 |  |  |  |  | -3 | AWGN | NR signal interference with CP/2 delay set2 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.56 |  |  |  |  | -3 | AWGN | NR signal interference with CP/2 delay set2 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.83 |  |  |  |  | -3 | AWGN | NR signal interference with CP/2 delay set2 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.19 |  |  |  |  | -3 | AWGN | LP-SS Interference With CP/2 delay set2 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.81 |  |  |  |  | -3 | AWGN | LP-SS Interference With CP/2 delay set2 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.51 |  |  |  |  | -3 | AWGN | LP-SS Interference With CP/2 delay set2 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.77 |  |  |  |  | -3 | AWGN | LP-SS Interference With CP/2 delay set2 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 3.73 |  | 2.37 |  |  | -3 | TDLC | w/o interference |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 3.05 |  | 1.92 |  |  | -3 | TDLC | w/o interference |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 3.08 |  | 1.98 |  |  | -3 | TDLC | w/o interference |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 3.08 |  | 1.91 |  |  | -3 | TDLC | w/o interference |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 4.17 |  | 2.81 |  | 2.25 | -3 | TDLC | NR signal interference with 3msdelay Set1 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 3.70 |  | 2.35 |  | 1.92 | -3 | TDLC | NR signal interference with 3msdelay Set1 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 3.53 |  | 2.38 |  | 1.94 | -3 | TDLC | NR signal interference with 3msdelay Set1 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 3.59 |  | 2.45 |  | 1.98 | -3 | TDLC | NR signal interference with 3msdelay Set1 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 4.24 |  | 2.93 |  | 2.31 | -3 | TDLC | LP-SS Interference With 3ms delay Set1 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 3.85 |  | 2.45 |  | 1.92 | -3 | TDLC | LP-SS Interference With 3ms delay Set1 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 4.09 |  | 2.69 |  | 2.19 | -3 | TDLC | LP-SS Interference With 3ms delay Set1 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 3.97 |  | 2.56 |  | 2.15 | -3 | TDLC | LP-SS Interference With 3ms delay Set1 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 3.80 |  | 2.53 |  | 2.02 | -3 | TDLC | NR signal interference with 3ms delay set2 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 3.39 |  | 2.11 |  | 1.76 | -3 | TDLC | NR signal interference with 3ms delay set2 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 3.22 |  | 2.14 |  | 1.73 | -3 | TDLC | NR signal interference with 3ms delay set2 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 3.33 |  | 2.19 |  | 1.82 | -3 | TDLC | NR signal interference with 3ms delay set2 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 4.41 |  | 3.08 |  | 2.34 | -3 | TDLC | LP-SS Interference With 3ms delay set2 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 3.91 |  | 2.51 |  | 1.97 | -3 | TDLC | LP-SS Interference With 3ms delay set2 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 4.66 |  | 3.13 |  | 2.53 | -3 | TDLC | LP-SS Interference With 3ms delay set2 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 4.28 |  | 2.70 |  | 2.26 | -3 | TDLC | LP-SS Interference With 3ms delay set2 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 4.09 |  | 2.77 |  | 2.24 | -3 | TDLC | NR signal interference with CP/2 delay Set1 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 3.73 |  | 2.38 |  | 1.90 | -3 | TDLC | NR signal interference with CP/2 delay Set1 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 3.53 |  | 2.38 |  | 1.96 | -3 | TDLC | NR signal interference with CP/2 delay Set1 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 3.62 |  | 2.45 |  | 2.00 | -3 | TDLC | NR signal interference with CP/2 delay Set1 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 4.32 |  | 2.97 |  | 2.29 | -3 | TDLC | LP-SS Interference With CP/2 delay Set1 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 3.72 |  | 2.40 |  | 1.96 | -3 | TDLC | LP-SS Interference With CP/2 delay Set1 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 3.57 |  | 2.39 |  | 1.92 | -3 | TDLC | LP-SS Interference With CP/2 delay Set1 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 3.64 |  | 2.50 |  | 2.00 | -3 | TDLC | LP-SS Interference With CP/2 delay Set1 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 3.66 |  | 2.48 |  | 1.96 | -3 | TDLC | NR signal interference with CP/2 delay set2 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 3.35 |  | 2.16 |  | 1.76 | -3 | TDLC | NR signal interference with CP/2 delay set2 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 3.22 |  | 2.22 |  | 1.79 | -3 | TDLC | NR signal interference with CP/2 delay set2 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 3.30 |  | 2.20 |  | 1.80 | -3 | TDLC | NR signal interference with CP/2 delay set2 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 4.50 |  | 3.04 |  | 2.28 | -3 | TDLC | LP-SS Interference With CP/2 delay set2 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 3.48 |  | 2.20 |  | 1.84 | -3 | TDLC | LP-SS Interference With CP/2 delay set2 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 3.24 |  | 2.09 |  | 1.76 | -3 | TDLC | LP-SS Interference With CP/2 delay set2 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 3.33 |  | 2.20 |  | 1.82 | -3 | TDLC | LP-SS Interference With CP/2 delay set2 |  |

- Table 4:OOK,RSRQ,ideal signal not pass the fading channel

| {M,L} | LP-SS Binary Sequence | RSRQ Measurement Accuracy [dB] vs Number of Samples X |  |  |  |  |  | SINR(dB) | channel | Interference Source | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | X = 1 |  | X = 2 |  | X = 3 |  |  |  |  |  |
|  |  | mean | std | mean | std | mean | std |  |  |  |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.28 |  |  |  |  | -3 | AWGN | w/o interference |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.90 |  |  |  |  | -3 | AWGN | w/o interference |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.61 |  |  |  |  | -3 | AWGN | w/o interference |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.90 |  |  |  |  | -3 | AWGN | w/o interference |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.18 |  |  |  |  | -3 | AWGN | NR signal interference with 3msdelay Set1 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.84 |  |  |  |  | -3 | AWGN | NR signal interference with 3msdelay Set1 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.57 |  |  |  |  | -3 | AWGN | NR signal interference with 3msdelay Set1 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.84 |  |  |  |  | -3 | AWGN | NR signal interference with 3msdelay Set1 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.19 |  |  |  |  | -3 | AWGN | LP-SS Interference With 3ms delay Set1 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.77 |  |  |  |  | -3 | AWGN | LP-SS Interference With 3ms delay Set1 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.46 |  |  |  |  | -3 | AWGN | LP-SS Interference With 3ms delay Set1 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.72 |  |  |  |  | -3 | AWGN | LP-SS Interference With 3ms delay Set1 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.15 |  |  |  |  | -3 | AWGN | NR signal interference with 3ms delay set2 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.82 |  |  |  |  | -3 | AWGN | NR signal interference with 3ms delay set2 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.56 |  |  |  |  | -3 | AWGN | NR signal interference with 3ms delay set2 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.83 |  |  |  |  | -3 | AWGN | NR signal interference with 3ms delay set2 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.19 |  |  |  |  | -3 | AWGN | LP-SS Interference With 3ms delay set2 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.66 |  |  |  |  | -3 | AWGN | LP-SS Interference With 3ms delay set2 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.36 |  |  |  |  | -3 | AWGN | LP-SS Interference With 3ms delay set2 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.60 |  |  |  |  | -3 | AWGN | LP-SS Interference With 3ms delay set2 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.16 |  |  |  |  | -3 | AWGN | NR signal interference with CP/2 delay Set1 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.85 |  |  |  |  | -3 | AWGN | NR signal interference with CP/2 delay Set1 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.57 |  |  |  |  | -3 | AWGN | NR signal interference with CP/2 delay Set1 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.84 |  |  |  |  | -3 | AWGN | NR signal interference with CP/2 delay Set1 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.18 |  |  |  |  | -3 | AWGN | LP-SS Interference With CP/2 delay Set1 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.84 |  |  |  |  | -3 | AWGN | LP-SS Interference With CP/2 delay Set1 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.56 |  |  |  |  | -3 | AWGN | LP-SS Interference With CP/2 delay Set1 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.83 |  |  |  |  | -3 | AWGN | LP-SS Interference With CP/2 delay Set1 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.11 |  |  |  |  | -3 | AWGN | NR signal interference with CP/2 delay set2 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.82 |  |  |  |  | -3 | AWGN | NR signal interference with CP/2 delay set2 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.56 |  |  |  |  | -3 | AWGN | NR signal interference with CP/2 delay set2 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.83 |  |  |  |  | -3 | AWGN | NR signal interference with CP/2 delay set2 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 1.19 |  |  |  |  | -3 | AWGN | LP-SS Interference With CP/2 delay set2 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 0.81 |  |  |  |  | -3 | AWGN | LP-SS Interference With CP/2 delay set2 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.51 |  |  |  |  | -3 | AWGN | LP-SS Interference With CP/2 delay set2 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.77 |  |  |  |  | -3 | AWGN | LP-SS Interference With CP/2 delay set2 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 4.48 |  | 2.81 |  | 2.15 | -3 | TDLC | w/o interference |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 3.91 |  | 2.47 |  | 1.81 | -3 | TDLC | w/o interference |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 3.84 |  | 2.44 |  | 1.76 | -3 | TDLC | w/o interference |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 3.90 |  | 2.45 |  | 1.77 | -3 | TDLC | w/o interference |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 4.38 |  | 3.05 |  | 2.39 | -3 | TDLC | NR signal interference with 3msdelay Set1 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 3.90 |  | 2.50 |  | 2.00 | -3 | TDLC | NR signal interference with 3msdelay Set1 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 3.71 |  | 2.54 |  | 2.04 | -3 | TDLC | NR signal interference with 3msdelay Set1 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 3.79 |  | 2.59 |  | 2.09 | -3 | TDLC | NR signal interference with 3msdelay Set1 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 4.40 |  | 3.14 |  | 2.42 | -3 | TDLC | LP-SS Interference With 3ms delay Set1 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 3.91 |  | 2.54 |  | 1.97 | -3 | TDLC | LP-SS Interference With 3ms delay Set1 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 3.85 |  | 2.49 |  | 2.10 | -3 | TDLC | LP-SS Interference With 3ms delay Set1 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 3.89 |  | 2.57 |  | 2.10 | -3 | TDLC | LP-SS Interference With 3ms delay Set1 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 4.44 |  | 3.01 |  | 2.38 | -3 | TDLC | NR signal interference with 3ms delay set2 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 3.87 |  | 2.46 |  | 1.98 | -3 | TDLC | NR signal interference with 3ms delay set2 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 3.69 |  | 2.49 |  | 2.00 | -3 | TDLC | NR signal interference with 3ms delay set2 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 3.79 |  | 2.55 |  | 2.04 | -3 | TDLC | NR signal interference with 3ms delay set2 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 4.77 |  | 3.31 |  | 2.57 | -3 | TDLC | LP-SS Interference With 3ms delay set2 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 4.01 |  | 2.62 |  | 2.05 | -3 | TDLC | LP-SS Interference With 3ms delay set2 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 4.06 |  | 2.56 |  | 2.25 | -3 | TDLC | LP-SS Interference With 3ms delay set2 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 4.09 |  | 2.55 |  | 2.13 | -3 | TDLC | LP-SS Interference With 3ms delay set2 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 4.34 |  | 3.00 |  | 2.36 | -3 | TDLC | NR signal interference with CP/2 delay Set1 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 3.95 |  | 2.52 |  | 1.96 | -3 | TDLC | NR signal interference with CP/2 delay Set1 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 3.72 |  | 2.53 |  | 2.04 | -3 | TDLC | NR signal interference with CP/2 delay Set1 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 3.87 |  | 2.62 |  | 2.12 | -3 | TDLC | NR signal interference with CP/2 delay Set1 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 4.46 |  | 3.13 |  | 2.41 | -3 | TDLC | LP-SS Interference With CP/2 delay Set1 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 3.90 |  | 2.53 |  | 2.01 | -3 | TDLC | LP-SS Interference With CP/2 delay Set1 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 3.73 |  | 2.51 |  | 2.01 | -3 | TDLC | LP-SS Interference With CP/2 delay Set1 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 3.89 |  | 2.61 |  | 2.06 | -3 | TDLC | LP-SS Interference With CP/2 delay Set1 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 4.35 |  | 2.95 |  | 2.39 | -3 | TDLC | NR signal interference with CP/2 delay set2 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 3.84 |  | 2.51 |  | 1.95 | -3 | TDLC | NR signal interference with CP/2 delay set2 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 3.71 |  | 2.48 |  | 2.03 | -3 | TDLC | NR signal interference with CP/2 delay set2 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 3.84 |  | 2.61 |  | 2.12 | -3 | TDLC | NR signal interference with CP/2 delay set2 |  |
| {1,6} | Serving cell:101010 Interference Cell:010101 |  | 4.84 |  | 3.38 |  | 2.50 | -3 | TDLC | LP-SS Interference With CP/2 delay set2 |  |
| {2,8} | Serving cell:01011001 Interference Cell:01100101 |  | 3.89 |  | 2.54 |  | 1.96 | -3 | TDLC | LP-SS Interference With CP/2 delay set2 |  |
| {2,16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 3.50 |  | 2.32 |  | 1.87 | -3 | TDLC | LP-SS Interference With CP/2 delay set2 |  |
| {4,16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 3.73 |  | 2.42 |  | 2.06 | -3 | TDLC | LP-SS Interference With CP/2 delay set2 |  |

- Table 5: SSB,RSRQ,ideal signal pass the fading channel

| RSRQ Measurement Accuracy [dB] vs Number of Samples X |  |  |  |  |  | SINR(dB) | channel | Interference Source | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X = 1 |  | X = 2 |  | X = 3 |  |  |  |  |  |
| mean | std | mean | std | mean | std |  |  |  |  |
|  | 0.74 |  |  |  |  | -3 | AWGN | NR signal Interference With 3ms delay set1 |  |
|  | 0.74 |  |  |  |  | -3 | AWGN | SSB Interference With 3ms delay set1 |  |
|  | 0.71 |  |  |  |  | -3 | AWGN | NR signal Interference With 3ms delay set2 |  |
|  | 0.71 |  |  |  |  | -3 | AWGN | SSB Interference With3ms delay set2 |  |
|  | 0.74 |  |  |  |  | -3 | AWGN | NR signal Interference With CP/2 delay set1 |  |
|  | 0.74 |  |  |  |  | -3 | AWGN | SSB Interference With CP/2 delay set1 |  |
|  | 0.71 |  |  |  |  | -3 | AWGN | NR signal Interference With CP/2 delay set2 |  |
|  | 0.71 |  |  |  |  | -3 | AWGN | SSB Interference With CP/2 delay set2 |  |
|  | 1.08 |  |  |  |  | -3 | TDLC | NR signal Interference With 3ms delay set1 |  |
|  | 1.08 |  |  |  |  | -3 | TDLC | SSB Interference With 3ms delay set1 |  |
|  | 1.08 |  |  |  |  | -3 | TDLC | NR signal Interference With 3ms delay set2 |  |
|  | 1.08 |  |  |  |  | -3 | TDLC | SSB Interference With3ms delay set2 |  |
|  | 1.08 |  |  |  |  | -3 | TDLC | NR signal Interference With CP/2 delay set1 |  |
|  | 1.08 |  |  |  |  | -3 | TDLC | SSB Interference With CP/2 delay set1 |  |
|  | 1.04 |  |  |  |  | -3 | TDLC | NR signal Interference With CP/2 delay set2 |  |
|  | 1.04 |  |  |  |  | -3 | TDLC | SSB Interference With CP/2 delay set2 |  |

- Table 6: SSB,RSRQ,ideal signal not pass the fading channel

| RSRQ Measurement Accuracy [dB] vs Number of Samples X |  |  |  |  |  | SINR(dB) | channel | Interference Source | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X = 1 |  | X = 2 |  | X = 3 |  |  |  |  |  |
| mean | std | mean | std | mean | std |  |  |  |  |
|  | 0.74 |  |  |  |  | -3 | AWGN | NR signal Interference With 3ms delay set1 |  |
|  | 0.74 |  |  |  |  | -3 | AWGN | SSB Interference With 3ms delay set1 |  |
|  | 0.71 |  |  |  |  | -3 | AWGN | NR signal Interference With 3ms delay set2 |  |
|  | 0.70 |  |  |  |  | -3 | AWGN | SSB Interference With3ms delay set2 |  |
|  | 0.74 |  |  |  |  | -3 | AWGN | NR signal Interference With CP/2 delay set1 |  |
|  | 0.74 |  |  |  |  | -3 | AWGN | SSB Interference With CP/2 delay set1 |  |
|  | 0.71 |  |  |  |  | -3 | AWGN | NR signal Interference With CP/2 delay set2 |  |
|  | 0.71 |  |  |  |  | -3 | AWGN | SSB Interference With CP/2 delay set2 |  |
|  | 1.86 |  |  |  |  | -3 | TDLC | NR signal Interference With 3ms delay set1 |  |
|  | 1.86 |  |  |  |  | -3 | TDLC | SSB Interference With 3ms delay set1 |  |
|  | 1.83 |  |  |  |  | -3 | TDLC | NR signal Interference With 3ms delay set2 |  |
|  | 1.84 |  |  |  |  | -3 | TDLC | SSB Interference With3ms delay set2 |  |
|  | 1.86 |  |  |  |  | -3 | TDLC | NR signal Interference With CP/2 delay set1 |  |
|  | 1.86 |  |  |  |  | -3 | TDLC | SSB Interference With CP/2 delay set1 |  |
|  | 1.83 |  |  |  |  | -3 | TDLC | NR signal Interference With CP/2 delay set2 |  |
|  | 1.83 |  |  |  |  | -3 | TDLC | SSB Interference With CP/2 delay set2 |  |

- Table 7: SSB,RSRP,ideal signal pass the fading channel

| RSRP Measurement Accuracy [dB] vs Number of Samples X |  |  |  |  |  | SINR(dB) | channel | Interference Source | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X = 1 |  | X = 2 |  | X = 3 |  |  |  |  |  |
| mean | std | mean | std | mean | std |  |  |  |  |
|  | 0.85 |  |  |  |  | -3 | AWGN | NR signal Interference With 3ms delay set1 |  |
|  | 0.85 |  |  |  |  | -3 | AWGN | SSB Interference With 3ms delay set1 |  |
|  | 0.82 |  |  |  |  | -3 | AWGN | NR signal Interference With 3ms delay set2 |  |
|  | 0.82 |  |  |  |  | -3 | AWGN | SSB Interference With3ms delay set2 |  |
|  | 0.85 |  |  |  |  | -3 | AWGN | NR signal Interference With CP/2 delay set1 |  |
|  | 0.85 |  |  |  |  | -3 | AWGN | SSB Interference With CP/2 delay set1 |  |
|  | 0.83 |  |  |  |  | -3 | AWGN | NR signal Interference With CP/2 delay set2 |  |
|  | 0.83 |  |  |  |  | -3 | AWGN | SSB Interference With CP/2 delay set2 |  |
|  | 1.15 |  |  |  |  | -3 | TDLC | NR signal Interference With 3ms delay set1 |  |
|  | 1.15 |  |  |  |  | -3 | TDLC | SSB Interference With 3ms delay set1 |  |
|  | 1.11 |  |  |  |  | -3 | TDLC | NR signal Interference With 3ms delay set2 |  |
|  | 1.11 |  |  |  |  | -3 | TDLC | SSB Interference With3ms delay set2 |  |
|  | 1.15 |  |  |  |  | -3 | TDLC | NR signal Interference With CP/2 delay set1 |  |
|  | 1.15 |  |  |  |  | -3 | TDLC | SSB Interference With CP/2 delay set1 |  |
|  | 1.11 |  |  |  |  | -3 | TDLC | NR signal Interference With CP/2 delay set2 |  |
|  | 1.11 |  |  |  |  | -3 | TDLC | SSB Interference With CP/2 delay set2 |  |

- Table 8: SSB,RSRP,ideal signal not pass the fading channel

| RSRP Measurement Accuracy [dB] vs Number of Samples X |  |  |  |  |  | SINR(dB) | channel | Interference Source | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X = 1 |  | X = 2 |  | X = 3 |  |  |  |  |  |
| mean | std | mean | std | mean | std |  |  |  |  |
|  | 0.85 |  |  |  |  | -3 | AWGN | NR signal Interference With 3ms delay set1 |  |
|  | 0.85 |  |  |  |  | -3 | AWGN | SSB Interference With 3ms delay set1 |  |
|  | 0.82 |  |  |  |  | -3 | AWGN | NR signal Interference With 3ms delay set2 |  |
|  | 0.82 |  |  |  |  | -3 | AWGN | SSB Interference With3ms delay set2 |  |
|  | 0.85 |  |  |  |  | -3 | AWGN | NR signal Interference With CP/2 delay set1 |  |
|  | 0.85 |  |  |  |  | -3 | AWGN | SSB Interference With CP/2 delay set1 |  |
|  | 0.83 |  |  |  |  | -3 | AWGN | NR signal Interference With CP/2 delay set2 |  |
|  | 0.82 |  |  |  |  | -3 | AWGN | SSB Interference With CP/2 delay set2 |  |
|  | 2.85 |  |  |  |  | -3 | TDLC | NR signal Interference With 3ms delay set1 |  |
|  | 2.85 |  |  |  |  | -3 | TDLC | SSB Interference With 3ms delay set1 |  |
|  | 2.89 |  |  |  |  | -3 | TDLC | NR signal Interference With 3ms delay set2 |  |
|  | 2.90 |  |  |  |  | -3 | TDLC | SSB Interference With3ms delay set2 |  |
|  | 2.85 |  |  |  |  | -3 | TDLC | NR signal Interference With CP/2 delay set1 |  |
|  | 2.85 |  |  |  |  | -3 | TDLC | SSB Interference With CP/2 delay set1 |  |
|  | 2.90 |  |  |  |  | -3 | TDLC | NR signal Interference With CP/2 delay set2 |  |
|  | 2.90 |  |  |  |  | -3 | TDLC | SSB Interference With CP/2 delay set2 |  |