# 3GPP TSG-RAN WG4 Meeting #114bis 	                                R4-2503299

Wuhan, China, 07th – 11th April, 2025

Source:                Tejas Networks Pvt. Ltd.

Title:                    Views on modification of existing Rx requirements for SBFD

Agenda Item:       7.23.2.3

Document for:     Discussion

# Introduction

In the RAN4 #114 meeting, RAN4 agreed to further study, SBFD in-band blocking.

# Discussion

## SBFD in-band blocking

In the RAN4 #114 agreed to further study BS receiver in-band blocking with following agreement and WF [1].

WF: - For FR2-1 WA SBFD BS, FFS re-use the current FR2-1 WA BS in-band blocking requirements. - For FR1 WA BS, define the in-band blocking requirement, FFS on following options. Other options are not precluded. - Option 1: Define the requirement by selecting one of the following grid-shift values. Additional emission issue is not included in this release. If regulators make a decision to require SBFD on adjacent operators in a band, a future WI would address necessary emissions and blocking requirements for the SBFD-SBFD adjacent operator scenario. - Option A: 20% - Option B: 50% - Option 2: Define general requirements for SBFD-TDD adjacent operator scenario. Define an additional in-band blocking requirement and additional emission to address SBFD-SBFD adjacent operator scenario. - Option 3: Define in-band blocking requirements based on co-existence simulation results with grid-shift 5%. - Option 4: Define in-band blocking requirements based on co-existence simulation results with grid-shift 100%. - For FR1 MR SBFD BS, FFS.

The For FR1 WA BS, SBFD in-band blocking simulation findings from our company are included in this contribution, a unified blocking requirement to consider both scenarios and down-selected grid-shift values of 10% and 100%.

### Simulation Assumptions

Based on the WF captured in [1], we have analysed the following scenarios: TDD DL (aggressor) to SBFD UL sub band, and SBFD DL (aggressor) to SBFD UL sub band. The objective is to measure the in-band blocking power at the SBFD base stations. Table 1 presents the co-existence scenarios considered, while Table 2 outlines the configuration used for the adjacent channel co-existence case in the simulation.

Table 1:  Co-existence Scenario considered for the simulation

| Scenario | FR | Aggressor | Victim | Corresponding BS class |
| --- | --- | --- | --- | --- |
| 1 | FR1 | Urban Macro | Urban Macro | Wide area BS |

Table 2:  Adjacent channel co-existence case considered for the simulation

| Aggressor | Victim | Slot allocation Aggressor Victim |
| --- | --- | --- |
| TDD DL | SBFD |  |
| SBFD | SBFD |  |

Following simulation assumption have been selected from Annex E for Scenario 1 (Urban Macro) [2].

- Grid shift: We have selected the worst-case scenario with small grid-shift (10%) for uncoordinated deployments and best-case grid shift of 100%.

- LOS probability: Option 2: If the 2D distance between two BS is less than or equal to the ISD, set the LOS probability to X; X = 0.75. Otherwise, UMa (h_UT = 25 m).

- UE distribution: uniform, 20% indoor and 80% outdoor.

- BS transmit power: TDD 49 dBm and SBFD 46 dBm.

- Antenna configuration: TDD 32×32, and SBFD config 32×32.

Moreover, for the performance evaluation, RAN4 had only considered the case of {D, U} as an SBFD configuration as it is comparable to the {D, U, D} SBFD configuration. Throughput loss due to the adjacent channel CLI was studied, and Figs. 1 through Fig-3 are box plots that represent the summary of a median throughput loss from performance reports of the adjacent channel coexistence study [3]. Plots include the minimum, first (lower) quartile, median, third (upper) quartile, and maximum of the median throughput losses published in the coexistence study.

Figure1: Adjacent Channel CLI median throughput loss (%) summary for Scenario-1 (FR-1) concluded in Rel-18 CLI

Figure 2: SBFD UL median throughput loss summary at cell edge and cell centre for Scenario-1

Figure 3: SBFD UL median throughput loss summary for Cell centre (Left) & Cell edge (Right) users for varying ACIR for Scenario-1

The following observations were made from the performance results with respect to both legacy TDD and SBFD as victims.

- The cell edge throughput degradation is worse than the average throughput degradation.

- The throughput degradation is due to the inter-BS ACI introduced by TDD-UL over SBFD, which increases as grid-shift (BS-to-BS distance) decreases in scenario 1.

- SBFD BS antenna configuration slightly impacts the throughput degradation.

- Major SBFD UL throughput degradation is observed cell edge uplink.

- For higher BS Tx power and lower grid-shifts the degradation is increased for cell edge throughput and average throughput.

- No throughput degradation is observed on the SBFD DL for both average throughput and cell edge throughput, respectively, for different BS Tx powers (ranging from 46 dBm to 53 dBm for FR1 and 30 dBm for FR2-1), grid-shifts (5% to 100%), and SBFD BS antenna configurations.

RAN4 had considered the case of {D, U} as an SBFD configuration instead of {D, U, D} SBFD configuration. But when we consider {D, U, D} with {40MHz,20MHz,40MHz} bandwidth, there is an additional protection of 40MHz for the leakage from the adjacent channel CLI. The study done as part of TR 38.858 [2] discussed the feasibility of analog filtering options for self-interference and co-channel interference due to co-site/co-sector in sections 9.3.1 and 9.3.2, respectively. It was dropped due to design complexity, other insertion loss due to a narrow roll-off from a few kHz to a few MHz, and limitations in supporting multi-carrier. However, this feasibility was not studied from an adjacent channel blocker perspective, considering the benefit of 40MHz filter roll-off due to {D, U, D} configuration for SBFD. For the SBFD symbol represented by {D, U, D}, Adjacent channel gNB-gNB CLI is seen in the UL sub-band of the SBFD symbol due to DL slots of legacy TDD from neighbouring gNBs overlapping with SBFD symbols. For the Legacy TDD symbols, the Adjacent channel gNB-gNB CLI is seen in UL symbols in the Flexible slot overlapping with the SBFD slot of neighbouring gNB. Since the configurations considered for the adjacent channel CLI for throughput analysis are different from what has been studied in TR 38.858 [2] there is a need for further study on network throughput impact due to adjacent channel co-existence.

Proposal 1: For the in-band blocking requirement, adopt a unified blocking threshold for both scenarios. For the grid shift, consider 10% grid shift.

Following BS Receiver Blocker Models and Receiver noise figure models for FR1 wide area BS considered for simulation.

- (i) Baseline BS Receiver Blocking with baseline noise figure model for FR1 wide area BS: P1=-43 dBm, P2=-25 dBm, F1=5 dB, F2=14 dB (default option in 38.858 Annex E).

- (i) Reduced blocker-based BS receiver blocking with modified noise figure P1=-37 dBm, P2=-25 dBm, F1=8 dB, F2=14 dB).

Since we have already considered {D, U, D} with {40MHz, 20MHz, 40MHz} bandwidth, there is an additional protection of 40MHz for the leakage from the adjacent channel CLI. Therefore, we have considered analog filtering options for adjacent interference reduction. Moreover, we have also considered an additional 3 dB noise figure to consider the noise introduced by this filter.

- Reduced blocker-based BS receiver blocking with modified noise figure, same simulation scenario as CASE-B above with additional beam nulling. Beam nulling is implemented at the co-channel gNBs and/or adjacent-channel gNBs of the SBFD network as proposed in RAN1 discussions as one of the gNB-gNB CLI handling schemes.

Observation 1: In addition to the Baseline Blocker Model, the reduced blocker model with a {D, U, D} SBFD configuration, analog filtering of adjacent interference, and a modified noise figure along with the same model incorporating CLI handling schemes should be further studied for SBFD performance evaluations in FR1 scenarios.

Proposal 2: For in-band blocking simulations, investigate both the reduced blocker model and the reduced blocker model with CLI handling mechanisms. Specifically, for UMa with a modified noise figure, use P1 = -37 dBm, P2 = -25 dBm, F1 = 8 dB, and F2 = 14 dB.

Proposal 3: Consider a 3 dB degradation in noise figure when using the reduced blocker model with additional filtering.

### Simulation results

#### In-band Blocking Results for Baseline BS Rx Model [CASE-A]

##### SBFD Aggressor

We consider a coexistence scenario where UMa deployments of two different operators are deployed in the same area and in adjacent channels. The first operator (victim) deploys a SBFD network, while the second operator (aggressor) also deploys a SBFD network. We are interested in evaluating the wideband interference power received by the UL of the SBFD operator from the DL of the SBFD aggressor. The default noise figure model defined in the 38.858 Annex E [2] is considered as baseline.

Figure 4: FR1, UMa -> UMa, in-band blocking, SBFD Aggressor

Table 3: FR1 SBFD UMa -> UMa conducted in-band blocking simulation results

|  | 100% grid-shift | 10% grid-shift |
| --- | --- | --- |
|  | 99% CDF | 99% CDF |
| Conducted in-band blocking (dBm) | -30.47 | -3.82 |

The in-band blocking requirement is defined for the 99%-tile of the CDF of the received UL wideband interference power. From Figure 4, it can be observed that the in-band blocking requirement is higher as the grid shift decreases. It can also be observed that for the 100% grid shift case, the 99 percentile of the in-band blocking power is 13.54 dBm, and for the 10% grid shift case, it is approximately 40.18 dB above the currently specified in-band blocking requirement (-43 dBm) for Wide Area BS, respectively.

Observation 2: For UMa deployments with SBFD as the aggressor network and a 100% grid shift, the 99th percentile of in-band blocking power is 13.54 dBm. With a 10% grid shift, the in-band blocking power is approximately 40.18 dB above the current threshold of -43 dBm.

Figure 5: FR1, UMa—different power levels, SBFD Aggressor

In Figure 5, we have plotted the distributions of received power levels, considering 10% grid shift for this simulation scenario. We have captured adjacent channel gNB DL power level, co-channel gNB DL power level, co-channel UE-UL power level, and wanted signal level. The results show that for 10% grid shift, adjacent channel interference power is dominating the total interference. This study is also available in our previous contribution [4].

Observation 3: For a 10% grid shift, adjacent channel CLI power dominates the total in-band blocking power. gNB-to-gNB CLI handling schemes or deployment restrictions are required to mitigate this impact.

Performance Comparison Between Different Grid Shifts

- (b)

Figure 6: CDF of (a) SINR (b)Spectral Efficiency

Figures 6 (a) and 6 (b) show the comparison of the SINR CDFs and spectral efficiency, respectively, at the victim receiver, including total noise and total interference (38.858 Annex E, E. 2.3.2.1) between the 10% and 100% grid shift scenarios. Figure 6(a) shows that at the 40 percentiles of SINR CDF, 100% grid shift shows a better SINR compared to 10% of grid shift by 16 dB. Similarly, Figure 6 (b) shows that for the 10% grid shift, only 26% of UEs have a spectral efficiency above 0.5 compared to the 51% of UEs for the 100% grid shift.

Observation 4: The SINR and spectral efficiency of the SBFD system decrease with an increasing grid shift percentage. For 10% grid shift cases, adjacent channel in-band interference significantly dominates, leading to performance degradation of the victim SBFD gNBs.

Proposal 4: For the FR1 Urban Macro scenario, increase the in-band blocking requirement (currently -43 dBm) for SBFD wide-area BSs to accommodate elevated levels of in-band blocking power.

Proposal 5:  For the 10% grid shift requirement, the interference power level and in-band blocking requirement should be evaluated after RAN1 studies the applicability of the CLI handling mechanism for adjacent channels or considers any relevant deployment restrictions.

##### TDD Aggressor

We consider a coexistence scenario where UMa deployments of two different operators are deployed in the same area and in adjacent channels. The first operator (victim) deploys a SBFD network, while the second operator (aggressor) deploys a TDD network. We are interested in evaluating the wideband interference power received by the UL of the SBFD operator from the DL of the TDD operator. The default noise figure model defined in the 38.858 Annex E [2] is considered as baseline.

Figure 7: FR1, UMa -> UMa, in-band blocking, TDD Aggressor

Table 4: FR1 SBFD UMa -> UMa conducted in-band blocking simulation results

|  | 100% grid-shift | 10% grid-shift |
| --- | --- | --- |
|  | 99% CDF | 99% CDF |
| Conducted in-band blocking (dBm) | -28.05 | -0.78 |

The in-band blocking requirement is defined for the 99%-tile of the CDF of the received UL wideband interference power. From Figure 4, it can be observed that the in-band blocking requirement is higher as the grid shift decreases. It can also be observed that for the 100% grid shift case, the 99 percentile of the in-band blocking power is 14.95 dBm, and for the 10% grid shift case, it is approximately 42.22 dB above the currently specified in-band blocking requirement (-43 dBm) for Wide Area BS, respectively.

Observation 5: For UMa deployments with a TDD operator as the adjacent network, in the 100% grid shift case, the 99th percentile of the in-band blocking power is 14.95 dBm. In the 10% grid shift case, it is approximately 42.22 dB above the currently specified in-band blocking requirement of -43 dBm.

Comparison of Inband Blocking Results for TDD as Aggressor vs SBFD as Aggressor

In the below results we show the comparison of in-band blocking power and performance metrics between simulation scenarios where TDD as aggressor adjacent network vs SBFD as aggressor adjacent network.

Figure 8: Comparison of in-band blocking power in case of SBFD vs TDD as aggressor.

Table 5: FR1 SBFD UMa -> UMa in-band blocking comparison [10%GS]

|  | TDD Aggressor | SBFD Aggressor |
| --- | --- | --- |
|  | 99% CDF | 99% CDF |
| Conducted in-band blocking (dBm) | -0.78 | -3.82 |

From the above CDF plots and 99% CDF values, the conducted in-band blocking for both the scenarios differs by 3 dB. For the case with TDD as aggressor, the 99 percentile of the in-band blocking power is 42.22 dB, and for the case with SBFD as aggressor, it is approximately 39.18 dB above the currently specified in-band blocking requirement (-43 dBm) for Wide Area BS, respectively.

Observation 6: For the case with TDD as the aggressor, the 99th percentile of the in-band blocking power is 42.22 dB. In the case with SBFD as the aggressor, it is approximately 39.18 dB above the currently specified in-band blocking requirement of -43 dBm for the wide-area BS.

#### In-band Blocking Results for reduced Blocker based BS Rx Model with Modified Noise Figure [CASE-B]

In this simulation scenario, we consider additional adjacent channel interference reduction using an analog filter. To consider the noise introduced by this filter, we have added an additional 3 dB noise figure (compared to the baseline noise figure in 38.858 Annex E) as P1 = -37 dBm, P2 = -25 dBm, F1 = 8 dB, and F2 = 14 dB. The objective here is to see the impact of the reduced blocker model on the improvement of SINR and spectral efficiency.

Figure 9: In band Blocking Power Levels in the Reduced Blocker Model, SBFD as Aggressor

In Fig. 9, we can observe that due to additional filter in the analog domain, the adjacent channel interference power level reduces significantly compared to the baseline model. The 99-percentile value of the in-band blocking power CDF reduces significantly for the reduced blocker model, as depicted in Table 6.

Table 6: FR1 SBFD UMa-UMa conducted in-band blocking simulation results with reduced blocker

|  | 10% grid-shift |  |
| --- | --- | --- |
|  | 99% CDF [Reduced Blocker] | 99% CDF [Baseline] |
| Conducted in-band blocking (dBm) SBFD Aggressor | -39.25 | -3.82 |
| Conducted in-band blocking (dBm) TDD Aggressor | -35.78 | -0.78 |

Observation 7: For the Urban Macro scenario, the 99th percentile of the in-band blocking power is -39.25 dBm in the 10% grid shift case for the reduced blocker model when SBFD is the aggressor, and -35.78 dBm when TDD is the aggressor. The in-band blocking power requirement was significantly reduced for the reduced blocker model.

#### Performance Evaluation with Reduced Blocker & CLI handling schemes.

(a)

(b)

Figure 10: (a) CDF of (b) SINR

Figures 10(a) and 10(b) show the comparison of the SINR CDFs and spectral efficiency, respectively, at the victim receiver, including total noise and total interference (38.858 Annex E, E. 2.3.2.1) for all the different receiver blocker models for the same grid shift of 10% in both the models for UMa.

Figure 10 (a) shows that at the 40 percentiles of SINR CDF, the reduced blocker model gives better SINR of 8.23 dB compared to the baseline model. Similarly, for the baseline model, only 26% of UEs have a spectral efficiency above 0.5 compared to the 44% of UEs in the reduced blocker model. The performance of the reduced blocker model increases significantly as the adjacent channel interference is blocked by the extra analog filters.

Observation 8: Simulations show an improvement of 8 dB in SINR with the reduced blocker model compared to the baseline model for UMa with a grid shift of 10%. Additionally, with the reduced blocker, 18-20% more UEs achieve a spectral efficiency greater than 0.5.

Reduced Blocker with Co-Channel gNB Beam Nulling

In this scenario, a beam nulling-based CLI handling scheme as proposed by RAN1 is used at the co-channel gNBs to reduce the interference caused by them to the SBFD network. We would like to see the gain in SINR and spectral efficiency due to beam nulling and the impact of adjacent CLI leakage.

Figure 10 shows that at the 40 percentiles of SINR CDF, the reduced blocker model with beam nulling gives better SINR of 6.18 dB compared to the same model without beam nulling. Similarly, for the reduced blocker model, 44% of UEs have a spectral efficiency of above 0.5 compared to the 55% of UEs in the reduced blocker model with beam nulling. The CLI handling scheme for the co-channel gNBs reduces the throughput loss percentage and increases the SINR as the co-channel interference is reduced by beam nulling.

Observation 9: Simulations show a 6 dB improvement in SINR, with 16-18% more UEs achieving spectral efficiency greater than 0.5, when using the reduced blocker model with the CLI handling mechanism, compared to the reduced blocker model without the CLI mechanism.

Observation 10: The throughput loss for the model with beam nulling is reduced. However, the results show that approximately 20% of the UEs still experience throughput loss, suggesting that adjacent channel interference leakage continues to dominate, even with the additional blocker.

Proposal 6: For the Urban Macro scenario with 10% grid shifts, the co-channel gNB CLI handling mechanism should be considered for SBFD networks.

Reduced blocker with Co-Channel + Adjacent-Channel gNB Beam Nulling

In this scenario, a beam nulling-based CLI handling scheme as proposed by RAN1 is used at the co-channel and adjacent channel gNBs to reduce the interference caused by them to the SBFD network. We would like to see the gain in SINR and spectral efficiency due to beam nulling and the impact of adjacent CLI leakage.

Figure 10 shows that at the 40 percentiles of SINR CDF, the reduced blocker model with adjacent channel beam nulling gives better SINR of 4.85 dB compared to the same model without beam nulling. Similarly, for the reduced blocker model, 40% of UEs have a spectral efficiency of above 1 compared to the 54% of UEs in the reduced blocker model with beam nulling. Also, Improvement of 18dB in SINR seen and ~40% more UEs with more than 0.5 Spectral efficiency in simulations with reduced blocker with CLI handling mechanism compared to baseline model. The CLI handling scheme for the adjacent channel gNBs reduces the throughput loss percentage and increases the SINR as the adjacent channel leakage interference is reduced by beam nulling.

Observation 11: Simulations show a 4.85 dB improvement in SINR, with approximately 15% more UEs achieving spectral efficiency greater than 1, when using the reduced blocker model with the CLI handling mechanism, compared to the reduced blocker model without the CLI mechanism for adjacent channel gNBs.

Observation 12: Simulations show an 18 dB improvement in SINR, with approximately 40% more UEs achieving spectral efficiency greater than 0.5, when using the reduced blocker model with the CLI handling mechanism, compared to the baseline model (CASE-A) without the analog filter or CLI handling mechanism.

#### Summary of Results

Table 7: FR1 UMa- UMa conducted in-band blocking simulation results

| Scenario | Aggressor | Model | Grid Shift | 99% CDF |
| --- | --- | --- | --- | --- |
| UMa | SBFD | Baseline | 10% | -3.82 |
|  |  |  | 100% | -30.46 |
|  |  | Reduced Blocker | 10% | -39.25 |
|  |  |  | 100% | -65.46 |
|  | TDD | Baseline | 10% | -0.78 |
|  |  |  | 100% | -28.05 |
|  |  | Reduced Blocker | 10% | -35.78 |
|  |  |  | 100% | -62.38 |

Table 8: FR1 UMi-UMi conducted in-band blocking simulation results [5]

| Scenario | Aggressor | Model | Grid Shift | 99% CDF |
| --- | --- | --- | --- | --- |
| UMi | TDD | Baseline | 10% | -16.03 |
|  |  |  | 100% | -32.30 |
|  |  | Reduced Blocker | 10% | -51.03 |
|  |  |  | 100% | -67.23 |

Observation 13: For the FR1 Urban Macro scenario with a large-area BS and 10% grid shift, the existing in-band blocking requirement of -43 dBm should be increased for both cases (TDD as the aggressor and SBFD as the aggressor) to address the elevated in-band blocking power levels. This is without considering any gNB-to-gNB CLI handling mechanisms, deployment scenario restrictions, or additional analog filters as proposed.

Observation 14: For FR1 Urban Macro scenarios with 10% grid shift, the in-band blocking power requirement is significantly reduced for the Reduced Blocker Model when using the additional analog filters as proposed.

Proposal 7: Establish a unified blocking requirement that accounts for both coexistence scenarios: legacy TDD systems in the adjacent channel and new SBFD systems in the adjacent channel.

Observation 15: For the FR1 Urban Macro scenario with a large-area BS and 10% grid shift, the existing in-band blocking requirement of -43 dBm should be increased for both cases (TDD as the aggressor and SBFD as the aggressor) to address the elevated in-band blocking power levels. This is without considering any gNB-to-gNB CLI handling mechanisms, deployment scenario restrictions, or additional analog filters as proposed.

Observation 16: For the FR1 Urban Micro scenario with a medium-area BS and 10% grid shift, the existing in-band blocking requirement of -38 dBm should be increased.

Observation 17: For both the FR1 Urban Macro and FR1 Urban Micro scenarios with 10% grid shift, the in-band blocking power requirement is significantly reduced for the Reduced Blocker Model when using the additional analog filters as proposed.

Proposal 8: For the FR1 Urban Micro scenario with 10% grid shifts, the existing in-band blocking requirement of -38 dBm should be increased for SBFD medium-area BSs to account for the elevated levels of in-band blocking power.

# Conclusion

In this contribution, we discuss about in-band blocking and receiver dynamic range degradation with the following proposals and observations.

Proposal 1: For the in-band blocking requirement, adopt a unified blocking threshold for both scenarios. For the grid shift, consider 10% grid shift.

Observation 1: In addition to the Baseline Blocker Model, the reduced blocker model with a {D, U, D} SBFD configuration, analog filtering of adjacent interference, and a modified noise figure along with the same model incorporating CLI handling schemes should be further studied for SBFD performance evaluations in FR1 scenarios.

Proposal 2: For in-band blocking simulations, investigate both the reduced blocker model and the reduced blocker model with CLI handling mechanisms. Specifically, for UMa with a modified noise figure, use P1 = -37 dBm, P2 = -25 dBm, F1 = 8 dB, and F2 = 14 dB.

Proposal 3: Consider a 3 dB degradation in noise figure when using the reduced blocker model with additional filtering.

Observation 2: For UMa deployments with SBFD as the aggressor network and a 100% grid shift, the 99th percentile of in-band blocking power is 13.54 dBm. With a 10% grid shift, the in-band blocking power is approximately 40.18 dB above the current threshold of -43 dBm.

Observation 3: For a 10% grid shift, adjacent channel CLI power dominates the total in-band blocking power. gNB-to-gNB CLI handling schemes or deployment restrictions are required to mitigate this impact.

Observation 4: The SINR and spectral efficiency of the SBFD system decrease with an increasing grid shift percentage. For 10% grid shift cases, adjacent channel in-band interference significantly dominates, leading to performance degradation of the victim SBFD gNBs.

Proposal 4: For the FR1 Urban Macro scenario, increase the in-band blocking requirement (currently -43 dBm) for SBFD wide-area BSs to accommodate elevated levels of in-band blocking power.

Proposal 5:  For the 10% grid shift requirement, the interference power level and in-band blocking requirement should be evaluated after RAN1 studies the applicability of the CLI handling mechanism for adjacent channels or considers any relevant deployment restrictions.

Observation 5: For UMa deployments with a TDD operator as the adjacent network, in the 100% grid shift case, the 99th percentile of the in-band blocking power is 14.95 dBm. In the 10% grid shift case, it is approximately 42.22 dB above the currently specified in-band blocking requirement of -43 dBm.

Observation 6: For the case with TDD as the aggressor, the 99th percentile of the in-band blocking power is 42.22 dB. In the case with SBFD as the aggressor, it is approximately 39.18 dB above the currently specified in-band blocking requirement of -43 dBm for the wide-area BS.

Observation 7: For the Urban Macro scenario, the 99th percentile of the in-band blocking power is -39.25 dBm in the 10% grid shift case for the reduced blocker model when SBFD is the aggressor, and -35.78 dBm when TDD is the aggressor. The in-band blocking power requirement was significantly reduced for the reduced blocker model.

Observation 8: Simulations show an improvement of 8 dB in SINR with the reduced blocker model compared to the baseline model for UMa with a grid shift of 10%. Additionally, with the reduced blocker, 18-20% more UEs achieve a spectral efficiency greater than 0.5.

Observation 9: Simulations show a 6 dB improvement in SINR, with 16-18% more UEs achieving spectral efficiency greater than 0.5, when using the reduced blocker model with the CLI handling mechanism, compared to the reduced blocker model without the CLI mechanism.

Observation 10: The throughput loss for the model with beam nulling is reduced. However, the results show that approximately 20% of the UEs still experience throughput loss, suggesting that adjacent channel interference leakage continues to dominate, even with the additional blocker.

Proposal 6: For the Urban Macro scenario with 10% grid shifts, the co-channel gNB CLI handling mechanism should be considered for SBFD networks.

Observation 11: Simulations show a 4.85 dB improvement in SINR, with approximately 15% more UEs achieving spectral efficiency greater than 1, when using the reduced blocker model with the CLI handling mechanism, compared to the reduced blocker model without the CLI mechanism for adjacent channel gNBs.

Observation 12: Simulations show an 18 dB improvement in SINR, with approximately 40% more UEs achieving spectral efficiency greater than 0.5, when using the reduced blocker model with the CLI handling mechanism, compared to the baseline model (CASE-A) without the analog filter or CLI handling mechanism.

Observation 13: For the FR1 Urban Macro scenario with a large-area BS and 10% grid shift, the existing in-band blocking requirement of -43 dBm should be increased for both cases (TDD as the aggressor and SBFD as the aggressor) to address the elevated in-band blocking power levels. This is without considering any gNB-to-gNB CLI handling mechanisms, deployment scenario restrictions, or additional analog filters as proposed.

Observation 14: For FR1 Urban Macro scenarios with 10% grid shift, the in-band blocking power requirement is significantly reduced for the Reduced Blocker Model when using the additional analog filters as proposed.

Proposal 7: Establish a unified blocking requirement that accounts for both coexistence scenarios: legacy TDD systems in the adjacent channel and new SBFD systems in the adjacent channel.

Observation 15: For the FR1 Urban Macro scenario with a large-area BS and 10% grid shift, the existing in-band blocking requirement of -43 dBm should be increased for both cases (TDD as the aggressor and SBFD as the aggressor) to address the elevated in-band blocking power levels. This is without considering any gNB-to-gNB CLI handling mechanisms, deployment scenario restrictions, or additional analog filters as proposed.

Observation 16: For the FR1 Urban Micro scenario with a medium-area BS and 10% grid shift, the existing in-band blocking requirement of -38 dBm should be increased.

Observation 17: For both the FR1 Urban Macro and FR1 Urban Micro scenarios with 10% grid shift, the in-band blocking power requirement is significantly reduced for the Reduced Blocker Model when using the additional analog filters as proposed.

Proposal 8: For the FR1 Urban Micro scenario with 10% grid shifts, the existing in-band blocking requirement of -38 dBm should be increased for SBFD medium-area BSs to account for the elevated levels of in-band blocking power.

# Reference

- R4-2502380 Way Forward for [114][307] NR_duplex_evo_BSRF

- TR 38.858, “Study on evolution of NR duplex operation,” 3GPP.

- R1-2403894, Discussion on gNB-gNB CLI handling, Tejas Networks Ltd.

- R4-2415046, Views on modification of existing Rx requirements for SBFD, Tejas Networks Ltd.

- R4-2417789, Views on modification of existing Rx requirements for SBFD, Tejas Networks Ltd.