# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503302

# Wuhan, Hubei, China, 07th – 11th April, 2025

Source: 	Xiaomi

Title: 	Discussion on RRM core requirements for LP-WUS/WUR

Agenda Item:	7.26.4.2

Document for:	Discussion

# 1Introduction

In RAN4#114 meeting, the RRM impacts on low-power wake-up signal and receiver for NR (LP-WUS/WUR) was discussed and a WF was approved [1]. In this contribution, we provide our further analysis on RRM impacts of LP-WUS/WUR features.

- Discussion

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

Based on the discussion in previous meetings, RRM measurement relaxation case#1 and case#3 has already be agreed. In case 1, i.e., fully offloading case, MR stops perform any RRM measurement and LP-WUR performs serving cell measurement. There is still FFS on the condition and details for the MR neighboring cell measurement. From our perspective, this is the case where UE could perform the greatest level of relaxation measurement. We prefer not to consider any neighbor cell measurement in this case.

Proposal 1: RAN4 not to consider any neighbor cell measurement in RRM relaxation case 1, except for the higher priority frequency layer search.

In case 2, MR has RRM relaxation on serving cell measurement (no neighbour cell measurement is needed) and LP-WUR performs serving cell measurement. We think this is the case UE in the cell center, when UE does not need to measure neighbor cell, as well as no higher priority inter-f or inter-RAT configured. From our perspective, this is the typical case for starting LP-WUR monitoring and no need to start MR measurement. So, we prefer to de-prioritize this case.

Proposal 2: It is suggested to de-prioritize RRM relaxation case 2.

Issue 1-1-5: Criteria (entry/exit conditions) for fully offloading case and MR RRM measurement relaxation

Issue 1-1-6: Criteria (entry/exit conditions) for LP-WUS monitoring

Issue 1-1-7: On jointly consideration on Criteria (entry/exit conditions) for fully offloading/MR RRM measurement relaxation and LP-WUS monitoring

As we understand that the LP-WUR measurement is initially introduced to guarantee the performance of LP-WUS monitoring, we think the LR measurement is not expected to start later than the LP-WUS monitoring. Also, we see little advantage in early starting of the LR measurement. If UE still use MR for paging reception, it is reasonable to use MR for RRM measurements as MR also need measurement for T/F synchronization. In this case, the LR measurement results seems useless. So, we prefer to define the same entry/exit condition of LP-WUR measurement and LP-WUS monitoring. For the entry/exist conditions for LP-WUS monitoring, we understand that this issue is under discussion in RAN1 and RAN2. We think RAN4 should wait for RAN1/2 progress. RAN4’s discussion on entry/exit conditions for LP-WUS monitoring can be triggered by other working group(s) if needed.

RAN4 also discuss the entry/exit conditions for MR RRM measurement relaxation. Based on our analysis in the previous section, we think it is more reasonable that the MR RRM relaxation is executed when LR is used for serving cell measurement. If the LR is not doing measurement, the evaluation of measurement relaxation can only depend on serving cell measurement in MR, which is aligned with Rel-16 power saving functionality. If the LR is doing RRM measurement while MR is also doing normal RRM measurement, there seems little benefit but more power consumption. So, from our perspective, the entry/exit conditions for MR RRM measurement relaxation should also be the same as LP-WUR serving cell measurement. However, we understand RAN2 is currently working on the criteria design, it is suggest RAN4 to focus on the relaxation cases.

Proposal 3: It's suggested that RAN4 concentrate on requirements discussion, while the entry/exit criteria for fully offloading/MR RRM measurement relaxation/ LP-WUS monitoring should be based on RAN2 design.

Issue 1-1-11: Considerations on higher priority frequency layer

In accordance with the legacy measurement procedure for MR only UE, measurements for neighbor cell could be divided into two parts. One is for intra-frequency measurements and NR inter-frequency or inter-RAT frequencies measurements with equal or lower priority, and the decision for MR to conduct these measurements hinges on the results from the serving cell measurements. When the signal quality of the serving cell reduce to certain threshold, the intra-frequency measurements and NR inter-frequency or inter-RAT frequencies measurements with equal or lower priority will be triggered. The other one is for NR inter-frequency or inter-RAT frequencies with higher priority, and the decision for MR to conduct the measurements depend on NW configuration. In this case, as long as NW configuration indicates the priority information, UE needs to perform measurements of higher priority NR inter-frequency or inter-RAT frequencies.

Observation 1: According to existing cell selection/re-selection procedure:

- The initiation of intra-frequency measurements, as well as NR inter-frequency or inter-RAT frequency measurements with equal or lower priority, depends on the serving cell measurement results from the MR;

- The initiation of NR inter-frequency or inter-RAT frequencies measurements with higher priority depends on NW configuration.

We think the handling on NR inter-frequency or inter-RAT frequencies measurements with higher priority should also follow the legacy.

- Proposal 4: When NR inter-frequency or inter-RAT frequencies measurements with higher priority are configured by NW, MR perform relaxed higher priority frequency layer search in RRM relaxation case 1 and case 3.

- Detailed LP-WUR requirements

Issue 1-2-3: On requirements for entry/exit criteria(threshold) evaluation for WUS paging monitoring/Fully Offloading (Case 1)/MR RRM relaxation (Case 3) Agreement: When both MR and LR threshold are configured, MR or LR evaluates whether its own entry/exit threshold is satisfied separately. MR and LR evaluation requirements are defined separately. Issue 1-2-4-2-2: On single or multiple LR evaluation requirements Agreement: Define a single LR evaluation requirement for both LR entry and exit threshold evaluation for LP-WUS monitoring, case 1 and case 3, i.e., LR evaluation requirement is not differentiated on entry or exit; or on different cases. Note: RAN2’s discussion on timer will not impact on RAN4’s evaluation requirement. Issue 1-2-4-2-3: On how to define LR evaluation requirements Agreement: LR evaluation duration is [x1 samples]*LP-SS (for OOK LR) or [y1 samples] *LO (for SSB LR), assuming x or y samples are used to satisfy accuracy requirement and x1 > x and y1>y. Number x and y are defined as measurement requirement as [x samples]*LP-SS (for OOK LR) or [y samples] *LO (for SSB LR)

The above attached agreements on detailed LP-WUR requirements were achieved in last RAN4#114 meeting. We will further discuss the open issues below.

During previous discussion, the measurement delay requirement in IDLE/Inactive mode for LP-WUR was divided into two cases. One is for LP-WUR based on LP-SS, the other one is for LP-WUR based on SSB. For the former case, agreement was achieved that measurement delay requirements for LP-WUR based on LP-SS shall be defined based on LP-SS periodicity. For the later case, i.e. LP-WUR based on SSB, it was agreed that the measurement delay requirements are defined based on LO periodicity.

For the case LP-SS is not configured, we understand only OFDM-based LP-WUR can work in this case. The OFDM-based LP-WUR obtains synchronization based on SSB. Basically, we think it is quite similar as MR. Also, the current agreement, i.e. based on LO periodicity, also aligns with measurement delay requirements for MR. As no upper bound has been defined for MR, we prefer not to define upper bound for LR either.

Proposal 5: RAN4 not to define upper bound for the measurement delay requirements for LP-WUR when LP-SS is not configured.

For the case LP-SS is configured, the agreement was achieved that measurement delay requirements for LP-WUR based on LP-SS shall be defined based on LP-SS periodicity. However, based on the latest RAN1 progress, we can see that the range of LP-SS periodicity is still open. There is possibility that the LP-SS periodicity can be configured to 80ms or 160ms, which is comparable to the periodicity of SSB. We prefer to set a lower limit for the measurement delay requirements in case the periodicity of LP-SS can be configured to small value like 80ms or 160ms. Or, if majority companies can accept, we also support to define the measurement delay requirements based on max(LP-SS, LO periodicity).

Proposal 6: In case the periodicity of LP-SS can be configured to small value like 80ms or 160ms, RAN4 to

- Option 1: define the lower limit for measurement delay requirements;

- Option 2: define the measurement delay requirements based on max(LP-SS periodicity, LO periodicity).

Based on the RAN4 discussion progress, RAN4 agreed to define single set of LR evaluation requirement for the transition between different cases, while the applicability of LR evaluation requirements is still under discussion.

The agreements on entry and exit conditions reached under current RAN2 discussions are summarized below.

| Case | Entry conditions | Exit conditions |
| --- | --- | --- |
| LP-WUS monitoring | MR threshold and optional LR threshold If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). | LR threshold |
| RRM measurement fully offloading (Case 1) | MR threshold and optional LR threshold | LR threshold |
| RRM measurement relaxation (Case 3) | For serving cell RRM relaxation: - MR threshold and optional LR threshold - If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation | FFS |

Taken legacy case into consideration, the tansition between cases including:

- from RRM measurement fully offloading case to legacy case

- from RRM measurement fully offloading case to RRM measurement relaxation case

- from RRM measurement relaxation case to legacy case

- from legacy case to RRM measurement relaxation case

- from legacy case to RRM measurement fully offloading case

- from RRM measurement relaxation case to RRM measurement fully offloading case

- from LP-WUR monitoring case to legacy case

- from legacy case to LP-WUR monitoring case

We think it is straightforward that the LR evaluation requirements should be applicable for ①, ②, ⑦, as the corresponding exit condition is based on LR threshold.

For ③ and ⑥, RAN2 has not decided the corresponding exit condition and the corresponding entry condition can optionally include LR threshold. Based on this current discussion status, we think at least the LR evaluation requirements should be applicable when NW configure LR threshold.

For ④, ⑤ and ⑧, We understand LR evaluation requirements applicability is related to the LR status. In last meeting, RAN4 achieved the agreements that when and how to turn on LR for serving cell measurement is up to UE implementation. We understand the LR evaluation requirement should be applicable for the case UE evaluate whether the LR measurement result is above the configured threshold. And the LP-WUR status has to be “on” to obtain the LR measurement result. If RAN4 agreed not to restrict when and how UE to turn on LR, we think it is reasonable that the LR evaluation requirement is not applicable for the case LR is “ON” at legacy state.

Proposal 7: The LR evaluation requirement is applicable to the following transition evaluation:

- from RRM measurement fully offloading case to legacy case

- from RRM measurement fully offloading case to RRM measurement relaxation case

- from RRM measurement relaxation case to legacy case

- from RRM measurement relaxation case to RRM measurement fully offloading case

- from LP-WUR monitoring case to legacy case

Proposal 7: The LR evaluation requirement is not applicable to the following transition evaluation:

- from legacy case to RRM measurement relaxation case;

- from legacy case to RRM measurement fully offloading case;

- from legacy case to LP-WUR monitoring case.

- On MR wake up delay related requirements for LP-WUR

The wake up delay related requirements has been discussed for several meetings. In this meeting, a related LS [2] was sent from RAN1. It is noted that RAN1 has agreed to define the wake-up delay and the definition of wake-up delay is the minimum gap time between LP-WUS reception and MR to start PDCCH monitoring.

Agreement in RAN1#120 For UE capability report on the wake-up delay: - Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[70ms], [500ms] and [900ms]}. - The reported values assume SSB periodicity of 20ms, where [70ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization. - FFS: translation of wake-up delay for different SSB periodicities - FFS: different sets of 3 candidate value for different SSB periodicities

It is clear that the wake-up delay define in RAN1 considered the MR synchronization time. So, from our perspective, RAN4 does not need to define the wake-up delay requirement.

Proposal 8: RAN4 to follow RAN1’s decision on the wake-up delay definition, and no need to define additional requirements.

Besides, RAN1 raised two questions in the LS. The first one is about the number of SSBs used for MR synchronization. Currently, RAN1 used two numbers to derive the MR wake-up delay. We understand that the two sets of numbers are for different sleep mode, e.g. deep sleep and ultra deep sleep. RAN4 didn't differentiate requirements for different sleep mode before, so single number is preferred from our side. For the second question, we think it is fine to assume the same number of SSBs for MR synchronization for different SSB periodicities, as normally the delay requirement defined in RAN4 is a function of the periodicity of reference signal.

Proposal 9: It is suggested to use 5 number of SSBs for MR synchronization for SSB periodicity of 20ms.

Proposal 10: It is reasonable to assume the same number of SSBs for MR synchronization for different SSB periodicities.

- Accuracy

During discussion in previous meeting, it was agreed to wait RAN1 progress in target SNR/SINR and use the RAN1 agreed target SNR/SINR as baseline for RRM simulation assumption. Meanwhile, companies still have different understanding on whether/how to define the accuracy. From our perspective, we support the idea that the accuracy discussed here is for simulation assumption and no need to define dedicated accuracy requirement in performance section. As per the existing serving cell measurement requirements defined in RRC_IDLE/INACTIVE state, no corresponding measurement accuracy requirement introduced. Since we are discussing RRM requirements for LP-WUR in RRC_IDLE/INACTIVE state, we prefer to follow the same logic as for MR.

Proposal 11: RAN4 to determine the measurement accuracy in RRC_IDLE/INACTIVE state for simulation purpose, and NO need to define dedicated accuracy requirement in the performance section.

- LP-WUR requirements at RRC_CONNECTED mode

In RRC_CONNECTED mode, as clearly described in the objectives, UE RRM/RLM/BFD/CSI measurements are performed by MR. So, we think there should be no RAN4 impact of LP-WUS/WUR features in RRC_CONNECTED mode expected. However, we are fine not to jumps to conclusions in this early stage.

Proposal 12: No RAN4 impact of LP-WUS/WUR features in RRC_CONNECTED mode is expected.

- Conclusion

Proposal 1: RAN4 not to consider any neighbor cell measurement in RRM relaxation case 1, except for the higher priority frequency layer search.

Proposal 2: It is suggested to de-prioritize RRM relaxation case 2.

Proposal 3: It's suggested that RAN4 concentrate on requirements discussion, while the entry/exit criteria for fully offloading/MR RRM measurement relaxation/ LP-WUS monitoring should be based on RAN2 design.

Observation 1: According to existing cell selection/re-selection procedure:

- The initiation of intra-frequency measurements, as well as NR inter-frequency or inter-RAT frequency measurements with equal or lower priority, depends on the serving cell measurement results from the MR;

- The initiation of NR inter-frequency or inter-RAT frequencies measurements with higher priority depends on NW configuration.

- Proposal 4: When NR inter-frequency or inter-RAT frequencies measurements with higher priority are configured by NW, MR perform relaxed higher priority frequency layer search in RRM relaxation case 1 and case 3.

Proposal 5: RAN4 not to define upper bound for the measurement delay requirements for LP-WUR when LP-SS is not configured.

Proposal 6: In case the periodicity of LP-SS can be configured to small value like 80ms or 160ms, RAN4 to

- Option 1: define the lower limit for measurement delay requirements;

- Option 2: define the measurement delay requirements based on max(LP-SS periodicity, LO periodicity).

Proposal 7: The LR evaluation requirement is applicable to the following transition evaluation:

- from RRM measurement fully offloading case to legacy case

- from RRM measurement fully offloading case to RRM measurement relaxation case

- from RRM measurement relaxation case to legacy case

- from RRM measurement relaxation case to RRM measurement fully offloading case

- from LP-WUR monitoring case to legacy case

Proposal 7: The LR evaluation requirement is not applicable to the following transition evaluation:

- from legacy case to RRM measurement relaxation case;

- from legacy case to RRM measurement fully offloading case;

- from legacy case to LP-WUR monitoring case.

Proposal 8: RAN4 to follow RAN1’s decision on the wake-up delay definition, and no need to define additional requirements.

Proposal 9: It is suggested to use 5 number of SSBs for MR synchronization for SSB periodicity of 20ms.

Proposal 10: It is reasonable to assume the same number of SSBs for MR synchronization for different SSB periodicities.

Proposal 11: RAN4 to determine the measurement accuracy in RRC_IDLE/INACTIVE state for simulation purpose, and NO need to define dedicated accuracy requirement in the performance section.

Proposal 12: No RAN4 impact of LP-WUS/WUR features in RRC_CONNECTED mode is expected.

- Reference

- R4-2502600, WF on RRM requirements for NR_LPWUS, Vivo

- R1-2501624, LS on the wake-up delay for LP-WUS operation in IDLE/INACTIVE mode, RAN1