3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503313

Wuhan, China, April 7th – 11th, 2025

Source: 	Xiaomi

Title: 	Discussion on other RRM requirement on Rel-19 MIMO phase 5

Agenda Item:	7.22.3.2

Document for:	Discussion

# 1 Introduction

In this contribution, we will mainly discuss the following issues for Rel-19 MIMO phase 5.

- UL TCI state activation in Asymmetric DL sTRP/UL mTRP - Two TAs in Asymmetric DL sTRP/UL mTRP

- Discussion

2.1 Asymmetric DL sTRP/UL mTRP

Issue 2-3: Whether to define UL TCI state activation requirement based on SRS? Way forward: Further discuss the two proposals and down-select one proposal in the next meeting: - Proposal 1 (Qualcomm, Huawei, MediaTek, Samsung, Apple, QC): No RRM impacts on TCI state switching requirement. - Proposal 3 (Xiaomi, Nokia): - Legacy DCI based UL TCI state activation requirement can be re-used for UL TRP. - RAN4 only to define MAC CE based UL TCI state activation requirement based on known TCI. The legacy MAC CE based delay requirement can be used as baseline. - For DL-TRP, legacy UL TCI state activation requirement based on DL-RS can be re-used. no need to define SRS based ULTCI state activation delay. - Further refine the wording for P3 in the WF, if needed

In last meeting, there is common understanding that there are two scenarios:

Agreement: - Scenario#1: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s) - Scenario#2: if UE is not configured with PL offset in joint/UL TCI state(s), UE expect to receive SSB from UL TRP(s).

UL TCI state activation for scenario 1

For scenario 1, SRS will be configured as source RS for UL-only TRP. In legacy RAN4, no requirement is defined when SRS will be configured as source RS for UL TCI state activation. We are fine not to define SRS based TCI state activation requirement for UL-only TRP.

Proposal 1: For scenario 1, Fine not to define SRS based TCI state activation requirement for UL-only TR.

If the target TCI state is known, upon receiving PDSCH carrying MAC-CE activation command in slot n, UE shall be able to receive UE-dedicated PDCCH/PDSCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc) / NR slot length. The UE shall be able to receive UE-dedicated PDCCH/PDSCH with the old TCI state until slot n+ THARQ + 3Nslotsubframe,µ where THARQ (in slot) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]; - Tfirst-SSB is time to first SSB transmission after MAC CE command is decoded by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state - TSSB-proc = 2 ms; - TOk = 1 if target TCI state is not in the active TCI state list for PDSCH/PDCCH, 0 otherwise.

For scenario 2, UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), SSB can be used for UL TCI state activation. Therefore, legacy UL TCI state requirement can be re-used for UL TRP.

Proposal 2: For scenario 2, legacy SSB based UL TCI state activation requirement can be re-used.

Two TA

Issue 2-1: Whether RAN4 need to consider the scenario when UE is/isn’t configured with pathloss offset? Agreement: For Scenario#1: RAN4 define the timing requirements for FR1, FFS on FR2. For Scenario#2, FFS on whether to reuse the legacy timing requirements (Rel-18 m-DCI two TA with updated or not). RAN4 common understanding: the RAN1 agreements in RAN1#118bis “One downlink reference timing is supported and applied to both TAGs.” are currently only applied for Scenario #1. - Scenario#1: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s) - Scenario#2: if UE is not configured with PL offset in joint/UL TCI state(s), UE expect to receive SSB from UL TRP(s). Note 1: the scenario 1 and 2 are based on RAN1 LS. Note 2: The above agreements apply when 2TA are configured.

In last meeting, RAN4 sent to LS to RAN1 to check whether one downlink reference timing can be applied for scenario 2.

Based on RAN1 LS (R1-2410870), RAN4 discussed timing requirement for two TA enhancement for asymmetric DL sTRP/ UL mTRP. RAN4 would like to check with RAN1 whether RAN1 agreements in RAN1#118bis “One downlink reference timing is supported and applied to both TAGs” are applied for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s). 2. Actions: To: RAN WG1 ACTION: RAN4 respectfully requests RAN1 to provide feedback on the above question.

From our understanding, one DL timing reference is designed for scenario 1 and can’t apply for both DL and UL TRPs for scenarios 2. In Rel-19, an associated TAG ID will also be configured for joint/UL TCI state(s). For PUCCH/PUSCH/SRS transmission with two TAs, two DL reference timing will be applied for each TAGs respectively. Therefore, legacy multi-DCI based two TA can be re-used.

For Rel-18 intra-cell mDCI PRACH transmission with two TAs, coresetPoolIndex is used. While coresetPoolIndex will be removed as agreed in RAN1 for Rel-19. The reference timing for PRACH needs to be updated.

The reference point for PUCCH/PUSCH/SRS/PRACH, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState [TS 38.331] having the same TAG as the uplink signal.

Proposal 3: For scenario 2, the reference point for PUCCH/PUSCH/SRS/PRACH, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState [TS 38.331] having the same TAG as the uplink signal.

- Conclusion

In this contribution, we provide the following proposals：

Proposal 1: For scenario 1, Fine not to define SRS based TCI state activation requirement for UL-only TR.

Proposal 2: For scenario 2, legacy SSB based UL TCI state activation requirement can be re-used.

Proposal 3: For scenario 2, the reference point for PUCCH/PUSCH/SRS/PRACH, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState [TS 38.331] having the same TAG as the uplink signal.

- Reference