3GPP TSG-RAN WG4 Meeting #114bis																		R4-2503318

Wuhan, Hubei, China, 07th – 11th April, 2025

Source: 	Xiaomi

Title: 	Discussion on PC2 RedCap RSD

Agenda item:	7.6.1

Document for:	Discussion

- Introduction

This contribution provides our consideration on how to define the requirements for 1R PC2 redcap UE.

- Discussion

In the last meeting’s WF [1], there’re the following two options to analyse/define the requirements.

Issue 1-2-3: REFSENS for 1Rx in FD-FDD

Agreement:

- Further discuss the following formats for further evaluations

  - Option 1: REFSENS (1Tx (23dBm) /1Rx) + RSD

  - Option 2: REFSENS (1Tx (26dBm) /2Rx) + ΔR

    - Evaluate the values of ΔR band by band

  - Note: Other options are not precluded

In our understanding, the 1R performance should be evaluated using option 2. But it may not need to be analyzed band by band because there’re too many bands. The PC3 1R approach which uses 2R->1R ΔR can be reused. For PC3 the value is borrowed from which is borrowed from LTE CAT1 [2] [3]. In theory, we don’t think 5MHz CBW has more benefit than the larger CBW, all of the FDD bands should have 3 dB relaxation than 2Rx. But in order to avoid the long discussion, we can accept the following general approach to reuse PC3 1R ΔR1R.

Table 7.3I.2-1: Single antenna port reference sensitivity allowance ΔR1R

| Operating band | Channel bandwidth (MHz) | ΔR1R (dB) |
| --- | --- | --- |
| TDD band | 5, 10, 15, 20 | 2.5 |
| FDD band | 5 | 2.5 |
| FDD band | 10, 15, 20 | 3.0 |

Proposal 1: Option 2 is used to evaluate the ΔR1R compared with 2R REFSENS, but there’s no need to evaluate band by band.  PC3 ΔR1R can be reused.

Regarding how to define the requirement, it seems both option 1 and option 2 work. Which one is better may largely depend on the spec readability. Option 2 can show the background of how the 1R REFSENS is derived, but option 1 may be more friendly to engineers because no need to add two delta values. However, for option 1, if there’s no general way to capture the delta value for the bands, we may prefer the approach of the HD-FDD approach that the REFSENS value is captured in the table instead of the RSD. The table length is the same but the final REFSENS value can be found directly in the table without calculation again, this calculation doesn’t help anything to readers.

Table 7.3I.2-3: HD-FDD RedCap UE with 1 Rx antenna port reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n1 | 15 | -97.5 | -94.3 | -92.5 | -91.2 |
|  | 30 |  | -94.7 | -92.7 | -91.4 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n2 | 15 | -96.3 | -93.1 | -91.3 | -90.0 |
|  | 30 |  | -93.5 | -91.5 | -90.2 |
|  | 60 |  | -93.8 | -91.7 | -90.5 |
| n3 | 15 | -95.3 | -92.1 | -90.3 | -89.0 |
|  | 30 |  | -92.5 | -90.5 | -89.2 |
|  | 60 |  | -92.8 | -90.7 | -89.5 |
| n5 | 15 | -96.3 | -93.1 | -91.3 | -90.0 |
|  | 30 |  | -93.5 | -91.5 | -90.2 |

Proposal 2: The 1R PC2 REFSENS is defined in the spec to capture the REFSENS value for the bands not the RSD or ΔR1R.

- Conclusion

This contribution provides our consideration on how to define the PC2 1R RSD for evaluation and spec drafting. We have the following proposals.

Proposal 1: Option 2 is used to evaluate the ΔR1R compared with 2R REFSENS, but there’s no need to evaluate band by band.  PC3 ΔR1R can be reused.

Proposal 2: The 1R PC2 REFSENS is defined in the spec to capture the REFSENS value for the bands not the RSD or ΔR1R.

- References

[1] R4-2502874, “WF on NR PC2 RedCap UE in FDD bands China Telecom”, Skyworks, ZTE, Qualcomm, CATT, Murata, Ericsson, Huawei, Apple, RAN4#114

[2] R4-2112385, “RedCap UE REFSENS requirements”, Apple, RAN4#100e

[3] R4-1610187, “Single Rx Cat 1 reference sensitivity”, Qualcomm, RAN4#81