# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503343

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	On RRM requirements of on-demand SSB SCell operation

Source: 	OPPO

Agenda item:	7.25.2.1

Document for:	Discussion

Introduction

In last meeting, a WF was approved for OD-SSB based requirements [1].

In this contribution, we will provide discussion on RRM requirements for On-demand SSB SCell operation.

Discussion

RAN4 has agreed to define requirements for OD-SSB based deactivated Scell measurement and Scell activation. In this paper, we mainly focus on the discussion on OD-SSB based deactivated SCell L3 measurement (Case 1). Most proposals about the fast mode windows below can also apply for case 2.

Open issues for Case 1

- Sub-issue 1-1 – The length of FMW Agreement: - The time window is defined with Tuncertain plus actual measurement duration. - Tuncertain equals Tmin(RAN1) plus additional processing/preparation time, if it allows. - The actual measurement duration equals min(Treport , Tidentify) from the end of uncertain time. - Treport is the first OD-SSB based L3 measurement reporting - Tidentify equals the minimum RAN4 OD-SSB meas. period requirement as follow. - A detected cell: OD-SSB based measurement period - A newly detectable cell: OD-SSB based cell identification period No OD-SSB measurement requirement applies in fast window if NW deactivates the OD-SSB earlier than the end of min(Treport, Tidentify). - Sub-issue 1-2: What’s the expected minimum interval(T2, sage 4) between multiple FMWs? Agreement: RAN4 to define measurement requirement based on T2. - When NW re-activates the OD-SSB within the T2, - Option 1: UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement. - Option 2: UE is required to perform “fast measurement” without cell search. - The UE is assumed to skip the PSS/SSS detection and/or SSB index acquisition. - Option 3: UE is required to perform FMW after T2. - When NW re-activates/re-configures the OD-SSB transmission after T2, UE performs the OD-SSB based fast L3 measurement within FMW - The UE is assumed to meet fast cell identification and measurement.

In last meeting, RAN4 agreed on the introduction of fast time window and expected minimum interval (T2) between multiple FMWs. The network and UE behaviours need to further discuss.

To ensure that UE can have enough number of OD-SSB samples to achieve the measurement requirement during one shot window, RAN4 defined the fast time window length. For network side, the ideal case is Scell activation command can be immediately transmitted once after UE report the measurement results based on OD-SSB measurement in the fast measurement windows. The fast measurement window based on OD-SSD activation and fast SCell activation based on fast measurement report is not expected to be inserted with a long-time gap.

Observation 1: The fast mode window(s) apply before the SCell activation. The network is expected to avoid long-time gap between OD-SSD activation command and SCell activation command.

For the case NW re-activates the OD-SSB within the T2, UE behaviour is still not decided.

In our view, when NW re-activates the OD-SSB within the T2, UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement within T2. But I think the intention of reactivation in such a short interval could be that network expect UE can quickly complete fast measurement and report again assuming the SCell is still a detected cell to UE. So UE can perform “fast measurement” without cell search immediately after T2, assuming skipping the PSS/SSS detection and/or SSB index acquisition.

This could be a well-balanced requirement to consider both network’s indication and UE power consumption. Otherwise, the network indication of OD-SSB will be totally ignored by UE within T2 and UE could miss one change to fulfil the purpose of fast SCell activation from the network’s expectation.

Proposal 1: When NW re-activates the OD-SSB within the T2, UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement within T2, and is required to perform “fast measurement” without cell search immediately after T2 assuming the SCell is still a detected cell.

It is a bit different from the case that NW re-activates/re-configures the OD-SSB transmission after T2 since the SCell could become a newly detectable cell the UE is assumed to meet fast cell identification and/or measurement. Similar to what we agreed on Tidentify as follows, whether the Scell is detected or not depends on the last report validity at the time point of re-activating/re-configuring the OD-SSB transmission.

Tidentify equals the minimum RAN4 OD-SSB meas. period requirement as follow.

      - A detected cell: OD-SSB based measurement period

      - A newly detectable cell: OD-SSB based cell identification period

Proposal 2: When NW re-activates/re-configures the OD-SSB transmission after T2, UE performs the OD-SSB based fast L3 measurement within FMW assuming the SCell could be a detected cell or a newly detectable cell.

- The start point of T2 is based on - Option 1: the time point Tend - Option 2: the latter point of L3 measurement reporting for FMW and Tend - Option 3: the latter point of latest L3 measurement reporting before NW re-indicating OD-SSB transmission and Tend - Option 4: the time point of OD-SSB deactivation - Other options are not precluded Where, Tend equals the time point of OD-SSB indication plus Tuncertain +Tidentify applies to option 1,2,3. - The duration is - Option 1: 5*max(measCycleSCell, DRX cycles) [– OD-SSB periodicity] - Option 2: 5* measCycle when measCycle =160 ms - Other options are not precluded The activation and deactivation of OD-SSB is fully up to NW.

If we agreed on the network and UE behaviour in different cases that NW re-activates/re-configures the OD-SSB transmission within or after T2, then we can continue the discussion on the start point and duration of T2.

From our side, we still prefer RAN4 to restrict the time gap between OD-SSD activation command and SCell activation command to avoid too long waiting time and too much UE power consumption on deactivated SCell measurement. The T2 can be configured with the periodic interval or predefined with the minimum interval. The start point can be the latter point of L3 measurement reporting for FMW and Tend, as the report of FMW could be earlier or latter than the end of Tidentify. We support option 2 and can also compromise to option 1 of the time point Tend, because the uncertainty of reporting can be avoided.

The interval of T2 can be configured by RRC together with OD-SSB related parameters, or can be predefined with the minimum value based on reporting validity time for known SCell definition. About the duration of T2, option 1 is more reasonable to us, since it follows current logic of defining the report validity time and considers both measCycleSCell and DRX cycle. Whether to remove one OD-SSB periodicity, we think it does matter too much.

Proposal 3: We prefer the start point of T2 is based on: Option 2 (the latter point of L3 measurement reporting for FMW and Tend) or Option 1 (the time point Tend).

Proposal 4: The duration of T2 is based on network configuration of both measCycleSCell and DRX cycles, e.g., 5*max (measCycleSCell, DRX cycles)

Besides, Network can expect to activate the SCell within T2 and the Scell can be taken as known cell. In this case, the activation procedure can be reduced as well. The T2 can also be used to restrict the minimum interval between multiple times OD-SSB activation/deactivation(reconfiguration), and it can equal to the expected max gap between OD-SSD activation command and SCell activation command. RAN4 need to discuss whether it is acceptable to network that T2 is taken as the expected max gap between OD-SSD activation command and SCell activation command. If agreed, RAN4 can specify it.

Proposal 5: T2 is also suggested to be taken as the expected max gap between OD-SSD activation command and SCell activation command. RAN4 can discuss whether and how to specify it.

Sub-issue 3-1: Lower bound for measurement in FMW - Background - The agreement in last meeting is as follow. Issue 1-2-1: OD-SSB based deactivated SCell measurement – measurement periodicity in fast mode measurement Agreement: For the case of CSSF = 1: - OD-SSB periodicity regardless of measCycleSCell and DRX cycle. - FFS whether a lower bound will be specified. FFS for the other cases of CSSF.. Wayforward: - Option 1: RAN4 not to define lower bound to measurement in FMW. - Option 2: Define a lower bound with OD-SSB periodicity as [10ms]. - The lower bound is only applied to measurement in FMW. Sub-issue 3-2: OD-SSB measurement in FMW(Case 1-1): single OD-SSB carrier with multiple legacy SSB carriers Agreement: When NW configures single OD-SSB carrier with multiple legacy SSB carriers, in FMW - Option 1: The CSSF only counts OD-SSB layers - Option 2: The CSSF of OD-SSB carrier counts OD-SSB layers and other layers Sub-issue 3-3: OD-SSB measurement in FMW(Case 1-2): Multiple active OD-SSB carriers Agreement: RAN4 to prioritize single OD-SSB carrier with multiple legacy SSB carriers in deactivated SCell measurement. - FFS multiple OD-SSB carriers

RAN4 has agreed to prioritize single OD-SSB carrier with multiple legacy SSB carriers in deactivated SCell measurement. We think if network wants to enable OD-SSB on different Carriers, TDM manner can be used. In R19, due to limited TUs and unexpected big changes on UE architectures including both RF and baseband, it is more realistic that RAN4 only defines single OD-SSB carrier under CA operation.

Proposal 6: In R19, it is more realistic that RAN4 only defines requirements for single OD-SSB carrier under CA operation.

When NW configures single OD-SSB carrier with multiple legacy SSB carriers in FMW, we prefer that the CSSF of OD-SSB carrier counts OD-SSB layers and other layers, because the basic assumption of UE implementation is not changed due to OD-SSB or normal SSB. From UE receiver’s perspective, the situation of processing SSB is the same to legacy, without differentiating OD-SSB from normal SSBs on other layers.

Proposal 7: When NW configures single OD-SSB carrier with multiple legacy SSB carriers, the CSSF of OD-SSB carrier counts OD-SSB layers and other layers in FMW, without differentiating OD-SSB from normal SSBs on other layers from UE receiver’s perspective.

For the lower bound to measurement period in FWM, we think it could be not a big problem since UE who can support OD-SSB feature on SCell is assumed to support CA operation as well. CSSF=1 is not a regular case, and even if it happens that UE performs OD-SSB measurement on single carrier at a specific duration, UE is able to meet the measurement requirements based on OD-SSB periodicity with the legacy measurement processing capabilities for CA.

Proposal 8: RAN4 not to define lower bound to measurement in FMW since CSSF = 1 is not a regular case for UE under CA operation.

Open issues for Case 2

Issue 2-2: Always-on SSB and OD-SSB time/frequency domain relation - Scenario 1: OD-SSB and always-on SSB are with same frequency, same offset but different periodicities. - Scenario 2: OD-SSB and always-on SSB are with same frequency, different offsets and periodicities. - Scenario 3: OD-SSB and always-on SSB are within different frequencies. - Scenario 4: OD-SSB and always-on SSB are with same frequency, different offset but same periodicities. Agreement: RAN4 to start from the following scenarios: - Scenario 1: OD-SSB and always-on SSB are with same frequency, same offset but different periodicities. - Scenario 4: OD-SSB and always-on SSB are with same frequency, different offset but same periodicities. - At least the offset between OD-SSB and AO-SSB equals the half of AO-SSB periodicity

RAN4 agreed to specify scenario 1 and 4. For other scenarios with different periodicities or different frequencies, more complicated UE implementation and requirements need to be investigated and discussed. We suggest RAN4 to down-prioritize the other scenarios including scenario 2 and 3 here,

Proposal 9: For case 2, suggest RAN4 to down-prioritize the other scenarios including scenario 2 and 3.

Issue 2-1: (Case 2) OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered Agreement: When NW indicates the OD-SSB transmission for deactivated SCell measurement(Case 2 Scenario 2), - Option 1: The same approach in Case 1 can be reused. - UE performs the OD-SSB based fast L3 measurement following OD-SSB periodicity for deactivated SCell within a time window. After the time window, UE fallbacks to a slow mode measurement(with DRX and measCycleSCell). - Option 2: - If the serving cell is unknown, the same approach in Case 1 can be reused. - If the serving cell is known, UE follows legacy requirements based on measCycleSCell. - The known condition reuses the principle of legacy known condition. - This scenario applies at least when UE sends the meas. reporting. - FFS the case if the L3 meas. report cannot be triggered. - The above requirement is applied at least when the OD-SSB and AO-SSB are on the same frequency and spatial relation.

For requirements of OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered, the biggest issue is whether to reuse the same approach in Case 1 at least including FMW and T2.

For the case if the serving cell is known, we think RAN4 should first reach common understanding whether and why network trigger OD-SSB. If UE followed legacy requirements based on measCycleSCell, then the purpose of enabling fast measurement and activation on deactivated SCell was compromised. In our view, no new issues are identified based on option 1, and it is more straightforward. The whole requirements of case 1 and case 2 can be more unified for UE.

Proposal 10: Support to use same approach in case 1 for case 2 when defining requirements of OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered.

Conclusions

In this paper, we discuss the open issues for OD-SSB based deactivated SCell L3 measurement in Case 1 and Case 2.

Observation 1: The fast mode window(s) apply before the SCell activation. The network is expected to avoid long-time gap between OD-SSD activation command and SCell activation command.

Proposal 1: When NW re-activates the OD-SSB within the T2, UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement within T2, and is required to perform “fast measurement” without cell search immediately after T2 assuming the SCell is still a detected cell.

Proposal 2: When NW re-activates/re-configures the OD-SSB transmission after T2, UE performs the OD-SSB based fast L3 measurement within FMW assuming the SCell could be a detected cell or a newly detectable cell.

Proposal 3: We prefer the start point of T2 is based on: Option 2 (the latter point of L3 measurement reporting for FMW and Tend) or Option 1 (the time point Tend).

Proposal 4: The duration of T2 is based on network configuration of both measCycleSCell and DRX cycles, e.g., 5*max (measCycleSCell, DRX cycles)

Proposal 5: T2 is also suggested to be taken as the expected max gap between OD-SSD activation command and SCell activation command. RAN4 can discuss whether and how to specify it.

Proposal 6: In R19, it is more realistic that RAN4 only defines requirements for single OD-SSB carrier under CA operation.

Proposal 7: When NW configures single OD-SSB carrier with multiple legacy SSB carriers, the CSSF of OD-SSB carrier counts OD-SSB layers and other layers in FMW, without differentiating OD-SSB from normal SSBs on other layers from UE receiver’s perspective.

Proposal 8: RAN4 not to define lower bound to measurement in FMW since CSSF = 1 is not a regular case for UE under CA operation.

Proposal 9: For case 2, suggest RAN4 to down-prioritize the other scenarios including scenario 2 and 3.

Proposal 10: Support to use same approach in case 1 for case 2 when defining requirements of OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered.

Reference

- R4-2502598, WF on RRM requirements for Netw_Energy_NR_enh_Part1