3GPP TSG-RAN4 Meeting #114BIS	R4-2503362

# Wuhan, Hubei, China, 7th – 11th April, 2025

Agenda item:			7.1.1.1.1

Source:	MediaTek

Title:	Discussion on MSD simplification of HPUE CA/DC UE

Document for:	Approval

# Introduction

In RAN4#114 meeting, the WF[1] has been approved and recommended to further discuss and encourage more companies’ input for each open issues i.e., MSD simplification and others for HP CA/DC UE.

Way forward [1][5]: ## 4.1 MSD rules Moderator Note: It is covered by R4-2502864 Not concluded in previous RAN4 meeting Agreement: - Eligibility criteria to HPUE MSD requirements specified using LUTs: Moderator: to be updated to accommodate 2UL - A valid 1UL HPUE configuration is specified in the two-band inter-band CA configuration tables (Table 5.5A.3.1-1a ~ 5.5A.3.1-1n), and, - A default power class 3 (1Tx) 1UL MSD test point is specified, and, - A 1UL HPUE MSD test point is not already specified in either Table 7.3A.4-2a, Table 7.3A.4-4a-1, Table 7.3A.4-4b, Table 7.3A.6-1a-1, or Table 7.3A.6-1b.

In this paper, we we share our view on the MSD simplification methodology according to 1UL and 2UL interference sources in Rel-19.

# MSD simplification

## 1UL HPUE MSD simplification

1UL MSD sources are Harmonic, Harmonic mixing and cross-band isolation problems will be impacted to the own Rx sensitivity levels. From the TS38.101-1 specification, we are listed up as follow.

  - MSD requirements for 1UL PC2 TDD band with 1Tx: RAN4 specify PC2 MSD requirement for TDD band in TS38.101-1. RAN4 assumed TDD PC2 UE with 1Tx.

  - MSD requirements for 1UL PC2 FDD band with 1Tx/2Tx: RAN4 specify PC2 MSD requirement for FDD band in TS38.101-1. RAN4 assumed FDD PC2 UE with 1Tx/2Tx antenna configuration.

  - MSD requirements for 1UL PC1.5 TDD band: RAN4 specify PC1.5 MSD requirement for TDD band in TS38.101-1. RAN4 assumed TDD PC1.5 UE with 2Tx only.

From the contribution [4], RAN4 already had many discussion and described the detail MSD statistics as shown in Figure 1 [4]. The figure will be used to define the MSD rules for 1UL MSD simplification based on the specified PC3 1UL MSD requirements as below.

Figure 1: MSD statistics of 1UL self-defense levels by cross-band isolation (diamonds), harmonics and harmonic mixing (crosses) based on TS38.101-1 v18.7.0. [4]

Based on the MSD statistical analyzed figure, RAN4 can accept the detail MSD LUT approach [3] from SKW as follow

In Rel-19, RAN4 can apply the following MSD simplification rules for new Rel-19 high power inter-band CA/DC band combinations within FR1 based on the 1UL PC3 MSD requirements due to uplink harmonic, harmonic mixing, and cross-band isolation interference, are specified as:

- PC21Tx MSD = PC31Tx MSD + PC21Tx,

- PC22Tx MSD = PC32Tx MSD + PC22Tx,

- PC1.5 MSD = PC32Tx MSD + PC1.5.

where the MSD values of PC3 1UL UE in TS38.101-1 shall be rounded to the closest integer value.

Table 1: 1UL Delta MSD Table according to power classes [3]

Proposal 1: For the 1UL MSD simplification rules of HPUE inter-band CA/DC UE, RAN4 can apply the above LUT based MSD simplification approach in section 2.1 due to harmonic, harmonic mixing and crossband isolation problems.

## 2UL HPUE MSD simplification

For 2UL MSD analysis, the IMD products’ power is mainly influenced by factors such as changes in transmit power, variations in the power class of each PA, the number of TX paths, and the IMD index of the frequency bands involved in intermodulation distortion. Also, HP PA improvement in linearity can bring about by the increase will be leaded to a reduction in product power.

In Agreed WF on MSD simplification rules in RAN4 #113 meeting [2], RAN4 derived detail MSD calculation principle for 10 cases for total CA UE power class in Table 2 according to the number of Tx Chain, PA combinations and expected interference levels in own Rx band.

Table 2. HPUE MSD analysed table from Rel-15 to Rel-19

| case |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ULCA power class | Type | PC3 2Tx | PC2 2Tx | PC2 2Tx | PC2 3Tx | PC2 2Tx | PC2 3Tx | PC1.5 2Tx | PC1.5 3Tx | PC1.5 3Tx | PC1.5 3Tx |
|  | PCmax | 23 | 26 | 26 | 26 | 26 | 26 | 29 | 29 | 29 | 29 |
| per band power class | Bd1 | PC3 1Tx | PC3 1Tx | PC3 1Tx | PC3 1Tx | PC2 1TX | PC2 1TX | PC2 1TX | PC2 1TX | PC3 1Tx | PC2 1TX |
|  | Bd2 | PC3 1Tx | PC3 1Tx | PC2 1Tx | PC2 2Tx | PC2 1Tx | PC2 2Tx | PC2 1Tx | PC2 2Tx | PC1.x 2Tx | PC1.5 2Tx |
| Valid Release |  | R15 | R17 | R17 | R18 | R19 | R19 | R19 | R19 | R18 | R19 |
| 1UL power | Bd1 | 23 | 23 | 23 | 23 | 26 | 26 | 26 | 26 | 23 | 26 |
|  | Bd2 | 23 | 23 | 26 | 2x23 | 26 | 2x23 | 26 | 2x23 | 2x26 | 2x26 |
| 2UL power | Bd1 | 20 | 23 | 23 | 23 | 23 | 23 | 26 | 26 | 23 | 26 |
|  | Bd2 | 20 | 23 | 23 | 2x20 | 23 | 2x20 | 26 | 2x23 | 2x24.7 | 2x23 |
| 2UL interference level |  | Ref | +3xorder | <+3xorder | <+3xorder | +3dB | +3dB | +3dB +3xorder | +3dB +3xorder | <4.7xorder -1.7 | ≤+3dB +3xorder |
| Unified 2UL interference level |  | Ref | 3xorder |  |  | 3dB |  | +6xorder -3 |  | <4.7xorder -1.7 | +6xorder-3 |

Based on SKW proposal [6] for 2UL MSD simplification, RAN4 can consider the IMD product level will be increased as proportional by the IMD order and we can ignore the asymmetry IMD of UL band to derive the MSD analysis.

To consistency of 1UL and 2UL MSD simplification rules, we prefer to define LUT per each power class according to the IMD order as follow

The original MSD proposed tables from [7]. It can be updated Table format as below to consistent with 1UL MSD table.

Table 2: 2UL Delta MSD Table for Power class 2 according to IMD order [7]

Table 3: 2UL Delta MSD Table for Power class 1.5 according to IMD order [7]

- PC2IMDx MSD = PC3IMDx MSD + PC2IMDx,

- PC1.5IMDx MSD = PC3IMDx MSD + PC1.5IMDx.

where the MSD values of PC3 2UL UE in TS38.101-1 shall be rounded to the closest integer value.

Proposal 2: For the 2UL MSD simplification rules of HPUE inter-band CA/DC UE, RAN4 can apply the above LUT based MSD simplification approach in section 2.2 due to dual uplink IMD problems.

Proposal 3: For the configurations where PC3 MSD is not specified, the 1UL or 2UL HPUE MSD requirements are specified in the legacy MSD tables 7.3A.4-2a, 7.3A.4-4a-1, 7.3A.4-4b, 7.3A.6-1a-1, 7.3A.6-1b.

Proposal 4: RAN4 can send LS to RAN5 to reduce the conformance test burden of HPUE band combinations with the following contents.

“RAN4 recommend to RAN5 for verifying of MSD requirements of high power inter-band CA/DC UE, RAN5 only consider the MSD requirements of the highest power class UE by 1Tx/2Tx configuration.”

- Conclusion

Observation 1: For harmonic products with option 1’s equation, the PC2 1TX calculated ∆MSD has a good fit for most cases, the PC2 2TX calculated ∆MSD is a little bit lower than the spec value at high PC3 MSD.

Proposal 1: For the 1UL MSD simplification rules of HPUE inter-band CA/DC UE, RAN4 can apply the above LUT based MSD simplification approach in section 2.1 due to harmonic, harmonic mixing and crossband isolation problems.

Proposal 2: For the 2UL MSD simplification rules of HPUE inter-band CA/DC UE, RAN4 can apply the above LUT based MSD simplification approach in section 2.2 due to dual uplink IMD problems.

Proposal 3: For the configurations where PC3 MSD is not specified, the 1UL or 2UL HPUE MSD requirements are specified in the legacy MSD tables 7.3A.4-2a, 7.3A.4-4a-1, 7.3A.4-4b, 7.3A.6-1a-1, 7.3A.6-1b.

Proposal 4: RAN4 can send LS to RAN5 to reduce the conformance test burden of HPUE band combinations with the following contents.

“RAN4 recommend to RAN5 for verifying of MSD requirements of high power inter-band CA/DC UE, RAN5 only consider the MSD requirements of the highest power class UE by 1Tx/2Tx configuration.”

## References

- R4-2502863, WF on TN HPUE, Samsung, Skyworks, OPPO, LGE, Apple, RAN4 #114

- R4-2420380, WF on MSD rule, Skyworks, Murata, Nokia, Qualcomm, RAN4#113

- R4-2501955, 1UL MSD simplifications rules for Rel-19, Skyworks, Nokia RAN4#113

- R4-2419712, 1UL HPUE NRCA MSD Observations, Skyworks, RAN4 #113

- R4-25xxxxx, RAN4 #114 Meeting report, RAN4 Chairman, RAN4 #114BIS

- R4-2500218, Simplification of HPUE 2UL DeltaMSD, Skyworks, RAN4#114

- R4-2501579, MSD beyond PC3, Qualcomm, RAN4#114