3GPP TSG-RAN WG4 Meeting #114bis		R4-2503382

# Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.25.2.3

Source: 	CMCC

Title: 			  Discussion on RRM requirement of OD-SIB1 for NES enhancement

Document for:	Discussion

- Introduction

In last meeting, RAN4 agreed that the potential RRM impact for OD-SIB1 will be discussed from this meeting [1].

Issue 2-1-1: RRM requirements impacts for OD-SIB1 Agreement: RAN4 to discuss the potential RRM impact for OD-SIB1 from the next meeting.

In this paper, we discuss related issues, and identify RAN4 impacts according to the progress of other working group. We list the relevant agreements below:

Agreement from RAN1 Agreement For discussion purpose, the following assumption will be used in RAN1 - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE Agreement For study of UL WUS design, consider at least PRACH as a starting point Agreement For UL WUS design for SIB1 request, at least dedicated PRACH resource is the assumption for further study in RAN1 Agreement For SIB1 in idle/inactive mode, prioritize RAN1 discussions on Case 2 and Case 3 - Case 2 (Option 1+B+X) is feasible from RAN1 perspective. - On target cell of UL WUS transmission: - Option 1: UE transmits UL WUS to NES Cell - Option 2: UE transmits UL WUS to Cell A - On configuration provision for UL WUS transmission - Option A: UE obtains the UL WUS configuration from NES Cell - Option B: UE obtains the UL WUS configuration from Cell A - On receiving of SIB1 - Option X: UE receives on-demand SIB1 from NES Cell - Option Y: UE receives on-demand SIB1 from Cell A Agreement - For further study of on-demand SIB1 in idle/inactive mode, it is assumed that always-on SSB is transmitted on the NES cell with on-demand SIB1. Agreement At least for Case-2: For further study of type 0 PDCCH monitoring occasions for on demand SIB1, after UE transmits the UL WUS in idle/inactive mode, RAN1 assumes following as a starting point: - Option 1: One or more type 0 PDCCH monitoring occasions for on demand SIB1 within a time window Agreement For further study of on-demand SIB1 in idle/inactive mode, enabling or disabling specific operations (e.g. paging, RACH receiving, OSI request …) of the NES cell with on-demand SIB1 is up to RAN2. Agreement For further work on on-demand SIB1 in idle/inactive mode, as a baseline, it is assumed that the transmit power control of UL WUS transmission based on PRACH is applied in the same manner as the legacy PRACH transmission. Agreement RAN1 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell. Agreement - RAN1 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline. Agreement - RAN1 recommends specifying on-demand SIB1 only for Case 2 (Option 1+B+X) in Rel-19 Conclusion - No further optimization in RAN1 on power control and power ramp-up procedure for UL WUS in R19 Agreement For the purpose of “to which cell does the configuration applies”, at least the following parameters are included in the UL-WUS configuration. Purpose; Parameters. To which cell does the configuration applies; NES-CellId; PhysCellId. ARFCN-ValueNR. Agreement For type 0 PDCCH monitoring occasions of on demand SIB1, searchSpaceZero and controlResourceSetZero for on-demand SIB1 are provided from UL WUS configuration if SSB on NES cell is on sync raster and K_SSB is not equal to 30 in FR1 or 14 in FR2. Agreement For The repetition periodicity of SIB1 within the time window of on-demand SIB1,: - Up to NW implementation (no change to existing specification) Agreement Search space for RAR in response to UL WUS on a NES Cell can be provided by the UL WUS configuration. If not, follow the search space zero for the NES Cell. Agreement The parameter “n-TimingAdvanceOffset” is included in the UL-WUS configuration. Agreement CORESET0 of NES cell is used for RAR CORESET of that NES cell. Agreement The reference time point to determine the window starting time for on-demand SIB1 is based on the RAR window for UL WUS wherein UE successfully received a RAR. - FFS: Use starting or ending slot of the RAR window as reference time The duration of the time window for on-demand SIB1 is indicated in the UL WUS configuration. - Unit of the duration is in slot Conclusion There is no RAN1 consensus to support UL WUS repetition in R19. Agreement The reference time point to determine the window starting time for on-demand SIB1 is based on the starting slot of the RAR window for UL WUS wherein UE successfully received a RAR Agreement The time offset between the reference time point and the starting time for the time window of on-demand SIB1 is indicated in the UL WUS configuration. - Unit of the time offset is in slot Conclusion There is no RAN1 consensus to support the following in Rel-19: Support joint PRACH request to trigger both OD-SIB1 and a subset of OD-OSI, e.g. SIB2/SIB3/SIB4. Agreement from RAN2 - If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission. - After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell. - After Rel-19 NES UEs camp in NES cell, the UE behaviour is same as the one defined as legacy normal camped state, e.g. paging reception, SIB1 update, etc. - As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation. - RAN2 understands the NW can avoid impact on legacy RRC connected UE and R19 RRC connected UE due to on-demand SIB1 (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell). - In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1. - The legacy UE behaviour can be reused upon on-demand SIB1 acquisition failure, i.e., the NES UE should follow the intraFreqReselection in MIB of NES cell. - If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell. - Reuse legacy cell reselection criterion as trigger condition of OD-SIB1 acquisition. No need to specify other conditions (e.g. a new RSRP threshold) - Specify the following UE behavior to allow the UEs in RRC_CONNECTED state to acquire OD-SIB1 when T311 is running: - - When T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid. - - The legacy cell selection criteria are reused as the trigger condition of OD-SIB1 acquisition. - - The OD-SIB1 acquisition behavior is same as that of RRC_IDLE/IANCTIV UEs. - For Rel-19 NES UE in RRC_CONNECTED, rely on the NW dedicated RRC for SIB1 delivery if searchSpaceSIB1 is not configured. It is legacy UE behavior and no spec change is expected.

- OD-SIB1 requirement

Paging interruption

In legacy, the paging interruption requirement is TSI-NR + 2*Ttarget_cell_SMTC_period ms, which includes the time period for fine time sync, fine frequency sync, and system information (SIB1 and SI messages) acquisition. We copy the definition in RAN4 spec for information:

TSI-NR is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for an NR cell.

For the NES case, UE camps on the Cell A and try to re-select to NES cell based on cell re-selection criteria. After the sync is ready, UE will trigger the UL-WUS, verify the RAR in RAR window, acquire OD-SIB1 in OD-SIB1 window, and acquire other SI messages.

Firstly, we need try to figure out whether this OD-SIB1 procedure can be contained in current TSI-NR.

From definition point of view, we think the current definition can be reused, since the OD-SIB1 is also part of the relevant system information data reception procedure.

From the test case point of view, may be more investigation is needed about whether the value we used now can contain the UL-WUS and OD-SIB1 reception procedure. This can be done in perf phase.

Proposal 1: Reuse the current paging interruption requirement and the definition of TSI-NR.

Proposal 2: Further discuss whether the current value of TSI-NR in the test need to be extended in perf phase.

Te requirement

Agreement from RAN1#119

The parameter “n-TimingAdvanceOffset” is included in the UL-WUS configuration.

Based on the current progress of RAN1, Case2 will be specified in R19.

Case 2:

- UE transmits UL-WUS to NES Cell

- UE obtains the UL-WUS configuration from Cell A

- UE receives on-demand SIB1 from NES Cell

- Since UE need to transmit the UL WUS to NES Cell, the UL timing requirement should be specified in RAN4. Different from legacy PRACH transmission, the DL timing reference of UL-WUS shall be the downlink timing of the reference cell minus NTA+NTA offset×Tc,

- reference cell is the NES cell

- NTA for UL-WUS is defined as 0

- is the n-TimingAdvanceOffset from UL-WUS configuration if configured, if not, follow the default value defined in Table 7.1.2-2 TS 38.133

Proposal 3: For the UL timing requirement of UL-WUS, the DL timing reference of UL-WUS shall be the downlink timing of the reference cell minus NTA+NTA offset×Tc, that:

- reference cell is the NES cell

- NTA for UL-WUS is defined as 0

- is the n-TimingAdvanceOffset from UL-WUS configuration if configured, if not, follow the default value defined in Table 7.1.2-2 TS 38.133

- Conclusion

Based on the discussion above, the following proposals are concluded.

Proposal 1: Reuse the current paging interruption requirement and the definition of TSI-NR.

Proposal 2: Further discuss whether the current value of TSI-NR in the test need to be extended in perf phase.

Proposal 3: For the UL timing requirement of UL-WUS, the DL timing reference of UL-WUS shall be the downlink timing of the reference cell minus NTA+NTA offset×Tc, that:

- reference cell is the NES cell

- NTA for UL-WUS is defined as 0

- is the n-TimingAdvanceOffset from UL-WUS configuration if configured, if not, follow the default value defined in Table 7.1.2-2 TS 38.133

- Reference

- R4-2502689, WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei