3GPP TSG-RAN WG4 Meeting #114bis			R4-2503386

# Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.29.4.2

Source: 	CMCC

Title: 			   Discussion on the RRM requirement for NTN phase3

Document for:	Discussion

- Introduction

In last meeting, RAN4 achieved some general agreements for objectives of DL coverage enhancement. The WF has been approved in [1]. In this contribution, we further discuss the open issues this objective.

- Downlink coverage enhancements

## Issue 1-1: Cell Re-selection

Further discussion:

- In cell re-selection requirements, discuss whether any update is needed for case where SSB periodicity is larger than 20 ms:

  - Option 1 (Samsung): FFS

  - Option 2 (vivo): No update is needed

In our understanding, the legacy cell re-selection requirements have already considered and supported the SMTC periodicity larger than 20ms. Therefore, no update is needed.

Proposal 1: For the SSB periodicity larger than 20ms case, no update is needed for cell re-selection requirements.

## Issue 1-5: Paging

Further discussion:

- Further discuss whether any update is needed to paging interruption requirements for NGSO scenarios when the SSB periods is equal to 160 ms:

  - Option 1 (Huawei, vivo): no update is needed

  - Option 2 (Nokia): reduce the multiplying factor K in 4.2C.2.5

To acquire the synchronization from target cell, K*Ttarget_cell_SMTC_period is needed. If the target cell is served by the same satellite as serving or GEO satellite or NR TN network, then K = 2, which is same as legacy. If the target cell is served by the known Non-GEO satellite, then K shall be extended to 5 with the consideration of more samples are needed for acquiring the sync with different doppler assumption. With above analysis, we can see that there is no relationship between K and SMTC period, and SMTC can be configured as 160ms even in earlier release.

However, when SMTC is 160ms, the sync time in paging interruption will reach 900ms, the motivation of further reducing it is reasonable. Therefore, We support to investigate some methods to reduce this paging interruption time, for example, in a new release, whether UE can support smaller K in such scenario based on its capability. Meanwhile, to be noted, such improvement is not necessary to be combined with SMTC periodicity.

Observation 1: The paging interruption time will be longer than 1s when SMTC is configured as 160ms. This observation also valid for earlier release.

Proposal 2: In new release, further investigate whether UE can support smaller K in known Non-GEO target satellite scenario based on its capability.

## Issue 1-2-A: RRC Re-establishment (default TSMTC,i)

Agreement (online):

- In RRC re-establishment requirements, update is needed to the TSMTC,i value for case where the SMTC for the inter-frequency carrier is not provided.

Further discussion:

- In RRC re-establishment requirements, update the value of TSMTC,i to 160ms for case where the SMTC for the inter-frequency carrier is not provided.

  - FFS: whether the above will apply only for UEs capable of coverage enhancement feature, if such a UE capability is defined.

  - FFS: whether the above is for only inter-frequency or for both intra-frequency and inter-frequency

Firstly, we think the default value shall only be applicable for UE capable of coverage enhancement feature. Due to such UE capability has not be discussed and defined in RAN1, we can wait for more input from RAN1.

Secondly, considering the TSMTC will always be configured for intra-frequency measurement by serving cell MO.  Therefore, the above is for only inter-frequency case.

Proposal 3: The updated default value of TSMTC,i will be applied only for UEs capable of coverage enhancement feature, the specific UE feature can wait for more input from RAN1.

Proposal 4: In RRC re-establishment requirements, update the value of TSMTC,i to 160ms only for case where the SMTC for the inter-frequency carrier is not provided.

## Issue 1-2-B: RRC Re-establishment (SSB Ês/Iot < -8dB and SSB periodicity >20 ms)

Further discussion:

- In RRC re-establishment requirements, discuss whether any update is needed to the TSMTC,i value for case where the SMTC for the inter-frequency carrier is not provided:

  - Option 1: Yes, an update is needed

    - Note: With the understanding that the by-default SSB periodicity for initial access is 160ms for Rel-19, from RRM requirement perspective.

    - Companies are encouraged to provide details on how to update the specification.

  - Other option is not precluded.

In legacy, for both RRC connection re-establishment to NR intra-frequency cell and inter-frequency cell, the UE is not required to successfully identify a cell on any NR frequency layer when TSMTC/ TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB. The current requirement is k*800ms (the highlight part below). Obviously, this shall be updated with the assumption of SSB periodicity larger than 20ms.

Table 6.2C.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell

| Serving cell | FR of target NR | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
| SSB Ês/Iot (dB) | cell | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 5 x TSMTC) | Kmulti_SMTC * MAX (800 ms, 10 x TSMTC) |
| < -8 | FR1 | N/A | k * 800 ms Note1, 3 |
| NOTE 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. NOTE 2: Kmulti_SMTC is defined in clause 9.2C.5.1. NOTE 3: k = 1 if the cells on the target frequency are served by GEO. k = (N+1) if the cells on the target frequency are served by LEO, where N is the number of different satellites associated to the list of configured neighbour cells in ntn-NeighCellConfigList and ntn-NeighCellConfigListExt. |  |  |  |

Table 6.2C.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell

| Serving cell SSB Ês/Iot (dB) | FR of target NR cell | Tidentify_inter_NR, i [ms] |  |
| --- | --- | --- | --- |
|  |  | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 6 x TSMTC, i) | K_satellite * MAX (800 ms, 13 x TSMTC, i) |
| < -8 | FR1 | N/A | k * 800 ms Note1, 3 |
| NOTE 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB. NOTE 2: K_satellite is defined in clause 9.3C.4. NOTE 3: k = 1 if the cells on the target frequency are served by GEO. k = (N+1) if the cells on the target frequency are served by LEO, where N is the number of different satellites associated to the list of configured neighbour cells in ntn-NeighCellConfigList and ntn-NeighCellConfigListExt. |  |  |  |

For the case that TSMTC/ TSMTC,i > 20 ms, we propose to define the requirement as k*(10+N)*TSMTC for RRC connection re-establishment to NR intra-frequency cell, and k*(13+N)*TSMTC,i for RRC connection re-establishment to NR inter-frequency cell. N is the relaxation part comparing to the Ês/Iot > -8dB case.

Proposal 5: For RRC re-establishment, when SSB Ês/Iot < -8dB and SSB periodicity >20 ms, update the Tidentify as k*(10+N)*TSMTC for RRC connection re-establishment to NR intra-frequency cell, and k*(13+N)*TSMTC,i for RRC connection re-establishment to NR inter-frequency cell. N is the relaxation part comparing to the Ês/Iot > -8dB case.

- Conclusion

Based on the discussion above, the following observations and proposals are concluded.

Proposal 1: For the SSB periodicity larger than 20ms case, no update is needed for cell re-selection requirements.

Observation 1: The paging interruption time will be longer than 1s when SMTC is configured as 160ms. This observation also valid for earlier release.

Proposal 2: In new release, further investigate whether UE can support smaller K in known Non-GEO target satellite scenario based on its capability.

Proposal 3: The updated default value of TSMTC,i will be applied only for UEs capable of coverage enhancement feature, the specific UE feature can wait for more input from RAN1.

Proposal 4: In RRC re-establishment requirements, update the value of TSMTC,i to 160ms only for case where the SMTC for the inter-frequency carrier is not provided.

Proposal 5: For RRC re-establishment, when SSB Ês/Iot < -8dB and SSB periodicity >20 ms, update the Tidentify as k*(10+N)*TSMTC for RRC connection re-establishment to NR intra-frequency cell, and k*(13+N)*TSMTC,i for RRC connection re-establishment to NR inter-frequency cell. N is the relaxation part comparing to the Ês/Iot > -8dB case.

- Reference

- R4-2502605, WF on RRM requirements for NR_NTN_Ph3_Part2, Moderator (Qualcomm Incorporated)