3GPP TSG-RAN WG4 Meeting # 114-bis                                                           R4-2503396

Wuhan, China, 7th –11th April, 2025

Agenda item:	7.12.1.1

Source: 	CMCC

Title: 	Discussion on UE RF requirements for ATG with inter-band CA

Document for:	Discussion

# 1. Introduction

RAN#106 meeting approved revised WID on Enhancements for Air-to-ground network for NR in Rel-19 [1]. After discussions of several meetings, we have made good progress on most of the ATG inter-band CA requirement, and we will focus on the n39 UL and n3 UL MSD requirements for CA_n3A-n39A.

# 2. Discussion

## 2.1. n39 UL for CA_n3A-n39A

During last meeting, we have reached the agreement as below.

Issue 2-1: Assumed parameters when supporting n39 UL for CA_n3A-n39A with common antenna. Agreement: - Companies are encouraged to provide MSD values based on the following parameters. Parameters; Values. Antenna Isolation; 20dB. The rejection for band n3 Rx filter at 1885~1915MHz; [35dB]. n39 Tx filter rejection at 1805~1830MHz; [50dB]. - Above parameters are only applicable for ATG UE with dedicated filter design Issue 2-2: MSD for CA_n3A-n39A with n39 UL Agreement: - Companies are encouraged to provide MSD values based on the following Tables. - MSD analysis based on separated antenna are not precluded. - All of these MSD values are only applicable to ATG UE. - Further study on how to capture these MSD values to in the specification. UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n39; n3; 1900; 30; 15; 160 (RBstart=0); 1817.5; 25; MSD Table X; >ACLR2. Table X CA_n3-n39 with band n39 UL MSD requirements for different output powers Output power; MSD (Based on Antenna Architecture with common antenna). 23dBm; TBD. 24dBm; TBD. …; …. 39dBm; TBD. 40 dBm; TBD.

Based on the parameters above, the MSD for different is show in the following table.

Table 1 The MSD calculation for CA_n3-n39 with n39 UL based on common antenna

|  | parameters | n3 main path | n3 diversity path |
| --- | --- | --- | --- |
| transmit power for n39, dBm |  | 23 | 23 |
| Tx band n39 BW(MHz, Lcrb@15kHz) | 30 |  |  |
| RFFE loss, dB | 4 |  |  |
| Diplexer isolation at n3 uplink freq，dB | 0 |  |  |
| antenna isolation, dB | 20 |  |  |
| n3 received signal at ANT port, dBm |  | 23 | 3 |
| n3 filter rejection at 1885~1915MHz, dB | 50 |  |  |
| signal After n3 filter, dBm |  | -31 | -51 |
| Typical receiver IIP2, dB | 50 |  |  |
| TX IM2 noise level refer to RX LNA input, dBm |  | -112 | -152 |
| front-end loss | 4 |  |  |
| TX IM2 noise level at ANT port, dBm |  | -108 | -148 |
| noise figure dB | 9 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -165 |  |  |
| Rx band n3 BW(MHz, NRB) | 23.9 |  |  |
| Thermal noise, dBm |  | -91.2 | -91.2 |
| ACLR2, dB | 43 |  |  |
| ACLR3, dB | 57 |  |  |
| n39 PA leakage PSD at PA output port |  | -48.8 |  |
| at 3rd adjacent channel, dBm/MHz |  |  |  |
| n39 PA noise PSD at PA output port, dBm/RxBW |  | -35 |  |
| n39 Tx filter rejection at 1805~1830MHz, dB | 35 |  |  |
| n39 PA noise power at Rx n3 ant port at 1885~1915MHz, dBm |  | -70 | -90 |
| Total noise level at ANT port |  | -70 | -87.5 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(25MHz BW) |  | -71 | -88.5 |
| Implementation Margin, dB | 2.5 |  |  |
| combined REFSENS(25MHz BW), dBm | -86.1 |  |  |
| MSD | 3.6 |  |  |

Table 2 The MSD calculation for CA_n3-n39 with n39 UL based on separated antenna

|  | parameters | n3 main path | n3 diversity path |
| --- | --- | --- | --- |
| transmit power for n39, dBm |  | 23 | 23 |
| Tx band n39 BW(MHz, Lcrb@15kHz) | 30 |  |  |
| RFFE loss, dB | 4 |  |  |
| Diplexer isolation at n3 uplink freq，dB | 0 |  |  |
| antenna isolation, dB | 20 |  |  |
| n3 received signal at ANT port, dBm |  | 3 | 3 |
| n3 filter rejection at 1885~1915MHz, dB | 50 |  |  |
| signal After n3 filter, dBm |  | -51 | -51 |
| Typical receiver IIP2, dB | 50 |  |  |
| TX IM2 noise level refer to RX LNA input, dBm |  | -152 | -152 |
| front-end loss | 4 |  |  |
| TX IM2 noise level at ANT port, dBm |  | -148 | -148 |
| noise figure dB | 9 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -165 |  |  |
| Rx band n3 BW(MHz, NRB) | 23.9 |  |  |
| Thermal noise, dBm |  | -91.2 | -91.2 |
| ACLR2, dB | 43 |  |  |
| ACLR3, dB | 57 |  |  |
| n39 PA leakage PSD at PA output port |  | -48.7 |  |
| at 3rd adjacent channel, dBm/MHz |  |  |  |
| n39 PA noise PSD at PA output port, dBm/RxBW |  | -35 |  |
| n39 Tx filter rejection at 1805~1830MHz, dB | 35 |  |  |
| n39 PA noise power at Rx n3 ant port at 1885~1915MHz, dBm |  | -90 | -90 |
| Total noise level at ANT port |  | -87.5 | -87.5 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(25MHz BW) |  | -88.5 | -88.5 |
| Implementation Margin, dB | 2.5 |  |  |
| combined REFSENS(25MHz BW), dBm | -89 |  |  |
| MSD | 0.6 |  |  |

Table 3 CA_n3-n39 with band n39 UL MSD test configuration

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n3 | 1900 | 30 | 15 | 160 (RBstart=0) | 1817.5 | 25 | MSD Table X | >ACLR2 |

Table X CA_n3-n39 with band n39 UL MSD requirements for different output powers

| Output power | MSD (Based on Antenna Architecture with common antenna) | MSD (Based on Antenna Architecture with separated antenna) |
| --- | --- | --- |
| 23dBm | 3.6 | 0.6 |
| 24dBm | 4.2 | 1.2 |
| 25dBm | 4.8 | 1.9 |
| 26dBm | 5.5 | 2.6 |
| 27dBm | 6.3 | 3.3 |
| 28dBm | 7.1 | 4.1 |
| 29dBm | 7.9 | 4.9 |
| 30dBm | 8.8 | 5.8 |
| 31dBm | 9.7 | 6.7 |
| 32dBm | 10.5 | 7.6 |
| 33dBm | 11.5 | 8.5 |
| 34dBm | 12.4 | 9.4 |
| 35dBm | 13.3 | 10.4 |
| 36dBm | 14.3 | 11.4 |
| 37dBm | 15.2 | 12.3 |
| 38dBm | 16.2 | 13.3 |
| 39dBm | 17.2 | 14.3 |
| 40 dBm | 28.2 | 15.3 |

As shown in the table above, we have analyzed the MSD values of n39 UL under different power levels and antenna architectures. When the transmission power is relatively high, the MSD value corresponding to the common antenna architecture will also increase. For example, when the transmission power is 33 dBm, the MSD value of the common antenna architecture has already exceeded 10 dB. In order to making the use of this frequency band feasible, we propose to improve the performance parameters of the n3 Rx filter on the one hand, and on the other hand, the MSD values of the separated antenna architecture should be taken into consideration.

Proposal 1: Consider improve the performance parameters of the n3 Rx filter.

Proposal 2: MSD values of ATG UE should be specified based on separated antenna architecture.

## 2.2. n3 UL for CA_n3A-n39A

During last meeting, we have reached the agreement as below.

Issue 2-3: Assumed parameters for CA_n3A-n39A with n3 UL WF: - Use the following parameters in the approved Tdoc R4-2410641 with 20dB antenna isolation as baseline to analysis the MSD value for n39 DL. - Other parameters are not precluded. Parameters; Value. Antenna isolation; [20dB]. Tx attenuation at 1880~1920MHz for band n3 duplexer; [30dB]. Rx attenuation at 1710~1785MHz for band n3+n39 Rx filter; [40dB]. Issue 2-4: MSD for CA_n3A-n39A with n3 UL WF: - Companies are encouraged to provide MSD values based on the following Tables. - Other MSD test configurations are not precluded. - All of these MSD values are only applicable to ATG UE. - Further study on how to capture these MSD values to in the specification. UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n3; n39; [1722.5]; [25]; [15]; [160 (RBstart=0)]; [1900]; [30]; MSD Table Y; [>ACLR2]. Table Y CA_n3-n39 with band n3 UL MSD requirements for different output powers Output power; MSD (Based on Antenna Architecture with common antenna). 23dBm; TBD. 24dBm; TBD. …; …. 39dBm; TBD. 40 dBm; TBD.

Based on the parameters above, the MSD for different is show in the following table.

Table 4 The MSD calculation for CA_n3-n39 with n3 UL based on common antenna

|  | parameters | n39 main path | n39 diversity path |
| --- | --- | --- | --- |
| transmit power for n3, dBm |  | 23 | 23 |
| Tx band n3 BW(MHz, Lcrb@15kHz) | 25 |  |  |
| RFFE loss, dB | 4 |  |  |
| Diplexer isolation at n3 uplink freq，dB | 0 |  |  |
| antenna isolation, dB | 20 |  |  |
| n39 received signal at ANT port, dBm |  | 23 | 3 |
| n39 filter rejection at 1710~1785MHz, dB | 40 |  |  |
| signal After n39 filter, dBm |  | -21 | -41 |
| Typical receiver IIP2, dB | 50 |  |  |
| TX IM2 noise level refer to RX LNA input, dBm |  | -92 | -132 |
| front-end loss | 4 |  |  |
| TX IM2 noise level at ANT port, dBm |  | -88 | -128 |
| noise figure dB | 9 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -165 |  |  |
| Rx band n39 BW(MHz, NRB) | 28.8 |  |  |
| Thermal noise, dBm |  | -90.4 | -90.4 |
| n3 PA noise PSD at PA output port, dBm/Hz | -130 |  |  |
| n3 PA noise PSD at PA output port, dBm/RxBW |  | -55.4 |  |
| n3 Tx filter rejection at 1880~1920MHz, dB | 30 |  |  |
| n3 PA noise power at Rx n39 ant port , dBm |  | -85.4 | -105.4 |
| Total noise level at ANT port |  | -82.7 | -90.3 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(30MHz BW) |  | -83.7 | -91.3 |
| Implementation Margin, dB | 2.5 |  |  |
| combined REFSENS(30MHz BW), dBm | -89.5 |  |  |
| MSD | 2.4 |  |  |

Table 5 The MSD calculation for CA_n3-n39 with n3 UL based on separated antenna

|  | parameters | n39 main path | n39 diversity path |
| --- | --- | --- | --- |
| transmit power for n3, dBm |  | 23 | 23 |
| Tx band n3 BW(MHz, Lcrb@15kHz) | 25 |  |  |
| RFFE loss, dB | 4 |  |  |
| Diplexer isolation at n3 uplink freq，dB | 0 |  |  |
| antenna isolation, dB | 20 |  |  |
| n39 received signal at ANT port, dBm |  | 3 | 3 |
| n39 filter rejection at 1710~1785MHz, dB | 40 |  |  |
| signal After n39 filter, dBm |  | -41 | -41 |
| Typical receiver IIP2, dB | 50 |  |  |
| TX IM2 noise level refer to RX LNA input, dBm |  | -132 | -132 |
| front-end loss | 4 |  |  |
| TX IM2 noise level at ANT port, dBm |  | -128 | -128 |
| noise figure dB | 9 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -165 |  |  |
| Rx band n39 BW(MHz, NRB) | 28.8 |  |  |
| Thermal noise, dBm |  | -90.4060751 | -90.40607512 |
| n3 PA noise PSD at PA output port, dBm/Hz | -130 |  |  |
| n3 PA noise PSD at PA output port, dBm/RxBW |  | -55.4060751 |  |
| n3 Tx filter rejection at 1880~1920MHz, dB | 30 |  |  |
| n3 PA noise power at Rx n39 ant port , dBm |  | -105.406075 | -105.4060751 |
| Total noise level at ANT port |  | -90.2701334 | -90.27013336 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(30MHz BW) |  | -91.2701334 | -91.27013336 |
| Implementation Margin, dB | 2.5 |  |  |
| combined REFSENS(30MHz BW), dBm | -91.8 |  |  |
| MSD | 0.16 |  |  |

Table 6 CA_n3-n39 with band n39 UL MSD test configuration

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | n39 | [1722.5] | [25] | [15] | [160 (RBstart=0)] | [1900] | [30] | MSD Table Y | [>ACLR2] |

Table Y CA_n3-n39 with band n3 UL MSD requirements for different output powers

| Output power | MSD (Based on Antenna Architecture with common antenna) | MSD (Based on Antenna Architecture with separated antenna) |
| --- | --- | --- |
| 23dBm | 2.4 | 0.16 |
| 24dBm | 2.5 | 0.16 |
| 25dBm | 2.7 | 0.16 |
| 26dBm | 2.8 | 0.16 |
| 27dBm | 2.9 | 0.16 |
| 28dBm | 3 | 0.16 |
| 29dBm | 3 | 0.17 |
| 30dBm | 3.1 | 0.18 |
| 31dBm | 3.1 | 0.19 |
| 32dBm | 3.2 | 0.2 |
| 33dBm | 3.2 | 0.2 |
| 34dBm | 3.3 | 0.3 |
| 35dBm | 3.3 | 0.3 |
| 36dBm | 3.4 | 0.4 |
| 37dBm | 3.6 | 0.6 |
| 38dBm | 3.8 | 0.8 |
| 39dBm | 4.2 | 1.2 |
| 40 dBm | 4.7 | 1.7 |

## 2.3. UE antenna type for inter-band CA

Agreement: - ATG UE with an antenna array on the bands of PCC and SCC, and one common Rx beam is possible for specific band combination. - Antenna array type for inter-band CA is highly depending on frequency range of operating bands and UE implementation. - Option 1: Case 4 UE implementation is possible for CA_n3-n39 that frequency ranges between two bands are close to each other. - Option 2: Case 4 UE implementation is possible for CA_n3-n39 that DL frequency separation between two bands are less than [X=FFS, e.g, 500 MHz].

There are two options left for the description of ‘frequency ranges between two bands are close to each other’. In our view, it is hard to determine the frequency separation between two bands. This issue should be analyzed from the perspective of terminal implementation on a case-by-case basis. For example, the consensus is that it is feasible for bands n3 and n39.

Proposal 3: It is hard to determine the frequency separation between two bands, and this issue should be analyzed case by case.

# 3. Conclusions

In this contribution, ATG UE with inter band CA requirements are discussed with the following proposals:

Proposal 1: Consider improve the performance parameters of the n3 Rx filter.

Proposal 2: MSD values of ATG UE should be specified based on separated antenna architecture.

Table 3 CA_n3-n39 with band n39 UL MSD test configuration

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n3 | 1900 | 30 | 15 | 160 (RBstart=0) | 1817.5 | 25 | MSD Table X | >ACLR2 |

Table X CA_n3-n39 with band n39 UL MSD requirements for different output powers

| Output power | MSD (Based on Antenna Architecture with common antenna) | MSD (Based on Antenna Architecture with separated antenna) |
| --- | --- | --- |
| 23dBm | 3.6 | 0.6 |
| 24dBm | 4.2 | 1.2 |
| 25dBm | 4.8 | 1.9 |
| 26dBm | 5.5 | 2.6 |
| 27dBm | 6.3 | 3.3 |
| 28dBm | 7.1 | 4.1 |
| 29dBm | 7.9 | 4.9 |
| 30dBm | 8.8 | 5.8 |
| 31dBm | 9.7 | 6.7 |
| 32dBm | 10.5 | 7.6 |
| 33dBm | 11.5 | 8.5 |
| 34dBm | 12.4 | 9.4 |
| 35dBm | 13.3 | 10.4 |
| 36dBm | 14.3 | 11.4 |
| 37dBm | 15.2 | 12.3 |
| 38dBm | 16.2 | 13.3 |
| 39dBm | 17.2 | 14.3 |
| 40 dBm | 28.2 | 15.3 |

Table 6 CA_n3-n39 with band n39 UL MSD test configuration

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | n39 | [1722.5] | [25] | [15] | [160 (RBstart=0)] | [1900] | [30] | MSD Table Y | [>ACLR2] |

Table Y CA_n3-n39 with band n3 UL MSD requirements for different output powers

| Output power | MSD (Based on Antenna Architecture with common antenna) | MSD (Based on Antenna Architecture with separated antenna) |
| --- | --- | --- |
| 23dBm | 2.4 | 0.16 |
| 24dBm | 2.5 | 0.16 |
| 25dBm | 2.7 | 0.16 |
| 26dBm | 2.8 | 0.16 |
| 27dBm | 2.9 | 0.16 |
| 28dBm | 3 | 0.16 |
| 29dBm | 3 | 0.17 |
| 30dBm | 3.1 | 0.18 |
| 31dBm | 3.1 | 0.19 |
| 32dBm | 3.2 | 0.2 |
| 33dBm | 3.2 | 0.2 |
| 34dBm | 3.3 | 0.3 |
| 35dBm | 3.3 | 0.3 |
| 36dBm | 3.4 | 0.4 |
| 37dBm | 3.6 | 0.6 |
| 38dBm | 3.8 | 0.8 |
| 39dBm | 4.2 | 1.2 |
| 40 dBm | 4.7 | 1.7 |

Proposal 3: It is hard to determine the frequency separation between two bands, and this issue should be analyzed case by case.

# 4. References

[1] RP-242385, Revised WID on Enhancements for Air-to-ground network for NR