3GPP TSG-RAN WG4 Meeting #114bis				                                           R4-2503416

Wuhan, CN, April 7th – April 11th, 2025

Title:	Discussion on FR2-1 L3 measurement delay reduction by optimizing Rx beam sweeping factor

Source:	CATT

Agenda Item:	7.17.2.1

Document for:	Discussion

1 Introduction

In last RAN4 meeting, the enhancement for FR2-1 SSB based measurement delay reduction was discussed. The conclusions were captured in the approved WF [1] and the left open issues can be found in the topic summary [2]. In this paper, we will continue the discussion on FR2-1 SSB based measurement delay reduction by optimizing Rx beam sweeping factor (BSF) and give our views.

2 Discussion

In last meeting, RAN4 discussed the conditions that trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF and the related agreements are reproduced below:

### Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF Agreement in ad-hoc session: - Discuss if additional conditions are needed to activate FBS-based L3 measurements. - Do not consider additional conditions beside the agreements from RAN4#113 meeting. Agreement in RRM session (Monday): - Adopt FBS triggering condition of serving cell is worse than a threshold, including RSRP, RSRQ threshold. - Further discuss whether to limit the serving cell to FR2 serving cell. - Further discuss whether to add the hysteresis and timeToTrigger parameters. - Note: the scenario related to single carrier or CA/DC is discussed in another issue. - The signaling design is up to RAN2.

There are some open issues that need further discussions. Regarding whether to limit the serving cell to FR2 only, the point of applying this serving cell quality threshold is UE needs to perform fast L3 measurements when serving cell quality becomes bad. Therefore we do not think it is necessary to limit the serving cell to FR2 only.

Proposal 1: Do not limit serving cell to FR2 serving cell.

On necessity of hysteresis and timeToTrigger parameters, in order to avoid ping-ping on and off of this enhancement, including these two parameters will be beneficial. But the details of design, in our understanding, can be left to RAN2. RAN4 can mention in the LS to RAN2 that it would be helpful to consider hysteresis and/or timeToTrigger when designing the signallings.

Proposal 2: RAN4 encourage RAN2 to consider the benefits of hysteresis and timeToTrigger parameters, but whether to add them is up to RAN2.

Regarding the quit conditions for scenario 1/2/3/4, further discussions are needed since there is a concern on UE power consumption. Multiple solutions were proposed, including UAI indication, timer and quite after first valid measurement report, etc. In our understanding, using a timer would be a simple and straightforward solution, which also can maximum the gain of this Rx BSF reduction enhancement feature. The mechanism of this timer-based solution is:

When serving cell quality threshold condition is met, i.e., the measurement result of serving cell is not larger than the threshold, UE activates Rx BSF reduction feature and performs fast L3 measurements for FR2 neighbour cells. At the same time, the timer starts to run. UE deactivates Rx BSF reduction feature when either of the conditions are met:

(1) Serving cell quality threshold condition is still met, but timer expires.

(2) While timer is still running, but serving cell quality threshold condition is no long met.

Furthermore, we can use a second timer for UE to re-activate Rx BSF reduction enhancement, which also helps UE save power. The idea is that when UE deactivates Rx BSF reduction feature, the first timer stops and the 2nd timer starts to run, and after the 2nd timer expires, UE re-evaluates the serving cell quality threshold condition and decides whether to activate Rx BSF reduction feature or not. UE is not requested to activate Rx BSF reduction feature while the 2nd timer is still running.

Proposal 3: UE deactivates Rx BSF reduction feature based on timer-solution, i.e., when either of the conditions are met:

(1) Serving cell quality threshold condition is still met, but timer expires.

(2) While timer is still running, but serving cell quality threshold condition is no long met.

Proposal 4: Introduce another timer, and UE re-evaluates the serving cell quality threshold condition after it expires.

Another issue is how to design the UE capability for Rx BSF reduction feature and the solution proposed by most of companies is to introduce reduced N, i.e., 2, 4 or 6. From our perspective, this is a very straightforward method and we also support it. Since UE will choose an appropriate value from a pre-defined set according to its capability and then report it, we prefer to keep all candidate values and UE.

Proposal 5: Introducing the UE capability of reduced N and the values are {2, 4 and 6}.

In regards to the requirements that are impacted by this feature, we suggest RAN4 to close this issue since CRs were allocated to interested companies, and RAN4 can discuss the draft CRs after the contents of UE capability is determined.

Proposal 6: RAN4 to discuss the updates on delay requirements based on progress on UE capability and draft CRs. This issue can be closed.

The last issue is the possibility of applying this enhanced feature to CSI-RS based measurement requirements. After the requirements for SSB measurement are updated, the enhanced Rx BSF can also apply to the associated SSB in CSI-RS based measurement since it refers to the formula of SSB based measurement without extra spec updates. Similarly, since the requirements for CHO and DAPS HO also refer to the formula of SSB based measurement and handover requirements respectively, the enhanced Rx BSF can also apply without extra specification updates expected.

Proposal 7: The reduced Rx BSF feature also applies to the associated SSB in CSI-RS based measurement, CHO and DAPS HO without extra specification updates.

3 Conclusion

In this paper, we further discussed on FR2-1 L3 measurement delay reduction by optimizing Rx beam sweeping factor, and the following proposals are given:

Proposal 1: Do not limit serving cell to FR2 serving cell.

Proposal 2: RAN4 encourage RAN2 to consider the benefits of hysteresis and timeToTrigger parameters, but whether to add them is up to RAN2.

Proposal 3: UE deactivates Rx BSF reduction feature based on timer-solution, i.e., when either of the conditions are met:

(1) Serving cell quality threshold condition is still met, but timer expires.

(2) While timer is still running, but serving cell quality threshold condition is no long met.

Proposal 4: Introduce another timer, and UE re-evaluates the serving cell quality threshold condition after it expires.

Proposal 5: Introducing the UE capability of reduced N and the values are {2, 4 and 6}.

Proposal 6: RAN4 to discuss the updates on delay requirements based on progress on UE capability and draft CRs. This issue can be closed.

Proposal 7: The reduced Rx BSF feature also applies to the associated SSB in CSI-RS based measurement, CHO and DAPS HO without extra specification updates.

4 References

- R4-2502593, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT, RAN4#114.

- R4-2500532, Topic summary for [114][218] NR_RRM_Ph5_Part2, CATT, RAN4#114.