3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503434

# Wuhan , CN, 7th – 11st Apr, 2025

Title: 	Discussion on CSI-RS based L1 measurement for Rel-19 LTM enhancements

Source: 	CATT

Agenda item:	7.27.2.2

Document for:	Discussion

- Introduction

In the last meeting, the WF on RRM requirements for NR_Mob_Ph4_Part1 was approved [1]. This document will discuss the RRM issues about CSI-RS based L1 measurement for LTM, and give our understanding and proposals.

- Discussion

- 2.1 Known cell definition in CSI-RS based L1 measurement

In RAN4#112-bis meeting, RAN4 has discussed known cell definition in CSI-RS based L1 measurement. In the last meeting, this issue has been divided into four parts for discussion, and RAN4 agreed to consider SSB based L3 measurement as the pre-requisite condition only in the first bullet. There is no conclusion yet on the other sub-issues in remaining bullets.

The previous agreements and candidate solutions for known cell definition in CSI-RS based L1 measurement are copied as follows [1]:

RAN4#112-bis: For CSI-RS based L1-RSRP measurement, the cell is considered as known if the following conditions are met: - Agreement: - The UE has performed L3 measurement on the target cell during the last 5 seconds, and - Further check whether to consider SSB-based L3 measurement only - The SSB from the target cell configured for L3[/L1] measurement remains detectable according to the cell identification requirements in clause 9.2. - L1 measurement in the 2nd bullet is not needed for FR1. - Further discuss whether to keep or remove L1 measurement for FR2-1 in the 2nd bullet. - The CSI-RS from the target cell configured for L1 measurement remains [detectable/measurable]. - Further discuss whether to use detectable or measurable. - Further discuss the side condition RAN4#114: Issue 4-1-1: Whether to use detectable or measurable? Tentative agreement: - Use measurable and specify the side condition for RS being measurable. Issue 4-1-2: Side condition for CSI-RS based L1 measurement for neighbor cell Candidate options: - Option 1: SNR of the CSI-RS from the target cell configured for L1 measurement is above -6dB (Nokia) - Option 2: It is proposed to use legacy side condition of SINR=-3dB. (Apple, Xiaomi, MTK, OPPO, Huawei, Ericsson)

For whether to use detectable or measurable, we think that the difference of the defination between detectable and measurable need to be further clarified. Considering that CSI-RS is not used for detection, so it is not accurate to use the wording of “detectable” as SSB, so may be ’measurable’ is fine for us. However, since the defination of ’detectable’ is clear and it exists in current spec [2], but there is no defination of ’measurable’ defined in spec, so the side condition for RS being measurable need to be further specified for CSI-RS based L1 measurement.

For the specific value of side condition, since the SINR of SSB based L1-RSRP is -3dB in R18 LTM, we think that the same side condition of SINR can be reused for CSI-RS based L1-RSRP measurement is introduced.

Observation 1: The defination of ’detectable’ exists in current spec, but there is no defination of ’measurable’ defined in spec.

Observation 2: CSI-RS is not used for detection, so it is not accurate to use the wording of “detectable” as SSB.

Proposal 1: For third condition in known condition, RAN4 to use ’measurable’ and specify the side condition for RS being measurable for CSI-RS based L1 measurement.

Proposal 2: RAN4 to reuse the legacy side condition of SINR=-3dB for CSI-RS based L1-RSRP measurement.

- 2.2 Measurement procedure

For the whole CSI-RS based L1 measurement procedure, RAN4 has agreed the entire framework in the last meeting, but there was still FFS for the issue. The candidate options are copied as below [1]:

RAN4#113: <Agreement> - Agree the following measurement procedure for CSI-RS based LTM candidate cell measurements: - Step 1: SSB based L3 measurements and reports at least before LTM configuration. - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell - Further discuss whether Step 2 can be skipped under certain conditions, or based on UE capability or based on network RRC configuration. FFS the details. - Note: Step 2 is removed for FR1. - Step 3: CSI-RS based L1 measurements on the same candidate cell - Depending on whether step 2 is skipped, the requirements in step 3 may be different. - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2. - [Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.] RAN4#114: Issue 4-2-1: Whether to keep or remove SSB based L1 measurement for FR2-1 in step 2 - Option A: Whether step 2 can be skipped or not can be based on UE capability reporting. (Apple, CATT, Nokia, Ericsson, Qualcomm, CMCC - compromise, ZTE, China Telecom, vivo, Xiaomi, OPPO) - Option B: Always keep step 2 for FR2-1 (MTK, HW) - Option C: Remove L1 measurement for FR2-1 in the 2nd bullet. (CMCC, OPPO)

The controversy on the above issue in the last meeting lies in whether to always keep or remove SSB based L1 measurement for FR2-1 in step 2. RAN4 has agreed to keep Step 2 in RAN4#113 meeting, and need to further discuss whether Step 2 can be skipped or not based on UE capability. In our view, the main difference between skipping and not skipping Step 2 is that the UE uses different Rx beams, resulting in different gains and complexity of UE implementation, so it depends more on the UE. Therefore, we prefer that whether Step 2 can be skipped can depend on UE capability as baseline.

However, the proponent of Option B mainly concern that the comparison of SSB based L1 measurement for serving cell and CSI-RS based L1 measurement is unfair and may reduce the performance of mobility, and we can also understand the concern, so we don’t have strong view on whether Step 2 can be skipped. Due to the large workload, we can compromise to prioritize defining the requirements for keeping step 2.

Proposal 3: Whether Step 2 can be skipped can depend on UE capability.

Proposal 4: Due to the workload, compromise to prioritize defining the requirements for keeping step 2.

- 2.3 CSI-RS based L1-SINR measurement

In the last meeting, the candidate solutions for CSI-RS based L1-SINR measurement are copied as follows [1]:

Candidate options: - Option 1: (CATT, Apple) RAN4 doesn’t need to define the RRM requirements for CSI-RS based L1-SINR measurement. - Option 2: (ZTE, CT) Focus on CSI-RS based L1-RSRP measurement and FFS on L1-SINR measurement.

In RAN1#118-bis meeting, the measurement metrics for CSI-RS based L1 measurement has been discussed, and RAN1 had no consensus on the support L1-SINR measurement based on CSI-RS for candidate cells. The conclusions in RAN1 are copied as follows:

Conclusion (RAN1#118bis) - There is no consensus in RAN1 on the support L1-SINR measurement based on CSI-RS for candidate cells.

As far as we know, RAN1 will not have a clear and explicit agreement about not introducing some requrements in RAN1 like RAN4, and 'no consensus ' means that RAN1 will not further discuss this issue and will not introduce CSI-RS based L1-SINR measurement. Therefore, we believe that RAN4 doesn’t need to further wait more RAN1 input and can make our decision, that is, RAN4 doesn’t need to define the RRM requirements for CSI-RS based L1-SINR measurement. If other working groups have any new agreement on this issue, RAN4 can revisit our decision.

Observation 3: There is no consensus in RAN1 on the support L1-SINR measurement based on CSI-RS for candidate cells, which means that RAN1 will not further discuss this issue and will not introduce CSI-RS based L1-SINR measurement.

Proposal 5: RAN4 doesn’t need to define the RRM requirements for CSI-RS based L1-SINR measurement.

- 2.4 Measurement gap and RTD assumption

In the last meeting, RAN4 discussed whether CSI-RS based L1 measurement with gap can be supported and whether to define requirements for RTD > CP, but there was no consensus among the companies. The candidate solutions are copied as below [1]:

Candidate options: - Option 1: CMCC, ZTE, CATT, China Telecom - Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell, as optional UE feature. - Define requirements for RTD > CP, as optional UE feature. FFS details - Option 2: E///, HW, CATT, Apple - compromise, CMCC - compromise - Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell, as optional UE feature. - No Ran4 requirements for RTD > CP. - Option 3: - Not Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell. - Define requirements for support RTD > CP, as optional UE feature. FFS details - Option 4: QC, Apple, vivo, Xiaomi, Samsung, MTK, OPPO, Nokia - compromise, E/// - compromise - Not Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell. - No Ran4 requirements for RTD > CP. Agreement: - Conclude this issue in the next meeting.

Considering the workload, we can compromize to Option 2 to only introduce one feature instead of introducing both, which is L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell (as optional UE feature).

As far as we know, the bandwidth of SSB is fixed, and the transmission bandwidth of the actual system may be much larger than that of SSB, and the measurements based on SSB may not be able to truly reflect the quality of the wireless link of DL outside the transmission bandwidth of SSB, which is one of the main reasons for the introduction of CSI-RS based L3 measurement.

For CSI-RS based L1 measurement, if RAN4 only considers CSI-RS based L1-RSRP measurement within active BWP, the gain of CSI-RS based measurement will be limited. Generally speaking, we think that CSI-RS based L1-RSRP measurement outside active BWP is a typical scenario, and it is more important from network perspective.

To address how to define the related requirements, RAN4 can focus on how to restrict the configuration and define the applicability for requirements, so that the entire mechanism can work.

Observation 4: If RAN4 only considers CSI-RS based L1-RSRP measurement within active BWP, the gain of CSI-RS based measurement will be limited.

Observation 5: CSI-RS based L1-RSRP measurement outside active BWP is a typical scenario and should be considered in R19 LTM.

Proposal 6: RAN4 to support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell as optional UE feature, and focus on how to restrict the configuration and define the applicability for requirements.

For whether to define requirements for RTD > CP, considering that CSI-RS resource can be configured on wider frequency, FFT is needed to cover a wider range, so the cost will be much higher to support RTD>CP for CSI-RS compared to SSB. So we can compromise not to define requirements for RTD > CP.

Proposal 7: Compromise not to define requirements for RTD > CP.

- 2.5 Definition of “inside/outside active BWP”

In the last meeting, the candidate solutions for definition of “inside/outside active BWP” are copied as below [1]:

Candidate options: - Option 1: (Apple) The CSI-RS of candidate cell should be considered inside the active BWP if all the resources are confined within the BWP. Otherwise, it is outside of active BWP. - Option 2: (CATT) RAN4 to firstly align the understanding of ‘the CSI-RS of candidate cell outside active BWP’ among companies. It is easier to define what ‘the CSI-RS of candidate cell within active BWP’ is, and all cases except for ‘within’ are considered as ‘the CSI-RS of candidate cell outside active BWP’. - Option 3: (vivo) From RRM requirement perspective, RAN4 further discuss which option is taken as the clarification for ‘the CSI-RS of candidate cell outside active BWP’ in R19 LTM: Option 1: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include at least one RB that is outside any active BWP. Option 2: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS are all outside active BWP(s) of the UE. Option 3: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include no more than 48 RBs within any active BWP.

For the definition of “inside/outside active BWP”, we suggest firstly aligning the understanding of ‘the CSI-RS of candidate cell outside active BWP’ among companies as it is easier to define what ‘the CSI-RS of candidate cell within active BWP’ is. We support option1 and think that the CSI-RS of candidate cell should be considered inside the active BWP if all the resources are confined within the BWP, and all cases except for ‘within’ are considered as ‘the CSI-RS of candidate cell outside active BWP’.

Proposal 8: RAN4 to firstly align the understanding of ‘the CSI-RS of candidate cell outside active BWP’.

- The CSI-RS of candidate cell should be considered inside the active BWP if all the resources are confined within the BWP.

- All cases except for ‘within’ are considered as ‘the CSI-RS of candidate cell outside active BWP’.

- 2.6 L1 measurement on CSI-RS with repetition ON/OFF

In the last meeting, the candidate solutions for L1 measurement on CSI-RS with repetition ON and OFF are copied as below [1]:

Candidate options: - Option 1: (CATT, CT, vivo) In FR2, RAN4 will not define the requirements for CSI-RS resources with repetition ON for L1 measurement on neighbor cell. In FR2, RAN4 will define the requirements for the following case: CSI-RS resources is configured with repetition OFF, UE perform CSI-RS based L1-RSRP measurement without RX beam sweeping after UE has performed L1-RSRP measurement on the associated SSB. - Option 2: (Huawei) Not to support CSI-RS resources with repetition ON in FR2 for CSI-RS based L1 RSRP measurement on candidate cell. - Option 3: (Nokia) Wait until RAN1 has agreed to support repetition “ON” configuration before agreeing to define requirements. As a baseline, define requirements for the case where the repetition is ‘OFF’. Revisit the agreement if RAN1 decides to support “ON” configuration. - Option 4: (Ericsson, Apple, Nokia) RAN4 to wait for RAN1 conclusion on support of CSI-RS with repetition ON.

Firstly, we want to clarify that what do CSI-RS repetition ON and OFF mean. According to TS 38.214, this parameter indicates whether the Tx beam repetition of CSI-RS is turned on or not from gNB side not from UE side.

Specifically, if the CSI-RS repetition is configured with ON, the Tx beam repetition of CSI-RS is turned on, and the corresponding gNB will transmit CSI-RS resource in a fixed beam direction. If the CSI-RS repetition is configured with OFF, the Tx beam repetition of CSI-RS is turned off, and the corresponding gNB will transmit CSI-RS resource with beam sweeping, and the gNB will use a wide beam.

Observation 6: CSI-RS repetition parameter indicates whether the Tx beam repetition of CSI-RS is turned on or not from gNB side not from UE side.

Observation 7: If the CSI-RS repetition is configured with ON, the Tx beam repetition of CSI-RS is turned on, and the corresponding gNB will transmit CSI-RS resource in a fixed beam direction.

Observation 8: If the CSI-RS repetition is configured with OFF, the Tx beam repetition of CSI-RS is turned off, and the corresponding gNB will transmit CSI-RS resource with beam sweeping.

In FR2, three typical cases can be analyzed:

For P1 case, wide beam will be used for gNB Tx and wide beam will be used for UE Rx, and the CSI-RS repetition is configured with ‘OFF’, SSB based BM is used and rough beam is used for UE.

For P2 case, narrow beam will be used for gNB Tx and fixed wide beam will be used for UE Rx, and the CSI-RS repetition is configured with ‘OFF’, then UE applies a fixed Rx beam obtained from SSB based L3 measurement.

For P3 case, fixed narrow beam will be used for gNB Tx and narrow beam will be used for UE Rx, and the CSI-RS repetition is configured with ‘ON’, then UE sweeps its Rx beam with a finer beamwidth, we can at least agree to not support this case for neighbor cell.

Therefore, RAN4 will not to define the requirements for CSI-RS resources with repetition ON in FR2 for L1 measurement on neighbor cell. Considering the overall sweeping time and measurement latency, it is also feasible for UE to use SSB based coarse beam as P2, so RAN4 can define the requirements for the following case in FR2: CSI-RS resources is configured with repetition OFF, UE perform CSI-RS based L1-RSRP measurement without RX beam sweeping after UE has performed L1-RSRP measurement on the associated SSB.

Proposal 9: In FR2, RAN4 will not to define the requirements for CSI-RS resources with repetition ON for L1 measurement on neighbor cell.

Proposal 10: In FR2, RAN4 will define the requirements for the following case:

- CSI-RS resources is configured with repetition OFF, UE perform CSI-RS based L1-RSRP measurement without RX beam sweeping after UE has performed L1-RSRP measurement on the associated SSB.

- 2.7 CSI-RS resource type

In the last meeting, the candidate solution for CSI-RS resource type is copied as below [1]:

Candidate options: - Option 1: (Apple, OPPO, Nokia, vivo) RAN4 prioritize CSI-RS based L1 measurement requirement for periodic CSI-RS and FFS on aperiodic and semi-persistent CSI-RS pending RAN1 decision. - Option 2: (CMCC, Ericsson) Semi-persistent CSI-RS resource for LTM candidate cells is supported. - Option 3: (CATT) RAN4 to define CSI-RS based L1 measurement for periodic and semi-persistent CSI-RS.

This issue is related to RAN1 progress, and the working assumption agreed in RAN1#118bis meeting is copied as follows:

Working Assumption (Agreement (RAN1#118bis) - In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective - Send an LS to RAN3 (CC RAN2) to ask for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload). - Support of semi-persistent CSI-RS is subject to UE capability. - Final LS in R1-2409283.

According to the above, in addition to periodic CSI-RS, RAN1 has agreed to support semi-persistent CSI-RS for candidate cell L1-RSRP measurement for gNB scheduled reporting, and the semi-persistent CSI-RS is subject to UE capability. Therefore, in addition to periodic CSI-RS, RAN4 also needs to consider semi-persistent CSI-RS transmission and measurements when defining RRM requirements for CSI-RS based L1 measurements for candidate cells.

Observation 9: In addition to periodic CSI-RS, RAN1 has agreed to support semi-persistent CSI-RS for candidate cell L1-RSRP measurement for gNB scheduled reporting, and the semi-persistent CSI-RS is subject to UE capability.

Proposal 11: RAN4 to define CSI-RS based L1 measurement for periodic and semi-persistent CSI-RS.

- Conclusion

This document discussed preliminary the issues for CSI-RS based L1 measurement for LTM in Rel-19 and the following proposals and observations are presented:

Observation 1: The defination of ’detectable’ exists in current spec, but there is no defination of ’measurable’ defined in spec.

Observation 2: CSI-RS is not used for detection, so it is not accurate to use the wording of “detectable” as SSB.

Proposal 1: For third condition in known condition, RAN4 to use ’measurable’ and specify the side condition for RS being measurable for CSI-RS based L1 measurement.

Proposal 2: RAN4 to reuse the legacy side condition of SINR=-3dB for CSI-RS based L1-RSRP measurement.

Proposal 3: Whether Step 2 can be skipped can depend on UE capability.

Proposal 4: Due to the workload, compromise to prioritize defining the requirements for keeping step 2.

Observation 3: There is no consensus in RAN1 on the support L1-SINR measurement based on CSI-RS for candidate cells, which means that RAN1 will not further discuss this issue and will not introduce CSI-RS based L1-SINR measurement.

Proposal 5: RAN4 doesn’t need to define the RRM requirements for CSI-RS based L1-SINR measurement.

Observation 4: If RAN4 only considers CSI-RS based L1-RSRP measurement within active BWP, the gain of CSI-RS based measurement will be limited.

Observation 5: CSI-RS based L1-RSRP measurement outside active BWP is a typical scenario and should be considered in R19 LTM.

Proposal 6: RAN4 to support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell as optional UE feature, and focus on how to restrict the configuration and define the applicability for requirements.

Proposal 7: Compromise not to define requirements for RTD > CP.

Proposal 8: RAN4 to firstly align the understanding of ‘the CSI-RS of candidate cell outside active BWP’.

- The CSI-RS of candidate cell should be considered inside the active BWP if all the resources are confined within the BWP.

- All cases except for ‘within’ are considered as ‘the CSI-RS of candidate cell outside active BWP’.

Observation 6: CSI-RS repetition parameter indicates whether the Tx beam repetition of CSI-RS is turned on or not from gNB side not from UE side.

Observation 7: If the CSI-RS repetition is configured with ON, the Tx beam repetition of CSI-RS is turned on, and the corresponding gNB will transmit CSI-RS resource in a fixed beam direction.

Observation 8: If the CSI-RS repetition is configured with OFF, the Tx beam repetition of CSI-RS is turned off, and the corresponding gNB will transmit CSI-RS resource with beam sweeping.

Proposal 9: In FR2, RAN4 will not to define the requirements for CSI-RS resources with repetition ON for L1 measurement on neighbor cell.

Proposal 10: In FR2, RAN4 will define the requirements for the following case:

- CSI-RS resources is configured with repetition OFF, UE perform CSI-RS based L1-RSRP measurement without RX beam sweeping after UE has performed L1-RSRP measurement on the associated SSB.

Observation 9: In addition to periodic CSI-RS, RAN1 has agreed to support semi-persistent CSI-RS for candidate cell L1-RSRP measurement for gNB scheduled reporting, and the semi-persistent CSI-RS is subject to UE capability.

Proposal 11: RAN4 to define CSI-RS based L1 measurement for periodic and semi-persistent CSI-RS.

- Reference

[1]      R4-2502601, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

[2]      3GPP TS 38.133, NR Requirements for support of radio resource management