3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503437

# Wuhan , CN, 7th – 11st Apr, 2025

Title: 	Discussion on other RRM requirements for Rel-19 NTN phase3

Source: 	CATT

Agenda item:	7.29.4.2

Document for:	Discussion

- Introduction

In the last meeting, the WF on RRM requirements for NR_NTN_Ph3_Part2 was approved [1]. This document will discuss other RRM requirements including SSB periodicity enhancement for NR NTN phase 3 and give our understanding and proposals.

- Discussion

Topic #1: SSB periodicity larger than 20 ms

- Issue 1-2-A: RRC Re-establishment (default TSMTC,i)

In the last meeting, RAN4 discussed the RRM impact on RRC Re-establishment (default TSMTC,i), and RAN4 agreed that the update is needed to the TSMTC,i value for case where the SMTC for the inter-frequency carrier is not provided, but there were still some FFS [1]:

Agreement (online): - In RRC re-establishment requirements, update is needed to the TSMTC,i value for case where the SMTC for the inter-frequency carrier is not provided. Further discussion: - In RRC re-establishment requirements, update the value of TSMTC,i to 160ms for case where the SMTC for the inter-frequency carrier is not provided. - FFS: whether the above will apply only for UEs capable of coverage enhancement feature, if such a UE capability is defined. - FFS: whether the above is for only inter-frequency or for both intra-frequency and inter-frequency

Regarding whether the update the value of TSMTC,i to 160ms will apply only for UEs capable of coverage enhancement feature if such a UE capability is defined. In our view, RAN4 to update the value of TSMTC,i to 160ms is only applicable to UEs capable of coverage enhancement feature, which is a better and clearer approach. Considering that RAN1 didn’t explicitly exclude legacy UE from accessing R19 network, the agreement and observation are copied as follows:

RAN1#119: Agreement For PDSCH with Msg4 Link level enhancement: - Support PDSCH repetition - FFS: signalling design including number of repetitions - FFS: impact on UE capability - Note: the target coverage enhancement to bridge the gap with respect to single Msg4 transmission is 2.8 Db - Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhance-ments at link level. Observation Backward compatibility for legacy UEs (i.e. Rel-17 and Rel-18 UEs) assuming a default SSB periodicity of 20ms is not guaranteed when SS/PBCH blocks periodicity is larger than 20ms within the cell used for initial frequency scan. Legacy UEs (i.e. Rel-17 and Rel-18 UEs) are not expected to be able to camp on a cell with SS/PBCH blocks periodici-ty larger than 160 ms. RAN1#120: Agreement For SIB1 link level enhancement, RAN1 to consider the following options: - Option 1: PDSCH repetition of SIB1 is transmitted within the same slot as the type0-CSS PDCCH repetition. o UE supporting SIB1 PDSCH coverage enhancement assumes that the PDCCH and associated PDSCH to be repeated in both slots where the corresponding PDCCHs are transmitted. o Each PDSCH SIB1 repetition is within the same slot of each PDCCH candidate for scheduling DCI o The two associated PDSCHs have the same RV - Option 2: Option 1 and an additional PDSCH with SIB1 repetition can occur after the slot of type0-PDCCH CSS repetition. o FFS: How to schedule the SIB1 repetition - Option 3: The repetition of PDSCH with SIB1 is indicated by the scheduling PDCCH o PDSCH is repeated in two slots Note: Backward compatibility should be maintained

If considering legacy UE, for UE that does not support coverage enhancement feature, it still assumes that the target SSB periodicity is no larger than 20 ms for the inter-frequency carrier i, then the value of Tidentify_inter_NR, i is still clear and will be taken as the upper bound.

For whether the above is for only inter-frequency or for both intra-frequency and inter-frequency, we believe that the legacy principle can be reused, and the case where the periodicity of the SMTC occasion is not configured only defined for inter-frequency, the legacy related requirements are copied as follows [3]:

TSMTC: It is the periodicity of the SMTC occasion configured for the intra-frequency carrier. If the UE has been provided with higher layer in TS 38.331 [2] signalling of smtc2, Tsmtc follows smtc1 or smtc2 according to the physical cell ID of the target cell. TSMTC,i: It is the periodicity of the SMTC occasion configured for the inter-frequency carrier i. If it is not configured, the UE may assume that the target SSB periodicity is no larger than 20 ms.

Observation 1: RAN1 didn’t explicitly exclude legacy UE from accessing R19 network, and the backward compatibility should be maintained.

Proposal 1: In RRC re-establishment requirements, update the value of TSMTC,i to 160ms for case where the SMTC for the inter-frequency carrier is not provided.

- The above will apply only for UEs capable of coverage enhancement feature, if such a UE capability is defined.

- The above is for inter-frequency.

- Issue 1-2-B: RRC Re-establishment (SSB Ês/Iot < -8dB and SSB periodicity >20 ms)

In the last meeting, RAN4 discussed the impact on note 1 for time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell, the candidate option is copied as follows:

Further discussion: - In RRC re-establishment requirements, discuss whether any update is needed to the TSMTC,i value for case where the SMTC for the inter-frequency carrier is not provided: - Option 1 (CATT, Huawei, vivo): Yes, an update is needed - Option 2 (Apple): No update is needed

In existing spec, the time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell are defined in the following table [2]:

Table 6.2C.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell Serving cell; FR of target NR; Tidentify_intra_NR [ms]. SSB Ês/Iot (dB); cell; Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 5 x TSMTC); Kmulti_SMTC * MAX (800 ms, 10 x TSMTC). < -8; FR1; N/A; k * 800 ms Note1, 3. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. Note 2: Kmulti_SMTC is defined in clause 9.2C.5.1. Note 3: k = 1 if the cells on the target frequency are served by GEO. k = (N+1) if the cells on the target frequency are served by LEO, where N is the number of different satellites associated to the list of configured neighbor cells in ntn-NeighCellConfigList and ntn-NeighCellConfigListExt..

For Note1, the UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. As we know, SMTC configuration will be configured by the network according to SSB periodicity, and the note is based on 20ms default SSB periodicity.

If UE is capable of 160ms default SSB periodicity, the typical value of TSMTC is also 160ms. If follows the existing logic in note 1, the UE will be always not required to successfully identify a cell on any NR frequency layer. RAN4 need to discuss whether to define a new requirement of Tidentify_intra_NR for blind search with new default SSB periodicity.

Proposal 2: RAN4 need to discuss whether to define a new requirement of Tidentify_intra_NR for blind search with new default SSB periodicity.

- Issue 1-5: Paging

In the last meeting, RAN4 didn’t discussed paging related requirements, and the candidate options are copied as below [1]:

Further discussion: - Further discuss whether any update is needed to paging interruption requirements for NGSO scenarios when the SSB periods is equal to 160 ms: - Option 1 (Huawei, vivo): no update is needed - Option 2 (Nokia): reduce the multiplying factor K in 4.2C.2.5

In our view, since UE has obtained the SSB periodicity via SIB1 before paging reception, and the legacy configurable values of SSB periodicity have already included 160ms, so the paging interruption requirements for NGSO scenarios didn’t exclude SSB periodicity of 160 ms, and we don’t see the necessity to adjust the existing requirements. Even though the paging interruption time will be longer for 160 ms, it will be regarded as the cost of enhancing downlink coverage.

Proposal 3: No update is needed for paging interruption requirements for NGSO scenarios when the SSB period is equal to 160 ms.

- Conclusion

This document discussed the issues about SSB periodicity enhancement for NR NTN Phase 3 and the following observations and proposals are presented:

Observation 1: RAN1 didn’t explicitly exclude legacy UE from accessing R19 network, and the backward compatibility should be maintained.

Proposal 1: In RRC re-establishment requirements, update the value of TSMTC,i to 160ms for case where the SMTC for the inter-frequency carrier is not provided.

- The above will apply only for UEs capable of coverage enhancement feature, if such a UE capability is defined.

- The above is for inter-frequency.

Proposal 2: RAN4 need to discuss whether to define a new requirement of Tidentify_intra_NR for blind search with new default SSB periodicity.

Proposal 3: No update is needed for paging interruption requirements for NGSO scenarios when the SSB period is equal to 160 ms.

- Reference

- R4-2502605, WF on RRM requirements for NR_NTN_Ph3_Part2, Moderator (Qualcomm Incorporated)

- 3GPP TS 38.133, NR Requirements for support of radio resource management