3GPP TSG-RAN WG4 Meeting #114bis                                                                                       R4-2503455

Wuhan, Hubei, China, 07th –11st April, 2025

Source: 	CATT

Title: 	Further discussion on system parameters for Ku-band for NR NTN

Agenda item:	7.11.3

Document for:	Approval

- Introduction

In RAN4#114 meeting, A WF [1] on system parameters for Ku-band for NR NTN was agreed and remaining issues on Ku-band system parameter require further discussion.

In this contribution, we provide our views on system parameters for Ku-band for NR NTN.

- Discussion

As per the revised WID for Ku-band NTN [2], the objectives concerning CBWs and SCSs are shown as below:

- Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing, and 10MHz, 15MHz, 25MHz, 35MHz and 50MHz channel bandwidths with 15 kHz subcarrier spacing. - Note: Same subcarrier spacing is assumed for SSB and data channels - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing. - Note: Simultaneous operation of FR1-NTN and FR2-NTN numerology is not supported in the UE

So, to support above CBWs and SCSs, it is necessary to define transmission bandwidth configuration, minimum guard band, SAN channel bandwidth per operating band,  channel raster for 15kHz SCS, and synchronization raster for 15kHz SSB SCS.

Transmission bandwidth configuration

According to revised WID, to support the all CBWs and SCSs for Ku-band, it is necessary to add transmission bandwidth configuration for 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz CBWs with 30 kHz SCS and 25MHz, 35MHz and 50MHz CBW with 15 kHz SCS for FR1 numerology for Ku-band. The transmission bandwidth configuration for above mentioned CBWs and SCSs in TS 38.104 can be reused. It is reasonable to add the following Transmission bandwidth configuration in Table 2-1 to support FR1 numerology for Ku-band.

Table 2-1: Transmission bandwidth configuration NRB for FR1-NTN

| SCS (kHz) | 25MHz | 35MHz | 50MHz | 70MHz | 100MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 15 | 133 | 188 | 270 | N/A | N/A |
| 30 | 65 | 92 | 133 | 189 | 273 |

Based on above discussion, we propose,

Proposal 1: Add the following Transmission bandwidth configuration in Table 2-1 to support FR1 numerology for Ku-band.

Table 2-1: Transmission bandwidth configuration NRB for FR1-NTN

| SCS (kHz) | 25MHz | 35MHz | 50MHz | 70MHz | 100MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 15 | 133 | 188 | 270 | N/A | N/A |
| 30 | 65 | 92 | 133 | 189 | 273 |

Minimum guard band

As above proposal, for adding transmission bandwidth configuration NRB for FR1-NTN, it is also need to add minimum guard band for them, the minimum guard band for above adding transmission bandwidth configuration in TS 38.104 can be reused. It is reasonable to add the following minimum guard band in Table 2-2 to support FR1 numerology for Ku-band.

Table 2-2: Minimum guard band (kHz) (FR1-NTN)

| SCS (kHz) | 25MHz | 35MHz | 50MHz | 70MHz | 100MHz |
| --- | --- | --- | --- | --- | --- |
| 15 | 522.5 | 572.5 | 692.5 | N/A | N/A |
| 30 | 785 | 925 | 1045 | 965 | 845 |

Based on above discussion, we propose,

Proposal 2: Add the following minimum guard band in Table 2-2 to support FR1 numerology for Ku-band.

Table 2-2: Minimum guard band (kHz) (FR1-NTN)

| SCS (kHz) | 25MHz | 35MHz | 50MHz | 70MHz | 100MHz |
| --- | --- | --- | --- | --- | --- |
| 15 | 522.5 | 572.5 | 692.5 | N/A | N/A |
| 30 | 785 | 925 | 1045 | 965 | 845 |

SAN channel bandwidth per operating band

In order to support CBWs and SCSs in revised WID for Ku-band, it is reasonable to define the SAN channel bandwidth per operating band in Table 2-3 for n248, n247, and SAN channel bandwidth per operating band in Table 2-4 for n509, n508.

Table 2-3: SAN channel bandwidths and SCS per operating band in FR1-NTN

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 (NOTE) | 35 | 50 | 70 | 100 |
| n248 | 15 |  | 10 | 15 |  | 25 |  | 35 | 50 |  |  |
|  | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
| n247 | 15 |  | 10 | 15 |  | 25 |  | 35 | 50 |  |  |
|  | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
|  | NOTE: Deployment of 30 MHz channel bandwidth for NTN SAN needs to be preceded by introduction of all applicable Tx RF, Rx RF, and demodulation requirements. |  |  |  |  |  |  |  |  |  |  |

Table 2-4: SAN channel bandwidths and SCS per operating band in FR2-NTN

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| n509 | 120 | 50 | 100 | 200 | 400 |
| n508 | 120 | 50 | 100 | 200 | 400 |

Based on above discussion, we propose,

Proposal 3: Define the SAN channel bandwidth per operating band in Table 2-3 for n248, n247, and SAN channel bandwidth per operating band in Table 2-4 for n509, n508.

Table 2-3: SAN channel bandwidths and SCS per operating band in FR1-NTN

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 (NOTE) | 35 | 50 | 70 | 100 |
| n248 | 15 |  | 10 | 15 |  | 25 |  | 35 | 50 |  |  |
|  | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
| n247 | 15 |  | 10 | 15 |  | 25 |  | 35 | 50 |  |  |
|  | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
|  | NOTE: Deployment of 30 MHz channel bandwidth for NTN SAN needs to be preceded by introduction of all applicable Tx RF, Rx RF, and demodulation requirements. |  |  |  |  |  |  |  |  |  |  |

Table 2-4: SAN channel bandwidths and SCS per operating band in FR2-NTN

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| n509 | 120 | 50 | 100 | 200 | 400 |
| n508 | 120 | 50 | 100 | 200 | 400 |

Channel raster entries for each operating band

According to revised WID, for new introducing 15kHz SCS FR1 numerology for Ku-band, it is needed to define channel raster for ΔFRaster =15kHz for band n248 and n247. Using the excel file in our contribution [3], the channel raster for ΔFRaster =15kHz for band n248 and n247 are calculated in Table 2-5.

Table 2-5: Applicable NR-ARFCN per operating band in FR1-NTN

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n248 | 15 | 1333334 – <1> – 1366666 | 1113334 – <1> – 1250000 |
| n247 | 15 | 1316667 – <1> – 1333333 | 1113334 – <1> – 1250000 |

Based on above discussion, we propose,

Proposal 4: Define the channel raster for ΔFRaster =15kHz for band n248 and n247 in Table 2-5.

Table 2-5: Applicable NR-ARFCN per operating band in FR1-NTN

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n248 | 15 | 1333334 – <1> – 1366666 | 1113334 – <1> – 1250000 |
| n247 | 15 | 1316667 – <1> – 1333333 | 1113334 – <1> – 1250000 |

Synchronization raster entries for each operating band

According to revised WID, for new introducing 15kHz SCS FR1 numerology for Ku-band, it is needed to define synchronization raster for SS Block SCS =15kHz for band n248 and n247. Using the excel file in our contribution [3], the synchronization raster for SSB Block SCS =15kHz for band n248 and n247 are calculated in Table 2-6.

Table 2-6: Applicable SS raster entries per operating band (FR1-NTN)

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n248 | 15kHz | Case A | 12848 – <1> – 14268 |
| n247 | 15kHz | Case A | 12848 – <1> – 14268 |

Based on above, we propose,

Proposal 5: synchronization raster for SSB Block SCS =15kHz for band n248 and n247 in Table 2-6.

Table 2-6: Applicable SS raster entries per operating band (FR1-NTN)

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n248 | 15kHz | Case A | 12848 – <1> – 14268 |
| n247 | 15kHz | Case A | 12848 – <1> – 14268 |

- Conclusion

This contribution provides analysis on system parameters for Ku-band for NR NTN, and the following proposals and observations are concluded as follows:

Proposal 1: Add the following Transmission bandwidth configuration in Table 2-1 to support FR1 numerology for Ku-band.

Table 2-1: Transmission bandwidth configuration NRB for FR1-NTN

| SCS (kHz) | 25MHz | 35MHz | 50MHz | 70MHz | 100MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 15 | 133 | 188 | 270 | N/A | N/A |
| 30 | 65 | 92 | 133 | 189 | 273 |

Proposal 2: Add the following minimum guard band in Table 2-2 to support FR1 numerology for Ku-band.

Table 2-2: Minimum guard band (kHz) (FR1-NTN)

| SCS (kHz) | 25MHz | 35MHz | 50MHz | 70MHz | 100MHz |
| --- | --- | --- | --- | --- | --- |
| 15 | 522.5 | 572.5 | 692.5 | N/A | N/A |
| 30 | 785 | 925 | 1045 | 965 | 845 |

Proposal 3: Define the SAN channel bandwidth per operating band in Table 2-3 for n248, n247, and SAN channel bandwidth per operating band in Table 2-4 for n509, n508.

Table 2-3: SAN channel bandwidths and SCS per operating band in FR1-NTN

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 (NOTE) | 35 | 50 | 70 | 100 |
| n248 | 15 |  | 10 | 15 |  | 25 |  | 35 | 50 |  |  |
|  | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
| n247 | 15 |  | 10 | 15 |  | 25 |  | 35 | 50 |  |  |
|  | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
|  | NOTE: Deployment of 30 MHz channel bandwidth for NTN SAN needs to be preceded by introduction of all applicable Tx RF, Rx RF, and demodulation requirements. |  |  |  |  |  |  |  |  |  |  |

Table 2-4: SAN channel bandwidths and SCS per operating band in FR2-NTN

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| n509 | 120 | 50 | 100 | 200 | 400 |
| n508 | 120 | 50 | 100 | 200 | 400 |

Proposal 4: Define the channel raster for ΔFRaster =15kHz for band n248 and n247 in Table 2-5.

Table 2-5: Applicable NR-ARFCN per operating band in FR1-NTN

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n248 | 15 | 1333334 – <1> – 1366666 | 1113334 – <1> – 1250000 |
| n247 | 15 | 1316667 – <1> – 1333333 | 1113334 – <1> – 1250000 |

Proposal 5: synchronization raster for SSB Block SCS =15kHz for band n248 and n247 in Table 2-6.

Table 2-6: Applicable SS raster entries per operating band (FR1-NTN)

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n248 | 15kHz | Case A | 12848 – <1> – 14268 |
| n247 | 15kHz | Case A | 12848 – <1> – 14268 |

- References

- R4-2502290, Way Forward for [114][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group), RAN4#114

- RP-250799, Revised WID: Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT, RAN#107

- R4-2500988, Further discussion on system parameters for Ku-band for NR NTN, CATT, RAN4#114