3GPP TSG-RAN WG4 Meeting #114bis                                                                                                      R4-2503462

Wuhan, Hubei, China, 07th –11st April, 2025

Source: 	CATT

Title: 	Discussion on framework simplification for co-location/co-existence requirements

Agenda item:	9.1

Document for:	Approval

- Introduction

In RAN4#114 meeting, A Way forward [1] for [114][331] BSRF_Simplify_CoLo_Coex was agreed and remaining issues in WF need further discussion.

In this contribution, we provide our views on framework simplification for co-location/co-existence requirements.

- Discussion

As per the WF [1], the remaining open issues concerning simplification methods are shown as below:

# Way Forward Focus on the following options: - Maintain tables for co-existence and co-location in each specification but simplify them, e.g., according proposals in [3, 4]. - Create a new standalone specification, capturing all the network-node requirements for co-existence and co-location. - This option does not exclude simplification of existing tables as the same time, as in bullet 1. Other options are not precluded. Companies are encouraged to present the possible implementations of these options during next meetings, e.g., draft CR, etc. Companies are encouraged to identify pros and cons of the above options, considering (but not limited to) the following aspects: - Workload for introduction of a new operating band, - Future-proof consideration, especially considering 6G introduction, - Repetitions reduction among multiple specifications, - Amount of specifications that contain together the complete information necessary to perform conformance testing in the lab. Final solution shall cover network node specifications, including the following aspects: - BS, IAB, and Repeater requirements, - All RATs of interest, - SRAT, and MSR requirements, - Core and Performance parts to be covered, - Conducted and OTA requirements.

As above, the contribution [3] provides good solution to simplify co-existence and co-location requirement, and the contribution [4] provides good solution to simplify the note in note column. We need to check the note in note column.

For co-existence requirement, the contribution [3] provides the simplification table for co-existence requirement copied as below:

Table 6.6.5.2.3-1: BS spurious emissions basic limits for BS for co-existence with systems operating in other frequency bands

| Frequency range for co-existence requirement | Basic limits (dBm) | Note |
| --- | --- | --- |
| Frequency range of downlink operating band | x | Note 3 |
| Frequency range of uplink operating band | y | Note 3, 4, 5, 6 |
| NOTE 1: x = -57 dBm/100kHz for NR BS to co-exist with GSM850, CDMA850 or GSM900 x = -47 dBm/100kHz for NR BS to co-exist with DCS1800 or PCS1900 x = -52 dBm for NR BS to co-exist with other bands NOTE 2: y = -61 dBm/100kHz for NR BS to co-exist with GSM850, CDMA850, GSM900, DCS1800 or PCS1900 y = -49 dBm for NR BS to co-exist with other bands NOTE 3: As defined in the scope for spurious emissions in this clause, except for the cases where the noted requirements apply to a BS operating in Band n28, n29 n67; the co-existence requirements in table 6.6.5.2.3 -1 do not apply for the ΔfOBUE frequency range immediately outside the downlink operating band (see table 5.2-1). Emission limits for this excluded frequency range may be covered by local or regional requirements. NOTE 4: For NR Band n28 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 27 UL operating band. NOTE 5: For NR Band n29 BS, specific solutions may be required to fulfil the spurious emissions limits for NR BS for co-existence with UTRA Band XII, E-UTRA Band 12 or NR Band n12 UL operating band, E-UTRA Band 17 UL operating band or E-UTRA Band 85 UL or NR Band n85 UL operating band. NOTE 6: For NR Band n67 BS, specific solutions may be required to fulfil the spurious emissions limits for NR BS for co-existence with E-UTRA Band 28 or NR Band n28 UL operating band or NR Band n83 UL operating band, where requirement applies for 703 MHz to 736 MHz. |  |  |

For co-existence requirement, the contribution [4] provides method to simplify the note in note column copied as below:

Observation 2: For the transmitter co-existence spurious emission requirements, the column for additional information (with title “Note”) could be cleaned up and simplified with respect to following aspects: - In the case bands overlap the requirement does not apply, which can be stated as part of general description. - In the case other emission requirements (e.g. protection of the BS receiver of own or different BS in subclause 6.6.5.2.2) are more stringent, the requirement does not apply. This information is obvious and can be removed or captured in general description.

As above, for the transmitter co-existence spurious emission requirements, we think that it is necessary to further interpret “In the case bands overlap the requirement does not apply”.  “Note” in the Note column for the transmitter co-existence spurious emission requirements can be further simplified with respect to following aspects:

- For frequency range of downlink operating band of co-existence systems or bands, in case frequency range of downlink operating band  of co-existence systems or bands overlaps frequency range of downlink operating band of NR operating band for NR BS, this requirement does not apply to NR BS operating in this NR operating band, which can be stated in Note column.

- For Frequency range of uplink operating band of co-existence systems or bands, in case frequency range of uplink operating band of co-existence systems or bands overlaps with frequency range of uplink operating band of NR operating band for NR BS, this requirement does not apply to NR BS operating in this NR operating band, which can be stated in Note column.

So, we have an observation for further simplify the   “Note” in the Note column for the transmitter co-existence spurious emission requirements as following:

Observation 1: For the transmitter co-existence spurious emission requirements, “Note” in the Note column can be further simplified with respect to following aspects:

- For frequency range of downlink operating band of co-existence systems or bands, in case frequency range of downlink operating band  of co-existence systems or bands overlaps frequency range of downlink operating band of NR operating band for NR BS, this requirement does not apply to NR BS operating in this NR operating band, which can be stated in Note column.

- For Frequency range of uplink operating band of co-existence systems or bands, in case frequency range of uplink operating band of co-existence systems or bands overlaps with frequency range of uplink operating band of NR operating band for NR BS, this requirement does not apply to NR BS operating in this NR operating band, which can be stated in Note column.

The above proposed general notes can’t cover all exceptions, which are discussed as following.

The Notes for band n29 are summarized as following:

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
| E-UTRA Band 12 or NR Band n12 | 699 – 716 MHz | -49 dBm | 1 MHz | For NR BS operating in n29, it applies 1 MHz below the Band n29 downlink operating band (Note 5). |
| E-UTRA Band 17 | 704 – 716 MHz | -49 dBm | 1 MHz | For NR BS operating in n29, (Note 5). |
| E-UTRA Band 85 or NR Band n85 | 728 – 746 MHz | -52 dBm | 1 MHz | For NR BS operating in n29, it applies 1 MHz below the Band n29 downlink operating band (Note 5). |
|  | 698 – 716 MHz | -49 dBm | 1 MHz | For NR BS operating in n29, it applies 1 MHz below the Band n29 downlink operating band (Note 5). |

As above, the exceptions for band n29 need to be considered, The above exceptions for band n29 need to be added to Note 4 in Table 6.6.5.2.3-1 as below:

NOTE 5:	For NR Band n29 BS, specific solutions may be required to fulfil the spurious emissions

limits for NR BS for co-existence with UTRA Band XII, E-UTRA Band 12 or NR Band n12 UL operating band, E-UTRA Band 17 UL operating band or E-UTRA Band 85 UL or NR Band n85 UL operating band, where requirement applies 1 MHz below the Band n29 downlink operating band.

Observation 2: The NOTE 5 in Table 6.6.5.2.3-1 can be revised as below:

NOTE 5:	For NR Band n29 BS, specific solutions may be required to fulfil the spurious emissions

limits for NR BS for co-existence with UTRA Band XII, E-UTRA Band 12 or NR Band n12 UL operating band, E-UTRA Band 17 UL operating band or E-UTRA Band 85 UL or NR Band n85 UL operating band, where requirement applies 1 MHz below the Band n29 downlink operating band.

The Notes for band n28 are summarized as following:

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
| E-UTRA Band 27 | 807 – 824 MHz | -49 dBm | 1 MHz | This requirement also applies to BS operating in Band n28, starting 4 MHz above the Band n28 downlink operating band (Note 5). |
| E-UTRA Band 68 | 698-728 MHz | -49 dBm | 1 MHz | For BS operating in Band n28, this requirement applies between 698 MHz and 703 MHz, while the rest is covered in clause 6.6.5.2.2. |

As above, the exceptions for band n28 need to be considered. The above exception for band n28 need to be added to Note 4 in Table 6.6.5.2.3-1 as below:

NOTE 4:	For NR Band n28 BS, specific solutions may be required to fulfil the spurious emissions

limits for BS for co-existence with E-UTRA Band 27 UL operating band, where requirement applies to BS operating in Band n28, starting 4 MHz above the Band n28 downlink operating band, and specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 68 UL operating band, where requirement applies between 698 MHz and 703 MHz,

Observation 3: The NOTE 4 in Table 6.6.5.2.3-1 can be revised as below:

NOTE 4:	For NR Band n28 BS, specific solutions may be required to fulfil the spurious emissions

limits for BS for co-existence with E-UTRA Band 27 UL operating band, where requirement applies to BS operating in Band n28, starting 4 MHz above the Band n28 downlink operating band, and specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 68 UL operating band, where requirement applies between 698 MHz and 703 MHz.

The notes for band n2, n5 and n26 are summarized as following:

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
| UTRA FDD Band XXV or E-UTRA Band 25 or NR band n25 | 1850 – 1915 MHz | -49 dBm | 1 MHz | For BS operating in Band n2, it applies for 1910 MHz to 1915 MHz, while the rest is covered in clause 6.6.5.2.2. |
| UTRA FDD Band XXVI or E-UTRA Band 26 or NR Band n26 | 814 – 849 MHz | -49 dBm | 1 MHz | For BS operating in Band n5, it applies for 814 MHz to 824 MHz, while the rest is covered in clause 6.6.5.2.2. |
| E-UTRA Band 106 or NR Band n106 | 896 – 901 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in Band n5 or n26. . |

We can find that above notes for band n2, n5 and n26 are not covered by general note and Note1 to Note 6 in Table 6.6.5.2.3-1, so it is needed to check how to simplify them.

Observation 4: Need to check how to simplify the following notes for co-existence requirement:

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
| UTRA FDD Band XXV or E-UTRA Band 25 or NR band n25 | 1850 – 1915 MHz | -49 dBm | 1 MHz | For BS operating in Band n2, it applies for 1910 MHz to 1915 MHz, while the rest is covered in clause 6.6.5.2.2. |
| UTRA FDD Band XXVI or E-UTRA Band 26 or NR Band n26 | 814 – 849 MHz | -49 dBm | 1 MHz | For BS operating in Band n5, it applies for 814 MHz to 824 MHz, while the rest is covered in clause 6.6.5.2.2. |
| E-UTRA Band 106 or NR Band n106 | 896 – 901 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in Band n5 or n26. . |

Based on above discussion, we can use following simplified tables for co-existence requirement.

Table 6.6.5.2.3-1: BS spurious emissions basic limits for BS for co-existence with systems operating in other frequency bands

| Frequency range for co-existence requirement | Basic limits (dBm) | Note |
| --- | --- | --- |
| Frequency range of downlink operating band of co-existence systems or bands | x | In case frequency range of downlink operating band of co-existence systems or bands overlaps frequency range of downlink operating band of NR operating band for NR BS, this requirement does not apply to NR BS operating in this NR operating band. Note 3 |
| Frequency range of uplink operating band of co-existence systems or bands | y | In case frequency range of uplink operating band of co-existence systems or bands overlaps with frequency range of uplink operating band of NR operating band for NR BS, this requirement does not apply to NR BS operating in this NR operating band. Note 3, 4, 5, 6 |
| NOTE 1: x = -57 dBm/100kHz for NR BS to co-exist with GSM850, CDMA850 or GSM900 x = -47 dBm/100kHz for NR BS to co-exist with DCS1800 or PCS1900 x = -52 dBm for NR BS to co-exist with other bands NOTE 2: y = -61 dBm/100kHz for NR BS to co-exist with GSM850, CDMA850, GSM900, DCS1800 or PCS1900 y = -49 dBm for NR BS to co-exist with other bands NOTE 3: As defined in the scope for spurious emissions in this clause, except for the cases where the noted requirements apply to a BS operating in Band n28, n29 n67; the co-existence requirements in table 6.6.5.2.3 -1 do not apply for the ΔfOBUE frequency range immediately outside the downlink operating band (see table 5.2-1). Emission limits for this excluded frequency range may be covered by local or regional requirements. NOTE 4: For NR Band n28 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 27 UL operating band, where requirement applies to BS operating in Band n28, starting 4 MHz above the Band n28 downlink operating band, and specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 68 UL operating band, where requirement between 698 MHz and 703 MHz. NOTE 5: For NR Band n29 BS, specific solutions may be required to fulfil the spurious emissions limits for NR BS for co-existence with UTRA Band XII, E-UTRA Band 12 or NR Band n12 UL operating band, E-UTRA Band 17 UL operating band or E-UTRA Band 85 UL or NR Band n85 UL operating band, where requirement applies 1 MHz below the Band n29 downlink operating band. NOTE 6: For NR Band n67 BS, specific solutions may be required to fulfil the spurious emissions limits for NR BS for co-existence with E-UTRA Band 28 or NR Band n28 UL operating band or NR Band n83 UL operating band, where requirement applies for 703 MHz to 736 MHz. |  |  |

Proposal 1: Use following simplified tables for co-existence requirement

Table 6.6.5.2.3-1: BS spurious emissions basic limits for BS for co-existence with systems operating in other frequency bands

| Frequency range for co-existence requirement | Basic limits (dBm) | Note |
| --- | --- | --- |
| Frequency range of downlink operating band of co-existence systems or bands | x | In case frequency range of downlink operating band of co-existence systems or bands overlaps frequency range of downlink operating band of NR operating band for NR BS, this requirement does not apply to NR BS operating in this NR operating band. Note 3 |
| Frequency range of uplink operating band of co-existence systems or bands | y | In case frequency range of uplink operating band of co-existence systems or bands overlaps with frequency range of uplink operating band of NR operating band for NR BS, this requirement does not apply to NR BS operating in this NR operating band. Note 3, 4, 5, 6 |
| NOTE 1: x = -57 dBm/100kHz for NR BS to co-exist with GSM850, CDMA850 or GSM900 x = -47 dBm/100kHz for NR BS to co-exist with DCS1800 or PCS1900 x = -52 dBm for NR BS to co-exist with other bands NOTE 2: y = -61 dBm/100kHz for NR BS to co-exist with GSM850, CDMA850, GSM900, DCS1800 or PCS1900 y = -49 dBm for NR BS to co-exist with other bands NOTE 3: As defined in the scope for spurious emissions in this clause, except for the cases where the noted requirements apply to a BS operating in Band n28, n29 n67; the co-existence requirements in table 6.6.5.2.3 -1 do not apply for the ΔfOBUE frequency range immediately outside the downlink operating band (see table 5.2-1). Emission limits for this excluded frequency range may be covered by local or regional requirements. NOTE 4: For NR Band n28 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 27 UL operating band, where requirement applies to BS operating in Band n28, starting 4 MHz above the Band n28 downlink operating band, and specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 68 UL operating band, where requirement applies between 698 MHz and 703 MHz. NOTE 5: For NR Band n29 BS, specific solutions may be required to fulfil the spurious emissions limits for NR BS for co-existence with UTRA Band XII, E-UTRA Band 12 or NR Band n12 UL operating band, E-UTRA Band 17 UL operating band or E-UTRA Band 85 UL or NR Band n85 UL operating band, where requirement applies 1 MHz below the Band n29 downlink operating band. NOTE 6: For NR Band n67 BS, specific solutions may be required to fulfil the spurious emissions limits for NR BS for co-existence with E-UTRA Band 28 or NR Band n28 UL operating band or NR Band n83 UL operating band, where requirement applies for 703 MHz to 736 MHz. |  |  |

Although, the above simplified table can simplify the co-existence requirement, but there are many other co-existence requirement was not included in table 6.6.5.2.3 -1, for example as below:

The following requirement may be applied for the protection of PHS. This requirement is also applicable at specified frequencies falling between ΔfOBUE below the lowest BS transmitter frequency of the downlink operating band and ΔfOBUE above the highest BS transmitter frequency of the downlink operating band. ΔfOBUE is defined in clause 6.6.1. The spurious emission basic limit for this requirement is: Table 6.6.5.2.3-2: BS spurious emissions basic limits for BS for co-existence with PHS Frequency range; Basic limit; Measurement Bandwidth; Note. 1884.5 – 1915.7 MHz; -41 dBm; 300 kHz; Applicable when co-existence with PHS system operating in 1884.5 – 1915.7 MHz. Table 6.6.5.2.3-3: Void In certain regions, the following requirement may apply to NR BS operating in Band n50 and n75 within the 1432 – 1452 MHz, and in Band n51 and Band n76. The basic limit is specified in Table 6.6.5.2.3-4. This requirement is also applicable at the frequency range from ΔfOBUE below the lowest frequency of the BS downlink operating band up to ΔfOBUE above the highest frequency of the BS downlink operating band. Table 6.6.5.2.3-4: Additional operating band unwanted emission basic limit for NR BS operating in Band n50 and n75 within 1432 – 1452 MHz, and in Band n51 and n76 Filter centre frequency, Ffilter; Basic limit; Measurement Bandwidth. Ffilter = 1413.5 MHz; -42 dBm; 27 MHz. …

As above, there are many other co-existence requirements are not included in table 6.6.5.2.3 -1, They are different, so it is difficult to summarized them to Table 6.6.5.2.3-1.  At this stage, only R19 and R20 version will be defined for 5G, we can take different method for 5G and 6G.

- Conclusion

This contribution provides analysis on framework simplification for co-location/co-existence requirements, and the following proposal is concluded as follows:

Observation 1: For the transmitter co-existence spurious emission requirements, “Note” in the Note column can be further simplified with respect to following aspects:

- For frequency range of downlink operating band of co-existence systems or bands, in case frequency range of downlink operating band  of co-existence systems or bands overlaps frequency range of downlink operating band of NR operating band for NR BS, this requirement does not apply to NR BS operating in this NR operating band, which can be stated in Note column.

- For Frequency range of uplink operating band of co-existence systems or bands, in case frequency range of uplink operating band of co-existence systems or bands overlaps with frequency range of uplink operating band of NR operating band for NR BS, this requirement does not apply to NR BS operating in this NR operating band, which can be stated in Note column.

Observation 2: The NOTE 5 in Table 6.6.5.2.3-1 can be revised as below:

NOTE 5:	For NR Band n29 BS, specific solutions may be required to fulfil the spurious emissions

limits for NR BS for co-existence with UTRA Band XII, E-UTRA Band 12 or NR Band n12 UL operating band, E-UTRA Band 17 UL operating band or E-UTRA Band 85 UL or NR Band n85 UL operating band, where requirement applies 1 MHz below the Band n29 downlink operating band.

Observation 3: The NOTE 4 in Table 6.6.5.2.3-1 can be revised as below:

NOTE 4:	For NR Band n28 BS, specific solutions may be required to fulfil the spurious emissions

limits for BS for co-existence with E-UTRA Band 27 UL operating band, where requirement applies to BS operating in Band n28, starting 4 MHz above the Band n28 downlink operating band, and specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 68 UL operating band, where requirement between 698 MHz and 703 MHz.

Observation 4: Need to check how to simplify the following notes for co-existence requirement:

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
| UTRA FDD Band XXV or E-UTRA Band 25 or NR band n25 | 1850 – 1915 MHz | -49 dBm | 1 MHz | For BS operating in Band n2, it applies for 1910 MHz to 1915 MHz, while the rest is covered in clause 6.6.5.2.2. |
| UTRA FDD Band XXVI or E-UTRA Band 26 or NR Band n26 | 814 – 849 MHz | -49 dBm | 1 MHz | For BS operating in Band n5, it applies for 814 MHz to 824 MHz, while the rest is covered in clause 6.6.5.2.2. |
| E-UTRA Band 106 or NR Band n106 | 896 – 901 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in Band n5 or n26. . |

Proposal 1: Use following simplified tables for co-existence requirement

Table 6.6.5.2.3-1: BS spurious emissions basic limits for BS for co-existence with systems operating in other frequency bands

| Frequency range for co-existence requirement | Basic limits (dBm) | Note |
| --- | --- | --- |
| Frequency range of downlink operating band of co-existence systems or bands | x | In case frequency range of downlink operating band of co-existence systems or bands overlaps frequency range of downlink operating band of NR operating band for NR BS, this requirement does not apply to NR BS operating in this NR operating band. Note 3 |
| Frequency range of uplink operating band of co-existence systems or bands | y | In case frequency range of uplink operating band of co-existence systems or bands overlaps with frequency range of uplink operating band of NR operating band for NR BS, this requirement does not apply to NR BS operating in this NR operating band. Note 3, 4, 5, 6 |
| NOTE 1: x = -57 dBm/100kHz for NR BS to co-exist with GSM850, CDMA850 or GSM900 x = -47 dBm/100kHz for NR BS to co-exist with DCS1800 or PCS1900 x = -52 dBm for NR BS to co-exist with other bands NOTE 2: y = -61 dBm/100kHz for NR BS to co-exist with GSM850, CDMA850, GSM900, DCS1800 or PCS1900 y = -49 dBm for NR BS to co-exist with other bands NOTE 3: As defined in the scope for spurious emissions in this clause, except for the cases where the noted requirements apply to a BS operating in Band n28, n29 n67; the co-existence requirements in table 6.6.5.2.3 -1 do not apply for the ΔfOBUE frequency range immediately outside the downlink operating band (see table 5.2-1). Emission limits for this excluded frequency range may be covered by local or regional requirements. NOTE 4: For NR Band n28 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 27 UL operating band, where requirement applies to BS operating in Band n28, starting 4 MHz above the Band n28 downlink operating band, and specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 68 UL operating band, where requirement applies between 698 MHz and 703 MHz. NOTE 5: For NR Band n29 BS, specific solutions may be required to fulfil the spurious emissions limits for NR BS for co-existence with UTRA Band XII, E-UTRA Band 12 or NR Band n12 UL operating band, E-UTRA Band 17 UL operating band or E-UTRA Band 85 UL or NR Band n85 UL operating band, where requirement applies 1 MHz below the Band n29 downlink operating band. NOTE 6: For NR Band n67 BS, specific solutions may be required to fulfil the spurious emissions limits for NR BS for co-existence with E-UTRA Band 28 or NR Band n28 UL operating band or NR Band n83 UL operating band, where requirement applies for 703 MHz to 736 MHz. |  |  |

- References

- R4-2502306, Way forward for [114][331] BSRF_Simplify_CoLo_Coex, Moderator (Ericsson), RAN4#114

- RP-243288, Moderator's summary of discussion about R19-RAN4-BSRF, RAN4 Chair (Huawei), RAN#106

- R4-2500102, On RAN task to work on framework simplification for co-location/co-existence requirements, Nokia

- R4-2500880, On the topic of BS RF specification structure improvements, Ericsson

- RP-242767, On new objective under NR_BS_RF_req_evo, Nokia, Ericsson, ZTE, Huawei, HiSilicon, RAN#106