3GPP TSG-RAN WG4 Meeting # 114bis                                                        R4-2503470

Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	                 Discussion on A-IoT BS requirements

Source: 	CATT

Agenda item:	7.24.2.1

Document for:	Discussion

- Introduction

This contribution provides our further analysis for A-IoT BS Tx and Rx requirements according to the WF [1].

- A-IoT BS RF requirements analysis

2.1 General

In WID [2], scenario is D1T1-B, R2D is located in DL spectrum and D2R and CW are located in UL spectrum. Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations. In this paper, the NR UEs distributed only outdoor.

2.2 A-IoT BS co-located with CW node analysis

If A-IoT BS and CW node are co-located, and the CW node works with other devices and other A-IoT BS, if the A-IoT BS receiver and the co-located CW node operate simultaneously, assume the coupling loss between them is 30dB, then the CW interference signal level at the A-IoT BS input will be large and it will have a great impact on the Rx blocking requirement of A-IoT BS receiver, even will block the A-IoT BS receiver. In addition, in the simulation of D1T1-B case in the SI phase, for each activated device, the nearest CW node will be activated during the simulation, and CW node is not co-located with any activated reader. So the scenario in which A-IoT BS receiver and the co-located CW node operate simultaneously is not recommended.

Proposal 1: CW nodes communicating with active devices can’t be co-located with any active readers.

2.3 Tx requirements

In WF [1], the Tx RF requirements for A-IoT BS in table 1 need to be further studied.

Table 1: Table 1: Tx RF requirements for A-IoT BS

| RF Requirement for A-IoT BS- TX requirements |  |
| --- | --- |
| Output power dynamic | RE power control dynamic range |
|  | Total power dynamic range |
| Transmit on/off | Transmitter off power |
|  | Transmitter transient period |
| Transmit signal quality | EVM |
| Unwanted emissions | Occupied bandwidth (OBW) |
|  | Adjacent Channel Leakage Power Ratio (ACLR) |
|  | Operating band unwanted emissions (OBUE) |
|  | Transmitter spurious emissions |
| Transmitter intermodulation |  |

2.3.1 Base station output power

The number of RBs of R2D signal and the number of RBs corresponding to the BS output power need to be clarified.

Proposal 2: The number of RBs of R2D signal and the number of RBs corresponding to the BS output power need to be clarified.

2.3.2 Output power dynamic

RAN1 doesn’t support RE granularity operation for R2D transmission, so RE power control dynamic range requirement is not necessary.

Proposal 3: RE power control dynamic range requirement is not necessary.

According to RAN1#120 conclusions, the signal resource of R2D can be several RBs, so the total power dynamic range requirement is necessary.

Proposal 4: According to signal resource of R2D can be several RBs, total power dynamic range requirement is necessary.

2.3.3 Transmit on/off

In the current BS Tx requirements, transmit OFF power requirement only applies to TDD operation,  but the frequency spectrums of R2D and D2R/CW are different, if define transmit OFF power requirement, the transmitter OFF power spectral density can reuse NB-IoT BS requirement, i.e. less than -85dBm/MHz.

Proposal 5: The transmitter OFF power spectral density can reuse NB-IoT BS requirement, i.e. less than -85dBm/MHz.

For an A-IoT BS communicating with device 1, the A-IoT BS doesn’t transmit R2D signal all the time and the transmission needs to be turned on and off, and R2D transmission and CW transmission /D2R signal reception operations are TDM, but the R2D spectrum and CW/D2R spectrums are different, if define transmit period, NB-IoT BS requirement can be reused.

Proposal 6: Transmitter transient period can reuse NB-IoT BS requirement, i.e. 10us.

2.3.4 Transmit signal quality

R2D signal is ook-4 modulation based on OFDM, and we can define envelope requirements similar to RFID, including modulation depth, RF envelope ripple, RF envelope rise time, RF envelope fall time and RF pulse. It is necessary to consider the differences between R2D signal and RFID signal, including modulation method, encoding method and waveforms, etc.

EVM is another specification that expresses signal modulation quality and does not need to be defined at the same time as envelope requirement. If EVM is introduced, it can be defined as 17.5% at on chip only.

Proposal 7: Either RF envelope or EVM requirements can be defined for R2D signal.

Proposal 8: If EVM is introduced, it can be defined as 17.5% at on chip only.

2.3.5 Unwanted emissions

2.3.5.1 Occupied bandwidth

The occupied bandwidth is the width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage /2 (=0.5%) of the total mean transmitted power.

The same principle with existing NR (99% power should be within BS channel bandwidth) can be reused, and the minimum requirement for A-IoT BS occupied bandwidth shall be less than the A-IoT BS channel bandwidth.

Proposal 9: The same principle with existing NR can be reused, and the minimum requirement for A-IoT BS occupied bandwidth shall be less than the A-IoT BS channel bandwidth.

2.3.5.2 ACLR

Based on the simulation results of scenario 2-1, the ACLR for NB-IoT BS with standalone mode can be reused by A-IoT BS, i.e.ACLR1 is 40dB and ACLR2 is 50dB. In the simulation, there are no 100kHz offset between the wanted carrier and the first adjacent carriers, so 100kHz offset is not required.

Proposal 10: The ACLR for NB-IoT BS with standalone mode can be reused by A-IoT BS, i.e.ACLR1 is 40dB and ACLR2 is 50dB.

Proposal 11: The 100kHz offset between wanted channel carrier and the first adjacent channel carriers is not required.

2.3.5.3 OBUE

The channel bandwidth of standalone NB-IoT BS is 200kHz, and it has guardbands at the low and up ends inside the channel, transmission bandwidth configuration of it is 180kHz, so OBUE requirement for standalone NB-IoT MR BS can be reused for A-IoT BS.

Proposal 12: OBUE requirement for standalone NB-IoT MR BS can be reused for A-IoT BS.

2.3.5.4 Transmitter spurious emissions

In scenario is D1T1-B, the A-IoT BS may co-located with CW node, and the scenario in which A-IoT BS receiver and the co-located CW node operate simultaneously is not recommended. In addition, the D1T1-B scenario is an indoor application, and the possibility of A-IoT BS co-located with BS operating in other frequency bands is relatively small.

Proposal 13: In D1T1-B scenario, the possibility of A-IoT BS co-located with BS operating in other frequency bands is relatively small.

2.3.6 Transmitter intermodulation

Traditionally the transmitter intermodulation requirement was created to guarantee emission levels in a co-location scenario. The requirement is based on robustness against injection of a reverse interference signal from a co-located BS. Scenario in which A-IoT BS receiver and the co-located CW node operate simultaneously is not recommended.  On the other hand, although the A-IoT BS R2D signal use NR BS DL spectrum, but the two BSs are not co-located, so the transmitter intermodulation requirement is not necessary.

Proposal 14: The A-IoT BS and NR BS are not co-located, so the transmitter intermodulation requirement is not necessary.

2.4 Rx requirements

In WF [1], the Rx RF requirements for A-IoT BS in table 2 need to be further studied.

Table 2: Rx RF requirements for A-IoT BS

| A-IoT BS - RX RF Requirement |  |
| --- | --- |
| SNR |  |
| Dynamic range |  |
| Adjacent Channel Selectivity（ACS） |  |
| In-band blocking | In-band blocking |
|  | Narrowband blocking |
| Receiver intermodulation | Receiver intermodulation |
|  | Narrowband intermodulation |
| In-channel selectivity (ICS) |  |

2.4.1 SNR

The SNR simulation assumptions are shown in table 3. We have simulated OOK and BPSK modulations, and the simulation results are shown in figure 1 and table 4.

Table 3:  SNR simulation assumptions

| Parameters | Assumptions |
| --- | --- |
| Carrier frequency | 900 MHz |
| SCS | 15 kHz as baseline |
| Block structure | Depend on RAN1 |
| Channel model | AWGN |
| Number of Tx/Rx chains for Ambient IoT device | 1 |
| Number of BS antenna elements | 1 |
| Message size | 64bit |
| BLER target | 10% |
| Sampling frequency（SFO） | Randomly select a value from the range of 103ppm for device 1 |
| Waveform (CW) | unmodulated single tone |
| Modulation | OOK, BPSK |
| Line code | Manchester encoding |
| FEC | Option 2：1/3 Convolutional Code |
| ADC bit width | Ideal |
| D2R receiver | coherent receiver |
| Repetition | Yes |

Figure 1: SNR simulation results

Table 4:  SNR simulation results

| Parameters | SFO | SNR (10% BLER) |
| --- | --- | --- |
| OOK, CC(R=1/3)+Manchester | no SFO | -0.6 |
|  | 1000 ppm | 2.5 |
| OOK, CC(R=1/3)+repetition | no SFO | -0.6 |
|  | 1000 ppm | 1.5 |
| BPSK, CC(R=1/3)+Manchester | no SFO | -6.7 |
|  | 1000 ppm | -3.5 |
| BPSK, CC(R=1/3)+repetition | no SFO | -6.7 |
|  | 1000 ppm | -4.6 |

Observation 1: The simulation results of SNR are shown in Table 4. It can be seen that the SNR of BPSK is obviously better than that of OOK.

2.4.2 ACS

Based on the simulation results of scenario 2-1, i.e. D1T1-B, NR UEs only distributed outdoor, R2D is located in DL spectrum and CW/D2R are located in UL spectrum, the ACS for NR BS can be reused for A-IoT BS.

Proposal 15: The ACS for NR BS can be reused for A-IoT BS.

2.4.3 In-band blocking

In scenario 2-1, i.e. the NR UEs only distributed outdoor, the distances between NR UEs and A-IoT BSs are relatively far, the narrow-band blocking requirement is not necessary.

Proposal 16: For D1T1-B scenario and NR UEs only distributed outdoor, the narrowband blocking requirement is not necessary.

2.4.4 Receiver intermodulation

As analysis in transmitter spurious emissions requirement, in D1T1-B scenario, the possibility of A-IoT BS co-located with BS operating in other frequency bands is relatively small.

The scenario in which A-IoT BS receiver and the co-located CW node operate simultaneously is not recommended. If A-IoT BS and co-located CW node can’t run at the same time, in scenario 2-1, i.e. the NR UEs only distributed outdoor, the distances between NR UEs and A-IoT BSs are relatively far, receiver intermodulation and narrowband intermodulation requirements are not necessary.

Proposal 17: For D1T1-B scenario and NR UEs only distributed outdoor, the receiver intermodulation requirement is not necessary.

Proposal 18: For D1T1-B scenario and NR UEs only distributed outdoor, the narrowband intermodulation requirement is not necessary.

2.4.5 ICS

ICS is a measure of the receiver ability to receive a wanted signal at its assigned resource block locations in the presence of another in-channel wanted signal received at a much larger power spectral density. In SI simulation, only one active A-IoT device under one reader at one drop, so the ICS requirement is not necessary.

Proposal 19: If the reader activates only one device at a time, ICS requirement is not necessary.

- Summary

This contribution provides our further analysis for A-IoT BS requirements. We have the following proposals and observations.

Proposal 1: CW nodes communicating with active devices can’t be co-located with any active readers.

Proposal 2: The number of RBs of R2D signal and the number of RBs corresponding to the BS output power need to be clarified.

Proposal 3: RE power control dynamic range requirement is not necessary.

Proposal 4: According to signal resource of R2D can be several RBs, total power dynamic range requirement is necessary.

Proposal 5: The transmitter OFF power spectral density can reuse NB-IoT BS requirement, i.e. less than -85dBm/MHz.

Proposal 6: Transmitter transient period can reuse NB-IoT BS requirement, i.e. 10us.

Proposal 7: Either RF envelope or EVM requirements can be defined for R2D signal.

Proposal 8: If EVM is introduced, it can be defined as 17.5% at on chip only.

Proposal 9: The same principle with existing NR can be reused, and the minimum requirement for A-IoT BS occupied bandwidth shall be less than the A-IoT BS channel bandwidth

Proposal 10: The ACLR for NB-IoT BS with standalone mode can be reused by A-IoT BS, i.e.ACLR1 is 40dB and ACLR2 is 50dB.

Proposal 11: The 100kHz offset between wanted channel carrier and the first adjacent channel carriers is not required.

Proposal 12: OBUE requirement for standalone NB-IoT MR BS can be reused for A-IoT BS.

Proposal 13: In D1T1-B scenario, the possibility of A-IoT BS co-located with BS operating in other frequency bands is relatively small.

Proposal 14: The A-IoT BS and NR BS are not co-located, so the transmitter intermodulation requirement is not necessary.

Observation 1: The simulation results of SNR are shown in Table 4. It can be seen that the SNR of BPSK is obviously better than that of OOK.

Proposal 15: The ACS for NR BS can be reused for A-IoT BS.

Proposal 16: For D1T1-B scenario and NR UEs only distributed outdoor, the narrowband blocking requirement is not necessary.

Proposal 17: For D1T1-B scenario and NR UEs only distributed outdoor, the receiver intermodulation requirement is not necessary.

Proposal 18: For D1T1-B scenario and NR UEs only distributed outdoor, the narrowband intermodulation requirement is not necessary.

Proposal 19: If the reader activates only one device at a time, ICS requirement is not necessary.

Reference

[1] R4-2502859, “WF on requirements for A-IoT BS and CW”, Huawei, RAN4#114

[2] RP-243326, “Solutions for Ambient IoT (Internet of Things) in NR”, Huawei, RAN#106

[3] R4-2502858, “WF on requirements for system parameters and A-IOT device”, CMCC, RAN4#114

[4] TR 38.769