3GPP TSG RAN WG4 Meeting #114bis		R4-2503485

Wuhan, Hubei, China, April 07th – 11th , 2025

Title: 	Further discussion on other RF requirements for LP-WUS

Agenda Item:	7.26.2.3

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In addition to REFSENS, ACS and ASCS, RAN4#114 has also discussed other RF requirements for LP-WUS such as IBB, OBB, maximum input level (MIL) and reached one agreement that the same interference level as MR is used for deriving IBB and OBB, while leaving other issues still open [1]:

Issue 3-1-1: IBB, OBB requirements Way forward: - Same interference level as MR

In this contribution, we present our views on open issues related to other RF requirements for LP-WUS.

# 2 Discussion

Maximum input level

For maximum input level, -25dBm is specified for NR carrier bandwidth 3MHz, 5MHz, 10MHz, 15MHz and 20MHz, and the case with 3MHz channel bandwidth has the highest PSD. In this case LP-WUS occupies 11 RBs out of 15 RBs in total, hence the maximum input level could be scaled down according to NR channel bandwidth 3MHz, i.e., 10*log10(NRB = 15/11) = 1.35dB, and if we take round to 0.5dB, then the maximum input level for LP-WUR should be -25-1.5 = -26.5dBm.

Proposal 1: RAN4 to define maximum input level as -26.5dBm for LP-WUR.

IBB & OBB

The blocking impact on LR may vary depending on the position of LP-WUS within the NR carrier. Figure 1(a) depicts a scenario where LP-WUS is positioned in the middle of the NR carrier, making it relatively distant from interfering signals. In contrast, Figure 1(b) illustrates a case where LP-WUS is located at the edge of the NR carrier, resulting in greater exposure to interferers.

(a) LP-WUS located in the middle of the NR carrier

(b) LP-WUS located at the edge of the NR carrier

Fig. 1, IBB & OBB for LP-WUS

Since it has been agreed that the interference level matches that of MR, the next question is how to set the wanted signal power level. For MR, the wanted signal power level is MR REFSENS + 6dB. Similar principle could be used for LR, which means the wanted signal power level can be set to LR REFSENS + X dB.

Proposal 2: RAN4 to set the wanted LP-WUS signal power level to LR REFSENS + X dB for IBB and OBB under the same interferers of MR.

However, REFSENS definition of LR is different from that of MR, where MDR is used for LR while 95% throughput principle is used for MR. Further study may be required to decide the value for X by considering potential different tolerances to the same interferer between detection performance and throughput, and also matched LR and MR coverage.

Under such consideration, the value for X can be derived further. For MR, denote ρ as the required SNR associated with 95% maximum throughput, and R as REFSENS, N0 is the white noise power:

ρ=RN0                   (1)

With the interferer added, its effective interference after rejection can be expressed as Ie, and the wanted signal power level is increased to αR. Under such blocking level it should be able to maintain 95% maximum throughput, i.e., SNR ρ should be the no less than ρ:

ρ=αRN0+Ie≥RN0              (2)

Hence,

Ie≤(α-1)N0                      (3)

Eq. (3) also applies to 5MHz NR carrier bandwidth (25 PRBs for SCS 15kHz).

For LP-WUS, the number of RBs is 11. The effective interference could be scaled proportionally:

Ie≤1125(α-1)N0                     (4)

Similar to Eq. (2), suppose the linear value of X dB is β, and LR REFSENS is R’, then the SNR of LP-WUS under blocking can be expressed as:

ρLR=βR'N0+Ie≥R'N0                    (5)

From Eq. (5),

Ie≤(β-1)N0                       (6)

Compared with Eq. (6) and Eq. (4), it can be assumed β-1N0=1125(α-1)N0 which leads to

β=1125α-1+1                     (7)

Actually, α corresponds to 6dB in MR, which means α = 4, then β can be computed as:

β=11254-1+1=5825→3.7dB        (8)

Eq. (8) can be round to 4dB. Therefore, X value is 4dB.

Proposal 3: RAN4 to set the wanted LP-WUS signal power level to LR REFSENS + 4 dB for IBB and OBB.

# 3 Conclusion

In this contribution, we have the following proposals for other RF requirements for LP-WUR:

Proposal 1: RAN4 to define maximum input level as -26.5dBm for LP-WUR.

Proposal 2: RAN4 to set the wanted LP-WUS signal power level to LR REFSENS + X dB for IBB and OBB under the same interferers of MR.

Proposal 3: RAN4 to set the wanted LP-WUS signal power level to LR REFSENS + 4 dB for IBB and OBB.

# Reference

[1] R4-2503028, “WF on UE RF requirements for LP-WUS”, vivo