3GPP TSG RAN WG4 Meeting #114bis		R4-2503493

Wuhan, Hubei, China, April 07th – 11th , 2025

Title: 	Further discussion on MPR applicability for FR1 intra-band UL CA

Agenda Item:	7.1.1.2.2

Source: 	CATT

Document for:	Discussion

# 1 Introduction

RAN4#114 has made decent progress on topics other than MPR applicability for FR1 intra-band UL CA [1], hence the open issue on this topic identified in RAN4#113 still remains [2]:

#### Issue 2-2-1: Applicable MPR and corresponding spectrum requirements for PC2/PC3 intra-band NC CA Agreement in main session: - No change of applicable MPR requirement in current spec but with clarification that applicable emission requirements are composite based regardless of dualPA IE indication. - FFS on whether update MPR for Single CC Fallback from Non-contiguous ULCA without dual PA support depending on UE capability. Note: Composite emissions means combination of individual sub-block emissions.

In this contribution, we present our view on this open issue.

# 2 Discussion

For the current specs, MPR for two non-contiguous UL CCs is specified in a way by considering whether or not dualPA-architecture is supported. For the open issue related to UL CA without dual PA, the specs reads[2]:

For UE without indicating dualPA-Architecture supported If OR( LCRB1 = 0, LCRB2 = 0 ) For PC3 UE, MPR defined in Table 6.2.2-1, except for B < 9 MHz where 5.5 dB MPR is used; For PC2 UE without indicating TxD, MPR defined in Table 6.2.2-2 is used, except for B < 11.52 MHz where 6.5 dB MPR is used; For PC2 UE indicating TxD, MPR defined in Table 6.2D.2-1 is used, except for B < 11.52 MHz where the maximum value between 6.5 dB and MPR defined in Table 6.2D.2-1 is used. Else If AND( FIM3,low_block,low > SEM-13,low , FIM3,high_block,high < SEM-13,high ) MPR defined in Clause 6.2A.2.2.2.3 and Clause 6.2A.2.2.2.4 for PC3 and PC2 UE respectively. Else MPR defined in Clause 6.2A.2.2.1.3 and Clause 6.2A.2.2.1.4 for PC3 and PC2 UE respectively.

For single CC fallback, it actually corresponds to the case where RB allocation of one of the two CCs is zero, i.e., either LCRB1 = 0 or LCRB2 = 0, which is already covered by the current specs. Moreover, since the applicable emission requirements are composite based, [3] identifies some cases for asymmetric non-contiguous CA with single PA architecture where intra-band non-contiguous SEM fails if LO is not retuned and the single CC MPR is used. In other words, there could be an alternative to apply single CC MPR by allowing transmission interruption.

However, the improvement is only for a very limited case where only small PRB allocation is made on one CC, which is not a favourite use case for UL CA. Moreover, this may require more works, e.g., further studies on whether and/or how to introduce a new requirement for the transmission interruption, hence we propose not to do any update.

Proposal: By considering the limited and unfavourable use case for UL CA and potential extra required works, RAN4 not to update MPR for Single CC Fallback from Non-contiguous ULCA without dual PA support depending on UE capability.

# 3 Conclusion

In this contribution, we have the following proposal on MPR applicability for FR1 intra-band UL CA:

Proposal: By considering the limited and unfavourable use case for UL CA and potential extra required works, RAN4 not to update MPR for Single CC Fallback from Non-contiguous ULCA without dual PA support depending on UE capability.

# Reference

[1] R4-2503013, “WF on Power domain enhancement”, Huawei, HiSilicon

[2] R4-2420527, “WF on Power domain enhancement”, Huawei

[3] 3GPP TS 38.101-1 v18.8.0

[4] R4-2411049, “On improved MPR for intra-band ULCA when only one CC has RBs allocated”, Skyworks Solutions Inc.