3GPP TSG-RAN WG4 Meeting #114bis												R4-2503498

Wuhan, China, 7 – 11 April, 2025

Agenda item:			7.14.2.1

Source:	Huawei, HiSilicon

Title:	on XR OTA tests

Document for:	approval

# Introduction

The WF [1] from the last RAN4 meeting recorded the following issues. This contribution address those issues.

Issue 2-1-2: Minimum measurement distance for 50cm QZ

Agreements:

  - The missing minimum measurement distance for AC system with 50cm QZ should be added into spec.

Issue 2-1-3: General XR TRP TRS test procedure

Agreements:

  - The general XR test method in R4-2502300 is agreed.

  - The TRP/TRS MU for low-battery capacity devices is FFS

# Discussion

The minimum range lengths for 50cm quiet zone are listed in Table 1.

| F [GHz] | Drad [m] | RQZ+2Drad²/λ | 3D = 6RQZ | RQZ+2λ | max(RQZ+2λ,3D,RQZ+2D²/λ) |
| --- | --- | --- | --- | --- | --- |
| 0.41 | 0.50 | 0.93 | 1.50 | 1.71 | 1.71 |
| 0.60 | 0.50 | 1.25 | 1.50 | 1.25 | 1.50 |
| 0.70 | 0.50 | 1.42 | 1.50 | 1.11 | 1.50 |
| 0.80 | 0.50 | 1.58 | 1.50 | 1.00 | 1.58 |
| 1.00 | 0.50 | 1.92 | 1.50 | 0.85 | 1.92 |
| 1.20 | 0.48 | 2.12 | 1.50 | 0.75 | 2.12 |
| 1.40 | 0.47 | 2.28 | 1.50 | 0.68 | 2.28 |
| 1.60 | 0.47 | 2.57 | 1.50 | 0.63 | 2.57 |
| 1.80 | 0.45 | 2.68 | 1.50 | 0.58 | 2.68 |
| 2.00 | 0.43 | 2.75 | 1.50 | 0.55 | 2.75 |
| 2.20 | 0.42 | 2.80 | 1.50 | 0.52 | 2.80 |
| 2.40 | 0.40 | 2.81 | 1.50 | 0.50 | 2.81 |
| 2.60 | 0.38 | 2.80 | 1.50 | 0.48 | 2.80 |
| 2.80 | 0.38 | 2.99 | 1.50 | 0.46 | 2.99 |
| 3.00 | 0.37 | 2.94 | 1.50 | 0.45 | 2.94 |
| 4.00 | 0.30 | 2.65 | 1.50 | 0.40 | 2.65 |
| 5.00 | 0.23 | 2.06 | 1.50 | 0.37 | 2.06 |
| 6.00 | 0.17 | 1.36 | 1.50 | 0.35 | 1.50 |
| 7.00 | 0.10 | 0.72 | 1.50 | 0.34 | 1.50 |
| 7.125 | 0.08 | 0.58 | 1.50 | 0.33 | 1.50 |

Proposal 1: adopt Table 1 in R4-2503498 for minimum range length with 50cm quiet zone.

The split measurement grid method proposed in R4-2502367 [2] performs the same test four times with different grid positions and combines the results. Each time the XR device, e.g. a pair of glasses, would be removed from the phantom and put back after a fully charging its battery. Measurement Uncertainty (MU) related to use of phantom in 38.870 [3] has several components, one of which is “Positioning Uncertainty from using Phantoms” in clause B.2.11.5. Assuming the MU value for this component is MUXR_positioning and using the principle of Root Sum Square to account for four positioning MUs during the split measurement grid process, the final MU after four positioning would be

MUXR_positioning_split_grid = sqrt (4 x MUXR_positioning2) = 2 x MUXR_positioning

The actual value of MUXR_positioning is not yet available. The proposal below is on how to calculate the composite positioning MU for split measurement grid method.

Proposal 2: the composite positioning MU value in split measurement grid method should be twice of the “positioning uncertainty from using phantom”.

# Conclusion

This contribution provides the following proposals.

Proposal 1: adopt Table 1 in R4-2503498 for minimum range length with 50cm quiet zone.

Proposal 2: the composite positioning MU value in split measurement grid method should be twice of the “positioning uncertainty from using phantom”.

# Reference

- R4-25023760 Way Forward for [114][327] TRP_TRS_Ph3

- R4-2502367 CR for split measurement grids method for TRP/TRS measurements

- 38.870 v18.4 Enhanced Over-the-Air (OTA) test methods for NR FR1 Total Radiated Power (TRP) and Total Radiated Sensitivity (TRS)