3GPP TSG-RAN WG4 Meeting #114bis	R4-2503500

Wuhan, China, Apr 7th – 11st, 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements for UE to UE CLI handling

Agenda item:	7.23.3.1

Document for:	Discussion

1. Introduction

In the last RAN4 meeting, the following agreements were made in the WF [1]:

Sub-topic 1-1: RRM impacts of UE-to-UE CLI handling Issue 1-1-2: Measurement methods - Agreements (from Thu AH session) - RAN4 shall specify L1 CLI measurement requirements at least for below methods: - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - Additional methods can be considered based on further RAN1 agreements. - Differentiation on requirements based on different methods should be considered, based on further RAN1 conclusions. Issue 1-1-3: Rx beam - Agreements (from Thu AH session) - RAN4 to capture the Rx beam determination for L1 CLI measurement based on RAN1 agreement, FFS how to capture, e.g. specify the UE behaviour or use a reference to RAN1 spec. - Rx beam sweeping is not needed for L1 CLI measurement. Issue 1-1-6: Measurement period - Agreements (from Wed AH session) - For L1-SRS-RSRP, further discuss the following options based on simulation results: - Option 1: requirement specified with [1] sample measurement assumption - Option 2: requirement specified based on [1] and [3] samples according to NW configuration, e.g. timeRestrictionForChannelMeasurement - For L1-CLI-RSSI, - Confirm measurement period requirements are defined based on 1 sample, and - Clarify in the requirements that the measurement is based on DL timing. Issue 1-1-9: Scheduling and measurement restriction - Agreements (from Thu AH session) - For scheduling restriction, - For L1-SRS-RSRP, - For FR1, R16 requirements are re-used as baseline, FFS exact number of restricted symbols considering the impact of timing assumption - For FR2, FFS whether R16 requirements are re-used as baseline, considering the impact of Rx beam determination, and timing assumption - For L1-CLI-RSSI, FFS whether R16 requirements are re-used as baseline, considering the measurement is based on DL timing. - For measurement restriction, - Requirements for L1 CLI measurement do not apply when the L1 CLI measurement resource collides with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps. - FFS whether need to consider the exception cases where L1 CLI measurement can be performed with other measurement in parallel.

2. General

  - Measurement Restriction

In RAN1 #119, it has been agreed that Method #3 for measuring UE-to-UE CLI is supported. Moreover, note that Method#1 and Method#3 cannot be configured in the same OFDM symbol nor be associated with the same CSI-ReportConfig.

Agreement (RAN1#119) For L1 UE-to-UE CLI measurement and reporting, Method#3 is supported - Method #3: UE measures RSSI within UL subband - A UE cannot be configured to perform measurement using Method #1 and Methods#3 in the same OFDM symbol. Measurement resource(s) corresponding to Methods #1 and #3 shall not be associated with the same CSI-ReportConfig.

Currently, there are no measurement restrictions explicitly specified for L3 CLI measurements, only scheduling restrictions, which are defined in TS 38.133 Clause 9.7.4. But the following text in clause 9.7.2.5 and 9.7.3.3.5, the L3 CLI measurements, including both SRS-RSRP and CLI-RSSI measurement, are assumed deprioritized comparing to other L3 and RLM/BFD/CBD or L1-RSRP measurements.

TS 38.133 clause 9.7.2.5 If the SRS resources configured for measurement are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps, requirements are not specified for TSRS_RSRP_measurement_period. TS 38.133 clause 9.7.3.5 If the CLI-RSSI measurement resources configured for measurement are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps, requirements are not specified for TCLI_RSSI_measurement_period.

Observation 1: In R16, the L3 CLI measurements, including both SRS-RSRP and CLI-RSSI measurement, are assumed deprioritized comparing to other L3 and RLM/BFD/CBD or L1-RSRP measurements.

In Rel-19, the CLI SRS-RSRP and CLI-RSSI measurements method are L1 measurements. Thus, similarly as for L1-RSRP/L1-SINR measurements, measurement restrictions shall be defined for L1 CLI measurement. The RAN1 agreement states that the UE cannot be configured to perform measurement using Methods#1 and Method#3 in the same OFDM symbol. This needs to be captured in the measurement restriction for L1-CLI-RSSI measurement. Therefore, our view is that a measurement restriction shall be defined at least in that case.

RAN4 shall specify measurement restrictions for L1-CLI measurements including CLI-RSSI in the DL subband i.e. Method#1, and in the UL subband, i.e. Method#3 and the SRS-RSRP measurement in the UL subband i.e. Method2.

Our view is that the L3 CLI requirements can be used as baseline for the L1-CLI measurement restriction, i.e., the L1-CLI measurement requirements are not specified in case of overlap with the signals mentioned above i.e. when it is configured in the same symbol as the RS for RLM, BFD, CBD, L1-RSRP or L1-SINR, or when it overlaps with measurement gaps.

Additionally, there is a need to consider the measurement restrictions when the L1-CLI-RSSI measurement is configured in the same symbol as the L1-SRS-RSRP, i.e. Method #2. We can apply the same principle to prioritize the measurements on SRS resources.

L1-CLI measurement requirements are not specified when it is configured in the same symbol as the RS for RLM, BFD, CBD, L1-RSRP or L1-SINR, or when it overlaps with measurement gaps.

L1-CLI-RSSI measurement requirements are not specified when the configured L1-CLI-RSSI measurement resources are partially or fully overlapping in time with configured L1-SRS-RSRP for method #2.

Moreover, there are differences between Method #1 and Method #3 regarding the side conditions. Comparing to Method#1, the UE is likely to measure higher interference power when measuring L1-CLI-RSSI in Method#3, hence a different side condition is expected to achieve the same measurement performance.

RAN4 to discuss if different side conditions shall be defined for L1-CLI-RSSI measurement in Method #1 and Method #3.

Regarding the SCS assumptions for the L1-CLI-RSSI measurements in Method#1 and Method#3, we would like to discuss RAN1 agreements and confirm our understanding in RAN4.

In RAN1#120, it has been agreed that:

Agreement - UE determines the subcarrier spacing of the CLI-RSSI measurement resources configured in a CSI-ResourceConfig based on the subcarrier spacing of the DL BWP identified by the higher-layer parameter bwp-Id in the CSI-ResourceConfig. - UE determines the subcarrier spacing of the SRS-RSRP measurement resources configured in a CSI-ResourceConfig based on the subcarrier spacing of the DL BWP identified by the higher-layer parameter bwp-Id in the CSI-ResourceConfig. - Note: No RAN1 spec impact is expected. Agreement For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to ‘cli-RSSI’ or ‘cli-SRS-RSRP’ - carrier in CSI-ReportConfig is used to indicate in which serving cell the CLI-RSSI measurement resources or SRS-RSRP measurement resources in CSI-ResourceConfig are to be found. If the field is absent, the resources are on the same serving cell as this report configuration. - bwp-Id in the associated CSI-ResourceConfig is used to indicate the DL BWP where the CLI-RSSI measurement resources or SRS-RSRP measurement resources are located in. Note: No RAN1 specification impact Agreement - UE determines the subcarrier spacing of the CLI-RSSI measurement resources configured in a CSI-ResourceConfig based on the subcarrier spacing of the DL BWP identified by the higher-layer parameter bwp-Id in the CSI-ResourceConfig. - UE determines the subcarrier spacing of the SRS-RSRP measurement resources configured in a CSI-ResourceConfig based on the subcarrier spacing of the DL BWP identified by the higher-layer parameter bwp-Id in the CSI-ResourceConfig. - Note: No RAN1 spec impact is expected.

From the RAN1 agreements we make the following observations:

- L1-CLI-RSSI and L1-SRS-RSRP measurements can be configured to be performed in resources from:

- In the same carrier as the serving cell, within the active BWP or in another configured BWP

- In a different carrier than the serving cell.

- The subcarrier spacing to be used in the CLI-RSSI and in the SRS-RSRP measurement is based on the subcarrier spacing of the DL BWP identified by the higher-layer parameter bwp-Id in the CSI-ResourceConfig

The UE shall perform CLI-RSSI measurement with the SCS of the DL BWP identified by the higher-layer parameter bwp-Id in the CSI-ResourceConfig. when measuring according to Method #1 and Method #3.

  - Rx beam

Regarding the assumptions on the UE Rx beam to perform the L1 CLI measurements, RAN1 has agreed on the following:

Agreement (RAN1#118-bis) For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH, support the following in the IE CSI-AperiodicTriggerStateList: - In CSI-AssociatedReportConfigInfo a list of TCI state(s) with qcl-Type set to 'typeD' is optionally configured where the TCI state(s) correspond to the resource(s) in the set of CLI measurement resources indicated by CSI-AssociatedReportConfigInfo - The list of TCI states can be configured (present) only if unifiedTCI-StateType is configured and the CLI measurement resource(s) are aperiodic - If the list of TCI states is not configured (absent), the UE assumes the following for each CLI measurement resource: - If unifiedTCI-StateType is not configured - Same assumption as for L3-based CLI measurement as in 38.133: “For performing CLI measurement in FR2, UE can assume the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET” - If unifiedTCI-StateType is configured - “Indicated” DL only/joint TCI state as specified in TS 38.214 - Note: Above default rules apply to aperiodic reporting based on aperiodic, semi-persistent, and periodic CLI measurement resources FFS: details of configuration of TCI state(s) for aperiodic reporting based on periodic and semi-persistent CLI measurement resources Agreement (RAN1#119) For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH based on a set of periodic CLI measurement resources, the TCI state(s) with QCL typeD for the CLI measurement resources in the periodic CLI measurement resource set are configured per CLI measurement resource (either for all resources in the set or none) by higher-layer parameter - For each periodic CLI measurement resource, this higher-layer parameter provides a reference to one TCI-State in TCI-States for providing the QCL source and QCL typeD. - If the TCI state is not configured, the UE QCL assumptions are the default rules agreed in RAN1#118bis For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH based on a set of semi-persistent CLI measurement resources, the gNB may activate and deactivate the configured semi-persistent CLI measurement resource sets by sending a new SP CLI Measurement Resource Set Activation/Deactivation MAC CE - TCI state IDi field is included in the new MAC CE, which is used as typeD QCL source for the CLI measurement resource within the Semi Persistent CLI measurement resource set. Either all TCI state ID fields are present or none. - If the TCI state ID field(s) are absent, the UE QCL assumptions are the default rules agreed in RAN1#118bis

In RAN4 #114, this issue has also been captured in the WF:

Issue 1-1-2: Rx beam - Proposals - Proposal 1a (LGE): - RAN4 can capture the original statements of RAN1 agreements. If necessary minor change based on the statement of RAN1 agreements can be considered. - Proposal 1b (ZTE, CATT): - directly refer to RAN1 spec instead of repeating all details in RAN spec. - Proposal 2 (Nokia): - The method used by the UE to determine the TCI state to be used in the L1 CLI measurement will not affect the measurement period. - Recommended WF - Agree on P2: The method used by the UE to determine the TCI state to be used in the L1 CLI measurement will not affect the measurement period. - Further discuss P1a and P1b in next meetings.

In the Rel-16 CLI measurements requirements, the FR2 UE assumes that the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET. As shown above, in Rel-19 Duplex evolution, RAN1 has agreed that the gNB can optionally configure the UE with a given TCI state for the CLI measurement, either by configuring a list of TCI states or configuring a unifiedTCI-StateType. Otherwise, the assumption from Rel-16 CLI measurements will be used by the UE. In all the cases above, RAN4 has agreed in RAN4 #113 that no Rx beam sweeping is needed for the L1 CLI measurement. Therefore, we think that the same measurement period can be defined in all cases, disregarding how the UE will determine the Rx beam.

The method used by the UE to determine the TCI state to be used in the L1 CLI measurement will not affect the measurement period.

  - Measurement resources

The following agreements/conclusions related to measurement resources were reached by RAN4:

#### Issue 1-1-3: Measurement resources - Agreements - Define measurement requirements assuming wideband reporting for L1-SRS-RSRP and L1-CLI-RSSI, and do not need to consider subband reporting. FFS if UE is able to measure both DL subbands simultaneously in case of DUD and how such measurement capability affects measurements period and accuracy requirements. - RAN4 to define requirements for L1-CLI-RSRI and L1-SRS-RSRP measurement for semi-static, aperiodic and periodic resource types.

The issue of whether a UE is able to measure both DL subbands simultaneously in case of DUD is still FFS. In our opinion, a UE configured with a L1-CLI-RSSI spanning across the 2 DL subbands of a DUD SBFD slot is able to simultaneously measure the CLI-RSSI while not accounting for the UL subband. On the accuracy performance, the UE scales the measured CLI-RSSI to report a nominal RSSI equivalent to 6RB measurement with 15 kHz SCS. This is different from the CSI-RS measurement which depends on the measured bandwidth, hence the puncturing on UL subband for CLI-RSSI would not impact the accuracy performance. Thus, we do not expect any impact on the measurement period and accuracy requirements.

A UE configured to measure L1-CLI-RSSI across 2 DL subbands on a DUD SBFD slot is able to simultaneously measure CLI-RSSI over both subbands. No expected impact on measurement period and accuracy requirements.

  - Measurement reporting

#### Issue 1-1-4: Measurement reporting - Agreements - Focus on aperiodic reporting mode for Rel-19 CLI measurement, for other reporting modes hold on the discussion pending on further progress in RAN1. - L1-RSRP measurement reporting requirement for aperiodic reporting can be reused for the aperiodic L1 based CLI measurement reporting.

As shown below, RAN1#120 agreed to support periodic reporting for L1-CLI-RSSI. On the other hand, semi-persistent reporting for L1-CLI-RSSI and L1-SRS-RSRP and periodic L1-SRS-RSRP reporting are not supported.

Agreement from RAN1#120 - Periodic L1 CLI-RSSI reporting for L1 based UE-to-UE CLI measurement and reporting is supported - Semi-persistent L1 CLI-RSSI reporting for L1 based UE-to-UE CLI measurement and reporting is NOT supported - Periodic/Semi-persistent L1 SRS-RSRP reporting for L1 based UE-to-UE CLI measurement and reporting is NOT supported

RAN4 should take these latest agreements into account for the definition of the Rel-19 L1 CLI requirements. And for periodic reporting of L1-CLI-RSSI, the existing periodic L1-RSRP reporting requirement can be reused.

Consider the periodic reporting of L1-CLI-RSSI for the definition of the Rel-19 L1 CLI requirements.

  - Report mapping

Regarding the L1-SRS-RSRP and L1-CLI-RSSI reporting, RAN1#119 agreed to introduce 2 new reporting quantities in the CSI-ReportConfig and to support absolute and differential reporting:

Agreement from RAN1#119 For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to ‘cli-RSSI’ or ‘cli-SRS-RSRP’, the following are supported - The number of reported CLI measurement resources M can be configured as {1, 2, 3, 4} - The M CLI measurement resource ID(s) are reported - The absolute (not differential) value of the most interfered (largest) measured L1-CLI-RSSI or L1-SRS-RSRP is reported - When the number of reported CLI measurement resources is larger than one, a differential reporting method is used taking the most interfered measured L1-CLI-RSSI or L1-SRS-RSRP as the reference - FFS: other reporting criteria, e.g., the least interfered measured L1-CLI-RSSI or L1-SRS-RSRP - FFS: whether/how to report differential value in case the largest measurement is out of range

In the following we discuss how the absolute power measurements and differential power measurements should be reported for L1-SRS-RSRP and L1-CLI-RSSI

L1-SRS-RSRP

For the L1-SRS-RSRP absolute power measurement reporting, the L3-SRS-RSRP report mapping table from TS 38.133 Section 10.1.22.1. can be re-used. Current L3-SRS-RSRP absolute power reporting assumes a 7-bit quantized value with a reporting range from -140 dBm to -44 dBm and with a resolution of 1 dB. For the L1-SRS-RSRP differential power measurement reporting, we suggest re-using the same principle as the current L1-RSRP differential reporting (see Table 10.1.6.1-2, TS 38.133). Thus, the differential L1-SRS-RSRP report consists of 4 bits with 2 dB step size. The largest measured L1-SRS-RSRP is used as reference to derive the differential values.

For L1 SRS-RSRP absolute power measurement reporting, use a 7-bit value in the quantization range [-140, -44] dBm with 1 dB step size.

For L1 SRS-RSRP differential power measurement reporting, use a 4-bit value in the quantization range with 2 dB step size. The largest measured L1-SRS-RSRP is used as reference to derive the differential values.

- L1-CLI-RSSI

Regarding the L1-CLI-RSSI absolute power measurement reporting, our preference is to re-use the L3-CLI-RSSI reporting mapping table from TS 38.133 Section 10.1.22.2.  The L3-CLI-RSSI reporting range spans from -100 dBm to -25 dBm with 1 dB resolution. Similarly, as for L1-SRS-RSRP, RAN4 should introduce a new report mapping table for differential L1-CLI-RSSI following the same principle as L1-RSRP differential reporting. Thus, the differential L1-CLI-RSSI report consist of 4 bits wit 2dB resolution. The largest L1-CLI-RSSI is used as reference to derive the differential values.

For L1 CLI-RSSI absolute power measurement reporting, use a 7-bit value in the quantization range [-100, -25] dBm with 1dB step size.

For L1 CLI-RSSI differential power measurement reporting, use a 4-bit value in the quantization range with 2 dB step size. The largest measured L1-CLI-RSSI is used as reference to derive the differential values.

Moreover, we think that RAN4 shall discuss whether different reporting values are needed for Method #1 and Method #3. In our opinion, the proposed L1-CLI-RSSI report mapping table covers a large range of interference power, and therefore it can be applicable to both measurement methods.

The same report mapping table is applicable to both Method #1 and Method #3.

Regarding this topic, during RAN1#120 RAN1 agreed to send an LS [3] to RAN4 to confirm the assumptions about the reporting range and granularity of L1-SRS-RSRP and L1-CLI-RSSI reporting. The current RAN1 assumptions are well-aligned with our views and therefore, we suggest RAN4 to reply that current RAN1 assumptions are reasonable.

RAN4 to reply to RAN1 that their current assumptions for L1-SRS-RSRP and L1-CLI-RSSI reporting range and granularity are reasonable for both absolute power and differential power measurements.

  - Scheduling restriction

In the last RAN4 meeting, the following agreement was reached regarding the scheduling restrictions:

Agreements (from Thu AH session) - For L1-SRS-RSRP, - For UL, scheduling restrictions always apply, for both FR1 and FR2. - For DL, scheduling restriction does not apply to the symbols with SRS if UE is able to support FDM-ed DL reception and SRS measurement if RAN1 will define optional UE capability for FDMed DL reception and SRS measurement; otherwise (if UE does not support the capability or RAN1 does not define the capability), scheduling restriction applies. - The restricted symbols at least include symbols for measurement resource and Y symbols before symbols for measurement resource and Y is re-using same number as R16 requirements, - Note: this time has accounted for the UL/DL switching time. - FFS whether there is restriction on symbols after the symbols for measurement resource considering UL/DL transition. - For L1-CLI-RSSI Methods #1, - For UL, scheduling restrictions always apply, for both FR1 and FR2. - For DL, scheduling restrictions do not apply except when CLI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2. - The restricted symbols at least include symbols for measurement resource, FFS whether restriction on additional symbols is needed. - For L1-CLI-RSSI Method #3, scheduling restriction requirements are FFS

UL Scheduling restrictions during L1-SRS-RSRP measurements

In the last RAN4 meeting it has been agreed that the restricted symbols for UL transmission include at least Y symbols before the L1-SRS-RSRP measurement, where Y reuses the same number of symbols as Rel-16 CLI measurements. This is illustrated in the figure below, and denoted as time T2:

Figure 1 – UL Scheduling restrictions during L1-SRS-RSRP measurements

It has been agreed in RAN4 #112 that the L1-SRS-RSRSP measurements are performed with a constant offset relative to the DL reference timing:

Issue 1-1-14: Measurement timing for L1-SRS-RSRP - Agreements (from online session) - As baseline, for L1 SRS-RSRP measurement, UE will apply a constant offset relative to the downlink reference timing in the serving cell, and the constant offset value is derived by UE implementation and shall be at least Tc*NTA_offset. - This can be revisited if it proves to be conflicting with further RAN1 agreements.

It is still open for discussion whether there is restriction on symbols after the measurement resource. This is denoted as time T1 in the figure above. Here we note that:

- The L1-SRS-RSRP measurement is performed with, at least, the following offset with respect to the DL timing: Tc*NTA_offset

- The UL signal transmission is performed with, at least, the following offset with respect to the DL timing: Tc*(NTA_offset + NTA)

Our view is that, if the UE that is performing L1-SRS-RSRP measurements is scheduled to transmit in UL right after this measurement, it should have enough time to do the Rx-Tx switching, which is accounted for in the offset Tc*NTA_offset, and for at least compensate for the N_TA. Therefore, we propose the following change to the agreement reached in the last RAN4 meeting:

The symbols restricted for UL transmissions include symbols for measurement resource and Y symbols before symbols for measurement resource, and X symbols after symbols for measurement resource and Y is re-using same number as R16 requirements and X is FFS.

Scheduling restrictions during L1-CLI-RSSI measurements

The L1-CLI-RSSI measurement is slightly different from the L1-SRS-RSRP measurement. The CLI-RSSI is measured with the DL timing. In the following Sections, we discuss the scheduling restrictions for Method 1 and method 3.

Method#1

Figure 2 - Scheduling restrictions in L1-CLI-RSSI method #1

In Method 1, the UE measures the CLI-RSSI in the DL subband as in the figure above. It has already been agreed that the UL restriction always apply for both FR1 and FR2 at least in the symbols used for measurement. CLI-RSSI is measured with the DL timing.

T1 in the figure represents the time between the end of the L1-CLI-RSSI measurement and the start of the UL transmission. After the UE has measured RSSI, it has to do a Rx-Tx switch to be able to transmit UL in the UL subband. Therefore, in our view, the symbols after the L1-CLI-RSSI measurement should be restricted.

T2 in the figure represents the time between the end of the PUSCH transmission and the start of the RSSI measurement. The UE needs to do an Tx to Rx switching before being able to measure RSSI. However, the UL transmission is sent with timing advance with at least Tc*(NTA_offset + NTA). Therefore, we believe that the time needed for the Tx-Rx transition before the L1-CLI-RSSI measurement is already taken into account in the offset for the PUSCH transmission.

For L1-CLI-RSSI Method 1 measurements, the symbols restricted for UL transmissions include at least symbols for measurement resource and Y symbols after symbols for measurement resource.

Method#3

| Figure 3 – DL scheduling restrictions during L1-CLI-RSSI measurements Method #3 | Figure 4 - UL scheduling restrictions during L1-CLI-RSSI measurements Method #3 |
| --- | --- |

In Figure 3 and in Figure 4 we show the DL scheduling restrictions and the UL scheduling restrictions for Method #3, respectively.

T1 represents the time between the end of the CLI-RSSI measurement and the start of the DL reception or UL transmission. And T2 represents the time between the end of the DL reception or UL transmission and the start of the CLI-RSSI measurement.

DL scheduling restrictions for L1-CLI-RSSI Method #3

In Method#3, the CLI-RSSI measurement and the DL reception are done in different subbands. Given that both the L1-CLI-RSSI are performed using the DL timing, we think there is no need to apply DL scheduling restrictions before or after the DL reception. The only FFS point is whether the UE is capable of simultaneously performing DL reception and CLI-RSSI measurements. If this is not supported, the UE is not expected to be scheduled in DL during the L1-CLI-RSSI measurements.

There is no need to define DL scheduling restrictions in symbols before or after the L1-CLI-RSSI measurement in Method #3.

UL scheduling restrictions for L1-CLI-RSSI Method #3

In this case, the UE is expected to perform L1-CLI-RSSI according to the UE’s DL timing while the UL transmissions are performed with the UE’s UL timing. Thus, for the T1 time interval, i.e., the time after the L1-CLI-RSSI measurement, we think that UL scheduling restrictions should be considered to account for the Rx-to-Tx switch and the timing advance. As for the Method#1, we think that there is no need to define UL scheduling restrictions for the T2 interval.

For L1-CLI-RSSI measurements based on Method 3, the symbols restricted for UL transmissions include symbols for measurement resource and Z symbols after symbols for measurement resource.

  - Side conditions

- Agreements (from online session) - For SRS configuration, reuse R16 assumptions on SRS configuration for L1 SRS-RSRP measurements - Agreements - Further discuss timing offset side condition for accuracy requirements in the performance part of the WI, including whether to consider intra-cell scenario only or both intra- and inter-cell scenarios for defining the side condition. - For SRS Es/Iot, further discuss the following options - Option 1: 1dB - Option 2: > 1dB - For SRS BW, further discuss the following options - Option 1: 24RB - Option 2: 48RB

Timing offset for L1-SRS-RSRP

Given the reply from RAN1 in [2], intra-cell measurements can be prioritized for the requirements definition in RAN4. If the intra-cell scenario is agreeable as the starting point, the cell phase error can be zero as victim and aggressor UEs are assumed in the same cell. As the propagation delay difference is derived assuming the victim and aggressor UEs are located in the cell center and cell edge respectively, it remains the same irrespective of intra-cell or inter-cell. Hence the residual timing error can be derived assuming cell phase error is zero and same cell size as in R16.

For L1-SRS-RSRP measurement in intra-cell scenario, the residual timing error is derived assuming cell phase error is zero and same cell size as in R16.

SRS Es/IoT

The SRS Es/Iot value may be higher considering the L1-SRS-RSRP measures the interference on UL subband where there is no other DL transmission hence less interference can be expected in the victim UE as compared to existing SRS-RSRP measurements. Current simulation assumptions for L1-SRSP RSRP measurement requirements cover SRS SNR values of 1 dB, 2 dB and 3 dB. We think RAN4 should define a higher value of SRS Es/Iot as side condition for the L1 CLI SRS-RSRP measurement on the UL subband in SBFD operation.

RAN4 should define a higher value of SRS Es/Iot as side condition for the L1 CLI SRS-RSRP measurement on the UL subband in SBFD operation.

SRS bandwidth

Current simulation assumptions for L1-SRSP RSRP measurement requirements assume SRS bandwidths of 24 PRBs and 48 PRBs. Note that existing SRS-RSRP measurement requirements assume a SRS bandwidth of 48 PRBs. Based on our results, we conclude that the measurement reports show very similar performance with both bandwidth values. Given that certain SBFD configurations might have a limited UL subband size, we prefer that L1-SRS-RSRP measurement requirements should consider a SRS bandwidth of 24 PRBs.

The SRS bandwidth of 24 PRBs is assumed for defining L1-SRS-RSRP measurement requirements.

  - Measurement period

On the measurement period, the WF from RAN4#113 captured the following:

- Recommended WF - For L1-SRS-RSRP, further discuss the following options based on simulation results. - Option 1: single set of requirements based on [1] sample - Option 2: two sets of requirements based on [1] and [3] samples based on NW configuration, e.g. timeRestrictionForChannelMeasurement - Other options are not precluded - For L1-CLI-RSSI, - Confirm measurement period requirements are defined based on 1 sample, and - Clarify in the requirements that the measurement is based on DL timing.

During RAN4#114, RAN4 agreed to send an LS to RAN1 (R4-2502632) to ask whether the RRC parameter timeRestrictionForChannelMeasurements is applicable to L1-SRS-RSRP and/or L1-CLI-RSSI measurements. RAN4 shall then wait for RAN1 output before making any decision on this topic.

RAN4 to wait for RAN1 reply on whether the parameter timeRestrictionForChannelMeasurement is applicable to L1-SRS-RSRP and/or L1-CLI-RSSI.

  - Requirement applicability

In R16 CLI measurements, the measurement requirements apply only when the measurement resource is fully confined within active BWP. While SBFD operation is considered in R19, the SRS-RSRP and CLI-RSSI in Method#3 will be configured in UL subband. However, the UL subband is also considered within DL active BWP. Hence the existing applicability conditions in R16 are still applicable to R19 L1 CLI measurement.

TS 38.133 clause 9.7.1:

CLI measurements are only applicable for RRC_CONNECTED intra-frequency:

-	when SRS-RSRP measurement resource is fully confined within BW of DL active BWP

-	when CLI-RSSI measurement resource is configured within active BWP

The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency:

-	when SRS-RSRP measurement resource is fully confined within BW of DL active BWP

-	when CLI-RSSI measurement resource is configured within active BWP

Conclusion

The following Observations and Proposals were made:

Observation 1: In R16, the L3 CLI measurements, including both SRS-RSRP and CLI-RSSI measurement, are assumed deprioritized comparing to other L3 and RLM/BFD/CBD or L1-RSRP measurements.

- RAN4 shall specify measurement restrictions for L1-CLI measurements including CLI-RSSI in the DL subband i.e. Method#1, and in the UL subband, i.e. Method#3 and the SRS-RSRP measurement in the UL subband i.e. Method2.

- L1-CLI measurement requirements are not specified when it is configured in the same symbol as the RS for RLM, BFD, CBD, L1-RSRP or L1-SINR, or when it overlaps with measurement gaps.

- L1-CLI-RSSI measurement requirements are not specified when the configured L1-CLI-RSSI measurement resources are partially or fully overlapping in time with configured L1-SRS-RSRP for method #2.

- RAN4 to discuss if different side conditions shall be defined for L1-CLI-RSSI measurement in Method #1 and Method #3.

- The UE shall perform CLI-RSSI measurement with the SCS of the DL BWP identified by the higher-layer parameter bwp-Id in the CSI-ResourceConfig. when measuring according to Method #1 and Method #3.

- The method used by the UE to determine the TCI state to be used in the L1 CLI measurement will not affect the measurement period.

- A UE configured to measure L1-CLI-RSSI across 2 DL subbands on a DUD SBFD slot is able to simultaneously measure CLI-RSSI over both subbands. No expected impact on measurement period and accuracy requirements.

- Consider the periodic reporting of L1-CLI-RSSI for the definition of the Rel-19 L1 CLI requirements.

- For L1 SRS-RSRP absolute power measurement reporting, use a 7-bit value in the quantization range [-140, -44] dBm with 1 dB step size.

- For L1 SRS-RSRP differential power measurement reporting, use a 4-bit value in the quantization range with 2 dB step size. The largest measured L1-SRS-RSRP is used as reference to derive the differential values.

- For L1 CLI-RSSI absolute power measurement reporting, use a 7-bit value in the quantization range [-100, -25] dBm with 1dB step size.

- For L1 CLI-RSSI differential power measurement reporting, use a 4-bit value in the quantization range with 2 dB step size. The largest measured L1-CLI-RSSI is used as reference to derive the differential values.

- The same report mapping table is applicable to both Method #1 and Method #3.

- RAN4 to reply to RAN1 that their current assumptions for L1-SRS-RSRP and L1-CLI-RSSI reporting range and granularity are reasonable for both absolute power and differential power measurements.

- The symbols restricted for UL transmissions include symbols for measurement resource and Y symbols before symbols for measurement resource, and X symbols after symbols for measurement resource and Y is re-using same number as R16 requirements and X is FFS.

- For L1-CLI-RSSI Method 1 measurements, the symbols restricted for UL transmissions include at least symbols for measurement resource and Y symbols after symbols for measurement resource.

- There is no need to define DL scheduling restrictions in symbols before or after the L1-CLI-RSSI measurement in Method #3.

- For L1-CLI-RSSI measurements based on Method 3, the symbols restricted for UL transmissions include symbols for measurement resource and Z symbols after symbols for measurement resource.

- For L1-SRS-RSRP measurement in intra-cell scenario, the residual timing error is derived assuming cell phase error is zero and same cell size as in R16.

- RAN4 should define a higher value of SRS Es/Iot as side condition for the L1 CLI SRS-RSRP measurement on the UL subband in SBFD operation.

- The SRS bandwidth of 24 PRBs is assumed for defining L1-SRS-RSRP measurement requirements.

- RAN4 to wait for RAN1 reply on whether the parameter timeRestrictionForChannelMeasurement is applicable to L1-SRS-RSRP and/or L1-CLI-RSSI.

- The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency:

-	when SRS-RSRP measurement resource is fully confined within BW of DL active BWP

-	when CLI-RSSI measurement resource is configured within active BWP

References

- R4-2420097, WF on RRM requirements for NR_duplex_evo, Huawei

- R1-2410890, “Reply LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation”

- R1-2501620, “LS on L1 based UE-to-UE CLI measurement and reporting”