3GPP TSG-RAN WG4 Meeting #114-bis					 R4-2503512

Wuhan, China, 7th -11th April, 2025

Source:	Qualcomm Incorporated

Title:	Power boosting and MPR reduction

Agenda item:	7.1.1.2.1

Document for:	Approval

# 1.	Introduction

In RAN4#114 there was a new WF [1] to further study power domain enhancements for NR single carrier. In this paper we present additional views on this topic.

# 2. 	Discussion

In RAN4#114 there was a WF created [1] to further study the open items on power domain enhancements for NR single carrier. Some of the unresolved issues are noted below:

In there was much discussion on how the UE can either calculate or be informed of the free RBs on either side of the UE CBW. The two proposals discussed for determining the number of free RBs were by indication through NW signalling or by UE calculation based on information received during RRC configuration and via SIB information. We think that it would easier for the NW to perform the computations as it has all the necessary information more readily than the UE. Therefore the simplist solution is for the NW to signal the UE bandwidth expansion.

Proposal 1: The symmetric or asymmetric UE CBW extension in terms of either RBs or UE CBW is signalled by the NW.

Another issue to resolve is whether CBW or RB should be used for UE CBW extension. We think that extending by a fraction of the CBW, such as CBW/2, and extending to the next highest value of CBW supported in table 5.3.2-1[1] may lead to the need for more free spectrum than using the NRB/2 approach. If the CBW is not extended to the next largest value in table 5.3.2-1 then more bandwidth configurations must be supported by the UE. Figure 1 illustrates the minimum number of free RBs required when extending by NRB/2 versus extending by CBW/2 to the next CBW value in table 5.3.2-1.

Figure 1: Minimum number of RBs required for asymmetrical CBW extension

For example, CBW=25MHz has 65 RBs. If the UE CBW is asymmetrically extended on one side by NRB/2 then an additional 33RBs would be required. However, if extended by CBW/2 then the new CBW becomes 37.5MHz and when extended to the next highest CBW value in table 5.3.2-1 becomes 40MHz which requires 41 free RBs. Therefore, the CBW/2 approach requires an additional 9 RBs compared to the NRB/2 approach.

Proposal 2: Define UE CBW extension in terms of NRB

In this contribution, ‘RBs available for ‘extension’ on each side of the UE CBW’ are referred to as ‘free RBs’ for compactness.

In the above mentioned WF it was proposed that companies attempt to formulate the appropriate wording to describe the inner RB region when the UE CBW is extended. In figure 2 the original MPR region comprising of inner and outer regions are shown in the left-side plot and the inner and outer regions for the extended bandwidth scenario is given in the right-side plot. The intersection of the original MPR region with the inner region of the extended bandwidth, indicated in the right-side plot in light green, is the new inner region in the extended bandwidth scenario.

Figure 2: MPR regions for original and extended bandwidth scenarios [2]

Therefore, the MPR region of the original UE-specific CBW which can use inner RB MPR inside the extended UE-specific bandwidth is the intersection of the inner MPR region of the extended UE-specific CBW with the original MPR region of the original UE-specific CBW. Putting this in words makes the description unwieldy:

The MPR region inside the original UE-specific CBW which can use inner RB MPR inside the extended UE bandwidth is given by the intersection of the inner MPR region of the extended UE-specific CBW with the MPR region of the original UE-specific CBW.

Moreover, re-using existing MPR definitions implies limited change to applicable SEM requirements.

Observation 1: Wording to describe the inner RB region when the UE CBW is extended can be problematic.

For scenario2 the UE channel bandwidth extension should not exceed the BS bandwidth as any region outside the BS BW may not belong to the same operator and extension of BW into those regions may not be allowed.

Proposal 3: For scenario2 the UE extended channel should be contained within the cell-specific carrier spectrum (i.e. bandwidth and location).

It was earlier agreed that bandwidth extension can be symmetric or asymmetric on either side of the UE-specific CBW [3]. There are several ways that this BW extension feature can be implemented. As illustrated in figure 3 one way is to add a certain number of RBs, denoted as p and q to one or both sides of the UE-specific CBW based on the number of free RBs on each side of the spectrum. This method gives the greatest flexibility for BW extension. Here NRB is the maximum number of RBs in the original UE-specific CBW.

Figure 2: UE-specific CBW extension

Note the UE CBW occupies a slightly larger BW than NRB as it includes the guard band on both sides of the NRB. The extra virtual RBs, p and q are added starting from the left of RB(1) and the right of RB(NRB) and there is no guard band between the extra RBs and NRB.

Proposal 4: There is no guard band between the UE CBW and the extended UE CBW

In the last meeting the following was agreed [1]:

#### Issue 1-1-1: Boundary to apply SE

- Options

  - Option 1: The SE limits apply for the frequency ranges that are more than UE CBW+5MHz from the edge of the extended UE CBW

  - Option 2: Boundary between NR out of band and general spurious emission domain on power domain enhancements for single carrier context is defined as Extended UE CBW + 5 MHz

- WF

  - option 1, to meet the ITU recommendation

#### Issue 1-1-2: Integral region basis for OOBE

- Options

  - Option 1: The OOBE requirements are based on UE CBW

  - Option 2: The OOBE requirements are based on extended UE CBW

| OP 1 |  |
| --- | --- |
| OP 2 |  |

- WF

  - option 1

ITU in SM.329 define SE based on the ‘necessary bandwidth’ (NB), which in turn is defined as:

For a given class of emission, the width of the frequency band which is just sufficient to ensure the transmission of information at the rate and with the quality required under specified conditions.

Since NB is unchanged despite the CBW extension, the frequency ranges of applicability of the ITU requirements also remain unchanged, at 2*NB away from channel edge. UE CBW can be used as proxy for NB.

Proposal 5: SE starts no further than 2x UE CBW from channel edge when network enables UE to assume BW extension

Based on the agreements from the last meeting stated above [1] the SE limits apply for frequency ranges that are more than UE CBW+5MHz from the edge of the extended UE CBW. Based on this the UE CBW extension, X on either side should be configured such that the frequency offset from where the ITU emissions start should be greater than the frequency offset from where 3GPP SE limits apply. This means that:

(2*UE CBW) ≥ (X+UE CBW + 5MHz).

i.e. X ≤ (UE CBW - 5MHz)

From the above the ITU SE specification limits the UE CBW extension.

Observation 2: The maximum allowed extension on each side is limited to UE CBW - 5MHz

The equations for finding the inner RBs can be modified as follows:

NRB’=NRB+p+q

RBStart,Low = max(1,floor(LCRB/2))

RBStart,High = NRB’-RBStart,Low-LCRB=NRB+p+q-RBStart,Low-LCRB

RBStart,Low ≤ RBStart ≤RBStart,High     & LCRB ≤ Ceil(NRB/2)

The above equations can be used to determine the new inner region based on p and q. The SEM region starts p RBs before the left edge and q RBs beyond the right edge of the UE CBW, in the frequency domain. This approach can be used as alternative to defining in words, the inner RB region when the UE CBW is extended

Proposal 6: An allocation belongs to the inner RB region when the UE CBW is extended such that it satisfies the following inequality:

RBStart,Low ≤ RBStart ≤RBStart,High     & LCRB ≤ Ceil(NRB/2)

RBStart,Low = max(1,floor(LCRB/2))

RBStart,High = NRB’-RBStart,Low-LCRB

Where:

NRB’=NRB+p+q

And ‘p’ and ‘q’ are the RBs available for BW extension on the left and right edge of the UE CBW respectively

While this scheme allows p and q to take on any value subject to meeting observation2 (i.e. allows maximum flexibility for adding RBs on either side of the UE CBW), it requires enormous effort to validate therefore making it impractical for actual implementation.

Observation 3: Extending the UE CBW by an arbitrary number of RBs on one or both sides of the spectrum leads to many RB combinations which would take enormous effort to validate

To reduce the validation effort, it is proposed to limit the bandwidth extension to NRB/2 based on the number of free RBs. If the number of free RBs is greater than NRB/2 then the UE CBW extension is limited to NRB/2. If the number of free RBs is less than NRB/2 then no CBW extension is allowed.

Proposal 7: If the number of free RBs is greater than NRB/2 then the UE CBW extension is limited to NRB/2 if the number of RBs in the BW = (UE CBW - 5MHz) is greater than or equal to NRB/2. If the number of free RBs is less than NRB/2 or the number of RBs in the BW = (UE CBW - 5MHz) is less than NRB/2 then no UE CBW extension is allowed.

In our opinion it makes sense to use IBE from the edges of the transmission bandwidth configuration to the edges of the guardband at the edge of the extended UE CBW. Also, the IBE in this region should be an extension of the IBE in the original UE CBW region as indicated in figure 3.

Proposal 8: IBE should be used from the edges of the transmission bandwidth configuration to the edges of the guardband at the edge of the extended UE CBW

Figure 3: Extension of IBE mask

Proposal 9: The IBE from the edges of transmission bandwidth configuration to the edges of the guardband in the extended UE CBW region should be an extension of the IBE for the signal within the UE CBW.

We think that the guard band at the edge of the extended UE CBW can be the same as the guard band of the original UE CBW as there is no signal occupying the extended UE CBW region the width of the guardband is not critical.

Proposal 10: The guard band at the edge of the extended UE CBW is set to be the same as that of the original UE CBW.

# Conclusion

In this paper we further discuss our views on the study of power domain enhancement and make the following proposals:

Proposal 1: The symmetric or asymmetric UE CBW extension in terms of either RBs or UE CBW can be signalled by the NW.

Proposal 2: Define UE CBW extension in terms of NRB

Observation 1: Wording to describe the inner RB region when the UE CBW is extended can be problematic.

Proposal 3: For scenario2 the UE extended channel should be contained within the cell-specific carrier spectrum (i.e. bandwidth and location).

Proposal 4: There is no guard band between the UE CBW and the extended UE CBW

Proposal 5: SE starts no further than 2x UE CBW from channel edge when network enables UE to assume BW extension

Observation 2: The maximum allowed extension on each side is limited to UE CBW - 5MHz

Proposal 6: An allocation belongs to the inner RB region when the UE CBW is extended such that it satisfies the following inequality:

RBStart,Low ≤ RBStart ≤RBStart,High     & LCRB ≤ Ceil(NRB/2)

RBStart,Low = max(1,floor(LCRB/2))

RBStart,High = NRB’-RBStart,Low-LCRB

Where:

NRB’=NRB+p+q

And ‘p’ and ‘q’ are the RBs available for BW extension on the left and right edge of the UE CBW respectively

Observation 3: Extending the UE CBW by an arbitrary number of RBs on one or both sides of the spectrum leads to many RB combinations which would take enormous effort to validate

Proposal 7: If the number of free RBs is greater than NRB/2 then the UE CBW extension is limited to NRB/2 if the number of RBs in the BW = (UE CBW - 5MHz) is greater than or equal to NRB/2. If the number of free RBs is less than NRB/2 or the number of RBs in the BW = (UE CBW - 5MHz) is less than NRB/2 then no UE CBW extension is allowed.

Proposal 8: IBE should be used from the edges of the transmission bandwidth configuration to the edges of the guardband at the edge of the extended UE CBW

Proposal 9: The IBE from the edges of transmission bandwidth configuration to the edges of the guardband in the extended UE CBW region should be an extension of the IBE for the signal within the UE CBW.

Proposal 10: The guard band at the edge of the extended UE CBW is set to be the same as that of the original UE CBW.

# References

[1] R4-2503013, WF on power domain enhancements, RAN4#114, February 7th – February 11th, 2024

[2] R4-2418015, Redefining allocation types for improved MPR with BS BW larger than UE CBW, USA, 18 – 22 November 2024

[3] R4-2417204, WF on power domain enhancements, RAN4#112-bis, October 12th – October 18th , 2024