# 3GPP TSG-RAN WG4 Meeting #114bis				                                           R4-2503529

# Wuhan, China, 7th – 11th April, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR for Rel-19 CSSF enhancement solution 1 (one CC measurement per band) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, CATT, Huawei, HiSilicon, Qualcomm, Ericsson, ZTE, MediaTek, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Ph5-Core |  |  |  |  |  | Date: |  |  | 2025-03-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The CSSF enhancement solution 1 has been agreed in WF R4-2502592 as followings: Issue 1-3-1: Tentative solution 1: measure one serving CC per band Agreement: - If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are configured for measurement in the same band and UE is allowed to determine the CC in the band to be measured based on followings, - Network indication of enable/disable this CSSF enhancement is per-UE. - Follow the following principle for serving and neighboring cell measurement - Measure PCC if PCC in band - Measure PSCC if PSCC in band - Measure SCC on which UE is configured to report SSB based measurements, if - there is only one SCC on which UE is configured to report SSB based measurements on same FR2 band and no PCC/PSCC in band - Follow the network indication to measure a SCC, if - neither PCC nor PSCC is in band, and - if there is no SCCs on which UE is configured to report SSB based measurements in band, or - if there are multiple SCCs on which UE is configured to report SSB based measurements in band. - Network indication of a specific SCC per-band for measurement. - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band. - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation. - Otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements. - Note: Following the WID, the above agreement applies for the measurement without gap. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the Rel-19 CSSF enhancement solution 1 for TS38.133. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirement for Rel-19 CSSF enhancement solution 1 for UE supporting Rel-19 CSSF enhancement is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.2; 9.1.5.1.1; 9.1.5.1.2; 9.1.5.1.3; 9.1.5.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of Change 1>

### 9.2.3	Number of cells and number of SSB

#### 9.2.3.1	Requirements for FR1

For each intra-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least:

-	8 identified cells, and

-	14 SSBs with different SSB index and/or PCI on the intra-frequency layer, where the number of SSBs in the serving cell (except for the SCell) is not smaller than the number of configured RLM-RS SSB resources.

#### 9.2.3.2	Requirements for FR2

For one single intra-frequency layer in a band, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least:

-	6 identified cells, and

-	24 SSBs with different SSB index and/or PCI,

where this single intra-frequency layer shall be:

-	PCC when UE is configured with SA NR operation mode with PCC in the band; or

-	PSCC when UE is configured with EN-DC with PSCC in the band; or

-	PSCC when UE is configured with NR-DC with PSCC in the band; or

-	One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation.

The UE shall also be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least 2 SSBs on serving cell for each of the other intra-frequency layer(s) in the same band.

For UE supporting [CSSF enhancement for one CC measurement per-band] for intra-frequency measuments without MG, if network indicates to enable one serving carrier measurement for each FR2 band via [TBD], for a given serving FR2 band UE only needs to measure serving cell and neighbor cells on,

- 	PCC, when UE is configured with SA NR or NE-DC operation mode with PCC in this FR2 band; or

-	PSCC, when UE is configured with EN-DC with PSCC in this FR2 band; or

-	PSCC, when UE is configured with NR-DC with PSCC in this FR2 band; or

-	SCC on which UE is configured to report SSB based measurements, when neither PCC nor PSCC is in the same band and this SCC is the only one on which UE is configured to report SSB based measurements in this FR2 band; or

-	SCC indicated by network via [TBD], when

-	neither PCC nor PSCC is in the same band, and,

-	there is no SCCs on which UE is configured to report SSB based measurements in this FR2 band, or

-	there are multiple SCCs on which UE is configured to report SSB based measurements in this FR2 band; or

-	SCC determined by UE implementation, otherwise.

# <End of Change 1>

# <Start of Change 2>

##### 9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps

For UE configured with the E-UTRA-NR dual connectivity operation, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB-based measurements, inter-frequency SSB-based measurements performed outside measurements gaps, intra-frequency CSI-RS L3 measurement and RSSI/channel occupancy measurement with no measurement gap on a carrier subject to CCA when SMTC and RMTC are overlapping will be as specified in table 9.1.5.1.1-1.

Table 9.1.5.1.1-1: CSSFoutside_gap,i scaling factor for EN-DC mode

| Scenario | CSSFoutside_gap,i for FR1 PSCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PSCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required Note 2 | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gp |
| --- | --- | --- | --- | --- | --- | --- |
| EN-DC with FR1 only CA | 1+NPSCC_CSIRS+NPSCC_CCA_RSSI/CO | NSCC_SSB +Y+2x NSCC_CSIRS+ NSCC_CCA_RSSI/CO | N/A | N/A | N/A | NSCC_SSB +Y+2x NSCC_CSIRS+ NSCC_CCA_RSSI/CO |
| EN-DC with FR2 only intra band CA | N/A | N/A | 1+NPSCC_CSIRS | N/A | NSCC_SSB +Y+2x NSCC_CSIRS | NSCC_SSB +Y+2x NSCC_CSIRS |
| EN-DC with FR2 only inter band CA | N/A | N/A | 1+NPSCC_CSIRS | 2x(1+ NSCC_CSIRS_FR2_NCM) Note 3,5 | 2×( NSCC_SSB +Y+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) |
| EN-DC with FR1 +FR2 CA (FR1 PSCell) Note 1 | 1+NPSCC_CSIRS | 2×( NSCC_SSB +Y+2xNSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | N/A | 2x(1+NSCC_CSIRS_FR2_NCM) Note 3 | 2×( NSCC_SSB +Y+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) |
| EN-DC with FR1 +FR2 CA (FR2 PSCell) Note 1 | N/A | NSCC_SSB +Y+2x NSCC_CSIRS | 1+NPSCC_CSIRS | N/A | NSCC_SSB+Y+2x NSCC_CSIRS | NSCC_SSB+Y+2x NSCC_CSIRS |
| NOTE 1: Only one NR FR1 operating band and one NR FR2 operating band are included for FR1+FR2 inter-band EN-DC. NOTE 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2. NOTE 3: CSSFoutside_gap,i =1 if only one SCell is configured and no inter-frequency MO without gap and only SSB based L3 measurement is configured on SCC; CSSFoutside_gap,i =2 if only one SCell is configured and no inter-frequency MO without gap and either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement is configured on SCC. NOTE 4: Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG; otherwise, it is 0. NOTE 5: Only two NR FR2 operating band are included for EN-DC with FR2 only inter-band CA NOTE 6: NPSCC_CSIRS=1 if PSCC is with either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement configured; otherwise, NPSCC_CSIRS =0. NOTE 7: NSCC_CSIRS=Number of configured SCell(s) with either both SSB and CSI-RS based L3 measurement configured or only CSI-RS based L3 measurement configured NOTE 8: NSCC_CSIRS_FR2_NCM=1 if FR2 SCC, where neighbour cell measurement is required, is with either both SSB and CSI-RS configured or only CSI-RS measurement configured; otherwise, NSCC_CSIRS_FR2_NCM=0. NOTE 9: NSCC_SSB=Number of configured SCell(s) with only SSB based L3 measurement configured, which is measured without MG. NOTE 10: NPSCC_CCA_RSSI/CO= 1 if PSCC is configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping; NSCC_CCA_RSSI/CO = Number of MOs for SCell(s) configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping. NOTE 11 If a measurement object configured by PSCell and an NR inter-RAT measurement object configured by E-UTRAN PCell are on the same serving carrier, they shall be counted as one intra-frequency measurement object, provided that they meet the measurement object merging conditions [in clause 9.1.3.2], otherwise they are counted separately as two measurement objects. NOTE 12: For UE supporting [CSSF enhancement for one CC measurement per-band] for intra-frequency measurements without MG, if network indicates to enable one serving carrier measurement for each FR2 band via [TBD], the being-measured FR2 CC(s) counted in CSSF shall follow the principles specified in clause 9.2.3.2. |  |  |  |  |  |  |

##### 9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps

For UE in SA operation mode, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB-based measurements, inter-frequency SSB-based measurements performed outside measurements gaps, E-UTRA inter-RAT measurement object without measurement gap, intra-frequency CSI-RS L3 measurement and RSSI/channel occupancy measurement with no measurement gap on a carrier subject to CCA when SMTC and RMTC are overlapping will be as specified in table 9.1.5.1.2-1, which shall also be applied for a UE configured with NE-DC operation.

Table 9.1.5.1.2-1: CSSFoutside_gap,i scaling factor for SA mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSF outside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap | CSSFoutside_gap,i for E-UTRA inter-RAT MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 only CA | 1+NPCC_CSIRS + NPCC_CCA_RSSI/CO | NSCC_SSB +Y+Z+2x NSCC_CSIRS+ NSCC_CCA_RSSI/CO | N/A | N/A | N/A | NSCC_SSB +Y+Z+2x NSCC_CSIRS+ NSCC_CCA_RSSI/CO | NSCC_SSB +Y+Z+2x NSCC_CSIRS |
| FR2 only intra band CA | N/A | N/A | 1+NPCC_CSIRS | N/A | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS |
| FR2 only inter band CA | N/A | N/A | 1+NPCC_CSIRS | 2*(1+ NSCC_CSIRS_FR2_NCM) Note 3,5 | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) |
| FR1 +FR2 CA (FR1 PCell) Note 1 | 1+NPCC_CSIRS | 2×( NSCC_SSB +Y+Z+2* NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | N/A | 2x(1+ NSCC_CSIRS_FR2_NCM) Note 3,5 | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) |
| FR1 +FR2 CA (FR2 PCell) Note 1 | N/A | NSCC_SSB +Y+Z+2x NSCC_CSIRS | 1+NPCC_CSIRS | N/A | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS |
| NOTE 1: Only one FR1 operating band and one FR2 operating band are included for FR1+FR2 inter-band CA. NOTE 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2. NOTE 3: CSSFoutside_gap,i =1 if only one SCell is configured and no inter-frequency MO without gap and only SSB based L3 measurement is configured on SCC; CSSFoutside_gap,i =2 if only one SCell is configured and no inter-frequency MO without gap and either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement is configured on SCC. NOTE 4: Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG; otherwise, it is 0. NOTE 5: Only two NR FR2 operating bands are included for FR2 inter-band CA. NOTE 6: NPCC_CSIRS=1 if PCC is with either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement configured; otherwise, NPCC_CSIRS =0. NOTE 7: NSCC_CSIRS=Number of configured SCell(s) with either both SSB and CSI-RS based L3 measurement configured or only CSI-RS based L3 measurement configured NOTE 8: NSCC_CSIRS_FR2_NCM=1 if FR2 SCC, where neighbour cell measurement is required, is with either both SSB and CSI-RS configured or only CSI-RS measurement configured; otherwise, NSCC_CSIRS_FR2_NCM=0. NOTE 9: NSCC_SSB=Number of configured SCell(s) with only SSB based L3 measurement configured, which is measured without MG. NOTE 10: NPCC_CCA_RSSI/CO= 1 if PSCC is configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping; NSCC_CCA_RSSI/CO = Number of MOs for SCell(s) configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping. NOTE 11: Z is the number of configured E-UTRA inter-RAT MOs without MG that are being measured outside of MG; otherwise, it is 0. NOTE 12: For UE supporting [CSSF enhancement for one CC measurement per-band] for intra-frequency measurements without MG, if network indicates to enable one serving carrier measurement for each FR2 band via [TBD], the being-measured FR2 CC(s) counted in CSSF shall follow the principles specified in clause 9.2.3.2. |  |  |  |  |  |  |  |

##### 9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps

For UE configured with NR-DC operation, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB-based measurement, inter-frequency SSB-based measurements performed outside measurements gaps and intra-frequency CSI-RS based L3 measurement will be as specified in table 9.1.5.1.3-1.

Table 9.1.5.1.3-1: CSSFoutside_gap,i scaling factor for NR-DC mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR1 PSCC | CSSFoutside_gap,i for FR2 PSCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 + FR2 NR-DC (FR1 PCell and FR2 PSCell) Note 1 | 1+NPCC_CSIRS | 2×( NSCC_SSB +Y+2xNSCC_CSIRS) | N/A | 2x(1+ NPSCC_CSIRS) Note 2 | 2x(NSCC_SSB +Y+2x NSCC_CSIRS ) | 2x(NSCC_SSB +Y+2x NSCC_CSIRS ) |
| FR1 + FR1 NR-DC (FR1 pCell and FR1 PSCell) | 1+NPCC_CSIRS | 2×( NSCC_SSB +Y+2xNSCC_CSIRS) | 2x(1+ NPSCC_CSIRS) Note 2 | N/A | N/A | 2x(NSCC_SSB +Y+2x NSCC_CSIRS ) |
| NOTE 1: NR-DC in Rel-15 only includes the scenarios where all serving cells in MCG in FR1 and all serving cells in SCG in FR2. NOTE 2: CSSFoutside_gap,i =1 if no SCell is configured and no inter-frequency MO without gap and only SSB based L3 measurement is configured on PSCC; CSSFoutside_gap,i =2 if no SCell is configured and no inter-frequency MO without gap and either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement is configured on PSCC. NOTE 3: Y is the number of configured inter-frequency SSB based frequency layers without MG that are being measured outside of MG; otherwise, it is 0. NOTE 4: NPCC_CSIRS=1 if PCC is with either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement configured; otherwise, NPCC_CSIRS =0. NOTE 5: NPSCC_CSIRS=1 if PSCC is with either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement configured; otherwise, NPSCC_CSIRS =0. NOTE 6: NSCC_CSIRS=Number of configured SCell(s) with either both SSB and CSI-RS based L3 measurement configured or only CSI-RS based L3 measurement configured NOTE 8: NSCC_SSB=Number of configured SCell(s) with only SSB based L3 measurement configured, which is measured without MG. NOTE 9: For UE supporting [CSSF enhancement for one CC measurement per-band] for intra-frequency measurements without MG, if network indicates to enable one serving carrier measurement for each FR2 band via [TBD], the being-measured FR2 CC(s) counted in CSSF shall follow the principles specified in clause 9.2.3.2. |  |  |  |  |  |  |

##### 9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps

For UE configured with NE-DC operation, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB-based measurement and inter-frequency SSB-based measurements performed outside measurements gaps and intra-frequency CSI-RS based L3 measurement will be as specified in table 9.1.5.1.4-1.

Table 9.1.5.1.4-1: CSSFoutside_gap,i scaling factor for NE-DC mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- |
| NE-DC with FR1 only CA | 1+NPCC_CSIRS | NSCC_SSB +Y+2x NSCC_CSIRS | N/A | N/A | N/A | NSCC_SSB +Y+2x NSCC_CSIRS |
| NE-DC with FR2 only intra band CA | N/A | N/A | 1+NPCC_CSIRS | N/A | NSCC_SSB +Y+2x NSCC_CSIRS | NSCC_SSB +Y+2x NSCC_CSIRS |
| NE-DC with FR2 only inter band CA | N/A | N/A | 1+NPCC_CSIRS | 2*(1+ NSCC_CSIRS_FR2_NCM) Note 3,5 | 2×( NSCC_SSB +Y+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) |
| NE-DC with FR1 +FR2 CA (FR1 PCell) Note 1 | 1+NPCC_CSIRS | 2×( NSCC_SSB +Y+2* NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | N/A | 2x(1+ NSCC_CSIRS_FR2_NCM) Note 3,5 | 2×( NSCC_SSB +Y+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) |
| NOTE 1: Only one FR1 operating band and one FR2 operating band are included for FR1+FR2 inter-band CA. NOTE 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2. NOTE 3: CSSFoutside_gap,i =1 if only one SCell is configured and no inter-frequency MO without gap and only SSB based L3 measurement is configured on SCC; CSSFoutside_gap,i =2 if only one SCell is configured and no inter-frequency MO without gap and either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement is configured on SCC. NOTE 4: Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG; otherwise, it is 0. NOTE 5: Only two NR FR2 operating band are included for NE-DC with FR2 only inter-band CA. NOTE 6: NPCC_CSIRS=1 if PCC is with either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement configured; otherwise, NPCC_CSIRS =0. NOTE 7: NSCC_CSIRS=Number of configured SCell(s) with either both SSB and CSI-RS based L3 measurement configured or only CSI-RS based L3 measurement configured NOTE 8: NSCC_CSIRS_FR2_NCM=1 if FR2 SCC, where neighbour cell measurement is required, is with either both SSB and CSI-RS configured or only CSI-RS measurement configured; otherwise, NSCC_CSIRS_FR2_NCM=0. NOTE 9: NSCC_SSB=Number of configured SCell(s) with only SSB based L3 measurement configured, which is measured without MG. NOTE 10: For UE supporting [CSSF enhancement for one CC measurement per-band] for intra-frequency measurements without MG, if network indicates to enable one serving carrier measurement for each FR2 band via [TBD], the being-measured FR2 CC(s) counted in CSSF shall follow the principles specified in clause 9.2.3.2. |  |  |  |  |  |  |

# <End of Change 2>