# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503532

# Wuhan, China, 7th – 11th April, 2025

Agenda item:	7.7.3

Source:	Apple

Title:	On RRM requirement for R19 LB CA

Document for:	Discussion

# Introduction

The WF on RRM requirements for NR_LBCA_Sw [1] have been agreed in RAN4#114, and the open issues have been summarized in the topic summary for [114][213] NR_LBCA_Sw [2].

In this contribution, we continue the discussion for RRM requirement of LB CA via switching.

# RRM requirements discussion

As agreed in the WID, there are two cases to consider:

-	Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

-	Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1

The example band combination for the analysis is CA_n5A-n29A in this WI, and the case switching can be illustrated in the following figure.

Figure 1. n5A-n29A case switching example

The key point for this LB CA with SDL carrier is that UE cannot support simultaneous reception on PCC and SDL CC, i.e., in the above example, when UE switches to n29 SDL CC, UE is not able to keep connection with n5 PCC.

# Assumptions for R19 LB CA via switching

In last meeting, some agreements of assumptions for R19 LB CA via switching were made in the WF[1], as followings:

- (1) FFS: For a UE supporting low band CA via switching, SDL SCell operation is SSB based? or SSB-less based? or both? Agreement: - For a UE supporting low band CA via switching, RAN4 to discuss requirement for SDL SCell operation for both SSB-based SCell and SSB-less SCell based, and decide whether to define requirements for both cases or start with SSB-based in the next meeting. - (2) FFS: MRTD between the two low bands is CP? Or wait for TAE conclusion from RF session? Agreement: As generic assumption for discussing the RRM requirement, the received timing difference between the two bands at UE is within a value in option 1 or option 2, unless other TAE requirement will be defined in RF session. - Option 1: 4.67us - Option 2: 3 us

Even though in last meeting RAN4 agreed that for a UE supporting low band CA via switching RAN4 to discuss requirement for SDL SCell operation for both SSB-based SCell and SSB-less SCell based, we have concern on the timeline of completing the WI. Almost all of the UE measurement behaviors are dependent on whether UE will perform SSB measurement on SCell or not; but the switching pattern will be discussed in RAN1 from April meeting, that means, RAN4 can only know if the SSB measurement can be impacted due to switching pattern at least from May meeting and need to conclude in August meeting. That is very challenging in our view.

Technically, the WID clarified the scenario for this LB CA, as following. Considering the synchronization and collocated condition between FDD PCC and SDL SCC, we assume the coarse timing between these two CCs can be shared within TAE.

- Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers

Additionally, CA_n5A-n29A is considered in this WI, and these two band are very close to each other on frequency domain, which means the channel conditions are also similar.

| n5 | 824 MHz – 849 MHz | 869 MHz – 894 MHz | FDD |
| --- | --- | --- | --- |
| n29 | N/A | 717 MHz – 728 MHz | SDL |

By considering the workload for WI and channel similarity between LB CCs in this WI, we prefer to use SSB-less SCell based operation as starting point.

Proposal 1: For a UE supporting low band CA via switching, RAN4 to discuss requirement for SDL SCell operation by using SSB-less SCell based operation as starting point.

On the other hand, since the main motivation for network to switch UE from PCC to SDL SCC is for data scheduling and UE is only required to received PDCCH/PDSCH on an activated SCell, we think it would be straightforward to apply such switching pattern only for activated SDL SCC. That means, the deactivated SCell operation shall not be restricted by the switching pattern.

Proposal 2: the switching pattern is applicable only when target SDL SCell has been activated.

# Interruption requirement

In last meeting, some of the scopes have been discussed and followings are the further analysis for RAN4#114bis:

WF in RAN4#114 - Interruption requirement - Alt 1: Needs interruption requirement for carrier switching between FDD CC and SDL CC (Apple, Xiaomi, CATT, HW, Samsung) - Capabilities for UEs with and without interruption (Samsung) - Alt2: RRM to reuse the RF defined switch delay and time mask. RAN4 RRM not to define additional interruption or scheduling restriction in RRM - Session Chair (RRM session): check the progress in RF session, and discuss if any aspects are not covered by the RF discussion.

In RAN4#114 RF session, the agreements were captured in WF[3] as following,

Define switching periods between FDD low band and SDL low band with the following values as a UE capability to accommodate different UE implementations - 35us, 140us - RAN4 to further discuss 13us - LS will capture the currently confirmed values - It is not precluded to send another LS with more values in future meetings

Like the other features in previous release, e.g., SRS carrier switching, SRS antenna port switching, and UL carrier switching, even though the switching period was defined in RF spec, RRM still need to have the interruption requirement (e.g., DL interruption due to UL carrier switching). Since this WI is for LB CA via switching, that means the UE capability in this WI is when UE cannot support simultaneous reception for both FDD PCC and SDL SCC as in figure 1, and therefore our understanding is the interruption is always applied for the UEs in this WI.

Proposal 3: RAN4 to define interruption requirement for carrier switching between FDD CC and SDL CC.

As shown in the following figure 2, even though the LB CA in this WI is co-located and synchronized network deployment for both DL carriers, the TA between DL and UL can cause the switching gap colliding with two slots on time domain.

Figure 2. carrier switching interruption example for LB CA

The RF requirement for the total switching time including switching and transition is 35us or 140us up to UE capability and 13us option is still under discussing. Thus, in RRM we propose to have slot level interruption requirements, and in order to make the requirement forward compatible to the scenario with additional active DL non-SDL SCC, the interruption requirement shall be applied as generally as possible to both UL and DL CCs.

As illustrated in the figure 2, the slot level interruption requirement due to carrier switching for R19 LB CA can be defined as 	TswitchingSlot-length-victim+1, where Tswitching is the switching time concluded in RF session. In this WI, if only 15kHz SCS is considered and RF agreed that the combination of those two low bands with other band can be further discussed in the future release, the slot-length-victim can only be 2 slots in total. Different from UL carrier switching, we don’t have in-slot switching demand for this LB CA case and interruption is counted in terms of ACK/NACK number (e.g., one ACK/NACK per TTI in the test case), and therefore the slot level requirement is sufficient in this case.

Proposal 4: RAN4 to define slot level interruption requirement for carrier switching in R19 LB CA.

Proposal 5: The interruption requirement for carrier switching in R19 LB CA is TswitchingSlot-length-victim+1, where Tswitching is the switching time concluded in RF session.

# SDL SCC related RRM requirement

WF in RAN4#114 - SDL SCC related RRM requirement - activated SDL SCell measurement Agreement: For R19 LB CA via switching, RAN4 to not define SDL activated SCell L3 measurement requirement if SSB-less based SCell operation is used. For R19 LB CA via switching, FFS: - Whether and how RAN4 to define SDL activated SCell L3 measurement requirement if SSB based SCell operation is used. - de-activated SDL SCell measurement Agreement: For R19 LB CA via switching, RAN4 to not define SDL deactivated SCell L3 measurement requirement if SSB-less based SCell operation is used. For R19 LB CA via switching, FFS: - Whether and how RAN4 to define SDL deactivated SCell L3 measurement requirement if SSB based SCell operation is used. - other SDL SCell requirements Agreement: - FFS: Whether and how - RAN4 to define a new SDL SCell activation/deactivation delay and related interruption for R19 LB CA - the existing SCell activation/deactivation delay and related interruption requirement can be used as baseline for R19 LB CA - FFS: Whether and how - RAN4 to define a new BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirements for R19 LB CA - the existing BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirement can be used as baseline for R19 LB CA Agreement: - Not to change definition of intra-frequency/inter-frequency measurement with/without gap measurement - FFS on the use case for MG application.

      - Activated SDL SCell measurement

If SSB-based SCell operation is assumed for SDL SCell measurement, UE needs to measure SSB for L3 on SDL SCell based on SMTC configuration, and the current requirement is defined as:

The current measurement interval is specified based on SMTC periodicity or max(SMTC periodicity, DRX cycle); however, if some of the SMTC occasions are outside the SDL SCC reception duration of switching pattern, UE can only perform the SDL SCell measurement on the overlapped occasions between SMTC window and SDL SCell reception duration indicated by switching pattern. Thus, the legacy measurement interruption shall be revised accordingly.

Additionally, the switching pattern and SMCT configuration are both controlled by network, so we assume network will configure the switching pattern reasonably to make sure UE can at least have one available measurement opportunity in each SDL SCell reception duration.

Proposal 6: If SSB-based SCell operation is assumed for activated SDL SCell measurement, UE can only perform the SDL SCell measurement on the overlapped occasions of SMTC window and SDL SCell reception duration indicated by switching pattern.

Depending on the pattern design from RAN1, if the SDL SCell reception duration can be longer than SMTC periodicity, RAN4 also needs to discuss whether UE shall follow the SMTC periodicity to do the SCell  measurement inside SDL SCell reception duration, or UE only needs to measure SCell one time per periodicity of switching pattern.

      - De-activated SDL SCell measurement

The current deactivated SCell measurement consider the interruption for RF tuning/retuning was specified below,

If measCycleSCell is smaller than 640ms, currently UE is required to stay on deactivated SCC after SCell addition(no interruption is allowed). However, in R19 new LB CA scenario, since UE cannot keep the connection with PCell when it stays on the SDL SCC for measurement, it would be necessary to allow UE switch back to PCC for traffic/control reception/transmission after deactivated SCell measurement regardless of the measCycleSCell.

Proposal 7: If SSB-based SCell operation is assumed for SDL SCell measurement, for deactivated SDL SCell measurement, interruptions on PCell or activated SCell(s) are allowed with up to 0.5% probability of missed ACK/NACK regardless of the configured measCycleSCell.

In current requirement, the deactivated SCell measurement follows the measCycleSCell as shown in the following table,

As in proposal 2, we think the switching pattern shall be applied only when the target SDL SCell is activated, and therefore the deactivated SCell measurement requirement shall not be impacted by the switching pattern. The existing deactivated SCell measurement requirement can be reused for deactivated SDL SCell in LB CA via switching.

Proposal 8: The existing deactivated SCell measurement requirement can be reused for deactivated SDL SCell in LB CA via switching.

      - SDL SCell activation

Regarding SDL SCell activation, there are also some issues to discuss, e.g.,

- Legacy SCell activation delay is defined based on SMTC and SSB periodicity, but will such periodicity be replaced by carrier switch pattern periodicity?

- Legacy interruption for SCell activation is defined in an interruption window, like following. But do we allow UE to stay on the SDL SCell during activation period? If yes, then UE will cause interruption to PCell as the whole activation time.

- Whether or not we can reuse SSB-less SCell activation requirement for SDL SCell activation, based on the condition below.

  - Co-located and synchronized network deployment for both carriers

  - Both carriers are in a single TAG

If the legacy SSB based inter-band SCell activation procedure is assumed for LB CA, then SCell activation delay may be defined based on switching pattern or SSB periodicity (depending on proposal 2 is agreeable in this meeting or not), or if UE is allowed to stay on the SDL SCell during activation time, the interruption can happen after n+1+THARQ. However, it’s unclear how UE can handle the CQI reporting during the SCell activation procedure, and therefore the details of interruption requirement shall be redesigned for LB CA.

If the SSB-less SCell activation procedure is assumed for LB CA, we may consider using R18 inter-band FR1 SSB-less SCell activation as the baseline and corresponding side conditions for RTD and power imbalance shall also be specified. In Rel-18 TRS or A-TRS is used for SSB-less SCell activation for AGC fine tuning and T/F tracking. Since the TRS/A-TRS based SCell activation will have much shorter activation delay than the SSB based SCell activation, UE can stay on the SDL SCC to complete the SCell activation and then switch back to the PCC based on the switching pattern. Again, here we assumed that switching pattern shall be applied only when the target SDL SCell is activated, and it’s unclear how UE can handle the CQI reporting during the SCell activation procedure.

We think the SDL SCell activation delay requirement can be reused since in our view UE doesn’t need to limit the SDL SCell activation procedure by the switching pattern before the SDL SCell becomes activated.

Proposal 9: the SCell activation delay requirement can be reused to SDL SCell activation in LB CA via switching.

Proposal 10: the interruption requirement in SCell activation delay shall be re-designed. Details can be FFS.

      - other SDL SCell requirements

Since SDL SCell has no uplink carrier and the SDL SCell is synchronized/collocated with FDD PCell, we don’t think it’s really necessary to require UE to perform BFD/CBD/PL-RS related measurement for SDL SCell. Similarly, for SCell SDL TCI switching, the typical scenario is to use SSB on PCell as the QCL source for TCI on channel of SDL SCell because of synchronized/collocated deployment, and then the existing TCI switching requirement can be used for LB CA via switching.

Proposal 11: no requirement is specified for BFD, CBD, PL-RS switch delay for SDL SCell in R19 LB CA via switching.

Proposal 12: existing TCI switching requirement can be used for SDL SCell in R19 LB CA via switching, provided that PCell SSB is configured as the QCL source of SDL SCell channel(s); otherwise no requirement applies.

For L1-RSRP/SINR measurement requirement for SDL SCell, similar as the SCell L3 measurement, UE can only perform the L1-RSRP/SINR measurement of SDL SCell on the overlapped occasions between L1 RS and SDL SCell reception duration indicated by switching pattern.

Proposal 13: For L1-RSRP/SINR measurement requirement for SDL SCell, UE can only perform such measurement on the overlapped occasions of L1 RS and SDL SCell reception duration indicated by switching pattern.

      - MG application

Regarding the MG application, we identified one possible issue for SDL SCell measurement with MG. The existing CSSF for measurement inside MG is as following,

The intra-frequency MO without MG shall be counted in the CSSF inside MG only when it meet above highlighted conditions, however, the existing fully overlapped or partially overlapped conditions may not be accurate when LB CA via switching is used. Like in the figure 3, the MO on PCC is intra-frequency MO without MG, and the SMTCs on the PCC is not fully overlapped with MG for inter-frequency measurement, i.e., it’s partially overlapped case if only considering the SMTC configuration and MG configurations. However, effectively, UE cannot perform measurement in the SMTC of PCC when it’s colliding with the SDL SCC staying duration (in which UE switched to stay on SDL SCC and cannot receive anything from PCC). In this case, PCC SMTC and MG are fully overlapped effectively; and SDL SCC SMTC and MG are fully non-overlapped effectively

Figure 3. fully overlapped and partially overlapped conditions for CSSF in LB CA via switching

Thus, in LB CA via switching, the layers or MOs counted in the CSSF inside MG shall be based on the effective SMTCs of that layer or MO. The effective SMTC means:

(1) the SMTCs of PCC outside the duration time of staying on SDL SCC are treated as effective SMTCs of PCC; and

(2) the SMTCs of SDL SCC inside the duration time of staying on SDL SCC are treated as effective SMTCs of SCC

Figure 4. effective SMTCs to decide overlapping condition for CSSF inside MG

In summary, the overlapping condition (i.e., partially overlapping or fully overlapping) to decide the CSSF inside MG shall be revised for LB CA via switching, by considering configured SMTCs, MGs and LB-CA switching pattern.

Proposal 14: the overlapping condition (i.e., partially overlapping or fully overlapping) to decide the CSSF inside MG shall be revised for LB CA via switching, by considering configured SMTCs, MGs and LB-CA switching pattern.

# Potential impacts to the PCell related requirement

Since carrier switching pattern will be introduced for SDL SCC in R19 LB CA, it may also impact the PCell related RRM requirement, e.g., PCell measurement and time tracking. Currently PCell L3 and L1 measurement follows SMTC periodicity and RS periodicity respectively, like following tables. However, the DL switching pattern may have larger periodicity and the SMTC or SSB periodicity, as shown in figure 3.

Figure 5. SMTC periodicity vs. switching pattern periodicity

Thus, the carrier switching pattern shall also be considered to decide the PCell L3 or L1 measurement. In our view, the more reasonable design is to make sure the switching pattern can guarantee UE has at least one RLM/BFD/L3 occasion for evaluation/measurement in every max(DRX cycle, RS periodicity (L1) or SMTC periodicity (L3)).

Regarding the UE timing, the current required UE to meet Te requirement only if at least one SSB is available in the last 160ms before UL transmission, as following. In our view, the design of switching pattern shall also make sure UE to not stay on SDL SCell for longer than 160ms, i.e., UE will have SSB reception available in last 160ms before the UL transmission on PCell. Or alternatively, since PCell and SDL SCell belongs to the same TAG and these cells are collocated and synchronized, we may allow reference cell switching between PCell and SDL SCell in same TAG for R19 DL CA, i.e., UE tracks timing on SDL SCell when it’s on SDL SCell following the switching pattern, and then such timing information can be used for UL transmission on PCell. These issues are related with the switching pattern design, and therefore we propose to consider them later.

In last meeting, RAN4 sent one LS [4] to RAN1 to emphasize the importance of UL timing on PCell, but RAN4 has to wait the pattern design from RAN1 to decide what the impact is to PCell RRM.

Proposal 15: RAN4 to discuss potential impacts to the PCell related RRM requirement once the switching pattern is concluded in RAN1.

We understand that the switching pattern is to be designed in RAN1. To not impose too much restriction on the pattern design, there could be patterns where the UE is configured to stay on SDL SCell for longer than 160ms or the normal SSB reception on PCC is constantly disrupted by UE dwelling on SDL SCell. In this case, it is possible for the network to transmit on-demand SSB or A-TRS to help UE gain timing info on PCell. Of course, the required number of on-demand SSBs or bursts of A-TRS may depend on UE capability. As the network knows the switching pattern and the dwelling time on PCell or SCell, the network can configure the on-demand SSB or A-TRS to avoid collision. Details can be further discussed if it turns out that on-demand SSB or A-TRS is needed for PCell timing acquisition given a particular switching pattern.

# Conclusion

In this contribution, we discuss the scope of corresponding RRM requirement for this R19 LB CA scenario.

Proposal 1: For a UE supporting low band CA via switching, RAN4 to discuss requirement for SDL SCell operation by using SSB-less SCell based operation as starting point.

Proposal 2: the switching pattern is applicable only when target SDL SCell has been activated.

Proposal 3: RAN4 to define interruption requirement for carrier switching between FDD CC and SDL CC.

Proposal 4: RAN4 to define slot level interruption requirement for carrier switching in R19 LB CA.

Proposal 5: The interruption requirement for carrier switching in R19 LB CA is TswitchingSlot-length-victim+1, where Tswitching is the switching time concluded in RF session.

Proposal 6: If SSB-based SCell operation is assumed for activated SDL SCell measurement, UE can only perform the SDL SCell measurement on the overlapped occasions of SMTC window and SDL SCell reception duration indicated by switching pattern.

Proposal 7: If SSB-based SCell operation is assumed for SDL SCell measurement, for deactivated SDL SCell measurement, interruptions on PCell or activated SCell(s) are allowed with up to 0.5% probability of missed ACK/NACK regardless of the configured measCycleSCell.

Proposal 8: The existing deactivated SCell measurement requirement can be reused for deactivated SDL SCell in LB CA via switching.

Proposal 9: the SCell activation delay requirement can be reused to SDL SCell activation in LB CA via switching.

Proposal 10: the interruption requirement in SCell activation delay shall be re-designed. Details can be FFS.

Proposal 11: no requirement is specified for BFD, CBD, PL-RS switch delay for SDL SCell in R19 LB CA via switching.

Proposal 12: existing TCI switching requirement can be used for SDL SCell in R19 LB CA via switching, provided that PCell SSB is configured as the QCL source of SDL SCell channel(s); otherwise no requirement applies.

Proposal 13: For L1-RSRP/SINR measurement requirement for SDL SCell, UE can only perform such measurement on the overlapped occasions of L1 RS and SDL SCell reception duration indicated by switching pattern.

Proposal 14: the overlapping condition (i.e., partially overlapping or fully overlapping) to decide the CSSF inside MG shall be revised for LB CA via switching, by considering configured SMTCs, MGs and LB-CA switching pattern.

Proposal 15: RAN4 to discuss potential impacts to the PCell related RRM requirement once the switching pattern is concluded in RAN1.

# References

[1] R4-2502588, WF on RRM requirements for NR_LBCA_Sw, Apple, RAN4 #114

[2] R4-2500527, Topic summary for [114][213] NR_LBCA_Sw, Apple, RAN4 #114

[3] R4-2503016, WF on low-low band switching, Apple, RAN4 #114

[4] R4-2502697, LS of RAN4 agreements on LB CA via switching, Apple, RAN4#114