# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503535

# Wuhan, China, 7th – 11th April, 2025

Agenda item:	7.29.4.1

Source:	Apple

Title:	On R19 NTN (e)RedCap RRM requirements

Document for:	Discussion

# Introduction

The WF on RRM requirements for Rel-19 NR NTN phase3 [1] has been agreed in RAN4#114, and the open issues have been summarized in the topic summary for [114][229] NR_NTN_Ph3_Part1 [2].

In this contribution, we continue the discussion on the RRM requirement for R19 NTN (e)RedCap.

# RRM for support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN

Issue 1-2-3: SMTCs and 'in parallel' related

##### Issue 1-2-3-1: Feature 25-1(parallelSMTC) Online discussion <Way Forward>: Options for further discussion: If not differentiate 1Rx and 2Rx RedCap UEs, the baseline for UE capability is: - Option 1: [1 SMTC or] 2 SMTCs not in parallel - Option 2: 2 parallel SMTCs - ‘Parallel’ means the SMTCs are TDM with proximity distance.

In legacy R17 UE FG definition, the parallelSMTC is defined as,

| Features | Index | Feature group | Components | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 25. NR_NTN_solutions | 25-1 | Parallel measurements on multiple SMTC-s for a single frequency carrier | Support of measurements on target cells belonging to 4 SMTC-s on a single frequency carrier | UE is mandatory to support 2 and can optionally support 4 if the feature is supported | Optional with capability signaling |

And the TS38.306 has defined that NTN UE shall at least support NTN SSB based RRM measurements on target cells belonging to 2 SMTC-s on a single frequency carrier, as following.

In our view, the limitation on RedCap UE is only about the processing capability (e.g., as defined in proximity distance) rather than whether 2 SMTCs baseline can be supported or not. On the other hand, in order to support LEO deployment, 2 SMTC configuration will be a very practical case. If option 1 is used but network is not aware of when UE is performing measurement on SMTC1 or SMTC2, it might cause unnecessary scheduling restriction for certain cases (e.g., UE does not support parallelMeasurementWithoutRestriction-r17). Thus, we prefer option 2 to have same definition of the UE capability between non-RedCap UE and (e)RedCap UE in NTN.

Proposal 1: for Feature 25-1(parallelSMTC), if not differentiate 1Rx and 2Rx RedCap UEs, the baseline for UE capability is to measure 2 parallel SMTCs in a same SMTC periodicity. ‘Parallel’ means the SMTCs are TDM with proximity distance.

Issue 1-2-3-1a: The proximity distance between two SMTCs

With the multiple SMTC configured on the same carrier, in legacy NTN requirement, RAN4 defined the proximity for the SMTCs as following,

9.1C.9.3 Collision between multiple SMTCs on a SAN carrier Collision between occasions of multiple SMTCs may occur as specified in this clause. When UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met: - the two SMTC occasions are fully or partially overlapping in time domain, or - the magnitude of the distance between two SMTC occasions in time domain is less than or equals to 4 ms. The distance between two SMTC occasions is defined as the time difference between the ending point of the earlier SMTC occasion and the starting point of the later SMTC occasion.

The proximity distance between the SMTCs reflects the UE capability of how fast it can complete the post-processing for the data buffered from prior SMTC. If network configures 2 SMTCs on a NTN carriers and the proximity distance between these two SMTCs are less than 4ms, UE is required to measurement only one of them in legacy NTN. Now, considering the RedCap UE has reduced UE processing capability than legacy non-RedCap UE, the proximity distance might need to be revisited in the spec. We think the relaxation for proximity could be considered for (e)RedCap UE in NTN, e.g.,

    - When (e)RedCap UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met:

      - the two SMTC occasions are fully or partially overlapping in time domain, or

      - the magnitude of the distance between two SMTC occasions in time domain is less than or equals to (4+x) ms. (x is the relaxation for (eRedCap UE)). We propose x=1ms.

    - The distance between two SMTC occasions is defined as the time difference between the ending point of the earlier SMTC occasion and the starting point of the later SMTC occasion.

Proposal 2: to consider relaxation for proximity when (e)RedCap UE is configured with more than one SMTC on a SAN carrier, i.e.,

    - When (e)RedCap UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met:

      - the two SMTC occasions are fully or partially overlapping in time domain, or

      - the magnitude of the distance between two SMTC occasions in time domain is less than or equals to (4+x) ms. (x is the relaxation for (eRedCap UE)), x=1ms.

    - The distance between two SMTC occasions is defined as the time difference between the ending point of the earlier SMTC occasion and the starting point of the later SMTC occasion.

Issue 1-2-3-5: Feature 25-3(parallelMeasurementGap)

<Way Forward>: - For whether to define RRM requirements for Feature 25-3(parallelMeasurementGap-r17) for (e)RedCap UEs, - Proposal 1: Support to define. - Proposal 1a: The UE capability is applicable, and the support of the optional features is left to UE implementation following the same principle of the existing non-RedCap requirements. - Proposal 1b: The capability can be optionally supported by (e)RedCap UEs with 2Rx, can not be supported by (e)RedCap UEs with 1Rx.

In our view, as long as RAN4 agrees to use this parallelMeasurementGap for (e)RedCap UE, we don’t think it’s necessary to differentiate this capability applicability for 2Rx or 1Rx cases. Rx number is related with the different antenna implementation for (e)RedCap UE and from RRM perspective it would impact on the measurement accuracy and delay, however, it shall not be a justification for UE to differentiate capability of parallelMeasurementGap. parallelMeasurementGap does not mean UE needs to measure both MGs simultaneously; but it only indicates that UE can support such configuration with 2MGs inside one MGRP. Thus, we propose to adopt proposal 1a.

Proposal 3: RAN4 to define RRM requirements for Feature 25-3(parallelMeasurementGap-r17) for (e)RedCap UEs, and the support of the optional features is left to UE implementation following the same principle of the existing non-RedCap. This capability supported by (e)RedCap UEs is not differentiated between 2Rx and 1Rx.

# Conclusion

In this contribution, we discuss the RRM requirement scope for WI of Non-Terrestrial Networks (NTN) for NR Phase 3.

Proposal 1: for Feature 25-1(parallelSMTC), if not differentiate 1Rx and 2Rx RedCap UEs, the baseline for UE capability is to measure 2 parallel SMTCs in a same SMTC periodicity. ‘Parallel’ means the SMTCs are TDM with proximity distance.

Proposal 2: to consider relaxation for proximity when (e)RedCap UE is configured with more than one SMTC on a SAN carrier, i.e.,

    - When (e)RedCap UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met:

      - the two SMTC occasions are fully or partially overlapping in time domain, or

      - the magnitude of the distance between two SMTC occasions in time domain is less than or equals to (4+x) ms. (x is the relaxation for (eRedCap UE)), x=1ms.

    - The distance between two SMTC occasions is defined as the time difference between the ending point of the earlier SMTC occasion and the starting point of the later SMTC occasion.

Proposal 3: RAN4 to define RRM requirements for Feature 25-3(parallelMeasurementGap-r17) for (e)RedCap UEs, and the support of the optional features is left to UE implementation following the same principle of the existing non-RedCap. This capability supported by (e)RedCap UEs is not differentiated between 2Rx and 1Rx.

# References

[1] R4-2502604, WF on RRM requirements for NR_NTN_Ph3_Part1, CATT, RAN4#114

[2] R4-2500543, Topic summary for [114][229] NR_NTN_Ph3_Part1, CATT, RAN4#114