# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503540

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.16.2

Source:	Apple

Title:	Discussion on Spatial Channel Modeling

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

In RAN4#114 spatial channel model for demodulation requirements was discussed and way forward [1] was approved. In this contribution we share our views on the study for spatial channel models for demodulation and CSI reporting requirements in RAN4.

# 2. Discussion

## Common for all methodologies

In RAN4#114 the following agreements were made:

CSI-RS Periodicity Agreement: CSI-RS is transmitted with a periodicity of 5ms in the first slot of the TDD pattern. PDSCH is not transmitted in the slots carrying CSI-RS nor S slot. CQI/PMI delay of 7ms. Regarding EPRE Ratio: Agreement: Average EPRE of PDSCH data is equal to the average EPRE of CSI-RS, no power boosting is allowed for CSI-RS with respect to the number of CDM-groups in an OFDM symbol. Receiver Type for SU-MIMO Agreement : A linear MMSE-IRC receiver to be used for the SCM study Receiver Type for 8 Layer 8Rx Agreement: For 8Rx, “Baseline” 8Rx Receiver to be assumed Cases for MU-MIMO Agreement: SU-MIMO to take priority over MU-MIMO. Simulation alignment spreadsheet will only include agreed alignment cases.

PDSCH PMI Selection

Way Forward:

Random PMI already agreed. Regarding potential fixed PMI:

- Option 1: Random PMI from sub-set

- Option 2: Collectively chosen fixed PMI

- Option 3: Independently chosen fixed PMI

Note: Companies to propose detailed method description for interested options for RAN4#114-bis.

PMI for alignment of SCM

Way Forward:

First check alignment with PMI Random (type I)

On PDSCH PMI selection, we need to clarify if fixed PMI is for both 4RX and 8RX cases. Based on the discussion in the last meeting, the option for potential fixed PMI was entertained for 8RX 8-layer case, where random PMI could limit the maximum achievable TP. The exercise of fixing PMI is very cumbersome and requires a 2 step-process. We think it is okay to live with the limitation that maximum TP might not be achieved with random PMI with 8RX 8-layers. We aren’t defining requirements here and this is a study.

- It is not clear if fixed PMI option is for both 4RX and 8RX cases.

- With random PMI the max TP might not be achieved with 8RX-8layers

- It is very cumbersome to fix PMI and requires a 2-step process

- For the study, it should be okay if max TP is not achieved with random PMI since requirements are not being defined here.

Only evaluate with random PMI for PDSCH test cases with 4RX and 8RX.

Speed/Doppler

Way Forward:

For Comparison:

TDL Channels use 10 and 100Hz

CDL Channels use 3kph and 30kph

For Alignment:

TDL Channels use 10Hz

CDL Channels use 3kph

We propose that for alignment we use low speed/Doppler for PMI test cases and high speed/Doppler for PDSCH test cases.

For alignment use -
Low speed/Doppler for PMI test cases – 3kph for CDL/10Hz for TDL
High speed/Doppler for PDSCH test cases – 30kph for CDL/100Hz for TDL

## CDL based methodologies

For CDL based methodologies the following agreements were made in RAN4#113:

SNR Definition – ‘long term average’ Agreement: Normalization factor is constant over the duration of the simulation

#### Angles of Departure and Angles of Arrival

The angles are scaled based on the desired angle spread in the tables below and the CDL tables for options A1, A2, A3 are derived. No additional scaling is required.

For CDL channel model variants A1/A2/A3 the angle scaling is applied, and no additional scaling is needed during channel coefficient generation.

#### SNR Normalisation with CDL

Way Forward:

Options:

- Option 1: Interested companies to provide information on long-term pre-EQ power measurements, for discussion whether a formula, or value, is required during RAN4#114-bis.

- Option 2: Define a normalisation method

With option 1 – it cannot be ensured that the normalization factor is the same for all companies, although it would be expected to be with the same agreed configuration.

For the normalization method, this is an additional step with CDL to be considered, especially for antenna array virtualization with sub-array. This creates additional overhead to the procedure and implementation.

- The normalization based on formula is an additional step needed for CDL and creates additional overhead to procedure and implementation.

#### Frequency Bands

Proposals:

- Option 1: RAN4 to study if the same CDL channel model can be defined for all carrier frequencies/ bands in given frequency range

The current models in the TR 38.827 are defined for specific scenarios, carrier frequency, with some assumptions on RMS delay spread, UE velocity, etc. RAN4 defines band agnostic demodulation and CSI reporting requirements.

- The CDL channel models in the TR 38.827 are defined for specific scenarios, carrier frequency, with some assumptions of RMS delay spread and UE velocity.

It would be very important to study if the spatial channel model can be defined for all applicable carrier frequencies/ bands for each FR.

RAN4 to study if the same channel model can be defined for all carrier frequencies/ bands in given frequency range – example FR1/ FR2.

## Comparison of methodologies

#### TDL and CDL Methodologies

Way Forward:

Options:

CDL from TR 38.753 will include randomness reduction for the purpose of the SI

- CDL (TR 38.753)

  - Option A1 : with Table 7.2.1-8 in TR 38.827

  - Option A2: based on Table 7.7.1-3 in TR 38.901 without truncation

  - Option A3 : based on Table 7.7.1-3 in TR 38.901 with truncation 
According to the updated truncation procedure described in Annex B, truncated table to be shared before RAN4#114-bis.

- Option B1: TDL (MediaTek)

- Option B2: TDL (Huawei)

For CDL based methodologies, we have 3 options –

  - Option A1 : with Table 7.2.1-8 in TR 38.827

  - Option A2: based on Table 7.7.1-3 in TR 38.901 without truncation

  - Option A3 : based on Table 7.7.1-3 in TR 38.901 with truncation

The difference between the variants is captured below:

|  | UMa-CDLC-A1 | UMa-CDLC-A2 | UMa-CDLC-A3 |
| --- | --- | --- | --- |
| ASmodel | Based on 38.827 | Based on 38.901 A1 | Based on 38.901 A1 |
| ZoA and cZSA | 90o and 0 | Scaled based on ASdesired ASmodel and mmodel | Scaled based on ASdesired ASmodel and mmodel |
| Number of clusters | 24 | 24 | 12 |
| Assumptions |  |  | Assumptions on AAV and downtilt built into truncation methodology |

Option A1 has a limitation of fixed ZoA of 90o which was the assumption in MIMO-OTA. We don’t see the necessity to keep this limitation for the study. It is also unclear why the formula for angle spreads is different from that in 38.901.

- For A1 we don’t need to have the limitation of ZoA=90. It is also unclear why angular spread is calculated differently than 38.901.

For option A3, the assumptions of antenna radiation pattern, AAV, BS down tilt need to be made for the truncation. In case another configuration is used, the generated table might not be directly applicable. Since CDL channel models are not as simple as TDL channel models, we also need to understand and ensure that the channel properties are not changed due to this.

- For A2 the assumption on BS antenna radiation pattern, AAV, BS down-tilt need to be made prior to truncation.

- The truncated channel model might only be used with the assumed config.

- We need to ensure that the channel properties are not changed due to the truncation.

In [2] we provide simulation results comparing the variants of CDL channel model.

Figure 2: UMa-CDL-C variants performance comparison in SU-PMI

Based on the results, we have the following observations:

- The performance with UMa-CDLC channel model variants – A1/A2/A3 are very similar for PDSCH with random PMI.

- Performance with follow PMI with Type I and eType II codebook between variants of UMa-CDL-C A1/A2/A3 is similar, but in some cases there is performance delta with A3 compared to A2.

A performance delta of up to 0.3 dB would be acceptable with the truncated channel model, but in some cases the delta is > 0.3 dB, which is concerning. This would indicate that some properties of the channel are changing with the truncation.

- Performance with truncated channel model is not expected to be more than 0.3 dB compared to untruncated channel.

- For the cases we notice a significant delta it could indicate that the channel properties are not maintained with truncation.

RAN4 should further study if there are any impacts to channel properties with the truncation.

RAN4 to study if channel properties are maintained with option A3 after truncation.

Based on the above observations, we propose that we only retain option A2 for the study. Until we see clear motivation, we also need not quantize the delays to 5ns.

For CDL based methodologies, only option A2 is retained for the study with no delay quantization.

The CDL table for option A2 to be captured in the TR:

Table 1: UMa-CDLC-A2 without delay quantization

| Cluster # | Cluster delay (ns) | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4 | -38.2 | -96.9 | 96.8 | 98.1 |
| 2 | 77 | -1.2 | -21.8 | 118.9 | 98.5 | 71.1 |
| 3 | 81 | -3.5 | -21.8 | 118.9 | 98.5 | 71.1 |
| 4 | 85 | -5.2 | -21.8 | 118.9 | 98.5 | 71.1 |
| 5 | 79 | -2.5 | -34.2 | -124.4 | 100.9 | 67.6 |
| 6 | 232 | 0 | -5.9 | 171.2 | 99.2 | 76.6 |
| 7 | 235 | -2.2 | -5.9 | 171.2 | 99.2 | 76.6 |
| 8 | 239 | -3.9 | -5.9 | 171.2 | 99.2 | 76.6 |
| 9 | 240 | -7.4 | 44.2 | 51.9 | 106.4 | 62.8 |
| 10 | 290 | -7.1 | -50.6 | 63.4 | 94.5 | 56.6 |
| 11 | 300 | -10.7 | 49.1 | -42.1 | 107.5 | 69.8 |
| 12 | 341 | -11.1 | -73.0 | 43.1 | 92.3 | 50.8 |
| 13 | 448 | -5.1 | -44.2 | 65.1 | 104.6 | 103.4 |
| 14 | 478 | -6.8 | -50.4 | -63.4 | 105.2 | 50.1 |
| 15 | 792 | -8.7 | -60.2 | 79.0 | 91.7 | 51.7 |
| 16 | 989 | -13.2 | 64.6 | 26.3 | 105.2 | 121.0 |
| 17 | 1554 | -13.9 | 62.2 | -22.6 | 94.0 | 53.9 |
| 18 | 1679 | -13.9 | 55.0 | -1.6 | 91.8 | 61.6 |
| 19 | 2004 | -15.8 | -76.3 | -19.8 | 90.7 | 37.5 |
| 20 | 2047 | -17.1 | 57.4 | 4.5 | 108.2 | 53.1 |
| 21 | 2302 | -16 | 58.1 | 0.2 | 91.7 | 35.8 |
| 22 | 2423 | -15.7 | 67.3 | -4.9 | 91.6 | 52.9 |
| 23 | 2571 | -21.6 | 76.2 | -28.3 | 106.4 | 46.4 |
| 24 | 3158 | -22.8 | -91.4 | 29.3 | 109.5 | 44.7 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter |  | cASD in [°] | cASA in [°] | cZSD in [°] | cZSA in [°] | XPR in [dB] |
| Value |  | 1.4 | 15.6 | 3.6 | 12.2 | 7 |

#### Antenna Array Virtualisation (for CDL options)

Way Forward:

Downselection to be based upon (1. Best performance, 2. Ease of alignment, 3. Practicality)

Options:

- Option 1Y: with Antenna Array Virtualisation (AAV) using subarray configuration

  - [(M,N,P,Ms,Ns)= (16,8,2,8,1) for SU-MIMO eTypeII, for 32 TX, for comparison only]

  - (M,N,P,Ms,Ns)  = (8,2,2,8,1) for 4Tx CSI-RS Ports

  - (M,N,P,Ms,Ns)  = (8,4,2,8,1) for 8Tx CSI-RS Ports

- Option 3: with AAV with the following configuration:

  - (M,N,P,Ms,Ns) = (1,4,2,1,1) for 8Tx CSI-RS Ports

  - (M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports

For the past few meetings, we have discussed a few options for antenna array virtualization, and in the last meeting we down selected to 2 options -AAV 1Y and AAV3. We understand that AAV with sub-array is used in deployment, but we don’t see the need to study this and have it in our CDL channel model configuration. From UE perspective nothing different is tested, as this is transparent to the UE. Using the sub-array config needs extra normalization step for SNR which increased overhead. At least from our simulation results for SU-MIMO, we do observe some gain with sub-array AAV, but don’t see the necessity of introducing this complexity.

- From UE perspective nothing different is tested with sub-array AAV.

- Using sub-array AAV required extra SNR normalization step that adds overhead.

Also, in [2] with SU-PMI we observe gains with eType II over Type I without using sub-array AAV by increasing the number of TX ports.

- For SU-PMI we observe gains with eType II over Type I with larger number of TX ports without sub-array AAV.

Based on the above observations we propose on only retain AAV3 going forward in this study.

For CDL based methodologies, only AAV3 is retained for the study.

#### Comparison with Legacy TDL

Agreement:

The following Legacy TDL models for comparison:

- Option 1: TDLC 300-10/100 with following correlation:

  - Option 1a: Low

  - Option 1b: XP Medium

  - Option 1c: XP High

  - Option 1d: Medium

Comparison with legacy TDL models is necessary in the study to understand the limitations of current methodology and also for comparison. The channel models and correlation should be chosen carefully, as not all corr levels are suitable for all antenna configs. We propose the following:

For SU-MIMO PDSCH:

4x4:  TDL with low correlation, and XP-Med.

8x8: TDL with low correlation, XP-MedA (variant of XP-Med with smaller a, b)

For SU-MIMO PMI:

8x4: TDL XP-Med, and XP-High.

32x4: TDL XP-Med or variant with smaller a

For legacy TDL, the following antenna correlation can be used: 
For SU-MIMO PDSCH:
     4x4:  TDL with low correlation, XP-Med.
     8x8: TDL with low correlation, XP-MedA (variant of XP-Med with smaller a, b)
For SU-MIMO PMI:
     8x4: TDL XP-Med, and XP-High.
     32x4: TDL XP-Med or variant with smaller a

#### Performance Comparison Methodologies

Way Forward:

Companies welcome to bring views at next meeting

So far we have agreed on several test cases for SU-MIMO, but we have not discussed the metric we would employ to compare the methodologies to rationalize the need for a special channel model for demodulation requirements. The necessity of introducing spatial channel model should be well justified. We need to further discuss the metric used to compare methodologies that help identify limitations in the current methodology that impacts UE/BS demod/ MIMO performance. Without a concrete idea on how to this is done, it would be difficult to move ahead with the agreed comparison.

RAN4 to discuss test metrics to compare the methodologies that help identify limitations in the current methodology that impacts UE MIMO performance.

Without identifying the limitations of current TDL based methodology it would not be practical to move ahead with huge simulation campaign for alignment of performance with CDL based methodologies.

We also need to evaluate if results are well aligned among companies with new methodologies including the test complexity and result convergence time.

RAN4 to consider the following aspects in determining the feasibility of spatial channel model methodology:
- Test convergence time
- Alignment of results
- Test complexity

#### On results collection for alignment and comparison

Way Forward:

Options:

- Option 1: Gamma value to random PMI configuration.

- Other options are not preconcluded.

Currently we have several spreadsheets with multiple sheets within to collect results for the different options. The task of data entry itself is cumbersome and time consuming. We suggest that we simplify the alignment and comparison by collecting the following metrics instead of full TP curves:

SU-MIMO PDSCH

SNR at 70%, 30% Max TP

SU-PMI

SNR at 90 % Max TP for follow Type I PMI

Gamma at 90% Max TP with follow Type I PMI

SNR at 90 % Max TP for follow eType II PMI

Gamma at 90% Max TP with follow eType TI PMI

- The task of data entry and collection for results alignment and comparison is very cumbersome with raw data (SNR and rel TP values)

Simplify the process of collecting results by collecting the following metrics:
SU-MIMO PDSCH
     SNR at 70%, 30% Max TP
SU-PMI
     SNR at 90 % Max TP for follow Type I PMI
     Gamma at 90% Max TP with follow Type I PMI
     SNR at 90 % Max TP for follow eType II PMI
     Gamma at 90% Max TP with follow eType TI PMI

# 3. Conclusion

In this paper, we provide our views on scope of study for spatial channel models for defining demodulation and CSI reporting requirements in RAN4. Our observations and proposals are captured below:

Common for all methodologies

- It is not clear if fixed PMI option is for both 4RX and 8RX cases.

- With random PMI the max TP might not be achieved with 8RX-8layers

- It is very cumbersome to fix PMI and requires a 2-step process

- For the study, it should be okay if max TP is not achieved with random PMI since requirements are not being defined here.

- Only evaluate with random PMI for PDSCH test cases with 4RX and 8RX.

For alignment use -
Low speed/Doppler for PMI test cases – 3kph for CDL/10Hz for TDL
High speed/Doppler for PDSCH test cases – 30kph for CDL/100Hz for TDL

CDL based methodologies

For CDL channel model variants A1/A2/A3 the angle scaling is applied, and no additional scaling is needed during channel coefficient generation.

- The normalization based on formula is an additional step needed for CDL and creates additional overhead to procedure and implementation.

- The CDL channel models in the TR 38.827 are defined for specific scenarios, carrier frequency, with some assumptions of RMS delay spread and UE velocity.

RAN4 to study if the same channel model can be defined for all carrier frequencies/ bands in given frequency range – example FR1/ FR2.

Comparison of methodologies

- For A1 we don’t need to have the limitation of ZoA=90. It is also unclear why angular spread is calculated differently than 38.901.

- For A2 the assumption on BS antenna radiation pattern, AAV, BS down-tilt need to be made prior to truncation.

- The truncated channel model might only be used with the assumed config.

- We need to ensure that the channel properties are not changed due to the truncation.

- The performance with UMa-CDLC channel model variants – A1/A2/A3 are very similar for PDSCH with random PMI.

- Performance with follow PMI with Type I and eType II codebook between variants of UMa-CDL-C A1/A2/A3 is similar, but in some cases there is performance delta with A3 compared to A2.

- Performance with truncated channel model is not expected to be more than 0.3 dB compared to untruncated channel.

- For the cases we notice a significant delta it could indicate that the channel properties are not maintained with truncation.

RAN4 to study if channel properties are maintained with option A3 after truncation.

For CDL based methodologies, only option A2 is retained for the study with no delay quantization.

- From UE perspective nothing different is tested with sub-array AAV.

- Using sub-array AAV required extra SNR normalization step that adds overhead.

- For SU-PMI we observe gains with eType II over Type I with larger number of TX ports without sub-array AAV.

For CDL based methodologies, only AAV3 is retained for the study.

For legacy TDL, the following antenna correlation can be used: 
For SU-MIMO PDSCH:
     4x4:  TDL with low correlation, XP-Med.
     8x8: TDL with low correlation, XP-MedA (variant of XP-Med with smaller a, b)
For SU-MIMO PMI:
     8x4: TDL XP-Med, and XP-High.
     32x4: TDL XP-Med or variant with smaller a

RAN4 to discuss test metrics to compare the methodologies that help identify limitations in the current methodology that impacts UE MIMO performance.

Without identifying the limitations of current TDL based methodology it would not be practical to move ahead with huge simulation campaign for alignment of performance with CDL based methodologies.

RAN4 to consider the following aspects in determining the feasibility of spatial channel model methodology:
- Test convergence time
- Alignment of results
- Test complexity

- The task of data entry and collection for results alignment and comparison is very cumbersome with raw data (SNR and rel TP values)

Simplify the process of collecting results by collecting the following metrics:
SU-MIMO PDSCH
     SNR at 70%, 30% Max TP
SU-PMI
     SNR at 90 % Max TP for follow Type I PMI
     Gamma at 90% Max TP with follow Type I PMI
     SNR at 90 % Max TP for follow eType II PMI
     Gamma at 90% Max TP with follow eType TI PMI

# Reference

- R4-2502378, “Way Forward for [114][322] NR_SCM”, Nokia.

- R4-2503541, “Simulation results for Spatial Channel Modeling study”, Apple