3GPP TSG-RAN4 Meeting #114-bis	R4-2503562

# Wuhan, China, 7th – 11th April, 2025

Title:	n1 NS_05 and NS_05U PC2 A-MPR proposal

Source:	Qualcomm Incorporated

Agenda item:	5.3.1

Release:	Rel-19

Document for:	Approval

# 1.	Background

n1 NS_05 requirements are relaxed since the PHS service is closed. However, the DECT service in the same range 1884.5MHz - 1915.7MHz must still be protected in Japan. The updated Japanese regulations were captured, the PHS protection requirement was modified to protect DECT [1], and PC3 A-MPR definitions and values were agreed in WF [2] for NS_05 requirement in the previous meeting. This paper proposes PC2 A-MPR regions and values for NS_05 and NS_05U requirement for 5MHz, 10MHz, 15MHz and 20MHz channel bandwidths for various waveforms and modulation orders.

# 2. 	Discussion

## 2.1	Measurement and simulation assumptions

We analyze the PC2 A-MPR regions and values for n1 NS_05 and NS_05U with the modified PHS requirements, through simulations and measurements.

The simulations and measurements were done for channel bandwidths 5MHz, 10MHz, 15MHz and 20MHz, wherein the channels were analyzed at different Fc values, and A-MPR values were proposed accounting for implementation margin.

We also use the following impairment levels in our measurements and simulations:

- PC2

- Calibration: 1dB MPR: DFT-s-OFDM QPSK BW20MHz, 100RB0

- Carrier feedthrough = -28dBc

- Image = -28dBc

- CIM3 = -60dBc

## 2.2 	Agreement

### 2.2.1	Proposed PC2 NS_05 and NS_05U A-MPR region definitions

Shown below in Table 1 is the proposed PC2 A-MPR regions for n1 NS_05 and NS_05U

Table 1 also includes a note to account for NS_05U A-MPR region.

| NS_05 | Fc (MHz) | Region A1 |  | Region A2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LCRB | RBstart | LCRB | RBstart |
| 5M |  | N/A | N/A | N/A | N/A |
| 10M | < 1930.7 | ≥ 1.35*RBstart + 4.32MHz/12/scs | ≥ 0 | ≤ 2.16MHz/12/scs | ≤ 1.8MHz/12/scs |
| 10M |  | N/A | N/A | N/A | N/A |
| 15M | 1927.5≤Fc<1932.5 | ≥ RBstart*1.2 + 4.68MHz/12/scs | ≥ 0 | ≤ 4.5MHz/12/scs | ≤ 3.6MHz/12/scs |
| 15M | 1932.5 ≤Fc<1938.2 | ≥ 8.28MHz/12/ scs | ≤ 1.26MHz/12/scs | ≤ 2.16MHz/12/scs | ≤ 2.16MHz/12/scs |
| 20M | < 1945.7 | ≥ RBstart*1.43 + 4.14MHz/12/scs | ≥ 0 | ≤ 6.48MHz/12/scs; ≤ RBstart*1.5 + 4.14MHz/12/scs | ≤ 4.68MHz/12/scs |
| NOTE: When NS_05U is signalled, Region A3 is applicable and adopts its definitions from NS_100 requirement. |  |  |  |  |  |

Table 1. Proposed PC2 NS_05 and NS_05U A-MPR region definitions

The NS_05 regions that include combination of Region A1 and Region A2 are represented graphically in the plots in Figure 1 below for various applicable channel bandwidths.

Figure 1. Graphical representation of proposed PC2 NS_05 A-MPR region definitions

The NS_05U network signaling includes an additional emission requirement for UTRA protection, defined by NS_100. This specification focuses on A-MPR for outer RB allocations.

Proposal 1: Add new region definitions for NS_05 and NS_05U PC2 A-MPR according to Table 1

### 2.2.2	Proposed PC2 NS_05 and NS_05U A-MPR values

Table 2 below shows the proposed PC2 A-MPR values for NS_05 regions noted in Figure 1.

For NS_05U, the A-MPR from NS_100 is adopted and incorporated into the A-MPR table in the “Note” section as Region A3, as shown in Table 1 above. The final A-MPR for outer RB allocations will be the maximum value among MPR, A-MPRNS_05, and A3. MPR values are as stated in Table 6.2D.2-1.

Hence for NS_05U, mathematically A-MPR will be calculated as max (MPR, A1, A2, A3).

| NS_05 |  | 5M | 10M @<1930.7 |  | 15M @1927.5 |  | 15M @1932.5 |  | 20M @<1947.5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | A1 | A2 | A1 | A2 | A1 | A2 | A1 | A2 |
| DFT-s-OFDM | BPSK | 0.0 | 3.5 | 3.0 | 3.5 | 3.5 | 3.0 | 3.5 | 4.0 | 4.0 |
|  | QPSK |  | 5.0 | 4.0 | 5.0 | 5.0 | 4.0 | 4.0 | 4.5 | 4.5 |
|  | 16QAM |  | 5.5 | 4.5 | 5.5 | 5.0 | 4.5 | 4.5 | 6.0 | 5.5 |
|  | 64QAM |  | 5.5 | 4.5 | 5.5 | 5.0 | 4.5 | 4.5 | 6.0 | 5.5 |
|  | 256QAM |  |  |  |  |  |  |  | 6.0 |  |
| CP-OFDM | QPSK |  | 6.0 | 5.0 | 6.5 | 6.5 | 4.5 | 5.5 | 6.5 | 6.0 |
|  | 16QAM |  | 6.0 | 5.0 | 6.5 | 6.5 | 4.5 | 5.5 | 6.5 | 6.5 |
|  | 64QAM |  | 6.0 | 5.0 | 6.5 | 6.5 | 4.5 | 5.5 | 6.5 | 6.5 |
|  | 256QAM |  |  |  |  |  |  |  |  |  |
| NOTE: When NS_05U is signaled, A-MPR = max(MPR,A1,A2,A3), where A3 is as defined in Table 6.2.3.1-2 and applicable to all channel bandwidths. |  |  |  |  |  |  |  |  |  |  |

Table 2. Proposed PC2 NS_05 and NS_05U A-MPR values

Proposal 2: Define new A-MPR values for PC2 NS_05 and NS_05U according to Table 2

# Conclusion

This contribution proposes A-MPR regions and values for PC2 n1 NS_05 and NS_05U modified requirements based on measurements and simulations. Companies are encouraged to provide their A-MPR values for each proposed region and the A-MPR values can be specified by averaging the companies’ results.

Proposal 1: Add new region definitions for NS_05 and NS_05U PC2 A-MPR according to Table 1

Proposal 2: Define new A-MPR values for PC2 NS_05 and NS_05U according to Table 2

# Reference

[1] R4-2420481, “WF on PHS protection update”, KDDI, OPPO, 3GPP TSG-RAN WG4 Meeting #113, Orlando, US, 18th – 22nd November, 2024

[2] R4- 2502980, “WF on UE requirements for PHS protection”, KDDI, OPPO, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greek, 17th – 21st February, 2025