# 3GPP TSG-RAN WG4 Meeting #114bis	                                                 R4-2503574

Wuhan, China, 7 – 11 April, 2025

Agenda Item:	7.28.2

Source: 	Apple

Title:	Discussion of RRM impact of XR for NR Phase 3

Document for:	Discussion

# Introduction

RRM impact of XR for NR Phase 3 has been widely discussed in previous RAN4 meetings. The latest agreements can be found in [1], according to which there are quite many open issues. In this contribution, we continue discussing the open issues.

# Discussion

## Issue 1-1: Deployment scenarios

< Agreement>:

- Deprioritize FR2-2 for enabling transmission/reception in gaps/restrictions that are caused by RRM measurements

<Way forward>:

- Further discussion needed for

  - NR-DC

Regarding DC, if only dynamic solution is supported in this work item, it would be quite challenging to support DC. Note that MG sometimes apply to both MCG and SCG. When MN/SN uses DCI to cancel some MG occasions,  SN/MN cannot realize that timely.

Observation 1: if only dynamic solution is supported, it could be challenging to enable it in DC scenario.

If RRC based solution is supported, then UE under DC scenario can also enjoy this feature since RRC based solution is stable and can be easily exchanged between MN and SN.

Proposal 1: In addition to DCI-based MG skipping control, RRC-based MG skipping control should also be supported, especially for DC scenario.

# Topic #2: Measurements with gaps

## Issue 2-1: Types of measurement gaps

<Way forward>:

- In this issue we discuss the types of measurement gaps to be considered in this WID. In order to help the discussion, we reuse the definition of Type-1 and Type-2 measurement gaps as agreed in Rel-18 NR_MG-enh2:

- Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG

- Companies may bring proposals regarding the priorities of each gap type for RAN4 to work based on the following table

| Gap types | First priority | Second priority | Don’t include |
| --- | --- | --- | --- |
| Type 1 |  |  |  |
| Type 2 |  |  |  |
| NCSG |  |  |  |
| MUSIM |  |  |  |
| Positioning |  |  |  |
| Rel 18 concurrent gap |  |  |  |
| PreConfigured |  |  |  |

We continue supporting the following types:

- Type-1 gap

- Type-2 gap

- NCSG

- concurrent gaps

- pre-MG

Proposal 2: priorities of each gap type:

| Cap types | First prio | Second prio | Don’t include |
| --- | --- | --- | --- |
| Type 1 | √ |  |  |
| Type 2 | √ |  |  |
| NCSG | √ |  |  |
| MUSIM |  | √ |  |
| Positioning |  |  | √ |
| Rel 18 concurrent gap | √ |  |  |
| PreConfigured | √ |  |  |

## Issue 3-1: How to capture impact of measurement skipping

It is straightforward that measurement cancellation has negative impact on measurement. In order to maintain the same measurement performance, UE still needs same number of measurement samples in physical layer. Assuming during the measurement period, network cancels X measurement occasions, then the total measurement delay shall be extended by X measurement occasions. Take inter-frequency measurement with gap for example:

Note that if too many measurement occasions are cancelled on target carrier (UE cannot measure it for a long time), UE may need to adjust AGC and timing again before it can perform measurement on the carrier successfully. From UE measurement point of view, this is similar to measurement cancellation due to LBT failure. Similarly, max number of occasions shall be defined within a measurement period. If actual number of cancelled occasions exceeds the maximum value, UE shall be allowed to restart a measurement period.

Besides, another point we are considering is whether and how to guarantee mobility performance on some particular carriers, e.g. carrier(s) with high priority like PCC. One potential solution is to assign more measurement opportunities to carriers with high priority, such that legacy measurement requirements can still apply on those carriers. The impact is on other carriers, on which measurement delay may be further extended.

Proposal 3: as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic.

Proposal 4: max number of occasions cancelled during a measurement period shall be defined.

# Topic #5: Need/feasibility of UE assistance information

## Issue 5-1: General on UAI

<Agreement>:

- Agree to define UAI based on the following assumptions, and further discuss the content of UAI

- RAN4 will not specify any explicit or implicit requirement or expectation on gNB behaviour in response to any XR UAI (from RAN4 perspective)

- Not define UE behaviour and the corresponding requirements based on UAI

- Supporting of UAI is optional for UE with optional UE capability.

<Way forward>:

  - Proposals related to measurement occasions

    - Option 1: Do not define UAI related to measurement occasions.

    - Option 2: The UE indicates a pattern of unused gaps

    - Option 3: The UE indicates a ratio/number of occasions that can be skipped without measurement delay extension

    - Option 4: The UE indicates NW a status whether it can tolerate measurement cancellation.

    - Option 5: One of options 2, 3, or 4 but the information is reported as UE capability.

  - Proposals related to channel conditions

    - Option 1: Provide information on channel conditions

    - Option 2: do not provide information on channel conditions

In general, we support option 4 for simplicity. One example is when  UE is configured with search threshold (s-MeasureConfig), the UE informs the network that it is stopping doing non-serving cell measurements because the s-MeasureConfig condition is no longer fulfilled.

We are also open for option 2 and 3.

Proposal 5: support option 4 for simplicity. Open for option 2 and 3:

- Option 2: The UE indicates a pattern of unused gaps

- Option 3: The UE indicates a ratio/number of occasions that can be skipped without measurement delay extension

- Option 4: The UE indicates NW a status whether it can tolerate measurement cancellation (preferred).

# Conclusion

In this contribution, we provide discussion on RRM impact of XR for NR phase 3. After discussion, the following conclusions are provided:

Observation 1: if only dynamic solution is supported, it could be challenging to enable it in DC scenario.

Proposal 1: In addition to DCI-based MG skipping control, RRC-based MG skipping control should also be supported, especially for DC scenario.

Proposal 2: priorities of each gap type:

| Cap types | First prio | Second prio | Don’t include |
| --- | --- | --- | --- |
| Type 1 | √ |  |  |
| Type 2 | √ |  |  |
| NCSG | √ |  |  |
| MUSIM |  | √ |  |
| Positioning |  |  | √ |
| Rel 18 concurrent gap | √ |  |  |
| PreConfigured | √ |  |  |

Proposal 3: as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic.

Proposal 4: max number of occasions cancelled during a measurement period shall be defined.

Proposal 5: support option 4 for simplicity. Open for option 2 and 3:

- Option 2: The UE indicates a pattern of unused gaps

- Option 3: The UE indicates a ratio/number of occasions that can be skipped without measurement delay extension

- Option 4: The UE indicates NW a status whether it can tolerate measurement cancellation (preferred).

# Reference

- R4-2502700, WF on RRM requirements for XR_Ph3, Nokia