3GPP TSG-RAN WG4 Meeting # 114bis	R4- 2503581

# Wuhan, Hubei, China, 07th – 11st April, 2025

Source:	LG Electronics

Title:	Discussion on RRM core requirements for LP-WUS/WUR

Agenda item:	7.26.4.2

Document for:	Discussion

Introduction

In this contribution, we provide our views on issues of RRM core requirements for NR LP-WUS/WUR based on open issues in last meeting’s moderator summary [2].

Discussion

- Topic #1: RRM core requirements for LP-WUS/WUR

General aspects

### Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

RAN4 has been agreed to support case#1 and #3. But regarding case#2 scenario, RAN4 doesn’t make agreement and it can follow the RAN2 decision. For now RAN2 has discussed entry/exit condition separately for serving cell RRM relaxation and neighbor cell RRM relaxation as below:

RAN2#127bis (Oct.2024) meeting Working assumption - For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. - FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available. RAN2#128 (Nov.2024) meeting - The entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. FFS if LR measurement is needed. - FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation. RAN2#129 (Feb.2025) meeting - The entry condition for MR serving cell RRM relaxation can include both MR and LR measurements. - If LR threshold is configured, the entry condition is when both MR and LR measurement are above the configured thresholds.

From the RAN2’s discussion, if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation, then it means not to support case#2. Else, it means to support case#2.

So, regarding case#2 scenario, we think RAN4 need to wait for RAN2’s conclusion.

Proposal 1: Regarding case#2 scenario, RAN4 need to follow RAN2’s conclusion.

Issue 1-1-2: Threshold for switch between different cases

- Proposals - P1: Up to RAN2 decision (CMCC CT) - P2: New LP-based threshold(s) based on LP-RSRP or LP-RSRQ used for switch into case 1 from other cases or exit from case 1 to other cases are needed (Apple ZTE vivo) - P2-1: LR measurement result shall be comparable to MR measurement result or shall be equivalent to MR measurement result with certain offset/margin (e.g., LR threshold is MR threshold + offset/margin). (Apple ZTE) - P2-2: Details on threshold are up to RAN2 (vivo) - P3: Discuss first what has been agreed in RAN2 (Nokia) - P4: RAN4 to study switching between Case#3 and Case#1 based on a timer. Since UE need to be confident enough to enter Case#1 before deactivating MR, it is better for the UE to enter Case#3 first for a period of time (confidence period) before UE can enter Case#1. (MTK)

Regarding threshold for switch between different cases, it is up to RAN2 decision. But RAN4 also need to consider how to avoid the ping-pong problem. To avoid the ping-pong problem, at least the LR and MR measurement result should be comparable or equivalent with certain offset/margin. So, the ping-pong problem looks like can be solved by adopting the LR threshold to MR threshold + offset/margin. But what if the ping-pong problem occur, even though the LR threshold to MR threshold + offset/margin. We think this situation can likely occur enough, and to solve this issue the UE should be allowed to adjust the configured LR threshold. It can be adoptable when the NW configuration is too conservative LR threshold also. In this case some UEs can extend the case#1 service area by adjusting the LR threshold.

For example, from the perspective of noise figure, the impact of LR and MR on RSRP and RSRQ is as follow. Let’s suppose that LR has higher noise figure than MR.

- The noise figure does not impact to the mean of RSRP

- The noise figure can impact to the variance of RSRP. So, the RSRP of LR has higher variance than the RSRP of MR

- If the UE is in the environment with same ideal RSRP between LR and MR, than the RSRQ of LR is lower than the RSRQ of MR. Because the noise power of LR case is higher than MR case at same signal power environment.

Considering this scenario, the LR variance should be considered when deciding the LR threshold. And additionally the mean value between LR RSRP and MR RSRP can have offset by several non-ideal factors.

Proposal 2: The threshold for switching between cases is up to RAN2 decision.

Proposal 3: RAN4 to consider how to avoid the ping-pong problem as below (based on P2-1).

- the LR and MR measurement result should be comparable or equivalent with certain offset/margin and additionally UEs are allowed to adjust the configured LR threshold within specific range.

Meanwhile, when the NW configure the LR threshold it should consider the UE types. Becase, according to the RF agreements the OOK based LR and OFDM based LR can have different noise figure. So, the NW should configure independently the LR threshold according to LR types.

Proposal 4: NW should configure independently the LR threshold according to LR types (OOK based LR and OFDM based LR).

Issue 1-1-8: LP-WUR operating carrier frequency

From the RAN2 reply LS R4-2503116, RAN2 shared that the UE can report the supporting LR band to NW and the overload issues also could be addressed by current mechanism in spec as below.

- RAN2 understands that UE can report which band(s) is supported by LR to NW. - RAN2 understands that any potential overload issues could be addressed by current mechanism in spec.

So, RAN4 can consider only same carrier frequency between MR and LR to define the requirements in Rel-19.

Proposal 5: RAN4 can consider only same carrier frequency between MR and LR to define the requirements in Rel-19.

Detail LP-WUR requirements at RRC_IDLE/INACTIVE state

### Issue 1-2-2: Periodicity for SSB based LP-WUR measurement delay requirements

In the last RAN4#113 meeting, the periodicity for SSB based LP-WUR measurement delay requirements are discussed as follow:

Agreement: If LP-SS is not configured, periodicity for SSB based LP-WUR measurement delay requirements - based on LO periodicity - FFS whether to define an upper bound - Note: DRX cycle and LO periodicity are the same based on RAN1 agreement. Agreement: When LP-SS is configured, apply the same agreement as for when LP-SS is not configured - The above agreement applies if LO periodicity is equal to or larger than LP-SS periodicity, FFS if LO periodicity is smaller than LP-SS periodicity.

When LP-SS is configured, the LP-SS periodicity is not important for SSB based LP-WUR measurement delay requirements. Even the LO periodicity is smaller than LP-SS periodicity, the UE measure SSB during DRX cycle which has same periocity with LO. So, when the LO periodicity is smaller than LP-SS periodicity, it doesn’t impact to the SSB based LP-WUR measurement delay requirements. We think RAN4 do not need to care whether LO periodicity is smaller than LP-SS periodicity or not for SSB based LP-WUR measurement delay requirements.

Proposal 6: RAN4 do not need to care whether LO periodicity is smaller than LP-SS periodicity or not for SSB based LP-WUR measurement delay requirements.

Issue 1-2-6: On MR wake up delay related requirements for LP-WUR

- Proposals - P1: MR wake-up time is pending on UE capability which can be 400ms and 800ms based on RAN1 agreement; MR sync time, need to be further discuss with different side conditions (LG) - Condition 1: MR and LR well sync, and LR sync info can be fully reused e.g. 1~2 SSB sufficient - Condition 2: MR and LR not well synced and MR need to apply sync procedure e.g. at least 10 SSBs required - P2: Wait for RAN1 further progress. (CMCC CATT oppo Huawei Ericsson) - P3: RAN4 only needs to define the wake-up delay requirement, if such requirement is necessary from majority companies’ view, where the wake-up delay means minimum gap time between LP-WUS reception and MR to start PDCCH monitoring. (Apple) - P4: After MR is triggered to be waken up and during the waking up duration, MR should follow SSB periodicity instead of DRX periodicity to perform measurement (vivo) - P5: Defining MR wake up requirement for the normal case and the “wake up delay” duration for the normal is: “wake up delay” duration = Ramping duration + [5] samples * SSB periodicity; Whether to further consider a worst case where a cell reselection is needed after ME is wake up needs FFS. (vivo) - P6: RAN4 shall define the minimum required time for MR synchronisation and related side conditions (Nokia)

Wake-up delay related RAN1 agreements are as below.

RAN1#118 (Aug. 2024) Agreement For the wake-up delay, consider the following options: - Option 2: UE reports one value from X candidate values for the wake-up delay via UE capability reporting. - FFS: X is 2, 3, 4 - Above applies for IDLE/INACTIVE mode. - Definition of wake-up delay: Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring RAN1#118bis (Oct. 2024) Agreement Support 3 candidate values for the wake-up delay capability a UE reports, two values for ultra-deep sleep state and one value for deep sleep state. - FFS the values - Note: Ultra-deep sleep state and deep sleep state are mentioned above solely for the purpose of RAN1 discussions and it is not intended to be captured in specifications from RAN1 perspective RAN1#120 (Feb. 2025) Agreement For UE capability report on the wake-up delay: - Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[70ms], [500ms] and [900ms]}. - The reported values assume SSB periodicity of 20ms, where [70ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization. - FFS: translation of wake-up delay for different SSB periodicities - FFS: different sets of 3 candidate value for different SSB periodicities - Send an LS to RAN4 after the down-selection to confirm the number of SSBs. Final LS in R1-2501624.

Regarding the wake-up delay, RAN1 still keep discussing and sent LS to RAN4 as below:

RAN1 -> RAN4 LS R4-2503110

In RAN1#118bis, it was agreed to support 3 candidate values for the wake-up delay capability a UE reports for LP-WUS operation in IDLE/INACTIVE mode. In addition, the following agreement was made in RAN1#120: Agreement For UE capability report on the wake-up delay: - Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[70ms], [500ms] and [900ms]}. - The reported values assume SSB periodicity of 20ms, where [70ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization. - FFS: translation of wake-up delay for different SSB periodicities - FFS: different sets of 3 candidate value for different SSB periodicities Note that the definition of wake-up delay is the minimum gap time between LP-WUS reception and MR to start PDCCH monitoring. RAN1 respectfully asks RAN4 to check and confirm the number of SSBs currently assumed by RAN1 for MR synchronization for SSB periodicity of 20ms. RAN1 also respectfully asks RAN4 to confirm if it is reasonable to assume the same number of SSBs for MR synchronization for different SSB periodicities.

So, RAN4 need to discuss the MR sync time for 3 candidate wake-up delay for 20ms SSB periodicity case first. The 3 candidate wake-up delay can be categorize two groups as below.

  - Group#1 {[70ms]}: MR and LR well sync, and LR sync info can be fully reused condition

  - Group#2 {[500ms], [900ms]}: MR and LR not well synced and MR need to apply sync procedure condition

We think the RAN1’s assumptions are reasonable. 3 SSB for group#1 and 5 SSB for group#2.

And for different SSB periodicities, the same number of SSBs for MR synch also can be reasonable.

Proposal 7: RAN1’s assumptions that 3 SSB for [70ms] wake-up delay and 5 SSB for [500ms], [900ms] are reasonable at 20ms of SSB periodicity.

Proposal 8: For different SSB periodicities, the same number of SSBs for MR synch also can be reasonable.

Issue 1-2-7: On LP-WUR cell selection evaluation requirements for case 1

- Proposals - P1: No need to define the LR-WUR based cell selection criterion evaluation in case 1. (Apple CMCC vivo Huawei MTK) - P2: Define LP-WUR based serving cell selection criterion S evaluation requirement in case 1. (CATT Ericsson) - P2-1：The existing MR based evaluation requirements can be the baseline (CATT)

In the case 1, the MR fully offloaded to LR. It could be possible when the UE do not need to consider mobility measurement at case 1. So, RAN4 do not need to define the LR-WUR based cell selection criterion evaluation in case 1. If the situation arises where the UE needs to measure mobility in case 1, it can be solved by adjusting the entry/exit threshold.

Proposal 9: RAN4 do not need to define the LR-WUR based cell selection criterion evaluation in case 1.

Conclusion

In this contribution, we provide our views on the remain issues listed in previous meeting’s moderator summary[2], and we propose

Proposal 1: Regarding case#2 scenario, RAN4 need to follow RAN2’s conclusion.

Proposal 2: The threshold for switching between cases is up to RAN2 decision.

Proposal 3: RAN4 to consider how to avoid the ping-pong problem as below (based on P2-1).

- the LR and MR measurement result should be comparable or equivalent with certain offset/margin and additionally UEs are allowed to adjust the configured LR threshold within specific range.

Proposal 4: NW should configure independently the LR threshold according to LR types (OOK based LR and OFDM based LR).

Proposal 5: RAN4 can consider only same carrier frequency between MR and LR to define the requirements in Rel-19.

Proposal 6: RAN4 do not need to care whether LO periodicity is smaller than LP-SS periodicity or not for SSB based LP-WUR measurement delay requirements.

Proposal 7: RAN1’s assumptions that 3 SSB for [70ms] wake-up delay and 5 SSB for [500ms], [900ms] are reasonable at 20ms of SSB periodicity.

Proposal 8: For different SSB periodicities, the same number of SSBs for MR synch also can be reasonable.

Proposal 9: RAN4 do not need to define the LR-WUR based cell selection criterion evaluation in case 1.

Reference

[1] R4-2502600, “WF on RRM requirements for NR_LPWUS,” Moderator (vivo)

[2] R4-2500539, “Topic summary for [114][225] NR_LPWUS,” Moderator (vivo)

[3] R4-2503110, “LS on the wake-up delay for LP-WUS operation in IDLE/INACTIVE mode, ” source: RAN1, to RAN4

[4] R4-2503116, “Reply LS on LP-WUS operation in IDLE/INACTIVE mode, ” source: RAN2, to RAN1, RAN4