3GPP TSG-RAN4 Meeting #114bis	R4-2503595

# Wuhan, China, 7th – 11th April, 2025

Source:	LG Electronics

Title:	Tx requirements for SL intra-band non-contiguous CA

Agenda item:	7.2.1

Document for:	Discussion

Introduction

In this paper, we provide our view on A-MPR requirements with NS_33 based on agreed WF[1].

Discussion

A-MPR (General for NS_33)

In [1], the followings were agreed for A-MPR.

- Evaluate 10+10MHz channel bandwidth combination - Apply the NR NCCA SEM principle - The A-SEM requirement applies only within 5855 MHz to 5925 MHz according to ETSI EN 302 571. - The A-SE requirement applies as shown in Table below and according to ETSI EN 302 571. Protected band; Frequency range (MHz); Maximum Level (EIRP); MBW (MHz). Frequency range; 5835; -; 5855; -30; 1. Frequency range; 5925; -; 5945; -30; 1.

Figure 2.1-0 shows A-SEM and A-SE for 7 carrier frequencies, 5860, 5870, 5880, 5890, 5900, 5910, and 5920MHz.

Figure 2.1-0: A-SEM and A-SE for "NS_33"

Table 2.1-1 and Figure 2.1-1 show all aggregated 10+10 configuration of SL intra-band NC CA in which EU regulation can apply with ‘NS_33’.

Table 2.1-1: All aggregated 10+10 configuration of SL intra-band NC CA for "NS_33"

| Aggregated 10+10 Conf.# | description | Gap [MHz] | Fc(CC1) [MHz] | Fc(CC2) [MHz] | Remarks (A-SEM & A-SE aspect) |
| --- | --- | --- | --- | --- | --- |
| 1 | S0_10_G10_10 | 10 | 5860 | 5880 | = Scen#11 |
| 2 | S10_10_G10_10 | 10 | 5870 | 5890 | = Scen#10 |
| 3 | S20_10_G10_10 | 10 | 5880 | 5900 |  |
| 4 | S0_10_G20_10 | 20 | 5860 | 5890 | = Scen#13 |
| 5 | S10_10_G20_10 | 20 | 5870 | 5900 | = Scen#12 |
| 6 | S0_10_G30_10 | 30 | 5860 | 5900 | = Scen#14 |
| 7 | S10_10_G30_10 | 30 | 5870 | 5910 |  |
| 8 | S0_10_G40_10 | 40 | 5860 | 5910 | = Scen#15 |
| 9 | S0_10_G50_10 | 50 | 5860 | 5920 |  |
| 10 | S30_10_G10_10 | 10 | 5890 | 5910 |  |
| 11 | S40_10_G10_10 | 10 | 5900 | 5920 |  |
| 12 | S20_10_G20_10 | 20 | 5880 | 5910 |  |
| 13 | S30_10_G20_10 | 20 | 5890 | 5920 |  |
| 14 | S20_10_G30_10 | 30 | 5880 | 5920 |  |
| 15 | S10_10_G40_10 | 40 | 5870 | 5920 |  |

Figure 2.1-1: All aggregated 10+10 configuration of SL intra-band NC CA for "NS_33"

Here, SEMfreq_-13A and SEfreq_-30A in Table 2.1-1 are addressed with similar approach as NR.

-	SEM-13,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.2.2 of TS38.101-1.

-	SEM-13,high = Threshold frequency where upper spectral emission mask above the upper channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.2.2 of TS38.101-1.

-	SEM-25,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -25 dBm / MHz to SE -30 dBm / MHz.

-	SEM-25,high = Threshold frequency where upper spectral emission mask above the upper channel drops from -25 dBm / MHz to SE -30 dBm / MHz.

-      SEM-37/100kHz,low = Threshold frequency where lower additional spectral emission mask below the lower channel corresponds to  -37dBm/100kHz, as specified in Clause 6.5E.2.3.1 of TS38.101-1.

-	SEM-37/100kHz,high = Threshold frequency where upper additional spectral emission mask above the upper channel corresponds to  -37dBm/100kHz, as specified in Clause 6.5E.2.3.1 of TS38.101-1.

-	Flow = 5855 MHz

-	Fhigh = 5925 MHz

SEMfreq_-13 = AND (FIM3,low_block,low > SEM-13,low,  FIM3,high_block,high < SEM-13,high )

SEfreq_-30  = OR(FIM3,low_block,low ≤ SEM-13,low ,  SEM-13,high ≤ FIM3,high_block,high )

SEMfreq_-13A = AND (FIM3,low_block,low > SEM-37,low , FIM3,high_block,high < SEM-37,high )

SEfreq_-30A = OR(FIM3,low_block,low ≤ MAX(SEM-13,low, Flow ),  MIN(SEM-13,high, Fhigh) ≤ FIM3,high_block,high )

Figure 2.1-2, Figure 2.1-3, Figure 2.1-4, and Figure 2.1-5 show the example of Frequency for FIM3,low_block,low & FIM3,high_block,high, such as, SEMfreq_-13/SEfreq_-30A, SEfreq_-30/SEfreq_-30A, SEMfreq_-13/SEMfreq-13A, SEfreq_-30/SEfreq_-30A respectively. Here, carrier frequency combinations of {5860, 5880MHz} and {5870, 5890} are assumed.

Figure 2.1-2: Example of SEMfreq_-13/SEfreq_-30A with {5860, 5880}MHz

Figure 2.1-3: Example of SEfreq_-30/SEfreq_-30A with {5860, 5880}MHz

Figure 2.1-4: Example of SEMfreq_-13/SEMfreq-13A with {5870, 5890}MHz

Figure 2.1-5: Example of SEfreq_-30/SEfreq_-30A with {5870, 5890}MHz

Proposal 1: Specify A-MPR requirements for NS_33 to meet -37dBm/100kHz and -30dBm/1MHz.

Proposal 2: Specify A-MPR requirements with and without indicating dualPA.

For A-MPR without indication dualPA, it can be applied with following RAN4#114 agreement for MPR without indication dualPA.

- For not indicating dualPA, specify MPR requirement with the agreed MPR for 2PA+1LO architecture.

Proposal 3: Specify A-MPR requirement without indicating dualPA with 2PA+1LO architecture.

PSSCH A-MPR (NS_33)

< PC3 PSSCH A-MPR >

For PSSCH A-MPR of SL non-contiguous CA, following evaluation scenarios in Table 2.2-2 are considered.

Table 2.2-2: SL non-contiguous CA AMPR evaluation scenarios (PSSCH)

| Aggregated 10+10 configuration | Scenario | CC1 | CC2 | B | Frequency of FIM3,low_block,low & FIM3,high_block,high | SCS1 (= SCS2) |
| --- | --- | --- | --- | --- | --- | --- |
| Conf#1, Conf#4 ~Conf#9 in Table 2.1-1 | 1 | 10RB0 | 10RB42 | 3.6 | SEfreq_-30A | 15 |
|  | 2 | 10RB30 | 10RB12 | 3.6 | SEfreq_-30A | 15 |
|  | 3 | 12RB0 | 12RB40 | 4.32 | SEfreq_-30A | 15 |
|  | 4 | 12RB30 | 12RB10 | 4.32 | SEfreq_-30A | 15 |
|  | 5 | 15RB0 | 15RB37 | 5.4 | SEfreq_-30A | 15 |
|  | 6 | 15RB30 | 15RB7 | 5.4 | SEfreq_-30A | 15 |
|  | 7 | 20RB0 | 20RB32 | 7.2 | SEfreq_-30A | 15 |
|  | 8 | 20RB30 | 20RB2 | 7.2 | SEfreq_-30A | 15 |
|  | 9 | 25RB0 | 25RB27 | 9.0 | SEfreq_-30A | 15 |
|  | 10 | 25RB27 | 25RB0 | 9.0 | SEfreq_-30A | 15 |
|  | 11 | 30RB0 | 30RB22 | 10.8 | SEfreq_-30A | 15 |
|  | 12 | 30RB22 | 30RB0 | 10.8 | SEfreq_-30A | 15 |
|  | 13 | 36RB0 | 36RB16 | 12.96 | SEfreq_-30A | 15 |
|  | 14 | 36RB16 | 36RB0 | 12.96 | SEfreq_-30A | 15 |
|  | 15 | 40RB0 | 40RB12 | 14.4 | SEfreq_-30A | 15 |
|  | 16 | 40RB12 | 40RB0 | 14.4 | SEfreq_-30A | 15 |
|  | 17 | 45RB0 | 45RB7 | 16.2 | SEfreq_-30A | 15 |
|  | 18 | 45RB7 | 45RB0 | 16.2 | SEfreq_-30A | 15 |
|  | 19 | 50RB0 | 50RB2 | 18.0 | SEfreq_-30A | 15 |
|  | 20 | 50RB2 | 50RB0 | 18.0 | SEfreq_-30A | 15 |
| Conf#2, Conf#3 in Table 2.1-1 | 1 | 10RB0 | 10RB42 | 3.6 | SEfreq_-30A | 15 |
|  | 2 | 10RB30 | 10RB12 | 3.6 | SEMfreq_-13A | 15 |
|  | 3 | 12RB0 | 12RB40 | 4.32 | SEfreq_-30A | 15 |
|  | 4 | 12RB30 | 12RB10 | 4.32 | SEMfreq_-13A | 15 |
|  | 5 | 15RB0 | 15RB37 | 5.4 | SEfreq_-30A | 15 |
|  | 6 | 15RB30 | 15RB7 | 5.4 | SEMfreq_-13A | 15 |
|  | 7 | 20RB0 | 20RB32 | 7.2 | SEfreq_-30A | 15 |
|  | 8 | 20RB30 | 20RB2 | 7.2 | SEMfreq_-13A | 15 |
|  | 9 | 25RB0 | 25RB27 | 9.0 | SEfreq_-30A | 15 |
|  | 10 | 25RB27 | 25RB0 | 9.0 | SEfreq_-30A | 15 |
|  | 11 | 30RB0 | 30RB22 | 10.8 | SEfreq_-30A | 15 |
|  | 12 | 30RB22 | 30RB0 | 10.8 | SEfreq_-30A | 15 |
|  | 13 | 36RB0 | 36RB16 | 12.96 | SEfreq_-30A | 15 |
|  | 14 | 36RB16 | 36RB0 | 12.96 | SEfreq_-30A | 15 |
|  | 15 | 40RB0 | 40RB12 | 14.4 | SEfreq_-30A | 15 |
|  | 16 | 40RB12 | 40RB0 | 14.4 | SEfreq_-30A | 15 |
|  | 17 | 45RB0 | 45RB7 | 16.2 | SEfreq_-30A | 15 |
|  | 18 | 45RB7 | 45RB0 | 16.2 | SEfreq_-30A | 15 |
|  | 19 | 50RB0 | 50RB2 | 18.0 | SEfreq_-30A | 15 |
|  | 20 | 50RB2 | 50RB0 | 18.0 | SEfreq_-30A | 15 |

Table 2.2-3 shows PC3 PA architecture.

Table 2.2-3: PC3 PA architecture

| Arch | description | Remarks |
| --- | --- | --- |
| #1-1 | 1x23dBm PA+ 1LO | Single–Tx |
| #1-2 | 2x20dBm PA+ 1LO | dual-Tx or txDiversity |
| #2-2 | 2x20dBm PA+ 2LO | dualPA-Architecture |

Table 2.2-4, Table 2.2-5, and Table 2.2-6 show the AMPR simulation results for the SL non-contiguous CA with architecture #1-1, #1-2, and #2-2 in Table 2.2-3 respectively.

Table 2.2-4: PSSCH/PSCCH AMPR simulation results for SL Non-contiguous CA with 1x23dBm+1LO

| S0_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 16.0 | 14.5 | 16.0 | 13.6 | 15.5 | 13.2 | 15.0 | 12.3 | 14.5 | 11.8 |
|  | '16QAM' | 16.4 | 14.1 | 16.0 | 13.6 | 16.0 | 13.2 | 15.0 | 12.3 | 14.5 | 11.8 |
|  | '64QAM' | 16.0 | 14.5 | 16.0 | 13.6 | 15.5 | 13.2 | 15.0 | 12.3 | 14.5 | 11.4 |
|  | '256QAM' | 16.5 | 14.1 | 16.0 | 13.6 | 15.5 | 13.2 | 15.0 | 12.3 | 14.5 | 11.8 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 14.1 | 11.3 | 13.6 | 10.4 | 13.7 | 10.4 | 13.2 | 10.9 | 12.7 | 11.8 |
|  | '16QAM' | 14.1 | 11.4 | 13.6 | 10.4 | 13.2 | 10.4 | 13.2 | 11.3 | 12.7 | 12.3 |
|  | '64QAM' | 14.1 | 10.9 | 13.6 | 10.4 | 13.2 | 10.4 | 13.2 | 11.3 | 12.7 | 12.3 |
|  | '256QAM' | 14.1 | 10.9 | 13.6 | 10.4 | 13.2 | 10.4 | 13.2 | 11.3 | 12.7 | 11.8 |
| S10_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 14.2 | 13.3 | 13.7 | 11.9 | 13.2 | 10.0 | 12.2 | 7.6 | 11.8 | 8.5 |
|  | '16QAM' | 14.2 | 13.3 | 13.7 | 11.9 | 13.2 | 10.0 | 12.2 | 7.6 | 11.8 | 9.0 |
|  | '64QAM' | 14.2 | 13.3 | 13.7 | 11.9 | 13.2 | 10.0 | 12.2 | 7.6 | 11.8 | 8.5 |
|  | '256QAM' | 14.2 | 13.3 | 13.7 | 11.9 | 13.2 | 10.0 | 12.2 | 7.6 | 11.8 | 8.5 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 11.3 | 10.4 | 10.9 | 10.9 | 10.4 | 10.4 | 9.9 | 9.9 | 9.5 | 9.5 |
|  | '16QAM' | 11.3 | 10.4 | 10.9 | 10.4 | 10.4 | 10.4 | 9.9 | 9.9 | 9.5 | 9.5 |
|  | '64QAM' | 11.3 | 10.4 | 10.4 | 10.9 | 10.4 | 10.4 | 9.9 | 9.9 | 9.5 | 9.5 |
|  | '256QAM' | 11.3 | 10.4 | 10.9 | 10.9 | 10.4 | 10.4 | 9.9 | 9.9 | 9.5 | 9.5 |
| S20_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 14.2 | 13.3 | 13.7 | 11.9 | 13.2 | 10.0 | 12.2 | 7.6 | 11.8 | 8.5 |
|  | '16QAM' | 14.2 | 13.3 | 13.7 | 11.9 | 13.2 | 10.0 | 12.2 | 7.6 | 11.8 | 8.5 |
|  | '64QAM' | 14.1 | 13.3 | 13.7 | 11.9 | 13.2 | 10.0 | 12.2 | 7.6 | 11.8 | 8.5 |
|  | '256QAM' | 14.2 | 13.3 | 13.7 | 11.9 | 13.2 | 10.0 | 12.2 | 7.6 | 11.8 | 8.5 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 11.3 | 10.4 | 10.4 | 10.4 | 10.4 | 10.4 | 9.9 | 9.9 | 9.5 | 9.5 |
|  | '16QAM' | 11.3 | 10.4 | 10.9 | 10.4 | 10.4 | 10.4 | 9.9 | 9.9 | 9.5 | 9.5 |
|  | '64QAM' | 11.3 | 10.4 | 10.9 | 10.4 | 10.4 | 10.4 | 9.9 | 9.9 | 9.5 | 9.5 |
|  | '256QAM' | 11.3 | 10.4 | 10.9 | 10.4 | 10.4 | 10.4 | 9.9 | 9.9 | 9.5 | 9.5 |
| S0_10_G20_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 16.0 | 14.5 | 16.0 | 14.0 | 15.0 | 13.1 | 15.0 | 12.2 | 14.5 | 11.8 |
|  | '16QAM' | 16.0 | 14.5 | 16.0 | 14.0 | 15.5 | 13.1 | 15.0 | 12.2 | 14.5 | 11.8 |
|  | '64QAM' | 16.0 | 14.5 | 16.0 | 14.0 | 15.5 | 13.1 | 15.0 | 12.2 | 14.5 | 11.8 |
|  | '256QAM' | 16.0 | 14.5 | 15.5 | 14.0 | 15.5 | 13.1 | 15.0 | 12.2 | 14.5 | 11.8 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 14.1 | 11.3 | 13.7 | 10.4 | 13.2 | 10.4 | 12.7 | 10.8 | 12.7 | 12.2 |
|  | '16QAM' | 14.1 | 11.3 | 13.7 | 10.4 | 13.2 | 10.4 | 12.7 | 10.8 | 12.7 | 11.8 |
|  | '64QAM' | 14.1 | 11.3 | 13.7 | 10.4 | 13.2 | 10.4 | 12.7 | 10.8 | 12.7 | 11.8 |
|  | '256QAM' | 14.1 | 11.3 | 13.6 | 10.4 | 13.2 | 10.4 | 13.2 | 10.8 | 12.7 | 11.8 |
| S10_10_G20_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 14.1 | 14.5 | 13.7 | 14.0 | 13.2 | 13.1 | 12.2 | 12.2 | 11.8 | 11.8 |
|  | '16QAM' | 14.2 | 14.0 | 13.7 | 14.0 | 13.2 | 13.1 | 12.2 | 12.2 | 11.8 | 11.8 |
|  | '64QAM' | 14.2 | 14.5 | 13.7 | 13.6 | 13.2 | 13.1 | 12.2 | 12.2 | 11.8 | 11.8 |
|  | '256QAM' | 14.1 | 14.5 | 13.7 | 14.0 | 13.2 | 13.1 | 12.2 | 12.2 | 11.8 | 11.8 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 11.3 | 11.3 | 10.4 | 10.4 | 10.4 | 10.4 | 9.9 | 9.9 | 9.5 | 9.5 |
|  | '16QAM' | 11.3 | 11.3 | 10.4 | 10.4 | 10.4 | 10.4 | 9.9 | 9.9 | 9.5 | 9.4 |
|  | '64QAM' | 11.3 | 11.3 | 10.4 | 10.9 | 10.4 | 10.4 | 9.9 | 9.9 | 9.5 | 9.4 |
|  | '256QAM' | 11.3 | 11.3 | 10.9 | 10.9 | 10.4 | 10.4 | 9.9 | 9.9 | 9.4 | 9.4 |
| S0_10_G30_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 15.5 | 14.1 | 15.5 | 13.7 | 15.0 | 13.2 | 14.5 | 12.3 | 14.5 | 11.4 |
|  | '16QAM' | 15.5 | 14.1 | 15.5 | 13.7 | 15.0 | 13.2 | 15.0 | 12.3 | 14.1 | 11.8 |
|  | '64QAM' | 15.5 | 14.1 | 15.1 | 13.7 | 15.0 | 13.2 | 14.5 | 12.3 | 14.1 | 11.4 |
|  | '256QAM' | 15.5 | 14.1 | 15.1 | 13.7 | 15.0 | 13.2 | 14.5 | 12.3 | 14.1 | 11.9 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 14.1 | 10.9 | 13.2 | 10.4 | 13.2 | 10.4 | 12.7 | 10.9 | 12.3 | 11.8 |
|  | '16QAM' | 14.1 | 10.9 | 13.6 | 10.4 | 13.2 | 10.4 | 12.7 | 10.9 | 12.7 | 11.8 |
|  | '64QAM' | 14.1 | 10.9 | 13.6 | 10.4 | 13.2 | 10.4 | 12.7 | 10.9 | 12.7 | 11.8 |
|  | '256QAM' | 13.6 | 10.9 | 13.2 | 10.4 | 13.2 | 10.4 | 12.7 | 10.9 | 12.7 | 11.8 |
| S10_10_G30_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 14.2 | 14.1 | 13.7 | 13.7 | 13.2 | 13.2 | 12.2 | 12.3 | 11.8 | 11.4 |
|  | '16QAM' | 14.2 | 14.1 | 13.7 | 13.7 | 13.2 | 13.2 | 12.2 | 12.3 | 11.8 | 11.4 |
|  | '64QAM' | 14.2 | 14.1 | 13.7 | 13.7 | 13.2 | 13.2 | 12.2 | 12.3 | 11.8 | 11.8 |
|  | '256QAM' | 14.2 | 14.1 | 13.7 | 13.7 | 13.2 | 13.2 | 12.2 | 12.3 | 11.8 | 11.4 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 11.3 | 10.9 | 10.9 | 10.4 | 10.4 | 10.4 | 9.9 | 9.9 | 9.5 | 9.5 |
|  | '16QAM' | 11.3 | 10.9 | 10.4 | 10.4 | 10.4 | 10.4 | 9.9 | 9.9 | 9.5 | 9.5 |
|  | '64QAM' | 11.3 | 10.9 | 10.4 | 10.4 | 10.4 | 10.4 | 9.9 | 9.9 | 9.5 | 9.5 |
|  | '256QAM' | 11.3 | 10.9 | 10.9 | 10.4 | 10.4 | 10.4 | 9.9 | 9.9 | 9.5 | 9.5 |
| S0_10_G40_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 16.0 | 14.2 | 16.0 | 13.7 | 15.5 | 12.8 | 15.0 | 12.3 | 14.5 | 11.8 |
|  | '16QAM' | 16.0 | 14.2 | 16.0 | 13.7 | 15.5 | 13.3 | 15.0 | 12.3 | 14.5 | 11.4 |
|  | '64QAM' | 16.0 | 14.2 | 16.0 | 13.7 | 15.5 | 13.3 | 15.0 | 12.3 | 14.6 | 11.4 |
|  | '256QAM' | 16.0 | 14.2 | 16.0 | 13.7 | 15.5 | 12.8 | 15.0 | 12.3 | 14.5 | 11.8 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 14.1 | 11.4 | 13.6 | 10.4 | 13.2 | 10.4 | 13.2 | 10.9 | 12.7 | 12.3 |
|  | '16QAM' | 14.1 | 10.9 | 13.6 | 10.4 | 13.6 | 10.4 | 13.2 | 10.9 | 12.7 | 12.3 |
|  | '64QAM' | 14.1 | 11.3 | 13.6 | 10.4 | 13.2 | 10.4 | 13.2 | 10.9 | 12.7 | 12.3 |
|  | '256QAM' | 14.1 | 10.9 | 13.6 | 10.9 | 13.2 | 10.4 | 13.2 | 10.9 | 12.7 | 11.8 |
| S0_10_G50_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 16.0 | 14.1 | 15.5 | 13.6 | 15.5 | 13.2 | 15.0 | 12.3 | 14.5 | 11.8 |
|  | '16QAM' | 15.5 | 14.5 | 15.5 | 13.6 | 15.5 | 13.2 | 15.0 | 12.3 | 14.1 | 11.8 |
|  | '64QAM' | 15.5 | 14.1 | 15.5 | 13.6 | 15.5 | 13.2 | 15.0 | 12.3 | 14.1 | 11.8 |
|  | '256QAM' | 15.5 | 14.1 | 15.5 | 13.6 | 15.5 | 13.2 | 15.0 | 12.3 | 14.5 | 11.8 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 13.6 | 10.9 | 13.6 | 10.4 | 13.2 | 10.4 | 12.7 | 10.9 | 12.7 | 11.8 |
|  | '16QAM' | 13.6 | 10.9 | 13.6 | 10.4 | 13.2 | 10.4 | 12.7 | 10.9 | 12.3 | 11.8 |
|  | '64QAM' | 14.1 | 10.9 | 13.6 | 10.4 | 13.2 | 10.4 | 12.7 | 10.9 | 12.3 | 11.8 |
|  | '256QAM' | 14.1 | 10.9 | 13.2 | 10.4 | 13.2 | 10.4 | 12.7 | 10.9 | 12.3 | 11.8 |

Table 2.2-5: PSSCH/PSCCH AMPR simulation results for SL Non-contiguous CA with 2x20dBm+1LO

| S0_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 17.7 | 15.3 | 17.7 | 14.4 | 17.2 | 14.0 | 16.2 | 13.1 | 15.8 | 12.2 |
|  | '16QAM' | 17.7 | 15.3 | 17.7 | 14.9 | 17.2 | 14.0 | 16.7 | 13.1 | 15.8 | 12.2 |
|  | '64QAM' | 17.7 | 15.3 | 17.7 | 14.4 | 17.2 | 14.0 | 16.7 | 13.1 | 15.8 | 12.2 |
|  | '256QAM' | 17.7 | 15.3 | 17.2 | 14.4 | 17.2 | 14.0 | 16.3 | 13.1 | 15.8 | 12.6 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 15.3 | 12.2 | 14.9 | 11.7 | 14.4 | 11.2 | 14.0 | 12.1 | 14.0 | 13.1 |
|  | '16QAM' | 15.3 | 11.7 | 14.9 | 11.7 | 14.4 | 11.2 | 14.0 | 12.1 | 14.0 | 13.1 |
|  | '64QAM' | 15.3 | 12.2 | 14.9 | 11.7 | 14.4 | 11.2 | 14.0 | 12.1 | 14.0 | 13.1 |
|  | '256QAM' | 15.3 | 11.7 | 14.9 | 11.7 | 14.4 | 11.2 | 14.4 | 12.1 | 14.0 | 13.1 |
| S10_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 14.9 | 14.1 | 14.5 | 12.7 | 14.0 | 10.8 | 13.0 | 8.4 | 12.6 | 9.8 |
|  | '16QAM' | 14.9 | 14.1 | 14.5 | 12.7 | 14.0 | 10.8 | 13.0 | 8.4 | 12.6 | 9.8 |
|  | '64QAM' | 15.0 | 14.1 | 14.5 | 12.7 | 14.0 | 10.8 | 13.0 | 8.4 | 12.6 | 9.8 |
|  | '256QAM' | 15.0 | 14.1 | 14.5 | 12.7 | 14.0 | 10.8 | 13.0 | 8.4 | 12.6 | 9.8 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 12.1 | 11.2 | 11.7 | 11.7 | 11.2 | 11.2 | 10.8 | 10.7 | 10.7 | 10.7 |
|  | '16QAM' | 12.1 | 11.2 | 11.7 | 11.7 | 11.2 | 11.2 | 10.8 | 10.7 | 10.7 | 10.8 |
|  | '64QAM' | 12.1 | 11.2 | 11.7 | 11.7 | 11.2 | 11.2 | 10.8 | 10.7 | 10.7 | 10.7 |
|  | '256QAM' | 12.1 | 11.2 | 11.7 | 11.7 | 11.2 | 11.2 | 10.8 | 10.7 | 10.7 | 10.8 |
| S20_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 15.0 | 14.1 | 14.5 | 12.7 | 14.0 | 10.8 | 13.0 | 8.4 | 12.6 | 9.8 |
|  | '16QAM' | 15.0 | 14.1 | 14.5 | 12.7 | 14.0 | 10.8 | 13.0 | 8.4 | 12.6 | 9.8 |
|  | '64QAM' | 15.0 | 14.1 | 14.5 | 12.7 | 14.0 | 10.8 | 13.0 | 8.4 | 12.6 | 9.8 |
|  | '256QAM' | 14.9 | 14.1 | 14.5 | 12.7 | 14.0 | 10.8 | 13.0 | 8.4 | 12.6 | 9.8 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 12.1 | 11.2 | 11.7 | 11.7 | 11.2 | 11.2 | 10.8 | 10.7 | 10.3 | 10.7 |
|  | '16QAM' | 12.1 | 11.2 | 11.7 | 11.7 | 11.2 | 11.2 | 10.8 | 10.7 | 10.7 | 10.7 |
|  | '64QAM' | 12.1 | 11.2 | 11.7 | 11.7 | 11.2 | 11.2 | 10.8 | 10.7 | 10.7 | 10.7 |
|  | '256QAM' | 12.1 | 11.2 | 11.7 | 11.2 | 11.2 | 11.2 | 10.7 | 10.7 | 10.7 | 10.8 |
| S0_10_G20_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 17.7 | 15.3 | 16.7 | 14.8 | 16.7 | 13.9 | 16.2 | 13.0 | 15.8 | 12.6 |
|  | '16QAM' | 17.7 | 15.3 | 17.2 | 14.8 | 16.7 | 13.9 | 16.2 | 13.0 | 15.8 | 12.6 |
|  | '64QAM' | 17.7 | 15.3 | 16.7 | 14.8 | 16.7 | 13.9 | 16.3 | 13.0 | 15.8 | 12.6 |
|  | '256QAM' | 17.7 | 15.3 | 17.2 | 14.8 | 16.7 | 13.9 | 16.2 | 13.0 | 15.8 | 12.6 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 15.3 | 12.1 | 14.9 | 11.7 | 14.4 | 11.2 | 14.0 | 12.1 | 14.0 | 13.0 |
|  | '16QAM' | 15.4 | 12.1 | 14.9 | 11.7 | 14.4 | 11.2 | 14.0 | 12.1 | 14.0 | 13.0 |
|  | '64QAM' | 14.9 | 12.1 | 14.9 | 11.7 | 14.4 | 11.2 | 14.0 | 12.1 | 14.0 | 13.0 |
|  | '256QAM' | 15.4 | 12.1 | 14.9 | 11.7 | 14.4 | 11.2 | 14.0 | 12.1 | 14.0 | 13.0 |
| S10_10_G20_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 14.9 | 15.3 | 14.5 | 14.8 | 14.0 | 13.9 | 13.0 | 13.0 | 12.6 | 12.6 |
|  | '16QAM' | 14.9 | 15.3 | 14.5 | 14.8 | 14.0 | 13.9 | 13.0 | 13.0 | 12.6 | 12.6 |
|  | '64QAM' | 14.9 | 15.3 | 14.4 | 14.8 | 14.0 | 13.9 | 13.0 | 13.0 | 12.6 | 12.6 |
|  | '256QAM' | 14.9 | 15.3 | 14.5 | 14.8 | 14.0 | 13.9 | 13.0 | 13.0 | 12.6 | 12.6 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 12.1 | 12.1 | 11.7 | 11.7 | 11.2 | 11.2 | 10.8 | 10.7 | 10.7 | 10.7 |
|  | '16QAM' | 12.1 | 12.1 | 11.7 | 11.7 | 11.2 | 11.2 | 10.8 | 10.7 | 10.7 | 10.7 |
|  | '64QAM' | 12.1 | 12.1 | 11.7 | 11.7 | 11.2 | 11.2 | 10.7 | 10.7 | 10.7 | 10.7 |
|  | '256QAM' | 12.1 | 12.1 | 11.7 | 11.7 | 11.2 | 11.2 | 10.8 | 10.7 | 10.7 | 10.7 |
| S0_10_G30_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 16.8 | 14.9 | 16.8 | 14.5 | 16.3 | 14.0 | 15.8 | 13.1 | 15.8 | 12.2 |
|  | '16QAM' | 17.2 | 14.9 | 16.8 | 14.5 | 16.3 | 14.0 | 15.8 | 13.1 | 15.3 | 12.2 |
|  | '64QAM' | 17.3 | 14.9 | 16.3 | 14.5 | 16.3 | 14.0 | 16.3 | 13.1 | 15.3 | 12.2 |
|  | '256QAM' | 17.3 | 14.9 | 16.8 | 14.5 | 16.8 | 14.0 | 15.8 | 13.1 | 15.3 | 12.6 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 15.3 | 12.2 | 14.9 | 11.2 | 14.4 | 11.2 | 14.0 | 11.7 | 13.5 | 13.1 |
|  | '16QAM' | 14.9 | 11.7 | 14.4 | 11.7 | 14.0 | 11.2 | 14.0 | 11.7 | 13.5 | 12.6 |
|  | '64QAM' | 14.9 | 12.2 | 14.4 | 11.2 | 14.4 | 11.2 | 14.0 | 11.7 | 13.5 | 12.6 |
|  | '256QAM' | 14.9 | 12.2 | 14.9 | 11.7 | 14.4 | 11.2 | 14.0 | 12.1 | 13.5 | 13.1 |
| S10_10_G30_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 15.0 | 14.9 | 14.5 | 14.5 | 14.0 | 14.0 | 13.0 | 13.1 | 12.6 | 12.2 |
|  | '16QAM' | 14.9 | 14.9 | 14.5 | 14.5 | 14.0 | 14.0 | 13.0 | 13.1 | 12.6 | 12.2 |
|  | '64QAM' | 15.0 | 14.9 | 14.5 | 14.5 | 14.0 | 14.0 | 13.0 | 13.1 | 12.6 | 12.2 |
|  | '256QAM' | 15.0 | 14.9 | 14.5 | 14.5 | 14.0 | 14.0 | 13.1 | 13.1 | 12.6 | 12.2 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 12.1 | 11.7 | 11.7 | 11.2 | 11.2 | 11.2 | 10.7 | 10.7 | 10.3 | 10.8 |
|  | '16QAM' | 12.1 | 11.7 | 11.7 | 11.7 | 11.2 | 11.2 | 10.8 | 10.7 | 10.3 | 10.3 |
|  | '64QAM' | 12.1 | 12.2 | 11.7 | 11.7 | 11.2 | 11.2 | 10.8 | 10.7 | 10.8 | 10.8 |
|  | '256QAM' | 12.1 | 11.7 | 11.7 | 11.2 | 11.2 | 11.2 | 10.8 | 10.7 | 10.7 | 10.8 |
| S0_10_G40_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 17.7 | 15.0 | 16.8 | 14.5 | 16.8 | 13.6 | 16.3 | 13.1 | 15.8 | 12.6 |
|  | '16QAM' | 17.7 | 15.0 | 17.2 | 14.5 | 16.8 | 13.6 | 16.3 | 13.1 | 15.8 | 12.2 |
|  | '64QAM' | 17.7 | 15.0 | 17.2 | 14.5 | 16.8 | 13.6 | 16.3 | 13.1 | 15.8 | 12.2 |
|  | '256QAM' | 17.7 | 15.0 | 17.2 | 14.5 | 16.8 | 14.1 | 16.3 | 13.1 | 15.8 | 12.7 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 15.3 | 12.2 | 14.9 | 11.7 | 14.4 | 11.2 | 14.0 | 12.1 | 14.0 | 13.1 |
|  | '16QAM' | 15.3 | 12.2 | 14.9 | 11.7 | 14.4 | 11.2 | 14.0 | 12.1 | 14.0 | 13.1 |
|  | '64QAM' | 15.3 | 11.7 | 14.9 | 11.7 | 14.4 | 11.2 | 14.0 | 12.1 | 14.0 | 13.1 |
|  | '256QAM' | 15.3 | 12.2 | 14.9 | 11.7 | 14.4 | 11.2 | 14.0 | 12.1 | 14.0 | 13.1 |
| S0_10_G50_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 17.2 | 15.3 | 17.2 | 14.4 | 16.7 | 14.0 | 15.8 | 13.1 | 15.8 | 12.6 |
|  | '16QAM' | 17.2 | 15.3 | 16.8 | 14.4 | 16.7 | 14.0 | 16.2 | 13.1 | 15.8 | 12.6 |
|  | '64QAM' | 17.2 | 15.3 | 16.8 | 14.8 | 16.7 | 14.0 | 16.2 | 13.1 | 15.3 | 12.6 |
|  | '256QAM' | 16.8 | 15.3 | 16.8 | 14.4 | 16.7 | 14.0 | 15.8 | 13.1 | 15.8 | 12.6 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 14.9 | 11.7 | 14.4 | 11.7 | 14.4 | 11.2 | 14.0 | 11.7 | 13.5 | 12.6 |
|  | '16QAM' | 14.9 | 11.7 | 14.9 | 11.7 | 14.4 | 11.2 | 14.0 | 11.7 | 14.0 | 13.1 |
|  | '64QAM' | 14.9 | 11.7 | 14.4 | 11.7 | 14.0 | 11.2 | 14.0 | 11.7 | 13.5 | 13.1 |
|  | '256QAM' | 14.9 | 11.7 | 14.4 | 11.7 | 14.4 | 11.2 | 14.0 | 11.7 | 13.5 | 13.1 |

Table 2.2-6: PSSCH/PSCCH AMPR simulation results for SL Non-contiguous CA with 2x20dBm+2LO

| S0_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 13.7 | 9.0 | 14.1 | 8.0 | 14.1 | 7.5 | 13.7 | 6.6 | 13.2 | 6.1 |
|  | '16QAM' | 14.1 | 9.0 | 14.1 | 8.0 | 13.6 | 7.5 | 13.7 | 6.6 | 13.2 | 6.1 |
|  | '64QAM' | 14.1 | 9.0 | 14.1 | 8.0 | 13.6 | 7.1 | 13.7 | 6.6 | 13.2 | 6.1 |
|  | '256QAM' | 14.1 | 9.0 | 14.1 | 8.0 | 13.6 | 7.5 | 13.7 | 6.6 | 13.2 | 6.1 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 12.7 | 6.1 | 12.3 | 7.1 | 11.8 | 8.5 | 11.8 | 9.9 | 11.3 | 10.9 |
|  | '16QAM' | 12.7 | 6.1 | 12.3 | 7.1 | 11.8 | 8.5 | 11.8 | 9.9 | 11.3 | 10.9 |
|  | '64QAM' | 12.7 | 6.1 | 12.3 | 7.1 | 11.8 | 8.5 | 11.8 | 9.9 | 11.3 | 10.9 |
|  | '256QAM' | 12.7 | 6.1 | 12.3 | 7.1 | 11.8 | 8.5 | 11.8 | 9.9 | 11.3 | 10.8 |
| S10_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 9.5 | 8.1 | 8.0 | 6.6 | 7.6 | 5.7 | 6.6 | 4.8 | 6.1 | 5.2 |
|  | '16QAM' | 9.0 | 8.1 | 8.0 | 6.6 | 7.1 | 5.7 | 6.6 | 4.8 | 6.2 | 5.2 |
|  | '64QAM' | 9.0 | 8.1 | 8.0 | 6.6 | 7.1 | 5.7 | 7.1 | 4.8 | 6.2 | 5.2 |
|  | '256QAM' | 9.0 | 8.1 | 8.0 | 6.6 | 7.6 | 5.7 | 6.6 | 4.8 | 6.1 | 5.2 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 6.1 | 5.7 | 5.7 | 5.7 | 5.7 | 5.7 | 5.2 | 5.2 | 5.2 | 5.2 |
|  | '16QAM' | 6.1 | 5.7 | 5.7 | 5.7 | 5.7 | 5.7 | 5.2 | 5.2 | 5.2 | 5.2 |
|  | '64QAM' | 6.1 | 5.7 | 5.7 | 5.7 | 5.7 | 5.7 | 5.2 | 5.7 | 5.2 | 5.2 |
|  | '256QAM' | 6.1 | 5.7 | 5.7 | 5.7 | 5.7 | 5.7 | 5.2 | 5.2 | 5.2 | 5.2 |
| S20_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 9.0 | 8.1 | 8.0 | 6.6 | 7.1 | 5.7 | 6.6 | 4.8 | 6.1 | 5.2 |
|  | '16QAM' | 9.5 | 8.1 | 8.0 | 6.6 | 7.6 | 5.7 | 6.6 | 4.8 | 6.1 | 5.2 |
|  | '64QAM' | 9.0 | 8.1 | 8.0 | 6.6 | 7.6 | 5.7 | 6.6 | 4.8 | 6.1 | 5.2 |
|  | '256QAM' | 9.5 | 8.1 | 8.0 | 6.6 | 7.6 | 5.7 | 6.6 | 4.8 | 6.2 | 5.2 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 6.1 | 5.7 | 5.7 | 5.7 | 5.7 | 5.7 | 5.2 | 5.2 | 5.2 | 5.2 |
|  | '16QAM' | 6.1 | 5.7 | 5.7 | 5.7 | 5.7 | 5.7 | 5.2 | 5.2 | 5.2 | 5.2 |
|  | '64QAM' | 6.1 | 5.7 | 5.7 | 5.7 | 5.7 | 5.7 | 5.2 | 5.7 | 5.2 | 5.2 |
|  | '256QAM' | 6.1 | 5.7 | 5.7 | 5.7 | 5.7 | 5.7 | 5.2 | 5.2 | 5.2 | 5.2 |
| S0_10_G20_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 14.6 | 9.4 | 14.6 | 8.0 | 14.1 | 7.5 | 13.7 | 6.6 | 13.6 | 6.1 |
|  | '16QAM' | 14.6 | 9.4 | 14.6 | 8.0 | 14.1 | 7.5 | 13.7 | 6.6 | 13.6 | 6.1 |
|  | '64QAM' | 14.6 | 9.0 | 14.6 | 8.0 | 14.1 | 7.5 | 13.7 | 6.6 | 13.2 | 6.1 |
|  | '256QAM' | 14.6 | 9.0 | 14.6 | 8.0 | 14.1 | 7.5 | 13.7 | 6.6 | 13.6 | 6.1 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 13.2 | 6.1 | 12.7 | 7.1 | 12.3 | 8.5 | 11.8 | 9.9 | 11.8 | 10.9 |
|  | '16QAM' | 13.2 | 6.1 | 12.7 | 7.1 | 12.3 | 9.0 | 11.8 | 9.9 | 11.8 | 10.8 |
|  | '64QAM' | 13.2 | 6.1 | 12.7 | 7.1 | 12.3 | 8.5 | 11.8 | 9.9 | 11.8 | 10.9 |
|  | '256QAM' | 13.2 | 6.1 | 12.7 | 7.1 | 12.3 | 8.5 | 11.8 | 9.9 | 11.8 | 10.8 |
| S10_10_G20_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 9.5 | 9.4 | 8.0 | 8.0 | 7.6 | 7.1 | 6.6 | 6.6 | 6.2 | 6.1 |
|  | '16QAM' | 9.0 | 9.0 | 8.0 | 8.0 | 7.6 | 7.5 | 6.6 | 7.1 | 6.2 | 6.1 |
|  | '64QAM' | 9.5 | 9.5 | 8.0 | 8.0 | 7.6 | 7.5 | 6.6 | 6.6 | 6.1 | 6.1 |
|  | '256QAM' | 9.5 | 9.0 | 8.0 | 8.0 | 7.6 | 7.5 | 6.6 | 6.6 | 6.2 | 6.1 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 6.1 | 6.1 | 5.7 | 5.7 | 5.7 | 5.7 | 5.2 | 5.2 | 5.2 | 5.2 |
|  | '16QAM' | 6.1 | 6.1 | 5.7 | 5.7 | 5.7 | 5.7 | 5.2 | 5.2 | 5.2 | 5.2 |
|  | '64QAM' | 6.1 | 6.1 | 5.7 | 5.7 | 5.7 | 5.7 | 5.2 | 5.2 | 5.2 | 5.2 |
|  | '256QAM' | 6.1 | 6.1 | 5.7 | 5.7 | 5.7 | 5.7 | 5.2 | 5.2 | 5.2 | 5.2 |
| S0_10_G30_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 12.7 | 9.4 | 13.2 | 8.0 | 13.2 | 7.5 | 12.7 | 6.6 | 12.7 | 6.1 |
|  | '16QAM' | 12.7 | 9.0 | 13.2 | 8.0 | 13.2 | 7.5 | 12.7 | 6.6 | 12.7 | 6.1 |
|  | '64QAM' | 12.7 | 9.4 | 13.2 | 8.0 | 13.2 | 7.5 | 12.7 | 6.6 | 12.7 | 6.1 |
|  | '256QAM' | 12.7 | 9.4 | 12.7 | 8.0 | 13.2 | 7.5 | 12.8 | 7.1 | 12.7 | 6.1 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 12.3 | 6.1 | 11.8 | 6.6 | 11.3 | 8.0 | 11.3 | 9.4 | 10.9 | 10.4 |
|  | '16QAM' | 12.3 | 6.1 | 11.8 | 6.6 | 11.3 | 8.0 | 11.3 | 9.4 | 10.9 | 10.4 |
|  | '64QAM' | 12.3 | 6.1 | 11.8 | 6.6 | 11.3 | 8.0 | 11.3 | 9.4 | 10.9 | 10.4 |
|  | '256QAM' | 12.3 | 6.1 | 11.8 | 6.6 | 11.3 | 8.0 | 11.3 | 9.4 | 10.9 | 10.4 |
| S10_10_G30_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 9.0 | 9.4 | 8.0 | 8.0 | 7.6 | 7.5 | 6.6 | 6.6 | 6.2 | 6.1 |
|  | '16QAM' | 9.0 | 9.0 | 8.0 | 8.0 | 7.6 | 7.5 | 6.6 | 6.6 | 6.6 | 6.1 |
|  | '64QAM' | 9.0 | 9.4 | 8.5 | 8.0 | 7.6 | 7.1 | 6.6 | 6.6 | 6.1 | 6.1 |
|  | '256QAM' | 9.5 | 9.5 | 8.1 | 8.0 | 7.6 | 7.5 | 6.6 | 6.6 | 6.2 | 6.1 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 6.1 | 6.1 | 5.7 | 5.7 | 5.7 | 5.7 | 5.2 | 5.2 | 5.2 | 5.2 |
|  | '16QAM' | 6.1 | 6.1 | 5.7 | 5.7 | 5.7 | 5.7 | 5.2 | 5.2 | 5.2 | 5.2 |
|  | '64QAM' | 6.1 | 6.1 | 5.7 | 5.7 | 5.7 | 5.7 | 5.2 | 5.2 | 5.2 | 5.2 |
|  | '256QAM' | 6.1 | 6.1 | 5.7 | 5.7 | 5.7 | 5.7 | 5.2 | 5.2 | 5.2 | 5.2 |
| S0_10_G40_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 13.7 | 9.4 | 13.2 | 8.0 | 13.2 | 7.5 | 13.2 | 6.6 | 12.7 | 6.1 |
|  | '16QAM' | 13.2 | 9.4 | 13.6 | 8.0 | 13.6 | 7.5 | 13.2 | 6.6 | 12.7 | 6.1 |
|  | '64QAM' | 13.2 | 9.5 | 13.7 | 8.0 | 13.2 | 7.5 | 13.2 | 7.1 | 12.7 | 6.1 |
|  | '256QAM' | 13.7 | 9.4 | 13.6 | 8.0 | 13.2 | 7.5 | 13.2 | 6.6 | 12.7 | 6.1 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 12.3 | 6.1 | 11.8 | 6.6 | 11.8 | 8.5 | 11.3 | 9.9 | 11.3 | 10.4 |
|  | '16QAM' | 12.7 | 6.1 | 11.8 | 6.6 | 11.8 | 8.0 | 11.3 | 9.9 | 10.9 | 10.9 |
|  | '64QAM' | 12.2 | 6.1 | 11.8 | 6.6 | 11.8 | 8.0 | 11.3 | 9.9 | 10.9 | 10.4 |
|  | '256QAM' | 12.2 | 6.1 | 11.8 | 6.6 | 11.8 | 8.0 | 11.3 | 9.9 | 10.9 | 10.8 |
| S0_10_G50_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 13.7 | 9.0 | 13.7 | 8.0 | 13.6 | 7.5 | 13.2 | 6.6 | 12.7 | 6.1 |
|  | '16QAM' | 13.7 | 9.0 | 13.7 | 8.0 | 13.6 | 7.5 | 13.7 | 6.6 | 13.2 | 6.1 |
|  | '64QAM' | 13.7 | 8.9 | 13.6 | 8.0 | 13.6 | 7.5 | 13.7 | 6.6 | 13.2 | 6.1 |
|  | '256QAM' | 13.7 | 9.5 | 13.6 | 8.0 | 13.6 | 7.5 | 13.7 | 6.6 | 13.2 | 6.1 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 12.7 | 6.1 | 12.3 | 6.6 | 11.8 | 8.5 | 11.8 | 9.9 | 11.3 | 10.8 |
|  | '16QAM' | 12.7 | 6.1 | 12.3 | 6.6 | 11.8 | 8.5 | 11.8 | 9.9 | 11.3 | 10.8 |
|  | '64QAM' | 12.7 | 6.1 | 12.3 | 6.6 | 11.8 | 8.5 | 11.3 | 9.9 | 11.3 | 10.8 |
|  | '256QAM' | 12.7 | 6.1 | 12.3 | 6.6 | 11.8 | 8.5 | 11.3 | 9.9 | 11.3 | 10.8 |

From Table 2.2-4, Table 2.2-5, and Table 2.2-6, AMPR is almost same for all modulation order. Same AMPR can be defined regardless of modulation order. And, the simulation results can be categorized with 2 cases because the result of each case is very similar.

  - Case 1 : S0_10_G10_10, S0_10_G20_10, S0_10_G30_10, S0_10_G40_10, S0_10_G50_10

  - Case 2 : S10_10_G10_10, S20_10_G10_10, S10_10_G20_10, S10_10_G30_10

Here,

Case 1 corresponds to that any Carrier Frequency in 2CC combination is either 5860MHz or 5920MHz, as follows.

  - {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920}

Case 2 corresponds to that any Carrier Frequency in 2CC combination is other than either 5860MHz or 5920MHz, as follows.

  - {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910}

Or,

If any Carrier Frequency in 2CC combination is either 5860MHz or 5920MHz

Case 1

Else

Case 2

Table 2.2-7, Table 2.2-8, and Table 2.2-9 show the maximum value of simulation results for SL non-contiguous CA with architecture #1-1, #1-2, and #2-2 respectively.

Here,

SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}.

SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}.

Table 2.2-7 : PSSCH/PSCCH AMPR simulation results for SL non-contiguous CA with 1x23dBm PA + 1LO

| Carrier Frequency Combination | B | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
| [MHz] |  | SEMfreq_-13/-13A | SEfreq_-30/-30A |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 0 ≤ B < 5.04 | 14.5 | 16.5 |
|  | 5.04 ≤ B < 10.08 | 13.2 | 16.0 |
|  | 10.08 ≤ B |  | 14.1 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 0 ≤ B < 5.04 | 13.3 | 14.5 |
|  | 5.04 ≤ B < 10.08 | 10.0 | 13.2 |
|  | 10.08 ≤ B |  | 11.3 |

Table 2.2-8 : PSSCH/PSCCH AMPR simulation results for SL non-contiguous CA with 2x20dBm PA + 1LO

| Carrier Frequency Combination | B | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
| [MHz] |  | SEMfreq_-13/-13A | SEfreq_-30/-30A |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 0 ≤ B < 5.04 | 15.3 | 17.7 |
|  | 5.04 ≤ B < 10.08 | 14.0 | 17.2 |
|  | 10.08 ≤ B |  | 15.4 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 0 ≤ B < 5.04 | 14.1 | 15.3 |
|  | 5.04 ≤ B < 10.08 | 10.8 | 14.0 |
|  | 10.08 ≤ B |  | 12.2 |

Table 2.2-9 : PSSCH/PSCCH AMPR simulation results for SL non-contiguous CA with 2x20dBm PA + 2LO

| Carrier Frequency Combination | B | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
| [MHz] |  | SEMfreq_-13/-13A | SEfreq_-30/-30A |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 0 ≤ B < 5.04 | 9.0 | 14.6 |
|  | 5.04 ≤ B < 10.08 | 7.5 | 14.1 |
|  | 10.08 ≤ B |  | 13.2 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 0 ≤ B < 5.04 | 8.1 | 9.5 |
|  | 5.04 ≤ B < 10.08 | 5.7 | 7.6 |
|  | 10.08 ≤ B |  | 6.1 |

The AMPR can be proposed as Table 2.2-10, Table 2.2-11, and Table 2.2-12 based on the simulation results when considering implementation margin.

Table 2.2-10 : PSSCH/PSCCH AMPR for SL non-contiguous CA with 1x23dBm PA + 1LO

| Carrier Frequency Combination | B | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
| [MHz] |  | SEMfreq_-13/-13A | SEfreq_-30/-30A |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 0 ≤ B < 5.04 | ≤ 17.5 | ≤ 19.5 |
|  | 5.04 ≤ B < 10.08 | ≤ 16.0 | ≤ 19.0 |
|  | 10.08 ≤ B |  | ≤ 17.0 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 0 ≤ B < 5.04 | ≤ 16.5 | ≤ 17.5 |
|  | 5.04 ≤ B < 10.08 | ≤ 13.0 | ≤ 16.0 |
|  | 10.08 ≤ B |  | ≤ 15.0 |
| NOTE : SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}. SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}. |  |  |  |

Table 2.2-11 : PSSCH/PSCCH AMPR for SL non-contiguous CA with 2x20dBm PA + 1LO

| Carrier Frequency Combination | B | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
| [MHz] |  | SEMfreq_-13/-13A | SEfreq_-30/-30A |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 0 ≤ B < 5.04 | ≤ 18.0 | ≤ 20.0 |
|  | 5.04 ≤ B < 10.08 | ≤ 16.5 | ≤ 19.5 |
|  | 10.08 ≤ B |  | ≤ 18.0 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 0 ≤ B < 5.04 | ≤ 17.0 | ≤ 18.0 |
|  | 5.04 ≤ B < 10.08 | ≤ 13.5 | ≤ 16.5 |
|  | 10.08 ≤ B |  | ≤ 15.5 |
| NOTE : SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}. SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}. |  |  |  |

Table 2.2-12 : PSSCH/PSCCH AMPR for SL non-contiguous CA with 2x20dBm PA + 2LO

| Carrier Frequency Combination | B | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
| [MHz] |  | SEMfreq_-13/-13A | SEfreq_-30/-30A |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 0 ≤ B < 5.04 | ≤ 13.5 | ≤ 18.0 |
|  | 5.04 ≤ B < 10.08 | ≤ 12.5 | ≤ 17.5 |
|  | 10.08 ≤ B |  | ≤ 16.5 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 0 ≤ B < 5.04 | ≤ 13.0 | ≤ 14.0 |
|  | 5.04 ≤ B < 10.08 | ≤ 10.5 | ≤ 12.5 |
|  | 10.08 ≤ B |  | ≤ 11.5 |
| NOTE : SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}. SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}. |  |  |  |

Considering proposal 2 and 3, PC3 PSSCH/PSCCH A-MPR can be proposed as follows.

Proposal 4: For PC3 PSSCH/PSCCH A-MPR without indicating dualPA, consider A-MPR in Table 2.2-11.

Proposal 5: For PC3 PSSCH/PSCCH A-MPR with indicating dualPA, consider A-MPR in Table 2.2-12.

< PC2 PSSCH A-MPR >

Table 2.2-13 shows PC2 PA architecture.

Table 2.2-13: PC2 PA architecture

| Arch | description | Remarks |
| --- | --- | --- |
| #1-1 | 1x26dBm PA+ 1LO | Single–Tx |
| #1-2 | 2x23dBm PA+ 1LO | dual-Tx or txDiversity |
| #2-2 | 2x23dBm PA+ 2LO | dualPA-Architecture |

Table 2.2-14, Table 2.2-15, and Table 2.2-16 show the AMPR simulation results for the SL non-contiguous CA with architecture #1-1, #1-2, and #2-2 in Table 2.2-13 respectively.

Table 2.2-14: PSSCH/PSCCH AMPR simulation results for SL Non-contiguous CA with 1x26dBm+1LO

| S0_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 19.4 | 16.5 | 18.9 | 15.6 | 18.9 | 15.2 | 17.9 | 14.3 | 17.0 | 13.8 |
|  | '16QAM' | 19.4 | 16.5 | 18.9 | 15.6 | 18.4 | 15.2 | 17.9 | 14.3 | 17.5 | 13.8 |
|  | '64QAM' | 19.4 | 16.5 | 18.9 | 15.6 | 18.4 | 15.2 | 17.9 | 14.3 | 17.5 | 13.8 |
|  | '256QAM' | 19.4 | 16.5 | 18.9 | 15.6 | 18.4 | 15.2 | 17.9 | 14.3 | 17.0 | 13.4 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 17.0 | 12.9 | 16.6 | 12.9 | 16.1 | 12.4 | 15.2 | 13.3 | 15.2 | 14.3 |
|  | '16QAM' | 17.0 | 12.9 | 16.1 | 12.9 | 15.6 | 12.4 | 15.2 | 13.3 | 15.2 | 14.3 |
|  | '64QAM' | 17.0 | 12.9 | 16.1 | 12.9 | 16.1 | 12.4 | 15.7 | 13.3 | 15.2 | 14.3 |
|  | '256QAM' | 17.0 | 12.9 | 16.6 | 12.9 | 16.1 | 12.4 | 15.6 | 13.3 | 15.2 | 14.3 |
| S10_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 16.2 | 15.3 | 15.7 | 13.9 | 15.2 | 12.0 | 14.3 | 9.7 | 13.8 | 11.1 |
|  | '16QAM' | 16.2 | 15.3 | 15.7 | 13.9 | 15.2 | 12.0 | 14.3 | 9.7 | 13.8 | 11.1 |
|  | '64QAM' | 16.2 | 15.3 | 15.7 | 13.9 | 15.2 | 12.0 | 14.3 | 9.7 | 13.8 | 11.1 |
|  | '256QAM' | 16.2 | 15.3 | 15.7 | 13.9 | 15.2 | 12.0 | 14.3 | 9.7 | 13.8 | 11.1 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 13.3 | 12.4 | 12.9 | 12.9 | 12.4 | 12.4 | 12.0 | 12.0 | 12.0 | 12.0 |
|  | '16QAM' | 13.3 | 12.5 | 12.9 | 12.4 | 12.4 | 12.4 | 12.0 | 12.0 | 12.0 | 12.0 |
|  | '64QAM' | 13.3 | 12.4 | 12.9 | 12.9 | 12.4 | 12.4 | 12.0 | 12.0 | 12.0 | 12.0 |
|  | '256QAM' | 13.3 | 12.5 | 12.9 | 12.9 | 12.4 | 12.4 | 12.0 | 12.0 | 12.0 | 11.5 |
| S20_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 16.2 | 15.3 | 15.7 | 13.9 | 15.2 | 12.0 | 14.3 | 9.7 | 13.8 | 11.1 |
|  | '16QAM' | 16.2 | 15.3 | 15.7 | 13.9 | 15.2 | 12.0 | 14.3 | 9.7 | 13.8 | 11.1 |
|  | '64QAM' | 16.2 | 15.3 | 15.7 | 13.9 | 15.2 | 12.0 | 14.3 | 9.7 | 13.8 | 11.1 |
|  | '256QAM' | 16.2 | 15.3 | 15.7 | 13.9 | 15.2 | 12.0 | 14.3 | 9.7 | 13.8 | 11.1 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 13.3 | 12.5 | 12.9 | 12.9 | 12.4 | 12.4 | 12.0 | 12.0 | 11.5 | 11.5 |
|  | '16QAM' | 13.3 | 12.4 | 12.9 | 12.9 | 12.4 | 12.4 | 12.0 | 12.0 | 12.0 | 12.0 |
|  | '64QAM' | 13.3 | 12.5 | 12.9 | 12.9 | 12.4 | 12.4 | 12.0 | 12.0 | 12.0 | 12.0 |
|  | '256QAM' | 13.3 | 12.4 | 12.9 | 12.9 | 12.4 | 12.4 | 12.0 | 12.0 | 12.0 | 12.0 |
| S0_10_G20_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 18.9 | 16.5 | 18.9 | 16.0 | 18.4 | 15.1 | 17.9 | 14.2 | 17.5 | 13.8 |
|  | '16QAM' | 18.9 | 16.5 | 18.9 | 16.0 | 18.4 | 15.6 | 17.9 | 14.2 | 17.0 | 13.8 |
|  | '64QAM' | 18.9 | 16.5 | 18.9 | 16.0 | 17.9 | 15.6 | 17.9 | 14.2 | 17.0 | 13.8 |
|  | '256QAM' | 18.9 | 16.5 | 18.9 | 16.0 | 17.9 | 15.1 | 17.9 | 14.2 | 17.0 | 13.8 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 16.6 | 13.4 | 16.1 | 12.9 | 15.7 | 12.4 | 15.2 | 13.3 | 15.2 | 14.3 |
|  | '16QAM' | 16.6 | 13.4 | 16.1 | 12.9 | 15.7 | 12.4 | 15.2 | 13.3 | 15.2 | 14.3 |
|  | '64QAM' | 17.0 | 13.4 | 16.1 | 12.9 | 16.1 | 12.4 | 15.2 | 13.3 | 15.2 | 14.3 |
|  | '256QAM' | 16.6 | 13.4 | 16.1 | 12.4 | 15.7 | 12.4 | 15.7 | 13.3 | 15.2 | 14.3 |
| S10_10_G20_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 16.2 | 16.5 | 15.7 | 16.0 | 15.2 | 15.1 | 14.2 | 14.2 | 13.8 | 13.8 |
|  | '16QAM' | 16.2 | 16.5 | 15.7 | 16.0 | 15.2 | 15.1 | 14.2 | 14.2 | 13.8 | 13.8 |
|  | '64QAM' | 16.2 | 16.5 | 15.7 | 16.0 | 15.2 | 15.6 | 14.3 | 14.2 | 13.8 | 13.8 |
|  | '256QAM' | 16.2 | 16.5 | 15.7 | 16.0 | 15.2 | 15.1 | 14.7 | 14.2 | 13.8 | 13.8 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 13.4 | 13.4 | 12.9 | 12.9 | 12.4 | 12.4 | 12.0 | 11.9 | 12.0 | 12.0 |
|  | '16QAM' | 13.4 | 13.4 | 12.9 | 12.9 | 12.4 | 12.4 | 12.0 | 11.9 | 12.0 | 12.0 |
|  | '64QAM' | 13.3 | 13.4 | 12.9 | 12.9 | 12.4 | 12.4 | 12.0 | 11.9 | 12.0 | 12.0 |
|  | '256QAM' | 13.4 | 13.4 | 12.9 | 12.9 | 12.4 | 12.4 | 12.0 | 11.9 | 11.5 | 12.0 |
| S0_10_G30_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 18.0 | 16.1 | 18.0 | 15.7 | 18.0 | 15.2 | 17.5 | 14.3 | 17.0 | 13.4 |
|  | '16QAM' | 18.5 | 16.6 | 18.4 | 15.7 | 18.0 | 15.2 | 17.5 | 14.3 | 17.0 | 13.4 |
|  | '64QAM' | 18.5 | 16.1 | 18.0 | 15.7 | 18.0 | 15.2 | 17.5 | 14.3 | 17.0 | 13.4 |
|  | '256QAM' | 18.5 | 16.1 | 18.0 | 15.7 | 18.0 | 15.2 | 17.5 | 14.3 | 17.0 | 13.4 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 16.5 | 12.9 | 16.1 | 12.4 | 15.7 | 12.4 | 15.2 | 13.4 | 15.2 | 14.3 |
|  | '16QAM' | 16.1 | 12.9 | 16.1 | 12.4 | 15.7 | 12.4 | 15.2 | 13.4 | 14.7 | 14.3 |
|  | '64QAM' | 16.5 | 12.9 | 16.1 | 12.4 | 15.7 | 12.4 | 15.2 | 13.4 | 14.7 | 14.3 |
|  | '256QAM' | 16.5 | 12.9 | 16.1 | 12.9 | 15.6 | 12.4 | 15.2 | 12.9 | 15.2 | 14.3 |
| S10_10_G30_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 16.2 | 16.1 | 15.7 | 15.7 | 15.2 | 15.2 | 14.3 | 14.3 | 13.8 | 13.4 |
|  | '16QAM' | 16.2 | 16.1 | 15.7 | 15.7 | 15.2 | 15.2 | 14.3 | 14.3 | 13.8 | 13.9 |
|  | '64QAM' | 16.2 | 16.1 | 15.7 | 15.7 | 15.2 | 15.2 | 14.3 | 14.3 | 13.8 | 13.4 |
|  | '256QAM' | 16.2 | 16.6 | 15.7 | 15.7 | 15.2 | 15.2 | 14.3 | 14.3 | 13.8 | 13.4 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 13.3 | 12.9 | 12.9 | 12.9 | 12.4 | 12.4 | 12.0 | 12.0 | 12.0 | 12.0 |
|  | '16QAM' | 13.3 | 12.9 | 12.9 | 12.9 | 12.4 | 12.4 | 12.0 | 12.0 | 11.5 | 12.0 |
|  | '64QAM' | 13.3 | 12.9 | 12.9 | 12.9 | 12.4 | 12.4 | 12.0 | 12.0 | 12.0 | 11.5 |
|  | '256QAM' | 13.3 | 12.9 | 12.9 | 12.9 | 12.4 | 12.4 | 12.0 | 12.0 | 11.5 | 11.5 |
| S0_10_G40_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 18.9 | 16.2 | 18.9 | 15.8 | 18.5 | 15.3 | 17.9 | 14.3 | 17.0 | 13.4 |
|  | '16QAM' | 18.9 | 16.2 | 18.9 | 15.8 | 18.5 | 15.3 | 18.0 | 14.3 | 17.0 | 13.9 |
|  | '64QAM' | 19.4 | 16.2 | 18.9 | 15.7 | 18.4 | 14.8 | 17.9 | 14.3 | 17.0 | 13.9 |
|  | '256QAM' | 18.9 | 16.2 | 18.9 | 15.7 | 18.0 | 14.8 | 18.0 | 14.3 | 17.0 | 13.4 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 17.0 | 12.9 | 16.1 | 12.9 | 15.6 | 12.4 | 15.6 | 13.4 | 15.2 | 14.3 |
|  | '16QAM' | 17.0 | 12.9 | 16.1 | 12.9 | 16.1 | 12.4 | 15.6 | 13.4 | 15.2 | 14.3 |
|  | '64QAM' | 17.0 | 13.4 | 16.1 | 12.9 | 15.6 | 12.4 | 15.6 | 13.4 | 15.2 | 14.3 |
|  | '256QAM' | 16.5 | 13.4 | 16.1 | 12.9 | 16.1 | 12.4 | 15.2 | 13.4 | 15.2 | 14.3 |
| S0_10_G50_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 18.9 | 16.5 | 18.4 | 15.6 | 18.0 | 15.2 | 17.5 | 14.3 | 17.0 | 13.8 |
|  | '16QAM' | 18.9 | 16.5 | 18.4 | 15.6 | 18.0 | 15.2 | 17.5 | 14.3 | 17.0 | 13.8 |
|  | '64QAM' | 18.9 | 16.5 | 18.4 | 15.6 | 18.4 | 15.2 | 17.5 | 14.3 | 17.0 | 13.8 |
|  | '256QAM' | 18.5 | 16.5 | 18.4 | 16.1 | 17.9 | 15.2 | 17.5 | 14.3 | 17.0 | 13.4 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 16.6 | 13.4 | 16.1 | 12.9 | 15.7 | 12.4 | 15.2 | 12.9 | 14.7 | 14.3 |
|  | '16QAM' | 16.5 | 12.9 | 16.1 | 12.4 | 15.2 | 12.4 | 15.2 | 12.9 | 14.7 | 14.3 |
|  | '64QAM' | 16.6 | 12.9 | 16.1 | 12.4 | 16.1 | 12.4 | 15.2 | 12.9 | 14.7 | 14.3 |
|  | '256QAM' | 16.6 | 12.9 | 16.1 | 12.9 | 15.7 | 12.4 | 15.2 | 12.9 | 14.7 | 14.3 |

Table 2.2-15: PSSCH/PSCCH AMPR simulation results for SL Non-contiguous CA with 2x23dBm+1LO

| S0_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 19.7 | 16.8 | 19.2 | 16.4 | 19.2 | 15.5 | 18.7 | 14.6 | 17.8 | 14.1 |
|  | '16QAM' | 19.7 | 16.8 | 19.2 | 16.4 | 19.2 | 15.5 | 18.2 | 14.6 | 18.2 | 14.1 |
|  | '64QAM' | 20.2 | 17.3 | 19.7 | 16.4 | 19.2 | 15.5 | 18.7 | 14.6 | 17.8 | 14.1 |
|  | '256QAM' | 20.2 | 17.3 | 19.2 | 16.4 | 19.2 | 15.5 | 18.3 | 14.6 | 17.8 | 14.2 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 17.3 | 13.7 | 16.9 | 13.2 | 16.4 | 12.7 | 16.0 | 13.7 | 15.5 | 14.6 |
|  | '16QAM' | 17.3 | 13.7 | 16.9 | 13.2 | 16.4 | 12.7 | 16.0 | 13.6 | 16.0 | 14.6 |
|  | '64QAM' | 17.3 | 13.7 | 16.9 | 13.2 | 16.4 | 12.7 | 16.4 | 13.7 | 15.5 | 15.0 |
|  | '256QAM' | 17.3 | 13.7 | 16.9 | 13.2 | 16.4 | 12.7 | 16.0 | 13.7 | 15.5 | 15.1 |
| S10_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 16.5 | 15.6 | 16.5 | 14.2 | 15.5 | 12.3 | 15.0 | 10.0 | 14.1 | 11.8 |
|  | '16QAM' | 16.9 | 15.6 | 16.5 | 14.2 | 15.5 | 12.3 | 15.0 | 10.0 | 14.1 | 11.8 |
|  | '64QAM' | 16.9 | 15.6 | 16.5 | 14.2 | 15.5 | 12.3 | 14.6 | 10.0 | 14.1 | 11.8 |
|  | '256QAM' | 16.9 | 15.6 | 16.5 | 14.2 | 15.5 | 12.3 | 14.6 | 10.0 | 14.1 | 11.8 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 13.6 | 13.2 | 13.2 | 13.2 | 12.8 | 12.7 | 12.8 | 12.7 | 12.3 | 12.3 |
|  | '16QAM' | 13.7 | 13.2 | 13.2 | 13.2 | 12.8 | 12.7 | 12.7 | 12.7 | 12.3 | 12.3 |
|  | '64QAM' | 13.6 | 13.2 | 13.2 | 13.2 | 12.8 | 12.7 | 12.8 | 12.7 | 12.3 | 12.3 |
|  | '256QAM' | 13.7 | 13.2 | 13.2 | 13.2 | 12.8 | 12.7 | 12.8 | 12.7 | 12.3 | 12.3 |
| S20_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 16.9 | 15.6 | 16.5 | 14.2 | 15.5 | 12.3 | 15.0 | 10.0 | 14.1 | 11.8 |
|  | '16QAM' | 16.9 | 15.6 | 16.4 | 14.2 | 15.5 | 12.3 | 15.0 | 10.0 | 14.1 | 11.8 |
|  | '64QAM' | 16.9 | 15.6 | 16.5 | 14.2 | 15.5 | 12.3 | 15.0 | 10.0 | 14.1 | 11.8 |
|  | '256QAM' | 16.5 | 15.6 | 16.0 | 14.2 | 15.5 | 12.3 | 15.0 | 10.0 | 14.1 | 11.8 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 13.6 | 13.2 | 13.2 | 13.2 | 12.8 | 12.7 | 12.8 | 12.7 | 12.3 | 12.3 |
|  | '16QAM' | 13.6 | 13.2 | 13.2 | 13.2 | 12.8 | 12.7 | 12.8 | 12.7 | 12.3 | 12.3 |
|  | '64QAM' | 13.6 | 13.2 | 13.2 | 13.2 | 12.8 | 12.7 | 12.8 | 12.7 | 12.3 | 12.3 |
|  | '256QAM' | 13.6 | 13.2 | 13.2 | 13.2 | 12.8 | 12.7 | 12.8 | 12.7 | 12.3 | 12.3 |
| S0_10_G20_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 19.7 | 17.2 | 19.2 | 16.3 | 18.7 | 15.9 | 18.2 | 15.0 | 17.8 | 14.1 |
|  | '16QAM' | 19.7 | 17.3 | 19.2 | 16.3 | 18.7 | 15.4 | 18.2 | 15.0 | 17.3 | 14.1 |
|  | '64QAM' | 19.7 | 17.3 | 19.2 | 16.3 | 19.2 | 15.9 | 18.2 | 15.0 | 17.8 | 14.1 |
|  | '256QAM' | 19.2 | 17.2 | 19.2 | 16.3 | 19.2 | 15.9 | 18.2 | 15.0 | 17.3 | 14.1 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 17.3 | 13.7 | 16.9 | 13.2 | 16.4 | 12.7 | 16.0 | 13.6 | 15.5 | 14.6 |
|  | '16QAM' | 17.3 | 13.7 | 16.9 | 13.2 | 16.4 | 12.7 | 16.0 | 13.6 | 15.5 | 14.6 |
|  | '64QAM' | 16.9 | 13.7 | 16.4 | 13.2 | 16.0 | 12.7 | 16.0 | 13.6 | 15.5 | 14.6 |
|  | '256QAM' | 17.3 | 13.7 | 16.9 | 13.2 | 16.4 | 12.7 | 16.0 | 13.6 | 15.5 | 14.6 |
| S10_10_G20_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 16.9 | 17.2 | 16.4 | 16.3 | 15.5 | 15.9 | 15.0 | 15.0 | 14.1 | 14.1 |
|  | '16QAM' | 16.9 | 17.2 | 16.5 | 16.3 | 15.5 | 15.9 | 15.0 | 14.6 | 14.1 | 14.1 |
|  | '64QAM' | 16.9 | 17.2 | 16.4 | 16.3 | 15.5 | 15.9 | 15.0 | 15.0 | 14.1 | 14.1 |
|  | '256QAM' | 16.9 | 17.2 | 16.4 | 16.3 | 15.5 | 15.9 | 15.0 | 15.0 | 14.1 | 14.1 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 13.7 | 13.7 | 13.2 | 13.2 | 12.8 | 12.7 | 12.7 | 12.7 | 12.3 | 12.3 |
|  | '16QAM' | 13.7 | 13.7 | 13.2 | 13.2 | 12.8 | 12.7 | 12.8 | 12.7 | 12.3 | 12.3 |
|  | '64QAM' | 13.7 | 13.7 | 13.2 | 13.2 | 12.7 | 12.7 | 12.7 | 12.7 | 12.3 | 12.3 |
|  | '256QAM' | 13.7 | 13.7 | 13.2 | 13.2 | 12.7 | 12.7 | 12.7 | 12.7 | 12.3 | 12.3 |
| S0_10_G30_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 19.3 | 16.9 | 18.3 | 16.5 | 18.7 | 15.5 | 18.2 | 14.6 | 17.3 | 14.2 |
|  | '16QAM' | 18.8 | 16.9 | 18.3 | 16.4 | 18.3 | 15.5 | 17.8 | 14.6 | 17.3 | 14.2 |
|  | '64QAM' | 19.2 | 16.9 | 18.8 | 16.5 | 18.7 | 15.5 | 17.8 | 14.6 | 17.3 | 14.2 |
|  | '256QAM' | 18.8 | 16.9 | 18.3 | 16.4 | 18.3 | 15.5 | 18.2 | 14.6 | 17.3 | 14.2 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 16.9 | 13.7 | 16.9 | 13.2 | 16.4 | 12.7 | 15.5 | 13.7 | 15.1 | 14.6 |
|  | '16QAM' | 16.9 | 13.7 | 16.4 | 13.2 | 16.4 | 12.7 | 15.5 | 13.7 | 15.5 | 14.6 |
|  | '64QAM' | 16.9 | 13.7 | 16.4 | 13.2 | 16.0 | 12.7 | 16.0 | 13.7 | 15.5 | 14.6 |
|  | '256QAM' | 16.9 | 13.7 | 16.4 | 13.2 | 16.0 | 12.7 | 15.5 | 13.7 | 15.5 | 14.6 |
| S10_10_G30_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 16.9 | 16.9 | 16.5 | 16.5 | 15.5 | 15.5 | 14.6 | 14.6 | 14.1 | 14.2 |
|  | '16QAM' | 16.9 | 16.9 | 16.5 | 16.5 | 15.5 | 15.5 | 15.0 | 14.6 | 14.1 | 14.2 |
|  | '64QAM' | 16.9 | 16.9 | 16.5 | 16.5 | 15.5 | 15.5 | 14.6 | 14.6 | 14.1 | 14.2 |
|  | '256QAM' | 16.9 | 16.9 | 16.5 | 16.5 | 15.5 | 15.6 | 15.0 | 14.6 | 14.1 | 14.2 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 13.6 | 13.7 | 13.2 | 13.2 | 12.7 | 12.7 | 12.8 | 12.7 | 12.3 | 12.3 |
|  | '16QAM' | 13.7 | 13.7 | 13.2 | 13.2 | 12.7 | 12.7 | 12.8 | 12.7 | 12.3 | 12.3 |
|  | '64QAM' | 13.6 | 13.7 | 13.2 | 13.2 | 12.8 | 12.7 | 12.8 | 12.7 | 12.3 | 12.3 |
|  | '256QAM' | 13.6 | 13.7 | 13.2 | 13.2 | 12.7 | 12.7 | 12.7 | 12.7 | 12.3 | 12.3 |
| S0_10_G40_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 19.7 | 16.5 | 19.2 | 16.1 | 18.8 | 15.6 | 18.3 | 14.7 | 17.8 | 14.2 |
|  | '16QAM' | 19.7 | 17.0 | 19.2 | 16.1 | 19.2 | 15.6 | 18.2 | 14.6 | 17.3 | 14.2 |
|  | '64QAM' | 19.7 | 16.5 | 19.2 | 16.1 | 19.2 | 15.6 | 18.3 | 14.7 | 17.3 | 14.2 |
|  | '256QAM' | 19.7 | 17.0 | 19.2 | 16.0 | 19.2 | 15.6 | 18.3 | 14.7 | 17.8 | 14.2 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 17.3 | 13.7 | 16.8 | 13.2 | 16.4 | 12.7 | 16.0 | 13.7 | 15.5 | 15.1 |
|  | '16QAM' | 17.3 | 13.7 | 16.9 | 13.2 | 16.4 | 12.7 | 16.0 | 13.7 | 15.5 | 15.1 |
|  | '64QAM' | 17.3 | 13.7 | 16.9 | 13.2 | 16.4 | 12.7 | 16.0 | 13.7 | 15.5 | 14.6 |
|  | '256QAM' | 17.3 | 13.7 | 16.8 | 13.2 | 16.4 | 12.7 | 16.0 | 13.7 | 15.5 | 15.1 |
| S0_10_G50_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 19.2 | 16.8 | 18.8 | 16.4 | 18.7 | 15.5 | 18.2 | 14.6 | 17.3 | 14.1 |
|  | '16QAM' | 19.2 | 16.9 | 18.8 | 16.4 | 18.7 | 15.5 | 18.2 | 14.6 | 17.3 | 14.1 |
|  | '64QAM' | 19.2 | 17.3 | 18.7 | 16.4 | 18.7 | 15.5 | 18.2 | 15.1 | 17.3 | 14.2 |
|  | '256QAM' | 19.2 | 16.8 | 18.7 | 16.4 | 19.2 | 15.5 | 18.2 | 14.6 | 17.3 | 14.1 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 16.9 | 13.7 | 16.4 | 13.2 | 16.0 | 12.7 | 15.5 | 13.7 | 15.5 | 14.6 |
|  | '16QAM' | 16.9 | 13.7 | 16.4 | 13.2 | 16.0 | 12.7 | 15.5 | 13.7 | 15.5 | 14.6 |
|  | '64QAM' | 16.9 | 13.7 | 16.4 | 13.2 | 16.0 | 12.7 | 15.5 | 13.7 | 15.5 | 14.6 |
|  | '256QAM' | 16.9 | 13.7 | 16.4 | 13.2 | 16.4 | 12.7 | 16.0 | 13.7 | 15.5 | 14.6 |

Table 2.2-16: PSSCH/PSCCH AMPR simulation results for SL Non-contiguous CA with 2x23dBm+2LO

| S0_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 15.6 | 11.5 | 15.6 | 10.5 | 15.6 | 9.1 | 15.2 | 8.6 | 14.7 | 7.7 |
|  | '16QAM' | 16.1 | 11.5 | 15.6 | 10.5 | 15.6 | 9.6 | 15.2 | 8.6 | 14.7 | 7.7 |
|  | '64QAM' | 15.7 | 11.5 | 15.6 | 10.5 | 15.6 | 9.1 | 15.2 | 8.6 | 14.7 | 7.7 |
|  | '256QAM' | 16.1 | 11.5 | 15.6 | 10.5 | 15.6 | 9.1 | 15.2 | 8.6 | 14.7 | 7.7 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 14.2 | 7.7 | 14.2 | 9.1 | 13.8 | 10.5 | 13.3 | 11.9 | 13.3 | 12.8 |
|  | '16QAM' | 14.7 | 7.7 | 14.3 | 9.1 | 13.8 | 10.5 | 13.3 | 11.9 | 13.3 | 12.4 |
|  | '64QAM' | 14.7 | 7.7 | 14.3 | 9.1 | 13.8 | 10.5 | 13.8 | 11.9 | 13.3 | 12.4 |
|  | '256QAM' | 14.2 | 7.7 | 14.2 | 9.1 | 13.8 | 10.5 | 13.8 | 11.9 | 13.3 | 12.9 |
| S10_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 12.0 | 10.6 | 10.5 | 8.7 | 9.6 | 7.2 | 8.7 | 6.3 | 7.7 | 6.7 |
|  | '16QAM' | 11.5 | 10.6 | 10.5 | 8.7 | 9.1 | 7.2 | 8.7 | 6.3 | 7.7 | 6.7 |
|  | '64QAM' | 12.0 | 10.6 | 10.5 | 8.7 | 9.6 | 7.2 | 8.6 | 6.3 | 7.7 | 6.7 |
|  | '256QAM' | 11.9 | 10.6 | 10.5 | 8.7 | 9.1 | 7.2 | 8.7 | 6.3 | 7.7 | 6.7 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 7.7 | 7.2 | 7.2 | 7.2 | 6.7 | 7.2 | 6.7 | 6.7 | 6.7 | 6.7 |
|  | '16QAM' | 7.2 | 7.2 | 7.2 | 7.2 | 6.7 | 7.2 | 6.7 | 6.7 | 6.7 | 6.7 |
|  | '64QAM' | 7.2 | 7.2 | 7.2 | 7.2 | 6.7 | 7.2 | 6.7 | 6.7 | 6.7 | 6.7 |
|  | '256QAM' | 7.7 | 7.2 | 7.2 | 7.2 | 6.8 | 7.2 | 6.7 | 6.7 | 6.7 | 6.7 |
| S20_10_G10_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 12.0 | 10.6 | 10.5 | 8.7 | 9.1 | 7.2 | 8.7 | 6.3 | 7.7 | 6.7 |
|  | '16QAM' | 12.0 | 10.6 | 10.5 | 8.7 | 9.1 | 7.2 | 8.7 | 6.3 | 7.7 | 6.7 |
|  | '64QAM' | 12.0 | 10.6 | 10.5 | 8.7 | 9.6 | 7.2 | 8.6 | 6.3 | 7.7 | 6.7 |
|  | '256QAM' | 11.5 | 10.6 | 10.5 | 8.7 | 9.6 | 7.2 | 8.7 | 6.3 | 7.7 | 6.7 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 7.7 | 7.2 | 7.2 | 7.2 | 6.7 | 7.2 | 6.7 | 6.7 | 6.7 | 6.7 |
|  | '16QAM' | 7.2 | 7.2 | 7.2 | 7.2 | 7.2 | 7.2 | 6.7 | 6.7 | 6.7 | 6.7 |
|  | '64QAM' | 7.7 | 7.2 | 7.2 | 7.2 | 6.8 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 |
|  | '256QAM' | 7.2 | 7.2 | 7.2 | 7.2 | 6.7 | 7.2 | 6.7 | 6.7 | 6.7 | 6.7 |
| S0_10_G20_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 16.6 | 11.9 | 16.1 | 10.5 | 16.1 | 9.1 | 15.6 | 8.6 | 15.2 | 7.7 |
|  | '16QAM' | 16.6 | 11.9 | 16.1 | 10.5 | 16.1 | 9.1 | 15.6 | 8.6 | 15.2 | 7.7 |
|  | '64QAM' | 16.6 | 11.4 | 16.1 | 10.5 | 16.1 | 9.1 | 15.6 | 8.6 | 15.2 | 7.7 |
|  | '256QAM' | 16.6 | 11.9 | 16.1 | 10.5 | 16.1 | 9.6 | 15.6 | 8.6 | 15.2 | 7.7 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 14.7 | 7.7 | 14.2 | 9.1 | 14.3 | 10.5 | 13.8 | 11.9 | 13.3 | 12.8 |
|  | '16QAM' | 14.7 | 7.7 | 14.2 | 9.1 | 13.8 | 10.5 | 13.8 | 11.9 | 13.3 | 12.8 |
|  | '64QAM' | 14.7 | 7.7 | 14.2 | 9.1 | 14.2 | 10.5 | 13.8 | 11.9 | 13.3 | 12.8 |
|  | '256QAM' | 14.7 | 7.7 | 14.3 | 9.1 | 14.2 | 10.5 | 13.8 | 11.9 | 13.3 | 12.8 |
| S10_10_G20_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 12.0 | 11.9 | 10.5 | 10.5 | 9.6 | 9.6 | 8.7 | 8.6 | 7.7 | 7.7 |
|  | '16QAM' | 12.0 | 11.5 | 10.5 | 10.5 | 9.1 | 9.1 | 8.7 | 8.6 | 7.7 | 7.7 |
|  | '64QAM' | 11.5 | 11.9 | 10.5 | 10.5 | 9.6 | 9.1 | 8.7 | 8.6 | 7.7 | 7.7 |
|  | '256QAM' | 12.0 | 11.9 | 10.5 | 10.5 | 9.6 | 9.5 | 8.2 | 8.6 | 7.7 | 7.7 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 7.2 | 7.2 | 7.2 | 7.2 | 7.2 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 |
|  | '16QAM' | 7.7 | 7.7 | 7.2 | 7.2 | 6.7 | 7.2 | 6.7 | 6.7 | 6.7 | 6.7 |
|  | '64QAM' | 7.7 | 7.7 | 7.2 | 7.2 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 |
|  | '256QAM' | 7.7 | 7.7 | 7.2 | 7.2 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 |
| S0_10_G30_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 14.7 | 11.9 | 14.7 | 10.5 | 14.7 | 9.1 | 14.7 | 8.6 | 14.3 | 7.7 |
|  | '16QAM' | 14.7 | 11.9 | 14.7 | 10.5 | 14.7 | 9.1 | 14.7 | 8.6 | 14.3 | 7.7 |
|  | '64QAM' | 14.7 | 11.5 | 14.7 | 10.5 | 14.7 | 9.5 | 14.7 | 8.6 | 14.3 | 7.7 |
|  | '256QAM' | 14.7 | 11.9 | 14.7 | 10.5 | 14.7 | 9.6 | 14.7 | 8.6 | 14.3 | 7.7 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 13.8 | 7.7 | 13.8 | 8.6 | 13.3 | 10.0 | 12.9 | 11.4 | 12.9 | 12.4 |
|  | '16QAM' | 14.2 | 7.7 | 13.8 | 8.6 | 13.3 | 10.0 | 12.8 | 11.4 | 12.9 | 12.4 |
|  | '64QAM' | 13.8 | 7.7 | 13.8 | 8.6 | 13.3 | 10.0 | 12.9 | 11.4 | 12.9 | 12.4 |
|  | '256QAM' | 13.8 | 7.7 | 13.3 | 8.6 | 13.3 | 10.0 | 12.9 | 11.5 | 12.9 | 12.4 |
| S10_10_G30_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 12.0 | 11.9 | 10.5 | 10.5 | 9.6 | 9.1 | 8.7 | 8.6 | 7.7 | 7.7 |
|  | '16QAM' | 12.0 | 11.9 | 10.5 | 10.5 | 9.6 | 9.6 | 8.7 | 8.6 | 7.7 | 7.7 |
|  | '64QAM' | 12.0 | 11.5 | 10.5 | 10.5 | 9.6 | 9.6 | 8.7 | 8.6 | 7.7 | 7.7 |
|  | '256QAM' | 12.0 | 11.9 | 10.5 | 10.5 | 9.6 | 9.1 | 8.6 | 8.6 | 7.7 | 7.7 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 7.7 | 7.7 | 7.2 | 7.2 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 |
|  | '16QAM' | 7.7 | 7.7 | 7.2 | 7.2 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 |
|  | '64QAM' | 7.7 | 7.7 | 7.2 | 7.2 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 |
|  | '256QAM' | 7.7 | 7.2 | 7.2 | 7.2 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 |
| S0_10_G40_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 15.2 | 11.9 | 15.2 | 10.5 | 15.2 | 9.6 | 14.7 | 8.6 | 14.7 | 7.7 |
|  | '16QAM' | 15.2 | 11.9 | 15.2 | 10.5 | 15.2 | 9.1 | 14.7 | 8.6 | 14.7 | 7.7 |
|  | '64QAM' | 15.2 | 11.9 | 15.2 | 10.5 | 15.2 | 9.5 | 15.2 | 8.6 | 14.7 | 7.7 |
|  | '256QAM' | 15.2 | 11.9 | 15.2 | 10.5 | 15.2 | 9.6 | 14.7 | 8.6 | 14.7 | 7.7 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 14.2 | 7.7 | 13.8 | 8.6 | 13.3 | 10.5 | 13.3 | 11.4 | 12.9 | 12.4 |
|  | '16QAM' | 14.2 | 7.7 | 13.8 | 8.6 | 13.3 | 10.5 | 13.3 | 11.4 | 12.9 | 12.4 |
|  | '64QAM' | 14.2 | 7.6 | 13.8 | 8.6 | 13.3 | 10.5 | 13.3 | 11.4 | 12.9 | 12.4 |
|  | '256QAM' | 14.2 | 7.7 | 13.8 | 8.6 | 13.3 | 10.5 | 13.3 | 11.4 | 12.9 | 12.4 |
| S0_10_G50_10 | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |
|  | 'QPSK' | 15.7 | 11.9 | 15.6 | 10.5 | 15.2 | 9.6 | 15.2 | 8.6 | 14.7 | 7.7 |
|  | '16QAM' | 15.6 | 11.5 | 15.6 | 10.5 | 15.6 | 9.1 | 15.2 | 8.6 | 14.7 | 7.7 |
|  | '64QAM' | 15.6 | 11.4 | 15.6 | 10.5 | 15.6 | 9.6 | 15.2 | 8.6 | 14.7 | 7.7 |
|  | '256QAM' | 15.7 | 11.5 | 15.6 | 10.5 | 15.6 | 9.1 | 15.2 | 8.6 | 14.7 | 7.7 |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |
|  | 'QPSK' | 14.2 | 7.7 | 14.2 | 8.6 | 13.8 | 10.5 | 13.3 | 11.4 | 13.3 | 12.4 |
|  | '16QAM' | 14.2 | 7.2 | 14.2 | 8.6 | 13.8 | 10.5 | 13.3 | 11.4 | 13.3 | 12.4 |
|  | '64QAM' | 14.2 | 7.7 | 14.3 | 8.6 | 13.8 | 10.5 | 13.3 | 11.5 | 12.9 | 12.4 |
|  | '256QAM' | 14.2 | 7.7 | 14.3 | 8.6 | 13.8 | 10.5 | 13.3 | 11.9 | 13.3 | 12.4 |

Table 2.2-17, Table 2.2-18, and Table 2.2-19 show the maximum value of simulation results.

Here,

SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}.

SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}.

Table 2.2-17 : PSSCH/PSCCH AMPR simulation results for SL non-contiguous CA with 1x26dBm PA + 1LO

| Carrier Frequency Combination | B | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
| [MHz] |  | SEMfreq_-13/-13A | SEfreq_-30/-30A |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 0 ≤ B < 5.04 | 16.5 | 19.4 |
|  | 5.04 ≤ B < 10.08 | 15.2 | 18.9 |
|  | 10.08 ≤ B |  | 17.0 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 0 ≤ B < 5.04 | 15.3 | 16.6 |
|  | 5.04 ≤ B < 10.08 | 12.0 | 15.6 |
|  | 10.08 ≤ B |  | 13.4 |

Table 2.2-18 : PSSCH/PSCCH AMPR simulation results for SL non-contiguous CA with 2x23dBm PA + 1LO

| Carrier Frequency Combination | B | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
| [MHz] |  | SEMfreq_-13/-13A | SEfreq_-30/-30A |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 0 ≤ B < 5.04 | 17.3 | 20.2 |
|  | 5.04 ≤ B < 10.08 | 15.5 | 19.2 |
|  | 10.08 ≤ B |  | 17.3 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 0 ≤ B < 5.04 | 15.6 | 17.2 |
|  | 5.04 ≤ B < 10.08 | 12.3 | 15.9 |
|  | 10.08 ≤ B |  | 13.7 |

Table 2.2-19 : PSSCH/PSCCH AMPR simulation results for SL non-contiguous CA with 2x23dBm PA + 2LO

| Carrier Frequency Combination | B | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
| [MHz] |  | SEMfreq_-13/-13A | SEfreq_-30/-30A |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 0 ≤ B < 5.04 | 11.5 | 16.6 |
|  | 5.04 ≤ B < 10.08 | 9.6 | 16.1 |
|  | 10.08 ≤ B |  | 14.7 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 0 ≤ B < 5.04 | 10.6 | 12.0 |
|  | 5.04 ≤ B < 10.08 | 7.2 | 9.6 |
|  | 10.08 ≤ B |  | 7.7 |

The AMPR can be proposed as Table 2.2-20, Table 2.2-21, and Table 2.2-22 based on the simulation results when considering implementation margin.

Table 2.2-20 : PSSCH/PSCCH AMPR for SL non-contiguous CA with 1x26dBm PA + 1LO

| Carrier Frequency Combination | B | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
| [MHz] |  | SEMfreq_-13/-13A | SEfreq_-30/-30A |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 0 ≤ B < 5.04 | ≤ 19.0 | ≤ 22.0 |
|  | 5.04 ≤ B < 10.08 | ≤ 17.5 | ≤ 21.5 |
|  | 10.08 ≤ B |  | ≤ 19.5 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 0 ≤ B < 5.04 | ≤ 18.0 | ≤ 19.0 |
|  | 5.04 ≤ B < 10.08 | ≤ 14.5 | ≤ 18.0 |
|  | 10.08 ≤ B |  | ≤ 16.0 |
| NOTE : SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}. SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}. |  |  |  |

Table 2.2-21 : PSSCH/PSCCH AMPR for SL non-contiguous CA with 2x23dBm PA + 1LO

| Carrier Frequency Combination | B | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
| [MHz] |  | SEMfreq_-13/-13A | SEfreq_-30/-30A |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 0 ≤ B < 5.04 | ≤ 20.0 | ≤ 22.5 |
|  | 5.04 ≤ B < 10.08 | ≤ 18.5 | ≤ 22.0 |
|  | 10.08 ≤ B |  | ≤ 20.0 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 0 ≤ B < 5.04 | ≤ 18.5 | ≤ 19.5 |
|  | 5.04 ≤ B < 10.08 | ≤ 15.0 | ≤ 18.5 |
|  | 10.08 ≤ B |  | ≤ 16.5 |
| NOTE : SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}. SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}. |  |  |  |

Table 2.2-22 : PSSCH/PSCCH AMPR for SL non-contiguous CA with 2x23dBm PA + 2LO

| Carrier Frequency Combination | B | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
| [MHz] |  | SEMfreq_-13/-13A | SEfreq_-30/-30A |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 0 ≤ B < 5.04 | ≤ 15.0 | ≤ 20.0 |
|  | 5.04 ≤ B < 10.08 | ≤ 13.0 | ≤ 19.5 |
|  | 10.08 ≤ B |  | ≤ 18.0 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 0 ≤ B < 5.04 | ≤ 14.0 | ≤ 15.5 |
|  | 5.04 ≤ B < 10.08 | ≤ 11.5 | ≤ 13.0 |
|  | 10.08 ≤ B |  | ≤ 12.0 |
| NOTE : SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}. SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}. |  |  |  |

Considering proposal 2 and 3, PC2 PSSCH/PSCCH A-MPR can be proposed as follows.

Proposal 6: For PC2 PSSCH/PSCCH A-MPR without indicating dualPA, consider A-MPR in Table 2.2-21.

Proposal 7: For PC2 PSSCH/PSCCH A-MPR with indicating dualPA, consider A-MPR in Table 2.2-22.

PSFCH A-MPR (NS_33)

< PC3 PSFCH A-MPR >

For PSFCH A-MPR of SL non-contiguous CA, following evaluation scenarios in Table 2.3-2 are considered.

Table 2.3-2: SL non-contiguous CA AMPR evaluation scenarios (PSFCH)

| Aggregated 10+10 configuration | Scenario | CC1 | CC2 | Frequency of FIM3,low_block,low & FIM3,high_block,high | SCS1 (= SCS2) |
| --- | --- | --- | --- | --- | --- |
| Conf#2, Conf#3 in Table 2.1-1 | 1 | 1RB51 | 1RB0 | SEMfreq_-13A | 15 |
|  | 2 | 1RB45 | 1RB6 | SEMfreq_-13A | 15 |
|  | 3 | 1RB39 | 1RB12 | SEMfreq_-13A | 15 |
|  | 4 | 1RB35 | 1RB16 | SEMfreq_-13A | 15 |
|  | 5 | 1RB26 | 1RB25 | SEfreq_-30A | 15 |
|  | 6 | 1RB21 | 1RB30 | SEfreq_-30A | 15 |
|  | 7 | 1RB13 | 1RB38 | SEfreq_-30A | 15 |
|  | 8 | 1RB0 | 1RB51 | SEfreq_-30A | 15 |
|  | 9 | 1RB23 | 1RB0 | SEMfreq_-13A | 30 |
|  | 10 | 1RB21 | 1RB2 | SEMfreq_-13A | 30 |
|  | 11 | 1RB19 | 1RB4 | SEMfreq_-13A | 30 |
|  | 12 | 1RB17 | 1RB6 | SEMfreq_-13A | 30 |
|  | 13 | 1RB12 | 1RB11 | SEfreq_-30A | 30 |
|  | 14 | 1RB8 | 1RB15 | SEfreq_-30A | 30 |
|  | 15 | 1RB4 | 1RB19 | SEfreq_-30A | 30 |
|  | 16 | 1RB0 | 1RB23 | SEfreq_-30A | 30 |
|  | 17 | 1RB10 | 1RB0 | SEMfreq_-13A | 60 |
|  | 18 | 1RB9 | 1RB1 | SEMfreq_-13A | 60 |
|  | 19 | 1RB8 | 1RB2 | SEMfreq_-13A | 60 |
|  | 20 | 1RB7 | 1RB3 | SEfreq_-30A | 60 |
|  | 21 | 1RB5 | 1RB5 | SEfreq_-30A | 60 |
|  | 22 | 1RB3 | 1RB7 | SEfreq_-30A | 60 |
|  | 23 | 1RB1 | 1RB9 | SEfreq_-30A | 60 |
|  | 24 | 1RB0 | 1RB10 | SEfreq_-30A | 60 |
| Conf#1, Conf#4 ~ Conf#9 in Table 2.1-1 | 1 | 1RB51 | 1RB0 | SEfreq_-30A | 15 |
|  | 2 | 1RB45 | 1RB6 | SEfreq_-30A | 15 |
|  | 3 | 1RB39 | 1RB12 | SEfreq_-30A | 15 |
|  | 4 | 1RB35 | 1RB16 | SEfreq_-30A | 15 |
|  | 5 | 1RB26 | 1RB25 | SEfreq_-30A | 15 |
|  | 6 | 1RB21 | 1RB30 | SEfreq_-30A | 15 |
|  | 7 | 1RB13 | 1RB38 | SEfreq_-30A | 15 |
|  | 8 | 1RB0 | 1RB51 | SEfreq_-30A | 15 |
|  | 9 | 1RB23 | 1RB0 | SEfreq_-30A | 30 |
|  | 10 | 1RB21 | 1RB2 | SEfreq_-30A | 30 |
|  | 11 | 1RB19 | 1RB4 | SEfreq_-30A | 30 |
|  | 12 | 1RB17 | 1RB6 | SEfreq_-30A | 30 |
|  | 13 | 1RB12 | 1RB11 | SEfreq_-30A | 30 |
|  | 14 | 1RB8 | 1RB15 | SEfreq_-30A | 30 |
|  | 15 | 1RB4 | 1RB19 | SEfreq_-30A | 30 |
|  | 16 | 1RB0 | 1RB23 | SEfreq_-30A | 30 |
|  | 17 | 1RB10 | 1RB0 | SEfreq_-30A | 60 |
|  | 18 | 1RB9 | 1RB1 | SEfreq_-30A | 60 |
|  | 19 | 1RB8 | 1RB2 | SEfreq_-30A | 60 |
|  | 20 | 1RB7 | 1RB3 | SEfreq_-30A | 60 |
|  | 21 | 1RB5 | 1RB5 | SEfreq_-30A | 60 |
|  | 22 | 1RB3 | 1RB7 | SEfreq_-30A | 60 |
|  | 23 | 1RB1 | 1RB9 | SEfreq_-30A | 60 |
|  | 24 | 1RB0 | 1RB10 | SEfreq_-30A | 60 |

Table 2.3-3, Table 2.3-4, and Table 2.3-5 show the AMPR simulation results for the SL non-contiguous CA with architecture #1-1, #1-2, and #2-2 in Table 2.2-3 respectively.

Table 2.3-3: PSFCH AMPR simulation results for SL Non-contiguous CA with 1x23dBm+1LO

|  | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S0_10_G10_10 | 12.8 | 12.3 | 12.4 | 12.9 | 12.4 | 12.3 | 12.3 | 16.5 |
| S10_10_G10_10 | 9.1 | 8.6 | 9.7 | 12.9 | 12.4 | 12.3 | 12.3 | 12.4 |
| S20_10_G10_10 | 9.1 | 8.6 | 9.7 | 12.9 | 12.4 | 12.3 | 12.3 | 12.4 |
| S0_10_G20_10 | 12.4 | 12.3 | 12.3 | 12.4 | 12.4 | 12.3 | 12.2 | 16.5 |
| S10_10_G20_10 | 12.4 | 12.3 | 12.3 | 12.4 | 12.4 | 12.3 | 12.2 | 12.4 |
| S0_10_G30_10 | 12.4 | 12.4 | 12.4 | 12.4 | 12.3 | 12.3 | 12.3 | 16.5 |
| S10_10_G40_10 | 12.4 | 12.4 | 12.4 | 12.4 | 12.3 | 12.3 | 12.3 | 12.4 |
| S0_10_G40_10 | 12.5 | 12.4 | 12.4 | 12.4 | 12.3 | 12.2 | 12.3 | 16.5 |
| S0_10_G50_10 | 12.4 | 12.3 | 12.4 | 12.4 | 12.4 | 12.3 | 12.2 | 16.0 |
|  | #9 | #10 | #11 | #12 | #13 | #14 | #15 | #16 |
| S0_10_G10_10 | 11.6 | 11.9 | 12.4 | 12.3 | 10.9 | 12.3 | 12.4 | 17.9 |
| S10_10_G10_10 | 8.3 | 7.8 | 8.7 | 10.0 | 10.9 | 12.3 | 12.4 | 11.0 |
| S20_10_G10_10 | 8.3 | 7.8 | 8.7 | 10.0 | 10.9 | 12.3 | 12.4 | 12.4 |
| S0_10_G20_10 | 12.1 | 12.4 | 12.4 | 11.9 | 11.3 | 12.3 | 12.0 | 17.9 |
| S10_10_G20_10 | 12.1 | 12.4 | 12.4 | 11.9 | 11.8 | 12.3 | 12.0 | 12.0 |
| S0_10_G30_10 | 12.5 | 11.9 | 11.9 | 12.3 | 12.2 | 12.3 | 12.4 | 16.5 |
| S10_10_G40_10 | 12.5 | 11.9 | 11.9 | 12.3 | 12.2 | 12.3 | 12.4 | 12.4 |
| S0_10_G40_10 | 12.4 | 12.3 | 12.2 | 12.2 | 12.3 | 12.4 | 12.4 | 17.4 |
| S0_10_G50_10 | 12.4 | 11.8 | 12.2 | 12.3 | 11.9 | 12.0 | 12.4 | 18.3 |
|  | #17 | #18 | #19 | #20 | #21 | #22 | #23 | #24 |
| S0_10_G10_10 | 11.4 | 11.3 | 11.4 | 11.4 | 11.5 | 11.5 | 14.6 | 18.3 |
| S10_10_G10_10 | 6.2 | 6.2 | 9.1 | 11.0 | 11.5 | 11.1 | 11.4 | 11.4 |
| S20_10_G10_10 | 6.2 | 6.2 | 9.1 | 11.0 | 11.5 | 11.1 | 11.4 | 11.4 |
| S0_10_G20_10 | 11.5 | 11.3 | 11.3 | 11.3 | 10.9 | 11.5 | 14.7 | 16.6 |
| S10_10_G20_10 | 11.5 | 11.3 | 11.3 | 11.3 | 10.9 | 11.5 | 11.5 | 11.5 |
| S0_10_G30_10 | 11.4 | 11.3 | 11.4 | 11.4 | 11.5 | 12.0 | 14.6 | 16.5 |
| S10_10_G40_10 | 11.4 | 11.3 | 11.4 | 11.4 | 11.5 | 11.5 | 11.5 | 11.0 |
| S0_10_G40_10 | 11.5 | 11.4 | 11.4 | 11.0 | 11.5 | 12.0 | 14.6 | 16.4 |
| S0_10_G50_10 | 11.5 | 11.5 | 11.5 | 11.1 | 11.5 | 11.9 | 14.1 | 16.4 |

Table 2.3-4: PSFCH AMPR simulation results for SL Non-contiguous CA with 2x20dBm+1LO

|  | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S0_10_G10_10 | 13.0 | 12.5 | 12.6 | 13.1 | 12.6 | 12.5 | 12.5 | 17.2 |
| S10_10_G10_10 | 8.9 | 8.9 | 10.3 | 13.1 | 12.6 | 12.5 | 12.5 | 12.6 |
| S20_10_G10_10 | 8.9 | 8.9 | 10.3 | 13.1 | 12.6 | 12.5 | 12.5 | 12.6 |
| S0_10_G20_10 | 12.6 | 12.5 | 12.6 | 12.6 | 12.6 | 12.6 | 12.5 | 17.2 |
| S10_10_G20_10 | 12.6 | 12.5 | 12.6 | 12.6 | 12.6 | 12.6 | 12.5 | 12.6 |
| S0_10_G30_10 | 12.6 | 12.6 | 12.7 | 12.7 | 12.6 | 12.5 | 12.5 | 17.2 |
| S10_10_G40_10 | 12.6 | 12.6 | 12.7 | 12.7 | 12.6 | 12.5 | 12.5 | 12.6 |
| S0_10_G40_10 | 12.7 | 12.7 | 12.6 | 12.6 | 12.5 | 12.5 | 12.6 | 17.2 |
| S0_10_G50_10 | 12.6 | 12.5 | 12.6 | 12.6 | 12.6 | 12.5 | 12.5 | 16.7 |
|  | #9 | #10 | #11 | #12 | #13 | #14 | #15 | #16 |
| S0_10_G10_10 | 11.8 | 12.2 | 12.6 | 12.5 | 11.1 | 12.6 | 12.7 | 18.6 |
| S10_10_G10_10 | 8.6 | 8.0 | 8.9 | 10.7 | 11.1 | 12.6 | 12.7 | 11.2 |
| S20_10_G10_10 | 8.6 | 8.0 | 8.9 | 10.7 | 11.1 | 12.6 | 12.7 | 12.6 |
| S0_10_G20_10 | 12.3 | 12.7 | 12.6 | 12.1 | 11.6 | 12.5 | 12.2 | 18.6 |
| S10_10_G20_10 | 12.3 | 12.7 | 12.6 | 12.1 | 12.0 | 12.5 | 12.2 | 12.2 |
| S0_10_G30_10 | 12.7 | 12.1 | 12.1 | 12.5 | 12.5 | 12.6 | 12.7 | 17.2 |
| S10_10_G40_10 | 12.7 | 12.1 | 12.1 | 12.5 | 12.5 | 12.6 | 12.7 | 12.6 |
| S0_10_G40_10 | 12.6 | 12.5 | 12.5 | 12.5 | 12.5 | 12.6 | 13.1 | 18.1 |
| S0_10_G50_10 | 12.6 | 12.0 | 12.5 | 12.5 | 12.1 | 12.2 | 12.6 | 18.6 |
|  | #17 | #18 | #19 | #20 | #21 | #22 | #23 | #24 |
| S0_10_G10_10 | 11.6 | 11.6 | 11.6 | 11.7 | 11.7 | 12.2 | 15.3 | 19.0 |
| S10_10_G10_10 | 7.0 | 6.9 | 9.8 | 11.2 | 11.7 | 11.3 | 11.7 | 11.7 |
| S20_10_G10_10 | 7.0 | 6.9 | 9.8 | 11.2 | 11.7 | 11.3 | 11.7 | 11.7 |
| S0_10_G20_10 | 11.7 | 11.6 | 11.6 | 11.6 | 11.2 | 12.2 | 15.4 | 17.3 |
| S10_10_G20_10 | 11.7 | 11.6 | 11.6 | 11.6 | 11.2 | 11.7 | 11.7 | 11.8 |
| S0_10_G30_10 | 11.7 | 11.6 | 11.6 | 11.7 | 11.7 | 12.7 | 15.3 | 17.2 |
| S10_10_G40_10 | 11.7 | 11.6 | 11.6 | 11.7 | 11.7 | 11.8 | 11.7 | 11.3 |
| S0_10_G40_10 | 11.7 | 11.7 | 11.7 | 11.3 | 11.8 | 12.7 | 14.8 | 17.1 |
| S0_10_G50_10 | 11.7 | 11.7 | 11.7 | 11.3 | 11.8 | 12.6 | 14.8 | 17.1 |

Table 2.3-5: PSFCH AMPR simulation results for SL Non-contiguous CA with 2x20dBm+2LO

|  | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S0_10_G10_10 | 10.6 | 10.5 | 10.6 | 12.0 | 10.9 | 10.6 | 10.6 | 14.4 |
| S10_10_G10_10 | 5.5 | 5.4 | 9.2 | 12.5 | 11.4 | 11.5 | 11.6 | 11.8 |
| S20_10_G10_10 | 5.5 | 5.4 | 9.2 | 12.5 | 11.4 | 11.5 | 11.6 | 11.8 |
| S0_10_G20_10 | 10.6 | 10.5 | 10.6 | 10.7 | 10.9 | 10.6 | 10.6 | 15.3 |
| S10_10_G20_10 | 10.6 | 10.5 | 10.6 | 10.7 | 10.9 | 10.6 | 10.6 | 11.2 |
| S0_10_G30_10 | 10.6 | 11.0 | 10.6 | 10.7 | 10.5 | 10.6 | 10.6 | 12.1 |
| S10_10_G40_10 | 10.6 | 11.0 | 10.6 | 10.7 | 10.5 | 10.6 | 10.6 | 10.8 |
| S0_10_G40_10 | 10.6 | 10.5 | 10.6 | 10.7 | 10.5 | 10.6 | 10.6 | 13.1 |
| S0_10_G50_10 | 11.1 | 10.5 | 10.6 | 10.7 | 10.9 | 10.6 | 10.6 | 14.0 |
|  | #9 | #10 | #11 | #12 | #13 | #14 | #15 | #16 |
| S0_10_G10_10 | 10.6 | 10.2 | 8.7 | 9.6 | 10.2 | 10.5 | 10.6 | 15.8 |
| S10_10_G10_10 | 5.5 | 5.6 | 5.4 | 10.5 | 11.2 | 11.5 | 10.6 | 11.3 |
| S20_10_G10_10 | 5.5 | 5.6 | 5.4 | 10.5 | 11.2 | 11.5 | 11.5 | 11.3 |
| S0_10_G20_10 | 10.6 | 10.7 | 10.1 | 9.1 | 10.2 | 10.5 | 11.0 | 16.7 |
| S10_10_G20_10 | 10.6 | 10.7 | 10.1 | 9.1 | 10.2 | 10.5 | 10.6 | 10.8 |
| S0_10_G30_10 | 10.2 | 10.2 | 10.6 | 10.5 | 10.7 | 10.5 | 10.6 | 14.9 |
| S10_10_G40_10 | 10.2 | 10.2 | 10.6 | 10.5 | 10.7 | 10.5 | 10.6 | 10.8 |
| S0_10_G40_10 | 10.6 | 10.7 | 10.1 | 10.5 | 10.7 | 10.5 | 11.5 | 17.2 |
| S0_10_G50_10 | 10.6 | 10.7 | 10.6 | 10.5 | 10.2 | 10.5 | 10.6 | 15.8 |
|  | #17 | #18 | #19 | #20 | #21 | #22 | #23 | #24 |
| S0_10_G10_10 | 7.5 | 7.0 | 8.4 | 8.8 | 8.2 | 11.5 | 16.1 | 19.1 |
| S10_10_G10_10 | 5.6 | 5.6 | 5.6 | 10.2 | 9.6 | 9.7 | 9.8 | 10.5 |
| S20_10_G10_10 | 5.6 | 5.6 | 5.6 | 10.2 | 9.6 | 9.7 | 9.8 | 10.5 |
| S0_10_G20_10 | 8.4 | 8.8 | 8.4 | 8.3 | 8.2 | 10.1 | 14.3 | 16.8 |
| S10_10_G20_10 | 8.4 | 8.8 | 8.4 | 8.3 | 8.6 | 8.8 | 8.4 | 8.6 |
| S0_10_G30_10 | 8.9 | 8.4 | 8.4 | 8.3 | 8.2 | 10.6 | 14.8 | 17.2 |
| S10_10_G40_10 | 8.9 | 8.4 | 8.4 | 8.3 | 8.2 | 8.8 | 8.4 | 9.0 |
| S0_10_G40_10 | 8.9 | 8.4 | 8.8 | 8.8 | 7.7 | 10.6 | 14.8 | 17.2 |
| S0_10_G50_10 | 8.4 | 8.4 | 8.4 | 7.8 | 8.6 | 11.0 | 15.2 | 17.7 |

Table 2.3-6 shows the maximum value of simulation results for SL non-contiguous CA with architecture #1-1, #1-2, and #2-2 in Table 2.2-3 respectively considering B and frequency of FIM3,low_block,low & FIM3,high_block,high.

Here,

SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}.

SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}.

Table 2.3-6 : PC3 PSFCH AMPR simulation results for SL non-contiguous CA

| Carrier Frequency Combination [MHz] |  | AMPR (dB) for IM3 frequency |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SEMfreq_-13/-13A (SCS[kHz]) |  |  | SEfreq_-30/-30A (SCS[kHz]) |  |  |
|  |  | 15 | 30 | 60 | 15 | 30 | 60 |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 1x23dBm | 12.9 | 12.4 | 11.4 | 16.5 | 18.3 | 18.3 |
|  | 2x20dBm + 1LO | 13.1 | 12.6 | 11.6 | 17.2 | 18.6 | 19.0 |
|  | 2x20dBm + 2LO | 12.0 | 10.6 | 8.4 | 15.3 | 17.2 | 19.1 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 1x23dBm | 12.9 | 10.0 | 9.1 | 12.4 | 12.5 | 11.5 |
|  | 2x20dBm + 1LO | 13.1 | 10.7 | 9.8 | 12.7 | 12.7 | 11.8 |
|  | 2x20dBm + 2LO | 12.5 | 10.5 | 5.6 | 11.8 | 11.5 | 10.5 |

The AMPR can be proposed as Table 2.3-7 based on the simulation results when considering implementation margin.

Table 2.3-7 : PC3 PSFCH AMPR for SL non-contiguous CA

| Carrier Frequency Combination [MHz] |  | AMPR (dB) for IM3 frequency |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SEMfreq_-13/-13A (SCS[kHz]) |  |  | SEfreq_-30/-30A (SCS[kHz]) |  |  |
|  |  | 15 | 30 | 60 | 15 | 30 | 60 |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 1x23dBm | ≤ 18.0 | ≤ 18.0 | ≤ 17.0 | ≤ 20.5 | ≤ 22.0 | ≤ 22.5 |
|  | 2x20dBm + 1LO | ≤ 18.0 | ≤ 18.0 | ≤ 17.0 | ≤ 20.5 | ≤ 22.0 | ≤ 22.5 |
|  | 2x20dBm + 2LO | ≤ 16.5 | ≤ 15.5 | ≤ 14.0 | ≤ 19.0 | ≤ 20.5 | ≤ 22.5 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 1x23dBm | ≤ 18.0 | ≤ 15.0 | ≤ 13.0 | ≤ 18.0 | ≤ 18.0 | ≤ 17.0 |
|  | 2x20dBm + 1LO | ≤ 18.0 | ≤ 15.0 | ≤ 13.0 | ≤ 18.0 | ≤ 18.0 | ≤ 17.0 |
|  | 2x20dBm + 2LO | ≤ 17.0 | ≤ 14.5 | ≤ 9.0 | ≤ 16.0 | ≤ 15.5 | ≤ 14.5 |
| NOTE : SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}. SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}. |  |  |  |  |  |  |  |

From Table 2.3-6 and Table 2.3-7, the following is observed.

Observation 1: For PC3 PSFCH, A-MPR of higher SCS is larger than that of smaller SCS for SEMfreq_-30/-30A  in Case1 of carrier frequency combination, otherwise, it is smaller.

From the obsevation, PSFCH A-MPR needs to be specified with large A-MPR of different SCSs to simplify the requirement.

And, considering proposal 2 and 3, PC3 PSFCH A-MPR can be proposed as follows.

Proposal 8: For PC3 PSFCH A-MPR without indicating dualPA, consider A-MPR of 2x20dBm+1LO in Table 2.3-8.

Proposal 9: For PC3 PSFCH A-MPR with indicating dualPA, consider A-MPR of 2x20dBm+2LO in Table 2.3-8.

Table 2.3-8 : PC3 PSFCH AMPR for SL non-contiguous CA

| Carrier Frequency Combination [MHz] |  | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
|  |  | SEMfreq_-13/-13A (SCS[kHz]) | SEfreq_-30/-30A (SCS[kHz]) |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 1x23dBm | ≤ 18.0 | ≤ 22.5 |
|  | 2x20dBm + 1LO | ≤ 18.0 | ≤ 22.5 |
|  | 2x20dBm + 2LO | ≤ 16.5 | ≤ 22.5 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 1x23dBm | ≤ 18.0 | ≤ 18.0 |
|  | 2x20dBm + 1LO | ≤ 18.0 | ≤ 18.0 |
|  | 2x20dBm + 2LO | ≤ 17.0 | ≤ 16.0 |
| NOTE : SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}. SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}. |  |  |  |

< PC2 PSFCH A-MPR >

Table 2.3-9, Table 2.3-10, and Table 2.3-11 show the AMPR simulation results for the SL non-contiguous CA with architecture #1-1, #1-2, and #2-2 in Table 2.2-13 respectively.

Table 2.3-9: PSFCH AMPR simulation results for SL Non-contiguous CA with 1x26dBm+1LO

|  | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S0_10_G10_10 | 14.3 | 14.3 | 14.4 | 14.9 | 14.4 | 14.3 | 14.3 | 19.9 |
| S10_10_G10_10 | 10.7 | 10.7 | 11.7 | 14.9 | 14.4 | 14.3 | 14.3 | 14.4 |
| S20_10_G10_10 | 10.7 | 10.7 | 11.7 | 14.9 | 14.4 | 14.3 | 14.3 | 14.4 |
| S0_10_G20_10 | 14.4 | 14.3 | 14.3 | 13.9 | 14.4 | 14.3 | 14.2 | 19.9 |
| S10_10_G20_10 | 14.4 | 14.3 | 14.3 | 13.9 | 14.4 | 14.3 | 14.2 | 14.4 |
| S0_10_G30_10 | 14.4 | 13.9 | 14.4 | 14.4 | 14.3 | 14.3 | 14.3 | 19.9 |
| S10_10_G40_10 | 14.4 | 13.9 | 14.4 | 14.4 | 14.3 | 14.3 | 14.3 | 14.4 |
| S0_10_G40_10 | 14.0 | 14.0 | 14.4 | 14.4 | 14.3 | 14.2 | 14.3 | 19.5 |
| S0_10_G50_10 | 14.4 | 14.3 | 14.4 | 14.4 | 14.4 | 14.3 | 14.2 | 19.4 |
|  | #9 | #10 | #11 | #12 | #13 | #14 | #15 | #16 |
| S0_10_G10_10 | 13.6 | 13.9 | 14.3 | 14.3 | 12.9 | 13.9 | 14.4 | 20.8 |
| S10_10_G10_10 | 9.9 | 9.8 | 10.2 | 12.5 | 12.9 | 13.9 | 14.0 | 13.0 |
| S20_10_G10_10 | 9.9 | 9.8 | 10.2 | 12.5 | 12.9 | 13.9 | 14.0 | 13.9 |
| S0_10_G20_10 | 14.1 | 14.0 | 14.0 | 13.9 | 13.3 | 14.3 | 14.0 | 20.9 |
| S10_10_G20_10 | 14.1 | 14.0 | 14.0 | 13.9 | 13.8 | 14.3 | 14.0 | 14.0 |
| S0_10_G30_10 | 14.0 | 13.9 | 13.9 | 13.8 | 13.8 | 13.9 | 14.9 | 19.4 |
| S10_10_G40_10 | 14.0 | 13.9 | 13.9 | 13.8 | 13.8 | 13.9 | 14.0 | 13.9 |
| S0_10_G40_10 | 14.4 | 14.3 | 13.8 | 13.8 | 14.3 | 14.0 | 15.3 | 20.3 |
| S0_10_G50_10 | 13.9 | 13.8 | 14.2 | 14.3 | 13.9 | 14.0 | 13.9 | 21.3 |
|  | #17 | #18 | #19 | #20 | #21 | #22 | #23 | #24 |
| S0_10_G10_10 | 12.9 | 13.3 | 13.4 | 13.4 | 13.5 | 14.4 | 18.0 | 21.3 |
| S10_10_G10_10 | 8.8 | 8.7 | 11.5 | 13.0 | 13.5 | 13.1 | 13.4 | 13.4 |
| S20_10_G10_10 | 8.8 | 8.7 | 11.5 | 13.0 | 13.5 | 13.1 | 13.4 | 13.4 |
| S0_10_G20_10 | 13.0 | 13.3 | 13.3 | 13.3 | 12.9 | 14.4 | 17.6 | 19.5 |
| S10_10_G20_10 | 13.0 | 13.3 | 13.3 | 13.3 | 12.9 | 13.5 | 13.5 | 13.1 |
| S0_10_G30_10 | 13.4 | 13.3 | 13.4 | 13.4 | 13.0 | 14.9 | 17.6 | 19.4 |
| S10_10_G40_10 | 13.4 | 13.3 | 13.4 | 13.4 | 13.0 | 13.5 | 13.4 | 13.0 |
| S0_10_G40_10 | 13.5 | 13.4 | 13.4 | 13.0 | 13.1 | 14.9 | 17.5 | 19.4 |
| S0_10_G50_10 | 13.0 | 13.5 | 13.5 | 13.1 | 13.5 | 14.8 | 17.0 | 19.4 |

Table 2.3-10: PSFCH AMPR simulation results for SL Non-contiguous CA with 2x23dBm+1LO

|  | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S0_10_G10_10 | 15.0 | 15.0 | 14.6 | 15.1 | 14.6 | 14.9 | 14.9 | 20.1 |
| S10_10_G10_10 | 11.3 | 10.9 | 12.3 | 15.1 | 14.6 | 14.9 | 14.9 | 15.1 |
| S20_10_G10_10 | 11.3 | 10.9 | 12.3 | 15.1 | 14.6 | 14.9 | 14.9 | 15.1 |
| S0_10_G20_10 | 15.0 | 14.9 | 15.0 | 14.6 | 14.6 | 15.0 | 14.9 | 20.6 |
| S10_10_G20_10 | 15.0 | 14.9 | 15.0 | 14.6 | 14.6 | 15.0 | 14.9 | 15.0 |
| S0_10_G30_10 | 14.6 | 14.6 | 14.6 | 14.6 | 14.5 | 14.5 | 14.9 | 20.6 |
| S10_10_G40_10 | 14.6 | 14.6 | 14.6 | 14.6 | 14.5 | 14.5 | 14.9 | 15.1 |
| S0_10_G40_10 | 14.7 | 14.6 | 14.6 | 14.6 | 14.9 | 14.4 | 15.0 | 20.1 |
| S0_10_G50_10 | 14.6 | 15.0 | 14.6 | 14.6 | 14.6 | 14.5 | 14.9 | 19.6 |
|  | #9 | #10 | #11 | #12 | #13 | #14 | #15 | #16 |
| S0_10_G10_10 | 13.8 | 14.6 | 14.6 | 14.5 | 13.5 | 14.5 | 15.1 | 21.5 |
| S10_10_G10_10 | 10.6 | 10.0 | 10.9 | 13.1 | 13.5 | 14.5 | 14.6 | 13.7 |
| S20_10_G10_10 | 10.6 | 10.0 | 10.9 | 13.1 | 13.5 | 14.5 | 14.6 | 14.6 |
| S0_10_G20_10 | 14.3 | 14.7 | 14.6 | 14.6 | 14.0 | 14.5 | 14.6 | 21.6 |
| S10_10_G20_10 | 14.3 | 14.7 | 14.6 | 14.6 | 14.5 | 14.5 | 14.6 | 14.7 |
| S0_10_G30_10 | 14.7 | 14.6 | 14.1 | 14.5 | 14.5 | 14.6 | 15.1 | 20.1 |
| S10_10_G40_10 | 14.7 | 14.6 | 14.1 | 14.5 | 14.5 | 14.6 | 14.6 | 14.6 |
| S0_10_G40_10 | 14.6 | 14.5 | 14.5 | 14.4 | 14.5 | 14.6 | 16.0 | 21.0 |
| S0_10_G50_10 | 14.6 | 14.5 | 14.9 | 14.9 | 14.6 | 14.2 | 14.6 | 22.0 |
|  | #17 | #18 | #19 | #20 | #21 | #22 | #23 | #24 |
| S0_10_G10_10 | 13.6 | 13.6 | 14.0 | 14.1 | 13.7 | 15.1 | 18.7 | 22.0 |
| S10_10_G10_10 | 9.5 | 9.4 | 12.2 | 13.6 | 13.7 | 13.7 | 13.7 | 14.1 |
| S20_10_G10_10 | 9.5 | 9.4 | 12.2 | 13.6 | 13.7 | 13.7 | 13.7 | 14.1 |
| S0_10_G20_10 | 13.7 | 13.6 | 14.0 | 14.0 | 13.2 | 15.1 | 18.3 | 20.2 |
| S10_10_G20_10 | 13.7 | 13.6 | 14.0 | 14.0 | 13.6 | 13.7 | 14.2 | 13.8 |
| S0_10_G30_10 | 14.1 | 13.6 | 14.1 | 14.1 | 13.7 | 15.6 | 18.3 | 20.1 |
| S10_10_G40_10 | 14.1 | 13.6 | 14.1 | 14.1 | 13.7 | 13.7 | 14.1 | 13.7 |
| S0_10_G40_10 | 14.1 | 13.6 | 14.1 | 13.7 | 13.7 | 15.5 | 17.7 | 20.1 |
| S0_10_G50_10 | 13.7 | 14.2 | 13.7 | 13.7 | 13.7 | 15.5 | 17.7 | 20.0 |

Table 2.3-11: PSFCH AMPR simulation results for SL Non-contiguous CA with 2x23dBm+2LO

|  | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S0_10_G10_10 | 11.6 | 11.5 | 11.5 | 13.0 | 11.9 | 11.5 | 11.6 | 17.2 |
| S10_10_G10_10 | 6.0 | 5.9 | 8.8 | 13.0 | 11.9 | 11.5 | 11.6 | 12.2 |
| S20_10_G10_10 | 6.0 | 5.9 | 8.8 | 13.0 | 11.9 | 11.5 | 11.6 | 12.2 |
| S0_10_G20_10 | 11.6 | 11.5 | 12.0 | 11.6 | 11.9 | 11.5 | 11.6 | 18.1 |
| S10_10_G20_10 | 11.6 | 11.5 | 12.0 | 11.6 | 11.9 | 11.5 | 11.6 | 12.2 |
| S0_10_G30_10 | 12.1 | 12.0 | 11.5 | 11.6 | 11.9 | 12.0 | 11.6 | 14.4 |
| S10_10_G40_10 | 12.1 | 12.0 | 11.5 | 11.6 | 11.9 | 12.0 | 11.6 | 11.7 |
| S0_10_G40_10 | 12.1 | 12.0 | 11.5 | 11.6 | 11.4 | 12.0 | 11.6 | 15.8 |
| S0_10_G50_10 | 12.1 | 11.5 | 11.5 | 11.6 | 11.9 | 11.5 | 11.6 | 16.7 |
|  | #9 | #10 | #11 | #12 | #13 | #14 | #15 | #16 |
| S0_10_G10_10 | 11.6 | 11.2 | 9.7 | 10.5 | 11.6 | 11.5 | 13.3 | 18.6 |
| S10_10_G10_10 | 6.0 | 6.0 | 5.9 | 10.5 | 11.6 | 11.5 | 10.6 | 11.7 |
| S20_10_G10_10 | 6.0 | 6.0 | 5.9 | 10.5 | 11.6 | 11.5 | 11.5 | 11.7 |
| S0_10_G20_10 | 11.6 | 11.6 | 11.1 | 10.5 | 11.6 | 11.5 | 13.8 | 19.0 |
| S10_10_G20_10 | 11.6 | 11.6 | 11.1 | 10.5 | 11.6 | 11.5 | 11.5 | 11.7 |
| S0_10_G30_10 | 11.2 | 11.2 | 11.5 | 11.4 | 11.6 | 11.5 | 12.0 | 17.2 |
| S10_10_G40_10 | 11.2 | 11.2 | 11.5 | 11.4 | 11.6 | 11.5 | 11.5 | 11.7 |
| S0_10_G40_10 | 11.6 | 11.6 | 11.1 | 11.4 | 11.6 | 11.5 | 14.2 | 20.0 |
| S0_10_G50_10 | 11.6 | 11.6 | 11.5 | 11.4 | 11.2 | 11.5 | 12.9 | 18.6 |
|  | #17 | #18 | #19 | #20 | #21 | #22 | #23 | #24 |
| S0_10_G10_10 | 8.0 | 8.9 | 9.8 | 10.7 | 11.0 | 14.3 | 19.0 | 21.9 |
| S10_10_G10_10 | 6.0 | 6.0 | 6.0 | 10.2 | 9.2 | 9.3 | 9.8 | 9.9 |
| S20_10_G10_10 | 6.0 | 6.0 | 6.0 | 10.2 | 9.2 | 9.3 | 9.8 | 9.9 |
| S0_10_G20_10 | 9.4 | 9.8 | 9.3 | 9.7 | 9.6 | 12.9 | 17.1 | 19.5 |
| S10_10_G20_10 | 9.4 | 9.8 | 9.3 | 9.7 | 9.6 | 10.2 | 9.8 | 9.9 |
| S0_10_G30_10 | 9.8 | 9.3 | 9.8 | 9.7 | 10.1 | 13.4 | 17.6 | 20.0 |
| S10_10_G40_10 | 9.8 | 9.3 | 9.8 | 9.7 | 9.6 | 9.7 | 9.8 | 9.9 |
| S0_10_G40_10 | 9.8 | 9.8 | 9.8 | 10.2 | 10.1 | 13.4 | 17.6 | 20.0 |
| S0_10_G50_10 | 9.8 | 9.8 | 9.8 | 9.7 | 10.1 | 13.8 | 18.0 | 20.5 |

Table 2.3-12 shows the maximum value of simulation results for SL non-contiguous CA with architecture #1-1, #1-2, and #2-2 in Table 2.2-13 respectively considering B and frequency of FIM3,low_block,low & FIM3,high_block,high.

Here,

SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}.

SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}.

Table 2.3-12 : PC2 PSFCH AMPR simulation results for SL non-contiguous CA

| Carrier Frequency Combination [MHz] |  | AMPR (dB) for IM3 frequency |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SEMfreq_-13/-13A (SCS[kHz]) |  |  | SEfreq_-30/-30A(SCS[kHz]) |  |  |
|  |  | 15 | 30 | 60 | 15 | 30 | 60 |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 1x26dBm | 14.9 | 14.3 | 13.4 | 19.9 | 21.3 | 21.3 |
|  | 2x23dBm + 1LO | 15.1 | 14.6 | 14.0 | 20.6 | 22.0 | 22.0 |
|  | 2x23dBm + 2LO | 13.0 | 11.6 | 9.8 | 18.1 | 20.0 | 21.9 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 1x26dBm | 14.9 | 12.5 | 11.5 | 14.4 | 14.3 | 13.5 |
|  | 2x23dBm + 1LO | 15.1 | 13.1 | 12.2 | 15.1 | 14.7 | 14.2 |
|  | 2x23dBm + 2LO | 13.0 | 10.5 | 6.0 | 12.2 | 11.7 | 10.2 |

The AMPR can be proposed as Table 2.3-13 based on the simulation results when considering implementation margin.

Table 2.3-13 : PC2 PSFCH AMPR for SL non-contiguous CA

| Carrier Frequency Combination [MHz] |  | AMPR (dB) for IM3 frequency |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SEMfreq_-13/-13A (SCS[kHz]) |  |  | SEfreq_-30/-30A (SCS[kHz]) |  |  |
|  |  | 15 | 30 | 60 | 15 | 30 | 60 |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 1x26dBm | ≤ 20.0 | ≤ 20.0 | ≤ 19.0 | ≤ 24.0 | ≤ 25.0 | ≤ 25.0 |
|  | 2x23dBm + 1LO | ≤ 20.0 | ≤ 20.0 | ≤ 19.0 | ≤ 24.0 | ≤ 25.0 | ≤ 25.0 |
|  | 2x23dBm + 2LO | ≤ 17.5 | ≤ 16.0 | ≤ 15.5 | ≤ 21.5 | ≤ 23.5 | ≤ 25.0 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 1x26dBm | ≤ 20.0 | ≤ 17.5 | ≤ 15.5 | ≤ 20.0 | ≤ 20.0 | ≤ 19.0 |
|  | 2x23dBm + 1LO | ≤ 20.0 | ≤ 17.5 | ≤ 15.5 | ≤ 20.0 | ≤ 20.0 | ≤ 19.0 |
|  | 2x23dBm + 2LO | ≤ 17.5 | ≤ 15.0 | ≤ 9.5 | ≤ 16.5 | ≤ 16.0 | ≤ 15.0 |
| NOTE : SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}. SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}. |  |  |  |  |  |  |  |

From Table 2.3-12 and Table 2.3-13, the following is observed.

Observation 2: For PC2 PSFCH, A-MPR of higher SCS is larger than that of smaller SCS for SEMfreq_-30/-30A  in Case1 of carrier frequency combination, otherwise, it is smaller.

From the obsevation, PSFCH A-MPR needs to be specified with large A-MPR of different SCSs to simplify the requirement.

And, considering proposal 2 and 3, PC3 PSFCH A-MPR can be proposed as follows.

Proposal 10: For PC2 PSFCH A-MPR without indicating dualPA, consider A-MPR of 2x23dBm+1LO in Table 2.3-14.

Proposal 11: For PC2 PSFCH A-MPR with indicating dualPA, consider A-MPR of 2x23dBm+2LO in Table 2.3-14.

Table 2.3-14 : PC3 PSFCH AMPR for SL non-contiguous CA

| Carrier Frequency Combination [MHz] |  | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
|  |  | SEMfreq_-13/-13A (SCS[kHz]) | SEfreq_-30/-30A (SCS[kHz]) |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 1x26dBm | ≤ 20.0 | ≤ 25.0 |
|  | 2x23dBm + 1LO | ≤ 20.0 | ≤ 25.0 |
|  | 2x23dBm + 2LO | ≤ 17.5 | ≤ 25.0 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 1x26dBm | ≤ 20.0 | ≤ 20.0 |
|  | 2x23dBm + 1LO | ≤ 20.0 | ≤ 20.0 |
|  | 2x23dBm + 2LO | ≤ 17.5 | ≤ 16.5 |
| NOTE : SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}. SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}. |  |  |  |

S-SSB A-MPR (NS_33)

< PC3 S-SSB A-MPR >

For S-SSB A-MPR of SL non-contiguous CA, following evaluation scenarios in Table 2.4-2 are considered.

Table 2.4-2: SL non-contiguous CA AMPR evaluation scenarios (S-SSB)

| Aggregated 10+10 configuration | Scenario | CC1 | CC2 | Frequency of FIM3,low_block,low & FIM3,high_block,high | SCS1 (= SCS2) |
| --- | --- | --- | --- | --- | --- |
| Conf#2, Conf#3 in Table 2.1-1 | 1 | 11RB41 | 11RB0 | SEMfreq_-13A | 15 |
|  | 2 | 11RB31 | 11RB10 | SEMfreq_-13A | 15 |
|  | 3 | 11RB20 | 11RB21 | SEfreq_-30A | 15 |
|  | 4 | 11RB0 | 11RB41 | SEfreq_-30A | 15 |
|  | 5 | 11RB26 | 11RB0 | SEfreq_-30A | 30 |
|  | 6 | 11RB21 | 11RB1 | SEfreq_-30A | 30 |
|  | 7 | 11RB13 | 11RB7 | SEfreq_-30A | 30 |
|  | 8 | 11RB0 | 11RB13 | SEfreq_-30A | 30 |
| Conf#1, Conf#4 ~ Conf#9 in Table 2.1-1 | 1 | 11RB41 | 11RB0 | SEfreq_-30A | 15 |
|  | 2 | 11RB31 | 11RB10 | SEfreq_-30A | 15 |
|  | 3 | 11RB20 | 11RB21 | SEfreq_-30A | 15 |
|  | 4 | 11RB0 | 11RB41 | SEfreq_-30A | 15 |
|  | 5 | 11RB26 | 11RB0 | SEfreq_-30A | 30 |
|  | 6 | 11RB21 | 11RB1 | SEfreq_-30A | 30 |
|  | 7 | 11RB13 | 11RB7 | SEfreq_-30A | 30 |
|  | 8 | 11RB0 | 11RB13 | SEfreq_-30A | 30 |

Table 2.4-3, Table 2.4-4, and Table 2.4-5 show the AMPR simulation results for the SL non-contiguous CA with architecture #1-1, #1-2, and #2-2 in Table 2.2-3 respectively.

Table 2.4-3: SSSB AMPR simulation results for SL Non-contiguous CA with 1x23dBm+1LO

|  | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S0_10_G10_10 | 17.4 | 17.0 | 17.8 | 17.9 | 15.6 | 15.9 | 15.0 | 16.0 |
| S10_10_G10_10 | 8.5 | 14.3 | 17.8 | 17.5 | 11.4 | 14.1 | 15.1 | 15.1 |
| S20_10_G10_10 | 8.6 | 13.3 | 17.4 | 16.9 | 11.7 | 13.6 | 15.1 | 14.2 |
| S0_10_G20_10 | 17.8 | 17.0 | 17.9 | 17.8 | 15.6 | 15.4 | 15.5 | 15.6 |
| S10_10_G20_10 | 17.8 | 17.4 | 17.9 | 17.0 | 15.5 | 14.7 | 15.4 | 15.5 |
| S0_10_G30_10 | 17.8 | 17.9 | 17.8 | 17.0 | 15.9 | 15.9 | 15.5 | 15.6 |
| S10_10_G40_10 | 17.8 | 16.6 | 16.9 | 17.0 | 15.8 | 15.0 | 15.5 | 15.5 |
| S0_10_G40_10 | 17.1 | 17.4 | 18.6 | 17.5 | 15.0 | 15.9 | 15.0 | 15.6 |
| S0_10_G50_10 | 17.8 | 17.5 | 17.8 | 17.5 | 15.8 | 15.0 | 16.3 | 16.0 |

Table 2.4-4: SSSB AMPR simulation results for SL Non-contiguous CA with 2x20dBm+1LO

|  | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S0_10_G10_10 | 17.3 | 18.7 | 17.4 | 18.2 | 15.4 | 16.3 | 15.8 | 16.3 |
| S10_10_G10_10 | 8.4 | 15.0 | 18.1 | 18.2 | 11.7 | 13.5 | 16.7 | 15.9 |
| S20_10_G10_10 | 8.4 | 14.1 | 17.7 | 17.8 | 12.6 | 13.9 | 15.9 | 15.9 |
| S0_10_G20_10 | 19.0 | 17.8 | 17.8 | 18.2 | 15.4 | 15.3 | 15.8 | 15.4 |
| S10_10_G20_10 | 18.5 | 17.8 | 18.7 | 18.2 | 15.4 | 15.9 | 15.8 | 15.9 |
| S0_10_G30_10 | 18.6 | 17.8 | 18.6 | 18.2 | 16.6 | 14.9 | 14.9 | 15.5 |
| S10_10_G40_10 | 18.7 | 17.8 | 18.5 | 17.8 | 15.8 | 15.8 | 15.8 | 15.4 |
| S0_10_G40_10 | 17.4 | 18.2 | 17.7 | 18.6 | 15.0 | 15.4 | 15.5 | 17.3 |
| S0_10_G50_10 | 18.1 | 18.7 | 18.1 | 17.0 | 16.3 | 15.3 | 15.8 | 16.4 |

Table 2.4-5: SSSB AMPR simulation results for SL Non-contiguous CA with 2x20dBm+2LO

|  | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S0_10_G10_10 | 7.7 | 8.2 | 8.1 | 13.8 | 7.3 | 7.2 | 7.7 | 13.8 |
| S10_10_G10_10 | 3.6 | 5.4 | 8.6 | 8.2 | 4.4 | 5.4 | 5.8 | 5.9 |
| S20_10_G10_10 | 3.5 | 5.4 | 8.2 | 8.2 | 4.4 | 5.4 | 5.9 | 6.4 |
| S0_10_G20_10 | 8.2 | 8.3 | 8.2 | 15.2 | 7.2 | 7.2 | 8.6 | 15.2 |
| S10_10_G20_10 | 8.2 | 8.3 | 8.6 | 8.2 | 7.2 | 7.3 | 7.2 | 7.3 |
| S0_10_G30_10 | 9.1 | 8.2 | 8.6 | 12.4 | 7.3 | 7.2 | 7.2 | 13.3 |
| S10_10_G40_10 | 7.7 | 8.3 | 8.6 | 8.2 | 7.2 | 7.2 | 7.2 | 7.3 |
| S0_10_G40_10 | 8.2 | 8.2 | 8.1 | 13.8 | 7.2 | 7.2 | 9.2 | 15.6 |
| S0_10_G50_10 | 8.7 | 8.2 | 9.0 | 13.4 | 7.3 | 7.3 | 7.8 | 13.8 |

Table 2.4-6 shows the maximum value of simulation results for SL non-contiguous CA with architecture #1-1, #1-2, and #2-2 in Table 2.2-3 respectively considering B and frequency of FIM3,low_block,low & FIM3,high_block,high.

Here,

SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}.

SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}.

Table 2.4-6 : PC3 SSB AMPR simulation results for SL non-contiguous CA

| Carrier Frequency Combination [MHz] |  | AMPR (dB) for IM3 frequency |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SEMfreq_-13/-13A (SCS[kHz]) |  | SEfreq_-30/-30A (SCS[kHz]) |  |
|  |  | 15 | 30 | 15 | 30 |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 1x23dBm | 17.4 | 15.9 | 18.6 | 16.3 |
|  | 2x20dBm + 1LO | 18.7 | 16.3 | 19.0 | 17.3 |
|  | 2x20dBm + 2LO | 8.2 | 7.3 | 15.2 | 15.6 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 1x23dBm | 14.3 | 14.1 | 17.9 | 15.8 |
|  | 2x20dBm + 1LO | 15.0 | 13.9 | 18.7 | 16.7 |
|  | 2x20dBm + 2LO | 5.4 | 5.4 | 8.6 | 7.3 |

The AMPR can be proposed as Table 2.4-7 based on the simulation results when considering implementation margin.

Table 2.4-7 : PC3 SSB AMPR for SL non-contiguous CA

| Carrier Frequency Combination [MHz] |  | AMPR (dB) for IM3 frequency |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SEMfreq_-13/-13A (SCS[kHz]) |  | SEfreq_-30/-30A (SCS[kHz]) |  |
|  |  | 15 | 30 | 15 | 30 |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 1x23dBm | ≤ 20.0 | ≤ 18.5 | ≤ 21.0 | ≤ 21.0 |
|  | 2x20dBm + 1LO | ≤ 21.0 | ≤ 19.5 | ≤ 21.5 | ≤ 21.5 |
|  | 2x20dBm + 2LO | ≤ 14.5 | ≤ 14.0 | ≤ 17.5 | ≤ 18.0 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 1x23dBm | ≤ 17.0 | ≤ 16.5 | ≤ 20.5 | ≤ 18.5 |
|  | 2x20dBm + 1LO | ≤ 17.5 | ≤ 16.5 | ≤ 21.0 | ≤ 19.5 |
|  | 2x20dBm + 2LO | ≤ 12.0 | ≤ 12.0 | ≤ 15.0 | ≤ 14.0 |
| NOTE : SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}. SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}. |  |  |  |  |  |

From Table 2.4-6 and Table 2.4-7, the following is observed.

Observation 3: For PC3 S-SSB, A-MPR of SCS 30kHz is larger than or equal to that of SCS 15kHz for SEMfreq_-30/-30A  in Case1 of carrier frequency combination, otherwise, it is smaller.

From the obsevation, S-SSB A-MPR needs to be specified with large A-MPR of different SCSs to simplify the requirement.

And, considering proposal 2 and 3, PC3 S-SSB A-MPR can be proposed as follows.

Proposal 12: For PC3 S-SSB A-MPR without indicating dualPA, consider A-MPR of 2x20dBm+1LO in Table 2.4-8.

Proposal 13: For PC3 S-SSB A-MPR with indicating dualPA, consider A-MPR of 2x20dBm+2LO in Table 2.4-8.

Table 2.4-8 : PC3 SSB AMPR for SL non-contiguous CA

| Carrier Frequency Combination [MHz] |  | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
|  |  | SEMfreq_-13/-13A (SCS[kHz]) | SEfreq_-30/-30A (SCS[kHz]) |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 1x23dBm | ≤ 20.0 | ≤ 21.0 |
|  | 2x20dBm + 1LO | ≤ 21.0 | ≤ 21.5 |
|  | 2x20dBm + 2LO | ≤ 14.5 | ≤ 18.0 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 1x23dBm | ≤ 17.0 | ≤ 20.5 |
|  | 2x20dBm + 1LO | ≤ 17.5 | ≤ 21.0 |
|  | 2x20dBm + 2LO | ≤ 12.0 | ≤ 15.0 |
| NOTE : SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}. SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}. |  |  |  |

< PC2 S-SSB A-MPR >

Table 2.4-9, Table 2.4-10, and Table 2.4-11 show the AMPR simulation results for the SL non-contiguous CA with architecture #1-1, #1-2, and #2-2 in Table 2.2-13 respectively.

Table 2.4-9: SSSB AMPR simulation results for SL Non-contiguous CA with 1x26dBm+1LO

|  | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S0_10_G10_10 | 20.8 | 20.4 | 21.2 | 21.3 | 18.0 | 18.0 | 17.1 | 18.6 |
| S10_10_G10_10 | 15.6 | 17.2 | 20.9 | 20.9 | 13.9 | 16.2 | 17.5 | 17.2 |
| S20_10_G10_10 | 13.8 | 16.3 | 21.3 | 20.9 | 13.8 | 15.7 | 17.5 | 18.1 |
| S0_10_G20_10 | 21.2 | 20.9 | 20.9 | 20.4 | 17.6 | 18.5 | 18.3 | 18.9 |
| S10_10_G20_10 | 20.7 | 20.8 | 20.4 | 21.7 | 18.9 | 17.5 | 17.0 | 17.5 |
| S0_10_G30_10 | 21.7 | 20.0 | 20.8 | 20.4 | 18.4 | 17.5 | 17.5 | 18.9 |
| S10_10_G40_10 | 20.9 | 20.4 | 20.3 | 20.9 | 18.0 | 18.9 | 17.5 | 18.1 |
| S0_10_G40_10 | 20.5 | 20.4 | 20.8 | 21.2 | 18.3 | 18.7 | 18.5 | 19.4 |
| S0_10_G50_10 | 22.1 | 20.4 | 20.9 | 21.3 | 17.4 | 17.4 | 18.5 | 17.7 |

Table 2.4-10: SSSB AMPR simulation results for SL Non-contiguous CA with 2x23dBm+1LO

|  | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S0_10_G10_10 | 21.1 | 20.7 | 21.1 | 21.6 | 17.9 | 18.3 | 18.2 | 19.3 |
| S10_10_G10_10 | 17.3 | 17.0 | 21.1 | 21.6 | 14.2 | 17.4 | 18.3 | 17.5 |
| S20_10_G10_10 | 14.6 | 17.1 | 21.2 | 22.1 | 14.2 | 16.0 | 18.7 | 18.4 |
| S0_10_G20_10 | 21.9 | 22.0 | 21.6 | 20.7 | 18.3 | 17.9 | 18.3 | 22.5 |
| S10_10_G20_10 | 22.3 | 21.6 | 21.7 | 22.0 | 17.5 | 18.3 | 19.5 | 17.9 |
| S0_10_G30_10 | 21.1 | 22.0 | 22.0 | 21.6 | 18.7 | 19.1 | 19.1 | 18.4 |
| S10_10_G40_10 | 21.6 | 21.7 | 21.5 | 20.7 | 18.3 | 18.2 | 18.7 | 18.8 |
| S0_10_G40_10 | 21.7 | 22.1 | 21.9 | 21.2 | 18.3 | 17.8 | 17.4 | 19.3 |
| S0_10_G50_10 | 21.5 | 20.3 | 21.6 | 21.2 | 18.3 | 18.7 | 19.3 | 20.7 |

Table 2.4-11: SSSB AMPR simulation results for SL Non-contiguous CA with 2x23dBm+2LO

|  | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S0_10_G10_10 | 11.6 | 12.1 | 12.0 | 16.3 | 7.9 | 8.3 | 10.2 | 16.3 |
| S10_10_G10_10 | 3.8 | 7.4 | 12.0 | 12.1 | 6.4 | 7.4 | 7.9 | 7.5 |
| S20_10_G10_10 | 3.8 | 7.4 | 12.5 | 12.1 | 6.0 | 6.9 | 8.3 | 7.9 |
| S0_10_G20_10 | 12.1 | 12.1 | 12.0 | 16.7 | 8.3 | 7.8 | 10.7 | 18.1 |
| S10_10_G20_10 | 12.1 | 12.1 | 12.5 | 12.1 | 7.9 | 7.8 | 7.9 | 8.3 |
| S0_10_G30_10 | 12.5 | 12.1 | 12.1 | 14.8 | 7.9 | 7.8 | 8.8 | 16.2 |
| S10_10_G40_10 | 12.2 | 12.1 | 12.0 | 12.6 | 7.8 | 7.9 | 8.8 | 7.8 |
| S0_10_G40_10 | 12.1 | 12.1 | 12.5 | 16.2 | 8.3 | 8.3 | 11.1 | 17.6 |
| S0_10_G50_10 | 12.1 | 12.1 | 12.0 | 16.7 | 7.8 | 8.4 | 10.7 | 16.7 |

Table 2.4-12 shows the maximum value of simulation results for SL non-contiguous CA with architecture #1-1, #1-2, and #2-2 in Table 2.2-13 respectively considering B and frequency of FIM3,low_block,low & FIM3,high_block,high.

Here,

SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}.

SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}.

Table 2.4-12 : PC2 SSB AMPR simulation results for SL non-contiguous CA

| Carrier Frequency Combination [MHz] |  | AMPR (dB) for IM3 frequency |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SEMfreq_-13/-13A (SCS[kHz]) |  | SEfreq_-30/-30A(SCS[kHz]) |  |
|  |  | 15 | 30 | 15 | 30 |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 1x26dBm | 20.8 | 18.0 | 22.1 | 19.4 |
|  | 2x23dBm + 1LO | 21.1 | 18.3 | 22.1 | 22.5 |
|  | 2x23dBm + 2LO | 12.1 | 8.3 | 16.7 | 18.1 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 1x26dBm | 17.2 | 16.2 | 21.7 | 18.9 |
|  | 2x23dBm + 1LO | 17.3 | 17.4 | 22.3 | 19.5 |
|  | 2x23dBm + 2LO | 7.4 | 7.4 | 12.6 | 8.8 |

The AMPR can be proposed as Table 2.4-13 based on the simulation results when considering implementation margin.

Table 2.4-13 : PC2 SSB AMPR for SL non-contiguous CA

| Carrier Frequency Combination [MHz] |  | AMPR (dB) for IM3 frequency |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SEMfreq_-13/-13A (SCS[kHz]) |  | SEfreq_-30/-30A (SCS[kHz]) |  |
|  |  | 15 | 30 | 15 | 30 |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 1x26dBm | ≤ 23.5 | ≤ 20.5 | ≤ 24.5 | ≤ 24.5 |
|  | 2x23dBm + 1LO | ≤ 23.5 | ≤ 21.5 | ≤ 24.5 | ≤ 25.0 |
|  | 2x23dBm + 2LO | ≤ 15.5 | ≤ 15.0 | ≤ 19.0 | ≤ 20.5 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 1x26dBm | ≤ 19.5 | ≤ 18.5 | ≤ 24.0 | ≤ 21.5 |
|  | 2x23dBm + 1LO | ≤ 20.0 | ≤ 20.0 | ≤ 25.0 | ≤ 22.5 |
|  | 2x23dBm + 2LO | ≤ 14.0 | ≤ 14.0 | ≤ 16.0 | ≤ 15.5 |
| NOTE : SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}. SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}. |  |  |  |  |  |

From Table 2.4-12 and Table 2.4-13, the following is observed.

Observation 4: For PC2 S-SSB, A-MPR of SCS 30kHz is larger than or equal to that of SCS 15kHz for SEMfreq_-30/-30A  in Case1 of carrier frequency combination, otherwise, it is smaller.

From the obsevation, S-SSB A-MPR needs to be specified with large A-MPR of different SCSs to simplify the requirement.

And, considering proposal 2 and 3, PC3 S-SSB A-MPR can be proposed as follows.

Proposal 14: For PC2 S-SSB A-MPR without indicating dualPA, consider A-MPR of 2x23dBm+1LO in Table 2.4-14.

Proposal 15: For PC2 S-SSB A-MPR with indicating dualPA, consider A-MPR of 2x23dBm+2LO in Table 2.4-14.

Table 2.4-14 : PC2 SSB AMPR for SL non-contiguous CA

| Carrier Frequency Combination [MHz] |  | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
|  |  | SEMfreq_-13/-13A (SCS[kHz]) | SEfreq_-30/-30A (SCS[kHz]) |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 1x26dBm | ≤ 23.5 | ≤ 24.5 |
|  | 2x23dBm + 1LO | ≤ 23.5 | ≤ 25.0 |
|  | 2x23dBm + 2LO | ≤ 15.5 | ≤ 20.5 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 1x26dBm | ≤ 19.5 | ≤ 24.0 |
|  | 2x23dBm + 1LO | ≤ 20.0 | ≤ 25.0 |
|  | 2x23dBm + 2LO | ≤ 14.0 | ≤ 16.0 |
| NOTE : SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}. SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}. |  |  |  |

Proposal 16: Capture all A-MPR results in TP.

Conclusion

In this contribution, we provided our view on A-MPR requirements with NS_33 for SL intra-band non-contiguous CA. Based on those, we propose as follows.

< A-MPR >

Proposal 1: Specify A-MPR requirements for NS_33 to meet -37dBm/100kHz and -30dBm/1MHz.

Proposal 2: Specify A-MPR requirements with and without indicating dualPA.

Proposal 3: Specify A-MPR requirement without indicating dualPA with 2PA+1LO architecture.

< PC3 PSSCH/PSCCH A-MPR >

Proposal 4: For PC3 PSSCH/PSCCH A-MPR without indicating dualPA, consider A-MPR in Table 2.2-11.

Table 2.2-11 : PSSCH/PSCCH AMPR for SL non-contiguous CA with 2x20dBm PA + 1LO

| Carrier Frequency Combination | B | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
| [MHz] |  | SEMfreq_-13/-13A | SEfreq_-30/-30A |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 0 ≤ B < 5.04 | ≤ 18.0 | ≤ 20.0 |
|  | 5.04 ≤ B < 10.08 | ≤ 16.5 | ≤ 19.5 |
|  | 10.08 ≤ B |  | ≤ 18.0 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 0 ≤ B < 5.04 | ≤ 17.0 | ≤ 18.0 |
|  | 5.04 ≤ B < 10.08 | ≤ 13.5 | ≤ 16.5 |
|  | 10.08 ≤ B |  | ≤ 15.5 |
| NOTE : SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}. SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}. |  |  |  |

Proposal 5: For PC3 PSSCH/PSCCH A-MPR with indicating dualPA, consider A-MPR in Table 2.2-12.

Table 2.2-12 : PSSCH/PSCCH AMPR for SL non-contiguous CA with 2x20dBm PA + 2LO

| Carrier Frequency Combination | B | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
| [MHz] |  | SEMfreq_-13/-13A | SEfreq_-30/-30A |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 0 ≤ B < 5.04 | ≤ 13.5 | ≤ 18.0 |
|  | 5.04 ≤ B < 10.08 | ≤ 12.5 | ≤ 17.5 |
|  | 10.08 ≤ B |  | ≤ 16.5 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 0 ≤ B < 5.04 | ≤ 13.0 | ≤ 14.0 |
|  | 5.04 ≤ B < 10.08 | ≤ 10.5 | ≤ 12.5 |
|  | 10.08 ≤ B |  | ≤ 11.5 |
| NOTE : SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}. SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}. |  |  |  |

< PC2 PSSCH/PSCCH A-MPR >

Proposal 6: For PC2 PSSCH/PSCCH A-MPR without indicating dualPA, consider A-MPR in Table 2.2-21.

Table 2.2-21 : PSSCH/PSCCH AMPR for SL non-contiguous CA with 2x23dBm PA + 1LO

| Carrier Frequency Combination | B | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
| [MHz] |  | SEMfreq_-13/-13A | SEfreq_-30/-30A |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 0 ≤ B < 5.04 | ≤ 20.0 | ≤ 22.5 |
|  | 5.04 ≤ B < 10.08 | ≤ 18.5 | ≤ 22.0 |
|  | 10.08 ≤ B |  | ≤ 20.0 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 0 ≤ B < 5.04 | ≤ 18.5 | ≤ 19.5 |
|  | 5.04 ≤ B < 10.08 | ≤ 15.0 | ≤ 18.5 |
|  | 10.08 ≤ B |  | ≤ 16.5 |
| NOTE : SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}. SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}. |  |  |  |

Proposal 7: For PC2 PSSCH/PSCCH A-MPR with indicating dualPA, consider A-MPR in Table 2.2-22.

Table 2.2-22 : PSSCH/PSCCH AMPR for SL non-contiguous CA with 2x23dBm PA + 2LO

| Carrier Frequency Combination | B | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
| [MHz] |  | SEMfreq_-13/-13A | SEfreq_-30/-30A |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 0 ≤ B < 5.04 | ≤ 15.0 | ≤ 20.0 |
|  | 5.04 ≤ B < 10.08 | ≤ 13.0 | ≤ 19.5 |
|  | 10.08 ≤ B |  | ≤ 18.0 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 0 ≤ B < 5.04 | ≤ 14.0 | ≤ 15.5 |
|  | 5.04 ≤ B < 10.08 | ≤ 11.5 | ≤ 13.0 |
|  | 10.08 ≤ B |  | ≤ 12.0 |
| NOTE : SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}. SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}. |  |  |  |

< PC3 PSFCH A-MPR >

Proposal 8: For PC3 PSFCH A-MPR without indicating dualPA, consider A-MPR of 2x20dBm+1LO in Table 2.3-8.

Proposal 9: For PC3 PSFCH A-MPR with indicating dualPA, consider A-MPR of 2x20dBm+2LO in Table 2.3-8.

Table 2.3-8 : PC3 PSFCH AMPR for SL non-contiguous CA

| Carrier Frequency Combination [MHz] |  | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
|  |  | SEMfreq_-13/-13A (SCS[kHz]) | SEfreq_-30/-30A (SCS[kHz]) |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 1x23dBm | ≤ 18.0 | ≤ 22.5 |
|  | 2x20dBm + 1LO | ≤ 18.0 | ≤ 22.5 |
|  | 2x20dBm + 2LO | ≤ 16.5 | ≤ 22.5 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 1x23dBm | ≤ 18.0 | ≤ 18.0 |
|  | 2x20dBm + 1LO | ≤ 18.0 | ≤ 18.0 |
|  | 2x20dBm + 2LO | ≤ 17.0 | ≤ 16.0 |
| NOTE : SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}. SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}. |  |  |  |

< PC2 PSFCH A-MPR >

Proposal 10: For PC2 PSFCH A-MPR without indicating dualPA, consider A-MPR of 2x23dBm+1LO in Table 2.3-14.

Proposal 11: For PC2 PSFCH A-MPR with indicating dualPA, consider A-MPR of 2x23dBm+2LO in Table 2.3-14.

Table 2.3-14 : PC3 PSFCH AMPR for SL non-contiguous CA

| Carrier Frequency Combination [MHz] |  | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
|  |  | SEMfreq_-13/-13A (SCS[kHz]) | SEfreq_-30/-30A (SCS[kHz]) |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 1x26dBm | ≤ 20.0 | ≤ 25.0 |
|  | 2x23dBm + 1LO | ≤ 20.0 | ≤ 25.0 |
|  | 2x23dBm + 2LO | ≤ 17.5 | ≤ 25.0 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 1x26dBm | ≤ 20.0 | ≤ 20.0 |
|  | 2x23dBm + 1LO | ≤ 20.0 | ≤ 20.0 |
|  | 2x23dBm + 2LO | ≤ 17.5 | ≤ 16.5 |
| NOTE : SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}. SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}. |  |  |  |

< PC3 S-SSB A-MPR >

Proposal 12: For PC3 S-SSB A-MPR without indicating dualPA, consider A-MPR of 2x20dBm+1LO in Table 2.4-8.

Proposal 13: For PC3 S-SSB A-MPR with indicating dualPA, consider A-MPR of 2x20dBm+2LO in Table 2.4-8.

Table 2.4-8 : PC3 SSB AMPR for SL non-contiguous CA

| Carrier Frequency Combination [MHz] |  | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
|  |  | SEMfreq_-13/-13A (SCS[kHz]) | SEfreq_-30/-30A (SCS[kHz]) |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 1x23dBm | ≤ 20.0 | ≤ 21.0 |
|  | 2x20dBm + 1LO | ≤ 21.0 | ≤ 21.5 |
|  | 2x20dBm + 2LO | ≤ 14.5 | ≤ 18.0 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 1x23dBm | ≤ 17.0 | ≤ 20.5 |
|  | 2x20dBm + 1LO | ≤ 17.5 | ≤ 21.0 |
|  | 2x20dBm + 2LO | ≤ 12.0 | ≤ 15.0 |
| NOTE : SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}. SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}. |  |  |  |

< PC2 S-SSB A-MPR >

Proposal 14: For PC2 S-SSB A-MPR without indicating dualPA, consider A-MPR of 2x23dBm+1LO in Table 2.4-14.

Proposal 15: For PC2 S-SSB A-MPR with indicating dualPA, consider A-MPR of 2x23dBm+2LO in Table 2.4-14.

Table 2.4-14 : PC2 SSB AMPR for SL non-contiguous CA

| Carrier Frequency Combination [MHz] |  | AMPR (dB) for IM3 frequency |  |
| --- | --- | --- | --- |
|  |  | SEMfreq_-13/-13A (SCS[kHz]) | SEfreq_-30/-30A (SCS[kHz]) |
| {5860, 5880}, {5860, 5890}, {5860, 5900}, {5860, 5910}, {5860, 5920}, {5870, 5920}, {5880, 5920}, {5890, 5920}, {5900, 5920} | 1x26dBm | ≤ 23.5 | ≤ 24.5 |
|  | 2x23dBm + 1LO | ≤ 23.5 | ≤ 25.0 |
|  | 2x23dBm + 2LO | ≤ 15.5 | ≤ 20.5 |
| {5870, 5890}, {5880, 5900}, {5890, 5910}, {5870, 5900}, {5870, 5910}, {5880, 5910} | 1x26dBm | ≤ 19.5 | ≤ 24.0 |
|  | 2x23dBm + 1LO | ≤ 20.0 | ≤ 25.0 |
|  | 2x23dBm + 2LO | ≤ 14.0 | ≤ 16.0 |
| NOTE : SEMfreq_-13 is applicable for carrier frequency combination of {5860, 5880}, {5870, 5890}, {5880, 5900}, {5890, 5910}, and {5900, 5920}. SEMfreq_-13A is applicable for carrier frequency combination of {5870, 5890}, {5880, 5900}, and {5890, 5910}. |  |  |  |

< TP >

Proposal 16: Capture all A-MPR results in TP.

- References

[1] R4-2420333, “WF on NR sidelink intra-band NC CA”, OPPO, LG Electronics