3GPP TSG-RAN WG4 Meeting # 114bis	                                      R4-2503603

Wuhan, CN, April 7 – 11, 2025

Agenda Item: 7.27.2.1

Source: MediaTek Inc.

Title: Discussion on event triggered L1 report for LTM

Document for: Discussion

- Introduction

RAN#105 agreed on the revised WID “NR mobility enhancements Phase 4” [1]. According to WID, RAN4 is supposed to define requirements for the following measurement enhancements:

- Event triggered L1 report,

- CSI-RS L1 measurements,

- CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch.

Last meeting, there were some discussions [2]. In the next, we would like to provide our views on the open issues of event triggered L1 report.

- Discussion

2.1 Capabilities for Support of Event Triggering L1 Reporting

Issue 2-4: whether to introduce/update requirements for reporting criteria per measurement category Candidate options: - Option 1: Yes (Xiaomi, CMCC, CATT, MTK, ZTE, OPPO, CTC, E///) - Option 1a: RAN4 to update intra-frequency and inter-frequency measurement reporting criteria requirements for SSB and CSI-RS based L1-RSRP with the same Ecat. (OPPO) - Option 1b: Number of supported L1 events is to be added on top of the currently supported L3 events. Details FFS. (Nokia) - Option 2: Open to let RAN1 discuss the number of events supported by UE at first. (MTK) - Option 2a: RAN4 to align common understanding that the existing number of reporting criteria per category is defined for L3 measurement and reporting only. For L1 measurements, the number of events to be supported by UE shall be discussed in RAN1 first. (vivo)

There was proposal to let RAN1 discuss the number of events supported by UE at first. We are open to this.

Proposal 1: Open to let RAN1 discuss the number of events supported by UE at first.

2.2 L1 event triggered reporting delay

Last meeting, RAN4 further discussed how to define L1 event triggered reporting delay and reached the following agreement. To us, Option 1 and Option 2 are the same. We slightly prefer Option 1 which is much clearer.

Issue 2-2: Event triggered reporting delay for L1 reporting Agreement: - Except for periodic MR: - Option 1: The event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event. - The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0. - Option 2: The event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period corresponding to the Cell(s) which triggers the event on the cells corresponding to the event. - The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0. - For periodic MR: - Option 1: For the first report after condition is met, the event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event. - RAN4 define delay requirement for the first report after condition is met. After that, UE follow the network configuration on the reporting interval. FFS how to capture this agreement into the CR. - Option 2: For the first report after condition is met, the event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period corresponding to the Cell(s) which triggers the event on the cells corresponding to the event. - RAN4 define delay requirement for the first report after condition is met. After that, UE follow the network configuration on the reporting interval. FFS how to capture this agreement into the CR.

Proposal 2: For L1 event triggered reporting delay, prefer the wording:

- Except for periodic MR:

  - The event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event.

    - The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0.

- For periodic MR:

  - For the first report after condition is met, the event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event.

    - RAN4 define delay requirement for the first report after condition is met. After that, UE follow the network configuration on the reporting interval. FFS how to capture this agreement into the CR.

2.3 Impact on other LTM requirements

Issue 2-10: Impact on other LTM requirements Tentative agreement: - UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, [and leaving condition of the event is configured and corresponding condition is not met], the related cell switch delay and TCI state activation requirements are applicable. - Further discuss how to capture this in RRM spec. Other proposals for further discussion: - Proposal: RAN4 to define TCI state known condition considering the L3 and L1 event triggered reporting.

Last meeting, we discussed the impact of event-triggered L1 report on other LTM requirements and reached the above progress. Initially, we are open to extend the cell switch delay and TCI state activation requirements to more reasonable scenarios. But after further thinking, we find another point that may need to be discussed. In FR2, when NW activates some TCI state(s), UE will prioritize L1-RSRP measurement on those TCI state(s) and may never measure the beam which is not activated. So UE may stop measuring the beam if NW activates some other TCI state(s) after event-triggered L1 report. In this case, the above optimization seems not workable.

Observation 1: UE may stop measuring the beam that triggers L1 report if NW activates some other TCI state(s) after event-triggered L1 report, which may result the reported TCI state unknown.

- Summary

In this paper, we provide some views on event triggered L1 report in R19 mobility. We have the following proposals:

Proposal 1: Open to let RAN1 discuss the number of events supported by UE at first.

Proposal 2: For L1 event triggered reporting delay, prefer the wording:

- Except for periodic MR:

  - The event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event.

    - The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0.

- For periodic MR:

  - For the first report after condition is met, the event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event.

    - RAN4 define delay requirement for the first report after condition is met. After that, UE follow the network configuration on the reporting interval. FFS how to capture this agreement into the CR.

Observation 1: UE may stop measuring the beam that triggers L1 report if NW activates some other TCI state(s) after event-triggered L1 report, which may result the reported TCI state unknown.

4 Reference

RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom.

R4-2502601, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple, RAN4#114.