3GPP TSG-RAN WG4 Meeting #114bis	R4-2503620

Wuhan, China, April 7th – April 11th, 2025

Agenda item:			7.21.4

Source:	Moderator (Nokia)

Title:	Topic summary for [114bis][210] FS_NR_AIML_Mob

Document for:	Information

# Introduction

This summary thread addresses the issues related to AI/ML mobility SI in RAN4. The document is split into 3 high-level topics which are further split into sub-topics and issues.

Moderator’s recommendations are to discuss following issue, listed in the order of priority.

- Issue 2-1-4: Need of Relative RSRP accuracy

- Issue 2-1-1 Relative RSRP accuracy definition

- Issue 2-1-3 Measured RSRP accuracy vs Predicted RSRP accuracy

- Issue 2-1-5: Ground truth definition for RSRP Accuracy for FR1

- Issue 2-1-7: Impacts of measurement error on prediction accuracy

- Issue 2-1-6: Impacts on beam/cell level measurements

- Issue 3-1-1: Prioritization of cases for event prediction

- Issue 3-1-2: Performance metric for event prediction use case

- Issue 3-1-3: Impact of measurement error for event prediction use case

- Issue 4-1-3 Multiple cells in the testing setup

- Issue 4-1-2: Prediction consistency in time domain

- Issue 4-2-2 Principle of FR2 testing

- Issue 4-2-1 Testing goal

# Topic #1: General Aspects

This section addresses general aspects and priorities of different scenarios and sub use-cases identified in the draft TR 38.744.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2504046 | vivo | Discussion on general aspects for AI mobility Proposal 1: RAN4 to study the three high-priority scenarios of scenario 2, 3 and 4 first for measurement prediction use-case. For scenario 6, the study can be considered after enough progress is made in AI/ML beam management. Proposal 2: RAN4 not to study RRM impact on RLF prediction in Rel-19 Proposal 3: Before performing enough evaluation and reaching an aligned conclusion prediction performance in terms of three sub-use cases, RAN4 not to have any deprioritization given to any sub-use cases |
| R4-2503350 | OPPO | Discussion on general aspects for AI mobility Proposal 1: RAN4 to only study scenarios 2,3,4 in this release. Proposal 2: RAN4 not to consider the combined prediction across multiple domains in this release. Proposal 3: RAN4 to consider all three sub-use cases with no prioritization. Proposal 4: Adopt the text proposal for TR 38.744 in [4] to capture the RAN4 study. |
| R4-2503928 | Ericsson | General discussion on AI/ML for mobility Proposal 1: RAN4 to conclude that the predicted measurements should at least meet the accuracy requirements that are defined for the non-AI/ML based RRM measurements for mobility purposes. Observation 1: If or which of the existing requirements should apply to spatial domain prediction remains to be understood. Proposal 2: RAN4 to identify requirements to be defined for data collection for network sided model for AI/ML based mobility. |
| R4-2503886 | Huawei, HiSilicon | Discussion on genereal aspects in AIML mobility Proposal 1: RAN4 will not start the study of RLF use-case in R19. Proposal 2: RAN4 to start the study of other scenarios than scenarios 2,3,4 if RAN2 achieves sufficient progress. Proposal 3: RAN4 to consider all three sub-use cases until RAN2 gives the priority. Proposal 4: RAN4 will study the following two potential testing options for defining the requirement of prediction accuracy. Pros and Cons need to be analyzed. • Option 1: Use system level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in link level channel model to define the requirement in RAN4. • Option 2: Use link level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in the same link level channel model to define the requirement in RAN4. Proposal 5: RAN4 will study the following two potential options for defining the testing and training dataset. • Option 1: Use the channel model and the sampling rules. • Option 2: Specify or partially specify the testing dataset in a sample-by-sample manner. |
| R4-2504288 | Samsung | Discussion on AI mobility general and scope Proposal 1: As a starting point, RAN4 to consider the Sub-use-case 2 for the study of scenarios 2, 3, and 4 for cell-level prediction. Proposal 2: RAN4 to deprioritize studying the impacts on requirements for beam level spatial domain prediction (i.e., FR2 to FR2 intra-frequency spatial domain scenario 6) in AI mobility Proposal 3: Accommodating with RAN2 guidance, there is no need for RAN4 to define RLF prediction requirement in Rel-19 |
| R4-2503850 | ZTE Corporation, Sanechips | Discussion on General Aspects of AIML Mobility Proposal 1: Align with RAN2 and RLF prediction will not be studied in Rel-19. Proposal 2: RAN4 should consider both serving cell and neighbor cell measurment prediction for intra-cell prediction. Specifically, serving cell (input) to predict the serving cell (output) and neigbour cell i (input) to predict the neigbour cell i (output). |
| R4-2503521 | Apple | General aspects on AI/ML for mobility in NR Observation 1: RRM measurement predictions provide several advantages, including reducing measurement gaps, increasing the periodicity of SMTC windows, minimizing scheduling and measurement restrictions, decreasing measurement delays, and shortening measurement gap lengths Observation 2: Further discussion is needed in RAN4 regarding future requirements for RLF design: - Should the indication be reported from the UE to the network, or only from the UE's lower layers to higher layers? - If a direct or indirect indication is sent from the UE to the network: - To be determined (FFS): How should such an RLF indication be triggered to the network? - For a direct indication, should the UE report a predicted RLF to the network when all measured and/or predicted downlink radio link quality values on all configured RLM-RS resources fall below a threshold (Qout)? - For an indirect indication, should the UE report all predicted SINR values with corresponding RS indices to the network when the measured and/or predicted downlink radio link quality on all configured RLM-RS resources is worse than Qout? Alternatively, should the UE indicate the predicted SINR with the relevant RS indices as long as at least one configured RLM-RS resource shows measured and/or predicted downlink radio link quality? Proposal 1: RAN4 should evaluate the combined benefits of RRM measurement predictions across multiple domains (such as spatial beam/cell, frequency, and temporal) and dimensions (across various cells and frequencies). Additionally, a working plan should be established to investigate scenarios that maximize these potential advantages. Proposal 2: For measurement event prediction, leverage AI/ML for temporal prediction of {TX, RX} Beam Pair Changes to optimize RRM measurement efficiency. To minimize RRM measurement overhead, the AI/ML model will predict the timing of beam pair changes, enabling the system to conserve resources by focusing measurements only during periods when beam changes are predicted. This approach eliminates the necessity for constant beam monitoring. |
| R4-2503774 | CMCC | Discussion on general part for AI/ML for mobility Proposal 1: it is proposed to firstly focus on scenario 2, 3, 4. If time allows, scenario 6 can be considered later. Proposal 2: it is proposed to consider all three sub-use cases. |
| R4-2503645 | OPPO | TP on RAN4 aspects for TR 38.744 |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1: General and Scope

Sub-topic description:

In this sub-topic the company proposals about priorities for AI/ML Mobility scenarios and sub-use cases are summarized

#### Issue 1-1-1: Radio Link Failure (RLF) prediction use case

- Proposals and Observations:

  - Proposal 1 (vivo, Samsung, ZTE): RAN4 not to study RRM impact on RLF prediction in Rel-19

- Recommended WF:

  - RAN4 doesn’t need to define requirements for RLF prediction use case.

#### Issue 1-1-2: Scenario priorities

- Prioritization of evaluation scenarios in RAN2 (Table 5.2.1-1 in TR 38.744 draft) for measurement prediction use-case.

| Scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

- Proposals and Observations:

  - Proposal 1 (vivo): RAN4 to study the three high-priority scenarios of scenario 2, 3 and 4 first for measurement prediction use-case. For scenario 6, the study can be considered after enough progress is made in AI/ML beam management.

  - Proposal 2 (OPPO): RAN4 to only study scenarios 2,3,4 in this release

  - Proposal 3 (Huawei): RAN4 to start the study of other scenarios than scenarios 2,3,4 if RAN2 achieves sufficient progress.

  - Proposal 4 (Apple): RAN4 should evaluate the combined benefits of RRM measurement predictions across multiple domains (such as spatial beam/cell, frequency, and temporal) and dimensions (across various cells and frequencies). Additionally, a working plan should be established to investigate scenarios that maximize these potential advantages.

  - Proposal 5 (CMCC): it is proposed to firstly focus on scenario 2, 3, 4. If time allows, scenario 6 can be considered later.

  - Proposal 6 (Samsung): RAN4 to deprioritize studying the impacts on requirements for beam level spatial domain prediction (i.e., FR2 to FR2 intra-frequency spatial domain scenario 6) in AI mobility

- Candidate Options:

  - Option 1: RAN4 to only study scenarios 2,3,4 in this release.

  - Option 2: In addition to scenario 2,3 and 4, RAN4 to start the study of scenario 6 if RAN2 achieves sufficient progress.

- Recommended WF:

  - RAN4 to discuss above listed candidate options.

#### Issue 1-1-3: Sub-use-case priorities

- RRM measurement prediction including intra and inter-frequency (UE sided and NW sided model). In RAN2, the following sub-use cases were agreed for RRM measurement prediction:

  - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

  - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

  - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

- Proposals and Observations:

  - Proposal 1 (vivo, OPPO, Huawei,CMCC): RAN4 to consider all three sub-use cases with no prioritization.

- Recommended WF:

  - RAN4 to consider all three sub-use cases with no prioritization.

#### Issue 1-1-4: TR framework

- Proposals and Observations:

  - Proposal 1 (OPPO): Adopt the text proposal for TR 38.744 in [4 R4-2503645] to capture the RAN4 study.

- Recommended WF:

  - Discuss the TR framework in R4-2503645.

#### Issue 1-1-5: Accuracy requirement for predicted measurements

- Proposals and Observations:

  - Proposal 1 (Ericsson): RAN4 to conclude that the predicted measurements should at least meet the accuracy requirements that are defined for the non-AI/ML based RRM measurements for mobility purposes.

  - Proposal 1 (Huawei): RAN4 to discuss whether to define separate prediction accuracy for each sub-use case or define one prediction accuracy requirement to cover 3 cell level sub-use cases.

- Recommended WF:

  - RAN4 to discuss above listed candidate options.

# Topic #2: RAN4 requirements for RRM Measurement prediction

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2503929 | Ericsson | On requirements for RRM measurement prediction for AI/ML based mobility Observation 1: Objective of the study item is to study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover. Observation 2: The scope of the study item is to reuse the existing mobility framework for L3-based mobility. Proposal 1: RAN4 to not consider the impact of L1 beam level measurement prediction at point A1, on RAN4 requirements unless agreed to be considered for AI/ML based mobility by RAN2. Proposal 2: RAN4 to consider the impact of L3 beam level measurement prediction at point E, on RAN4 requirements. Observation 3: For a fixed MRRT, prediction accuracy for temporal prediction case-B may differ based on the measurement skipping pattern within OW. Proposal 3: RAN4 to study the impact of measurement skipping pattern within the OW on RAN4 requirement for a fixed MRRT for temporal prediction case-B. Observation 4: Higher the correlation coefficient between the carriers of the input and output cells, higher is the accuracy of RRM measurements predicted for the output cell in frequency prediction case (scenario 3 in prioritized scenario). Observation 5: Clustering approach (measurement from multiple spatial cells is used for RRM measurement prediction of an output cell) provides higher prediction accuracy in comparison to the case where the measurement from single cell in one frequency is used as an input to predict the RRM measurement for an output cell in another frequency. Proposal 4: RAN4 to consider the impact of frequency relationship between the measured and predicted cells on the performance of frequency domain prediction of RRM measurement for mobility. Proposal 5: RAN4 to study requirement aspects for the following cases: • When measurement from single cell in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency. • When measurement from a group of cells in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR cell(s) in another carrier frequency(ies). Observation 6: RLF prediction is out of Rel. 19 study item scope. |
| R4-2504232 | Nokia | Study of RAN4 impacts for RRM measurement prediction Proposal 1: Beam 1 and Beam 2 are to be preferred from different cells to align with legacy schemes. Observation 1: The existing measurement reporting accuracy requirement metrics are evaluated over several measurements; in time domain case B, the evaluation may include both actual and predicted measurements. Proposal 2: In case of evaluating measurement reporting accuracy, both measured and predicted measurements periods, or a mix of the two, are evaluated the same way. Observation 2: In case of relative accuracy metric, several and potentially different prediction and/or actual measurement pairs are evaluated to compute the metric. Proposal 3: The existing relative accuracy metric can be applied to the relative accuracy of reported measurements with temporal domain case B RRM measurement prediction. Observation 3: L3 filtering prediction can limit UE configuration and correspondingly additional efforts required to train ML model. Proposal 4: RAN4 should wait for RAN2's agreement on the progress of predicting filtered RSRP values C or D, before proceeding further. Observation 4: Unfiltered prediction can be beneficial as UE can be configured freely for any kind of filtering coefficients. Also, unfiltered prediction supports current test setup. Proposal 5: Unfiltered prediction is a preferable option due to flexible filtering configuration. Proposal 6: RAN4 to wait for RAN2 agreement on how filtering is implemented. Observation 5: In time-domain case A, predictions for two cells can also be compared for relative RSRP accuracy. Besides, cell 1 could be predicted and cell 2 could be measurement and vice versa. Proposal 7: In time domain case A, we might be comparing predictions of two cells for relative RSRP accuracy. Observation 6: Alternative 1 where the FR1 ground truth can be derived by TE based on the transmitted or received power (legacy approach) is not suitable for AIML Mobility due to the fading channel and due to the L1 filtering which is UE implementation specific and not known to the TE. Observation 7: Alternative 2 where the FR1 ground truth is derived based on the measurements reported by UE based on certain conditions, is more suitable for AIML Mobility since the channel variations and L1 filtering are already considered while reporting the measurements from UE. Proposal 8: Different issues that may impact the FR1 ground truth derivation are listed in the table below: Issues; Impact to Alternative 1; Impact to Alternative 2. 1.; The information at TE side, on the timing UE performs and/ or finishes the measurement and/or prediction; YES. TE may not be aware of the exact time period of the prediction performed at UE. This may lead to inaccuracies of ground truth derivation at TE due to dynamic channel conditions; NO. 2.; Impact of L3 filtering; L3 filtering methods, filtering co-efficient are configurable parameters, hence known to TE that can be utilized for deriving the ground truth. However, this may increase the complexity at TE when compared to legacy test setup.; NO. 3.; Number of samples of L1 filtering; The L1 filtering method, filtering co-efficient and the number of samples for filtering are not known by the TE since it is UE proprietary implementation, hence it will impact the derivation of ground truth at TE.; NO. 4.; Timing window, if it exists, where UE can’t measure the ground truth in Alt2; NO; Depending upon the UE capabilities, some UEs may not be able to measure and predict at the same time. However, this can be handled during testing as described above under Alternative 2 description.. 5.; Channel condition; With fading channel, the dynamic channel variations in addition to L1/L3 filtering will have to be considered at TE for deriving the ground truth which may not be feasible due to proprietary algorithms used at UE.; NO. 6.; Avoid UE cheat in the test; NO; UE cheating can be an issue in this alternative. 7.; S(I)NR variations in the test at different time instances (for Alt2); NO; One option is to measure at higher SINR level for more accurate ground truth and the SINR may be lower during the prediction with reference to the average SINR.. Proposal 9: Alternative 2 is the preferred option for ground truth definition for RSRP accuracy even though there are inaccuracies in the UE measurements, since it is more feasible option for the dynamic channel conditions. Observation 8: Temporal domain Case A doesn’t need new prediction delay requirements since legacy measurement delay requirements should be sufficient. Proposal 10: RAN4 to further study the impact on legacy measurement delay requirements for Temporal domain Case B and Frequency domain cases. Observation 9: In all 3 agreed sub-use cases, L3 cell-measurement is predicted (generated) and considered for RRM. Proposal 11: RAN4 to focus first on L3 cell level RSRP (Point C) prediction and after that L3 beam-level prediction (Point E). Observation 10: The performance of AI/ML depends on the selection of sub-use cases used to predict cell-level measurement. Observation 11: According to RAN2 observations, L3 cell level RSRP (Point C) prediction based on actual L3 cell-level measurement result(s) (i.e., Sub-use case 2) provides the highest MAE accuracy in many companies’ simulations. Proposal 12: RAN4 first study performance metrics/KPIs for target cell L3 cell level RSRP prediction based on actual serving cell L3 cell-level measurement result(s) (i.e., Sub-use case 2). Proposal 13: RAN4 should study the measurement error modeling by using the tables below as a baseline. Table 1 RF and BB error modelling principle RF and BB modeling principle; Supporting companies. Both RF and baseband errors follow a Gaussian distribution.; X, Y, Z, U .... Baseband and RF errors follow different distributions.; A, B, C, D …. Table 2 BB error modeling Baseband error modeling; Supporting companies. Gaussian distribution; A, B …. Link-level simulation results; C, D …. Table 3 RF error modeling RF error modeling; Supporting companies. Uniform distribution; A …. Gaussian distribution; B …. Truncated Gaussian distribution; C …. Fixed value; D .... Table 4 RF error application How RF error is applied; Supporting companies. Independent for each Tx beam; A …. same for each Tx beam; B …. Independent for each module/panel; C …. same for each module/panel; D .... Observation 12: The number of possible test cases can increase considerably for a set of selected OW, PW and MRRT, accounting for multiple possible skipping patterns for a given MRRT value. Proposal 14: For temporal domain prediction case B, RAN4 to agree on pre-selected reference testing scenarios relating to OW, PW and MRRT as well as selected skipping patterns for a given MRRT value. Observation 13: The input data and characteristics of predicted measurement in inter-cell inter-frequency prediction may be different from temporal domain predictions. Proposal 15: RAN4 to study the minimum requirement/characteristic of the input and output of AI/ML model for RRM and testability assessment of inter-cell inter-frequency prediction domain scenarios. |
| R4-2503887 | Huawei, HiSilicon | Discussion on impacts for RRM measurement prediction Proposal 1: RAN4 to start the discussion on scenario 2, 3 and 4 with high priority. Scenario 6 (FR2 to FR2 intra-frequency spatial domain for the 1st goal) can be carried out after sufficient progress are achieved on high priority scenarios. Proposal 2: If new measurement quantity is introduced in AI/ML mobility, new requirements for data collection may be specified accordingly, otherwise no RRM requirement is observed. Proposal 3: For intra-frequency temporal domain prediction case A, observation window length (or called as prediction evaluation period) and prediction window length may need to be specified. Proposal 4: To guarantee prediction accuracy for FR2 intra-frequency temporal domain case A, limited doppler shift, limited PW length and proper OW length shall be set in testing. Proposal 5: For intra-frequency temporal domain prediction case B, RSRP prediction accuracy are to be defined under the side condition of MRRT with limited PW length. Proposal 6: For inter-frequency inter-cell frequency domain prediction, the side condition of frequency prediction (e.g., EPRE difference) and RSRP prediction accuracy may need to be specified. Proposal 7: The processing time of inference function may need to be considered in measurement reporting delay. Proposal 8: RAN4 can deprioritize the sub-use cases of L3 filtered beam-level measurement predication (Point E). Proposal 9: For 3 sub-use cases for cell-level RRM measurement prediction, RAN4 is to define measurement prediction accuracy at point C. Proposal 10: RAN4 to discuss whether to define separate prediction accuracy for each sub-use case or define one prediction accuracy requirement to cover 3 cell level sub-use cases. Observation 1: Fading channel is to be employed for AI mobility measurement prediction. Observation 2: For either intra-frequency temporal domain case B and case A, although the configuration of PW, OW and skipping pattern are known to TE, the specific position in time domain of UE obtaining a L3 predicted result is unknown to TE. Observation 3: The number of L1 measurement samples is unknown to TE. Proposal 11: If the ground truth in FR1 is defined based on the TE transmitted or reception power (Alter 1), the following aspects are obstacles: - The specific position in time domain of UE obtaining a L3 predicted result is unknown to TE. - The number of L1 measurement samples is unknown to TE. Proposal 12: The ground truth in FR1 can be defined based on the reported measurement value under certain conditions. - how to avoid UE cheating can wait the progress in R19 AI interface beam management test. Proposal 13: The relative RSRP prediction accuracy can be subdivided to two cases: - Relative RSRP accuracy between predicted and predicted = (reported predicted RSRP of cell 1/beam1 – reported predicted RSRP of cell 2/beam2) – (ground truth of RSRP of cell 1/beam1 – ground truth of RSRP of cell 2/beam2) - Relative RSRP accuracy between predicted and measured = (reported predicted RSRP of cell 1/beam1 – reported measured RSRP of cell 2/beam2) – (ground truth of RSRP of cell 1/beam1 – ground truth of RSRP of cell 2/beam2) RAN4 can further discuss whether the above two relative RSRP accuracy is different or not. Proposal 14: If the functionality/model activation, deactivation, or fallback to non-AI function are indicated by network, the activation/deactivation/fallback delay and corresponding side conditions would need to be specified. Proposal 15: If the functionality/model activation, deactivation, or fallback to non-AI function are autonomously performed by UE, the requirements would be related with monitoring requirements. Proposal 16: Wait for more progress on performance monitoring from RAN1 R19 AI/ML air interface. |
| R4-2503851 | ZTE Corporation, Sanechips | Discussion impacts on requirements of AI/ML mobility Proposal 1: RAN4 should discuss whether prediction accuracy requirement defined at point C could be used for point A1. Observation1: Both cell level measurement prediction and beam level measurement prediction are within the scope of RRM measurement prediction. And according to RAN2’s agreements, L3 filtered beam level prediction cases are lower priority and companies should focus on FR2-to-FR2 intra-frequency temporal domain prediction caseA (scenario 4). Proposal 2：RAN4 to deprioritize studying the impacts on requirements for L3 beam-level prediction, the definition of prediction accuracy for point E will correspondingly be given low priority. Proposal 3: Beam 1 and Beam 2 can be from same cell or different cells. For temproal prediction, measurement and prediction occur in the same cell, while for frequency prediction, measurement and prediction occur in different cells. Onservation 2: For FR2, the definiation of ground truth on R19 AI BM has been reused as baseline, that is, the ground truth of L3 RPRP is the reported L3 RSRP measurement under sufficient high SNR in FR2. Proposal 4: For FR1, the transmitted power can be used as baseline for the definition on ground truth of L3-RSRP, which is obtained as the legacy way of conducted test. Observation 3: If the definition of relative RSRP is completed according to the description of legacy, it will involve two prediction results, and the relationship between these two prediction results in temproal domain prediction needs to be clarified. Proposal 5: RAN4 should study the RRM impact of MRRT and MRRS, which involve the number of skipped measurement instances and total measurement instances. Observation 4: Compared with legacy measurement delay requirements, number of actual measured samples and frequency layers are different, which will impact measurement requirements. Proposal 6: RAN4 should discuss whether/how to update measurement delay requirements for AI/ML mobility. |
| R4-2503775 | CMCC | Discussion on RAN4 impacts for RRM measurement prediction Proposal 1: for relative RSRP prediction accuracy, it is proposed that Beam 1 and Beam 2 can be from same cell or different cells - For scenario 2 and 4, Beam 1 and Beam 2 are from same cell - For scenario 3, Beam 1 and Beam 2 are from different cell. Proposal 2: for relative RSRP prediction accuracy, ground truth is reported by UE at high SNR for FR2, FFS ground truth for FR1. Proposal 3: table 2 provide the comparison between Alternative 1 and Alternative 2 for ground truth definition for FR1. Table 2 Comparison between Alternative 1 and Alternative 2 for ground truth definition for FR1 Alternative 1 (Ground truth for FR1 will be based on the transmitted or reception power); Alternative 2 (Ground truth for FR1 will be based on the reported measurement value under certain conditions). Impact of L3 filtering; If L3 filtering is considered, how to perform L3 filtering is based on L3 filtering formula with filtering coefficient configured, and TE has no such information; N/A. Impact of L1 filtering; How to perform L1 filtering, e,g, the number of samples, whether sliding or non-sliding filtering in use, is up to UE implementation, and TE has no such information; N/A. Timing window; N/A; For Case B (target goal is to reduce measurement overhead), the time instance in prediction window will not be measured, which means UE cannot measure and report the ground truth.. Channel condition; N/A; For fading channel, the channel condition at T1 to get ground truth and the channel condition at T2 to predict is different, which result in mismatch. UE cheating issue; N/A; Since both the ground truth and predicted value are reported by UE, there may be cheating issue. Proposal 4: except L3 cell level RSRP (Point C) prediction, it is proposed that RAN4 to consider L1 beam-level (Point A1). And it is proposed that L3 beam-level (Point E) can be deprioritized based on RAN2 agreements. Proposal 5: it is proposed to discuss whether to introduce RAN4 requirements about MRRT and MRRS. Proposal 6: for RRM measurement prediction, the measurement delay requirements need to be defined/updated - For intra-frequency temporal domain prediction, UE will skip some measurement occasions, which means that only subset of measurement occasions are measured, as a result, SMTC and/or MGRP cannot be directly used in the measurement delay requirements as legacy For inter-frequency inter-cell frequency domain prediction, not all configured MOs need to be measuerd, which means that only the actually measured frequency layers are considered in the measurement delay requirements |
| R4-2504047 | vivo | Discussion on RAN4 impacts for RRM measurement prediction Proposal 1: The Predicted relative RSRP accuracy refers to - Predicted relative RSRP accuracy = (reported predicted RSRP of cell 1/beam1 – reported predicted RSRP of cell 2/beam2) – (ground truth of RSRP of cell 1/beam1 – ground truth of RSRP of cell 2/beam2) - The ground truth: - For FR1, the definition depends on the outcome of issue 2-1-2 - For FR12, it refers to the reported L3 RSRP measurement Observation 1: For the prediction case on frequency domain, the discussion and simulation of RAN2 is still limited to the scenario of single/multiple cells as input and a single cell as output. Proposal 2: Whether Relative RSRP Accuracy will ultimately be included as a formal KPI in the final RAN4 requirements can be further evaluated upon confirmation of scenario support - particularly for cases involving single/multiple cells as input and multiple cells as output." Proposal 3: For the ground truth definition of L3-RSRP for FR1, the analysis for Alt 1 and Alt 2 considering relevant aspects listed as follows: Aspect; Alt 1 Ground truth for FR1 will be based on the reception power; Alt 2 Ground truth for FR1 will be based on the reported measurement value under certain conditions. TE-side information & timing; The RSRP value at any time instance can be directly derived by TE without alignment issues.; Misalignment risk between predicted (X) and actual measurement time (Y). Proposal: Suggestion to firstly consider the case that the the interval duration and starting point between time instances in both observation and prediction windows are aligned. Impact of L3 filtering; Measured values are actual; no filtering-related problems.; Complexity varies (e.g., L1→L1, L1→L3, L3→L3). Proposal: Filtering can be omitted in tests to avoid filtering coefficient inconsistencies.. L1 filtering samples; Legacy UE implementation; not AI-specific.; Legacy UE implementation; not AI-specific.. Timing window constraints; Ground truth is always available in both Temporal Case A and B.; In Temporal Case B, ground truth may be missing if measurements are skipped. It Needs further analysis.. Channel conditions & SNR; Channel variations irrelevant; Synchronized observation/prediction windows mitigate channel/SNR variability.. |
| R4-2503787 | NTT DOCOMO, INC. | Discussions on RRM measurement impacts of AIML for mobility Proposal 1: The ground truth for relative RSRP prediction accuracy should be same as the ground truth definition for RSRP accuracy. Proposal 2: For FR2, the ground truth for relative RSRP prediction accuracy should be the reported L3 RSRP measurement under sufficient high SNR. Proposal 3: As same as current relative RSRP definition, beam 1 and beam 2 can be from both same cell and different cells. Proposal 4: The reported RSRP of cell2/beam2 in predicted relative RSRP accuracy should be both predicted and measured value considering mobility purpose. Proposal 5: Combination of both predicted and measured samples for reported RSRP of cell2/beam2 is not needed because the performance of this case should be the middle ground of only measured value case and only predicted value case. |
| R4-2503607 | MediaTek Inc. | Discussion on RAN4 impacts for RRM measurement prediction Proposal 1: For RRM measurement prediction, only use cell level RSRP (Point C) as the performance metric for all the three sub-use cases. Proposal 2: The test case for inter-frequency prediction can be designed as the following: - NW configures SMTC for both f1 (intra-f) and f2 (inter-f). MG is not configured. A4 event is configured to check the prediction accuracy on f2. The threshold of A4 is set as X dBm. Suppose the absolute prediction accuracy is defined as ±YdB. - TE calculates filtered L3 result on each SMTC occasion. - TE can know when there is a neighbour cell whose RSRP is with [X-Y, X+Y]. TE can also get to know the time range when there will be expected SR based on the configurations. If UE triggers report request during the expected time range, then it can be viewed as the prediction meets the accuracy requirements in this single test. Proposal 3: With the proposed test procedure, the mismatch between TE and UE on L1 filtering and L3 filtering can be reflected on the defined accuracy requirements or some additional margin of the test requirements. Proposal 4: How to avoid UE cheating should be resolved at first before concluding to use reported measurement results as ground truth. Observation 1: For scenario 3 (inter-frequency measurement prediction), it is possible to inform UE cell ID of the inter-frequency neighbor cell(s) to be predicted for collocated cases while may be not possible for non-collocated cases. Proposal 5: For scenario 3 (inter-frequency measurement prediction), the cells predicted but not measured recently are unknown. Proposal 6: Study when UE performs cell detection on the predicted cells, e.g., before model monitoring or during model monitoring. Proposal 7: Wait for more progress in AI/ML BM to study the impact on measurement report delay requirements for AI mobility. Proposal 8: Wait for more progress in AI/ML BM case 2 to study the impact on measurement delay requirements for AI mobility Scenario 2. Proposal 9: For Scenario 3, gap sharing mechanism among different inter-frequency layers should be considered for both for measurement and monitoring when more than one inter-frequency layers are involved. Proposal 10: Scenario 4 has no impact on measurement delay requirements. |
| R4-2503582 | LG Electronics Inc. | Discussion of RAN4 impacts for RRM measurement prediction Proposal 1: Beam 1 and Beam 2 can be from same cell or different cells. Proposal 2: For FR2 the ground truth can be get by reporting, and for FR1, it depends on test environment. If the test environment is conducted test and AWGN channel, then use the ideal value. Else the ground truth can be get by reporting. Proposal 3: The reported RSRP of cell2/beam2 should be predicted samples. Proposal 4: If the conducted test is considered with AWGN channel in FR1, then our preference is Alt 1. But if OTA test is considered or fading channel is considered, then our preference is Alt 2. Proposal 5: When Alt2 is adopted, it should be considered how to avoid the UE cheat in the test as follow. If TE has an environment set for the test in advance, and before proceeding with the test against UE, it receives measurement reporting from UE on all the elements in the environment set. Then the TE can know the correct answers for each environment in the set, and then randomly selects the environment in the set to perform the test. UE will not be able to cheating because it does not know which environment has been selected. |
| R4-2503308 | Xiaomi | Discussion on impacts on RAN4 requirement for RRM measurement prediction in AI mobility Observation 1: In legacy, for both periodic reporting and event triggered L3-RSRP reporting, UE will only report one latest measured L3-RSRP. Observation 2: From RAN2 agreement, several possible performance metrics for L3 RSRP prediction performance will be added into TR, e.g. Average L3 cell RSRP, last predicted point L3 cell RSRP. Observation 3: For AI based BM case-2, support to report largest N future time instance(s) in one report. The time instance information of the beam with the largest RSRP are additionally indicated in the report. Proposal 1: Add clarification that the agreed Absolute L3 Predicted RSRP Accuracy metric applies exclusively to reports containing a single L3-RSRP, which may represent: - A single instance (e.g., predicted RSRP at time t). - An average over a defined time window. Proposal 2: RAN4 should prioritize developing methodologies to align predicted L3-RSRP with ground truth RSRP at single time instances. Observation 4: When timing mismatch increases to 200ms, RSRP difference increases to 5.5dB. Observation 5: Prediction interval for sliding and non-sliding widow is different, which will have impact on degerming the reported time instance. Proposal 3: Timing between predicated L3-RSRP and ground truth L3-RSRP needs to be aligned, how to align timing is highly dependent on reporting scheme designed by RAN2. Proposal 4: If timing alignment is infeasible, incorporate additional margins into accuracy thresholds. Observation 6: L1 filtering sample number will have impact on deriving ideal L3-RSRP value from TE side. Observation 7: if 3 samples are used to derive L1 filtered RSRP, RSRP delta will be within 3dB for 90% cases when compared with that case that 5 samples are used. Proposal 5: L1 filtering number needs to be aligned, otherwise, extra margin needs to be considered. Observation 8: If the L3 filtering factor α is not zero, the L1/L3 filtering method will influence the derivation of the ideal L3-RSRP value from the TE side. Observation 9: At the same time T, ideal L3 filtered RSRP difference between non-sliding and sliding mode can be small than 1.8dB for k=4 in 90% cases. If K=0, no L3 filtering is applied and there is no RSRP difference. Proposal 6: L3 filtering impact can be removed by setting k=0, extra margin needs to be considered. Proposal 7: If the ground truth of L3-RSRP is reported by the UE, RAN4 should discuss whether and how to prevent the UE from providing cheating reports. Proposal 8: Observation window and prediction window defined in RAN2 will have RRM impact: - Observation window will have impact on measurement duration - Prediction window will have impact on scheduling restriction Observation 10: Observation window and prediction length is dependent on different L1/L3 filtering method. Observation 11: Observation window and prediction length is dependent on different sub-cases. Observation 12: Observation window and prediction length is dependent on different patterns. Proposal 9: For intra-frequency intra-cell prediction, suggest RAN4 to select some typical case/scenarios to study observation window and prediction window. Observation 13: Inter-frequency inter-cell RSRP prediction will have impact on CSSF reduction in measurement period. Proposal 10: For inter-frequency inter-cell RSRP prediction, RAN4 to study the impact on CSSF reduction in measurement period. |
| R4-2503522 | Apple | Study of RAN4 impacts for RRM measurement prediction for AIML Mobility in NR Observation 1: Scenario #2 for AI/ML in Release-19 Mobility is similar to Release-18 AI/ML beam prediction (Case-2), except that BM Case-2 allows set B to differ from set A. They are identical when set B = set A - Observation 2: In Temporal Domain Case B, if measurements in the prediction window are intentionally skipped, the corresponding L3-RSRP values for those time instances may not be available. This absence of reported measurements undermines their viability as ground truth for validation, necessitating alternative approaches or re-evaluation of measurement protocols for such scenarios. - Observation 3: Relative RSRP accuracy is essential for use cases where predictions rank or compare multiple cells (e.g., identifying the strongest neighbor cell for handover). However, RAN2’s current simulations and requirements focus on absolute RSRP accuracy (deviation between predicted and ground truth RSRP for a single cell). - Observation 4: Existing AI/ML models and test frameworks in RAN2 do not yet address scenarios where predictions output RSRP for multiple cells simultaneously - Proposal 1: RAN4 to study accuracy requirements for AI/ML predicted cell-level L3-filtered measurements for measurement reduction purpose. FFS on predicted beam level L3-filtered measurements (point E) Proposal 2: RAN4 to investigate the impact of measurement errors (RF and baseband) in AI/ML model input data on the accuracy of predicted measurements. Additionally, RAN4 should investigate whether RAN2 simulation results and conclusions regarding achievable prediction accuracy remain valid under such measurement errors for scenario 2. Proposal 3: For beam-level L1 requirements and given that L1 reporting is not supported within the L3 mobility framework, bypass L3 filtering and utilize a single Tx beam for testing purposes. (point A1 equivalent to C). Proposal 4: Investigate the translation of RAN2 KPI requirements into RAN4 RRM measurement accuracy requirements to assess the feasibility of the RAN2 study. This translation is essential for aligning performance metrics and ensuring that mobility optimization goals are met effectively. Additionally, explore the impact of RRM measurement requirements on prediction accuracy across various RRM measurement prediction cases and evaluation scenarios given side conditions of OW and PW, SNR. Proposal 5: The proposed cell-level RSRP prediction accuracy is defined as follows: - Absolute RSRP Accuracy for Cell-Level = predicted L3-RSRP of cell i - ideal received L3-RSRP of cell i - In FR2: The ideal received L3-RSRP is assumed to approximate the reported RSRP measurement, provided the SNR is sufficiently high for reliable RSRP accuracy. - In FR1: The ideal received L3-RSRP is taken to be equal to the transmitted power (assuming conducted test and AWGN channel during the test) Proposal 6: To support the increase of MGRP, RAN4 should investigate the feasibility of predicting measurement results for multiple cells per frequency layer. Additionally, it should examine the impact of adjusting the measurement gap period or selectively skipping gap occasions to reduce measurement gaps. Proposal 7: In Scenario 3 of RRM measurement prediction, further explore the decision to establish absolute and/or relative accuracy requirements for the predicted/generated L3 cell-level RSRP. Additionally, discuss methods for conducting relative measurements, such as those referencing to an anchor cell or calculating pairwise differences between cells. Proposals 8: Scenario 3 (Frequency Domain Prediction) can be extended to incorporate multi-cell and multi-frequency layer inputs, as well as time-domain samples as well as multiple cell outputs. This integration would enable measurement gap reduction by leveraging cross-layer and temporal correlations in the data. Proposal 9: RAN2’s assumption of collocated cells for frequency domain prediction may represent an unrealistic scenario. RAN4 to investigate frequency domain prediction methodologies for non-collocated cells, ensuring alignment with real-world deployment conditions. (scenario #3) Proposal 10: For frequency-domain prediction, RAN4 to evaluate whether an AI/ML model trained at one site can generalize effectively to another site. Additional conditions must be identified to ensure consistency between training and inference phases for generalization purposes. (scenario #3) Proposal 11: RAN4 should evaluate how unknown cell predictions impact AI/ML model accuracy (e.g., false positives/negatives in cell identification). Additionally, RAN4 must define criteria for classifying cells as known or unknown in AI/ML training and inference phases, such as time thresholds or signal stability metrics. Finally, RAN4 should assess whether AI/ML models can reliably predict measurements for unknown cells using historical or cross-frequency correlation data Proposal 12: Investigate the translation of RAN2 KPI requirements into RAN4 RRM measurement accuracy and prediction measurement accuracy requirements to assess the feasibility of the RAN2 study. The evaluation of the benefits provided by AI/ML-assisted mobility should consider (KPIs) related to handovers (e.g., ping-pong handovers, handover failures/radio link failures, time of stay, handover interruptions, prediction accuracy, and measurement reduction). These factors will determine the formulation of RAN4 requirements, as the primary objective is to enhance the robustness of mobility procedures and minimize failures. (scenario #4) Proposal 13: RAN4 to evaluate the assumptions for single-link temporal domain prediction accuracy (single-beam operation) versus tracking multiple beams, including potential beam changes (best Tx/Rx pair) within the observation/prediction window. Proposal 14: RAN4 to investigate the extension of Scenario 4 to a multi-link temporal domain prediction across different cells and optimum TX/RX pairs Proposal 15: For the multi-link RSRP temporal-prediction extension of Scenario RAN4 to investigate the impact of cell dependent measurement errors (different SINRs and channel conditions) to the prediction accuracy of the AI/ML models (scenario #4) Proposal 16: RAN4 to evaluate whether an AI/ML model trained at one site can be effectively applied to another site under Scenario 4 and its multi-link extension. Additional conditions must be defined to ensure consistency between training and inference phases for generalization purposes - Proposal 17: RAN4 to investigate the impact of measurement errors in the input data of AI/ML model on the accuracy of predicted measurements. RAN4 should leverage the simulation methodologies and evaluation results (from AI/ML BM Case 1) to assess how these measurement errors affect the accuracy of predictions related to mobility. RAN4 should validate any RAN2 conclusions in the presence of realistic measurements errors. Proposal 18 : RAN4 to investigate modelling measurement errors as a function of SINR. This analysis must account for variations in propagation environments across serving and neighboring cells, as measurement errors are inherently influenced by cell-specific SINR conditions. Proposal 19: Different cells may operate under distinct network conditions (e.g., transmit (TX) codebook configurations, beamwidths, TX implementation variations). RAN4 to investigate whether an AI/ML model trained on data from one cell (e.g., neighbor cell 1) can generalize effectively for inference on another cell. Critical conditions (e.g., beam alignment parameters, hardware compatibility, signal propagation characteristics) must be defined to ensure consistency between training and inference phases for robust generalization. Proposal 20: Define the ground truth of L3-RSRP as the measured RSRP from time t₀ to t₁ (observation window), under the condition that the channel characteristics during the prediction window (t₁ to t₂) remain identical to those in the observation window. The goal is to establish a reproducible and scenario-agnostic validation framework for AI/ML-based L3-RSRP prediction accuracy, particularly addressing limitations in scenarios where real-time UE measurements are unavailable or unreliable. - Proposed Methodology for Proposal 20: Ground truth definition - Channel-Consistency The channel conditions (e.g., large-scale fading, Doppler spread, interference levels, and multipath profiles) must remain unchanged between the observation window (t₀–t₁) and the prediction window (t₁–t₂). This ensures that the RSRP measured in the observation window (t₀–t₁) accurately reflects the conditions under which predictions are validated in t₁–t₂. - Measurement Protocol: - During the observation window (t₀–t₁), RSRP is measured continuously using UE or test equipment (TE) under controlled channel emulation. - In the prediction window (t₁–t₂), the AI/ML model’s predicted RSRP is compared against the pre-measured ground truth from t₀–t₁, assuming identical channel conditions. - Emulation and Reproducibility: - A channel emulator must replicate time-invariant channel parameters (e.g., path loss, shadowing, delay spread) across both windows. - For FR1, consistency in large-scale fading (e.g., path loss models) is prioritized. For FR2, beam-specific parameters (e.g., beam alignment, polarization) must also remain static. - Validation requires a correlation coefficient ≥ 0.95 between channel realizations in t₀–t₁ and t₁–t₂ to confirm stationarity. - Temporal Granularity: - The duration of t₀–t₁ and t₁–t₂ is defined based on UE mobility: - Low mobility (e.g., pedestrian): 20–50 ms windows. - High mobility (e.g., vehicular): 5–10 ms windows to minimize Doppler-induced variations. - Proposal 21: RAN4 to consider following steps for defining RSRP relative accuracy requirements: - Step 1: Prioritize Absolute RSRP Accuracy - Define absolute RSRP accuracy as the primary KPI for initial AI/ML model validation. - Metrics include: - Mean Absolute Error (MAE) between predicted and ground truth RSRP. - Tolerance thresholds (e.g., ±3 dB for FR1, ±4 dB for FR2). - Scope: Apply to current RAN2-supported scenarios (single-cell output). - Step 2: Monitor RAN2 Progress on Multi-Cell Output Scenarios - RAN4 to collaborate with RAN2 to investigate AI/ML models that predict RSRP for multiple cells(e.g., neighbor cell lists). - Identify use cases where relative RSRP accuracy becomes critical (e.g., handover optimization, carrier aggregation). - Step 3: Define Relative RSRP Accuracy Requirements (Conditional) - If RAN4/RAN2 standardizes multi-cell output prediction scenarios, RAN4 will: - Develop relative RSRP accuracy metrics, such as: - Rank order accuracy (correctly ranking cells by predicted RSRP). - Differential error thresholds (e.g., ±1 dB tolerance for relative differences between top-K cells). - Align test methodologies with multi-cell prediction dynamics (e.g., beam/cell correlation, interference coupling). Proposal 22: RAN4 should leverage the LCM progress and requirements discussed in R-19 WI related to AI/ML Air Interface and investigate enhancements to these LCM procedures to RRM for mobility. |
| R4-2503351 | OPPO | Study of impacts on RAN4 requirements for RRM measurement prediction Proposal 1: For all three sub-use cases, RAN4 to define prediction accuracy requirements and testing for L3 cell level measurements at point C. Proposal 2: RAN4 to evaluate and decide whether to define separate or unified requirements for different sub-use cases during WI stage. Proposal 3: For both intra and inter frequency prediction, the relative accuracy is defined as below, where the definition of ground truth can refer to the definition for absolute accuracy. - Predicted relative RSRP accuracy = (reported predicted RSRP of cell 1– reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1– ground truth of RSRP of cell 2) Proposal 4: Use Alt2(i.e., reported measurement value under certain conditions) as baseline to define the ground truth for prediction accuracy. Proposal 5: If the definition of ground truth cannot be decided in this meeting, list the advantages/disadvantages in the TR and down-select in WI stage. Proposal 6: RAN4 to consider certain channel model, beam consolidation configuration and L3 filter configuration as possible side conditions when defining the accuracy requirements and testing. Proposal 7: RAN4 to consider the impact of correlation coefficient between two frequency layers and the cluster approach on inter-frequency measurement prediction accuracy requirements. Observation 1: Even with a relatively large measurement error, for temporal case A and case B, the degradation due to measurement error is not significant. Proposal 8: RAN4 not to interpose the feasibility discussion in RAN2 but can consider the impact of measurement error on accuracy requirements in RAN4 during WI stage similar as AI/ML for NR Air Interface. Proposal 9: RAN4 to focus on the possible requirements to be defined and the testability discussion for AI mobility during SI stage. Proposal 10: RAN4 to discuss the measurement period requirements for intra-frequency measurement temporal case A considering the impact of OW or PW length. Proposal 11: RAN4 to discuss the measurement period requirements for intra-frequency measurement temporal case B considering the impact of skipped measurement instances, e.g., based on MRRT values. Proposal 12: RAN4 to discuss the measurement period requirements for inter-frequency measurement prediction considering the impact of skipped measurement frequencies or skipped measurement instances which may not need to be considered in the CSSF calculation. Proposal 13: RAN4 to investigate the possibility of updates on scheduling restrictions, measurement capabilities or definition of known conditions due to measurement prediction. |
| R4-2504287 | Samsung | General discussion on AI mobility regarding RAN4 requirements impact Proposal 1: - RAN4 to prioritize studying the impacts on requirements for L3 cell level RSRP (Point C) prediction - RAN4 to deprioritize studying the impacts on requirements for L3 beam-level (Point E) prediction, aligning with RAN2 prioritization Proposal 2: - If it is proposed to consider L1 beam level prediction, RAN4 to study the impacts on requirements for L1 beam level RSRP (Point A1) prediction - FFS: Whether to/How to define an additional prediction accuracy requirement on top of the requirement at C Proposal 3: RAN4 to discuss the RRM impact of measurement gap reduction in measurement period/ scheduling restriction and CSSF enhancement for inter-frequency prediction Proposal 4: For the RRM impact on prediction window, at least the following may need to be discussed - Scheduling restriction - Measurement latency for MO (s), MO (s) reduction Proposal 5: RAN4 to study if HO requirements including delay, interruption, side conditions and accuracy are needed to be updated for AI/ML-assisted mobility, temporal domain prediction case A can be taken as starting point considering verified handover decision option 3 and 2 Proposal 6: LCM related requirements and functional tests should to be defined based on the specific AI/ML mobility use cases and sub-use cases - The LCM procedures agreed in AI/ML for NR air interface RRM need to be considered as starting point - Prefer to use the legacy framework for RRC/MAC-CE/DCI based core requirements as the baseline for LCM procedures |
| R4-2504524 | Qualcomm Inc. | Impact of AI based mobility on RRM measurement prediction Definition of ground truth for AI/ML-based RRM measurement prediction Observation 1: It is not reasonable to assume that the same L3-RSRP measurement result at Point C in Figure 9.2.4-1 of TS38.300 should be observed across all UEs, particularly when the channel evolves over time and the measurement reference signals are contaminated due to low SNR, UE mobility, inter-cell/inter-sector interference, etc. Observation 2: It is not reasonable to assume that the predicted strongest neighbor cell would have as high SNR as the predicted strongest beam from the serving cell. Considering the geographical location of the UE, where the AI/ML-based neighbor cell’s RSRP prediction is of interest, is where the UE is not close to the serving cell but near the cell boundary, even the serving cell’s SNR is not expected to be high enough Observation 3: Considering that the models are not likely to be trained solely based on synthetic data generated by simulators, both the model input corresponding to the observation window and the ground truth of the model outputs corresponding to the prediction window will essentially be data sets generated based on actual UE measurement results, not so-called error-free ideal measurement results. Observation 4: The ground truth being the actual measurement result is also in line with the way NW and UE monitor the performance of the AI/ML-based RRM prediction. The effective KPI for performance monitoring is the difference between the prediction result and the actual measurement result on the performance monitoring reference signals, as there is no practical way of defining so-called error-free ideal measurement results. Observation 5: The UE’s L3-RSRP measurement accuracy performance will be ensured by the current requirements and test framework. Thus, the UE’s reported measurement results in more dynamic environments can serve as the benchmark for AI/ML-based L3-RSRP prediction accuracy performance requirements. - For testing, the following two-step test may thoroughly verify AI/ML-based L3-RSRP prediction performance: - Existing L3-RSRP Measurement Accuracy: - It is tested under static channel conditions (i.e., no RSRP variation during the entire test run), hence there is no ambiguity in the definition of the ground truth (or reference) RSRP. - With this, it can be claimed that the reported RSRP from the UE can serve as the ground truth for AI/ML-based L3-RSRP prediction in the second step below. - New Test Case for AI/ML-based L3-RSRP Prediction under Time-varying RSRP Conditions: - UE can be tested under multiple different RSRP variation profiles. - X% of the data set can be used for AI/ML model training. Note that it does not mean online training. - The rest of the data set can be used for AI/ML-based L3-RSRP prediction testing. - At time instance T0, UE reports AI/ML-based L3-RSRP prediction results based on previous observations. The reports correspond to future time instances. - UE and NW will be in sync in terms of how far into the future the measurement prediction results correspond to. - The accuracy/error can be defined as the difference between 'reported prediction results' and 'reported actual measurement results' corresponding to the same or closely timed instances. - The above is expected to allow UE to collect actual measurement results on/offline (measurement errors and UE implementation specifics are inherited in the data set) for model training and testing. Observation 6: For the same channel realization, it was observed that the IIR-filtered L3-RSRP can vary depending on the UE L3-RSRP measurement rates and the start position of the L3-RSRP measurement in the time domain. Therefore, a margin to accommodate uncertainties in the actual UE L3 measurement rate, etc., needs to be introduced on top of the ground truth if the ground truth for L3-RSRP is defined based on expected measurement results. Proposal 1: RAN4 to consider the following options for the definition of ground truth of L3-RSRP for the predicted absolute L3-RSRP accuracy (reported predicted L3-RSRP – ground truth of L3-RSRP). If the down selection needs to be made during the study item phase, RAN4 should consider Option 2-A, Option 2-B, and Option 1 in that order of priority. - Option 1: Ground truth based on expected measurement result (spec-defined expected L3-RSRP) - The ground truth of L3-RSRP predicted for the time instance X is derived based on the expected reception power of the reference signal at UE assuming the measurement rate defined in RRM spec, plus a margin to accommodate uncertainties in the actual UE L3 measurement rate. The value of the margin needs a further study, and it can vary for different measurement configurations, etc. - FFS on whether and how to accommodate the measurement error due to noisy channel, RF calibration error, UE Rx beamforming gain, etc. - Option 2-A: Ground truth based on measurement report from the UE under the SAME radio condition as the prediction-based L3-RSRP report (UE-reported L3-RSRP with noise) - The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X. - Y can be differently defined depending on factors such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc. - The radio condition such as S(I)NR remains the same during the target inference time instance and the report instance, i.e., from X – Y ms to X + Y ms. - To avoid the UE from cheating the test, TE can either create another cell with the same radio conditions to the extent possible (e.g., large-scale fading) as the target cell for the L3-RSRP prediction or run two rounds of testing with the same configuration: one for the prediction-based L3-RSRP report and the other for the actual measurement-based L3-RSRP report. - Option 2-B: Ground truth based on measurement report from the UE under the IDEAL radio condition (UE-reported L3-RSRP with noise-free) - The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X. - Y can be differently defined depending on factors such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc. - The radio condition such as S(I)NR for the measurement report is set such that the measurement result at the UE can be accurate enough. - Another round of tests would be needed for the L3-RSRP measurement report - RAN4 to strive to not differentiate the definition of the ground truth between FR1 and FR2. Performance metrics for AI/ML-based RRM measurement prediction Observation 7: The relative cell level strength metric is important for event-based L3-RSRP reports and conditional handovers, as they are widely used in the field for measurement gap configuration, handover preparation, and handover execution decisions. Among multiple event types, the most commonly used event type is Event-A3, which is defined based on the relative cell level strength between the serving cell and a neighbor cell. Proposal 2: RAN4 to consider a relative L3-RSRP as one of the performance metrics for AI/ML-based L3-RSRP prediction with the following definition: - Accuracy of predicted relative L3-RSRP = (reported predicted RSRP of cell 1/beam1 – reported RSRP of cell 2/beam2) – (ground truth of RSRP of cell 1/beam1 – ground truth of RSRP of cell 2/beam2) - Reported predicted relative L3-RSRP - Ground truth of relative L3-RSRP, where the ground truth of relative L3-RSRP is defined as follows: - For intra-frequency L3-RSRP, - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency, or - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell. - For inter-frequency, - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 and cell 2 are the target cells for AI/ML-based L3-RSRP inference. - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2. Observation 8: AI/ML-based RRM prediction is not expected to entirely replace actual UE measurements on neighbor cells, particularly during handover, due to the required downlink timing and frequency synchronization procedure with the target cell. If the AI/ML model can accurately predict the N strongest neighbor cells, the serving cell may reconfigure the measurement objects accordingly, allowing the UE to perform actual measurements on those cells. Observation 9: The RAN2 agreement also hints that the network would not solely rely on AI/ML-based RRM prediction results but would require the report of actual measurement results when deciding UE mobility. Observation 10: Based on system level simulation campaign conducted in RAN2 under a noise-free environment, AI/ML-based L3-RSRP prediction does not yet show decent accuracy. Proposal 3: RAN4 to consider the following as one of the performance metrics for AI/ML-based L3-RSRP prediction. If the down selection needs to be made during the study item phase, RAN4 should consider Top-K(%) and Top-K/1(%) in that order of priority. - Top-1 (%) : The percentage of ‘the Top-1 strongest cell is Top-1 predicted cell’ - Top-K/1 (%) : The percentage of ‘the Top-1 strongest cell is one of the Top-K predicted cells’ - Top-1/K (%) : The percentage of ‘the Top-1 predicted cell is one of the Top-K strongest cells’ - Top-K (%): The percentage of the maximum ground truth L3-RSRP among the Top-K predicted cells being larger than ‘the ground truth L3-RSRP of the strongest cell’ - X dB - Note: Top-1 strongest cell and Top-K strongest cells are determined by the metric of ground truth L3-RSRP. Sub-use-cases Proposal 4: RAN4 to consider all three sub-use cases, with higher prioritization given to sub-use case 2 and 3. - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s) - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) - FFS on whether and how to differentiate Sub-use case 2 and 3, e.g. one of or both sub-use case can be considered in defining RRM core and performance requirements Impacts from observation and prediction windows Proposal 5: UE speed, MRRT, and OW/PW lengths need to be considered when defining RRM requirements in the work item phase, unless it is identified that some specific issues are up for discussion during the study item phase. RRM prediction performance and impact of measurement error Proposal 6: RAN4 should not start simulation-based performance evaluation on measurement prediction due to measurement inaccuracy in the observation window during the study item phase. Instead, the group can further discuss additional aspects that might need to be taken into account in the WI phase, if approved, in addition to the outcome of the evaluations on AI/ML-based BM, e.g. - Whether and how to accommodate the impact of measurement inaccuracies for a cluster-based approach. - Whether and how to model the impact of imperfect RF calibration for inter-frequency RRM prediction. - Whether different aspects need to be considered for Case A and Case B in intra-cell/intra-frequency RRM prediction. - Note that the effective differences between Case A and Case B, from a UE measurement and prediction perspective, might be limited to the measurement rate within the observation window and the prediction range into the future. Case A and Case B may have different impacts on RRM requirements, particularly about measurement/scheduling restrictions. Potential impact on RRM requirements Proposal 7: RAN4 to review the existing RRM requirement and outline the identified potential impacts at a high level, e.g. - Common aspects: - Adjustment of the length of observation and prediction windows depending on UE in DRX state and/or cycle, serving cell RSRP, etc. - Measurement and scheduling restrictions in the prediction window. - Definition of known vs. unknown target cells for mobility and/or PSCell/SCell addition and activation. - Side conditions such as requirement applicability rules. - Overall budget in terms of mandatory capability on the number of cells and frequency layers for actual measurements and inference-based measurements. - Scenario 2: FR1 to FR1 intra-frequency temporal domain case B, Intra-cell - Uniform measurement approach for cells in the same frequency layer - Scenario 3: FR1 to FR1 inter-frequency, Inter-cell - CSSF - MG - Scenario 4: FR2 to FR2 intra-frequency temporal domain case A, Intra-cell Uniform measurement approach for cells in the same frequency layer |
| R4-2503411 | CATT | Discussion on impacts of AIML RRM measurement prediction on RRM requirements Proposal 1: RAN4 do not study radio link failure use case in Rel-19. Proposal 2: Beams in the definition of relative RSRP prediction accuracy can come from the same or different cells. Proposal 3: For FR1, ground truth is UE received power derived by TE. For FR2, ground truth is reported value and RAN4 wait for progress on AI/ML BM and reuse the agreement. Proposal 4: The reported RSRP of cell2/beam2 can be predicted results and proponents to clarify the necessity of a combination of predicted and measured samples. Proposal 5: It is not necessary to add limitations for the definition of relative RSRP prediction accuracy. Applicability conditions can be discussed for specific use cases. Proposal 6: RAN4 not to define strong cell prediction accuracy requirement. Proposal 7: RAN4 to deprioritize the study on impact of L3 beam-level measurement prediction at point E to align with RAN2 prioritization. Proposal 8: Consolidation/selection and layer 3 filtering should be included if RAN4 agree to define L3 cell level RSRP performance requirements. Proposal 9: RAN4 should focus on performance metrics and relevant requirements and postpone the discussion on LCM related issues. |
| R4-2503705 | Tejas | Study of impacts on RAN4 requirements for AIML mobility in NR Observation 1: Sub-use Case 2 and 3 with robust prediction model enhance handover performance through reliable predictions. Observation 2: Sub-use case 2 and 3 do not directly contribute to measurement reduction without additional strategies Observation 3: Intra-frequency temporal domain Case A has been categorized as low priority. Observation 4: larger gap between consecutive observation windows and the absence of overlap between the prediction and observation windows make Intra-frequency temporal domain Case B, Example 1, prediction less reliable. Observation 5: Predicting multiple inter-frequency measurements can be challenging due to varying channel conditions across different frequencies Observation 6: Ground truth can be determined easily and accurately from the TE, it provides a clear and consistent reference point for comparing the predicted L3-RSRP values reported by the UE during the test. Observation 7: L3 RSRP measurement results can be calculated. Proposal 1: RAN4 should prioritize the study and performance evaluation of Sub-use case 2 and Sub-use case 3. Proposal 2: RAN4 should study the mobility enhancements based on L3 cell-level measurement predictions in conjunction with the relaxation of L1 beam-level measurements. Proposal 3: Prioritize intra frequency temporal domain Case B example 2, for study and performance evaluation. Proposal 4: RAN4 should explore potential enhancements in inter-frequency and spatial domain measurement prediction by combining them with temporal domain prediction. Proposal 5: Update the definition of relative RSRP accuracy as follows: Predicted relative RSRP accuracy = Absolute L3 predicted RSRP accuracy of cell 1/beam 1 - Absolute L3 predicted RSRP accuracy of cell 2/beam 2. Proposal 6: RAN4 to consider both absolute and relative RSRP accuracy requirements for study. Proposal 7: Use the same ground truth definition of RSRP accuracy for relative RSRP accuracy Proposal 8: For FR1 conducted tests, the ground truth should be derived from the test equipment. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

In this sub-topic the company proposals about the RRM requirements for RRM Measurements prediction use case are summarized.

### Sub-topic 2-1: General and Scope

Sub-topic description:

#### Issue 2-1-1: Relative RSRP accuracy definition

The following agreement was reached in RAN4#114 meeting:

Agreement: - Predicted relative RSRP accuracy = (reported predicted RSRP of cell 1/beam1 – reported RSRP of cell 2/beam2) – (ground truth of RSRP of cell 1/beam1 – ground truth of RSRP of cell 2/beam2) - FFS: Beam 1 and Beam 2 can be from same cell or different cells. - FFS: whether ground truth can be reported or ideal - FFS: whether the reported RSRP of cell2/beam2 can be predicted or measured or a combination of both predicted and measured samples.

Many companies brought proposals on open points in the last agreement.

- Proposals and Observations:

  - Proposal 1 (Nokia): Beam 1 and Beam 2 are to be preferred from different cells to align with legacy schemes

  - Proposal 2 (Huawei): The relative RSRP prediction accuracy can be subdivided to two cases:

    - Relative RSRP accuracy between predicted and predicted = (reported predicted RSRP of cell 1/beam1 – reported predicted RSRP of cell 2/beam2) – (ground truth of RSRP of cell 1/beam1 – ground truth of RSRP of cell 2/beam2)

    - Relative RSRP accuracy between predicted and measured = (reported predicted RSRP of cell 1/beam1 – reported measured RSRP of cell 2/beam2) – (ground truth of RSRP of cell 1/beam1 – ground truth of RSRP of cell 2/beam2)

    - RAN4 can further discuss whether the above two relative RSRP accuracy is different or not.

  - Proposal 3 (ZTE): If the definition of relative RSRP is completed according to the description of legacy, it will involve two prediction results, and the relationship between these two prediction results in temproal domain prediction needs to be clarified.

  - Proposal 4 (CMCC): for relative RSRP prediction accuracy, it is proposed that Beam 1 and Beam 2 can be from same cell or different cells

    - For scenario 2 and 4, Beam 1 and Beam 2 are from same cell

    - For scenario 3, Beam 1 and Beam 2 are from different cell

  - Proposal 5 (vivo): Predicted relative RSRP accuracy = (reported predicted RSRP of cell 1/beam1 – reported predicted RSRP of cell 2/beam2) – (ground truth of RSRP of cell 1/beam1 – ground truth of RSRP of cell 2/beam2)

    - The ground truth:

      - For FR1, the definition depends on the outcome of issue 2-1-2

      - For FR12, it refers to the reported L3 RSRP measurement

  - Proposal 6 (vivo): Whether Relative RSRP Accuracy will ultimately be included as a formal KPI in the final RAN4 requirements can be further evaluated upon confirmation of scenario support - particularly for cases involving single/multiple cells as input and multiple cells as output.

  - Proposal 7 (NTT DOCOMO): As same as current relative RSRP definition, beam 1 and beam 2 can be from both same cell and different cells.

  - Proposal 8 (NTT DOCOMO): The reported RSRP of cell2/beam2 in predicted relative RSRP accuracy should be both predicted and measured value considering mobility purpose.

  - Proposal 9 (NTT DOCOMO): Combination of both predicted and measured samples for reported RSRP of cell2/beam2 is not needed because the performance of this case should be the middle ground of only measured value case and only predicted value case.

  - Proposal 10 (Apple): In Scenario 3 of RRM measurement prediction, further explore the decision to establish absolute and/or relative accuracy requirements for the predicted/generated L3 cell-level RSRP. Additionally, discuss methods for conducting relative measurements, such as those referencing to an anchor cell or calculating pairwise differences between cells.

  - Proposal 11 (Oppo): For both intra and inter frequency prediction, the relative accuracy is defined as below, where the definition of ground truth can refer to the definition for absolute accuracy.

    - Predicted relative RSRP accuracy = (reported predicted RSRP of cell 1– reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1– ground truth of RSRP of cell 2)

  - Proposal 12 (Qualcomm): RAN4 to consider a relative L3-RSRP as one of the performance metrics for AI/ML-based L3-RSRP prediction with the following definition:

    - Accuracy of predicted relative L3-RSRP = (reported predicted RSRP of cell 1/beam1 – reported RSRP of cell 2/beam2) – (ground truth of RSRP of cell 1/beam1 – ground truth of RSRP of cell 2/beam2)

    - Reported predicted relative L3-RSRP - Ground truth of relative L3-RSRP, where the ground truth of relative L3-RSRP is defined as follows:

    - For intra-frequency L3-RSRP,

      - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency, or

      - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell.

    - For inter-frequency,

      - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 and cell 2 are the target cells for AI/ML-based L3-RSRP inference.

      - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2.

  - Proposal 13 (CATT, LGE): Beams in the definition of relative RSRP prediction accuracy can come from the same or different cells.

  - Proposal 14 (Tejas): Update the definition of relative RSRP accuracy as follows:

    - Predicted relative RSRP accuracy = Absolute L3 predicted RSRP accuracy of cell 1/beam 1 - Absolute L3 predicted RSRP accuracy of cell 2/beam 2.

  - Proposal 15 (Ericsson):

    - Beam 1 and beam 2 in the predicted relative RSRP accuracy formula agreed in RAN4#114 can be from the same cell or different cells.

    - Reported RSRP of cell2/beam2 in the predicted relative RSRP accuracy formula agreed in RAN4#114 can be predicted or measured RSRP values.

    - Predicted relative RSRP accuracy for temporal prediction is defined as:

      - Predicted relative RSRP accuracy for temporal prediction = (reported predicted RSRP1 of a cell/beam – reported RSRP2 of the same cell/beam) – (ground truth of RSRP1 of a cell/beam – ground truth of RSRP2 of the same cell/beam).

      - Reported RSRP2 can be both predicted or measured.

- Candidate options:

  - Option 1: For both intra and inter frequency prediction, the relative accuracy is defined as below, where the definition of ground truth can refer to the definition for absolute accuracy.

    - Predicted relative RSRP accuracy = (reported predicted RSRP of cell 1– reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1– ground truth of RSRP of cell 2)

  - Option 2:

    - For intra-frequency L3-RSRP,

      - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency, or

      - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell.

    - For inter-frequency,

      - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 and cell 2 are the target cells for AI/ML-based L3-RSRP inference.

      - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2.

  - Option 3:

    - for relative RSRP prediction accuracy, it is proposed that Beam 1 and Beam 2 can be from same cell or different cells

      - For scenario 2 and 4, Beam 1 and Beam 2 are from same cell

      - For scenario 3, Beam 1 and Beam 2 are from different cell

  - Option 4: The relative RSRP prediction accuracy can be subdivided to two cases:

    - Relative RSRP accuracy between predicted and predicted = (reported predicted RSRP of cell 1/beam1 – reported predicted RSRP of cell 2/beam2) – (ground truth of RSRP of cell 1/beam1 – ground truth of RSRP of cell 2/beam2)

    - Relative RSRP accuracy between predicted and measured = (reported predicted RSRP of cell 1/beam1 – reported measured RSRP of cell 2/beam2) – (ground truth of RSRP of cell 1/beam1 – ground truth of RSRP of cell 2/beam2)

    - RAN4 can further discuss whether the above two relative RSRP accuracy is different or not.

- Recommended WF

- Discuss above listed Candidate Options

#### Issue 2-1-2: Strongest Cell Prediction accuracy

- Proposals and Observations:

  - Proposal 1 (Qualcomm): RAN4 to consider the following as one of the performance metrics for AI/ML-based L3-RSRP prediction. If the down selection needs to be made during the study item phase, RAN4 should consider Top-K(%) and Top-K/1(%) in that order of priority:

    - Top-1 (%) : The percentage of ‘the Top-1 strongest cell is Top-1 predicted cell’

    - Top-K/1 (%) : The percentage of ‘the Top-1 strongest cell is one of the Top-K predicted cells’

    - Top-1/K (%) : The percentage of ‘the Top-1 predicted cell is one of the Top-K strongest cells’

    - Top-K (%): The percentage of the maximum ground truth L3-RSRP among the Top-K predicted cells being larger than ‘the ground truth L3-RSRP of the strongest cell’ - X dB

    - Note: Top-1 strongest cell and Top-K strongest cells are determined by the metric of ground truth L3-RSRP.

- Recommended WF

  - Discuss the above proposal.

#### Issue 2-1-3: Measured RSRP accuracy vs Predicted RSRP accuracy

- Proposals and Observations:

  - Proposal 1 (Nokia): In case of evaluating measurement reporting accuracy, both measured and predicted measurements periods, or a mix of the two, are evaluated the same way.

  - Proposal 2 (OPPO): RAN4 to evaluate and decide whether to define separate or unified requirements for different sub-use cases during WI stage.

- Recommended WF

- RAN4 to decide during WI stage whether to define separate or unified requirements for measured and predicted reports.

#### Issue 2-1-4: Need of Relative RSRP accuracy

- Proposals and Observations:

  - Proposal 1 (vivo): Whether Relative RSRP Accuracy will ultimately be included as a formal KPI in the final RAN4 requirements can be further evaluated upon confirmation of scenario support - particularly for cases involving single/multiple cells as input and multiple cells as output

  - Proposal 2 (Apple): RAN4 to consider following steps for defining RSRP relative accuracy requirements:

    - Step 1: Prioritize Absolute RSRP Accuracy

      - Define absolute RSRP accuracy as the primary KPI for initial AI/ML model validation.

      - Metrics include:

        - Mean Absolute Error (MAE) between predicted and ground truth RSRP.

        - Tolerance thresholds (e.g., ±3 dB for FR1, ±4 dB for FR2).

      - Scope: Apply to current RAN2-supported scenarios (single-cell output).

    - Step 2: Monitor RAN2 Progress on Multi-Cell Output Scenarios

      - RAN4 to collaborate with RAN2 to investigate AI/ML models that predict RSRP for multiple cells(e.g., neighbor cell lists).

      - Identify use cases where relative RSRP accuracy becomes critical (e.g., handover optimization, carrier aggregation).

    - Step 3: Define Relative RSRP Accuracy Requirements (Conditional)

      - If RAN4/RAN2 standardizes multi-cell output prediction scenarios, RAN4 will:

      - Develop relative RSRP accuracy metrics, such as:

      - Rank order accuracy (correctly ranking cells by predicted RSRP).

      - Differential error thresholds (e.g., ±1 dB tolerance for relative differences between top-K cells).

    - Align test methodologies with multi-cell prediction dynamics (e.g., beam/cell correlation, interference coupling).

  - Proposal 3 (Tejas): RAN4 to consider both absolute and relative RSRP accuracy requirements for study

- Candidate Options:

  - Prioritize Absolute RSRP accuracy

  - Consider both Absolute RSRP and Relative RSRP accuracy

  - Wait for RAN2 progress to finalize if Relative RSRP accuracy is needed.

- Recommended WF

- Discuss above listed Candidate Options

#### Issue 2-1-5: Ground truth definition for RSRP Accuracy for FR1

- Background:

Following agreement was reached in RAN4#114 meeting:

- Agreement: - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions - Further discuss the advantages and disadvantages for Alt 1 and Alt 2 considering the aspects such as: - The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction - Impact of L3 filtering - Number of samples for L1 filtering - Timing window, if it exists, where UE can’t measure the ground truth in Alt2 - channel condition - Avoid UE cheat in the test - S(I)NR variations in the test at different time instances (for Alt2) - Other aspects are not precluded - Companies are encouraged to provide analysis on all or some of the aspects in the list

- Proposals and Observations:

  - Proposal 1 (Nokia): Different issues that may impact the FR1 ground truth derivation are listed in the table below:

|  | Issues | Impact to Alternative 1 | Impact to Alternative 2 |
| --- | --- | --- | --- |
| 1. | The information at TE side, on the timing UE performs and/ or finishes the measurement and/or prediction | YES. TE may not be aware of the exact time period of the prediction performed at UE. This may lead to inaccuracies of ground truth derivation at TE due to dynamic channel conditions | NO |
| 2. | Impact of L3 filtering | L3 filtering methods, filtering co-efficient are configurable parameters, hence known to TE that can be utilized for deriving the ground truth. However, this may increase the complexity at TE when compared to legacy test setup. | NO |
| 3. | Number of samples of L1 filtering | The L1 filtering method, filtering co-efficient and the number of samples for filtering are not known by the TE since it is UE proprietary implementation, hence it will impact the derivation of ground truth at TE. | NO |
| 4. | Timing window, if it exists, where UE can’t measure the ground truth in Alt2 | NO | Depending upon the UE capabilities, some UEs may not be able to measure and predict at the same time. However, this can be handled during testing as described above under Alternative 2 description. |
| 5. | Channel condition | With fading channel, the dynamic channel variations in addition to L1/L3 filtering will have to be considered at TE for deriving the ground truth which may not be feasible due to proprietary algorithms used at UE. | NO |
| 6. | Avoid UE cheat in the test | NO | UE cheating can be an issue in this alternative |
| 7. | S(I)NR variations in the test at different time instances (for Alt2) | NO | One option is to measure at higher SINR level for more accurate ground truth and the SINR may be lower during the prediction with reference to the average SINR. |

  - Proposal 2 (Nokia): Alternative 2 is the preferred option for ground truth definition for RSRP accuracy even though there are inaccuracies in the UE measurements, since it is more feasible option for the dynamic channel conditions.

  - Proposal 3 (Huawei): If the ground truth in FR1 is defined based on the TE transmitted or reception power (Alter 1), the following aspects are obstacles:

    - The specific position in time domain of UE obtaining a L3 predicted result is unknown to TE.

    - The number of L1 measurement samples is unknown to TE.

  - Proposal 4 (Huawei): The ground truth in FR1 can be defined based on the reported measurement value under certain conditions.

    - how to avoid UE cheating can wait the progress in R19 AI interface beam management test.

  - Proposal 5 (ZTE): For FR1, the transmitted power can be used as baseline for the definition on ground truth of L3-RSRP, which is obtained as the legacy way of conducted test.

  - Proposal 6 (CMCC): table below provide the comparison between Alternative 1 and Alternative 2 for ground truth definition for FR1.

Table  Comparison between Alternative 1 and Alternative 2 for ground truth definition for FR1

|  | Alternative 1 (Ground truth for FR1 will be based on the transmitted or reception power) | Alternative 2 (Ground truth for FR1 will be based on the reported measurement value under certain conditions) |
| --- | --- | --- |
| Impact of L3 filtering | If L3 filtering is considered, how to perform L3 filtering is based on L3 filtering formula with filtering coefficient configured, and TE has no such information | N/A |
| Impact of L1 filtering | How to perform L1 filtering, e,g, the number of samples, whether sliding or non-sliding filtering in use, is up to UE implementation, and TE has no such information | N/A |
| Timing window | N/A | For Case B (target goal is to reduce measurement overhead), the time instance in prediction window will not be measured, which means UE cannot measure and report the ground truth. |
| Channel condition | N/A | For fading channel, the channel condition at T1 to get ground truth and the channel condition at T2 to predict is different, which result in mismatch |
| UE cheating issue | N/A | Since both the ground truth and predicted value are reported by UE, there may be cheating issue |

  - Proposal 7 (vivo): For the ground truth definition of L3-RSRP for FR1, the analysis for Alt 1 and Alt 2 considering relevant aspects listed as follows:

| Aspect | Alt 1 Ground truth for FR1 will be based on the reception power | Alt 2 Ground truth for FR1 will be based on the reported measurement value under certain conditions |
| --- | --- | --- |
| TE-side information & timing | The RSRP value at any time instance can be directly derived by TE without alignment issues. | Misalignment risk between predicted (X) and actual measurement time (Y). Proposal: Suggestion to firstly consider the case that the the interval duration and starting point between time instances in both observation and prediction windows are aligned |
| Impact of L3 filtering | Measured values are actual; no filtering-related problems. | Complexity varies (e.g., L1→L1, L1→L3, L3→L3). Proposal: Filtering can be omitted in tests to avoid filtering coefficient inconsistencies. |
| L1 filtering samples | Legacy UE implementation; not AI-specific. | Legacy UE implementation; not AI-specific. |
| Timing window constraints | Ground truth is always available in both Temporal Case A and B. | In Temporal Case B, ground truth may be missing if measurements are skipped. It Needs further analysis. |
| Channel conditions & SNR | Channel variations irrelevant | Synchronized observation/prediction windows mitigate channel/SNR variability. |

  - Proposal 8 (MediaTek): How to avoid UE cheating should be resolved at first before concluding to use reported measurement results as ground truth.

  - Proposal 9 (LG Electronics): For FR2 the ground truth can be get by reporting, and for FR1, it depends on test environment. If the test environment is conducted test and AWGN channel, then use the ideal value. Else the ground truth can be get by reporting.

  - Proposal 10 (Xiaomi):

    - Timing between predicated L3-RSRP and ground truth L3-RSRP needs to be aligned, how to align timing is highly dependent on reporting scheme designed by RAN2.

    - If timing alignment is infeasible, incorporate additional margins into accuracy thresholds.

    - L1 filtering number needs to be aligned, otherwise, extra margin needs to be considered.

    - L3 filtering impact can be removed by setting k=0, otherwise, extra margin needs to be considered.

  - Proposal 11 (Xiaomi): If the ground truth of L3-RSRP is reported by the UE, RAN4 should discuss whether and how to prevent the UE from providing cheating reports.

  - Proposal 12 (Apple): Define the ground truth of L3-RSRP as the measured RSRP from time t₀ to t₁ (observation window), under the condition that the channel characteristics during the prediction window (t₁ to t₂) remain identical to those in the observation window. The goal is to establish a reproducible and scenario-agnostic validation framework for AI/ML-based L3-RSRP prediction accuracy, particularly addressing limitations in scenarios where real-time UE measurements are unavailable or unreliable.

Proposed Methodology for the above Proposal: Ground truth definition

    - Channel-Consistency

The channel conditions (e.g., large-scale fading, Doppler spread, interference levels, and multipath profiles) must remain unchanged between the observation window (t₀–t₁) and the prediction window (t₁–t₂). This ensures that the RSRP measured in the observation window (t₀–t₁) accurately reflects the conditions under which predictions are validated in t₁–t₂.

    - Measurement Protocol:

      - During the observation window (t₀–t₁), RSRP is measured continuously using UE or test equipment (TE) under controlled channel emulation.

      - In the prediction window (t₁–t₂), the AI/ML model’s predicted RSRP is compared against the pre-measured ground truth from t₀–t₁, assuming identical channel conditions.

    - Emulation and Reproducibility:

      - A channel emulator must replicate time-invariant channel parameters (e.g., path loss, shadowing, delay spread) across both windows.

      - For FR1, consistency in large-scale fading (e.g., path loss models) is prioritized. For FR2, beam-specific parameters (e.g., beam alignment, polarization) must also remain static.

      - Validation requires a correlation coefficient ≥ 0.95 between channel realizations in t₀–t₁ and t₁–t₂ to confirm stationarity.

    - Temporal Granularity:

      - The duration of t₀–t₁ and t₁–t₂ is defined based on UE mobility:

        - Low mobility (e.g., pedestrian): 20–50 ms windows.

        - High mobility (e.g., vehicular): 5–10 ms windows to minimize Doppler-induced variations.

  - Proposal 13 (OPPO): Use Alt2(i.e., reported measurement value under certain conditions) as baseline to define the ground truth for prediction accuracy.

  - Proposal 14 (OPPO): If the definition of ground truth cannot be decided in this meeting, list the advantages/disadvantages in the TR and down-select in WI stage.

  - Proposal 15 (Qualcomm): RAN4 to consider the following options for the definition of ground truth of L3-RSRP for the predicted absolute L3-RSRP accuracy (reported predicted L3-RSRP – ground truth of L3-RSRP). If the down selection needs to be made during the study item phase, RAN4 should consider Option 2-A, Option 2-B, and Option 1 in that order of priority.

    - Option 1: Ground truth based on expected measurement result (spec-defined expected L3-RSRP)

      - The ground truth of L3-RSRP predicted for the time instance X is derived based on the expected reception power of the reference signal at UE assuming the measurement rate defined in RRM spec, plus a margin to accommodate uncertainties in the actual UE L3 measurement rate. The value of the margin needs a further study, and it can vary for different measurement configurations, etc.

      - FFS on whether and how to accommodate the measurement error due to noisy channel, RF calibration error, UE Rx beamforming gain, etc.

    - Option 2-A: Ground truth based on measurement report from the UE under the SAME radio condition as the prediction-based L3-RSRP report (UE-reported L3-RSRP with noise)

      - The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X.

        - Y can be differently defined depending on factors such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc.

        - The radio condition such as S(I)NR remains the same during the target inference time instance and the report instance, i.e., from X – Y ms to X + Y ms.

        - To avoid the UE from cheating the test, TE can either create another cell with the same radio conditions to the extent possible (e.g., large-scale fading) as the target cell for the L3-RSRP prediction or run two rounds of testing with the same configuration: one for the prediction-based L3-RSRP report and the other for the actual measurement-based L3-RSRP report.

    - Option 2-B: Ground truth based on measurement report from the UE under the IDEAL radio condition (UE-reported L3-RSRP with noise-free)

      - The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X.

        - Y can be differently defined depending on factors such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc.

        - The radio condition such as S(I)NR for the measurement report is set such that the measurement result at the UE can be accurate enough.

        - Another round of tests would be needed for the L3-RSRP measurement report

    - RAN4 to strive to not differentiate the definition of the ground truth between FR1 and FR2.

  - Proposal 16 (CATT): For FR1, ground truth is UE received power derived by TE. For FR2, ground truth is reported value and RAN4 wait for progress on AI/ML BM and reuse the agreement.

  - Proposal 17 (Tejas): For FR1 conducted tests, the ground truth should be derived from the test equipment.

  - Proposal 18 (Ericsson): Ground truth in FR1 testing is the calculated value based on transmitted power known at the TE. Other parameters that may have an impact on ground truth calculation are not precluded.

  - Proposal 19 (Samsung):

    - For FR1 RSRP prediction accuracy testing, the ideal RSRP (i.e., the ground truth) can be of the transmit power settled by TE in noise-free/high SNR test condition

    - To make FR1 RSRP prediction accuracy test feasible, at least the following information should to know at TE：

      - Prediction related time, the time can be the absolute time UE performs the prediction and the relative prediction time (prediction window)

      - Fast fading channel condition

    - Keep the parameters related to UE measurement such as L3 filtering, L1 filtering, and the timing UE performs and/or finishes the measurement transparent to TE

    - Define the SNR side condition requirement under the fast fading channel environment if ground truth for FR1 is settled by measurement report

- Candidate Options:

  - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power

  - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions

  - Option 1: RAN4 should consider Alt 1

  - Option 2: RAN4 should consider Alt 2

  - Option 3: RAN4 should not prioritize any alternative during SI phase. The advantages/disadvantages should be captured in the TR and down-selection should be done in WI stage.

- Recommended WF

- Discuss above listed Candidate Options

#### Issue 2-1-6: Impacts on beam/cell level measurements

- Background:

The following agreement was achieved in RAN4#112bis meeting:

Agreement: - RAN4 to first study the impacts on requirements for the following case: - L3 cell level RSRP (Point C) prediction - FFS L1 beam-level (Point A1) - FFS L3 beam-level (Point E) - Other cases can be considered based on RAN4 further discussion and RAN2 further progress.

The figure below from TS 38.300 (Figure 9.2.4-1) should be considered as a reference for this discussion.

- Proposals and Observations:

  - Proposal 1 (Ericsson): RAN4 to not consider the impact of L1 beam level measurement prediction at point A1, on RAN4 requirements unless agreed to be considered for AI/ML based mobility by RAN2.

  - Proposal 2 (Ericsson): RAN4 to consider the impact of L3 beam level measurement prediction at point E, on RAN4 requirements.

  - Proposal 3 (Huawei, CATT): RAN4 can deprioritize the sub-use cases of L3 filtered beam-level measurement predication (Point E).

  - Proposal 4 (Huawei): For 3 sub-use cases for cell-level RRM measurement prediction, RAN4 is to define measurement prediction accuracy at point C.

  - Proposal 5 (ZTE): RAN4 should discuss whether prediction accuracy requirement defined at point C could be used for point A1.

  - Proposal 6 (ZTE): RAN4 to deprioritize studying the impacts on requirements for L3 beam-level prediction, the definition of prediction accuracy for point E will correspondingly be given low priority.

  - Proposal 7 (CMCC): except L3 cell level RSRP (Point C) prediction, it is proposed that RAN4 to consider L1 beam-level (Point A1). And it is proposed that L3 beam-level (Point E) can be deprioritized based on RAN2 agreements.

  - Proposal 8 (MediaTek): For RRM measurement prediction, only use cell level RSRP (Point C) as the performance metric for all the three sub-use cases.

  - Proposal 9 (Apple): For beam-level L1 requirements and given that L1 reporting is not supported within the L3 mobility framework, bypass L3 filtering and utilize a single Tx beam for testing purposes. (point A1 equivalent to C).

  - Proposal 10 (OPPO): For all three sub-use cases, RAN4 to define prediction accuracy requirements and testing for L3 cell level measurements at point C.

  - Proposal 11 (Samsung):

    - RAN4 to prioritize studying the impacts on requirements for L3 cell level RSRP (Point C) prediction

    - RAN4 to deprioritize studying the impacts on requirements for L3 beam-level (Point E) prediction, aligning with RAN2 prioritization

  - Proposal 12 (Samsung): If it is proposed to consider L1 beam level prediction, RAN4 to study the impacts on requirements for L1 beam level RSRP (Point A1) prediction

    - FFS: Whether to/How to define an additional prediction accuracy requirement on top of the requirement at C

  - Proposal 13 (Nokia): RAN4 to focus first on L3 cell level RSRP (Point C) prediction and after that L3 beam-level prediction (Point E).

- Candidate Options:

  - Option 1: Do not consider L3 beam-level (Point E) and L1 beam-level (Point A1) measurement prediction

  - Option 2: Consider L3 beam-level (Point E) measurement prediction as second priority

  - Option 3: Consider L1 beam-level (Point A1) measurement prediction as second priority

  - Option 4: RAN4 should consider studying the impacts on requirements for L3 beam-level (Point E)

  - Option 5: RAN4 should consider studying the impacts on requirements for L1 beam-level (Point A1)

- Recommended WF:

  - Discuss different options listed above.

#### Issue 2-1-7: Impacts of measurement error on prediction accuracy

- Background:

The following agreement was reached in RAN4#114 meeting:

Agreement: For AI based RRM measurement prediction in the SI, while RAN4 has not agreed yet on the exact error model(s) in the SI phase, RAN4 think RF and baseband errors on the measurements would impact the prediction accuracy.

- Proposals and Observations:

  - Proposal 1 (Nokia): RAN4 should study the measurement error modeling by using the tables below as a baseline

- Table 1 RF and BB error modelling principle

| RF and BB modeling principle | Supporting companies |
| --- | --- |
| Both RF and baseband errors follow a Gaussian distribution. | X, Y, Z, U ... |
| Baseband and RF errors follow different distributions. | A, B, C, D … |

- Table 2 BB error modeling

| Baseband error modeling | Supporting companies |
| --- | --- |
| Gaussian distribution | A, B … |
| Link-level simulation results | C, D … |

- Table 3 RF error modeling

| RF error modeling | Supporting companies |
| --- | --- |
| Uniform distribution | A … |
| Gaussian distribution | B … |
| Truncated Gaussian distribution | C … |
| Fixed value | D ... |

- Table 4 RF error application

| How RF error is applied | Supporting companies |
| --- | --- |
| Independent for each Tx beam | A … |
| same for each Tx beam | B … |
| Independent for each module/panel | C … |
| same for each module/panel | D ... |

  - Proposal 2 (Apple): RAN4 to investigate the impact of measurement errors (RF and baseband) in AI/ML model input data on the accuracy of predicted measurements. Additionally, RAN4 should investigate whether RAN2 simulation results and conclusions regarding achievable prediction accuracy remain valid under such measurement errors for scenario 2.

  - Proposal 3 (Apple): For the multi-link RSRP temporal-prediction extension of Scenario RAN4 to investigate the impact of cell dependent measurement errors (different SINRs and channel conditions) to the prediction accuracy of the AI/ML models (scenario #4)

  - Proposal 4 (Apple): RAN4 to investigate the impact of measurement errors in the input data of AI/ML model on the accuracy of predicted measurements. RAN4 should leverage the simulation methodologies and evaluation results (from AI/ML BM Case 1) to assess how these measurement errors affect the accuracy of predictions related to mobility. RAN4 should validate any RAN2 conclusions in the presence of realistic measurements errors.

  - Proposal 5 (Apple): RAN4 to investigate modelling measurement errors as a function of SINR. This analysis must account for variations in propagation environments across serving and neighboring cells, as measurement errors are inherently influenced by cell-specific SINR conditions.

  - Proposal 6 (OPPO): RAN4 not to interpose the feasibility discussion in RAN2 but can consider the impact of measurement error on accuracy requirements in RAN4 during WI stage similar as AI/ML for NR Air Interface.

  - Proposal 7 (Qualcomm): RAN4 should not start simulation-based performance evaluation on measurement prediction due to measurement inaccuracy in the observation window during the study item phase. Instead, the group can further discuss additional aspects that might need to be taken into account in the WI phase, if approved, in addition to the outcome of the evaluations on AI/ML-based BM, e.g.

    - Whether and how to accommodate the impact of measurement inaccuracies for a cluster-based approach.

    - Whether and how to model the impact of imperfect RF calibration for inter-frequency RRM prediction.

    - Whether different aspects need to be considered for Case A and Case B in intra-cell/intra-frequency RRM prediction.

    - Note that the effective differences between Case A and Case B, from a UE measurement and prediction perspective, might be limited to the measurement rate within the observation window and the prediction range into the future. Case A and Case B may have different impacts on RRM requirements, particularly about measurement/scheduling restrictions

- Recommended WF

  - Option 1：RAN4 to consider the impact of measurement error on accuracy requirements in RAN4 during WI stage similar as AI/ML for NR Air Interface

  - Option 2：RAN4 should study the measurement error modeling by using the tables below as a baseline

- Table RF and BB error modelling principle

| RF and BB modeling principle | Supporting companies |
| --- | --- |
| Both RF and baseband errors follow a Gaussian distribution. | X, Y, Z, U ... |
| Baseband and RF errors follow different distributions. | A, B, C, D … |

- Table 2 BB error modeling

| Baseband error modeling | Supporting companies |
| --- | --- |
| Gaussian distribution | A, B … |
| Link-level simulation results | C, D … |

- Table 3 RF error modeling

| RF error modeling | Supporting companies |
| --- | --- |
| Uniform distribution | A … |
| Gaussian distribution | B … |
| Truncated Gaussian distribution | C … |
| Fixed value | D ... |

- Table 4 RF error application

| How RF error is applied | Supporting companies |
| --- | --- |
| Independent for each Tx beam | A … |
| same for each Tx beam | B … |
| Independent for each module/panel | C … |
| same for each module/panel | D ... |

#### Issue 2-1-8: Inter-frequency prediction scenario

- Proposals and Observations:

  - Proposal 1 (Ericsson): RAN4 to consider the impact of frequency relationship between the measured and predicted cells on the performance of frequency domain prediction of RRM measurement for mobility.

  - Proposal 2 (Ericsson): RAN4 to study requirement aspects for the following cases:

    - When measurement from single cell in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

    - When measurement from a group of cells in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR cell(s) in another carrier frequency(ies).

  - Proposal 3 (Nokia): RAN4 to study the minimum requirement/characteristic of the input and output of AI/ML model for RRM and testability assessment of inter-cell inter-frequency prediction domain scenarios.

  - Proposal 4 (Huawei): For inter-frequency inter-cell frequency domain prediction, the side condition of frequency prediction (e.g., EPRE difference) and RSRP prediction accuracy may need to be specified.

  - Proposal 5 (MediaTek): The test case for inter-frequency prediction can be designed as the following:

    - NW configures SMTC for both f1 (intra-f) and f2 (inter-f). MG is not configured. A4 event is configured to check the prediction accuracy on f2. The threshold of A4 is set as X dBm. Suppose the absolute prediction accuracy is defined as ±YdB.

    - TE calculates filtered L3 result on each SMTC occasion.

    - TE can know when there is a neighbour cell whose RSRP is with [X-Y, X+Y]. TE can also get to know the time range when there will be expected SR based on the configurations. If UE triggers report request during the expected time range, then it can be viewed as the prediction meets the accuracy requirements in this single test.

  - Proposal 6 (MediaTek): For Scenario 3, gap sharing mechanism among different inter-frequency layers should be considered for both for measurement and monitoring when more than one inter-frequency layers are involved.

  - Proposal 7 (Xiaomi): For inter-frequency inter-cell RSRP prediction, RAN4 to study the impact on CSSF reduction in measurement period.

  - Proposal 8 (OPPO): RAN4 to consider the impact of correlation coefficient between two frequency layers and the cluster approach on inter-frequency measurement prediction accuracy requirements.

  - Proposal 9 (OPPO): RAN4 to discuss the measurement period requirements for inter-frequency measurement prediction considering the impact of skipped measurement frequencies or skipped measurement instances which may not need to be considered in the CSSF calculation.

- Recommended WF

  - RAN4 to study the impacts on RSRP prediction accuracy for inter-frequency prediction scenario considering the following aspects:

    - the side condition of frequency prediction (e.g., EPRE difference)

    - the impact of correlation coefficient between two frequency layers

    - impact of cluster approach, e.g.,

      - When measurement from single cell in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

      - When measurement from a group of cells in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

  - RAN4 to study the impacts on measurement period requirements for inter-frequency prediction scenario considering the following aspects:

    - the feasibility of predicting measurement results for multiple cells per frequency layer.

    - the impact of adjusting the measurement gap period or selectively skipping gap occasions to reduce measurement gaps.

    - Impacts on CSSF

    - Impacts on MG

    - gap sharing mechanism among different inter-frequency layers when more than one inter-frequency layers are involved.

  - Other aspects for inter-frequency prediction scenario

    - RAN4 to study the minimum requirement/characteristic of the input and output of AI/ML model for RRM and testability assessment of inter-cell inter-frequency prediction domain scenarios.

    - RAN4 should explore potential enhancements in inter-frequency and spatial domain measurement prediction by combining them with temporal domain prediction.

#### Issue 2-1-9: Impacts from observation and prediction windows

- Background:

The following agreement was reached in RAN4#114 meeting:

Agreement: - RAN4 to consider the impact of UE speed, MRRT and OW and PW lengths on prediction accuracy requirements for temporal predictions. - RAN4 to also consider number of measurements performed during OW and its impacts on the prediction accuracy.

- Proposals and Observations:

  - Proposal 1 (Ericsson): RAN4 to study the impact of measurement skipping pattern within the OW on RAN4 requirement for a fixed MRRT for temporal prediction case-B.

  - Proposal 2 (Nokia): For temporal domain prediction case B, RAN4 to agree on pre-selected reference testing scenarios relating to OW, PW and MRRT as well as selected skipping patterns for a given MRRT value.

  - Proposal 3 (Huawei): For intra-frequency temporal domain prediction case A, observation window length (or called as prediction evaluation period) and prediction window length may need to be specified.

  - Proposal 4 (Huawei): To guarantee prediction accuracy for FR2 intra-frequency temporal domain case A, limited doppler shift, limited PW length and proper OW length shall be set in testing.

  - Proposal 5 (Xiaomi): Observation window and prediction window defined in RAN2 will have RRM impact:

    - Observation window will have impact on measurement duration

    - Prediction window will have impact on scheduling restriction

  - Proposal 6 (Xiaomi): For intra-frequency intra-cell prediction, suggest RAN4 to select some typical case/scenarios to study observation window and prediction window.

  - Proposal 7 (Apple): Investigate the translation of RAN2 KPI requirements into RAN4 RRM measurement accuracy requirements to assess the feasibility of the RAN2 study. This translation is essential for aligning performance metrics and ensuring that mobility optimization goals are met effectively. Additionally, explore the impact of RRM measurement requirements on prediction accuracy across various RRM measurement prediction cases and evaluation scenarios given side conditions of OW and PW, SNR.

  - Proposal 8 (OPPO): RAN4 to discuss the measurement period requirements for intra-frequency measurement temporal case A considering the impact of OW or PW length.

  - Proposal 8a (OPPO): RAN4 to discuss the measurement period requirements for intra-frequency measurement temporal case B considering the impact of skipped measurement instances, e.g., based on MRRT values.

  - Proposal 9 (Qualcomm): UE speed, MRRT, and OW/PW lengths need to be considered when defining RRM requirements in the work item phase, unless it is identified that some specific issues are up for discussion during the study item phase.

- Candidate Options

  - Option 1: For temporal domain case, RAN4 should agree on testing scenarios defining OW, PW and MRRT as well as skipping patterns

  - Option 2: RAN4 should study the impact on measurement duration from observation window.

  - Option 3: RAN4 should study the impact on scheduling restrictions from prediction window.

  - Option 4: RAN4 to consider OW/PW, UE speed and MRRT only during WI phase unless a need is identified.

- Recommended WF

  - Discuss above listed candidate options.

#### Issue 2-1-10: Impact on Core requirements

- Proposals and Observations:

  - Proposal 1 (OPPO): RAN4 to discuss the measurement period requirements for intra-frequency measurement temporal case B considering the impact of skipped measurement instances, e.g., based on MRRT values.

  - Proposal 1a (OPPO): RAN4 to investigate the possibility of updates on scheduling restrictions, measurement capabilities or definition of known conditions due to measurement prediction.

  - Proposal 2 (Samsung): RAN4 to discuss the RRM impact of measurement gap reduction in measurement period/ scheduling restriction and CSSF enhancement for inter-frequency prediction.

  - Proposal 3 (Nokia): RAN4 to further study the impact on legacy measurement delay requirements for Temporal domain Case B and Frequency domain cases

  - Proposal 4 (Huawei): The processing time of inference function may need to be considered in measurement reporting delay.

  - Proposal 5 (ZTE): RAN4 should discuss whether/how to update measurement delay requirements for AI/ML mobility.

  - Proposal 6 (CMCC): for RRM measurement prediction, the measurement delay requirements need to be defined/updated

    - For intra-frequency temporal domain prediction, UE will skip some measurement occasions, which means that only subset of measurement occasions are measured, as a result, SMTC and/or MGRP cannot be directly used in the measurement delay requirements as legacy

    - For inter-frequency inter-cell frequency domain prediction, not all configured MOs need to be measuerd, which means that only the actually measured frequency layers are considered in the measurement delay requirements

  - Proposal 7 (MediaTek): Wait for more progress in AI/ML BM to study the impact on measurement report delay requirements for AI mobility.

  - Proposal 8 (Qualcomm):  RAN4 to review the existing RRM requirement and outline the identified potential impacts at a high level, e.g.

    - Common aspects:

      - Adjustment of the length of observation and prediction windows depending on UE in DRX state and/or cycle, serving cell RSRP, etc.

      - Measurement and scheduling restrictions in the prediction window.

      - Definition of known vs. unknown target cells for mobility and/or PSCell/SCell addition and activation.

      - Side conditions such as requirement applicability rules.

      - Overall budget in terms of mandatory capability on the number of cells and frequency layers for actual measurements and inference-based measurements.

    - Scenario 2: FR1 to FR1 intra-frequency temporal domain case B, Intra-cell

      - Uniform measurement approach for cells in the same frequency layer

    - Scenario 3: FR1 to FR1 inter-frequency, Inter-cell

      - CSSF

      - MG

    - Scenario 4: FR2 to FR2 intra-frequency temporal domain case A, Intra-cell

      - Uniform measurement approach for cells in the same frequency layer

- Candidate Options:

  - Option 1: RAN4 to discuss the measurement period requirements for intra-frequency measurement temporal case B

  - Option 2: RAN4 to discuss the measurement period requirements for inter-frequency case

  - Option 3: RAN4 to discuss the measurement period requirements for both inter-frequency case and intra-frequency temporal case B

  - Option 4: RAN4 should consider inference delay in measurement reporting delay.

  - Option 5: RAN4 should wait for further progress in RAN2 before starting discussion on core requirements in RAN4

- Recommended WF

- Discuss above listed options

#### Issue 2-1-11: LCM related requirements

- Proposals and Observations:

  - Proposal 1 (Apple): RAN4 should leverage the LCM progress and requirements discussed in R-19 WI related to AI/ML Air Interface and investigate enhancements to these LCM procedures to RRM for mobility.

  - Proposal 2 (Samsung): LCM related requirements and functional tests should to be defined based on the specific AI/ML mobility use cases and sub-use cases

    - The LCM procedures agreed in AI/ML for NR air interface RRM need to be considered as starting point

      - Prefer to use the legacy framework for RRC/MAC-CE/DCI based core requirements as the baseline for LCM procedures

  - Proposal 3 (CATT): RAN4 should focus on performance metrics and relevant requirements and postpone the discussion on LCM related issues.

  - Proposal 4 (Huawei): If the functionality/model activation, deactivation, or fallback to non-AI function are indicated by network, the activation/deactivation/fallback delay and corresponding side conditions would need to be specified.

  - Proposal 5 (Huawei): If the functionality/model activation, deactivation, or fallback to non-AI function are autonomously performed by UE, the requirements would be related with monitoring requirements.

  - Proposal 6 (Ericsson): RAN4 to identify requirements to be defined for data collection for network sided model for AI/ML based mobility.

  - Proposal 7 (Qualcomm): the test framework also needs to consider how to verify UE LCM performance, if needed.

- Recommended WF

  - LCM related discussions should be held in RAN4 potentially at a later stage. RAN2 needs to progress on this topic.

# Topic #3: RAN4 requirements for measurement event prediction

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2504233 | Nokia | Study of RAN4 impacts for measurement event prediction Observation 1: Direct and indirect measurement event prediction may have different requirements and performance characteristics. Proposal 1: RAN4 to study the potential differences in performance and requirements between direct and indirect measurement event prediction methods. Proposal 2: RAN4 to prioritize between studying direct and indirect measurement event predictions if they exhibit distinct performance and requirement differences. As a possible approach, RAN4 may follow RAN2 prioritization or alternatively focus on indirect measurement event prediction. Observation 2: In time domain case B measurement event prediction, the PW is typically short, and predictions are not made for every measurement period. Observation 3: In time domain case B measurement event prediction, all mobility decisions are made based on measurement-event-triggered reporting based on actual measurements and predictions. Therefore, the goal of time domain case B is measurement reduction without causing any significant impact on the mobility performance. Observation 4: The following scenarios can be considered for indirect measurement event prediction using temporal domain predictions: I: Temporal domain Case A: The UE predicts a probable future measurement event, which may be used in mobility optimization decisions, but there is no measurement reduction. II: Temporal domain Case B: The UE is predicting measurements for measurement reduction, which should not have significant impact on the mobility performance. Observation 5: For indirect measurement event prediction, the choice between temporal domain Case A and Case B cases may impose different requirements and performance metrics, necessitating further RAN4 evaluation. Proposal 3: RAN4 to prioritize, based on RAN2 outcome, from the available cases for indirect measurement event prediction using temporal domain Case A or Case B, and conduct a study on the performance and requirements for the selected case. Proposal 4: RAN4 to study the applicability of current measurement event criteria and reporting requirements/performance metrics for measurement event prediction and explore the need for new metrics. |
| R4-2503930 | Ericsson | On requirements for Event prediction for AI/ML based mobility Observation 1: Reporting delay requirement defined for the legacy event triggered reporting may not hold true for the event prediction case. Proposal 1: RAN4 to consider impact on reporting delay requirement due to predicted event triggered reporting. Proposal 2: Requirements for RRM measurement prediction in temporal domain is reused for indirect event prediction use cases. Proposal 3: Performance requirement for direct event prediction needs to be defined for event prediction provided direct event prediction is supported by the framework designed by RAN2 for AI/ML based mobility. |
| R4-2503309 | Xiaomi | Discussion on impacts on RAN4 requirement for measurement event prediction in AI mobility Observation 1: In legacy, RAN4 defined event triggered L3 reporting delay in core part. Proposal 1: Measurement event prediction may have impact on predicated event triggered L3 reporting. Observation 2: The time interval from measurement to prediction increase, the prediction accuracy decreases. The prediction accuracy of individual measurement results in a time window could be quite different. Proposal 2: RAN4 to define predicted RSRP accuracy requirement for event triggered reporting. How to define requirement will wait for more RAN2 progress. Proposal 3: When define predicted RSRP accuracy requirement for event triggered reporting, the impact of TTT can’t be ignored. Observation 3: In RAN2, performance metric F1 score is defined for event prediction accuracy, which considered both precision and recall. Observation 4: In RAN2, a timing difference between predicated event and ground truth event is defined. Observation 5: In legacy RAN4, in test for verify event prediction accuracy, only precision = 90% is used and no timing difference between predicated event and ground truth event is considered. Proposal 4: RAN4 to discuss whether and how to define performance metric for event prediction accuracy. |
| R4-2503352 | OPPO | Study of impacts on RAN4 requirements for measurement event prediction Proposal 1: For measurement event prediction, RAN4 to discuss RRM requirements impact starting from indirect prediction case. Proposal 2: For indirect prediction, the requirements for intermediate output are needed and the requirements for measurement prediction can be used as baseline. Proposal 3: For indirect prediction, RAN4 to discuss whether the existing handover delay requirements can be reused. Proposal 4: For indirect prediction, RAN4 to discuss the possibility of updates on event condition or requirements applicability. Proposal 5: For indirect prediction, RAN4 to discuss whether new quantities or requirements are needed based on further RAN2 progress. |
| R4-2503888 | Huawei, HiSilicon | Discussion on impacts for measurement event prediction Proposal 1: RAN4 to study the indirect event prediction and direct event prediction. Proposal 2: RAN4 to discuss the following options for indirect event prediction Option 1: Define accuracy for indirect event prediction Option 1-1: the accuracy of event prediction is defined as ETD, under the conditions of certain fixed precision and recall or F1 score. More specifically, the measurement event shall be predicted within certain ETD (time difference between the ground truth event and the predict event), under the condition that F1 score consistently is above a value. Option 1-2: the accuracy of event prediction is defined as precision and recall or F1 score, under the conditions of certain fixed ETD. More specifically, the qualified UE shall satisfy F1 score consistently is above a value, under the condition that event is predicted within certain ETD. Option 3: Not define accuracy requirement for indirect event prediction, as long as the measurement prediction accuracy is satisfied. Proposal 2: RAN4 to discuss the following options for defining accuracy for direct event prediction: Option 1: the accuracy of event prediction is defined as event prediction probability X% within an occurrence window [t0, t1], under the conditions of certain fixed F1 score. More specifically, the measurement event shall be predicted within an occurrence window [t0, t1] with X% probability, under the condition that F1 score is consistently above a value. Option 2: another accuracy definition is defined as precision and recall or F1 score under the conditions of event prediction probability X% within an occurrence window [t0, t1]. More specifically, the qualified UE shall satisfy F1 score consistently is above a value (e.g., 0.9), under the condition of X% event prediction probability within an occurrence window [t0, t1]. |
| R4-2504048 | vivo | Discussion on RAN4 impacts for measurement event prediction Observation 1: For indirect measurement event prediction, the intermediate result reported is the predicted value of the service cell/neighborhood (A3 event), which is the same as the current RRM measurement prediction use case for temporal domain Proposal 1: On the study of RRM impact on measurement event prediction, RAN4 can leverage of the progress made in RRM measurement prediction Proposal 2: For measurement event prediction use-case, RAN4 to deprioritize the scenario on indirect measurement event prediction for spatial domain |
| R4-2503852 | ZTE Corporation, Sanechips | Discussion on impacts for measurement event prediction Proposal 1: RAN4 should discuss the RRM impact on indirect measurement event prediction and direct measurement event prediction. Proposal 2: Measurement error should be considered into measurement event prediction, which is similar as RRM measurement prediction. Proposal 3: For measurement event prediction, RAN4 should at least consider scenarios2(FR1 to FR1 intra-frequency temporal domain caseB), scenario 3 (FR1 to FR1 inter-frequency (frequency domain)), and scenario 4 (FR2 to FR2 intra-frequency temporal domain case A). Proposal 4: Discussions on RRM impact for RRM measurement prediction use case can be reused for indirect event measurement prediction. |
| R4-2503412 | CATT | Discussion on impacts of AIML measurement event prediction on RRM requirements Observation 1: RRM measurement prediction can be reused as baseline for corresponding direct measurement event prediction, based on RAN2 agreement. Observation 2: The intermediate output, i.e., RSRPs of serving/neighbouring cells, may be unknown to TE and therefore it is impossible to verify its accuracy. Observation 3: Necessity of defining a performance requirement for a probability needs justifying for indirect measurement event prediction. Proposal 1: No extra discussion is needed for model input-related aspects for direct and indirect measurement event prediction, including test configurations. Proposal 2: Successful prediction rate can be used as a test metric for direct and indirect measurement event prediction. Proposal 3: With AI/ML based measurement event prediction, some components in conditional handover delay requirements can be updated or removed, e.g., Tprocessing and Tmeasure. FFS details. |
| R4-2504525 | Qualcomm Inc. | Impact of AI based mobility on RRM event prediction Proposal 1: RAN4 to consider the following two metrics for the evaluation of AI/ML-based RRM event prediction: - Time domain: How closely the AI/ML model has predicted the event occurrence time compared to the real event occurrence time. - L3-RSRP domain: How closely the AI/ML model has predicted the L3-RSRP corresponding to the predicted event occurrence time compared to the real L3-RSRP. Proposal 2: For the definition of ground truth of predicted event occurrence time accuracy (reported predicted event occurrence time – ground truth of the event occurrence time) in the time domain, the ground truth of the event occurrence time should be defined by the same principles as the ground truth of predicted L3-RSRP. If a down selection is made during the study item phase for the definition of ground truth of predicted L3-RSRP, the same principle should be inherited for the definition of ground truth of predicted event occurrence time. The options for the definition of ground truth of predicted L3-RSRP are listed below (options are duplicated below for reference from our companion paper submitted for AI 7.21.2.1): - Option 1: Ground truth based on expected measurement result (spec-defined expected L3-RSRP) - The ground truth of L3-RSRP predicted for the time instance X is derived based on the expected reception power of the reference signal at UE assuming the measurement rate defined in RRM spec, plus a margin to accommodate uncertainties in the actual UE L3 measurement rate. The value of the margin needs a further study, and it can vary for different measurement configurations, etc. - FFS on whether and how to accommodate the measurement error due to noisy channel, RF calibration error, UE Rx beamforming gain, etc. - Option 2-A: Ground truth based on measurement report from the UE under the SAME radio condition as the prediction-based L3-RSRP report (UE-reported L3-RSRP with noise) - The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X. - Y can be differently defined depending on factors such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc. - The radio condition such as S(I)NR remains the same during the target inference time instance and the report instance, i.e., from X – Y ms to X + Y ms. - To avoid the UE from cheating the test, TE can either create another cell with the same radio conditions to the extent possible (e.g., large-scale fading) as the target cell for the L3-RSRP prediction or run two rounds of testing with the same configuration: one for the prediction-based L3-RSRP report and the other for the actual measurement-based L3-RSRP report. - Option 2-B: Ground truth based on measurement report from the UE under the IDEAL radio condition (UE-reported L3-RSRP with noise-free) - The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X. - Y can be differently defined depending on factors such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc. - The radio condition such as S(I)NR for the measurement report is set such that the measurement result at the UE can be accurate enough. - Another round of tests would be needed for the L3-RSRP measurement report - RAN4 to strive to not differentiate the definition of the ground truth between FR1 and FR2. Observation 2: For the performance metric of event prediction in the L3-RSRP domain, one or both of the performance metrics in terms of predicted absolute L3-RSRP accuracy, predicted relative L3-RSRP accuracy, and others such as Top-K(%) and Top-K/1(%), which are covered by another agenda item, should be considered. Proposal 3: RAN4 to consider the following as one of the performance metrics for AI/ML-based measurement event prediction: - One or multiple of the performance metrics to be defined for AI/ML-based L3-RSRP prediction in terms of predicted absolute L3-RSRP accuracy, predicted relative L3-RSRP accuracy, and others such as Top-K(%) and Top-K/1(%). - The gap in the time domain from the predicted time instance when the event is predicted to occur to the time instance when the event occurs, based on the to-be-defined ground truth L3-RSRP. - The above two metrics may jointly serve as a performance assessment metric for event prediction, or one can be down-selected. The choice of the metric and the details could further depend on the following aspects to be discussed and decided by RAN2: - Whether the AI model is for indirect or direct event prediction. - What metrics are to be reported to the serving cell. |
| R4-2503776 | CMCC | Discussion on RAN4 impacts for measurement event prediction Proposal 1: for the intermediate output of indirect measurement event prediction, it is proposed to reuse the conclusion on RRM measurement prediction. Proposal 2: for the final output of measurement event prediction (e.g. the probability of event occurrence for direct prediction or expected occurrence time of a event for indirect prediction), it is proposed to discuss whether and how to define RRM requirements. |
| R4-2503523 | Apple | Study of RAN4 impacts for measurement event prediction for AIML Mobility in NR Proposal 1: RAN4 to discuss the modelling of measurement errors and their impact to evaluate AI/ML event prediction benefits compared to sample-and-hold and the impact to false detection of events (higher cost) Proposal 2: For indirect measurement event prediction, RAN4 to discuss the modeling and impact of measurement errors to the prediction accuracy (i.e., F1 score) Proposal 3: RAN4 to discuss modeling measurements errors as a function of SINR. The propagation environment varies across serving/neighbouring cells. Measurement errors depend on the cell-specific SINR Proposal 4: Investigate the translation of RAN2 KPI requirements into RAN4 RRM measurement accuracy and prediction measurement accuracy requirements to assess the feasibility of the RAN2 study. The evaluation of the benefits provided by AI/ML-assisted mobility should consider (KPIs) related to handovers (e.g., ping-pong handovers, handover failures/radio link failures, time of stay, handover interruptions, prediction accuracy, and measurement reduction). These factors will determine the formulation of RAN4 requirements, as the primary objective is to enhance the robustness of mobility procedures and minimize failures. Proposal 5: Different cells may operate under distinct network conditions (e.g., transmit (TX) codebook configurations, beamwidths, TX implementation variations). RAN4 to investigate whether an AI/ML model trained on data from one cell (e.g., neighbor cell 1) can generalize effectively for inference on another cell. Critical conditions (e.g., beam alignment parameters, hardware compatibility, signal propagation characteristics) must be defined to ensure consistency between training and inference phases for robust generalization. Proposal 6: RAN4 should consider the following performance metrics for AI/ML-based measurement event prediction: - Baseline Metrics from L3-RSRP Prediction - Retain one or multiple existing performance metrics defined for AI/ML-based L3-RSRP prediction - Event-Specific Temporal Accuracy - Define a time-domain gap metric: The difference between the predicted time instance when the event is forecasted to occur (e.g., entering condition met at T2) and the actual time instance when the event occurs, based on ground truth L3-RSRP. - Example: If an event is predicted at T2 but occurs at T3 (ground truth), the gap is ΔT = T3 – T2. - Combined or Down-Selected Metrics - The above metrics (L3-RSRP prediction accuracy + temporal gap) may be: - Jointly used for comprehensive performance assessment, or Independently selected depending on use-case priorities. |
| R4-2503608 | MediaTek Inc. | Discussion on RAN4 impacts for measurement event prediction Observation 1: For measurement event prediction, it is too early to discuss the metrics to define requirements for in RAN4 as the reported quantities are quite open in RAN2. Proposal 1: For measurement event prediction based on the scenario “intra-frequency temporal domain case A”, further study whether there is a need to define the corresponding test cases. Proposal 2: For measurement event prediction and RRM measurement prediction on the scenarios “intra-frequency temporal domain case B” and “inter-frequency”, further study the necessity to define separate test cases for both use cases on the same scenario. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 3-1: General and Scope

Sub-topic description:

In this sub-topic the company proposals about the RRM requirements for AI/ML Mobility use cases are summarized.

#### Issue 3-1-1: Prioritization of cases for event prediction

- Proposals and Observations:

  - Proposal 1 (Nokia): RAN4 to prioritize between studying direct and indirect measurement event predictions if they exhibit distinct performance and requirement differences. As a possible approach, RAN4 may follow RAN2 prioritization or alternatively focus on indirect measurement event prediction

  - Proposal 2 (OPPO): For measurement event prediction, RAN4 to discuss RRM requirements impact starting from indirect prediction case.

  - Proposal 3 (Huawei): RAN4 to study the indirect event prediction and direct event prediction

  - Proposal 4 (vivo): For measurement event prediction use-case, RAN4 to deprioritize the scenario on indirect measurement event prediction for spatial domain

  - Proposal 5 (ZTE): RAN4 should discuss the RRM impact on indirect measurement event prediction and direct measurement event prediction.

- Candidate options:

  - Option 1: RAN4 to prioritize indirect prediction case

  - Option 2: RAN4 to prioritize direct prediction case

  - Option 3: RAN4 may follow RAN2 prioritization

  - Option 4: RAN4 should discuss both indirect and direct case and shouldn’t define any prioritization.

- Recommended WF

- Discuss above listed Candidate Options.

#### Issue 3-1-2: Performance metric for event prediction use case

- Proposals and Observations:

  - Proposal 1 (Nokia): RAN4 to study the applicability of current measurement event criteria and reporting requirements/performance metrics for measurement event prediction and explore the need for new metrics.

  - Proposal 2 (Ericsson): Requirements for RRM measurement prediction in temporal domain is reused for indirect event prediction use cases..

  - Proposal 3 (Ericsson): Performance requirement for direct event prediction needs to be defined for event prediction provided direct event prediction is supported by the framework designed by RAN2 for AI/ML based mobility.

  - Proposal 4 (Xiaomi): RAN4 to define predicted RSRP accuracy requirement for event triggered reporting. How to define requirement will wait for more RAN2 progress.

  - Proposal 5 (Xiaomi): When define predicted RSRP accuracy requirement for event triggered reporting, the impact of TTT can’t be ignored.

  - Proposal 6 (Xiaomi): RAN4 to discuss whether and how to define performance metric for event prediction accuracy.

  - Proposal 7 (OPPO): For indirect prediction, the requirements for intermediate output are needed and the requirements for measurement prediction can be used as baseline.

  - Proposal 8 (OPPO): For indirect prediction, RAN4 to discuss whether new quantities or requirements are needed based on further RAN2 progress.

  - Proposal 9 (Huawei):

RAN4 to discuss the following options for indirect event prediction

    - Option 1: Define accuracy for indirect event prediction

Option 1-1: the accuracy of event prediction is defined as ETD, under the conditions of certain fixed precision and recall or F1 score. More specifically, the measurement event shall be predicted within certain ETD (time difference between the ground truth event and the predict event), under the condition that F1 score consistently is above a value.

Option 1-2: the accuracy of event prediction is defined as precision and recall or F1 score, under the conditions of certain fixed ETD. More specifically, the qualified UE shall satisfy F1 score consistently is above a value, under the condition that event is predicted within certain ETD.

    - Option 2: Not define accuracy requirement for indirect event prediction, as long as the measurement prediction accuracy is satisfied.

RAN4 to discuss the following options for defining accuracy for direct event prediction:

    - Option 1: the accuracy of event prediction is defined as event prediction probability X% within an occurrence window [t0, t1], under the conditions of certain fixed F1 score. More specifically, the measurement event shall be predicted within an occurrence window [t0, t1] with X% probability, under the condition that F1 score is consistently above a value.

Option 2: another accuracy definition is defined as precision and recall or F1 score under the conditions of event prediction probability X% within an occurrence window [t0, t1]. More specifically, the qualified UE shall satisfy F1 score consistently is above a value (e.g., 0.9), under the condition of X% event prediction probability within an occurrence window [t0, t1].

  - Proposal 10 (CATT): Successful prediction rate can be used as a test metric for direct and indirect measurement event prediction.

  - Proposal 11 (Qualcomm): RAN4 to consider the following two metrics for the evaluation of AI/ML-based RRM event prediction:

    - Time domain: How closely the AI/ML model has predicted the event occurrence time compared to the real event occurrence time.

    - L3-RSRP domain: How closely the AI/ML model has predicted the L3-RSRP corresponding to the predicted event occurrence time compared to the real L3-RSRP.

  - Proposal 12 (Qualcomm): RAN4 to consider the following as one of the performance metrics for AI/ML-based measurement event prediction:

    - One or multiple of the performance metrics to be defined for AI/ML-based L3-RSRP prediction in terms of predicted absolute L3-RSRP accuracy, predicted relative L3-RSRP accuracy, and others such as Top-K(%) and Top-K/1(%).

    - The gap in the time domain from the predicted time instance when the event is predicted to occur to the time instance when the event occurs, based on the to-be-defined ground truth L3-RSRP.

    - The above two metrics may jointly serve as a performance assessment metric for event prediction, or one can be down-selected. The choice of the metric and the details could further depend on the following aspects to be discussed and decided by RAN2:

      - Whether the AI model is for indirect or direct event prediction.

      - What metrics are to be reported to the serving cell.

  - Proposal 13 (CMCC):

    - for the intermediate output of indirect measurement event prediction, it is proposed to reuse the conclusion on RRM measurement prediction.

    - for the final output of measurement event prediction (e.g. the probability of event occurrence for direct prediction or expected occurrence time of a event for indirect prediction), it is proposed to discuss whether and how to define RRM requirements.

  - Proposal 14 (Apple): Investigate the translation of RAN2 KPI requirements into RAN4 RRM measurement accuracy and prediction measurement accuracy requirements to assess the feasibility of the RAN2 study. The evaluation of the benefits provided by AI/ML-assisted mobility should consider (KPIs) related to handovers (e.g., ping-pong handovers, handover failures/radio link failures, time of stay, handover interruptions, prediction accuracy, and measurement reduction). These factors will determine the formulation of RAN4 requirements, as the primary objective is to enhance the robustness of mobility procedures and minimize failures.

  - Proposal 15 (Apple): RAN4 should consider the following performance metrics for AI/ML-based measurement event prediction:

    - Baseline Metrics from L3-RSRP Prediction

      - Retain one or multiple existing performance metrics defined for AI/ML-based L3-RSRP prediction

    - Event-Specific Temporal Accuracy

      - Define a time-domain gap metric: The difference between the predicted time instance when the event is forecasted to occur (e.g., entering condition met at T2) and the actual time instance when the event occurs, based on ground truth L3-RSRP.

      - Example: If an event is predicted at T2 but occurs at T3 (ground truth), the gap is ΔT = T3 – T2.

    - Combined or Down-Selected Metrics

      - The above metrics (L3-RSRP prediction accuracy + temporal gap) may be:

Jointly used for comprehensive performance assessment, or Independently selected depending on use-case priorities.

- Candidate options:

- For indirect case:

  - Option 1: Requirements for RRM measurement prediction in temporal domain is reused as a baseline

  - Option 2: RAN4 to study the applicability of current measurement event criteria and reporting requirements to event prediction use case

  - Option 3: New performance metric is needed. FSS on the definition of new performance metric for event prediction. Candidates are:

    - Option 3-1: For indirect event prediction, the accuracy of event prediction is defined as ETD (time difference between the ground truth event and the predict event), under the condition that F1 score consistently is above a value.

    - Option 3-2:  For indirect event prediction, the accuracy of event prediction is defined as precision and recall or F1 score, under the conditions of certain fixed ETD.

    - Option 3-3: The gap in the time domain from the predicted time instance when the event is predicted to occur to the time instance when the event occurs, based on the to-be-defined ground truth L3-RSRP.

    - Option 3-4: Define a time-domain gap metric: The difference between the predicted time instance when the event is forecasted to occur (e.g., entering condition met at T2) and the actual time instance when the event occurs, based on ground truth L3-RSRP.

      - Example: If an event is predicted at T2 but occurs at T3 (ground truth), the gap is ΔT = T3 – T2.

    - Option 3-5: Not define accuracy requirement for indirect event prediction, as long as the measurement prediction accuracy is satisfied.

    - Option 3-6: One or multiple of the performance metrics to be defined for AI/ML-based L3-RSRP prediction in terms of predicted absolute L3-RSRP accuracy, predicted relative L3-RSRP accuracy, and others such as Top-K(%) and Top-K/1(%). This can serve as a performance assessment metric for event prediction in conjunction with time-domain gap metric, or one can be down-selected. The choice of the metric and the details could further depend on the following aspects to be discussed and decided by RAN2:

      - Whether the AI model is for indirect or direct event prediction.

      - What metrics are to be reported to the serving cell.

- Recommended WF

- Discuss above listed Candidate Options.

#### Issue 3-1-3: Impact of measurement error for event prediction use case

- Proposals and Observations:

  - Proposal 1 (ZTE): Measurement error should be considered into measurement event prediction, which is similar as RRM measurement prediction.

  - Proposal 2 (Apple): RAN4 to discuss the modelling of measurement errors and their impact to evaluate AI/ML event prediction benefits compared to sample-and-hold and the impact to false detection of events (higher cost).

  - Proposal 3 (Apple): For indirect measurement event prediction, RAN4 to discuss the modeling and impact of measurement errors to the prediction accuracy (i.e., F1 score)

  - Proposal 4 (Apple): RAN4 to discuss modeling measurements errors as a function of SINR. The propagation environment varies across serving/neighbouring cells. Measurement errors depend on the cell-specific SINR

- Candidate options:

  - Option 1: Measurement error to be considered for event prediction use case during WI phase

  - Option 2: Measurement error not to be considered for event prediction use case.

  - Option 3: Measurement error to be considered for event prediction use case during WI phase. Error model to be similar to that of RRM measurement prediction use case.

  - Option 4: Measurement error to be considered for event prediction use case during WI phase. Error model may be different from the one used for RRM measurement prediction use case.

- Recommended WF

- Discuss above listed Candidate Options.

#### Issue 3-1-4: Impact on core requirements for event prediction use case

- Proposals and Observations:

  - Proposal 1 (Xiaomi): Measurement event prediction may have impact on predicated event triggered L3 reporting.

  - Proposal 2 (OPPO): For indirect prediction, RAN4 to discuss whether the existing handover delay requirements can be reused.

  - Proposal 3 (CATT): With AI/ML based measurement event prediction, some components in conditional handover delay requirements can be updated or removed, e.g., Tprocessing and Tmeasure. FFS details

  - Proposal 4 (vivo): For measurement event prediction use-case, RAN4 to deprioritize the scenario on indirect measurement event prediction for spatial domain

  - Proposal 5 (ZTE): RAN4 should discuss the RRM impact on indirect measurement event prediction and direct measurement event prediction.

  - Proposal 6 (Ericsson): RAN4 to consider impact on reporting delay requirement due to predicted event triggered reporting.

- Recommended WF

  - RAN4 should wait for further progress in RAN2 before starting discussion on core requirements in RAN4 for event prediction use case.

#### Issue 3-1-5: Scenario priorities for event prediction

- Proposals and Observations:

  - Proposal 1 (ZTE): : For measurement event prediction, RAN4 should at least consider scenarios2(FR1 to FR1 intra-frequency temporal domain caseB), scenario 3 (FR1 to FR1 inter-frequency (frequency domain)), and scenario 4 (FR2 to FR2 intra-frequency temporal domain case A).

  - Proposal 2 (Nokia): RAN4 to prioritize, based on RAN2 outcome, from the available cases for indirect measurement event prediction using temporal domain Case A or Case B, and conduct a study on the performance and requirements for the selected case.

  - Proposal 3 (Samsung): RAN4 to study if HO requirements including delay, interruption, side conditions and accuracy are needed to be updated for AI/ML-assisted mobility, temporal domain prediction case A can be taken as starting point considering verified handover decision option 3 and 2

- Candidate options:

  - Option 1: RAN4 to consider scenarios2(FR1 to FR1 intra-frequency temporal domain caseB), scenario 3 (FR1 to FR1 inter-frequency (frequency domain)), and scenario 4 (FR2 to FR2 intra-frequency temporal domain case A).

- Recommended WF

- RAN4 to discuss if Option 1 is agreeable.

#### Issue 3-1-6: Groun truth definition for event prediction

- Proposals and Observations:

  - Proposal 1 (Qualcomm): : For the definition of ground truth of predicted event occurrence time accuracy (reported predicted event occurrence time – ground truth of the event occurrence time) in the time domain, the ground truth of the event occurrence time should be defined by the same principles as the ground truth of predicted L3-RSRP. If a down selection is made during the study item phase for the definition of ground truth of predicted L3-RSRP, the same principle should be inherited for the definition of ground truth of predicted event occurrence time. The options for the definition of ground truth of predicted L3-RSRP are listed below (options are duplicated below for reference from our companion paper submitted for AI 7.21.2.1):

    - Option 1: Ground truth based on expected measurement result (spec-defined expected L3-RSRP)

      - The ground truth of L3-RSRP predicted for the time instance X is derived based on the expected reception power of the reference signal at UE assuming the measurement rate defined in RRM spec, plus a margin to accommodate uncertainties in the actual UE L3 measurement rate. The value of the margin needs a further study, and it can vary for different measurement configurations, etc.

      - FFS on whether and how to accommodate the measurement error due to noisy channel, RF calibration error, UE Rx beamforming gain, etc.

    - Option 2-A: Ground truth based on measurement report from the UE under the SAME radio condition as the prediction-based L3-RSRP report (UE-reported L3-RSRP with noise)

      - The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X.

        - Y can be differently defined depending on factors such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc.

        - The radio condition such as S(I)NR remains the same during the target inference time instance and the report instance, i.e., from X – Y ms to X + Y ms.

        - To avoid the UE from cheating the test, TE can either create another cell with the same radio conditions to the extent possible (e.g., large-scale fading) as the target cell for the L3-RSRP prediction or run two rounds of testing with the same configuration: one for the prediction-based L3-RSRP report and the other for the actual measurement-based L3-RSRP report.

    - Option 2-B: Ground truth based on measurement report from the UE under the IDEAL radio condition (UE-reported L3-RSRP with noise-free)

      - The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X.

        - Y can be differently defined depending on factors such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc.

        - The radio condition such as S(I)NR for the measurement report is set such that the measurement result at the UE can be accurate enough.

        - Another round of tests would be needed for the L3-RSRP measurement report

    - RAN4 to strive to not differentiate the definition of the ground truth between FR1 and FR2.

- Recommended WF

- RAN4 to wait for the conclusion on Issue 2-1-5 before starting discussion on this issue.

# Topic #3: Testability and interoperability

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2503300 | Xiaomi | Proposal 1: For testing in FR1 RRM measurement prediction, the configuration of TDL channel model can be considered. Proposal 2: For testing in FR1 RRM measurement prediction, the way the ground truth is extracted has impact on the testing procedure: - If the ground truth of L3-RSRP is calculated by TE, new test procedure on how TE determine the ideal L3-RSRP need to be further studied. - If the ground truth of L3-RSRP is reported by UE, the legacy test procedure can be taken as baseline. Proposal 3: For the scenario of FR1 to FR1 inter-frequency inter-cell frequency domain scenario, the legacy test procedure can be taken as baseline for single cell prediction. FFS for multi-cells prediction. Proposal 4: For FR2 measurement prediction, conclusions on testability that RAN4 made for AI/ML BM can be taken as a baseline. |
| R4-2503353 | OPPO | Proposal 1: RAN4 to discuss how to model the time-varying characteristics for temporal prediction (e.g., according to UE trajectories or changing the channel model with a stable transmission power). Proposal 2: For FR2 prediction, proposals and conclusions that RAN4 made for AI BM can be reused as baseline, e.g., whether the chamber in OTA testing can be utilized and how to utilize/revise it. Proposal 3: For inter-carrier scenario (inter-frequency inter-cell prediction), the following aspects need to be discussed: - how to model co-located or co-site scenario for the cell to be measured and the cell to be predicted. - how to construct and obtain different cell’s RSRP corresponding to a specific temporal relationship. - channel modelling used for inter-cell prediction Proposal 4: RAN4 to consider including L3 filter coefficient as part of the testing side conditions. Proposal 5: RAN4 to consider how to setup the measurement configurations (e.g., OW, PW, MRRT, etc.) in the tests based on the sets of requirements. Proposal 6: For intra and inter frequency measurement prediction, RAN4 to use legacy testing setup as baseline and include the AI specific parameters if needed. Proposal 7: For intra frequency measurement prediction, consider 2 cells (serving cell and one neighbour cell) in the testing setup. Proposal 8: For inter frequency measurement prediction, more than 3 cells are needed if cluster approach is assumed and the exact number can be decided in WI stage according to the requirements discussion. Proposal 9: Regarding the ground truth derivation in tests, the actual measurement report based on legacy procedure can be used. Proposal 10: The testing goal for measurement prediction is to verify - the measurement reported in OW meets the new requirements if defined; - the predicted measurement in PW meets the accuracy requirements under certain side conditions. Proposal 11: RAN4 to discuss the configurations for generalization in WI stage and the methodology discussed in NR_AIML_Air WI can be used as starting point. |
| R4-2503413 | CATT | Proposal 1: For scenario 2 and 3, RAN4 to verify whether measurement results satisfy the existing accuracy requirements under certain side conditions, e.g., MRRT or MRRS. Proposal 2: For scenario 4, RAN4 can use HO delay requirement as a starting point when HO delay is shortened. For other HO enhanced aspects, RAN4 further discuss how to define HO performance metric. Proposal 3: RAN4 configures one SSB per cell and the measured RSRP results are regarded as cell-level RSRPs. Proposal 4: TE randomly adjusts the transmitting power of SSBs from all cells to model time-variant RSRP changes, which makes ground truth known to TE. The range of TX power can be FFS. Proposal 5: AWGN or TDL channel can be used in FR1 conducted test. Proposal 6: The following values can be a baseline for the number of configured cells in FR1 HO performance test: - [9] cells for intra-frequency predictions; - [5] cells for inter-frequency predictions; - And values of carrier frequencies can be determined later. Proposal 7: For measurement reduction test, less cells can be set up in test, e.g., [3] cells. Proposal 8: RAN4 to postpone discussion on test set up in FR2 until sufficient progress is made in AI/ML BM. Proposal 9: Colocation can be reflected by a minor RSRP difference between two carriers. |
| R4-2503520 | Apple | Observation 1: The primary challenge in achieving generalization for AI/ML models in AI/ML mobility is ensuring that a model trained with specific cell or inter-cell characteristics can perform effectively across different cells and inter-cell environments. A critical question is whether an AI/ML model trained in one deployment scenario can be directly applied to a different scenario without retraining. Addressing this challenge is essential, as it could reduce the need for multiple AI/ML models, thereby enhancing adaptability and robustness in diverse and realistic radio conditions (SNR). Proposal 1: RAN4 should define performance metrics/KPIs and clarify the objectives, testing properties, and methodologies for evaluating AI/ML functionalities across each mobility use case and sub-use case addressing both Goal 1 (measurement reduction) and Goal 2 (mobility optimization): - 1st goal: To assess the trade-off between measurement reduction and measurement prediction accuracy and its effect to mobility performance with legacy as a baseline with MRRT, SNR as side conditions - 2nd goal: To determine if the AI/ML model achieves measurable improvements in handover (HO) performance with PW, OW, SNR as side conditions Proposal 2: For temporal domain prediction, to investigate whether legacy test setups can serve as a baseline and how to introduce controlled randomness and time-domain variation/correlation to validate testing objectives comparing predicted versus actual outcomes. Proposal 3: Define multiple test iterations, each designed to reflect unique, time-varying conditions such as reception power and SNR influenced by UE trajectory, to ensure coverage of potential conditions in AI/ML-assisted mobility scenarios Proposal 4: For Rel-19 AI/ML mobility, it is essential for RAN4 to investigate generalization-related tests and requirements arising from varying propagation conditions, SNR levels, and cell-specific Tx/codebook implementations across different cells and frequency layers. Proposal 5: RAN4 should investigate the side conditions for RSRP prediction pertinent to AI/ML mobility, along with network assistance information required to guarantee generalization and effective operation across multiple target cells and different frequency layers. Additionally, it should leverage the discussions in AI/ML beam management regarding the signaling of associated IDs to ensure consistency between primary and secondary carrier components, thereby enhancing generalization capabilities. - Proposal 6: RAN4 should leverage the LCM progress and requirements discussed in R-19 WI related to AI/ML Air Interface and investigate enhancements to these LCM procedures to RRM for mobility. |
| R4-2503609 | MediaTek | Proposal 1: Further check whether it is possible to change Tx power every symbol at TE using conductive test in FR1 for AI/ML mobility. Proposal 2: In FR1, computer simulations alike test setup can be adopted for AI mobility test. The following figure can be used to determine the power transmitted at TE on each cell, i.e., TE can transmit the signals of cell 1, 2, 3 with time-variance following a certain channel model. |
| R4-2503777 | CMCC | Proposal 1: for test on intra-frequency temporal domain predictiontes (scenario 2 and 4), the test setup for legacy mobility can be used as baseline but with modification, since test setup for legacy mobility is based on two cells while AI/ML test targets for intra-cell. Proposal 2: for test on inter-frequency frequency domain prediction (scenario 3), it is proposed to discuss how to update leagcy test setup based on two cells, since RAN2 agree to focus on frequency domain for cluster based approach. Proposal 3: for FR1 AI/ML mobility, it is proposed that the ground truth is available at TE. Proposal 4: for AI/ML Mobility testing in FR2, it is proposed to take the outcome of AI/ML BM use case as baseline. |
| R4-2503853 | ZTE | Observation 1: One side model is considered in AI mobility. For one side mode, the inference is performed entirely on one side (UE side or NW side), so there should be no interoperability issues involved in the inference process. However, the interaction maybe happened between UE and NW mainly depended on signalling which should be discussed in RAN2 at first. Observation 2: Even the measurement results of skipped samples are predicted, the network performance based on predicted results is highly likely to differ from the performance based on actual measurements due to the vary channel condition. Proposal 1: The testability discussion of sub-use-case 1 (L1-L1) and sub-use-case 3 (L1-L3) should wait the progress of the testability study in AI-BM. And RAN4 can start the discussion on sub-use-case 2 (L3-L3). Proposal 2: For testing on AI mob, prefer to use AWGN/TDL channel model as baseline. Observation 3: the channel conditions of real environment are complex and diversified, and the channel state is not stationary and constantly changing for mobility. Proposal 3: RAN4 should discuss how to reflect randomness and variability in the testing process, such as reception power and SNR influenced by mobility. Observation 4: Genralization performance is important for prediction, RAN2 thinks that AI/ML model generalization could be addressed after sufficient performance gains for different use cases are found, and RAN4 should discuss whether/how to define and test the generalization performance of AI/ML model. Proposal 4: Generalization discussion in RAN4 can be started if sufficient progress is made in RAN2 or NR_AIML_Air WI. Proposal 5: From the perspective of test, core part and performance part requirements should be considered, i.e. measurement delay and measurement accuracy. Proposal 6: RAN4 to discuss numbers of Tx beam/SSBs to be configured and the number of cell to be measured for AI/ML mobility. Proposal 7: For inter-frequency predictions, consider multi-cell setup in the test, including serving cell and neighbor cells. |
| R4-2503889 | Huawei, HiSilicon | Proposal 1: For testing goal, the principle of defining the minimum performance can be discussed based on the following two options: - Option 1: To verify whether the difference between the predicted measurement and the ideal measurement is within the existing relative measurement accuracy requirement. - Option 2: To verify whether the difference between the predicted measurement and the ideal measurement is within X dB, where X is larger than legacy relative measurement accuracy margin requirement, under the side condition of MRRT and other parameters. - FFS: The value of MRRT. - FFS: Other parameters and the values, e.g., the prediction window. Observation 1: AI based prediction may help reduce the power consuming due to the measurement reduction. Observation 2: AI based prediction may help reduce the delay of data transmission and improve the throughput due to the reduction of measurement gap. Proposal 2: When defining measurement prediction requirements, the gain derived from power consumption reduction and measurement gap reduction shall also be taken into account. Proposal 3: For FR1 to FR1 prediction testing, the following configuration can be considered. - 1 Tx at TE as a starting point - Channel models to be AWGN or TDL Proposal 4: For FR1-to-FR1 prediction testing, reuse RAN4 legacy test setup for cases where one cell to be measured and one cell to be predicted. Proposal 5: Postpone the testability discussion for FR2-to-FR2 prediction to wait the progress of the testability study in AI-BM. |
| R4-2503931 | Ericsson | Proposal 1: Test setup for testing the AI/ML model deployed at UE to support intra-cell temporal prediction scenario is based on two-cell setup, similar to the test setup for legacy L3-based mobility. Proposal 2: The aim of testing for intra-cell prediction should be to validate that the measurements reported by the UE for each cell in the test setup meet: - requirement for the measurements performed inside the OW, - requirement for the measurements predicted for the time instances inside the PW. Proposal 3: The aim of testing for inter-cell scenario should be to validate that the measurements reported by the UE: - meet the requirements for measured cell, and - meet the requirements for the target cell. Proposal 4: Existing test setup defined to validate requirements for inter-frequency measurements is re-used or used as a baseline for discussion on testing AI/ML based mobility for scenario 3. - Cell 1 in the test can be considered as the cell for measurement. - Cell 2 in the test can be considered as the target cell for measurement prediction. Proposal 5: Beam 1 and beam 2 in the predicted relative RSRP accuracy formula agreed in RAN4#114 can be from the same cell or different cells. Proposal 6: Reported RSRP of cell2/beam2 in the predicted relative RSRP accuracy formula agreed in RAN4#114 can be predicted or measured RSRP values. Proposal 7: Predicted relative RSRP accuracy for temporal prediction is defined as: - Predicted relative RSRP accuracy for temporal prediction = (reported predicted RSRP1 of a cell/beam – reported RSRP2 of the same cell/beam) – (ground truth of RSRP1 of a cell/beam – ground truth of RSRP2 of the same cell/beam). - Reported RSRP2 can be both predicted or measured. Observation 1: Aspects such as availability of the timing information relating to the measurements and predictions performed and reported by the UE, impact of L3 filtering, and number of samples for L1 filtering in the test are more relevant for the test configuration discussion. Proposal 8: Ground truth in FR1 testing is the calculated value based on transmitted power known at the TE. Other parameters that may have an impact on ground truth calculation are not precluded. |
| R4-2504049 | vivo | Observation 1: Whether and how to emulate a multi-beam environment with a fading channel of spatial characteristics is the key issue in AI BM discussion. Observation 2: There are no testability issues for the conducted testing in scenario 2 and 3. Observation 3: In AI/ML based BM case 1, all the Tx beams with spatial domain characteristics need to be emulated in OTA chamber, while the spatial characteristics are not necessarily needed in AI mobility test system. Proposal 1: RAN4 to consider OTA testing for FR2 (scenario 4) and using one probe to emulate different Tx beams by applying different power for AI mobility FR2 beam level test for one cell. Proposal 2: RAN4 to consider using AWGN/TDL channel model as the baseline for testing. Proposal 3: RAN4 to consider modelling reference UE trajectory for OTA testing and to emulate the dynamic channel by changing the doppler, changing the SNR, rotate UE in turnable, and change probe power. Proposal 4: RAN4 to take the potential testability setups for the 3 scenarios into consideration as the following: Observation 4: A simplified RAN4 test system would skip Tx beams and the input dimension mismatch issue may happen in the lab. Proposal 5: RAN4 to consider evaluating the impact on the model input dimensions mismatch (e.g., the input Tx beam number for inference is not equal to the input Tx beam number for training because of the test setup) for the performance. |
| R4-2504234 | Nokia | Proposal 1: RAN4 should define test setups considering L1 or L3 measurements from serving and neighbouring cells with related configuration (inputs, prediction horizon). Observation 1: RAN4 should consider the additional aspects for testing (compared to the legacy test) in AI/ML mobility such as L3 filtering, channel model, maximum number of cells, Tx beams and the number of AoAs. Proposal 2: RAN4 should consider higher number of cells for inter-frequency inter-cell scenario (i.e. scenario 3). Proposal 3: For FR1, emulation of the radio channel variations is important for evaluating the prediction accuracy of the AIML models/ functionalities. Hence RAN4 should consider fading channels (like CDL/ TDL) for testing. Observation 2: Majority of test cases specified in 3GPP TS 38.133 assume no application of L3 filtering (i.e., with the filter coefficient set to zero) and emulate only a limited number of beams in the test setup. Observation 3: Depending on the configuration of beam consolidation selection, L3 cell-level RSRP predictions at Point C may equal to L3 beam-level RSRP predictions at Point E. Proposal 4: RAN4 to consider L3 filtering and multi-beam scenarios while studying the test cases for AI/ML mobility uses cases. Observation 4: UE can perform prediction and Ground Truth measurements sequentially or concurrently depending on its capabilities. Proposal 5: TE must account for the delays to acquire prediction and corresponding Ground Truth as some UEs might not be capable of doing both concurrently. Proposal 6: For testing time domain prediction case B, RAN4 to follow RAN2 agreement regarding skipping pattern indication either part of inference configuration or test configuration. Proposal 7: RAN4 should consider how to model different non-static channel characteristics per cell/site due to moving UE trajectories and to emulate channel variations during the test. Proposal 8: RAN4 should study how to ensure consistency between different carriers in the context of AI/ML mobility SI. Proposal 9: RAN4 should wait for further progress on generalization discussions in RAN2 before starting any discussion on generalization related topics in RAN4. Proposal 10: For FR2, the testing setup and channel model in the AI/ML BM use case which is under study should be reused as a baseline for measurement prediction use case testing. |
| R4-2504286 | Samsung | Observation 1: Due to the fast fading channel condition random fluctuation, the definition of high-SNR shall be different at each time instant. Observation 2: If to consider all the uncertainties including filtering, channel fading condition, measurement samples, etc., an unnecessarily wide predicted accuracy gain range may make it easier for the UE to pass RSRP prediction accuracy test. Observation 3: For FR2 testing, on the exact AoA to be used, the test can be done in one Rx beam peak direction or in one or more directions from the UE spherical coverage Proposal 1: RAN4 to consider the following testability aspects for AI mobility - Consideration of serving and neighbour cells with robust cell configurations - Generalization across cells for intra-frequency and/or across frequencies should be guaranteed - Multi-cell setup and number of cells to measure - The legacy number of cells per carrier that the UE is able to monitor for intra-frequency and inter-frequency can be considered as the baseline Proposal 2: Regarding the emulated SNR, to align the definition of high-SNR at each time instant under the fast fading channel condition, the following two solutions of controlling the noise/SNR can be done at TE - The upper bound of signal power as the maximum available Tx power transmitted from TE, then TE controls the level of artificial noise for target SNRs at different time instances - Control the SNR and artificial noise dynamically with total power constraint for target SNRs at different time instances - The testable SNR range in the test system and the assessment of the range shall be discussed Proposal 3: For FR1 RSRP prediction accuracy testing, the ideal RSRP (i.e., the ground truth) can be of the transmit power settled by TE in noise-free/high SNR test condition. Proposal 4: To make FR1 RSRP prediction accuracy test feasible, at least the following information should to know at TE： - Prediction related time, the time can be the absolute time UE performs the prediction and the relative prediction time - Fast fading channel condition Proposal 5: Suggest to keep UE measurement related parameter information including L3 filtering, L1 filtering and the timing UE performs and/or finishes the measurement transparent to TE - A suitable test tolerance range of predicted accuracy gain should to be defined to consider the uncertainties as much as possible and to ensure UE will not pass the testing easily in parallel Proposal 6: If Ground truth for FR1 will be settled by measurement report, the SNR side condition requirement under the fast fading channel environment should be defined Proposal 7: For FR2 testing in AI/ML mobility, at least the following aspects can be studied: - Emulation of the propagation conditions - AWGN and TDL - Emulation of channel models - The number of emulated transmission reception points (TRxP) - Emulation of noise and EPRE - AoA setup - Rx beam direction assumption Proposal 8: For FR2 testing in AI/ML mobility, multiple beam environment could be emulated Proposal 9: For FR2 measurement prediction testing, RAN4 made for AI/ML-BM can be reused as a baseline - RAN4 to discuss what test parameters/methodologies on testability and interoperability for which AI/ML-PHY scenarios/cases could be reused to L3 cell level measurement prediction testing to avoid the duplicate study Proposal 10: For FR2 testing in AI mobility, if the test is to mostly verify the RRM RSRP prediction performance, RAN4 to study the feasibility of testing in beam peak direction on the exact AoA to be used Proposal 11: For Rel-19 AI/ML mobility, it is more necessary for RAN4 to study generalization related tests/requirements - Reuse the principle/methodologies/signallings on generalization achieved in AI/ML for NR air interface as much as possible |
| R4-2504526 | Qualcomm Incorporated | Proposal 1: The following measurement points should all be included in the scope of the testability study: - L3 cell level RSRP (Point C) - L3 beam-level (Point E) - L1 beam-level (Point A1) Proposal 2: RAN4 to further study the impact of time-varying channels and SNR due to fading and UE mobility on the definition of ground truth L3-RSRP. Proposal 3: RAN4 to consider the following approaches to avoid UE from cheating the test, if the ground truth is defined based on the actual UE measurement report. - Two neighbor cells: TE creates another cell with the same radio conditions to the extent possible (e.g., large-scale fading) as the target cell for the L3-RSRP prediction. Note that the channels between the two neighbor cells with the same set of configurations may still realize different instantaneous channel profiles. Thus, the exact same instantaneous channel may need to be applied to both neighbor cells. - Two rounds of testing: TE runs two rounds of testing with the same configuration: one for the prediction-based L3-RSRP report and the other for the actual measurement-based L3-RSRP report. - Note that the test framework also needs to consider how to verify UE LCM performance, if needed. Proposal 4: RAN4 to study how and to what extent data consistency during training and inference, and between serving cell configuration (e.g. TCI state update, TA updates, etc) and dynamic radio environment, needs to be ensured. Proposal 5: Test framework for AI/ML based RRM measurement prediction should be defined in a way that the generalization or robustness of the model can be evaluated. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 4-1: New Testability aspects

Sub-topic description:

In this sub-topic the company proposals about the testability aspects for AI/ML Mobility use cases are summarized.

#### Issue 4-1-1: Testing Setup in FR1

- Proposals and Observations:

  - Proposal 1 (Huawei):

    - For FR1 to FR1 prediction testing, consider (i) 1 Tx at TE as a starting point and (ii) channel models to be AWGN or TDL. Reuse RAN4 legacy test setup for cases where one cell to be measured and one cell to be predicted.

  - Proposal 2 (Nokia):

    - Consider L3 filtering and multi-beam scenarios

    - Emulation the radio channel variations by considering fading channels such as CDL or TDL to test and evaluate the prediction accuracy of the AIML models/ functionalities

    - TE considers the delays to acquire prediction and corresponding Ground Truth as some UEs might not be capable of doing both concurrently

    - Use the skipping pattern indication agreed in RAN2

    - Legacy test setup may be considered as baseline for intra cell scenarios (i.e. scenarios 2 and 4 with sub-use case 2).

    - Consider L3 filtering and multi-beam scenarios while studying the test cases for AI/ML mobility uses cases.

  - Proposal 3 (Vivo):

    - Consider using AWGN/TDL channel model as the baseline for testing.

    - Consider modelling reference UE trajectory for OTA testing and to emulate the dynamic channel by changing the doppler, changing the SNR, rotate UE in turnable, and change probe power

    - Consider potential testability setups in FR1

| Scenario number | Sub-scenario | Freq Range | Test method | Channel model (as start point) | Min cell number | Min. probe number | SNR | UE trajectory |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 1 | FR1 | Conducted | AWGN/ TDL | 1 serving cell | N/A | legacy SNR | Change doppler and SNR |
|  | 2 |  |  |  | 2 (1 serving cell, 1 neighbor cell) |  |  |  |
| 3 | 1 | FR1 | Conducted | AWGN/ TDL | 2 (1 serving cell, 1 neighbor cell) | N/A | legacy SNR | Change doppler and SNR |
|  | 2 |  |  |  | 3 (2 co-located neighbor cell, 1 serving cell) |  |  |  |

  - Proposal 4 (Apple):

    - Define multiple test iterations, each designed to reflect unique, time-varying conditions such as reception power and SNR influenced by UE trajectory, to ensure coverage of potential conditions in AI/ML-assisted mobility scenarios

  - Proposal 5 (ZTE):

    - The testability discussion of sub-use-case 1 (L1-L1) and sub-use-case 3 (L1-L3) should wait the progress of the testability study in AI-BM. And RAN4 can start the discussion on sub-use-case 2 (L3-L3).

    - Use AWGN/TDL channel model as baseline.

    - Discuss how to reflect randomness and variability in the testing process, such as reception power and SNR influenced by mobility

    - Consider core part and performance part requirements, i.e. measurement delay and measurement accuracy

    - Discuss numbers of Tx beam/SSBs to be configured and the number of cell to be measured for AI/ML mobility

  - Proposal 6 (MediaTek):

    - Adopt computer simulations alike test setup for AI mobility test, e.g., the power transmitted at TE on each cell is determined, and the TE transmits the signals of cell 1, 2, 3 with time-variance following a certain channel model.

  - Proposal 7 (Xiaomi):

    - For testing in FR1 RRM measurement prediction, the configuration of TDL channel model can be considered.

  - Proposal 8 (CATT):

    - RAN4 configures one SSB per cell and the measured RSRP results are regarded as cell-level RSRPs

    - AWGN or TDL channel can be used in FR1 conducted test

  - Proposal 9 (OPPO):

    - Include L3 filter coefficient as part of the testing side conditions

    - Discuss how to setup the measurement configurations (e.g., OW, PW, MRRT, etc.) in the tests based on the sets of requirements.

    - For intra frequency measurement prediction, consider 2 cells (serving cell and one neighbor cell) in the testing setup.

    - For inter frequency measurement prediction, more than 3 cells are needed if cluster approach is assumed and the exact number can be decided in WI stage according to the requirements discussion.

  - Proposal 10 (CMCC):

    - For test on intra-frequency temporal domain prediction (scenario 2 and 4), the test setup for legacy mobility can be used as baseline but with modification, since test setup for legacy mobility is based on two cells while AI/ML test targets for intra-cell

    - For test on inter-frequency frequency domain prediction (scenario 3), it is proposed to discuss how to update leagcy test setup based on two cells, since RAN2 agree to focus on frequency domain for cluster based approach.

  - Proposal 11 (Samsung):

    - Emulate SNR to align the definition of high-SNR at each time instant under the fast-fading channel condition, the following two solutions of controlling the noise/SNR can be done at TE

      - The upper bound of signal power as the maximum available Tx power transmitted from TE, then TE controls the level of artificial noise for target SNRs at different time instances

      - Control the SNR and artificial noise dynamically with total power constraint for target SNRs at different time instances

    - Discuss testable SNR range in the test system and the assessment of the range

  - Proposal 12 (Ericsson):

    - Test setup for testing the AI/ML model deployed at UE to support intra-cell temporal prediction scenario is based on two-cell setup, similar to the test setup for legacy L3-based mobility.

    - Existing test setup defined to validate requirements for inter-frequency measurements is re-used or used as a baseline for discussion on testing AI/ML based mobility for scenario 3.

      - Cell 1 in the test can be considered as the cell for measurement.

      - Cell 2 in the test can be considered as the target cell for measurement prediction.

  - Proposal 13 (Qualcomm):

    - Test framework for AI/ML based RRM measurement prediction should be defined in a way that the generalization or robustness of the model can be evaluated.

- Candidate Options:

    - Option 1: Discuss the channel model for testing: AWGN, TDL, and/or CDL

    - Option 2: Consider multi-beam scenarios

    - Option 3: Study how legacy testing setup for L3 measurements can be reused

    - Option 4: consider AI specific configurations (e.g., L3 filter coefficient, OW, PW, MRRT, etc.)

    - Option 5: Discuss the SNR for testing

- Recommended WF

    - To discuss the options listed above.

#### Issue 4-1-2: Prediction consistency in time domain

- Proposals and Observations:

  - Proposal 1 (Apple):

    - For temporal domain prediction, to investigate whether legacy test setups can serve as a baseline and how to introduce controlled randomness and time-domain variation/correlation to validate testing objectives comparing predicted versus actual outcomes.

  - Proposal 2 (MediaTek):

    - Check whether it is possible to change Tx power every symbol at TE using conductive test in FR1 for AI/ML mobility

  - Proposal 3 (Nokia):

    - Study how to ensure consistency between different carriers

    - Consider how to model different non-static channel characteristics per cell/site due to moving UE trajectories and to emulate channel variations during the test.

  - Proposal 4 (CATT):

    - TE randomly adjusts the transmitting power of SSBs from all cells to model time-variant RSRP changes. The range of TX power can be FFS.

  - Proposal 5 (OPPO):

    - Discuss how to model the time-varying characteristics for temporal prediction (e.g., according to UE trajectories or changing the channel model with a stable transmission power).

  - Proposal6 (Qualcomm):

    - RAN4 to study how and to what extent data consistency during training and inference, and between serving cell configuration (e.g. TCI state update, TA updates, etc) and dynamic radio environment, needs to be ensured.

- Candidate Options:

    - Option 1: Discuss how to model different time-varying characteristics per cell/site due to moving UE trajectories

    - Option 2: Discuss how to incorporate controlled randomness and the extent of time-domain variation and correlation

    - Option 3: Model time-variant RSRP changes by randomly adjusting TE’s transmitting power of SSBs from all cells

- Recommended WF

    - To discuss the options listed above.

#### Issue 4-1-3: Multiple cells in the testing setup

- Proposals and Observations:

  - Proposal 1 (Xiaomi):

    - FFS for multi-cells prediction. For the scenario of FR1 to FR1 inter-frequency inter-cell frequency domain scenario, the legacy test procedure can be taken as baseline for single cell prediction.

  - Proposal 2 (CATT):

    - Baseline for the number of configured cells in FR1 HO performance test:

      - [9] cells for intra-frequency predictions

      - [5] cells for inter-frequency predictions

      - For measurement reduction test, less cells can be set up in test, e.g., [3] cells.

    - Later determine number of carrier frequencies

  - Proposal 3 (Samsung):

    - Consider serving and neighbor cells with robust cell configurations

    - Generalization across cells for intra-frequency and/or across frequencies should be guaranteed

    - Multi-cell setup and number of cells to measure: the legacy number of cells per carrier that the UE is able to monitor for intra-frequency and inter-frequency can be considered as the baseline

  - Proposal 4 (Apple):

    - Investigate the side conditions for RSRP prediction pertinent to AI/ML mobility, along with network assistance information required to guarantee generalization and effective operation across multiple target cells and different frequency layers.

  - Proposal 5 (Nokia):

    - Consider higher number of cells for inter-frequency inter-cell scenario (i.e. scenario 3)

    - Define test setups considering L1 or L3 measurements from serving and neighboring cells with related configuration (inputs, prediction horizon).

  - Proposal 6 (Oppo):

    - For intra frequency measurement prediction, consider 2 cells (serving cell and one neighbour cell) in the testing setup.

    - For inter frequency measurement prediction, more than 3 cells are needed if cluster approach is assumed, and the exact number can be decided in WI stage according to the requirements discussion.

  - Proposal 7 (ZTE):

    - Tests on inter-frequency predictions consider multi-cell setup including serving cell and neighbor cells.

  - Proposal 8 (CMCC):

    - for test on inter-frequency frequency domain prediction (scenario 3), it is proposed to discuss how to update leagcy test setup, since RAN2 agree to focus on frequency domain for cluster based approach.

- Candidate Options

    - Option 1: Study multi-cell setup in the test for inter-frequency predictions

    - Option 2: Consider at least two cells: serving cell and another/targe cell for inter-cell measurement prediction

    - Option 3: Specify that [9] cells for intra-frequency predictions; [5] cells for inter-frequency predictions. For measurement reduction test, less cells can be set up in test, e.g., [3] cells.

    - Option 4: Consider any side condition for multiple cells

- Recommended WF

    - To discuss the options listed above.

#### Issue 4-1-4: Ground Truth derivation in FR1

- Proposals and Observations:

  - Proposal 1 (OPPO):

    - Regarding the ground truth derivation in tests, the actual measurement report based on legacy procedure can be used.

  - Proposal 2 (CMCC):

    - The ground truth is available at TE.

  - Proposal 3 (Xiaomi):

    - For testing in FR1 RRM measurement prediction, the way the ground truth is extracted has impact on the testing procedure:

      - If the ground truth of L3-RSRP is calculated by TE, new test procedure on how TE determine the ideal L3-RSRP need to be further studied.

      - If the ground truth of L3-RSRP is reported by UE, the legacy test procedure can be taken as baseline.

  - Proposal 4 (Samsung):

    - For FR1 RSRP prediction accuracy testing, the ideal RSRP (i.e., the ground truth) can be of the transmit power settled by TE in noise-free/high SNR test condition

  - Proposal 5 (Ericsson):

    - Ground truth in FR1 testing is the calculated value based on transmitted power known at the TE. Other parameters that may have an impact on ground truth calculation are not precluded

  - Proposal 6 (Qualcomm):

    - Study the impact of time-varying channels and SNR due to fading and UE mobility on the definition of ground truth L3-RSRP

    - Test consider the following approaches to avoid UE from cheating the test, if the ground truth is defined based on the actual UE measurement report.

      - Two neighbor cells: TE creates another cell with the same radio conditions to the extent possible (e.g., large-scale fading) as the target cell for the L3-RSRP prediction. Note that the channels between the two neighbor cells with the same set of configurations may still realize different instantaneous channel profiles. Thus, the exact same instantaneous channel may need to be applied to both neighbor cells.

      - Two rounds of testing: TE runs two rounds of testing with the same configuration: one for the prediction-based L3-RSRP report and the other for the actual measurement-based L3-RSRP report.

  - Proposal 7 (CATT):

    - Ground truth is known to TE when TE randomly adjusts the transmitting power of SSBs from all cells to model time-variant RSRP changes

- Candidate Options

    - Option 1: ground truth is initially known by the TE, e.g., the ground truth is the transmit power settled by TE.

    - Options 2: actual measurement report based on legacy procedure is used to derive the ground truth.

    - Option 3: study the impact of time-varying channels and SNR on the ground-truth definition

- Recommended WF

    - To discuss the options listed above.

### Sub-topic 4-2: Testing goals and related aspects

Sub-topic description:

In this sub-topic the company proposals about the testability and interoperability aspects for AI/ML Mobility use cases are summarized.

#### Issue 4-2-1: Testing goal

Background:

Following agreement was reached in RAN4#113 meeting:

- Agreement - As baseline, the testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved.

- Proposals and Observations:

  - Proposal 1 (Huawei):

    - For testing goal, the principle of defining the minimum performance can be discussed based on the following two options:

      - Option 1: To verify whether the difference between the predicted measurement and the ideal measurement is within the existing relative measurement accuracy requirement.

      - Option 2: To verify whether the difference between the predicted measurement and the ideal measurement is within X dB, where X is larger than legacy relative measurement accuracy margin requirement, under the side condition of MRRT and other parameters.

      - FFS: The value of MRRT.

      - FFS: Other parameters and the values, e.g., the prediction window

    - When defining measurement prediction requirements, the gain derived from power consumption reduction and measurement gap reduction shall also be taken into account.

  - Proposal 2 (Vivo):

    - Evaluate the impact on the model input dimensions mismatch (e.g., the input Tx beam number for inference is not equal to the input Tx beam number for training because of the test setup) for the performance

  - Proposal 3 (Apple):

    - Define performance metrics/KPIs and clarify the objectives, testing properties, and methodologies for evaluating AI/ML functionalities across each mobility use case and sub-use case addressing both Goal 1 (measurement reduction) and Goal 2 (mobility optimization):

      - 1st goal: To assess the trade-off between measurement reduction and measurement prediction accuracy and its effect to mobility performance with legacy as a baseline with MRRT, SNR as side conditions

      - 2nd goal: To determine if the AI/ML model achieves measurable improvements in handover (HO) performance with PW, OW, SNR as side conditions

  - Proposal 4 (CATT):

    - For scenario 2 and 3, RAN4 to verify whether measurement results satisfy the existing accuracy requirements under certain side conditions, e.g., MRRT or MRRS.

    - For scenario 4, RAN4 can use HO delay requirement as a starting point when HO delay is shortened. For other HO enhanced aspects, RAN4 further discuss how to define HO performance metric.

  - Proposal 5 (Oppo):

    - The measurement reported in OW meets the new requirements if defined

    - The predicted measurement in PW meets the accuracy requirements under certain side conditions.

  - Proposal 6 (Qualcomm):

    - Include in the scope of the testability study:

      - L3 cell level RSRP (Point C)

      - L3 beam-level (Point E)

      - L1 beam-level (Point A1)

  - Proposal 7 (Ericsson)

    - The aim of testing for intra-cell prediction should be to validate that the measurements reported by the UE for each cell in the test setup meet:

      - requirement for the measurements performed inside the OW,

      - requirement for the measurements predicted for the time instances inside the PW.

    - The aim of testing for inter-cell scenario should be to validate that the measurements reported by the UE:

      - meet the requirements for measured cell, and

      - meet the requirements for the target cell.

- Candidate Options:

  - Option 1: No need to update in the agreed definition of testing goal in RAN4#113

  - Option 2: Update the testing goal agreed in RAN4#113 to further clarify the testing principle/objectives and metrics/KPIs.

    - Option 2-1: Discuss the principle of defining the minimum performance considering the impact of PW, OW, MRRT, and/or any side condition.

    - Option 2-1: Discuss the goal of measurement reduction and mobility optimization

- Recommended WF

  - To discuss the options listed above. First it is recommended to discuss whether any update in the definition is required, and then the detailed options can be discussed.

#### Issue 4-2-2: Principle of FR2 testing

- Proposals and Observations:

  - Proposal  1 (CMCC):

    - Use the outcome of AI/ML BM use case as baseline for FR2.

  - Proposal 2 (Xiaomi):

    - For FR2 measurement prediction, conclusions on testability that RAN4 made for AI/ML BM can be taken as a baseline.

  - Proposal 3 (OPPO):

    - Reuse RAN4 AI BM conclusion as baseline, e.g., whether the chamber in OTA testing can be utilized and how to utilize/revise it.

  - Proposal 4 (Nokia):

    - For FR2, the testing setup and channel model in the AI/ML BM use case which is under study should be reused as a baseline for measurement prediction use case testing.

  - Proposal 5 (Vivo):

    - consider OTA testing for FR2 (scenario 4) and using one probe to emulate different Tx beams by applying different power for AI mobility FR2 beam level test for one cell.

    - Consider the potential testability setups in FR2:

| Scenario number | Sub-scenario | Freq Range | Test method | Channel model (as start point) | Min cell number | Min. probe number | SNR | UE trajectory |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | 1 | FR2 | OTA | AWGN/ TDL | 1 serving cell | 1 | Post SNR of the strongest beam pair | Change doppler, SNR, UE rotation, and change probe power |
|  | 2 |  |  |  | 2 (1 serving cell, 1 neighbor cell) | 2 |  |  |

  - Proposal 6 (CATT):

    - Postpone discussion on test set up in FR2 until sufficient progress is made in AI/ML BM

  - Proposal 7 (Samsung):

    - Study at least the following aspects:

      - Emulation of the propagation conditions

      - AWGN and TDL

      - Emulation of channel models

      - The number of emulated transmission reception points (TRxP)

      - Emulation of noise and EPRE

      - AoA setup

      - Rx beam direction assumption

    - Emulate multiple beam environment

    - Reuse AI/ML-BM agreements as a baseline.

      - Discuss what test parameters/methodologies on testability and interoperability for which AI/ML-PHY scenarios/cases could be reused to L3 cell level measurement prediction testing to avoid the duplicate study

    - Study the feasibility of testing in beam peak direction on the exact AoA to be used if the test is to mostly verify the RRM RSRP prediction performance

  - Proposal 8 (Huawei):

    - Postpone the testability discussion for FR2-to-FR2 prediction to wait the progress of the testability study in AI-BM

- Recommended WF

  - For AI/ML Mobility testing in FR2, use testing setup under consideration for AI/ML BM use case as a baseline.

#### Issue 4-2-3: Testing of Generalization aspects

- Proposals and Observations:

  - Proposal 1 (ZTE):

    - Start to discuss generalization in RAN4 if sufficient progress is made in RAN2 or NR_AIML_Air WI

  - Proposal 2 (Samsung):

    - Ensure generalization across cells for intra-frequency and/or across frequencies

    - Study generalization related tests/requirements

    - Reuse the principle/methodologies/signallings on generalization achieved in AI/ML for NR air interface as much as possible

  - Proposal 3 (Apple):

    - For Rel-19 AI/ML mobility, it is essential for RAN4 to investigate generalization-related tests and requirements arising from varying propagation conditions, SNR levels, and cell-specific Tx/codebook implementations across different cells and frequency layers.

    - Leverage the discussions in AI/ML beam management regarding the signaling of associated IDs to ensure consistency between primary and secondary carrier components, thereby enhancing generalization capabilities.

  - Proposal 4 (OPPO):

    - discuss the configurations for generalization in WI stage and the methodology discussed in NR_AIML_Air WI can be used as starting point.

  - Proposal 5 (Nokia):

    - Wait for further progress on generalization discussions in RAN2 before starting any discussion on generalization related topics in RAN4

- Candidate options:

  - Option 1: Discuss whether to start to discuss generalization in RAN4 or wait for more progress from RAN2

  - Option 2: Use the generalization in NR_AIML_Air WI as a starting point when it is possible to reuse

- Recommended WF

  - Discuss the options listed above.

#### Issue 4-2-4: Testing aspects of inter-carrier scenarios

- Proposals and Observations:

  - Proposal 1 (CATT):

    - Colocation can be reflected by a minor RSRP difference between two carriers.

  - Proposal 2 (OPPO):

    - For inter-carrier scenario (inter-frequency inter-cell prediction), the following aspects need to be discussed:

      - how to model co-located or co-site scenario for the cell to be measured and the cell to be predicted.

      - how to construct and obtain different cell’s RSRP corresponding to a specific temporal relationship.

      - channel modelling used for inter-cell prediction

  - Proposal 3 (Ericsson):

    - The aim of testing for inter-cell scenario should be to validate that the measurements reported by the UE:

      - meet the requirements for measured cell, and

      - meet the requirements for the target cell.

- Recommended WF

  - Above listed proposals need to be further discussed.

#### Issue 4-2-5: LCM related testability aspects

- Proposals and Observations:

  - Proposal  1 (Apple):

    - Leverage the LCM progress and requirements discussed in R-19 WI related to AI/ML Air Interface and investigate enhancements to these LCM procedures to RRM for mobility.

  - Proposal 2 (Qualcomm):

    - Define test framework considering how to verify UE LCM performance, if needed.

- Recommended WF

  - Discuss the options listed above.