# 3GPP TSG-RAN WG4 Meeting #114bis                            R4-2503636

# Wuhan, China, 7th-11th Apr, 2025

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on BS RF requirements for LP-WUS/WUR

# Agenda Item:   7.26.3

# Document for:  Approval

- Introduction

In RAN4#114 meeting, RAN4 had made great progress in the discussion on definition of LP-WUS power boosting and finally achieved agreement to use traditional dynamic based definition. But there are still some issues left, for example, transmit signal quality.

- Discussion

## 2.1 Power Boosting of LP-WUS

In last meeting, RAN4 achieved an agreement that the definition of LP-WUS power boosting from NB-IoT can be used[1]. Whether LP-WUS is supported and how much power boosting level is supported are declared by manufacturers separately, and no minimum power boosting level requirement is defined[1].

Concept of LP-WUS power boosting and associated detail WF: - Adopt following content as the concept of LP-WUS power boosting. - The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs). - Clarify the declaration on the support of LP-WUS and support of LP-WUS power boosting should be separate. - The minimum power boost level is included in the BS manufacturer declaration table, but not defined as a minimum requirement. - FFS on the detail of the minimum power boosting level, e.g. - Whether it can be different for different BS CBW

We think it is a consensus that different channel bandwidth can support different levels of LP-WUS power boosting because of the power share between NR signal and LP-WUS. The simulation results in our previous contribution [2] verifies this view. Therefore, we prefer LP-WUS power boosting level can be different for different BS channel bandwidth. As for whether manufacturers would like to declare same boosting level or different boosting levels for different channel bandwidth, this depends on manufacturers themselves.

Proposal 1: LP-WUS power boosting level can be different for different BS channel bandwidth, and it depends on manufacturer declaration.

## 2.2 Transmit Signal Quality

Whether transmit signal quality should be defined and whether equivalent EVM requirements should be defined are still under discussion [1]. RAN4 Based on the latest agreements in RAN1#120, we update our simulation assumptions at BS side as shown in Table 1.

Consideration on EVM requirement for LP-WUS WF: - For RAN4 simulation campaign and discussion in next meeting, the latest RAN1 agreements on waveform design are expected to be used. - FFS on the following options, others are not precluded. - Option 1: No need to specify transmit signal quality requirement for LP-WUS. - Option 2: Instead of defining transmit signal quality requirement for LP-WUS, just do the verification e.g. utilizing a higher-order EVM test (64QAM) for NR with embedded LP-WUS.

Table 1	Link level simulation assumptions for LP-WUS BS transmission

| Attributes | Assumptions |
| --- | --- |
| Case name (waveform) | OOK-4 waveform |
| Center frequency | 900MHz, 2.6GHz |
| Channel structure | 5 information bits with RM coding in TS38.212 Clause 5.3.3.3 No CRC is configured |
| Chip rate | M=1/2/4 |
| Coding | 1/2 rate Manchester coding |
| Repetition | None |
| Time error | 0 |
| Residual Frequency error | 0/10/20 ppm |
| SCS | 30kHz |
| UE Channel BW | 20MHz (51 RB) |
| WUS RB | - Fixed 11RB |
| Position within channel | - Center |
| Guard RB | - 0RB between LP-WUS and NR signal |
| Non-linearities | Not modeled |

Traditional Tx EVM is tested based on frequency-domain constellations. For LP-WUS, the symbols before RB mapping can be regarded as “constellations”. We added EVM on the LP-WUS frequency-domain symbols before RE mapping, as shown in Fig. 1, and then check the impact of EVM to Rx MDR. The simulation result is shown in Fig. 2. It can be found that frequency-domain EVM on LP-WUS “constellations” has minor impact on the receiving MDR.

Fig. 1  Add EVM to LP-WUS frequency-domain symbols

Fig. 2  Impact of EVM on OOK-4 M=4 receiving MDR for OOK-based receiver and OFDM-based receiver

Observation 1: Frequency-domain EVM on LP-WUS “constellations” has minor impact on the receiving MDR.

Therefore, we prefer that there is no need to define EVM requirement for LP-WUS.

Proposal 2: Propose not to define EVM requirements for LP-WUS.

On the other hand, RAN4 cares about the LP-WUS impact on NR signals within the same carrier. For the mixed signal, PAPR is one of the most important metrics to judge the impact from LP-WUS. Therefore, we compared the PAPR of pure NR CP-OFDM signal and mixed NR CP-OFDM and LP-WUS signal. The results under NFFT=2048 are shown in Table 2.

Table 2  PAPR (linearity) comparison between pure NR CP-OFDM signal and NR+LP-WUS

| NR Modulation | Signal component | Channel Bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5MHz | 10MHz | 15MHz | 20MHz | 50MHz |
| QPSK | Pure NR | 9.2597 | 10.0908 | 10.5106 | 10.7324 | 10.9768 |
|  | NR+LP-WUS | - | 9.3342 | 10.2756 | 10.6221 | 10.9734 |
| 16QAM | Pure NR | 9.3018 | 10.1059 | 10.5244 | 10.7192 | 10.9833 |
|  | NR+LP-WUS | - | 9.3472 | 10.2850 | 10.6292 | 10.9729 |
| 64QAM | Pure NR | 9.3064 | 10.1208 | 10.5318 | 10.7613 | 10.9773 |
|  | NR+LP-WUS | - | 9.3591 | 10.2821 | 10.6299 | 10.9902 |
| 256QAM | Pure NR | 9.3137 | 10.1187 | 10.5364 | 10.7579 | 10.9709 |
|  | NR+LP-WUS | - | 9.3529 | 10.2848 | 10.6265 | 10.9885 |

It can be found that LP-WUS can decrease PAPR slightly, especially for small CBW. Therefore, we don’t think LP-WUS have much impact on the signal quality of NR signal.

Observation 2: LP-WUS can decrease PAPR of mixed NR signal and LP-WUS slightly compared to pure NR signal.

- Conclusion

Proposal 1: LP-WUS power boosting level can be different for different BS channel bandwidth, and it depends on manufacturer declaration.

Observation 1: Frequency-domain EVM on LP-WUS “constellations” has minor impact on the receiving MDR.

Proposal 2: Propose not to define EVM requirements for LP-WUS.

Observation 2: LP-WUS can decrease PAPR of mixed NR signal and LP-WUS slightly compared to pure NR signal.

- Reference

- R4-2502305 Way Forward for [113][308] NR_LPWUS, RAN4#114, Athens, Greece, Feb. 2025.

- R4-2418827 Discussion on BS RF requirements for LP-WUS/WUR, RAN4#113, Orlando, USA, Nov. 2024.