# 3GPP TSG-RAN WG4 Meeting #114bis                             R4-2503638

# Wuhan, Greece, 7th-11th April, 2025

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on REFSENS, ASCS, ACS of LP-WUR

# Agenda Item:   7.26.2.2

# Document for:  Approval

- Introduction

In this contribution, we provided our views and simulation results on REFSENS, ASCS and ACS for LP-WUR based on the latest output on signal design from RAN1. RAN1’s agreements on LP-WUS design in last meeting that will impact RAN4 work are listed as follows.

RAN1 Agreement For LP-WUS information carried by OOK, select alt 1 for codepoint to meet FAR and MDR performance. - Alt 1: Coding with rate matching if any - For more than 2 information bits: RM coding as in section 5.3.3.3 of 38.212 (FFS: whether scrambling in the same section is applied or not) - 2 information bits (if supported): coding as in section 5.3.3.2 of 38.212 (FFS: whether scrambling in the same section is applied or not) - 1 information bit (if supported): repetition as in section 5.3.3.1 of 38.212 (FFS: whether scrambling in the same section is applied or not) - FFS: Code block length - FFS: Further repetition before or after Manchester coding RAN1 Agreement At least for M>1, for the overlaid OFDM sequence length, support - Alt2: - Note: X is the number of RBs of LP-WUS/LP-SS bandwidth (blanked guard RBs are not included) RAN1 Agreement For M=2, 4, is given by the largest prime number such that,is the overlaid OFDM sequence length. - The base overlaid sequenceis generated by extension of , - With CS(s) applied to the base overlaid OFDM sequence if any:denotes the potential cyclic shift (s) , - Note it doesn’t preclude any pulse shaping scheme if any.

- Discussion

## 2.1 Simulation Assumptions

Based on RAN1’s latest conclusions, we update our simulation assumptions as listed in Table 1, especially the channel structure part.

Table 1	Link level simulation assumptions for FR1 LP-WUS

| Attributes | Assumptions |
| --- | --- |
| Case name (waveform) | OOK-4 waveform |
| Center frequency | 900MHz, 2.6GHz |
| Channel structure | 5 information bits with RM coding in TS38.212 Clause 5.3.3.3 No CRC is configured |
| Chip rate | M=1/2/4 |
| Coding | 1/2 rate Manchester coding |
| Repetition | None |
| Time error | 0 |
| Residual Frequency error | 0/10/20 ppm |
| SCS | 30kHz |
| UE Channel BW | 20MHz (51 RB)-case 1 10MHz (24 RB)-case 2 5MHz (11 RB)-case 3 |
| WUS RB | - Fixed 11RB ~ 3.96MHz for 10MHz and 20MHz cases |
| Position within channel | - For 10/20MHz CBW, Center for ASCS, edge for ACS [assume no ASCS impact] - For 5MHz CBW, fixed center of channel |
| Guardband of NR channel, both wanted cell and interfer cell (ACS) | - For wanted signal: 505kHz for 5MHz, 665kHz for 10MHz, 805kHz for 20MHz - For interference cell2 5MHz: fixed 505kHz |
| Guard RB | - For ASCS: 0 or 1RB on each side of LP-WUS bandwidth - For ACS: 1/2/3/4 RB |
| Filter | - 3th/5th Order lowpass Butterworth matching fixed 3.96MHz RF bandwidth for 10MHz/20MHz case - Other order lowpass filter is not precluded - The filter bandwidth is adapted with actual WUS RBs, for 5MHz case |
| ASCS | PDSCH mapped on RBs not used for LP-WUS and Guard RB; EPRE of PDSCH /EPRE of LP-WUS = 0 dB Same PSD with WUS signal |
| ACS | PDSCH mapped on interference RBs (11RB for 5MHz CBW), one side; EPRE of PDSCH /EPRE of in-band LP-WUS = 15~33dB NOTE: decide the interference level depending on SNR |
| Wanted signal level | For ACS, REFSENS + 14 dB for LP-WUS |
| Sampling rate | 7.68MHz |
| ADC bit width | 4 bits ADC for ASCS/ACS |
| Non-linearities | Not modeled |
| Power boosting | None |
| Channel Model | Option 1: TDL-C 300 Option 2: AWGN Note: encourage companies to provide simulation results with both options |
| Performance metric | - 1% MDR |
| Note: the detailed Channel structure for LP-WUS is under discussion in RAN1, some parameters may be updated accordingly. |  |

## 2.2 REFSENS

In last meeting, discussed NF and IM package and achieve an tentative agreement in [1].

Agreement: - For the FR1 requirements targeting at bands <2.5GHz - The IM+NF values are - Set 1: 18dB - Set 2: 13.5dB - Send an LS to RAN1 with RAN4 decision of two sets of : NF (assume IM is 2.5dB) - Ask RAN1 to check whether coverage target can be met based on above NF information. And ask for detailed signal configuration (e.g., chip rate, length of sequence) of LP-WUS. - LS should also cc RAN2

We think most companies proposed their preferred NF and IM values based on OOK-based receiver and OFDM-based receiver. So we prefer to reuse this receiver type to define RREFSENS requirement.

Observation 1: In last meeting to derive the IM+NF values, most companies proposed their preferred NF and IM value based on OOK-based and OFDM-based receiver.

With decided NF and IM package, the SNR for REFSENS should still be evaluated. It should be noticed that RM coding to LP-WUS information bits provides an potential to decode LP-WUS based on Maximum Likelihood (ML) receiving algorithm. Therefore, we provide SNR results both with and without ML in Table 2.

Table 2	SNR for REFSENS of LP-WUR to achieve 1% MDR

| Receiver Type | Waveform | SNR (without ML) | SNR (with ML) |
| --- | --- | --- | --- |
| OOK-based receiver | OOK-4, M=4 | -1.5dB | -7.5dB |
| OFDM-based receiver | OOK-4, M=4 | -4.5dB | -10.0dB |

It can be found that although ML will cost more consumption to LP-WUR, it can decrease required SNR a lot. Besides, OFDM-based receiver outperforms OOK-based receiver.

Observation 2: ML can help to save 5~6dB SNR for REFSENS of LP-WUR.

Observation 3: OFDM-based receiver outperforms OOK-based receiver in terms of REFSENS SNR.

Proposal 1: The REFSENS SNR could be -1.5dB and -4.5dB for OOK-based receiver and OFDM-based receiver, respectively.

The REFSENS of MR is based on: SNR=-1dB, NF+IM=11.5dB, 2Rx Diversity Gain = 3dB.

Based on the conclusion in last meeting, the two sets of LP-WUR REFSENS are based on:

- Set 1 (OOK-based receiver): SNR=[-1.5dB], NF+IM=18dB, 1Rx Diversity Gain = 0dB;

- Set 2 (OFDM-based receiver): SNR=[-4.5dB], NF+IM=13.5dB, 1Rx Diversity Gain = 0dB.

Therefore, the REFSENS difference compared to MR could be approximately 9dB and 2dB for OOK-based receiver and OFDM-based receiver, respectively.

Proposal 2: REFSENS of OOK-based receiver could be 9dB higher than MR, REFSENS of OFDM-based receiver could be 2dB higher than MR.

## 2.2 ASCS

In previous meetings, the simulation results provided by companies showed that guard RB has minor improvement on the required SNR to achieve 1% BLER/MDR. Besides, we think that it is not efficient to waste two RBs as guard RB.

Proposal 3: Adopt 0 guard RB for ASCS requirement and ASCS test.

Considering 0~1dB de-sense due to introduce of NR RBs, relaxation on LP-WUS power is fine. To perform ASCS test, no guard RB is needed and NR PDSCH should occupy all the RBs that are not used for LP-WUS with the same PSD as LP-WUS.

Proposal 4: Define the test parameters for ASCS of LP-WUR in Table 3. For these test parameters, the missed detection rate shall be ≤1%.

Table 3  Test parameters for ASCS of LP-WUR

| RX parameter | LP-WUS transmission bandwidth configuration (RB) |
| --- | --- |
|  | 11RB |
| LP-WUS Power (dBm) | LP-WUR REFSENS + [1dB] |
| BWinterferer | NR PDSCH is mapped on RBs not used for LP-WUS |
| In-band Pinterferer | Same PSD as LP-WUS |

## 2.3 ACS

The agreements on ACS for LP-WUR are listed as follows [1].

Issue 2-4-1: Baseline ACS requirements (same interference) and required guard RB Agreements - Same interference signal level as MR for LP-WUR ACS requirements - FFS at least for one set of Rx requirements. - The LP-WUS level is LP-WUS REFSENS+[X]dB, X=14 as starting point - FFS whether two sets of LP-WUS wanted signal level can be defined. - Final ACS will be decided by LP-WUR REFSENS and wanted signal relaxation - FFS number of guard RBs.

It has been agreed that same interference signal level as MR for LP-WUR ACS requirements should be used. Namely, the ACS difference between LP-WUR and MR depends on the REFSENS difference between LP-WUR and MR. Therefore, the ACS requirement for OOK-based receiver could be 33dB-9dB=24dB, and the ACS requirement for OFDM-based receiver could be 33dB-2dB=31dB.

Observation 4: OFDM-based receiver outperforms OOK-based receiver a lot in terms of ACS.

Proposal 5: The ACS requirements could be 24dB for OOK-based receiver and 31dB for OFDM-based receiver, respectively.

Moreover, with more guard RB numbers, LP-WUR can address higher interference level. To achieve better receiving performance, guard RBs can be allocated. But this depends on the network configuration. Besides, it should be noticed that for 5MHz CBW with 30kHz SCS, all the RBs are occupied by LP-WUS and no additional RBs are left for guard RBs. In this case, we suggest not to limit the number of guard RBs as a core requirement and it can be left flexible.

Proposal 6: Propose not to define the number of guard RBs for ACS as a core requirement.

To further check the number of guard RBs in ACS test, we evaluate the MDR of OOK-based receiver and OFDM-based receiver versus different EPRE ratio between interference level and LP-WUS, under the condition of 0/1/2/3/4 guard RB numbers. In legacy spec, to perform ACS test, the wanted signal power is set under a good condition with MR REFSENS + 14dB, and 1.5dB margin is used in ACS test. Thus, we also provide the simulation results under LR REFSENS + 14dB condition for LP-WUR as shown in Fig. 1.

Fig. 1  LP-WUR performance in ACS case under different interference level and guard RB numbers for

OOK-based receiver and OFDM-based receiver

According to the simulation results, 1 guard RB is needed for OOK-based receiver and 2 guard RBs are needed for OFDM-based receiver. Therefore, we propose the test parameters in Table 4-7 as follows. Therefore, we prefer to define the test parameters for ACS of LP-WUR in Table 4-7. For these test parameters, the missed detection rate shall be ≤1%.

Proposal 7: For ACS test, to achieve 22.5dB ACS, 1 guard RB is needed for OOK-based receiver, and to achieve 29.5dB ACS, 2 guard RBs are needed for OFDM-based receiver. Consider the ACS test parameters in Table 4-7.

Table 4  Test parameters for ACS of OOK-based receiver, Case 1

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB |
| LP-WUS Power | dBm | LP-WUR REFSENS + 14 dB |
| Pinterferer | dBm | LP-WUR REFSENS + [36.5] dB |
| BWinterferer | MHz | 5 |
| Finterferer (offset to LP-WUS)2 | MHz | minimum guard band3 + [1]*12*SCS + 2.5 / -(minimum guard band3 + [1]*12*SCS + 2.5) |
| NOTE 1: LP-WUS is set at edge of the NR carrier without adjacent sub-carrier interference. NOTE 2: Finterferer (offset to LP-WUS) is the frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the LP-WUS). The interferer is an NR signal with an SCS equal to that of the LP-WUS. NOTE 3: Minimum guard band depends on the NR channel bandwidth that the LP-WUS is located in. |  |  |

Table 5  Test parameters for ACS of OOK-based receiver, Case 2

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB |
| LP-WUS Power | dBm | -56.5 |
| Pinterferer | dBm | [-37] |
| BWinterferer | MHz | 5 |
| Finterferer (offset to LP-WUS)2 | MHz | minimum guard band3 + [1]*12*SCS + 2.5 / -(minimum guard band3 + [1]*12*SCS + 2.5) |
| NOTE 1: LP-WUS is set at edge of the NR carrier without adjacent sub-carrier interference. NOTE 2: Finterferer (offset to LP-WUS) is the frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the LP-WUS). The interferer is an NR signal with an SCS equal to that of the LP-WUS. NOTE 3: Minimum guard band depends on the NR channel bandwidth that the LP-WUS is located in. |  |  |

Table 6  Test parameters for ACS of OFDM-based receiver, Case 1

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB |
| LP-WUS Power | dBm | LR REFSENS + 14 dB |
| Pinterferer | dBm | LR REFSENS + [43.5] dB |
| BWinterferer | MHz | 5 |
| Finterferer (offset to LP-WUS)2 | MHz | minimum guard band3 + [2]*12*SCS + 2.5 / -(minimum guard band3 + [2]*12*SCS + 2.5) |
| NOTE 1: LP-WUS is set at edge of the NR carrier without adjacent sub-carrier interference. NOTE 2: Finterferer (offset to LP-WUS) is the frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the LP-WUS). The interferer is an NR signal with an SCS equal to that of the LP-WUS. NOTE 3: Minimum guard band depends on the NR channel bandwidth that the LP-WUS is located in. |  |  |

Table 7  Test parameters for ACS of OFDM-based receiver, Case 2

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB |
| LP-WUS Power | dBm | -56.5 |
| Pinterferer | dBm | [-27] |
| BWinterferer | MHz | 5 |
| Finterferer (offset to LP-WUS)2 | MHz | minimum guard band3 + [2]*12*SCS + 2.5 / -(minimum guard band3 + [2]*12*SCS + 2.5) |
| NOTE 1: LP-WUS is set at edge of the NR carrier without adjacent sub-carrier interference. NOTE 2: Finterferer (offset to LP-WUS) is the frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the LP-WUS). The interferer is an NR signal with an SCS equal to that of the LP-WUS. NOTE 3: Minimum guard band depends on the NR channel bandwidth that the LP-WUS is located in. |  |  |

- Conclusion

Observations:

Observation 1: In last meeting to derive the IM+NF values, most companies proposed their preferred NF and IM value based on OOK-based and OFDM-based receiver.

Observation 2: ML can help to save 5~6dB SNR for REFSENS of LP-WUR.

Observation 3: OFDM-based receiver outperforms OOK-based receiver in terms of REFSENS SNR.

Observation 4: OFDM-based receiver outperforms OOK-based receiver a lot in terms of ACS.

Proposals:

Proposal 1: The REFSENS SNR could be -1.5dB and -4.5dB for OOK-based receiver and OFDM-based receiver, respectively.

Proposal 2: REFSENS for OOK-based receiver could be 9dB higher than MR, REFSENS for OFDM-based receiver could be 2dB higher than MR.

Proposal 3: Adopt 0 guard RB for ASCS requirement and ASCS test.

Proposal 4: Define the test parameters for ASCS of LP-WUR in Table 3. For these test parameters, the missed detection rate shall be ≤1%.

Table 3  Test parameters for ASCS of LP-WUR

| RX parameter | LP-WUS transmission bandwidth configuration (RB) |
| --- | --- |
|  | 11RB |
| LP-WUS Power (dBm) | LP-WUR REFSENS + [1dB] |
| BWinterferer | NR PDSCH is mapped on RBs not used for LP-WUS |
| In-band Pinterferer | Same PSD as LP-WUS |

Proposal 5: The ACS requirements could be 24dB for OOK-based receiver and 31dB for OFDM-based receiver, respectively.

Proposal 6: Propose not to define the number of guard RBs for ACS as a core requirement.

Proposal 7: For ACS test, to achieve 22.5dB ACS, 1 guard RB is needed for OOK-based receiver, and to achieve 29.5dB ACS, 2 guard RBs are needed for OFDM-based receiver. Consider the ACS test parameters in Table 4-7.

- Reference

- R4-2503028 WF on UE RF requirements for LP-WUS, vivo, RAN4#114, Athens, Greece, Feb. 2025.