3GPP TSG-RAN WG4 Meeting #114bis	R4-2503655

Wuhan, Hubei, China, 07th – 11nd April, 2025

Source:	Samsung

Title:	Views on MIMO layer evaluation for 6Rx UE

Agenda item:	7.1.1.3.2

Document for:	Discussion

- Introduction

In the RAN#103 meeting, the WID [1] for “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” has been approved, in which “6Rx for handheld and FWA” is an important objective for both core and performance parts. This is an enhancement for receiver in addition to Rel-15 4Rx and Rel-18 8Rx improvements (which are specified for fixed wireless access (FWA) types of devices). The 6Rx in Rel-19 is targeting for handheld UE considering 6Rx would be possible and useful to improve DL coverage and throughputs for the large size handheld UEs, e.g., the foldable smartphone. In the RAN#107 meeting, the WID has been updated [2] as below.

Objective of Core part WI 6Rx for handheld and FWA UE [RAN4, RAN1] - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Introduce 6 MIMO layers support as an optional feature only for FWA. - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024) - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution. General for all the areas - Specify release independence requirements for the core part in TS 38.307/36.307, if needed and feasible. - Objective of Performance part WI The objectives of performance part for UE RF enhancements for NR FR1 and FR2 Phase 4 include: 6Rx for handheld and FWA UE - Specify the performance requirements to enable 6Rx on higher frequency bands (>2.5GHz) targeting at support of smartphone for NR FR1 single carrier scenario - Investigate and if necessary, specify RLM test cases to support 6Rx - Specify UE PDSCH demodulation performance and CSI requirements to support 6 Rx - 6 MIMO layers performance requirements will be only defined for FWA. - 4 MIMO layers performance requirements will be defined and will apply to FWA and handheld UE devices. - Specify the SDR requirements to support 6Rx General for all the areas - Specify release independence requirements for performance part in TS 38.307/36.307, if needed and feasible.

In this contribution, we provide more analysis and views according the WF [3] in last RAN4#114 meeting and latest WID [2].

- Discussion

In last RAN4#114 meeting, it is agreed that 6-layer only support for FWA shall be explicitly captured at least in the RAN4 specification as below [3].

Issue 2-1-1: Way forward for 6-layers support for handheld UE and FWA devices Way Forward: - RAN4 Chair to take note of the following in the meeting report: - “RAN4 unable to conclude on a common set of simulation assumptions to determine feasibility of 6 layer for handheld UE In Rel-19” - 6-layer performance requirements will be only defined for FWA. - 4-layer performance requirements will be defined and will apply to FWA and handheld UE devices. - The fact that 6-layer is only supported for FWA shall be explicitly captured at least in the RAN4 specifications.

Currently, the maximum number of spatial multiplexing layer(s) supported by the UE for DL reception only have three options in 38.331 and 38.306, i.e., 2 layers, 4 layers and 8 layers described as below, in which 6-layer is not supported.

TS 38.331 Clause 6.3.3

- MIMO-Layers The IE MIMO-Layers is used to convey the number of supported MIMO layers. MIMO-Layers information element -- ASN1START -- TAG-MIMO-LAYERS-START MIMO-LayersDL ::= ENUMERATED {twoLayers, fourLayers, eightLayers} MIMO-LayersUL ::= ENUMERATED {oneLayer, twoLayers, fourLayers} -- TAG-MIMO-LAYERS-STOP -- ASN1STOP

TS 38.306 Clause 4.2.7

Considering there is no TU reserved for RAN2 on this topic, RAN2 probably wouldn't do the update for 6-layer unless we inform them. In order to support our agreement for introducing 6-layer, it is necessary to inform RAN2 to update IE descriptions of MIMO-LayersDL and maxNumberMIMO-LayersPDSCH. At the same time, the information that 6-layer is only supported for FWA should be informed to RAN2 correctly.

Observation 1: Currently, the maximum number of spatial multiplexing layer(s) supported by the UE for DL reception only have three options in 38.331 and 38.306, i.e., 2 layers, 4 layers and 8 layers, in which 6-layer is not supported.

Observation 2: There is no TU reserved for RAN2 on this topic, RAN2 probably wouldn't do the update for 6-layer unless we inform them.

Proposal 1: In order to support our agreement for introducing 6-layer, it is necessary to inform RAN2 to update IE descriptions of MIMO-LayersDL and maxNumberMIMO-LayersPDSCH. At the same time, the information that 6-layer is only supported for FWA should be informed to RAN2 correctly.

- Conclusion

In this contribution, we provide our analysis and views on “6Rx for handheld and FWA” topic.

The observations and proposals could be summarized as:

Observation 1: Currently, the maximum number of spatial multiplexing layer(s) supported by the UE for DL reception only have three options in 38.331 and 38.306, i.e., 2 layers, 4 layers and 8 layers, in which 6-layer is not supported.

Observation 2: There is no TU reserved for RAN2 on this topic, RAN2 probably wouldn't do the update for 6-layer unless we inform them.

Proposal 1: In order to support our agreement for introducing 6-layer, it is necessary to inform RAN2 to update IE descriptions of MIMO-LayersDL and maxNumberMIMO-LayersPDSCH. At the same time, the information that 6-layer is only supported for FWA should be informed to RAN2 correctly.

- Reference

[1] RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#103 meeting, Huawei

[2] RP-250780, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#107 meeting, Huawei

[3] R4-2503014, WF on 6Rx UE requirements, RAN4#114 meeting, AT&T