3GPP TSG-RAN WG4 Meeting #114bis	R4-2503658

Wuhan, Hubei, China, 07th – 11nd April, 2025

Agenda item:	7.16.2

Source:	Samsung

Title:	Discussion on spatial channel model for demodulation performance requirements

Document for:	Discussion

- Introduction

In the RAN#104 meeting, the SID [1] for “New SID: Study on spatial channel model for demodulation performance requirements” has been approved, which starts from RAN4#112 in Aug 2024 and targeting to complete by RAN#109 in Sep 2025 [2]. The objectives are as below:

- Study practical spatial channel modelling methodology for both SU- and MU-MIMO demodulation requirements and CSI reporting requirements: - Identify the limitations of the current (i.e. up to and including Release 18) channel models and corresponding scenarios and how they relate to UE MIMO performance - Consider both Clustered Delay Line (CDL)-based and TDL-based channel modelling approaches - For CDL-based channel modelling, use the tuned repeatable spatial channel model of TR38.827 as the starting point and identify any necessary changes. - Study and verify test methodology feasibility including test complexity and achievable results uncertainty. The test complexity shall not be significantly increased. - The methodology shall include both FR1 (conducted) and FR2 (wireless cable), with first priority for FR1.

In RAN4#113 meeting, High level TR skeleton for 38.753 and initial work split have been agreed, and obvious progresses in last RAN4#114 meeting have been achieved for test cases, parameter assumptions, CDL methodologies and AAV configurations [3]. However, until now the discussion mainly focuses on TR 38.827. There are still several issues need to be further clarified and discussed for the CDL modelling, and the discussion for TDL modelling also need to be started.

In this contribution, we share our views and analysis on the open issues for CDL-based model and TDL-based model.

- Discussion

  - TDL Based Methodologies

The channel model with spatial property has been discussed at the beginning of NR. Geometric based SCM(E) is defined in 38.901, in which the cluster delays are drawn randomly from the delay distribution, the cluster powers are calculated assuming a single slope exponential power delay profile, the AoA/AoD/ZoA/ZoD are determined by applying the inverse Gaussian/Laplacian function and random variable values with uniform distributions, the rays are randomly coupling within a cluster for both azimuth and elevation, the cross polarization power ratio (XPR) is log-Normal distributed, the initial phases are random uniform within (-). There are too many variables to align results based on this channel model. 38.901 CDL channel model is derived from SCM(E) with some parameters fixed, i.e., cluster delays, cluster powers, AoA/AoD/ZoA/ZoD and XPR are tabulated and fixed. Based on 38.901 CDL channel model, 38.901 TDL model is defined without spatial property (only cluster delays and powers with Rx and Tx radiation pattern effect are kept), and 38.101-4 TDL model is further simplified  by removing some clusters.

Currently, the motivation of the SID [1] is that 38.101-4 TDL model does not have spatial property for RAN4 demodulation/CSI reporting requirements, which are needed for some scenarios e.g. High rank, PMI reporting cases.  It is obvious that spatial property is not equal to CDL channel model, since AE radiation patterns, the per sub-cluster AoA/AoD/ZoA/ZoD angles, and even Beam forming are redundant for the spatial property introduction. This target for introducing spatial property could also be achieved by extending current TDL model through introducing spatial clusters as mentioned in [4], for each cluster TX and RX steering corresponding to desired AoD/ZoA and AoA/ZoA.

The general multi-cluster MIMO channel impulse response matrix with dimensions Nrx× Ntx at some time instant is [4]

Hτ=nmpn,m∙Dn(rx)Hn,mDn(tx)∙δτ-τn,m

In which, steering matrix definitions:

- Dn(tx)=1001⊗Dθtx1(n),Ntx1⊗Dθtx2(n),Ntx2

- Dn(rx)=1001⊗Dθrx1(n),Nrx1⊗Dθrx2(n),Nrx2

Steering models

There are two steering models proposed in [4],

- Steering model 1: the steering matrix of size N× N as function of phase θ is defined as

  - Dθ,N=diag1ejθ⋯ejθ(N-1)

  - Assuming the antenna array employs uniform half wavelength distance between elements in each linear dimension, the phase shift θ corresponds to AOA or AOD angle φ so that θ=π∙sinφ.

- Steering model 2: the steering matrix of size N× N as function of phase θ is defined as Dθ,N=diag1ej3N-1θ⋯ej3θ, i.e., for N=1, 2, 3, 4

  - Dθ,1=1

  - Dθ,2=diag1ej3θ

  - Dθ,3=diag1ej1.5θej3θ

  - Dθ,4=diag1ejθej2θej3θ

Steering model 1 is simple and intending to introduce the corresponding relationship between phase shift θ and the desired AOA or AOD angle φ, however the desired angles are not obvious since the configurable parameter is phase shift θ. Steering model 2 follow the design principle of 38.101-4 B.2.3.2.3, it’s straightforward but the meaning of angle θ is not clear.

Observation 1: Steering model 1 is simple and intending to introduce the corresponding relationship between phase shift θ and the desired AOA or AOD angle φ, however the desired angles are not obvious since the configurable parameter is phase shift θ. Steering model 2 follow the design principle of 38.101-4 B.2.3.2.3, it’s straightforward but the meaning of angle θ is not clear.

Due to above observation, we’d like to propose a new extended steering model based on the steering model 1 as below

  - Dθ,N=diag1ej2π∙d∙sinθej2π∙2d∙sinθ⋯ej2π∙(N-1)d∙sinθ

in which d is the distance between each antenna element, the unit is waveform. θ is the desired AoA/AoD/ZoA/ZoD angle.

For this new steering model 3, one benefit is the desired angles are configurable and obvious, another benefit is more flexible to adapt to different antenna distance.

Proposal 1: A new steering model 3 based on the steering model 1 as below

  - Dθ,N=diag1ej2π∙d∙sinθej2π∙2d∙sinθ⋯ej2π∙(N-1)d∙sinθ

in which d is the distance between each antenna element, the unit is waveform. θ is the desired angle.

Observation 2: For this new steering model 3, one benefit is the desired angles are configurable and obvious, another benefit is more flexible to adapt to different antenna distance.

Cluster models

Two cluster models introduced in [4]. For cluster model 1, each cluster is associated with a TDL channel model with full delay profile, so the complexity of the channe model grows with the number of clusters. For cluster model 2, the delay profile of a single TDL channel model is split into multiple clusters, and the complexity of the model remains almost equal to the underlying TDL model. Nevertheless, we assume the complexity difference between two cluster models is negligible compared with CDL channel model. Hence, the complexity is not a key issue for extended TDL models. Considering the cluster powers, the power difference between layers or codewords of cluster model 1 is clear, while the power difference between layers or codewords of cluster model 2 is ambiguous and need further calculation.

Observation 3: The complexity of cluster model 2 is similar as legacy TDL channel model, while the complexity of cluster model 1 grows with the number of clusters. Nevertheless, we assume the complexity difference between two cluster models is negligible compared with CDL channel model. Hence, the complexity is not a key issue for extended TDL models.

Observation 4: The power difference between layers or codewords of cluster model 1 is clear, while the power difference between layers or codewords of cluster model 2 is ambiguous and need further calculation.

Proposal 2: Use cluster model 1 as the starting point for extended TDL model performance evaluation.

Simulation assumptions

In order to compare and align simulation results for TDL based multi-cluster MIMO channel model, the number of clusters, the power of clusters and the desired angles need to be aligned.

Number of clusters

As the purpose of multi-cluster is to provide different beam directions for higher rank cases, two clusters for rank4 cases and four clusters for rank8 cases should be assumed.

Proposal 3: For the number of clusters, two clusters for rank4 cases and four clusters for rank8 cases should be assumed.

Power of clusters

In order to distinguish the SNR between different layers/codewords, the power of clusters should be defined. For two clusters, 0dB and -3dB could be selected as the starting point. For four clusters, 0dB, 0dB, -3dB, -3dB could be selected as the starting point.

Proposal 4: In order to distinguish the SNR between different layers/codewords, the power of clusters should be defined. For two clusters, 0dB and -3dB could be selected as the starting point. For four clusters, 0dB, 0dB, -3dB, -3dB could be selected as the starting point.

Desired angles

For the desired angles, for 4Tx cases, N1=2, N2=1, and for 8Tx cases, N1=4, N2=1, only the azimuth angles need to be considered. For two clusters, desired angles could be 0 and π/2. For four clusters, desired angles could be 0, π/6, π/2 and 3π/2.

Proposal 5: For two clusters, desired angles could be 0 and π/2. For four clusters, desired angles could be 0, π/6, π/2 and 3π/2.

  - CDL Based Methodologies

After RAN4#114 meeting, two issues for CDL based channel modeling approach need further discussion. The first one is the angles of departure and angles of arrival, the second one is the SNR normalization with CDL.

Angles of Departure and Angles of Arrival

Way Forward: For all 4 direction combinations (AoA, AoD, azimuth, zenith) the following parameters need to be configured. Angle spread scaling: ASdesired Angle rotation: μϕ,desired as a starting point. For CDL Option A1: Based on 38.827 For CDL Option A2, for each (AoA, AoD) and θ,ϕ, the parameters in the formula are the following: ASmodel is computed according to TR 38.901, Annex A.1, Equation A-1 μϕ,model is computed according to TR 38.901, Annex A.2, Equation A-2 For the purposes of simulation alignment: μϕ,desired = μϕ,model ASdesired ASdesired, ASD; ASdesired, ASA; ASdesired, ZSD; ASdesired, ZSA. CDL Option A1; 25.7620; 74.1138; 4.8978; 18.2050. CDL Option A2; 25.7620; 74.1138; 4.8978; 18.2050. CDL Option A3; 25.7620; 74.1138; 4.8978; 18.2050. As a starting point: cASA,ASD,ZSD,ZSA according to the table below Per-Cluster Parameters. Parameter; cASD in [°]; cASA in [°]; cZSD in [°]; cZSA in [°]; XPR in [dB]. Value; 2; 15; 3; 7; 7.

Although the Way Forward above are trying to seperate CDL option A1 and A2, further clarification are still needed.

ZoA angles for CDL Option A1

Firstly, for CDL Option A1, Table 7.2.1-8 as below in 38.827 is the results after applying angle scaling and angle rotation. The ZoA angles are const 90 degrees becasue OTA test is 2D without elevation modeling as described in section 7.2.1.

Description in section 7.2.1 of 3GPP 38.827

For FR1, the baseline emulated propagation environment for FR1 MIMO OTA is 2D without elevation modelling, i.e., all ZOA are set 90° and ZSA is 0° in the following tables.

However, this restriction for const 90 degrees ZoA is not needed for generic CDL modeling in RAN4.

Observation 5: For CDL Option A1, Table 7.2.1-8 in 38.827 is the results after applying angle scaling and angle rotation, in which the ZoA angles are const 90 degrees becasue OTA test is 2D without elevation modeling.

Proposal 6: For CDL Option A1, the restriction for const 90 degrees ZoA is not needed for generic CDL modeling in RAN4.

ASmodel for CDL Option A1 and CDL Option A2

Secondly, for the angle spreads ASmodel, difference descriptions in 38.827 and 38.901.

For CDL Option A1, the angle spreads ASmodel are described in 38.827 Table 7.2-2. It is supposed to be calculated by using Equation (2-1), (2-2), (2-3)

(2-1)

(2-2)

(2-3)

Below Table 2-1 is the ASmodel selected from Table 7.2-2 for Uma CDLC, and Table 2-2 is the ASmodel calculated according Equation (2-1)- (2-3). The angle spread values of AoA angles are different between Table 2-1 and Table 2-2.

Table 2-1 Angle spreads ASmodel selected from 3GPP 38.827 Table 7.2-2 for Uma CDLC

| Model | ASmodel [deg] |  |  |  |
| --- | --- | --- | --- | --- |
|  | ASD | ASA | ZSD | ZSA |
| CDL-C | 39.0949 | 71.1175 | 4.0666 | 10.4245 |

Table 2-2 Angle spreads ASmodel calculated according Equation (2-1)- (2-3)

| Model | ASmodel [deg] |  |  |  |
| --- | --- | --- | --- | --- |
|  | ASD | ASA | ZSD | ZSA |
| CDL-C | 39.0949 | 104.3569 | 4.0666 | 10.4245 |

While for CDL Option A2, the Equation for angle spreads ASmodel is Equation A-1 in 38.901 as below (2-4). The calculated angle spreads ASmodel are as Table 2-3.

(2-4)

Table 2-3 Angle spreads ASmodel calculated according 38.901 Equation (A-1)

| Model | ASmodel [deg] |  |  |  |
| --- | --- | --- | --- | --- |
|  | ASD | ASA | ZSD | ZSA |
| CDL-C | 37.4036 | 71.4535 | 4.0665 | 10.4182 |

Observation 6: For CDL Option A1, the calculated angle spread values of AoA angles according the calculation Equation defined in 38.827 are different with 38.827 Table 7.2-2, need further clarification.

Observation 7: Angle spreads ASmodel are different for CDL Option A1 and CDL Option A2 as they are using different calculation Equations.

Channel model parameters after angle scaling and angle rotation

For CDL Option A1, the channel model parameters after angle scaling and angle rotation are as Table 7.2.1-8 as below in 3GPP 38.827.

Table 7.2.1-8: Channel model parameters for UMa CDL-C at 3.5 GHz

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4215 | -37.4195 | -96.4031 | 96.7645 | 90 |
| 2 | 76.6135 | -1.25 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 3 | 80.9935 | -3.4684 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 4 | 85.0085 | -5.2294 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 5 | 79.424 | -2.5215 | -33.5316 | -124.0196 | 100.8594 | 90 |
| 6 | 232.359 | 0 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 7 | 235.352 | -2.2185 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 8 | 239.44 | -3.9794 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 9 | 240.316 | -7.4215 | 41.4581 | 51.4188 | 106.3995 | 90 |
| 10 | 289.6275 | -7.1215 | -49.2149 | 62.9864 | 94.4761 | 90 |
| 11 | 299.7745 | -10.7215 | 46.1367 | -41.2744 | 107.4834 | 90 |
| 12 | 340.764 | -11.1215 | -70.697 | 42.5606 | 92.3083 | 90 |
| 13 | 448.4025 | -5.1215 | -43.1524 | 64.6538 | 104.5929 | 90 |
| 14 | 477.5295 | -6.8215 | -49.0831 | -62.7423 | 105.1951 | 90 |
| 15 | 792.196 | -8.7215 | -58.4403 | 78.6184 | 91.7061 | 90 |
| 16 | 989.3325 | -13.2215 | 60.9633 | 25.6781 | 105.1951 | 90 |
| 17 | 1554.4985 | -13.9215 | 58.6569 | -23.4063 | 93.9944 | 90 |
| 18 | 1679.1095 | -13.9215 | 51.8037 | -2.3553 | 91.8265 | 90 |
| 19 | 2003.923 | -15.8215 | -73.86 | -20.5926 | 90.7426 | 90 |
| 20 | 2046.8105 | -17.1215 | 54.0442 | 3.7933 | 108.2061 | 90 |
| 21 | 2301.8725 | -16.0215 | 54.7691 | -0.4794 | 91.7061 | 90 |
| 22 | 2422.651 | -15.7215 | 63.5332 | -5.5859 | 91.5856 | 90 |
| 23 | 2570.5855 | -21.6215 | 72.0338 | -29.1381 | 106.3995 | 90 |
| 24 | 3158.0895 | -22.8215 | -88.2912 | 28.7003 | 109.5309 | 90 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 1.3179 | 15.632 | 3.6131 | 0 | 7 |  |

While for CDL Option A2, the channel model parameters after angle scaling and angle rotation are as Table 2-5 based on 38.827 angle spreads Table 2-1 and desired angle spreads Table 2-4, and the channel model parameters after angle scaling and angle rotation are as Table 2-6 based on 38.901 angle spreads Table 2-3 and desired angle spreads Table 2-4. The channel model parameter values are slightly different between based on 38.827 angle spreads and based on 38.901 angle spreads.

Table 2-4 Desired angle spreads ASdesiredASdesired selected from 3GPP 38.827 Table 7.2-4 for Uma CDLC

| Model | ASdesiredASdesired [deg] |  |  |  |
| --- | --- | --- | --- | --- |
|  | ASD | ASA | ZSD | ZSA |
| CDL-C | 25.7620 | 74.1138 | 4.8978 | 18.2050 |

Table 2-5 Channel model parameters after angle scaling and rotation based on 38.827 angle spreads Table 2-1

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.0 | -4.4 | -37.4 | -96.4 | 96.8 | 98.1 |
| 2 | 76.6 | -1.2 | -21.7 | 118.7 | 98.5 | 71.1 |
| 3 | 81.0 | -3.5 | -21.7 | 118.7 | 98.5 | 71.1 |
| 4 | 85.0 | -5.2 | -21.7 | 118.7 | 98.5 | 71.1 |
| 5 | 79.4 | -2.5 | -33.5 | -124.0 | 100.9 | 67.6 |
| 6 | 232.4 | 0.0 | -6.5 | 171.3 | 99.2 | 76.6 |
| 7 | 235.4 | -2.2 | -6.5 | 171.3 | 99.2 | 76.6 |
| 8 | 239.4 | -3.9 | -6.5 | 171.3 | 99.2 | 76.6 |
| 9 | 240.3 | -7.4 | 41.5 | 51.4 | 106.4 | 62.8 |
| 10 | 289.6 | -7.1 | -49.2 | 63.0 | 94.5 | 56.6 |
| 11 | 299.8 | -10.7 | 46.1 | -41.3 | 107.5 | 69.8 |
| 12 | 340.8 | -11.1 | -70.7 | 42.6 | 92.3 | 50.8 |
| 13 | 448.4 | -5.1 | -43.2 | 64.7 | 104.6 | 103.4 |
| 14 | 477.5 | -6.8 | -49.1 | -62.7 | 105.2 | 50.1 |
| 15 | 792.2 | -8.7 | -58.4 | 78.6 | 91.7 | 51.7 |
| 16 | 989.3 | -13.2 | 61.0 | 25.7 | 105.2 | 121.0 |
| 17 | 1554.5 | -13.9 | 58.7 | -23.4 | 94.0 | 53.9 |
| 18 | 1679.1 | -13.9 | 51.8 | -2.4 | 91.8 | 61.6 |
| 19 | 2003.9 | -15.8 | -73.9 | -20.6 | 90.7 | 37.5 |
| 20 | 2046.8 | -17.1 | 54.0 | 3.8 | 108.2 | 53.1 |
| 21 | 2301.9 | -16.0 | 54.8 | -0.5 | 91.7 | 35.8 |
| 22 | 2422.7 | -15.7 | 63.5 | -5.6 | 91.6 | 52.9 |
| 23 | 2570.6 | -21.6 | 72.0 | -29.1 | 106.4 | 46.4 |
| 24 | 3158.1 | -22.8 | -88.3 | 28.7 | 109.5 | 44.7 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 1.3179 | 15.632 | 3.6131 | 12.2246 | 7 |  |

Table 2-6 Channel model parameters after angle scaling and rotation based on 38.901 angle spreads Table 2-3

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.0 | -4.4 | -38.2 | -96.9 | 96.8 | 98.1 |
| 2 | 76.6 | -1.2 | -21.8 | 118.9 | 98.5 | 71.1 |
| 3 | 81.0 | -3.5 | -21.8 | 118.9 | 98.5 | 71.1 |
| 4 | 85.0 | -5.2 | -21.8 | 118.9 | 98.5 | 71.1 |
| 5 | 79.4 | -2.5 | -34.2 | -124.4 | 100.9 | 67.6 |
| 6 | 232.4 | 0.0 | -5.9 | 171.2 | 99.2 | 76.6 |
| 7 | 235.4 | -2.2 | -5.9 | 171.2 | 99.2 | 76.6 |
| 8 | 239.4 | -3.9 | -5.9 | 171.2 | 99.2 | 76.6 |
| 9 | 240.3 | -7.4 | 44.2 | 51.9 | 106.4 | 62.8 |
| 10 | 289.6 | -7.1 | -50.6 | 63.4 | 94.5 | 56.6 |
| 11 | 299.8 | -10.7 | 49.1 | -42.1 | 107.5 | 69.8 |
| 12 | 340.8 | -11.1 | -73.0 | 43.1 | 92.3 | 50.8 |
| 13 | 448.4 | -5.1 | -44.2 | 65.1 | 104.6 | 103.4 |
| 14 | 477.5 | -6.8 | -50.4 | -63.4 | 105.2 | 50.1 |
| 15 | 792.2 | -8.7 | -60.2 | 79.0 | 91.7 | 51.7 |
| 16 | 989.3 | -13.2 | 64.6 | 26.3 | 105.2 | 121.0 |
| 17 | 1554.5 | -13.9 | 62.2 | -22.6 | 94.0 | 53.9 |
| 18 | 1679.1 | -13.9 | 55.0 | -1.6 | 91.8 | 61.6 |
| 19 | 2003.9 | -15.8 | -76.3 | -19.8 | 90.7 | 37.5 |
| 20 | 2046.8 | -17.1 | 57.4 | 4.5 | 108.2 | 53.1 |
| 21 | 2301.9 | -16.0 | 58.1 | 0.2 | 91.7 | 35.8 |
| 22 | 2422.7 | -15.7 | 67.3 | -4.9 | 91.6 | 52.9 |
| 23 | 2570.6 | -21.6 | 76.2 | -28.3 | 106.4 | 46.4 |
| 24 | 3158.1 | -22.8 | -91.4 | 29.3 | 109.5 | 44.7 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 1.3775 | 15.5585 | 3.6133 | 12.232 | 7 |  |

Observation 8: The channel model parameter values after angle scaling and angle rotation are slightly different between based on 38.827 angle spreads and based on 38.901 angle spreads.

Proposal 7: For CDL Option A2, the table of channel models parameters after angle scaling and angle rotation should be compared and aligned between companies in order to provide simulation results based on the same assumptions. Our table of channel parameters for CDL Option A2 is as Table 2-6 above.

SNR Normalisation with CDL

With the introduction of the BS antenna radiation pattern and AAV beamforming in CDL based channel modelling, the normalization should be applied by a normalization factor and the normalization factor should be calculated considering the long-term average PDP and the beamforming effect for the spatial CDL channel, otherwise an additional gain will be added to the power of channel. The Way Forward for this issue in last RAN4#114 meeting is as below.

Way Forward: Options: - Option 1: Interested companies to provide information on long-term pre-EQ power measurements, for discussion whether a formula, or value, is required during RAN4#114-bis. - Option 2: Define a normalisation method - Option 2a: RAN4 to use following as normalization factor: C=NTxNRxnt=0NTx-1nr=0NRx-1E{Hnt,nr(k,l) 2} Where H(k,t)∈CNR×NT is channel coefficient at subcarrier k, OFDM symbol l. NRx is number of Rx and NTx is number of Tx ports. Hnt,nr(k,l) is the (nt, nr) element of H(k,l). E{Hnt,nr(k,l) 2} can be derived by long-term measurement in the simulation.. - Option 2b: Let normalized AAV beamformer vector of length Ms∙Ns (i.e. sub-array size) for one antenna port be α1,1⋯αMs,Ns=1GBF1Ms∙Nsexp-j2π∙θ1,1 ⋯ exp⁡(-j2π∙θMs,Ns) Here, amplitude normalization is separated into two factors - 1Ms∙Ns : split transmit power of each AP evenly over Ms∙Ns transmit antenna elements - 1GBF : normalize with measured transmit beamforming (power) gain GBF.

Basically, the factors affect the final SNR could be summarized as BS/UE AE radiation patterns, AE combining weights (beamforming weight) and angles (AoA/AoD/ZoA/ZoD). The angles have been analysed above. For the BS/UE AE radiation patterns, companies agreed to use 38.901 Table 7.3-1 as BS AE radiation pattern (as Figure 2-1) and omni pattern as UE radiation pattern. For FR1, there is no AE combing model introduced at UE side and a vertical 8-to-1 combing vector agreed at BS side although it is quite narrow shown as Figure 2-2.

Figure 2-1. Directional radiation power pattern defined in 38.901 Table 7.3-1

Figure 2-2. Beam Steering of the vertical 8-to-1 combing vector at BS side

The final BS directivity pattern considering both AE radiation pattern and combing weights could be described by Figure 2-3. Besides AE radiation pattern and combing weights, the value of down tilt angle also could impact the final BS directivity pattern. Therefore, compared with calculating the SNR normalization factor directly by channel response, it is somehow more difficult to divide the SNR normalization factor into AE PDP gain factor, BF gain factor and down tilt impact factor, etc. If the SNR normalization factor is not applied, it is hard to compare CDL based SCM options with legacy TDL cases since the different gains, and we may have to define specific requirement for different AAVs and down tilt values.

Figure 2-3. 3D Directivity pattern of ( 8, 2, 2, 8, 1) at BS side

The SNR normalization factor could be calculated by Equation (2-5) proposed in Option 2a

C=NTxNRxnt=0NTx-1nr=0NRx-1E{Hnt,nr(k,l) 2}                                                                     (2-5)

Where H(k,l)∈CNR×NT is channel coefficient at subcarrier k, OFDM symbol l. NRx is number of Rx and  NTx is number of Tx ports. Hnt,nr(k,l) is the (nt, nr) element of H(k,l). E{Hnt,nr(k,l) 2} can be derived by long-term measurement in the simulation.

Observation 9: For the SNR normalization factor, compared with calculating it directly by long-term channel response, it is somehow more difficult to divide the SNR normalization factor into AE PDP gain factor, BF gain factor and down tilt impact factor, etc.

Proposal 8: The SNR normalization factor could be calculated by Equation (2-5) proposed in Option 2a

C=NTxNRxnt=0NTx-1nr=0NRx-1E{Hnt,nr(k,l) 2}

Where H(k,l)∈CNR×NT is channel coefficient at subcarrier k, OFDM symbol l. NRx is number of Rx and  NTx is number of Tx ports. Hnt,nr(k,l) is the (nt, nr) element of H(k,l). E{Hnt,nr(k,l) 2} can be derived by long-term measurement in the simulation.

  - Comparison of methodologies

TDL and CDL Methodologies

From the way forward of last RAN4#114 meeting, there are three candidates for CDL based methodologies and two candidates for TDL based methodologies. For each candidate, two doppler configurations, two CSI-RS configurations, and three PMI types need to be applied, some candidates may also have different AAV options and/or power configurations.  The number of corresponding simulation cases are huge if all these options are kept, hence it’s helpful if the candidates could be down selected by theoretical analysis. Based on our analysis in section 2.2 and 2.3, we prefer to keep option A2, option A3 and option B1 to do simulation results comparison and alignment.

Way Forward: Options: CDL from TR 38.753 will include randomness reduction for the purpose of the SI - CDL (TR 38.753) - Option A1 : with Table 7.2.1-8 in TR 38.827 - Option A2: based on Table 7.7.1-3 in TR 38.901 without truncation - Option A3 : based on Table 7.7.1-3 in TR 38.901 with truncation According to the updated truncation procedure described in Annex B, truncated table to be shared before RAN4#114-bis. - Option B1: TDL (MediaTek) - Option B2: TDL (Huawei)

Proposal 9: Down select current three candidates for CDL based methodologies to option A2 and option A3 to do simulation results comparison and alignment.

Comparison with Legacy TDL

Simulation results of legacy TDL cases are needed in order to compare with the results of spatial channel models. But considering both the modelling for spatial property of CDL based and TDL extended methods assume cross polarization, it seems option 1d without cross polarization modelling is not needed and unfair as one comparison line (Figure 2-4 for example).

Agreement: The following Legacy TDL models for comparison: - Option 1: TDLC 300-10/100 with following correlation: - Option 1a: Low - Option 1b: XP Medium - Option 1c: XP High - Option 1d: Medium

Figure 2-4. Simulation results of legacy TDL models

In our companion contribution [5], we provide our initial simulation results for CDL and baseline TDL comparison, with 4 ports 4 layers.

Proposal 10: For comparison with Legacy TDL, considering both the modelling for spatial property of CDL based and TDL extended methods assume cross polarization, option 1d without cross polarization modelling should be removed.

Metric for PMI tests comparison

It is reasonable and straightforward to use current Gamma value method defined in 38.101-4 as the metric for PMI tests comparison.

Proposal 11: Use current Gamma value method defined in 38.101-4 as the metric for PMI tests comparison.

From our companion contribution [5], our observations and proposals could be summarized as below.

For doppler 10Hz simulations

SNR normalization factor:

According the SNR normalization factor described in proposal 8, the SNR normalization factors from our simulation results could be summarized as below table, which are related with the number of antennas, AAV options, AE radiation patterns.

| Number of antennas | Models | Normalization factor (linear) | Normalization factor (dB) |
| --- | --- | --- | --- |
| 4Tx4Rx | CDL827-option3 | 0.829431407 | -0.812195238 |
|  | CDL827-option1Y | 4.385069685 | 6.419764993 |
|  | CDL901-option3 | TBD | TBD |
|  | CDL901-option1Y | TBD | TBD |
| 8Tx8Rx | CDL827-option3 | 0.890083597 | -0.505692023 |
|  | CDL827-option1Y | 4.384104304 | 6.418808781 |
|  | CDL901-option3 | TBD | TBD |
|  | CDL901-option1Y | TBD | TBD |
| 8Tx4Rx | CDL827-option3 | 0.812191189 | -0.903417265 |
|  | CDL827-option1Y | 4.424094715 | 6.458244162 |
|  | CDL901-option3 | TBD | TBD |
|  | CDL901-option1Y | TBD | TBD |

Gamma values for PMI reporting

| Number of antennas | Models | SNR @ 90% maximum TP (dB) | Gamma |
| --- | --- | --- | --- |
| 8Tx4Rx2Layer | TDLC TypeI | 4.6 | 2.73 |
|  | TDLC eTypeII | 8.1 | 1.97 |
|  | CDL827 option3 TypeI | 8.3 | 1.31 |
|  | CDL827 option3 eTypeII | 4.7 | 1.86 |
|  | CDL827 option1Y TypeI | 0.8 | 1.34 |
|  | CDL827 option1Y eTypeII | -2.5 | 1.88 |
|  | CDL901 option3 TypeI | TBD | TBD |
|  | CDL901 option3 eTypeII | TBD | TBD |
|  | CDL901 option1Y TypeI | TBD | TBD |
|  | CDL901 option1Y eTypeII | TBD | TBD |
|  | Extended TDL TypeI | 4.2 | 1.87 |
|  | Extended TDL eTypeII | 2.4 | 2.33 |
| 8Tx4Rx4Layer | TDLC TypeI | NA | NA |
|  | TDLC eTypeII | NA | NA |
|  | CDL827 option3 TypeI | 20.1 | 1.07 |
|  | CDL827 option3 eTypeII | 17.6 | 1.30 |
|  | CDL827 option1Y TypeI | 12.5 | 1.08 |
|  | CDL827 option1Y eTypeII | 10.1 | 1.32 |
|  | CDL901 option3 TypeI | TBD | TBD |
|  | CDL901 option3 eTypeII | TBD | TBD |
|  | CDL901 option1Y TypeI | TBD | TBD |
|  | CDL901 option1Y eTypeII | TBD | TBD |
|  | Extended TDL TypeI | TBD | TBD |
|  | Extended TDL eTypeII | TBD | TBD |

- Conclusion

In this contribution, we provide analysis and views on SCM topic.

The observations and proposals could be summarized as:

Observation 1: Steering model 1 is simple and intending to introduce the corresponding relationship between phase shift θ and the desired AOA or AOD angle φ, however the desired angles are not obvious since the configurable parameter is phase shift θ. Steering model 2 follow the design principle of 38.101-4 B.2.3.2.3, it’s straightforward but the meaning of angle θ is not clear.

Proposal 1: A new steering model 3 based on the steering model 1 as below

  - Dθ,N=diag1ej2π∙d∙sinθej2π∙2d∙sinθ⋯ej2π∙(N-1)d∙sinθ

in which d is the distance between each antenna element, the unit is waveform. θ is the desired angle.

Observation 2: For this new steering model 3, one benefit is the desired angles are configurable and obvious, another benefit is more flexible to adapt to different antenna distance.

Observation 3: The complexity of cluster model 2 is similar as legacy TDL channel model, while the complexity of cluster model 1 grows with the number of clusters. Nevertheless, we assume the complexity difference between two cluster models is negligible compared with CDL channel model. Hence, the complexity is not a key issue for extended TDL models.

Observation 4: The power difference between layers or codewords of cluster model 1 is clear, while the power difference between layers or codewords of cluster model 2 is ambiguous and need further calculation.

Proposal 2: Use cluster model 1 as the starting point for extended TDL model performance evaluation.

Proposal 3: For the number of clusters, two clusters for rank4 cases and four clusters for rank8 cases should be assumed.

Proposal 4: In order to distinguish the SNR between different layers/codewords, the power of clusters should be defined. For two clusters, 0dB and -3dB could be selected as the starting point. For four clusters, 0dB, 0dB, -3dB, -3dB could be selected as the starting point.

Proposal 5: For two clusters, desired angles could be 0 and π/2. For four clusters, desired angles could be 0, π/6, π/2 and 3π/2.

Observation 5: For CDL Option A1, Table 7.2.1-8 in 38.827 is the results after applying angle scaling and angle rotation, in which the ZoA angles are const 90 degrees becasue OTA test is 2D without elevation modeling.

Proposal 6: For CDL Option A1, the restriction for const 90 degrees ZoA is not needed for generic CDL modeling in RAN4.

Observation 6: For CDL Option A1, the calculated angle spread values of AoA angles according the calculation Equation defined in 38.827 are different with 38.827 Table 7.2-2, need further clarification.

Observation 7: Angle spreads ASmodel are different for CDL Option A1 and CDL Option A2 as they are using different calculation Equations.

Observation 8: The channel model parameter values after angle scaling and angle rotation are slightly different between based on 38.827 angle spreads and based on 38.901 angle spreads.

Proposal 7: For CDL Option A2, the table of channel models parameters after angle scaling and angle rotation should be compared and aligned between companies in order to provide simulation results based on the same assumptions. Our table of channel parameters for CDL Option A2 is as Table 2-6 above.

Observation 9: For the SNR normalization factor, compared with calculating it directly by long-term channel response, it is somehow more difficult to divide the SNR normalization factor into AE PDP gain factor, BF gain factor and down tilt impact factor, etc.

Proposal 8: The SNR normalization factor could be calculated by Equation (2-5) proposed in Option 2a

C=NTxNRxnt=0NTx-1nr=0NRx-1E{Hnt,nr(k,l) 2}

Where H(k,l)∈CNR×NT is channel coefficient at subcarrier k, OFDM symbol l. NRx is number of Rx and  NTx is number of Tx ports. Hnt,nr(k,l) is the (nt, nr) element of H(k,l). E{Hnt,nr(k,l) 2} can be derived by long-term measurement in the simulation.

Proposal 9: Down select current three candidates for CDL based methodologies to option A2 and option A3 to do simulation results comparison and alignment.

Proposal 10: For comparison with Legacy TDL, considering both the modelling for spatial property of CDL based and TDL extended methods assume cross polarization, option 1d without cross polarization modelling should be removed.

Proposal 11: Use current Gamma value method defined in 38.101-4 as the metric for PMI tests comparison.

- Reference

[1] RP-241610, New SID: Study on spatial channel model for demodulation performance requirements, RAN#104 meeting, Nokia, BT Plc, AT&T, Bell Mobility, Bouygues Telecom, China Telecom, CMCC, Deutsche Telekom, Ericsson, Intel Corporation, KDDI, Keysight, KT Corp., MediaTek, NTT Docomo, Orange, Qualcomm, Rohde & Schwarz, Samsung, SK Telecom, Spark NZ, Telecom Italia, Telefonica, Telenor, Telia Company, Telstra, T-Mobile USA, Verizon, Vodafone, ZTE Corporation

[2] RP-250577, Revised SID: Study on spatial channel model for demodulation performance requirements, RAN#107 meeting, Nokia, BT Plc

[3] R4-2502378, Way Forward for [114][322]NR_SCM, RAN4#114 meeting, Nokia

[4] R4-2500337, TP to TR38.753: TDL approaches and related Annex, RAN4#114 meeting, MediaTek Inc.

[5] R4-2503659, Simulation results on spatial channel model, RAN4#114bis meeting, Samsung