3GPP TSG-RAN WG4 Meeting #114bis		R4-2503663

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:		7.10.3.5

Source:		Samsung

Title:	Discussion on RRM performance requirements of Rel-19 NR-NTN in less than 5MHz

Document for:		Discussion

- Introduction

The latest revised WID [1] has been approved in the RAN plenary RAN#106 meeting. The objectives of performance part of less than 5MHz for NTN are described in [1] as:

Less than 5MHz for NTN - Specify the necessary conformance requirements for SAN for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary RRM performance requirements and test cases for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary demodulation performance requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

Until last meeting RAN4#114, the RRM core requirements are almost completed.

From this meeting, RAN4 will start the discussion on RRM performance part of less than 5MHz for NTN. In this contribution, we provide our view of the RRM performance part of less than 5MHz for NTN.

- Discussion

In previous meeting, RAN4 achieved the consensus on RRM core requirements of less than 5MHz for NTN.

The performance requirements can be divided into two parts: Measurement accuracy and Test cases:

Measurement accuracy

In last RAN4#114 meeting, RAN4 agreed not to introduce two set of side conditions.

The existing SSB based L3 and L1 accuracy requirements can be applied for less than 5MHz in NTN, no need to add extra accuracy requirements.

Proposal 1: Existing SSB based L3 and L1 accuracy requirements in FR1 NTN can be applied for less than 5MHz BW, and no new accuracy requirement is needed.

Test cases

For existing test for Satellite access, the test configuration is BW 10MHz. The common parameters such as PDSCH RMC, CORESET for RMSI, CORESET for RMC has already been added in TN less than 5MHz.

Proposal 2: The common configuration such as SSB pattern, PDSCH RMC, CORESET for RMSI, CORESET for RMC can reuse those in TN less than 5MHz system.

The core requirement impacts by this WI are summarized as:

- HO

- Conditional HO

- Satellite switching with re-synchronization

- Radio Link Monitoring

- BFR

- L3 intra-frequency measurements

- L3 inter-frequency measurements

In general, this type of core requirements is new for less than 5MHz, it is straightforward to add new test cases to verify the core requirements.

| Procedures | Current NTN test |  |
| --- | --- | --- |
| HO | - Intra-frequency SAN Handover from FR1 to FR1 - Inter-frequency SAN Handover from FR1 to FR1 | Due to extended delay for new requirements, need new tests. |
| Conditional HO | - Intra-frequency SAN time-based conditional Handover from FR1 to FR1 - Inter-frequency SAN time-based conditional Handover from FR1 to FR1 - Intra-frequency SAN distance-based conditional Handover from FR1 to FR1 - Inter-frequency SAN distance-based conditional Handover from FR1 to FR1 - Intra-frequency SAN time-based conditional Handover without L3 measurement criteria from FR1 to FR1 - Inter-frequency SAN time-based conditional Handover without L3 measurement criteria from FR1 to FR1 | Due to extended delay for new requirements, need new tests. |
| Satellite switching with re-synchronization | - RACH-based Hard Satellite switching with re-synchronization from FR1 to FR1 - RACH-less Soft Satellite switching with re-synchronization from FR1 to FR1 | Due to extended delay for new requirements, need new tests. |
| RLM | - Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode - Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode. - Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode - Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode | Due to new requirements, need new tests. |
| BFD | - Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in non-DRX mode - Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in DRX mode | Due to new requirements, need new tests. |
| Intra-frequency L3 measurement | - SA event triggered reporting tests without gap under non-DRX - SA event triggered reporting tests without gap under DRX - SA event triggered reporting tests without gap under non-DRX with SSB index reading - SA event triggered reporting tests with single measurement gap under non-DRX for satellite access - SA event triggered reporting tests with FNO concurrent gaps under DRX for satellite access - SA event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for satellite access | Not all test cases are needed |
| Inter-frequency L3 measurement | - SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with single gap for satellite access - SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used with single gap for satellite access - SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used with single gap for satellite access - SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in fully non-overlapped for satellite access - SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in partially partial overalpping for satellite access - Event triggered reporting test without gap under non-DRX - Event triggered reporting tests without gap under DRX | Not all test cases are needed |

Due to the workload of UE test, we prefer not duplicated all the tests in less than 5MHz but pick up several necessary tests.

Proposal 3: To verify the new core requirements by less than 5MHz in NTN, RAN4 to decide the follow test cases:

- [Cell Reselection]

- Intra-frequency SAN Handover from FR1 to FR1

- Inter-frequency SAN Handover from FR1 to FR1

- Intra-frequency SAN time-based conditional Handover from FR1 to FR1

- Inter-frequency SAN distance-based conditional Handover from FR1 to FR1

- Intra-frequency SAN time-based conditional Handover without L3 measurement criteria from FR1 to FR1

- RACH-based Hard Satellite switching with re-synchronization from FR1 to FR1

- Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode

- Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode

- Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in DRX mode

- Intra-frequency: SA event triggered reporting tests without gap under non-DRX with SSB index reading

- Inter-frequency: SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used with single gap for satellite access

Observation 1: For other existing tests with no new core requirements, because the test configuration is BW 10MHz in legacy NTN tests, if UE only support less than 5MHz, which means there are no tests for this UE. If the UE support both <5MHz and larger BW as well, the UE can be tested by existing tests.

Proposal 4: We are open to discuss whether to add new test configurations for <5MHz in existing tests (in such tests, no new core requirements) if the UE supports only less than 5MHz.

- Conclusion

In this contribution, we analyze the potential impact on RRM performance requirements for less than 5MHz for NR-NTN and provide our proposals:

Proposal 1: Existing SSB based L3 and L1 accuracy requirements in FR1 NTN can be applied for less than 5MHz BW, and no new accuracy requirement is needed.

Proposal 2: The common configuration such as SSB pattern, PDSCH RMC, CORESET for RMSI, CORESET for RMC can reuse those in TN less than 5MHz system.

Proposal 3: To verify the new core requirements by less than 5MHz in NTN, RAN4 to decide the follow test cases:

- [Cell Reselection]

- Intra-frequency SAN Handover from FR1 to FR1

- Inter-frequency SAN Handover from FR1 to FR1

- Intra-frequency SAN time-based conditional Handover from FR1 to FR1

- Inter-frequency SAN distance-based conditional Handover from FR1 to FR1

- Intra-frequency SAN time-based conditional Handover without L3 measurement criteria from FR1 to FR1

- RACH-based Hard Satellite switching with re-synchronization from FR1 to FR1

- Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode

- Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode

- Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in DRX mode

- Intra-frequency: SA event triggered reporting tests without gap under non-DRX with SSB index reading

- Inter-frequency: SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used with single gap for satellite access

Observation 1: For other existing tests with no new core requirements, because the test configuration is BW 10MHz in legacy NTN tests, if UE only support less than 5MHz, which means there are no tests for this UE. If the UE support both <5MHz and larger BW as well, the UE can be tested by existing tests.

Proposal 4: We are open to discuss whether to add new test configurations for less than 5MHz in existing tests (in such tests, no new core requirements) if the UE supports only less than 5MHz.

- Reference

[1] RP-242901, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN