3GPP TSG-RAN WG4 Meeting#114bis		R4-2503672

Wuhan, China, Apr 7th -11th, 2025

Agenda item:	7.24.2.3

Source: 	Spreadtrum, UNISOC

Title: 	Discussion on RF requirements for CW

Document for:	Discussion

Introduction

In RAN4#114 meeting, a WF [1] was agreed. This contribution continues to discuss remaining issues as follows.

### Issue 3-1: CW Output power (for ahoc discussion) - Agreement: - CW output power [33] dBm. ### Issue 3-2: ON/OFF power - Agreement: - FFS for OFF power ### Issue 3-3: Frequency error - Agreement: - Reuse Micro BS frequency error ±0.1 ppm for A-IoT CW. ### Issue 3-4: Phase noise - Agreement: - Further discuss phase noise requirements ### Issue 3-5: Unwanted emission - Agreement: - Specify emission requirements, values FFS ### Issue 3-6: Spurious emissions - Agreement: - NR spurious emission requirement can be considered as baseline for R19 AIoT CW node ### Issue 3-7: Intermodulation - Agreement: - FFS

Discussion

- Output power

According to the SID, just D1T1-B case 1-4 needs to consider, which means that CW uses UL spectrum comes from outside topology. During the co-existence study, CW output power is 23dBm when UL spectrum is used, CW output power is 33dBm when DL spectrum is used. With regard to the coverage range of device and outside topology. CW output power is 33dBm could be a starting point.

Proposal 1: CW output power can be 33dBm.

- OFF power

According to the WID, CW is UL spectrum, D2R is UL spectrum and R2D is DL spectrum. From the CW point of view, CW may interfere with NR UE, D2R receiver and R2D receiver. For NR UE, NR UE is only outdoor, the interference between CW and NR UE can be negligible. For D2R receiver, when CW is off, no matter D2R is on or off, the off power of CW cannot interfere D2R receiver because D2R receiver has CW cancellation capability. For R2D receiver, when CW is off, R2D for device 1 may be on. The architecture of R2D for device 1 has no filter. R2D cannot avoid the interference that comes from CW if the off power of CW is not low enough. Hence, the off power of CW needs to be defined to avoid to interfering with the R2D receiver. The value of off power of CW can reuse NR off power (i.e., -50dBm).

Proposal 2: To reduce the interference to R2D receiver, it is needed to define OFF power. The value of off power of CW can reuse NR off power (i.e., -50dBm).

- Phase noise

In current spec, there are no phase noise requirements. As to CW, phase noise may interfere with the D2R receiver. CW cancellation capability needs to consider phase noise. And phase noise maybe related to unwanted emissions. D2R receiver and unwanted emissions are all reflecting CW phase noise performance. Hence, there is no need to define phase noise for CW requirements.

Proposal 3: Other requirements can reflect phase noise performance, there is no need to define phase noise.

- Unwanted emission

CW is a single-tone waveform, channel bandwidth is similar to NB-IoT. In our understanding, unwanted emission can reuse NB-IoT requirements

Proposal 4: Unwanted emission can reuse NB-IoT requirements.

- Intermodulation

CW is a single-tone, there is no intermodulation for CW itself. However, CW may inter modulate with NR signals, thus affecting the performance of NR. In our understanding, it is necessary to consider intermodulation.

Proposal 5: Intermodulation needs to be considered.

Conclusions

In this paper, we provide our views on RF requirements for ambient IoT device, we have made the following proposals.

Proposal 1: CW output power can be 33dBm.

Proposal 2: To reduce the interference to R2D receiver, it is needed to define OFF power. The value of off power of CW can reuse NR off power (i.e., -50dBm).

Proposal 3: Other requirements can reflect phase noise performance, there is no need to define phase noise.

Proposal 4: Unwanted emission can reuse NB-IoT requirements.

Proposal 5: Intermodulation needs to be considered.

Reference

- R4-2502859, WF on requirements for A-IoT BS and CW, , Huawei, TSG RAN-4 meeting #114