3GPP TSG-RAN4 Meeting #114-bis	R4- 2503723

# Wuhan, China, 6th Apr 2025 – 11th Apr 2025

Source:	LG Electronics

Title:	Discussion for ATG UE RF requirements

Agenda item:	7.12.1.1

Document for:	Discussion

Introduction

In the RAN4#114 meeting,  the WF on UE RF requirements for ATG was approved. In this discussion, we share our point of view for the UE RF requirements for ATG based on the approved WF[1]

Discussion

Cross band isolation MSD analysis for ATG UE

The ATG MSD values for the CA_n3-n39 are analysed based on approved WF [1], the analysis details are shown at sub-clauses 2.1.1 and 2.1.2.

MSD for CA_n3A-n39A with n39 UL

Table 1 PRX noise analysis

| 30 MHz BW Tx PSD level | -47.77 | dBm/Hz |
| --- | --- | --- |
| 30 MHz BW leakage PSD level | -99.77 | dBm/Hz |
| PRx n3 Rx rejected power(Tx signal to LNA) | -142.77 | dBm/Hz |
| PRx n39 Tx rejected power(Tx leakage to LNA direct) | -149.77 | dBm/Hz |
| Total noise power | -141.98 | dBm/Hz |
| For 25 MHz DL noise power | -68.00 | dBm |

Table 2 DRX noise analysis

| DRx n3 Rx rejected power | -170.77 | dBm/Hz |
| --- | --- | --- |
| DRx n39 Tx rejected power | -177.77 | dBm/Hz |
| PCB isolation(Rx rejection) | -167.77 | dBm/Hz |
| PCB isolation(Tx rejection) | -159.77 | dBm/Hz |
| Total noise power | -158.79 | dBm/Hz |
| For 25 MHz DL noise power | -84.81 | dBm |

Table 3 MSD value after MRC with SNR and IM

| after MRC | -84.9 | dBm |
| --- | --- | --- |
| n3 REFSENS | -88.9 | dBm |
| MSD(-1 dB SNR, 2.5 dB IM) | 5.5 | dB |

Proposal 1: Cross band isolation MSD value for n39 UL with 23 dBm and DL n3 is 5.5 dB

MSD for CA_n3A-n39A with n3 UL

Table 4 PRX noise analysis

| 25 MHz BW Tx PSD level | -46.98 | dBm/Hz |
| --- | --- | --- |
| 25 MHz BW leakage PSD level | -108.98 | dBm/Hz |
| PRx n39 Rx rejected power(Tx signal to LNA) | -149.54 | dBm/Hz |
| PRx n3 Tx rejected power(Tx leakage to LNA direct) | -148.98 | dBm/Hz |
| Total noise power | -146.24 | dBm/Hz |
| For 30 MHz DL | -71.47 | dBm |

Table 5 DRX noise ananlysis

| DRx n39 Rx rejected power | -177.54 | dBm/Hz |
| --- | --- | --- |
| DRx n3 Tx rejected power | -176.98 | dBm/Hz |
| PCB isolation(Rx rejection) | -187.54 | dBm/Hz |
| PCB isolation(Tx rejection) | -176.98 | dBm/Hz |
| Total noise power | -172.26 | dBm/Hz |
| For 30 MHz DL | -97.49 | dBm |

Table 6 MSD value after MRC with SNR and IM

| after MRC | -97.5 | dBm |
| --- | --- | --- |
| n39 REFSENS | -91.9 | dBm |
| MSD | -4.1 | dB |

Proposal 2: Cross band isolation MSD value for n39 UL with 23 dBm and DL n3 is 0 (-4.1) dB

ACLR 2 or > ACLR2

Section 7.3A.6 of 38.101-1 defines the source of cross-band isolation interference as follows.

In NR, there is no specific definition for the ACLR1 and ACLR2 regions, so the content mentioned above does not clearly specify what areas the first adjacent channel and second adjacent channel represent. However, Section 6.5F.3.3.8 of 38.101-1 defines the first adjacent channel and second adjacent channel as follows.

Based on the above criteria, the adjacent channel regions in CA_n3-n39 are illustrated in the following figure.

Figure 1 ACLR range for CA_n3-n39 UL 39

Figure 2 ACLR range for CA_n3-n39 UL 3

UL 39 is located very close to the victim band n3, and therefore, the source of cross-band isolation interference can vary depending on where the DL fc is located within DL n3. Consequently, the DL fc 1817.5 MHz requested by the operator is located in the >ACLR2 region, so the source of cross-band isolation interference can be >ACLR2.

Proposal 3: Source of cross-band isolation interference for UL n39, DL n3 should be >ACLR2

In the case of UL3, it is relatively far from the victim band n39. As shown in Figure 2, since the victim band n39 is located in the >ACLR2 region of the source of cross-band isolation interference, it should be defined as >ACLR2.

Proposal 4: Source of cross-band isolation interference for UL n3, DL n39 should be >ACLR2

How to implant the MSD requirement for ATG UE to 38.101-1 spec

Unlike general UEs, ATG UEs do not distinguish maximum output power by UE power class. Instead, ATG UEs can report their maximum power within the range of 23 dBm to 40 dBm with a 1 dB resolution. Therefore, including ATG UE MSD in the MSD table defined for standard terminals can be confusing, and it may be difficult to include MSD values for the range of 23 dBm to 40 dBm. Hence, it is necessary to create a separate MSD table specifically for ATG UEs rather than using the existing cross-band isolation MSD table. Defining all MSD values for the range of 23 dBm to 40 dBm individually can make the specification complex. However, by defining the default MSD (@23 dBm) and the values for delta MSD, the cross-band isolation MSD for ATG UEs can be represented relatively simply.

Proposal 5: Make the new MSD table for ATG UE as shown below

Table 7 Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a aggressor NR ATG single UL band for DL NR ATG CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n3 | 1900 | 30 | 15 | 160 (RBstart=0) | 1817.5 | 25 | Max[Default MSD(5.5 dB) + △MSDUE for xxdBm, 0] | >ACLR2 |
| n3 | n39 | [1722.5] | [25] | [15] | [160 (RBstart=0)] | [1900] | [30] | Max[Default MSD(-4.1 dB) + △MSDUE for xxdBm, 0] | >ACLR2 |
| If the MSD value is less than 0, the MSD value is considered as 0. |  |  |  |  |  |  |  |  |  |

Table 8 △MSD for ATG UE

| Output power | MSD (Based on Antenna Architecture with common antenna) |
| --- | --- |
| △MSDUE for 23dBm | 0 |
| △MSDUE for 24dBm | 0.8 |
| △MSDUE for 25dBm | 1.6 |
| △MSDUE for 26dBm | 2.4 |
| △MSDUE for 27dBm | 3.2 |
| △MSDUE for 28dBm | 4 |
| △MSDUE for 29dBm | 4.8 |
| △MSDUE for 30dBm | 5.6 |
| △MSDUE for 31dBm | 6.4 |
| △MSDUE for 32dBm | 7.2 |
| △MSDUE for 33dBm | 8 |
| △MSDUE for 34dBm | 8.8 |
| △MSDUE for 35dBm | 9.6 |
| △MSDUE for 36dBm | 10.4 |
| △MSDUE for 37dBm | 11.2 |
| △MSDUE for 38dBm | 12 |
| △MSDUE for 39dBm | 12.8 |
| △MSDUE for 40dBm | 13.6 |

UE antenna type for inter-band CA

In the last meeting, the agreement was made for issue 2-5 as shown below.

Agreement:

- ATG UE with an antenna array on the bands of PCC and SCC, and one common Rx beam is possible for specific band combination.

  - Antenna array type for inter-band CA is highly depending on frequency range of operating bands and UE implementation.

  - Option 1: Case 4 UE implementation is possible for CA_n3-n39 that frequency ranges between two bands are close to each other.

  - Option 2: Case 4 UE implementation is possible for CA_n3-n39 that DL frequency separation between two bands are less than [X=FFS, e.g, 500 MHz].

There are two options regarding the DL frequency separation range in the sub-bullet. The two options are very similar, but we think option 2 is a clearer way to define the DL frequency range for one common Rx beam than option 1. This is because the word 'close' in option 1 is ambiguous, and therefore clear criteria for DL frequency separation to assume one common Rx beam are necessary. We think the Rx beam power difference should be below 3 dB to assume one common Rx beam. Based on this criterion, a DL frequency separation of 1 GHz results in a 3 dB Rx power difference, assuming a distance of 100 km.

Proposal 6: Support option 2 with X = 1 GHz

- ATG UE with an antenna array on the bands of PCC and SCC, and one common Rx beam is possible for specific band combination.

  - Antenna array type for inter-band CA is highly depending on frequency range of operating bands and UE implementation.

  - Option 2: Case 4 UE implementation is possible for CA_n3-n39 that DL frequency separation between two bands are less than [X=1 GHz].

Conclusion

Proposal 1: Cross band isolation MSD value for n39 UL with 23 dBm and DL n3 is 5.5 dB

Proposal 2: Cross band isolation MSD value for n39 UL with 23 dBm and DL n3 is 0 (-4.1) dB

Proposal 3: Source of cross-band isolation interference for UL n39, DL n3 should be >ACLR2

Proposal 4: Source of cross-band isolation interference for UL n3, DL n39 should be >ACLR2

Proposal 5: Make the new MSD table for ATG UE as shown below

Table 7 Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a aggressor NR ATG single UL band for DL NR ATG CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n3 | 1900 | 30 | 15 | 160 (RBstart=0) | 1817.5 | 25 | [Default MSD(5.5 dB) + △MSDUE for xxdBm] | >ACLR2 |
| n3 | n39 | [1722.5] | [25] | [15] | [160 (RBstart=0)] | [1900] | [30] | [Default MSD(-4.1 dB) + △MSDUE for xxdBm] | >ACLR2 |
| If the MSD value is less than 0, the MSD value is considered as 0. |  |  |  |  |  |  |  |  |  |

Table 8 △MSD for ATG UE

| Output power | MSD (Based on Antenna Architecture with common antenna) |
| --- | --- |
| △MSDUE for 23dBm | 0 |
| △MSDUE for 24dBm | 0.8 |
| △MSDUE for 25dBm | 1.6 |
| △MSDUE for 26dBm | 2.4 |
| △MSDUE for 27dBm | 3.2 |
| △MSDUE for 28dBm | 4 |
| △MSDUE for 29dBm | 4.8 |
| △MSDUE for 30dBm | 5.6 |
| △MSDUE for 31dBm | 6.4 |
| △MSDUE for 32dBm | 7.2 |
| △MSDUE for 33dBm | 8 |
| △MSDUE for 34dBm | 8.8 |
| △MSDUE for 35dBm | 9.6 |
| △MSDUE for 36dBm | 10.4 |
| △MSDUE for 37dBm | 11.2 |
| △MSDUE for 38dBm | 12 |
| △MSDUE for 39dBm | 12.8 |
| △MSDUE for 40dBm | 13.6 |

Proposal 6: Support option 2 with X = 1 GHz

- ATG UE with an antenna array on the bands of PCC and SCC, and one common Rx beam is possible for specific band combination.

  - Antenna array type for inter-band CA is highly depending on frequency range of operating bands and UE implementation.

  - Option 2: Case 4 UE implementation is possible for CA_n3-n39 that DL frequency separation between two bands are less than [X=1 GHz].

- References

# [1] R4-2418140, WF on UE RF reqquirements for ATG (CMCC)