3GPP TSG-RAN WG4 Meeting#114bis		R4-2503731

Wuhan, China, Apr 7th -11st, 2025

Agenda item:	7.7.2

Source: 	Spreadtrum, UNISOC

Title: 	Discussion on UE RF requirements for LB-LB CA switching

Document for:	Discussion

Introduction

In RAN4#114 meeting, a WF [1] was approved. In this contribution, we want to provide some initial views on UE RF requirements for LB-LB CA switching.

Discussion

- Switching delay

According to the new WID, we could take CA_n5A-n29A as an example to analyse in this WI. In RAN4, we need to specify the switching delay and time mask for this band combination. In addition, based on the motivation [2] for this WI, on one side, CA_n5A-n29A, these two DL bands will not work at the same time and it is necessary to perform time-sharing switching based on the TTI period. On the other side, with regard to additional components and volume in the form factor, CA_n5A-n29A aggregating via a single antenna. Hence, in our understanding, switching delay depends on UE architecture implementation.

- UE reference architecture

UE reference architecuture1

Figure1: UE reference architecture1 for CA_n5A-n29A.

Based on UE reference architecture1, there are antenna and RF switch common for n5 band and n29 SDL band. Hence, the switching delay of antenna tuning elements switching is approximately 2us and the switching delay of RF switch is 5us. Hence, in our understanding, 5us or the smallest switching delay 35us for existing spec could be the switching delay.

UE references architecture2

Figure2: UE reference architecture2 for CA_n5A-n29A.

Based on UE reference architecture2, there are antenna and RF switch and mixer common for n5 band and n29 SDL band. Hence, the switching delay of antenna tuning elements switching is approximately 2us and the switching delay of RF switch is 5us, the switching delay of PLL switching is 140us. Hence, in our understanding, 140us could be the switching delay.

Different architecture has different switching delay for CA_n5A-n29A carrier switching. Based on the above analysis, switching delay for LB-LB CA carrier switching could be {35us, 140us} as a starting point.

In addition, based on the architecture of band combination, there is no component switching is related to 13us switching delay. Hence, we don’t need to consider 13us for switching delay.

Proposal 1: The switching delay for LB-LB CA carrier switching could be {35us, and 140us}, we don’t need to consider 13us.

Proposal 2: Shared RF front-end (i.e., antenna, RF switch, or LNA) as UE reference architecture 1 and shared mixer and/or other RF components as UE reference architecture 2 could be a basis for developing RF requirements for the feature.

- Switching period capability

Different band combinations have different architecture, the results of switching delay come from different architecture. In our understanding, the switching period capability can be per band combination.

Proposal 3: Switching delay capability can be per band combination.

- Time mask for switching

Regarding the time mask definition of tx switching, the switching periods described in Figure 1 and Figure 2 are located in either NR band X or band Y as indicated in RRC signalling. For each band pair, the length of uplink switching is indicated by RRC signalling switchingPeriodConfigForBandFair.

Figure1: Time mask for switching between band X and band Y, where the switching period is located in band X

Figure 2: Time mask for switching between band X and band Y, where the switching period is located in band Y

Similar to uplink switching, time mask for switching between FDD and SDL, where the switching period is located depends on RRC configuration. In addition, the switching period is TTI-based according to the switching procedure. TTI=k*slot. We can specify time mask for LBCA switching as follows.

Proposal 4:  Support option1. Switching period can be either within carrier 1(FDD) or within carrier 2(SDL). Specify time mask for LBCA switching based on Figure3 and Figure 4.

- Switching period location

Regarding the time mask definition of TX switching, the switching periods described in Figure 1 and Figure 2 are located in either NR band X or band Y as indicated in RRC signalling. Similarly, switching period location depends on RRC configuration, we can wait for RAN1’s conclusion.

Proposal 5: Switching period location depends on RRC configuration. We can wait for RAN1’s conclusion.

Conclusions

In this paper, we provide our views on UE RF requirements for LB-LB CA, we have made the following proposals.

Proposal 1: The switching delay for LB-LB CA carrier switching could be {35us, 140us}, we don’t need to consider 13us.

Proposal 2: Shared RF front-end (i.e., antenna, RF switch, or LNA) as UE reference architecture 1 and shared mixer and/or other RF components as UE reference architecture 2 could be a basis for developing RF requirements for the feature.

Proposal 3: Switching delay capability can be per band combination.

Proposal 4:  Support option1. Switching period can be either within carrier 1(FDD) or within carrier 2(SDL). Specify time mask for LBCA switching based on Figure3 and Figure 4.

Proposal 5: Switching period location depends on RRC configuration. We can wait for RAN1’s conclusion.

- Reference

- R4-2503016, WF on low-low band switching, Apple. TSG-RAN4#114 meeting