3GPP TSG-RAN WG4 Meeting #114bis	R4-2503741

Wuhan, China, 7th – 11st April 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on UE RF requirements for NR NTN Ku band

# Agenda item:	7.11.4

# Document for:	Approval

# 1 Introduction

In RAN#107 meeting, a revised Work Item [1] was approved on introduction of Ku Band for NR NTN for NR NTN with the following RAN4 objective. In this contribution, we want to share some further views about NR NTN Ku band from system parameters perspective.

The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate Tx/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for coexistence scenario, frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services. - Specify RF requirements for mobile VSAT served by NGSO and GSO based on existing mobile VSAT type 5 RF requirements - Investigate and if possible specify common FR1 and FR2 radiated RF requirements for Satellite Access Node (SAN) and NTN VSAT types (excluding types 1 to 3) in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing, and 10MHz, 15MHz, 25MHz, 35MHz and 50MHz channel bandwidths with 15 kHz subcarrier spacing. - Note: Same subcarrier spacing is assumed for SSB and data channels - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing. - Note: Simultaneous operation of FR1-NTN and FR2-NTN numerology is not supported in the UE - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A. - Specify RRM requirements including timing requirements for mobile VSAT served by NGSO - NOTE: RRM requirements are defined for VSAT supporting FR1-NTN operation only, or for FR2-NTN operation only.

# 2 Discussion

  - 2.1 Off-axis EIRP emission density limit within the operating band

- Issue 1-2-5: Off-axis EIRP requirements for the CEPT Ku band - For the Ku band concerning requirements from ECC regulation and ETSI standards, the off-axis EIRP density limits from EN 302 977 will be considered as the starting point - Companies are encouraged to further check the condition for φ > 70° and the application on the maximum number of the VSATs for this requirement

The above is way forward in last meeting and we tend to agree use the off-axis EIRP density limits from EN 302 977 as  the starting point. Moreover, the following wording was brought to our attention in last meeting, which is also included in EN 301 428, EN 302 340 applied for mobile VSAT.

From our understanding, it is necessary to include the condition for φ > 70° into the spec, and we propose to add a note to clarify the relaxation conditions for φ > 70° as follows.

Table 2.1-1: Off-axis EIRP density limits for co-polarized transmissions for band n248 and n509

| θ value | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 2.5° ≤ θ ≤ 7° | 63 – 25log10(θ) – K | 40 |
| 7° ≤ θ ≤ 9.2° | 42 – K | 40 |
| 9.2° ≤ θ ≤ 48° | 66 – 25log10(θ) – K | 40 |
| 48° < θ | 24 – K | 40 |
| NOTE 1: K=10log(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one mobile VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.3.2 in EN 301 428. The manufacturer shall declare the value of N. NOTE 2: For θ> 70° the values given above may be increased to 4 - H dBW over the range of angles for which the particular feed system may give rise to relatively high levels of spillover. |  |  |

Proposal 1: We propose to add a note to clarify the relaxation conditions for φ > 70° and define off-axis EIRP requirements for the CEPT Ku band as Table 2.1-1.

Furthermore, off-axis EIRP density limits for cross-polarized also needs to be specified for CEPT Ku band. The requirements in EN 301 428, EN 302 340 and EN 302 977 are specified as follows:

EN 301 428 (14.0-14.5 GHz)

EN 302 340, EN 302 977 (14.0-14.5 GHz)

Base on above, we propose to define off-axis EIRP density limits for cross-polarized component for CEPT Ku band as Table 2.1-2.

Table 2.1-2: Off-axis EIRP density limits for cross-polarized transmissions

| θ value | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 2.5° ≤ θ ≤ 7° | 53 – 25log10(θ) – K | 40 |
| 7° ≤ θ ≤ 9.2° | 32 – K | 40 |
| NOTE: K=10log(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one mobile VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.3.2 in EN 301 428. The manufacturer shall declare the value of N. |  |  |

Proposal 2: We propose to define off-axis EIRP density limits for cross-polarized component for CEPT Ku band as Table 2.1-2.

  - 2.2 On-axis spurious requirement

- Issue 1-2-6: On-axis spurious requirements for the CEPT Ku band - For the CEPT Ku band on-axis spurious requirements - The On-axis spurious requirements for the “emissions disabled” and “carrier-off” will be set as EIRP limit [9] dBm with measurement bandwidth [100 kHz] - FFS for the “carrier-on” state - Regulations of other countries can be further checked whether further adjustment is needed for the on-axis spurious requirement

In last meeting, we reached an agreement on on-axis spurious requirements for the “emissions disabled” and “carrier-off”. For “carrier-on” state, we need further discussion. Based on the following picture cited in [3], it can be found that EN 303 980 and EN 303 981 apply for mobile VSAT with NGSO, and EN 302 977 is applied for mobile VSAT with GSO.

On-axis spurious requirement in “Carrier-on” state

EN 302 977

EN 303 980, EN 303 981

For on-axis spurious limits in “Carrier-on” state, EN 302 977 gives different requirements for outside and inside with dBW in any 100 kHz for Ku band in UL 14.0–14.5 GHz for mobile VSAT with GSO. However, EN 303 980 and EN 303 981 define the same requirement for both outside and inside as 44-K dBm/MHz for mobile VSAT with NGSO. For simplification, we propose to use the most stringent requirement for both NGSO and GSO as follows.

Table 2.2-1: On-axis spurious limits in “Carrier-on” state - outside

| Frequency range (GHz) | NTN VSAT type | EIRP Limit (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- | --- |
| 14.0 – 14.5 | 4, 5 | 34 - K (NOTE) | 100 |
| NOTE: K=10log10(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one NTN VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.2.2.1 in EN 302 977. The manufacturer shall declare the value of N. |  |  |  |

Table 2.2-2: On-axis spurious limits in “Carrier-on” state - inside

| Frequency range (GHz) | NTN VSAT type | EIRP Limit (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- | --- |
| 14.0 – 14.5 | 4, 5 | 48 - K (NOTE) | 100 |
| NOTE: K=10log10(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one NTN VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.2.2.1 in EN 302 977. The manufacturer shall declare the value of N. |  |  |  |

Proposal 3: It is proposed to define on-axis spurious limits in “Carrier-on” state for CEPT Ku band as Table 2.2-1 and 2.2-2.

  - 2.3 Off-axis spurious requirement

- Issue 1-2-7: Off-axis spurious requirements for the CEPT Ku band - For the Off-axis spurious requirements for the CEPT Ku band - Specify the Off-axis spurious general requirement at 10 meters distance according to table 1.2.6-1 for the CEPT Ku band Table 1-2-6.1 Frequency range (MHz); EIRP Limit (dBμV/m); Measurement bandwidth (kHz). 30 – 230; 30; 120. 230 – 1000; 37; 120. - Off-axis spurious limits in “Carrier-on”, “Carrier-off” and the "Emissions-disabled" state in EN 302 977 as a starting point for mobile VSAT with GSO for Ku band - Off-axis spurious limits in “Carrier-on”, “Carrier-off” and the "Emissions-disabled" state specified in EN 303 980, EN 303 981 as a starting point for mobile VSAT with NGSO for Ku band - Regulations of other countries can be further checked whether further adjustment is needed for the off-axis spurious requirement

For off-axis spurious limits in “Carrier-on” and “Carrier-off” state, EN 303 980, EN 303 981 and EN 302 977 define a little different requirements for Ku band in UL 14.0–14.5 GHz. The measurement bandwidth and frequency range are different for mobile VSAT with NGSO and mobile VSAT with GSO, which is shown in the following pictures.

EN 303 980, EN 303 981 (mobile VSAT with NGSO)

EN 302 977 (mobile VSAT with NGSO)

For simplification, it’s better to converge on those requirements and convert the values in dBm. We propose to use the same requirement for both NGSO and GSO considering 100kHz measurement bandwidth as follows.

Table 2.3-1: Off-axis spurious limits in “Carrier-on” and “Carrier-off” states

| Frequency range (GHz) | EIRP Limit (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 1.00 – 2.00 | -47 | 0.1 |
| 2.00 – 3.40 | -41 | 0.1 |
| 3.40 – 10.70 | -35 | 0.1 |
| 10.70 – 13.75 | -29 | 0.1 |
| 13.75 – 14.00 | 5 | 10 |
| 14.50 – 14.75 | 5 | 10 |
| 14.75 – 21.20 | -29 | 0.1 |
| 21.20 – 60.00 | -23 | 0.1 |

Proposal 4: We propose to use the same requirement for both NGSO and GSO considering 100kHz measurement bandwidth as Table 2.3-1 for CEPT Ku band.

  - 2.4 OTA reference sensitivity level

For OTA reference sensitivity level, it is channel bandwidth dependent. In current spec, it specifies the minimum EIS for 50MHz channel bandwidth, and for other channel bandwidths larger than 50MHz, the corresponding EIS requirement could be scaled with PRB based compared with 50MHz. In addition, during ka band discussion, it defines the minimum EIS based on the link budget [3][4][5]. For Ku band, we can use the same approach to define the minimum EIS and assume 10MHz channel bandwidth as the worst case to derive the requirement. The calculation can be found in the following table.

Table 2.4-1: The minimum EIS calculation

|  | GSO |  | LEO1200 |  | LEO600 |  |
| --- | --- | --- | --- | --- | --- | --- |
| h | 35786.00 | 35786.00 | 1200.00 | 1200.00 | 600.00 | 600.00 |
| B | 6378.00 | 6378.00 | 6378.00 | 6378.00 | 6378.00 | 6378.00 |
| r | 42164.00 | 42164.00 | 7578.00 | 7578.00 | 6978.00 | 6978.00 |
| Elevation_degree | 35.00 | 90.00 | 35.00 | 90.00 | 35.00 | 90.00 |
| Elevation_radian | 0.61 | 1.57 | 0.61 | 1.57 | 0.61 | 1.57 |
| D2 | 38180.79 | 35786.00 | 1830.82 | 1200.00 | 967.37 | 600.00 |
| f_MHz | 11000.00 | 11000.00 | 11000.00 | 11000.00 | 11000.00 | 11000.00 |
| Pathloss_FreeSpace | 204.90 | 204.34 | 178.52 | 174.85 | 172.98 | 168.83 |
| Atmosphere_loss | 2.00 | 2.00 | 2.00 | 2.00 | 2.00 | 2.00 |
| NF [dB] | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 |
| BW [MHz] | 10.00 | 10.00 | 10.00 | 10.00 | 10.00 | 10.00 |
| SNR [target SNR] | -1.00 | -1.00 | -1.00 | -1.00 | -1.00 | -1.00 |
| IM | 2.00 | 2.00 | 2.00 | 2.00 | 2.00 | 2.00 |
| SAN_AntennaGain | 55.50 | 55.50 | 55.50 | 55.50 | 55.50 | 55.50 |
| SAN_TxPower [dBm] | 80.00 | 83.00 | 50.00 | 53.00 | 44.00 | 47.00 |
| EIS_min | -126.90 | -123.34 | -130.52 | -123.85 | -130.98 | -123.83 |

Based on the above analysis, the minimum EIS could be defines as -131 dBm for 10MHz, and for the other channel bandwidth the corresponding EIS requirement could be scaled with PRB based compared with 50MHz for both GSO and LEO.

Proposal 5: The minimum EIS could be defined as -131 dBm for 10MHz, and for the other channel bandwidth the corresponding EIS requirement could be scaled with PRB based compared with 10MHz for both GSO and LEO.

  - 2.5 Maximum input level

For the maximum input level, we also can use same way as Ka band to derive the requirement, which is calculated based on the link budget. In the following table, we calculate the requirement based on the 64QAM modulation order. For target SNR for DL 64QAM, we assume the same value as Ka band [4]. It can observed that the min Rx antenna size is too large for LEO1200 and LEO600, which is mainly impacted by the target SNR. Therefore, the target SNR needs further discussion.

Table 2.5-1: The maximum input level calculation

|  | GSO |  | LEO1200 |  | LEO600 (set 1) |  |
| --- | --- | --- | --- | --- | --- | --- |
| h | 35786.00 | 35786.00 | 1200.00 | 1200.00 | 600.00 | 600.00 |
| B | 6378.00 | 6378.00 | 6378.00 | 6378.00 | 6378.00 | 6378.00 |
| r | 42164.00 | 42164.00 | 7578.00 | 7578.00 | 6978.00 | 6978.00 |
| Elevation_degree | 35.00 | 90.00 | 35.00 | 90.00 | 35.00 | 90.00 |
| Elevation_radian | 0.61 | 1.57 | 0.61 | 1.57 | 0.61 | 1.57 |
| D2 | 38180.79 | 35786.00 | 1830.82 | 1200.00 | 967.37 | 600.00 |
| f_MHz | 11000.00 | 11000.00 | 11000.00 | 11000.00 | 11000.00 | 11000.00 |
| Pathloss_FreeSpace | 204.90 | 204.34 | 178.52 | 174.85 | 172.98 | 168.83 |
| Atmosphere_loss | 2.00 | 2.00 | 2.00 | 2.00 | 2.00 | 2.00 |
| NF [dB] | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 |
| BW [MHz] | 10.00 | 10.00 | 10.00 | 10.00 | 10.00 | 10.00 |
| Target SNR for DL 64QAM | 6.00 | 9.50 | 6.80 | 12.00 | 12.50 | 18.00 |
| IM | 2.00 | 2.00 | 2.00 | 2.00 | 2.00 | 2.00 |
| SAN_AntennaGain | 55.50 | 55.50 | 55.50 | 55.50 | 55.50 | 55.50 |
| SAN_TxPower [dBm] | 80.00 | 83.00 | 50.00 | 50.00 | 44.00 | 47.00 |
| MaxInputPower | -126.90 | -123.34 | -130.52 | -126.85 | -130.98 | -123.83 |
| Gain_element | 5.00 | 5.00 | 5.00 | 5.00 | 5.00 | 5.00 |
| Rx_Antenna_size_min | 777.10 | 765.97 | 2148.22 | 3055.96 | 8870.95 | 6068.59 |

Proposal 6: The target SNR used for the maximum input level calculation needs further discussion.

  - 2.6 Receiver antenna off-axis performance

For receiver antenna off-axis performance, EN 302 340, EN 303 980, EN 303 981 and EN 302 977 specify different requirements for different VSATs. EN 303 980 and EN 303 981 apply for mobile VSAT with NGSO, while EN 302 977 and EN 302 340 are applied for mobile VSAT with GSO. Receiver antenna off-axis performance in EN 303 977 and EN 302 340 is same as the requirement for Ka band in TS 38.101-5.

EN 303 977 and EN 302 340

Observation 1: For receiver antenna off-axis performance, EN 303 977 and EN 302 340 have the same requirement for mobile VSAT with GSO operating in CEPT Ku band as defined in TS 38.101-5 for Ka band.

For mobile VSAT with NGSO for Ku band, EN 303 981 defines two sets of receiver antenna off-axis performance for two classes of VSAT. The difference between two classes is that class A transmits at elevation angles of 50°or greater, relative to the horizontal plane, and class B transmits at elevation angles of 25°or greater. However, in EN 303 980, it specifies only one set requirement same as the requirement of class A in EN 303 981.

EN 303 980

EN 303 981

Observation 2: EN 303 981 defines two sets of receiver antenna off-axis performance for two classes of mobile VSAT with NGSO for CEPT Ku band.

Based on the above analysis, we propose to use the most stringent requirement to define receiver antenna off-axis performance for both GSO and NGSO, which is shown in the following table.

Table 2.5-1: Off-axis Co-polarized gain limit

| φ value (degree) | gain (dBi) |
| --- | --- |
| φmin ≤ φ < 48° | 32 – 25log10(φ) |
| 48° ≤ φ ≤ 85° | -10 |
| 85° ≤ φ ≤ 180° | -2 |
| NOTE: φmin = 1o or 100λ/D (degrees) whichever is the greater, for D/λ≥ 50. φmin = 2o or 114(D/ λ)-1.09 (degrees) whichever is the greater, for D/λ 50. where D is the nominal diameter of the antenna |  |

Table 2.5-2: Off-axis Cross-polarized gain limit

| φ value (degree) | gain (dBi) |
| --- | --- |
| φr ≤ φ ≤ 7° | 23 – 20log10(φ) |
| 7° < φ ≤ 32.3° | 20.2 – 16.7log10(φ) |
| 32.3° < φ ≤ 75° | -5 |
| 75° < φ ≤ 180° | -2 |
| NOTE: φr = 1o or 100λ/D (degrees) whichever is the greater where D is the nominal diameter of the antenna |  |

Proposal 7: It is proposed to define the receiver antenna off-axis gain of each co-polarized and cross-polarized components as Table 2.5-1 and Table 2.5-2 respectively for CEPT Ku band.

# 3 Conclusion

In this contribution, we shared some views on UE RF requirements for NTN Ku band and the proposals are made as follows:

Proposal 1: We propose to add a note to clarify the relaxation conditions for φ > 70° and define off-axis EIRP requirements for the CEPT Ku band as Table 2.1-1.

Table 2.1-1: Off-axis EIRP density limits for co-polarized transmissions for band n248 and n509

| θ value | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 2.5° ≤ θ ≤ 7° | 63 – 25log10(θ) – K | 40 |
| 7° ≤ θ ≤ 9.2° | 42 – K | 40 |
| 9.2° ≤ θ ≤ 48° | 66 – 25log10(θ) – K | 40 |
| 48° < θ | 24 – K | 40 |
| NOTE 1: K=10log(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one mobile VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.3.2 in EN 301 428. The manufacturer shall declare the value of N. NOTE 2: For θ> 70° the values given above may be increased to 4 - H dBW over the range of angles for which the particular feed system may give rise to relatively high levels of spillover. |  |  |

Proposal 2: We propose to define off-axis EIRP density limits for cross-polarized component for CEPT Ku band as Table 2.1-2.

Table 2.1-2: Off-axis EIRP density limits for cross-polarized transmissions

| θ value | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 2.5° ≤ θ ≤ 7° | 53 – 25log10(θ) – K | 40 |
| 7° ≤ θ ≤ 9.2° | 32 – K | 40 |
| NOTE: K=10log(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one mobile VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.3.2 in EN 301 428. The manufacturer shall declare the value of N. |  |  |

Proposal 3: It is proposed to define on-axis spurious limits in “Carrier-on” state for CEPT Ku band as Table 2.2-1 and 2.2-2.

Table 2.2-1: On-axis spurious limits in “Carrier-on” state - outside

| Frequency range (GHz) | NTN VSAT type | EIRP Limit (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- | --- |
| 14.0 – 14.5 | 4, 5 | 34 - K (NOTE) | 100 |
| NOTE: K=10log10(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one NTN VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.2.2.1 in EN 302 977. The manufacturer shall declare the value of N. |  |  |  |

Table 2.2-2: On-axis spurious limits in “Carrier-on” state - inside

| Frequency range (GHz) | NTN VSAT type | EIRP Limit (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- | --- |
| 14.0 – 14.5 | 4, 5 | 48 - K (NOTE) | 100 |
| NOTE: K=10log10(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one NTN VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.2.2.1 in EN 302 977. The manufacturer shall declare the value of N. |  |  |  |

Proposal 4: We propose to use the same requirement for both NGSO and GSO considering 100kHz measurement bandwidth as Table 2.3-1 for CEPT Ku band.

Table 2.3-1: Off-axis spurious limits in “Carrier-on” and “Carrier-off” states

| Frequency range (GHz) | EIRP Limit (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 1.00 – 2.00 | -47 | 0.1 |
| 2.00 – 3.40 | -41 | 0.1 |
| 3.40 – 10.70 | -35 | 0.1 |
| 10.70 – 13.75 | -29 | 0.1 |
| 13.75 – 14.00 | 5 | 10 |
| 14.50 – 14.75 | 5 | 10 |
| 14.75 – 21.20 | -29 | 0.1 |
| 21.20 – 60.00 | -23 | 0.1 |

Proposal 5: The minimum EIS could be defined as -131 dBm for 10MHz, and for the other channel bandwidth the corresponding EIS requirement could be scaled with PRB based compared with 10MHz for both GSO and LEO.

Proposal 6: The target SNR used for the maximum input level calculation needs further discussion.

Observation 1: For receiver antenna off-axis performance, EN 303 977 and EN 302 340 have the same requirement for mobile VSAT with GSO operating in CEPT Ku band as defined in TS 38.101-5 for Ka band.

Observation 2: EN 303 981 defines two sets of receiver antenna off-axis performance for two classes of mobile VSAT with NGSO for CEPT Ku band.

Proposal 7: It is proposed to define the receiver antenna off-axis gain of each co-polarized and cross-polarized components as Table 2.5-1 and Table 2.5-2 respectively for CEPT Ku band.

Table 2.5-1: Off-axis Co-polarized gain limit

| φ value (degree) | gain (dBi) |
| --- | --- |
| φmin ≤ φ < 48° | 32 – 25log10(φ) |
| 48° ≤ φ ≤ 85° | -10 |
| 85° ≤ φ ≤ 180° | -2 |
| NOTE: φmin = 1o or 100λ/D (degrees) whichever is the greater, for D/λ≥ 50. φmin = 2o or 114(D/ λ)-1.09 (degrees) whichever is the greater, for D/λ 50. where D is the nominal diameter of the antenna |  |

Table 2.5-2: Off-axis Cross-polarized gain limit

| φ value (degree) | gain (dBi) |
| --- | --- |
| φr ≤ φ ≤ 7° | 23 – 20log10(φ) |
| 7° < φ ≤ 32.3° | 20.2 – 16.7log10(φ) |
| 32.3° < φ ≤ 75° | -5 |
| 75° < φ ≤ 180° | -2 |
| NOTE: φr = 1o or 100λ/D (degrees) whichever is the greater where D is the nominal diameter of the antenna |  |

# 4 Reference

- RP-250799, Revised WID: Introduction of Ku bands for NR NTN, Intelsat et al.

- R4-2502288, Way Forward for [114][314] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL)

- R4-2402522, Further discussion on UE RF Rx requirements for NTN in Ka-band, ZTE Corporation

- R4-2405642, Further discussion on UE RF Rx requirements for NTN in Ka-band, ZTE Corporation

- R4-2402063, Discussion on Rx requirement for Ka band NTN UE, Huawei, HiSilicon