# 3GPP TSG-RAN WG4 Meeting #114bis                              R4-2503756

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.22.3.2

Source: 	CMCC

Title: 			 Discussion on other RRM requirements for NR MIMO Phase 5

Document for:	Discussion

- Introduction

In RAN #102 meeting, the new WID on NR MIMO Phase 5 was approved [1]. In RAN#105 meeting, the revised WID are approved [2], the objectives are duplicated as following.

RAN1: - Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources Acronyms: - sTRP: single TRP (transmit-receive point) - mTRP: multiple TRP (transmit-receive point) - CJT: coherent joint transmission - DU: distributed unit - CRI: CSI-RS resource indicator RAN2: Specify higher layer support of the enhancements listed above. RAN4: Specify necessary core requirements for the enhancements listed above.

In last meeting, there was discussion on NR MIMO Phase 5, and a WF was agreed [3]. This contribution provides discussion on RRM requirements on UE reporting enhancement for CJT calibration and enhancements for asymmetric DL sTRP/UL mTRP scenarios.

- Discussion

## 2.1 UE reporting enhancement for CJT calibration

RAN1 agreements on UE reporting enhancement for CJT deployments which may have RAN4 impacts are duplicated as following.

Agreements in RAN1#116 For the Rel-19 aperiodic standalone CJT calibration reporting, the following use cases are assumed: - For per-TRP delay offset reporting: - Use case 1.1: TRP selection - Use case 1.2: delay offset compensation for at least one TRP to ensure the CJT-composite delay spread doesn’t exceed a pre-defined dynamic range/threshold - For per-TRP frequency offset (FO) reporting: - Use case 2.1: TRP selection - Use case 2.2: per-TRP FO compensation at NW side For the Rel-19 aperiodic standalone CJT calibration reporting, given the NTRP configured NZP CSI-RS resources/resource sets and the selected N resources/resource sets, support reporting, in one CSI reporting instance, {(Dn,offset, dn), n=0, 1, …, N – 1} where - Dn,offset is a B-bit indicator representing the delay offset associated with the n-th CSI-RS resource/resource set - For the reference CSI-RS resource/resource set nref, the value of Dnref,offset is assumed 0 and not reported - dn is a 1-bit indicator associated with the n-th CSI-RS resource/resource set, indicating whether the measured delay offset, plus delay spread, is inside or outside a pre-defined range/interval For the Rel-19 aperiodic standalone CJT calibration reporting, in addition to the already agreed use cases, the following use cases are assumed: - For per-TRP DL/UL Rx-Tx phase misalignment reporting: - Use case 3.1: TRP selection - Use case 3.2: per-TRP DL/UL Rx-Tx phase compensation at NW side for reciprocity (e.g. using both CSI-RS and SRS for measurement) For the Rel-19 aperiodic standalone CJT calibration reporting, given the NTRP configured NZP CSI-RS resources/resource sets and the selected N resources/resource sets, support reporting, in one CSI reporting instance, {FOn , n=0, 1, …, N – 1, n≠nref}, where FOn denotes the measured frequency offset associated with the n-th CSI-RS resource/resource set relative to the reference CSI-RS resource/resource set nref - For the reference CSI-RS resource/resource set nref, the value of FOnref is assumed 0 and not reported Agreements in RAN1#116bis For the Rel-19 aperiodic standalone CJT calibration reporting, given the NTRP configured NZP CSI-RS resources/resource sets and the selected N resources/resource sets, support reporting, in one CSI reporting instance, {n,, n=0, 1, …, N – 1, n≠nref, =0,1,…,-1}, where n, denotes the measured phase offset between the n-th CSI-RS resource/resource set and the reference CSI-RS resource/resource set nref for the -th frequency unit - The value n, indicates a uniformly quantized phase between –A and A, or 0 and A - FFS: supported quantization alphabet(s) (including A and resolution) for n, For the Rel-19 aperiodic standalone CJT calibration reporting, for a given CJT calibration report of one [or more] CJT calibration report type, the nref is selected by the UE and reported as a part of the CJT calibration report - Note: CJT calibration report type refers to the Doffset/d report, FO report, and, TDD PO report For the Rel-19 aperiodic standalone CJT calibration reporting of {FOn , n=0, 1, …, NTRP – 1, n≠nref}, the value of FOn indicates a uniformly quantized frequency offset between 0 and AFO For the Rel-19 aperiodic standalone CJT calibration reporting, regarding phase offset reporting, the value n,, indicates a uniformly quantized phase between 0 and 2 For the Rel-19 aperiodic standalone CJT calibration reporting, the dynamic range and resolution parameters for delay offset reporting Dn,offset, i.e. (AD, MD), are NW-configured via higher-layer (RRC) signalling from the following candidate values: - AD ={0.5CP, 0.75CP, CP, 1.5CP, 2CP,1/4∆f, 1/12∆f, 1/24∆f } where CP and ∆f denote the length of the cyclic prefix according to the current specifications (for normal CP) within a slot and the SCS, respectively - FFS: Further down-selection of the above candidate values for AD, including the use of a same unit for all supported values - MD ={32, 64} - FFS: If TDD TX/RX timing misalignment report is supported, whether different set of candidate MD values is needed In addition, the inside/outside range for the 1-bit indicator dn is equal to [0, CP]. FFS: Further implicit/explicit restriction(s) on candidate value(s) depending on the CSI-RS configuration For the Rel-19 aperiodic standalone CJT calibration reporting, an ‘invalid’ quantization state/hypothesis is supported for frequency offset and phase offset CJT calibration reporting - Note: already supported as ‘out-of-range’ for the (Dn,offset, dn) reporting - FFS (RAN1#117): The need for a condition/event for ‘invalid’ to be specified as a UE procedure e.g. RSRP-based For the Rel-19 aperiodic standalone CJT calibration reporting, the dynamic range and resolution parameters for frequency offset reporting FOn, i.e. (AFO, MFO), are NW-configured via higher-layer (RRC) signalling from the following candidate values: - AFO = {0.01ppm, 0.1ppm, 0.2ppm, ∆f, ∆f/2, ∆f/4, ∆f/8, 1/(4∆t), 1/(8∆t), 1/(16∆t), 1/(32∆t), 1/(512∆t)} where ∆f and ∆t denote the SCS and duration of one OFDM symbol, respectively - FFS: Further down-selection of the above candidate values for AFO, including the use of a same unit for all supported values - MFO = {16,32} For the Rel-19 aperiodic standalone CJT calibration reporting, the resolution parameters for n,, i.e. M, are NW-configured via higher-layer (RRC) signalling from the candidate values {16, 32} For the Rel-19 aperiodic standalone CJT calibration reporting, in addition to reporting one type of CJT calibration report in one report, at least support reporting {(Dn,offset, dn), n=0, 1, …, NTRP – 1, n≠nref1} and {FOn , n=0, 1, …, NTRP – 1, n≠nref2} in one report - nref1 and nref2 are independently selected and indicated by the UE, - One-part UCI is used Agreements in RAN1#117 For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the dynamic range for delay offset reporting Dn,offset, i.e. AD, at least support the following values: {0.5CP, CP} For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the dynamic range for frequency offset reporting FOn, i.e. AFO, at least support the following values: {0.1ppm, 0.2ppm} Agreements in RAN1#118 For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the resolution for frequency offset reporting FOn, additionally support MFO = 256.

According to RAN1 agreements, it can be seen that CJTC delay offset reporting, CJTC frequency offset (FO) reporting, CJTC phase offset reporting are aperiodic reporting. For the impact on RRM performance requirements, more discussion is needed. In Rel-18 MIMO, time domain channel property (TDCP) is introduced, which is defined as wideband normalized correlation between two CSI-RS transmission occasions, corresponding to CSI-RS resourece(s) from NZP-CSI-RS-ResourceSet(s) configured with higher layer parameter trs-info, that are separated by Dn symbols or slots, depending on the configuration, where Dn is the n-th delay configured value among DelayD configured delay values {D1, …, DY} and Y is number of configured delay values. From RRM perspective, TDCP measurement report mapping requirements including amplitude measurement report mapping and phase measurement report mapping are introduced. For Rel-19 aperiodic standalone CJT calibration reporting, similar as Rel-18 TDCP, at least report mapping need to be specified. RAN1 has some initial agreements on report range and resolution. For delay offset reporting, the dynamic range at least support the following values: {0.5CP, CP}. For frequency offset reporting, the dynamic range at least support the following values: {0.1ppm, 0.2ppm}. For phase offset reporting, it indicates a uniformly quantized phase between 0 and 2. For reporting range of these reporting type, whether other values are supported will be discussed later. Regarding resolution parameters, {16, 32, 256} are supported for frequency offset reporting and phase offset reporting, and {32, 64} are supported for delay offset reporting. Based on above RAN1 agreements, RAN4 can discuss how to define report mapping for delay offset reporting, frequency offset reporting and phase offset reporting. Except report mapping, accuracy requirements also need to be considered to guarantee measurement performance, but whether and how to specify the requirements are pending on RAN1 further agreements.

Proposal 1: for UE reporting enhancement for CJT calibration, it is proposed to define report mapping for delay offset reporting, frequency offset reporting and phase offset reporting.

## 2.2 Enhancements for asymmetric DL sTRP/UL mTRP scenarios

According to RAN1 agreements [6], if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s), else, UE may expect to receive SSB from UL TRP(s). It can be seen that if PL offset is not configured, UL TRP can transmit SSB.

From deployment point of view, UL TRP(s) transmit SSB is one of our typical deployment in HetNet scenario.

ICBM is in use since normal TRP instead of UL-only TRP are deloyed in our HetNet deployment. As shown in Fig.1, indoor pico site and outdoor macro site are deployed at same frequency with different cell ID (Inter-cell). Both indoor pico site and outdoor macro site transmit SSBs with different cell ID. UE transmit PUCCH/PUSCH/SRS to the TRP with additional PCI, and UE receive PDCCH/PDSCH only from normal TRP. The reason to have this deployment is that outdoor macro site has severe interference to indoor pico site, then outdoor macro site can be used for DL transmission, while indoor pico site has better coverage for UL, then indoor pico site can be used for UL reception.

Fig.1 Inter-cell deployment with asymmetric DL and UL transmission

Issue 2-1: Whether RAN4 need to consider the scenario when UE is/isn’t configured with pathloss offset? Agreement: For Scenario#1: RAN4 define the timing requirements for FR1, FFS on FR2. For Scenario#2, FFS on whether to reuse the legacy timing requirements (Rel-18 m-DCI two TA with updated or not). RAN4 common understanding: the RAN1 agreements in RAN1#118bis “One downlink reference timing is supported and applied to both TAGs.” are currently only applied for Scenario #1. - Scenario#1: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s) - Scenario#2: if UE is not configured with PL offset in joint/UL TCI state(s), UE expect to receive SSB from UL TRP(s). Note 1: the scenario 1 and 2 are based on RAN1 LS. Note 2: The above agreements apply when 2TA are configured. Issue 2-2: How to update the RRM requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP? Agreement: - For scenario 1, for both TAGs, the uplink transmission(s) timing for PUCCH/PUSCH/SRS take place (NTA + NTA offset) x Tc before the reception of the first detected path in time of one of the corresponding downlink reference signal(s) of the reference cell associated with DL or Joint TCI state. - The above agreement can be revisited if not aligned with RAN1 further agreement.

According to the discussion in last meeting, companies are not clear on whether RAN1 agreements in RAN1#118bis “ One downlink reference timing is supported and applied to both TAGs ” are applied for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s). RAN4 sent LS to RAN1 for clarification [4]. How to update the RRM requirements support 2TA, e.g. UE transmit timing requirements, is related with DL reference timing. It is proposed to wait for RAN1 reply LS.

Proposal 2: for 2TA enhancement, it is proposed to define RRM requirements for scenario 2 (UE is not configured with PL offset in joint/UL TCI state(s), UE expect to receive SSB from UL TRP(s)), and how to define the requirements can wait for RAN1 reply.

Issue 1-2-2-4: RRM core impacts of active uplink TCI state switching delay for unified TCI? - Proposals - Proposal 1 (Qualcomm): - RAN4 should focus for FR2 only on the case that UL TRP transmits SSB (scenario 2). - In case that the UL TRP transmits SSB (scenario 2), existing requirements for uplink TCI state switching can be reused, and no additional requirements need to be defined. - RAN4 should only focus on the case that the UL TRPs transmit NCD-SSBs. - Proposal 2 (Apple, MediaTek, vivo): No RRM impacts on UL TCI state switching delay requirement for unified TCI states. - Proposal 3 (Huawei): No RRM impacts when PL offset is not configured in joint/UL TCI state(s). - Proposal 4 (vivo): RAN4 does not specify the RRM requirements for UL-only TRP where UE does not expect to receive SSB for FR2-1 in Rel-19. - Proposal 5 (Samsung): RRM core impacts of active uplink TCI state switching delay for unified TCI - FR1: If joint TCI is switching in FR1 mentioned above, there is no difference from legacy, the joint TCI state switching is the same as legacy. No RRM impacts. - FR2: Depends on the conclusion of whether to specify the UL TCI state switching delay base on SRS. - Proposal 6 (ZTE): When applying R17/18 unified TCI state framework to support 2TA in asymmetric scenario, - If there is SSB received from the UL-only TRP, both existing R17/18 joint and UL TCI state could be used. - Otherwise, only the UL TCI state of existing R17/18 could be used for the UL-only TRP. - Proposal 7 (CMCC): - for the case that UL TRP transmit SSB, it is proposed that legacy UL TCI state switching delay requirements can be reused. - Proposal 8: (Ericsson, Nokia): RAN4 to specify UL TCI state switching requirements for UL only TRP. - Proposal 4a: (Nokia) - For the asymmetric DL/UL scenario, reuse Rel-17 and Rel-18 unified TCI single-DCI mTRP TCI state switching delay requirements for DL and UL TCI state switch. - Update the unified UL TCI state switching delay requirements to be applicable when the target UL TCI state is associated with an uplink reference signal. - RAN4 to define UL TCI state switch delay requirements for an uplink only TRP with no distinction between known and unknown TCI state.

For scenario 2 (UE is not configured with PL offset in joint/UL TCI state(s), UE expect to receive SSB from UL TRP(s)), the solution to derive pathloss for UL TRP baed on the pathloss from RSs from normal TRP and pathloss offset is not applicable. While for this case, since SSB is transmitted from UL TRP (i.e. TRP with additional PCI), the SSB can be used as pathloss RS when UE transmits PUCCH and PUSCH to UL TRP. RAN1 also agreed that SSB from UL-TRP can be used for PL RS for PRACH. Taking above into account, legacy requirements can be reused.

Agreement in RAN1#120 meeting For a UE provided with SSB-MTC-AddtionalPCI and not configured with multi-DCI based mTRP, support to reuse the DCI field ‘PRACH association indicator’ in DCI format 1_0 to indicate PL RS for PDCCH-order PRACH: - The bit field index 0 of this field indicates the DL RS that DMRS of PDCCH order DCI is QCLed with is used as PL RS for PRACH; - The bit field index 1 of this field is mapped to the additional PCI associated with the active TCI states and indicates the indicated SSB in this DCI is used as PL RS for PRACH: - In this case, the PRACH configuration associated with addition PCI is used. - This DCI field is present when the corresponding RRC parameter is configured and multi-DCI based mTRP is not configured.

Proposal 3: for scenario 2 (UL TRP transmit SSB), it is proposed that legacy UL TCI state switching delay requirements can be reused.

- Conclusion

This contribution provides discussion on RRM requirements on UE reporting enhancement for CJT calibration and enhancements for asymmetric DL sTRP/UL mTRP scenarios. The proposals are:

Proposal 1: for UE reporting enhancement for CJT calibration, it is proposed to define report mapping for delay offset reporting, frequency offset reporting and phase offset reporting.

Proposal 2: for 2TA enhancement, it is proposed to define RRM requirements for scenario 2 (UE is not configured with PL offset in joint/UL TCI state(s), UE expect to receive SSB from UL TRP(s)), and how to define the requirements can wait for RAN1 reply.

Proposal 3: for scenario 2 (UL TRP transmit SSB), it is proposed that legacy UL TCI state switching delay requirements can be reused.

- Reference

- RP-234007, New WID: NR MIMO Phase 5

- RP-242394, WID revision: NR MIMO Phase 5

- R4-2502596, WF on RRM requirements for NR_MIMO_Ph5 (Part2), MediaTek

- R4-2502667, LS on DL timing reference to support two TA enhancement for asymmetric DL sTRP/ UL mTRP

- R4-2420108, WF on RRM requirements for NR_MIMO_Ph5 (Part2), MediaTek

- R1-2410870, Reply to RAN4 LS on UE RF issues for Rel-19 MIMO Enhancement

- R4-2416921, WF on RRM requirements for NR_MIMO_Ph5 (Part2), MediaTek

- R4-2414035, WF on RRM requirements for NR_MIMO_Ph5, Samsung