# 3GPP TSG-RAN WG4 Meeting #114bis                              R4-2503768

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.19.3

Source: 	CMCC

Title: 			 Discussion on RRM requirements for beam management

Document for:	Discussion

- Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. In RAN #106 meeting, the revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [2]. The objectives for RAN4 are duplicated as following.

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved [3]. This contribution provides discussion on RRM requirements for beam management.

- Discussion

Issue 2-1: Metrics/KPIs for beam prediction If RSRP and beam ID are reported: - RSRP accuracy will be one of the KPIs. - How option 1 is applied is FFS - Beam ID prediction accuracy will be one of the KPIs - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options If only beam ID is reported: - Beam ID prediction accuracy will be used as the KPI - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options

It was agreed that if RSRP and beam ID are reported, RSRP accuracy will be one of the KPIs. But the details of RSRP accuracy are FFS. In RAN4#112bis, it was agreed that if the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i–groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP]. For the part with square brackets in the agreement, we do not think it is necessary. In our view, the accuracy requirements should apply to all predicted/reported beams, no need to limit to a subset of predicted beams. There are two reasons to suggest to apply the accuracy requirements to all predicted/reported beams. On one hand, all the predicted/reported beams are benificial for network, for example, can be used for load balancing. On the other hand, the final reported beam index and/or related L1-RSRP are based on ranking of all the predicted beams. If the accuracy of some predicted beams are not guaranteed, it will have impact on the final decision of the reported beams.

Proposal 1: it is proposed that the RSRP accuracy requirements apply to all predicted/reported beams.

The definition of relative RSRP accuracy are FFS. Firstly, the high level issue is whether to have relative accuracy requirements. Some companies propose not to have relative accuracy requirements in addition to absolute accuracy requirements for AI/ML, while some companies mentioned that relative accuracy is related to monitoring and RAN1 has related discussion. Our preference is to introduce relative accuracy requirements. It is beneficial to determine the optimal beams by comparing the RSRP from different beams. As for how ro define the relative accuracy requirements, in general, it is preferred to follow the definition of legacy L1-RSRP measurement accuracy. According to legacy L1-RSRP measurement, the relative accuracy is defined as the L1-RSRP measured from one SSB compared to the largest measured value of L1-RSRP among all SSBs of the cell on which UE performs L1-RSRP measurements.

For relative RSRP accuracy for AI/ML, it is proposed to follow the legacy way. In detail, the relative RSRP accuracy is the difference between the predicted L1-RSRP of beam index i and the predicted L1-RSRP of beam index n, where the beam index n owns the largest predicted value. According to the discussion in AI/ML for mobility, it was agreed that predicted relative RSRP accuracy = (reported predicted RSRP of cell 1/beam1 – reported RSRP of cell 2/beam2) – (ground truth of RSRP of cell 1/beam1 – ground truth of RSRP of cell 2/beam2). Follow the similar way, the relative RSRP accuracy for beam management is proposed as (predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n) -  (ground truth of L1-RSRP of beam index i – ground truth of L1-RSRP of beam index n) , where the beam index n owns the largest predicted value.

Proposal 2: the relative RSRP accuracy is proposed as:

- Relative RSRP accuracy = (predicted L1-RSRP of beam index i - predicted L1-RSRP of beam index n) -  (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), where the beam index n owns the largest predicted value.

For both the case that RSRP and beam ID are reported and the case that only beam ID is reported, beam ID prediction accuracy will be used as the KPI. According to RAN1 agreements, for BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), at least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring. Even though this agreements are for performance monitoring, they can be used as reference for the discussion of inference KPI. Considering that Top 1 is a sub-use case of Top K. It is prefered to use Top-K/1.

Proposal 3: it is proposed that the beam ID prediction accuracy is defined as Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

Issue 2-3: Measurement period for inference - Proposals - Option 1: Use “legacy” L1-RSRP measurement as baseline - FFS whether same values for M,N,P should be reused - Option 2: Differentiate case 1 and case 2 - Reuse L1 measurement period for Case 1 - Introduce configurable number of measurement occasions for case 2 - Option 3: Study the measurement period based on simulations - Option 4: Others

In RAN4#113 meeting, it was agreed that the prediction delay at least includes measurement for prediction, inference delay, and FFS on whether the legacy measurement delay can be reused for measurement for prediction. Legacy SSB based L1-RSRP measurement period is defined in 9.5.4.1-2 of TS 38.133. Taking FR2 as an example, the detailed requirements are duplicated as following.

Table 9.5.4.1-2: Measurement period TL1-RSRP_Measurement_Period_SSB for FR2 (TS 38.133)

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320 ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

AI/ML based beam management includes two cases. BM-Case1: Spatial-domain Downlink beam prediction for Set A of beams based on measurement results of Set B of beams. BM-Case2: Temporal Downlink beam prediction for Set A of beams based on the historic measurement results of Set B of beams. For BM-Case 1, UE nneed to firstly measure set B, which is similar as legacy L1-RSRP measurement, the legacy L1-RSRP measurement delay can be used as baseline. For BM-Case2,the prediction is performed based on historic measurement results. Some companies mentioned that the UE may perform multiple L1-RSRP measurement occasions and the exact number of these measurement periods/occasions could be configurable and is to be studied and defined in RAN1. with the condition that UE perform multiple L1-RSRP measurement occasions, one thing not clear whether multiple L1-RSRP measurement results (i.e. there will be one L1-RSRP for each L1-RSRP measurement occasion) are used for prediction or one L1-RSRP measurement results (i.e. one L1-RSRP is derived based on the measurement on multiple L1-RSRP measurement occasions) . If the former one is adopted, we do not see issues to reuse legacy L1-RSRP measurement period, since legacy measurement period is for UE to perform the measurement periodically. If the latter one is adopted, legacy L1-RSRP measurement period may need to be scaled by the number of periods/occasions. In summary, the legacy measurement period can be used as baseline, whether and how to update can be further discussed based on RAN1 progress.

Proposal 4: for measurement period for inference, it is proposed to take legacy L1-RSRP measurement period as baseline.

RAN1 agreements in RAN1#120 meeting Agreement For report content of inference results for UE-sided model, where the largest RSRP value is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential RSRP is quantized to a 4-bit value with 2 dB step size. Note: the model output is UE implementation and it doesn’t have to be RSRP subject to dBm value. Agreement For report content of inference results for UE-sided model for BM-Case 1, the RSRP of predicted beam(s)in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output. Note: how to capture it in the spec is a separate discussion. Agreement For UE-side AI/ML model inference and BM-Case2, for the quantization of a RSRP value of inference results in a report over multiple future time instances, - the largest RSRP value based on prediction of all time instances is the reference RSRP, and differential RSRPs in the report are computed relative to the reference RSRP. - The time instance information of the beam with the largest RSRP are additionally indicated in the report.

For beam management, RAN1 ahs agreed the report content of inference results for UE-sided model. The largest RSRP value is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential RSRP is quantized to a 4-bit value with 2 dB step size.

The legacy RSRP measurement report mapping table is defined in RAN4, which are duplicated as following. In legacy, the reporting range of SS-RSRP for L1 reporting is defined from -140 to -44 dBm with 1 dB resolution. And the reporting range of differential SS-RSRP for L1 reporting is defined from 0 dB to -30 dB with 2 dB resolution. It can be seen that RAN1 design on the report content of inference results is alligned with legacy L1 measurement report, which means that legacy L1 report mapping table can be reused for the report of inference results.

Table 10.1.6.1-1: SS-RSRP and CSI-RSRP measurement report mapping (TS 38.133)

| Reported value | Measured quantity value (L3 SS-RSRP) and CSI-RSRP | Measured quantity value (L1 SS-RSRP and CSI-RSRP) | Unit |
| --- | --- | --- | --- |
| RSRP_0 | SS-RSRP<-156 | Not valid | dBm |
| RSRP_1 | -156 SS-RSRP<-155 | Not valid | dBm |
| RSRP_2 | -155 SS-RSRP<-154 | Not valid | dBm |
| RSRP_3 | -154 SS-RSRP<-153 | Not valid | dBm |
| RSRP_4 | -153 SS-RSRP<-152 | Not valid | dBm |
| RSRP_5 | -152 SS-RSRP<-151 | Not valid | dBm |
| RSRP_6 | -151 SS-RSRP<-150 | Not valid | dBm |
| RSRP_7 | -150 SS-RSRP<-149 | Not valid | dBm |
| RSRP_8 | -149 SS-RSRP<-148 | Not valid | dBm |
| RSRP_9 | -148 SS-RSRP<-147 | Not valid | dBm |
| RSRP_10 | -147 SS-RSRP<-146 | Not valid | dBm |
| RSRP_11 | -146 SS-RSRP<-145 | Not valid | dBm |
| RSRP_12 | -145 SS-RSRP<-144 | Not valid | dBm |
| RSRP_13 | -144 SS-RSRP<-143 | Not valid | dBm |
| RSRP_14 | -143 SS-RSRP<-142 | Not valid | dBm |
| RSRP_15 | -142 SS-RSRP<-141 | Not valid | dBm |
| RSRP_16 | -141 SS-RSRP<-140 | RSRP<-140 | dBm |
| RSRP_17 | -140 SS-RSRP<-139 | -140RSRP<-139 | dBm |
| RSRP_18 | -139 SS-RSRP<-138 | -139 RSRP<-138 | dBm |
| … | … |  | … |
| RSRP_111 | -46≤ SS-RSRP<-45 | -46 RSRP<-45 | dBm |
| RSRP_112 | -45 SS-RSRP<-44 | -45 RSRP<-44 | dBm |
| RSRP_113 | -44 SS-RSRP<-43 | -44 RSRP | dBm |
| RSRP_114 | -43 SS-RSRP<-42 | Not valid | dBm |
| RSRP_115 | -42 SS-RSRP<-41 | Not valid | dBm |
| RSRP_116 | -41 SS-RSRP<-40 | Not valid | dBm |
| RSRP_117 | -40 SS-RSRP<-39 | Not valid | dBm |
| RSRP_118 | -39 SS-RSRP<-38 | Not valid | dBm |
| RSRP_119 | -38 SS-RSRP<-37 | Not valid | dBm |
| RSRP_120 | -37 SS-RSRP<-36 | Not valid | dBm |
| RSRP_121 | -36 SS-RSRP<-35 | Not valid | dBm |
| RSRP_122 | -35 SS-RSRP<-34 | Not valid | dBm |
| RSRP_123 | -34 SS-RSRP<-33 | Not valid | dBm |
| RSRP_124 | -33 SS-RSRP<-32 | Not valid | dBm |
| RSRP_125 | -32 SS-RSRP<-31 | Not valid | dBm |
| RSRP_126 | -31 SS-RSRP | Not valid | dBm |
| RSRP_127 (Note) | Infinity | Infinity | dBm |
| NOTE: The value of RSRP_127 is applicable for RSRP threshold configured by the network as defined in TS 38.331 [2], but not for the purpose of measurement reporting. |  |  |  |

Table 10.1.6.1-2: Differential SS-RSRP and CSI-RSRP measurement (for L1 reporting) report mapping (TS 38.133)

| Reported value | Measured quantity value (difference in measured RSRP from strongest RSRP) | Unit |
| --- | --- | --- |
| DIFFRSRP_0 | 0≥ΔRSRP>-2 | dB |
| DIFFRSRP_1 | -2≥ΔRSRP>-4 | dB |
| DIFFRSRP_2 | -4≥ΔRSRP>-6 | dB |
| DIFFRSRP_3 | -6≥ΔRSRP>-8 | dB |
| DIFFRSRP_4 | -8≥ΔRSRP>-10 | dB |
| DIFFRSRP_5 | -10≥ΔRSRP>-12 | dB |
| DIFFRSRP_6 | -12≥ΔRSRP>-14 | dB |
| DIFFRSRP_7 | -14≥ΔRSRP>-16 | dB |
| DIFFRSRP_8 | -16≥ΔRSRP>-18 | dB |
| DIFFRSRP_9 | -18≥ΔRSRP>-20 | dB |
| DIFFRSRP_10 | -20≥ΔRSRP>-22 | dB |
| DIFFRSRP_11 | -22≥ΔRSRP>-24 | dB |
| DIFFRSRP_12 | -24≥ΔRSRP>-26 | dB |
| DIFFRSRP_13 | -26≥ΔRSRP>-28 | dB |
| DIFFRSRP_14 | -28≥ΔRSRP>-30 | dB |
| DIFFRSRP_15 | -30≥ΔRSRP | dB |

Proposal 5: for beam management, it is proposed to reuse legacy L1 report mapping, i.e. L1 SS-RSRP part of Table 10.1.6.1-1 and Table 10.1.6.1-2 in TS38.133 are reused for the report of inference results.

WF in RAN4 #113: Issue 3-6: Measurement error impact evaluation - Proposals - Option 1: model RF error as same within an RF module but varies across modules, FFS the distribution - Option 2: Gaussian distribution with +/- 2.5dB for BB errors and total of +/-6.5 for BB+RF - Option 3: model RF error as - An offset value in the original dataset; - Truncated Uniform distribution; - Truncated Gaussian distribution.

According to the agreed simulation assumptions for beam prediction, simulation assumptions for baseband error and RF error are also covered. For RF error, ±4.5dB is considered. As for the detail about how to generate RF errors, e.g. independent for each TX beam, or same for each TX beam, it is still open. This issue is more related with UE implementation, input from UE vendors is appreciated. As for baseband error, there are two options about how to apply it to the SLS. Option 1 is to apply error distribution from the post beamforming SNR of the strongest Tx-Rx beam pair to all Tx-Rx beam pair’s measurements in the SLS. Option 2 is to generate error distribution separately for each individual Tx-Rx beam pair’s SNRs and apply to the corresponding Tx-Rx beam pair measurement in the SLS. According to the initial simulation results from companies, the performance is different between the two options. From practical point of view, the error is different from different SNRs. It is better to allign with the real deployment. Considering that the main target is to evaluate the measurement error impact on prediction performance, no need to evaluate the impact due to BB error and RF error seperately, a single measurement error inlcudes both baseband error and RF error can be considered.

Proposal 6: for measurement error impact evaluation, it is proposed to consider a single error covering both baseband error and RF error. And it is proposed to consider baseband error under different SNR.

Issue 3-9: AoA/AoD - Proposals - Option 1: exclude test setup with single AoA(single probe) - Option 2: further study single AoA setup: - Companies should bring further analysis on relevance of such a test to beam prediction and how it would relate to field deployments - Option 3: other

Acorrding to previous discussion, some companies propose only to verify AI funtionality, since the model input is RSRP. We have different views, not only the functionality but also the prediction performance need to be verified. It is necessary to consider UE RX beam sweeping. From this point of view, multiple AoAs is preferred.

Proposal 7: to verify beam management performance, it is proposed to use multiple AoAs test systems.

Issue 4-2: Channel model - Proposals - Option 1: simplified CDL - Option 2: TDL - Option 3: others

Spatial/time domain prediction require to learn the sufficient spatial/time domain characteristic, which means that sufficient randomness and variation in time and spatial domain in the test is necessary. Compared with TDL, CDL channel provides more spatial characteristics and reflects field conditions accurately, and is more suitable for beam prediction testing. However, CDL channel requires multiple probes, and the test complexity and cost are increasing. Based on above consideration, we are also fine with simplified CDL with small number of clusters.

Proposal 8: for beam prediction testing, it is prefered to consider CDL, and it is also OK with simplified CDL (e.g. simplified CDL with small number of clusters).

- Conclusion

This contribution provides discussion on RRM requirements for beam management. The proposals are:

Proposal 1: it is proposed that the RSRP accuracy requirements apply to all predicted/reported beams.

Proposal 2: the relative RSRP accuracy is proposed as:

- Relative RSRP accuracy = (predicted L1-RSRP of beam index i - predicted L1-RSRP of beam index n) -  (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), where the beam index n owns the largest predicted value.

Proposal 3: it is proposed that the beam ID prediction accuracy is defined as Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

Proposal 4: for measurement period for inference, it is proposed to take legacy L1-RSRP measurement period as baseline.

Proposal 5: for beam management, it is proposed to reuse legacy L1 report mapping, i.e. L1 SS-RSRP part of Table 10.1.6.1-1 and Table 10.1.6.1-2 in TS38.133 are reused for the report of inference results.

Proposal 6: for measurement error impact evaluation, it is proposed to consider a single error covering both baseband error and RF error. And it is proposed to consider baseband error under different SNR.

Proposal 7: to verify beam management performance, it is proposed to use multiple AoAs test systems.

Proposal 8: for beam prediction testing, it is prefered to consider CDL, and it is also OK with simplified CDL (e.g. simplified CDL with small number of clusters).

- Reference

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-243244, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R4-2502856, WF on the requirements for AI/ML air interfance, Qualcomm Incorporated

- R4-2420513, WF on AI/ML air interface part 1, Qualcomm Incorporated

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated

- R4-2414323, WF on AI/ML, Qualcomm Incorporated

- R4-2410570, WF on AI/ML, Qualcomm Incorporated

- R4-2406617, WF on AI/ML, Qualcomm Incorporated

[5] 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface