# 3GPP TSG-RAN WG4 Meeting #114bis                             R4-2503769

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.19.2

Source: 	CMCC

Title: 			 Discussion on CSI prediction

Document for:	Discussion

- Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. In RAN #106 meeting, the revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [2]. The objectives for RAN4 are duplicated as following.

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved [3]. This contribution provides discussion core requirements for CSI prediction.

- Discussion

Summary on RAN4 agreements on CSI prediction: RAN4#112 Issue 4-5: CSI prediction accuracy metric Agreement: Use relative throughput as requirement metric for CSI prediction - companies are invited to bring contribution on how the requirement could be defined: - what is the baseline that throughput would be compared to RAN4#113 Issue 2-1: Channel model for CSI prediction tests Agreement: - For channel model for CSI prediction tests, TDL channel models will be used. - FFS on whether CDL channel model is needed for generalization test. Issue 2-2: Reference throughput Agreement: - reference throughput is obtained based on random PMI with Rel-15 Type I single panel codebook Issue 2-3: Testing conditions Agreement: - use static conditions as baseline. - FFS on whether to have non-static condition RAN4#114 Issue 1-2: Reference model for CSI prediction Agreement: - Assume no reference AI model for CSI prediction as baseline - If it is difficult to align the results corresponding to test metric, RAN4 will discuss the reference model.

According to the revised WID [2], CSI prediction (one-sided model) is one of the objectives for WI, and RAN4 need to specify necessary RAN4 core requirements. In previous meeting, it was agreed to use relative throughput as requirement metric for CSI prediction. Reference throughput is obtained based on random PMI with Rel-15 Type I single panel codebook. For CSI prediction, it was agreed that AI/ML based performance requirement should not be worse than the legacy, and FFS which legacy requirement/feature are relevant.

In last meeting, RAN1 agreed to support Rel-18 codebook ('typeII-Doppler-r18') for inference report.

RAN1 agreements in RAN1#120 meeting Agreement For CSI prediction using UE-side model, at least for inference, Rel-18 CSI framework is reused. - For CSI-RS resource type for CMR, periodic, semi-persistent, and aperiodic CSI-RS are supported. - For inference report, - for N4>=1, support Rel-18 codebook ('typeII-Doppler-r18')

Proposal 1: for CSI prediction, it is proposed to use Rel-18 eType II Doppler codebook.

For testing conditions, it was agreed to use static conditions as baseline, FFS on whether to have non-static condition. Static condition is similar as we have for legacy test. Non-static condition is more suitable to verify the generalization and is benificial in reducing the number of tests. It is good if we could have tests for both static and non-static condition.

However, considering the limited timeline and there are so many issues to be solved, it seems impossible to have parallel discussion on both cases. Especially, non-static conditions are new for RAN4 which will take a long time to discuss the detail, for example, how the channel model and SNR settings are changed over the test. It is preferred to firstly focus on static condition, and non-static condition can be further discussed if time allowed.

Proposal 2: for CSI prediction, it is preferred to firstly focus on static conditions. Considering the timeline and workload, non-static conditions can be discussed later if time allowed .

For channel model for CSI prediction tests, it was agreed that TDL channel models will be used, FFS on whether CDL channel model is needed for generalization test. It is beneficial to use CDL channel model. However, legacy PMI tests are not based on CDL. And the feasibility for demodulation performance requirements with CDL channel model is under discussion in Rel-19 SI. Considering current situation, it is not feasible to consider CDL channel model in Rel-19. Using CDL channel model for CSI prediction test can be further dicsussed in later release based on the outcome of Rel-19 SI on SCM.

Proposal 3: For channel model for CSI prediction tests, it is proposed to focus on TDL channel models in Rel-19. Using CDL channel model for CSI prediction test can be further dicsussed in later release based on the progress on SCM.

In last meeting, the baseline simulation assumptions for AIML based CSI prediction was approved [10]. We provide simulation results for step 1 to check SGCS before CSI feedback (between the raw channel matrix and ground-truth at UE).

Table 1 Agreed simulation assumptions (R4-2502970)

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD, TDD, OFDM |
| Carrier frequency | 2GHz for FDD 4GHz for TDD |
| Bandwidth | 10MHz for FDD, 40MHz for TDD |
| Symbol | 1 symbol |
| Number of subbands | 13 subbands, 4RB as a bundle for FDD 14 subbands, 8RB as a bundle for TDD |
| Subcarrier spacing | 15kHz for 2GHz, 30kHz for 4GHz |
| gNB TX antennas | Option1: 32 (N1,N2)=(4,4) as defined in Rel-15 type1 Option2: 16 (N1,N2)=(4,2) as defined in Rel-18 CSI prediction requirements |
| UE RX antennas | 4 |
| Channel model | TDLA30 as baseline with XP medium correlation |
| Doppler spread | 20Hz, 50Hz and 100Hz |
| Delay spread | 30ns |
| Channel estimation | Start with Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Further consider practical channel estimation in the next step - Use a common CSI-RS configuration as in demodulation test (may need to define/choose a CSI-RS configuration) |
| Rank per UE | Rank 1 and 2 |
| CSI feedback assumption | Step1: check the CSI prediction (outputs Raw channel) performance, do not need to take CSI feedback into account Step2: take CSI feedback into account (wait more related agreements in RAN4) - Option 1: Use Rel-18 eTypeII-Doppler - Companies are encouraged to check RAN1 agreements |

In our simulation, 4GHz for TDD is in use. gNB TX antennas is 16 (N1,N2)=(4,2) as defined in Rel-18 CSI prediction requirements. Doppler spread is 50Hz. Rank1 is assumed. Table 2 is our simulation results.

Table 2 SGCS for CSI prediction

|  | SGCS |
| --- | --- |
| ConvLSTM | 0.9249 |

Proposal 4: for CSI prediction, the evaluation results for step 1 are shown as Table 2.

- Conclusion

This contribution provides discussion on CSI prediction. The proposals are:

Proposal 1: for CSI prediction, it is proposed to use Rel-18 eType II Doppler codebook.

Proposal 2: for CSI prediction, it is preferred to firstly focus on static conditions. Considering the timeline and workload, non-static conditions can be discussed later if time allowed .

Proposal 3: For channel model for CSI prediction tests, it is proposed to focus on TDL channel models in Rel-19. Using CDL channel model for CSI prediction test can be further dicsussed in later release based on the progress on SCM.

Proposal 4: for CSI prediction, the evaluation results for step 1 are shown as following.

|  | SGCS |
| --- | --- |
| ConvLSTM | 0.9249 |

- Reference

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-243244, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R4-2502856, WF on the requirements for AI/ML air interfance, Qualcomm Incorporated

- R4-2420513, WF on AI/ML air interface part 1, Qualcomm Incorporated

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated

- R4-2414323, WF on AI/ML, Qualcomm Incorporated

- R4-2410570, WF on AI/ML, Qualcomm Incorporated

- R4-2406617, WF on AI/ML, Qualcomm Incorporated

- 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- R4-2502970, Baseline Simulation assumptions for AIML based CSI prediction