# 3GPP TSG-RAN WG4 Meeting #114bis                              R4-2503778

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.24.3

Source: 	CMCC

Title: 			 Discussion on RRM requirements for A-IoT

Document for:	Discussion

- Introduction

In RAN #107 meeting, the revised WID on Solutions for Ambient IoT (Internet of Things) in NR was approved [1]. According to the WID, the objectives of RAN4 scope are duplicated as following.

- RAN4 scope: - Specify RF requirements for Ambient-IoT BS, device 1, and CW - RF requirements for Type 1-C Ambient-IoT BS - RF requirements for device 1 - RF requirements for CW - Specify RRM core requirements for device 1, if necessary - Study and develop OTA test methodology for A-IoT device 1 - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT. - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system - Use band n8 as an example band

This meeting is the first meeting to discuss RRM requirements for A-IoT. This contribution provides initial discussion on this topic.

- Discussion

General Scope of Rel-19 WID The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope: - The overall objective shall be to standardize the following Ambient IoT device: Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Deployment scenario 1 with Topology 1, according to D1T1-B. - FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum. - Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. - Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command). - Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B - Proximity determination via Solution 1 in TR 38.769 only. - Device (un)availability via Direction 1 in TR 38.769 only.

According to the general scope of the WID [2], the focus is D1T1-B (Deployment scenario 1 with Topology 1 ), and Ambient IoT device 1 is considered. Deployment scenario 1: Device indoors, basestation indoors. In Topology 1, the Ambient IoT device directly and bidirectionally communicates with a basestation. The communication between the basestation and the ambient IoT device includes Ambient IoT data and/or signalling. For D1T1-B, CW is outside topology.

Figure 1 Diagram of the scenario D1T1-B,

Observation 1: according to the WID, device 1 and D1T1-B are considered.

In last RAN2 meeting, it was agreed that from RAN2 perspective only the following types of procedures will be considered in the normative phase: “Inventory only” and “Inventory and command”.

Agreements in RAN2 #129 meeting From RAN2 perspective only the following types of procedures will be considered in the normative phase: “Inventory only” and “Inventory and command”.

The definition of Command and Inventory are defined in TR23.700. Command: Refers to an instruction sent by an AF to an AIoT Device. Which include Read, Write and Disable. Inventory: Refers to determining the identity of all or a subset of AIoT Devices in range of a reader.

TR23.700 - Command: Refers to an instruction sent by an AF to an AIoT Device. The following instructions may be supported: - Read: Reading data from an AIoT Device; - Write: Writing data to an AIoT Device; - Disable: Disable an AIoT Device temporarily or permanently. - Inventory: Refers to determining the identity of all or a subset of AIoT Devices in range of a reader.

Observation 2: according to RAN2 agreements, only “Inventory only” and “Inventory and command”are considered in Rel-19 WI.

- RAN2 scope of Rel-19 WID - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included. - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

According to the RAN2 scope of the WID [2], A-IoT Paging, A-IoT Random access and A-IoT data transmission are considered. From RRM requirements point of view, A-IoT Random access may have impact on RRM requirements.

The progress on A-IoT Random access procedure during SI is captured in the TR 38.769.

- Propgress on A-IoT Random access procedure in SI phase (TR 38.769) - Step 1: Random access type (i.e., contention-free or contention-based) and access occasion/resource determination: - The A-IoT device determines the random access type from the A-IoT paging message, in accordance to clause 6.3.3. The reader can configure either contention-free access or contention-based random access (and the corresponding configurations). It needs to be further discussed on whether it is explicitly or implicitly. - If the random access is contention-free access: - Selects the indicated D2R occasion/resource; - Skips the contention resolution in Step 2 and performs the data transmission in accordance to clause 6.3.5. - If the random access is contention-based random access: - Performs access occasion/resource selection: as the baseline for CBRA, at least for TDMA case, the device can randomly select one access occasion for A-IoT Msg1 within the access occasions provided/assigned by the reader. It needs to be further discussed if this is applicable to FDMA case. Further enhancement option(s) can be also considered after more physical layer detailed design on TDMA and FDMA; - Performs the Step 2 for contention resolution. - Step 2: Contention resolution of contention-based random access: - There are three candidate solutions being studied for the contention resolution, as below (it needs to be further discussed on the down selection and/or the unified design): - Solution 1: A-IoT Msg1 without data - A-IoT Msg1: When the A-IoT device identifies the start of its own access occasion, it sends one 16-bit random ID generated by the A-IoT device to the reader. - A-IoT Msg2: The reader responds with the successfully received random ID. If the A-IoT device receives the A-IoT Msg2 including a random ID, which is the same as the previously transmitted one in A-IoT Msg1, it considers the contention resolution as successful. - Solution 2: A-IoT Msg1 with data - A-IoT Msg1: When the A-IoT device identifies the start of its own access occasion, it sends the A-IoT Msg1 including the upper layer data, which can be the device ID and/or any other upper layer data, in addition to one 16-bit random ID generated by the A-IoT device to the reader. - A-IoT Msg2: The reader may respond with the successfully received random ID. If the A-IoT device receives the A-IoT Msg2 including a random ID, which is the same as the previously transmitted one in A-IoT Msg1, it considers the contention resolution as successful. If the A-IoT Msg2 is not received by the device, the re-access is not autonomously performed i.e., the re-access is always controlled by the reader. - Solution 3: A-IoT Msg1 optionally includes data (unified solution supporting both Solution 1 and 2) - A-IoT Msg1: When the A-IoT device identifies the start of its own access occasion, it sends one 16-bit random ID generated by the A-IoT device to the reader. In addition, it is controlled by the reader on whether to also include the upper layer data, which can be the device ID and/or any other upper layer data. - A-IoT Msg2: The reader responds with the successfully received random ID. If the A-IoT device receives the A-IoT Msg2 including a random ID, which is the same as the previously transmitted one in A-IoT Msg1, it considers the contention resolution as successful.

RAN2 agreements in last meeting are duplicated as following. It can be seen that both contention-free RACH and contention-based RACH are supported. And only 3-step CBRA is supported for A-IoT. In legacy, requirements for random access are specified in TS 38.133. Similar approach can be considered for A-IoT. But for the details on how to specify the RRM requirements for A-IoT random access can be further discussed pending on RAN2 progress.

Agreements in RAN2 #129 meeting - For Rel-19, only 3-step CBRA is supported for A-IoT - We will specify both CBRA and CFRA. - Re-use the subsequent paging message to trigger re-access. There is no need to differentiate msg1 resource for initial access vs re-access.

Proposal 1: it is proposed to define RRM requirements for A-IoT random access procedure.

- Conclusion

This contribution provides discussion on UE-to-UE CLI handling. The proposals are:

Observation 1: according to the WID, device 1 and D1T1-B are considered.

Observation 2: according to RAN2 agreements, only “Inventory only” and “Inventory and command”are considered in Rel-19 WI.

Proposal 1: it is proposed to define RRM requirements for A-IoT random access procedure.

- Reference

- RP-250796, Revised WID: New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA Inc