3GPP TSG-RAN WG4 Meeting #114bis	R4-2503786

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.19.3

Source: 		NTT DOCOMO, INC.

Title: 	Discussions on RRM core requirement for beam management

Document for:	Discussion

- Introduction

During the past meetings, requirements for beam management has been discussed and several agreements were obtained. This paper further discusses remaining issues in the summary [1] and WF [2].

- Discussion

  - Beam prediction KPIs

During past meetings, the metrics/KPIs for beam prediction has been extensively discussed. In last meeting, the discussion point was separated into two aspects, one is RSRP value, the other is beam ID. The candidate options are copied in below.

Issue 2-1: Metrics/KPIs for beam prediction If RSRP and beam ID are reported: - RSRP accuracy will be one of the KPIs. - How option 1 is applied is FFS - Beam ID prediction accuracy will be one of the KPIs - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options If only beam ID is reported: - Beam ID prediction accuracy will be used as the KPI - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options

In order to clarify the options stated in each candidate, the issue is modified as follows.

If RSRP and beam ID are reported: - RSRP accuracy will be one of the KPIs. - How to apply is FFS - Beam ID prediction accuracy will be one of the KPIs - Option 1: - Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" - Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" - Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - FFS combination of Top-K/1 and Top 1/K - Option 2: - The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - FFS whether beam ID prediction accuracy refers to a combination of Option 1 and/or Option 2 or modified Option 1/2 or other options If only beam ID is reported: - Beam ID prediction accuracy described above will be used as the KPI

For RSRP accuracy, it should be defined as absolute RSRP accuracy of predicted beam. The legacy absolute RSRP accuracy is defined as the acceptable difference between actual RSRP value and measured RSRP value. By reusing same strategy for predicted RSRP, it can be defined as the acceptable difference between ground truth of the predicted beam and predicted RSRP value. Since prediction can include hardware characteristics, implementation margin is not necessary. Thus the requirement can be tightened than current absolute RSRP accuracy.

Proposal 1: RSRP accuracy of beam prediction is defined as the acceptable difference between ground truth of the predicted beam and predicted RSRP value. The ground truth of the predicted beam is the ideal measurement of RSRP on the predicted Tx beam.

Proposal 2: the requirement should be tightened than current absolute RSRP accuracy because prediction can include hardware characteristics, such as implementation margin.

For beam ID prediction accuracy, option 2 can include option 1 and also combination of Top-K/1 and Top-1/K. Thus option 2 should be the KPI. In order to guarantee optimality of the selected beam, x should be less than 3. Under this condition, if the ground truth of top 1 predicted beam is not the strongest beam, it may be used as usable beam for the UE.

Figure 1: Relationship between ground truth and predicted value (K = 2 case)

Proposal 3: Beam ID prediction accuracy should be the successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB and x should be less than 3.

- Conclusion

In this contribution, we proposed our views on KPIs for beam prediction.

Proposal 1: RSRP accuracy of beam prediction is defined as the acceptable difference between ground truth of the predicted beam and predicted RSRP value. The ground truth of the predicted beam is the ideal measurement of RSRP on the predicted Tx beam.

Proposal 2: the requirement should be tightened than current absolute RSRP accuracy because prediction can include hardware characteristics, such as implementation margin.

Proposal 3: Beam ID prediction accuracy should be the successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB and x should be less than 3.

References

[1] 3GPP, R4-2500683, “Topic summary for [114][131] NR_AIML_air_part1”, Qualcomm Incorporated

[2] 3GPP, R4-2502856, “WF on the requirements for AI/ML air interfance” , Qualcomm Incorporated