3GPP RAN WG4 Meeting #114bis		R4-2503812

Wuhan, China, April 7th – 11th, 2025

Agenda item:	8.1

Source:	Apple

Title:	Views on RAN2 reply LS on UE capability for FDD-FDD inter-band CA simultaneous Rx-Tx

WI/SI:	NR_CADC_SUL_R19

Release:	Rel-19

Document for:	Approval

# 1	Introduction

In response to RAN4’s LS on “UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx” [1], RAN2 replied with their agreement as: RAN2 understands that for the example band the 2UL/2DL inter-band FDD-FDD CA_n5-n8 with the scheduling restriction of non-concurrent n5 DL and n8 UL is feasible, and there is no RAN2 spec impact. However, regarding whether to define UE capability signalling, RAN2 didn't reach consensus due to the lack of details on simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8. To gain a better understanding on the feature, RAN2 also raised two questions to inquire RAN4’s clarifications.

In this contribution, we share our views on the restricted scheduling to enable 2UL/2DL operation for FDD-FDD combination with overlapping cross UL and DL bands. Considering the resemblance to the operation scenarios for LB-LB CA via switching feature [2], we propose to postpone the CA_n5-n8 2U/2DL configuration to Rel-20 as one of the new LB-LB CA switching scenarios in the potential Rel-20 5G-A work item for LB-LB CA via switching.

# Discussion

CA_n5-n8 was first introduced in Rel-18. Due to that the n5 DL band range is overlapping with n8 UL band range, there is no feasible full-band filter to isolate n5 DL and n8 UL. As a result, when n8 UL and n5 DL are operating simultaneously, severe sensitivity degradation in n5 DL carrier is expected. Though during the Rel-18 discussions, there were three operation configurations as below considered to mitigate the n8 UL to n5 DL self-interference issue:

- Option 1: DL CA_n5-n8 with single UL in n5 only

- Option 2: UL/DL CA_n5-n8 with non-simultaneous Rx/Tx between n5 DL and n8 UL

- Option 3: UL/DL CA_n5-n8 with additional dedicated filter for the restricted frequency ranges for specific operator

Due to the potential RAN1 and RAN2 impact for Option 2 and the UE implementation/performance challenge in Option 3 [3], only Option 1 operation was specified in Rel-18.

Nevertheless, owing to the demand for better UL throughput performance, the CA_n5-n8 proponent company has proposed to revisit Option 2 in Rel-19 via a basket WID, with the assumption that there would be no RAN1 and RAN2 impact, or at most only certain capability signaling needs to be introduced to support the feature. However, in our view, the assumption of no RAN1 impact was an overlook in RAN4, and that is also the reason why RAN2 indicated the lack of details on this feature in the reply LS.

Observation 1: The assumption of no RAN1 impact to support 2UL/2DL CA_n5-n8 with non-simultaneous Rx/Tx between n5 DL and n8 UL was an overlook in RAN4.

Now by comparing Rel-19 LB-LB CA via switching feature with 2UL/2DL CA_n5-n8 with non-simultaneous Rx/Tx between n5 DL and n8 UL, we have found the strong resemblance between the two features and the alternating configurations between 2UL/1DL and 1UL/2DL for CA_n5-n8 is essentially another LB-LB CA switching scenario which should be subject to RAN1’s provision before the feature can be enabled. With that said, we think the intended 2UL/2DL CA_n5-n8 with non-simultaneous Rx/Tx between n5 DL and n8 UL feature should not be introduced via basket WID and propose to postpone the CA_n5-n8 2U/2DL configuration to Rel-20 as one of the new LB-LB CA switching scenarios in the potential Rel-20 5G-A work item for LB-LB CA via switching.

Observation 2: There is a strong resemblance between Rel-19 LB-LB CA via switching feature and 2UL/2DL CA_n5-n8 with non-simultaneous Rx/Tx between n5 DL and n8 UL.

Observation 3: The alternating configurations between 2UL/1DL and 1UL/2DL for CA_n5-n8 is essentially another LB-LB CA switching scenario which should be subject to RAN1’s provision before the feature can be enabled.

Observation 4: The intended 2UL/2DL CA_n5-n8 with non-simultaneous Rx/Tx between n5 DL and n8 UL feature should not be introduced via basket WID.

Proposal: Postpone the CA_n5-n8 2U/2DL configuration to Rel-20 as one of the new LB-LB CA switching scenarios in the potential Rel-20 5G-A work item for LB-LB CA via switching.

# 3	Conclusion

In this contribution, we share our views on the restricted scheduling to enable 2UL/2DL operation for FDD-FDD combination with overlapping cross UL and DL bands and propose to postpone the CA_n5-n8 2U/2DL configuration to Rel-20 as one of the new LB-LB CA switching scenarios in the potential Rel-20 5G-A work item for LB-LB CA via switching.

Observation 1: The assumption of no RAN1 impact to support 2UL/2DL CA_n5-n8 with non-simultaneous Rx/Tx between n5 DL and n8 UL was an overlook in RAN4.

Observation 2: There is a strong resemblance between Rel-19 LB-LB CA via switching feature and 2UL/2DL CA_n5-n8 with non-simultaneous Rx/Tx between n5 DL and n8 UL.

Observation 3: The alternating configurations between 2UL/1DL and 1UL/2DL for CA_n5-n8 is essentially another LB-LB CA switching scenario which should be subject to RAN1’s provision before the feature can be enabled.

Observation 4: The intended 2UL/2DL CA_n5-n8 with non-simultaneous Rx/Tx between n5 DL and n8 UL feature should not be introduced via basket WID.

Proposal: Postpone the CA_n5-n8 2U/2DL configuration to Rel-20 as one of the new LB-LB CA switching scenarios in the potential Rel-20 5G-A work item for LB-LB CA via switching.

# 4	References

R4-2503120 “Reply LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx”, RAN2, 3GPP TSG-RAN WG4 Meeting #114bis, Wuhan, China, April 7th – 11th, 2025

RP-243317, “New WID on low band carrier aggregation via switching”, Apple et al, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9th -12th, 2024

RP-231970 “Rescoping CA_n5-n8 in UL CA_n5-n8 support”, Apple, 3GPP TSG-RAN Meeting #101, Bengaluru, India, September 11th – 15th, 2023