# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503827

Wuhan, Hubei, China, 07th – 11th April, 2025

Source: 	vivo

Title: 	Discussion and reply LS on AdditionalSpectrumEmission on Sidelink

Agenda Item:	8.2

Document for:	Approval

Introduction

An LS [1] was sent to RAN2 on the support AddtionalSpectrumEmission in NR SL pre-configuration. A received LS [2] was received from RAN2, asking if the new NS value signaling should be signaled both in SIB and in dedicated signaling. In this contribution, we provide our view on the issue and draft reply LS to RAN2.

Discussion

The content of the response LS [2] from RAN2 was captured as follows:

# 1. Overall description RAN2 would like to thanks RAN4 for the LS on additionalSpectrumEmission. In the LS, RAN4 ask RAN2 to introduce ‘AdditionalSpectrumEmission’ for NR SL Pre-configuration. However, from RAN2 point of view, ‘AdditionalSpectrumEmission’ also needs to be introduced in SIB and dedicated RRC signaling, which has been supported in LTE SL. RAN2 would like to ask RAN4 to provide feedback on following question: - If RAN4 thinks that the new NS value(“AdditionalSpectrumEmission”) signalling should be signalled both in SIB and in dedicated signalling? if the answer is no, what is the reason for this? # 2. Actions: To RAN4: RAN2 respectfully requests RAN4 to provide feedback on the question above.

From our perspective, this IE AdditionalSpectrumEmission is used to indicate emission requirements to be fulfilled by NR sidelink UE. It is for regulatory compliance. We think it should be supported in any case by pre-configuration or indicated by network, which includes SIB or dedicated RRC signalling. This is also aligned with LTE SL.

Therefore, to RAN2’s question, yes. Both in SIB and in dedicated signaling should be supported.

Proposal 1: Reply to RAN2 that the new NS value (“AdditionalSpectrumEmission”) signalling should be signalled both in SIB and in dedicated signalling.

Conclusion

This contribution discusses the new NS value signalling for NR Sidelink. The following proposal is made:

Proposal 1: Reply to RAN2 that the new NS value (“AdditionalSpectrumEmission”) signalling should be signalled both in SIB and in dedicated signalling.

- References

[1] R4-2418075, LS on AdditionalSpectrumEmission in NR SL Pre-configuration, RAN4#113.

[2] R4-2503113_R2-2501383, Reply LS on AdditionalSpectrumEmission, RAN2#129.

# Annex

# 3GPP TSG-RAN WG4 Meeting # 114bis 	R4-250XXXX

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title:	Reply LS on additionalSpectrumEmission on Sidelink

Response to:	R4-2503113(R2-2501383)

Release:	Release 18

Work Item:	NR_SL_enh2-Core

Source:	RAN4

To:	RAN2

Cc:	-

Contact Person:

Name: -

E-mail Address: -

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN4 would like to thank RAN2 for the response LS in R2-2501383.

RAN4 has discussed new NS value(“AdditionalSpectrumEmission”) signalling for NR sidelink, and it should be signalled both in SIB and in dedicated signalling.

2. Actions:

To RAN2:

ACTION: RAN4 respectfully asks RAN2 to take the above information into account for the signalling design for NR sidelink.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #115	19 – 23 May 2025           Malta, MT

TSG-RAN WG4 Meeting #116	25 - 29 Aug. 2025           India (TBC), IN