3GPP TSG-RAN4 Meeting #114bis	R4-2503836

Wuhan, China, April 7 – 11, 2025

Agenda item:			7.6.3

Source:	vivo

Title:	Impacts on UE RF requirements and DL performance

Document for:	Discussion

- Introduction

In RAN4#114 meeting, the WF[1] has been agreed as follows, and this contribution will discuss about the remaining open issues.

#### Issue 3-1: Discussion on maximum input level for fragmented carriers - Maximum power level including all RF signals and interference at the UE input does not exceed -25 dBm #### Issue 3-2: Discussion of impacts on DL performance for UE enabling one Rx RF chain mode - Relaxation on REFSENS for both PCC and SCC will be evaluated and captured in the study outcome report. - DL performance degradation can be considered based on configurations - Configurations leading to no relaxation are not precluded pending on the evaluation results. The exact values and applicable conditions shall be decided in the WI phase if there is corresponding WI - Maintain the table format agreed in previous RAN4 meeting. Detailed evaluation assumptions such as imbalanced PRx/DRx noise level and other assumptions are encouraged to be provided from companies evaluation TPs #### Issue 2-6: Baseband assumption We do not need new agreements. Refer to WF R4-24020519 #### Issue 3-3: Discussion on in-gap interference requirements - For image of in-gap interference not overlap with wanted CC: Evaluate whether relaxation on ACS/IBB is needed based on evaluated ΔRIBNC on PCC and SCC. - For image of in-gap interference(ACS/IBB) overlap with wanted CC: companies are encouraged to provide the impact of interference to wanted signal power ratio of [15-26]dB to the ΔRIBNC for PCC and SCC. #### Issue 3-4: Discussion on DL fragmented carriers in TDD bands - Option 2: Separate LOs are assumed for UL and DL for TDD band Open issues: ### How to indicate UE capability for supporting ”one Rx RF chain” for DL fragmented carriers Option 1: Legacy band combination reporting scheme + Rx sharing capability indication - FFS on whether the capability is specified as ”per-band per-BC” or “per CC per band per BC”. Other options are not precluded ### Discussion on semi-statically switch hardware resources procedure and fallback considerations ### Triggering condition to enable ”One RF Rx chain” FFS: - FFS on whether the UE indication solution is needed or not, with understanding that enabling one Rx chain is under the control of network. - Triggering condition for UE working in one Rx RF chain is needed - Triggering condition for UE working in legacy Rx RF chains is needed - The switching should be not be dynamic and be under network control with UE indication of condition. ### Discussion on applicability for PCell and SCell Discussion on the number of Rx and DL MIMO layer configuration for the UE supporting ”one Rx RF chain” mode

- Discussion

## 2.1. DL performance test of CA_n25(2A)

### General idea on testing

Testing should be a very useful way to verify the current performance of the RF chain. However, the difficulty is that the current UE implementation doesn’t have the “sharing” assumptions. For the UE actually implemented intra-band NC CA, the separate RF chains would be used, thus we cannot verify a UE that actually support CA_n25(2A) under shared RF chain.

However, we can use the single carrier configuration that configures a large channel bandwidth to mimic the “sharing condition”. In this situation, a large bandwidth can be configured, while only a part of RBs were actually scheduled, to mimic the NC CA situation. By comparison, the performance of the small channel bandwidth can be used to mimic the separated RF chain implementation.

In addition, the constraints of system simulators have to be considered. Currently, there are no non-contiguous allocation test for demodulation, and system simulators also do not support such non-contiguous configuration, though the physical layer spec support. In order to overcome this difficulty, separate testing of two different blocks (which is used to mimic two difference CCs in NC CA) is used.

In this way, the whole tests became feasible with a UE supporting n25 single CC operation and a normal system simulator.

Observation 1: Single carrier configuration with two blocks used respectively can be used as a way to verify the Fragmented carrier performance for sharing RF chain for NC CA.

### Bandwidth Configuration Description

Among the several example bands: n2, n25, n3, n7, n66, n41, n39, and n26, in this paper CA_n25(2A) is chosen as the main case to test the DL performance with RX chain sharing between the two DL CCs. Table1 shows the specific configuration: Two DL signals have the same receiving bandwidth of 5MHz, which is the simplest configuration considering the impact of UL and blocker on the DL performance. When the bandwidth of the two DL CCs is asymmetric the impact evaluation would be more complex, which will be studied in the future.

As can be seen from the yellow row marked under the CA_n25(2A) configuration in Table 1, the entire DL bandwidth in sharing state is 40MHz (SCC:5M+Wgap:30M+PCC:5M). Therefore, a 40M wideband single-carrier DL signal is set to represent the total sharing RX CBW, where the RB configuration in the upper or lower edge of the signal represents the location of the DL CC in the sharing RX CBW. Only one of the PCC and SCC is configured at a time, and the DL performance of the two DL CCS is observed alternately.

Also, for the uplink a 40M broadband signal with a full RB bandwidth of 5M is configured to simulate the actual 5M UL CC, with 80M frequency separation between the center of UL PCC RB allocation and the DL PCC RB allocation. UL RBs are allocated at the higher edge of the broadband 40M signal to keep the image product as far away from the DL signal as possible. It is a simplified configuration setup scheme from the test point of view, which will inevitably introduce some errors. All the reference sensitivity results are based on single antenna measurement. The following test results are all based on this configuration assumption and provide corresponding observations just from our point of view.

Fig1. configuration of CA_n25(2A) in testing

Table1. Power class 3 intra-band non-contiguous CA reference sensitivity with one uplink

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | SCC ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n25(2A) 9 | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 105 | 5.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |
| CA_n25(2A) 10 CA_n25(3A) | 15/15 | 40MHz + 5MHz | Wgap = 20.0 | 40 (RBstart = 176) | [24.6] 8 | FDD |

### Test connection schematics

Fig2. connection schematic diagram of testing

As can be seen in the connection schematic diagram, the directional property of the directional coupler is used to prevent the incoming blocker signal from entering the Radio Communication Tester, and the duplexer is set to prevent TX high-power signals from entering into the signal source and causing alterations to the signal. The attenuation of the inserted devices has been taken into account.

### Test Case Description & Results & Analysis

As shown in [2], there are 7 test cases in total, of which cases 1 to 6 have one PCC and one SCC test result each, while test case 7 has only PCC test results. Each test case is divided into two sub-test cases, i.e., the UL signal is set to the highest and lowest power to test the demodulation performance of DL CC respectively. Case 1&2 represent scenarios with only TX interference. While case 3~7 represent the scenarios interfered by both TX signal and in-gap blocker, and the blocker is configured at different frequency points with different power assumption.

The results and configurations are also copied below and the details could be seen in [2], in this paper only case 1 and 7 is further analyzed as below.

Fig 3. Test configuration & REFSENS results of CA_n25(2A)

## 2.2 Further analysis of TX leakage and image overlapping issues

### Test result of Case 1, only considering TX interference:

Only Tx leakage effects are considered is the baseline case. The UL CC is also configured with full RB of 5MHz: 25RB inside 5MHz. The sharing RX chain is 40M with 5M RB allocation to mimic PCC and SCC.

  - Compared to separate RX chain with 5M: With the highest UL output power, the REFSENS of PCC and SCC is deteriorated by 8.9dB and 8.8dB separately. With the lowest UL output power, the REFSENS of PCC and SCC is deteriorated by 8.8dB and 8.7dB separately.

  - Compared to separate RX chain with 40M: With the highest and lowest UL output power, the REFSENS of PCC is deteriorated by 0.1dB and 0.3dB separately.

Fig4. configuration of case 1&2

Even limited by the current device conditions the actual shared RX chain could not be implemented currently, configuring a large bandwidth with two part of NC RBs scheduled is considered as an alternative scheme to mimic the NC CA situation in one RX RF chain. However, the alternative scheme should be the optimal state which mimics the most complete sharing receiving procedure with not only analogy part but also digital part.

In hence, from the above testing results of case 1 with only TX interference considered, it could be seen that the REFSENS of both PCC&SCC for sharing chain(40M CBW with 5M RB allocation) is about 9dB worse than separate chain of 5M CBW with full RB allocation, but only about 0.2dB worse than separate chain of 40M.

When the sharing chain is as wide as 40MHz, the noise would be integrated over the entire 40M CBW so the REFSENS would be evaluated based on 40M rather than 5M. We also did some further tests based on different RB configurations within the same /different bandwidth, and the results show that REFSENS is strongly correlated with CBW and weakly correlated with RB allocation.

Observation 2: REFSENS is strongly correlated with CBW and weakly correlated with RB allocation.

It is foreseeable that if it is possible to perform a separate FFT for each CC, the evaluation of REFSENS may be more accurate. Based on the analysis above, sharing the digital portion may introduce significant deterioration of REFSENS compared to the separate state, and if the deterioration is not acceptable then the digital part sharing should be treated with caution.

Observation 3: Sharing the digital part may lead to significant deterioration of REFSENS, probably due to the excess noise introduced by the large bandwidth of the sharing chain.

Proposal 1: Whether the significant deterioration of REFSENS for the shared RX chain could be solved by independent reception of the digital part needs further discussion.

### Test result of Case 7, TX interference and blocker are both considered:

Both TX leakage and blocker are considered, the blocker is configured at 5MHz at the DL SCC position to simulate a scenario where its image overlaps the DL PCC just exactly, and the power is set to -65dBm (REFSENS +31.5) or -25dBm (highest power). Only the demodulation performance of PCC is observed in the two sub cases.

  - Compared to separate RX chain with 5M: With the highest UL output power, the REFSENS of PCC is deteriorated by 40.6dB. With the lowest UL output power, the REFSENS of PCC is deteriorated by 41.5dB.

  - Compared to separate RX chain with 40M: With the highest UL output power, the REFSENS of PCC is deteriorated by 31.6dB. With the lowest UL output power, the REFSENS of PCC is deteriorated by 32.5dB.

  - When the blocker’s image overlaps with target CC, image rejection doesn’t seem to work.

  - When the blocker’s power = -25dBm, PCC can't demodulate properly and the connection is hard.

  - When the image of the blocker overlaps the DL CC just exactly, the maximum power of the blocker that can be accepted to ensure proper demodulation of the DL signal is about -35 dBm.

Fig5. Configuration of case 7

Observation 4: For one RX RF chain with part of RB allocation on each side to mimic the case of blocker’s image overlapping PCC, no image rejection effect is observed.

Checking the contributions of the past few meetings it could be found that, most analyses are based on one assumption that the image rejection does work, i.e., the blocker’s image power is lower than the blocker by 25dB. However, image rejection is a concept from the LTE period, and perhaps has a different role than it does in fragmented carrier nowadays.

In 36.101-1, receiver image rejection requirement is described as follows and specified as 25dB. Even the wording has been changed for several times, the parameter itself is actually related imperfections factors of receiver such as component mismatch in down-conversion mixers and analog baseband filters and the quality of quadrature signals from the local oscillator.

TS36.101-1:

As analysed in [3], the unbalanced LO would introduce two tones at frequencies -fLO and +fLO, and if receiving by a direct conversion receiver the LO should be placed in the middle of the channel. After down-mixing there would be useful information at both sides of the zero frequency. Due to imbalanced LO, spectral components from both positive and negative carriers are overlapping and causing the “image” problem. So, the image rejection metric actually characterizes the amount of suppression of the image product generated by the useful signal itself in the presence of a non-ideal LO.

Observation 5: The image of one single carrier in a direct-conversion receiver arises from the unbalanced performance of the IQ signal of LO, the image product would not appear with an ideal LO.

Considering a more realistic unbalanced local oscillator, this can be represented by the signal where Figure: Direct conversion receiver with an imbalanced local oscillator[3] Classically, the image rejection is given as the ratio between the squared amplitudes of the original component and the image signal, e.g. for figure below & left, IRR can be evaluated as

It is similar for intra-band C CA, the LO needs to be configured in the centre of the two C CCs, and actually the receiver should belong to the low-IF architecture at this time. Then the two CCs will overlap after down-conversion, i.e., the image signal of CC1 will cover CC2, and the image power of CC1 is the same as that of CC1. However, when the LO is ideal enough, the SCC and PCC signals can be completely separated by shifting the I signal by 90 degrees, and summing/subtracting it with the Q signal, respectively, and vice versa. Therefore, when the power difference between PCC and SCC could be controlled within a proper range to ensure that the ADC is in a reasonable dynamic range, the image problem can be perfectly suppressed.

Fig6: After down conversion, two C CCs would overlap each other

Fig7: After phase-shifting and sum/subtract of I/Q, two C CCs could be separated

Observation 6: In intra-band contiguous CA, the LO should be centered between the two carriers. Under ideal LO conditions, image interference between the two carriers can be fully eliminated through phase-shifting and sum/subtract of I/Q signals.

However, when the LO is non-ideal there will be a certain power and phase imbalance between the I and Q circuits. This leads to the failure of the image suppression process of phase-shifting and sum/subtract of I/Q signals, i.e., the PCC signal will definitely carry a portion of the image component of the SCC, and so will the SCC signal.

At the same time, the unbalanced power between the two signals will lead to increased image interference. In this case, if the amplitude imbalance of I/Q and the amplitude imbalance between the two signals can be controlled within a certain range, then the power ratio between the CC1 signal and the mirror interference generated by CC2 at CC1 can be maintained at a high level to ensure normal demodulation. The Image rejection index is derived from this and is used to measure the linearity of I/Q of the receiving link on the premise that the two target CC power is the same.[4]

Observation 7：When the LO is non-ideal, the image of intra-band C CCs cannot be perfectly suppressed through the I/Q signal processing of Observation 6. Therefore, the image rejection metric must be introduced to constrain the I/Q imbalance of the LO.

Observation 8: The 25dB image rejection requirement in LTE is based on the assumption of two continuous carriers with equal power and verified under 6 dB power imbalance between CCs.

Figure: Down conversion and image rejection schemes with finite IRR.

When the blocker appears at the image position of the wanted signal, the receiver also works under low-IF operation, and image suppression is needed. However, there are several differences compared to contiguous CA:

- The power of the blocker may be much higher than the wanted signal, which makes the image suppression more difficult.

- UE doesn’t know the exact position of the blocker or when to perform the suppression.

Considering all these differences, it is questionable whether the 25dB IRR can still be achievable in the fragmented carrier case, and we would like to hear more analysis from the chip vendor perspective.

Proposal 2: It is suggested to revisit the 25 dB IRR capability and check whether such value is still achievable or enough in one RX RF chain mode, considering higher power and unpredictable position of the blocker.

As for the following test results of case7 for CA_n25(2A), since the test case is based on a single-carrier non-contiguous RB configuration to simulate the position of the target CC and the blocker respectively, which does not exist in the current spec. Therefore, the receiver will not assume the image exist and actively trigger the image suppression process, including I/Q signals’ phase-shifting and sum/subtract. This is also the reason why there is no effect of image rejection in the test results.

In all, the position of blocker is one of the vital factors to decide whether to perform sharing RX chain, if under one RX RF chain mode and the blocker’s image overlaps with the target CC then the procedure of image suppression should be triggered, or else the image product would become rather high to prevent effective demodulation of target CCs. In addition, since the receiver only has a finite image rejection capability, the power of the blocker is also need to be estimated to ensure the wanted signal degradation is acceptable. Therefore, the measurement of the in-gap blocker’s position and its power are necessary.

Proposal 3：It is necessary to evaluate the position and power of the blocker and both of them should be taken as trigger conditions—when the blocker's image overlaps with the target CC, the phase-shifting-based image suppression procedure must be triggered; otherwise, RX chain sharing could not be performed.

- Conclusion

Observation 1: Single carrier configuration with two blocks used respectively can be used as a way to verify the Fragmented carrier performance for sharing RF chain for NC CA.

Observation 2: REFSENS is strongly correlated with CBW and weakly correlated with RB allocation.

Observation 3: Sharing the digital part may lead to significant deterioration of REFSENS, probably due to the excess noise introduced by the large bandwidth of the sharing chain.

Proposal 1: Whether the significant deterioration of REFSENS for the shared RX chain could be solved by independent reception of the digital part needs further discussion.

Observation 4: For one RX RF chain with part of RB allocation on each side to mimic the case of blocker’s image overlapping PCC, no image rejection effect is observed.

Observation 5: The image of one single carrier in a direct-conversion receiver arises from the unbalanced performance of the IQ signal of LO, the image product would not appear with an ideal LO.

Observation 6: In intra-band contiguous CA, the LO should be centered between the two carriers. Under ideal LO conditions, image interference between the two carriers can be fully eliminated through phase-shifting and sum/subtract of I/Q signals.

Observation 7: When the LO is non-ideal, the image of intra-band C CCs cannot be perfectly suppressed through the I/Q signal processing of Observation 6. Therefore, the image rejection metric must be introduced to constrain the I/Q imbalance of the LO.

Observation 8: The 25dB image rejection requirement in LTE is based on the assumption of two continuous carriers with equal power and verified under 6 dB power imbalance between CCs.

Proposal 2: It is suggested to revisit the 25 dB IRR capability and check whether such value is still achievable or enough in one RX RF chain mode, considering higher power and unpredictable position of the blocker.

Proposal 3: It is necessary to evaluate the position and power of the blocker and both of them should be taken as trigger conditions—when the blocker's image overlaps with the target CC, the phase-shifting-based image suppression procedure must be triggered; otherwise, RX chain sharing could not be performed.

- References

- R4-2502873, WF on NR FR1 DL fragmented carriers study, MediaTek Inc. RAN4#114.

- R4-2500727, Impacts on UE RF requirements and DL performance, vivo, RAN4#114.

- R4-103677, Image Rejection in intraband carrier aggregation, Nokia, Nokia Siemens Networks, RAN4 AH#4.

- R4-102976, Considerations on retuning for deactivated SCell and related implications, Nokia Corporation, RAN4#56.

# ANNEX: Test configuration & REFSENS results of CA_n25(2A) in 114 meeting

Table 2. Test configuration & REFSENS results of CA_n25(2A) in 114 meeting

| ID | DL Allocation |  |  |  |  |  |  |  |  | Interference |  |  |  |  |  |  | Output |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Configuration / CBW |  |  |  |  |  |  |  |  | UL Allocation |  | blocker |  |  |  |  | sharing RX chain (40M single carrier with 5M RB allocation represents each CC) |  | separate RX chain (baseline, 5M) |  | deterioration value of shared chain compared to separate chain |  |
|  | Equivalent Single Carrier ( Fc_DL = 1950 MHz, Fc_UL = 1870 MHz, BW = 40 MHz ) |  |  |  |  | Block1 Equivalent BW (MHz) | Equivalent Wgap (MHz) | Block2 Equivalent BW (MHz) | SCS (kHz) | PCC |  | Allocation |  |  | P_blocker |  |  |  |  |  |  |  |
|  | Band | Block1 (Equivalent PCC) |  | Block2 (Equivalent SCC) |  |  |  |  |  | RB allocation (LCRB @ RBstart) | Range | Equivalent Range | BW (MHz) | Channel Center Frequency (MHz) | P_blocker=PCC_REFSENS+∆P | P_blocker=SCC_REFSENS +∆P | REFSENS of Block1 with max/min UL power (Equivalent PCC) | REFSENS of Block2 with max/min UL power (Equivalent SCC) | REFSENS of Block1 with max/min UL power (Equivalent PCC) | REFSENS of Block2 with max/min UL power (Equivalent SCC) | △REFSENS of Block1 with max/min UL power (Equivalent PCC) | △REFSENS of Block2 with max/min UL power (Equivalent SCC) |
|  |  | RB allocation | Fc(MHz) | RB allocation | Fc(MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for a CA_n25(2A) Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1 | n25 | 25@191 | Note: Center approximate 1967.5 | - | - | 5 | 30 | - | 15 | 25@191 | Note: Center approximate 1887.5 for equivalent 5MHz allocation | - | - | - | - | - | -92.9/-93.8 | - | -101.8/-102.6 | - | 8.9/8.8 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -92.8/-93.5 (Note2) |  |  |  |
| 1.2 | n25 | - | - | 25@0 | Note:Center approximate 1932.5 | - | 30 | 5 | 15 |  |  |  |  |  |  |  | - | -93.1/-93.7 | - | -101.9/-102.4 |  | 8.8/8.7 |
| 2.1 | n25 | 25@191 | Note: Center approximate 1967.5 | - | - | 5 | 30 | - | 15 | 10@206 |  | - | - | - | - | - | -92.6/-93.8 | - | -101.8/-102.6 | - | 9.2/8.8 |  |
| 2.2 | n25 | - | - | 25@0 | Note: Center approximate 1932.5 | - | 30 | 5 | 15 |  |  |  |  |  |  |  | - | -93/-93.7 | - | -101.9/-102.4 |  | 8.9/8.7 |
| 3.1 | n25 | 25@191 | Note: Center approximate 1967.5 | - | - | 5 | 30 | - | 15 | 25@191 |  | Note: Adjacent to block1； Close to 25@163 | 5 | 1962.2 | -65 （-96.5+31.5） | - | -92.3/-93.1 | - | -101.5/-102.2 | - | 9.2/9.1 |  |
| 3.2 | n25 | - | - | 25@0 | Note: Center approximate 1932.5 | - | 30 | 5 | 15 |  |  |  |  |  |  |  | - | -92.9/-93.6 | - | -101.7/-102.1 |  | 8.8/8.5 |
| 4.1 | n25 | 25@191 | Note: Center approximate 1967.5 | - | - | 5 | 30 | - | 15 | 25@191 |  | Note: Adjacent to block1； Close to 25@163 | 5 | 1962.2 | -81.5 (-96.5+15) | - | -92.7/-93.7 | - | -101.5/-102.3 | - | 8.8/8.6 |  |
| 4.2 | n25 | - | - | 25@0 | Note: Center approximate 1932.5 | - | 30 | 5 | 15 |  |  |  |  |  |  |  | - | -92.9/-93.6 | - | -101.7/-102.2 |  | 8.8/8.6 |
| 5.1 | n25 | 25@191 | Note: Center approximate 1967.5 | - | - | 5 | 30 | - | 15 | 25@191 |  | Note: Adjacent to block1； Close to 25@163 | 5 | 1962.2 | -25 (maximum power) | - | -64.9/-63 (Note1) | - | -73.1/-73.6 (Note1) | - | 8.2/10.6 |  |
| 5.2 | n25 | - | - | 25@0 | Note: Center approximate 1932.5 | - | 30 | 5 | 15 |  |  |  |  |  |  |  | - | -64.5/-63 | - | -99.3/-99.6 |  | 34.8/36.6 |
| 6.1 | n25 | 25@191 | Note: Center approximate 1967.5 | - | - | 5 | 30 | - | 15 | 25@191 |  | Note: Adjacent to block2； Close to 25@25 | 5 | 1937.8 | - | -65 （-96.5+31.5） | -92.8/-93.7 | - | -101.5/-102.1 | - | 8.7/8.4 |  |
| 6.2 | n25 | - | - | 25@0 | Note: Center approximate 1932.5 | - | 30 | 5 | 15 |  |  |  |  |  |  |  | - | -92.5/-92.9 | - | -101.5/-102 |  | 9/9.1 |
| 7.1 | n25 | 25@191 | Note: Center approximate 1967.5 | - | - | 5 | 30 | - | 15 | 25@191 |  | Note: Same position of previous block2, Image to block1；Close to 25@0 | 5 | 1932.8 | - | -65 （-96.5+31.5） | -61/-60.8 | - | -101.6/-102.3 | - | 40.6/41.5 |  |
| 7.2 | n25 | 25@191 | Note: Center approximate 1967.5 | - | - | 5 | 30 | - | 15 |  |  |  |  |  |  | -25 (maximum power) | Interference is too high to connect | - | -99.2/-99.7 | - |  |  |
| Note1: Sensitivity fluctuates greatly at minimum UL power, demodulation is not possible at maximum UL power if the target CC’s power drops by 0.1dB. Note2: It is based on the configuration of 40M CBW. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |