3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503842

Wuhan, China, 07th – 11th April, 2025

Source: 	vivo, SmarterMicro

Title: 	Iot NTN HPUE Tx Requirements and feasibility analysis

Agenda Item:	7.10.2.3.1

Document for:	Approval

Introduction

From RAN4#112 to #113 agreements, summary and evaluation results were included in [1] to [10].

In RAN4#114, a WF was reached in [14]. A comprehensive discussion paper was provided on architecture analysis for NB-Iot based Iot-NTN in [12]. However, previous evaluation on GSM components were challenged since the usage of 2G is shrinking. Some more requirements were discussed, and SAR solution were also discussed.

In this paper, new evaluation of NB-Iot based Iot-NTN based on NR PA was submitted in response to comments received from last meeting. Based on this, the requirements applicability was proposed. In addition, further discussion are made on the Tx requirements and SAR related discussion.

Evaluation of NR PA for NB-Iot Iot-NTN

PA selection and Features

In RAN4#114, it was proposed by some companies that GSM deployment is shrinking and may not be the best preference for reuse. In order to confirm the potential of reusing current commercial PA used for NR RF chain for NB-Iot based Iot-NTN, an evaluation were done for one NR PA.

This PA was selected by the following selection criteria, and based on the criteria, one PA was selected as following:

Table 1. PA Selection criteria and condition

| Criteria | Selected PA |
| --- | --- |
| Commercialized | Already Used in smartphones and Iot devices |
| Sufficient shipment volume | Millions to tens of millions |
| Typical material and design | GaAs |
| Mainstream Performance | Yes |

The Main features of the PA is as following:

Table 2. Selected PA Main features

| Features |  |
| --- | --- |
| Mode supported | Multi-mode (NR/LTE/3G) |
| Bands Supported | LB/MB/HB |
| Prated | PC3:29dBm |

Here is block diagram:

Figure 1. Block diagram of a Multi-mode PA(NR/LTE/3G) for verification

Test conditions

The test conditions and some corresponding notes are as following:

Table 3. Test conditions

| Item | Test condition | Note |
| --- | --- | --- |
| Test waveform | NB-Iot based IoT-NTN Edge 1RE and full RE | Edge 1RE and full RE are selected worst cases. Referenced from 36.521-4, (#3, #4, #10) |
| Input port | RFIN_MB | Using Middle Band PA |
| Output port | MB1 | Using Middle Band PA |
| Test Vcc | Vcc=3.4V for normal Pout Vcc=4.0V for higher Pout | All these Vcc are within the normal working range for MB PA with guaranteed reliability. |
| Test Pout | From 27dBm to 32/33dBm | An IL of 2dB be assumed in addition to 31dBm nominal MOP |
| Test Frequency | 1995MHz, 1643MHz, | Typical frequency for band 255 and 256 (Band 254 is close to 256 for Tx) |
| Test item | UTRA ACLR and SEM | UTRA ACLR = 37dBc as defined in 36.102 SEM reusing PC3 requirements. (Bottleneck according to earlier study) |
| Note: Spurious emission is usually controlled by means of filter in front end and not a bottleneck for the HPUE, thus not considered here. EVM is also not considered since it is also not a bottleneck and large margin would apply as demonstrated in other similar tests. |  |  |

Results

ACLR Measurements

Table 4. Evaluation results of UTRA_ACLR for Freq = 1643MHz

Table 5. Evaluation results of UTRA_ACLR for Freq = 1995MHz

SEM Measurements

A summary of the evaluation of SEM is as following. The detailed SEM results for the boundary points between Pass and fail has been incorporated into the Annex for further reference (Marked as blue for the Pout/Vcc).

Table 6. Summary of evaluation of SEM

In addition, there are also some evaluation (not for all configurations) for Pout = 32dBm and Vcc = 3.4V for edge 1RE configuration, but all those test points were failed, since this Vcc 3.4V is no longer suitable for such high Pout, and the PA has entered non-linear region in such configuration.

Analysis and comparison

From the test results, some conclusions can be derived.

Different Frequencies

The first one would be quite aligned behaviour for 1643MHz and 1995MHz. More detailed speaking, this typical middle band PA inside the module is optimized for 1.7GHz ~2.1GHz. We can see this is quite consistent with band 256/255/254.

By comparison, previous verification for GSM show quite different performance on this two testing frequencies as in [12]. From this point, NR PA may be more adaptable for middle NTN bands.

Observation 1-1: The testing behaviour in 1643MHz and 1995MHz are quite aligned, meaning existing implementation for middle band NR PA is quite adaptable for band 256/255/254.

Nominal Output Power

From the verification, it can be seen that a maximum of 33dBm MOP can be reached within allowed Vcc configuration. Although Vcc = 3.4V is a typical value for PC3 middle band with a Prated 29dBm, Vcc 4.0V is still well within the max value for middle bands, with sufficient implementation margin for thermal effects etc to ensure long time use.

In earlier evaluation of GSM PA, the maximum value of the power is limited to 31dBm even in more satisfactory band in [12], which is apparently lower compared to this NR PA. Although the IL in GSM frontend is expected to be smaller compared to NR PA, there is still room to do dedicated optimization for the architecture of reusing NR Tx RF chain for Iot-NTN to reduce front end loss by bypassing unnecessary components.

With this understanding and current implementation, a 2dB front end insertion loss can be considered as a baseline. If so, a 33dBm PA output power means that a 31dBm nominal output power is achievable.

Of course, we can see that many of the SEM tests failed at 33dBm PA output power. However, it should be noted that the testing signals are not what discussed before as MPR=0 (centre single tone), and MPR has not been considered here yet.

Observation 1-2: An output power of 33dBm is achievable for NR PA evaluation. Under a 2dB front end IL assumption, a 31dBm nominal MOP can be achieved. (MPR not considered here yet.)

ACLR

ACLR results is quite satisfactory, and a large marge can be observed in all condition, even include the cases of highest power. This value is generally consistent with earlier evaluation for GSM and even better in higher power case. So it is believed that ACLR would not be a bottleneck.

Actually, there may even be question that the results are better than expected. After an analysis, it may because that the transmission is a narrowband signal, the UTRA ACLR using a measurement bandwidth of 3.84MHz, which even further improved the results.

Observation 1-3: UTRA ACLR for NR PA has quite large implementation margin compared to the current requirements and would not be a bottleneck.

SEM

The SEM requirements have been agreed to be reusing what PC3 is currently used. This is proving to be the bottle neck for PA to reach higher power.

In our verification, edge single tone case had passed all the cased for Pout = 32dBm, but would fail most of cases for Pout = 33dBm. In full allocation configurations, SEM passed for Pout = 29dBm. Those performance are also better compared to GSM PA.

It should be noted all those configurations can define a reasonable MPR to counter the emission problem.

Observation 1-4: The SEM were passed in all edge single tone configurations for Pout = 32dBm, and in full allocation configurations for Pout = 29dBm.

However, although no evaluation has been done for centre single tone case, it can still be reasonably deduced that for centre single tone case, SEM can be passed in case of Pout = 33dBm. Take 1643MHz, 4.0V, 1@0 & 1@11 as example, and two figures were copied from the Annex:

It can be seen although those two cases failed, they failed only with small margin, and only in one side. Since this is narrow band case, the behaviour should be stable in this range, and it is highly likely that a centre allocation would pass the SEM requirements. Actually for 1995MHz, even 1@11 has passed SEM in case of 33dBm of Pout.

Observation 1-5: Though not verified this time, it is highly likely that the centre single tone configuration would pass SEM test in case of Pout = 33dBm.

In addition, there is one more thing need to be considered. In legacy 36.102, 1 RE allocation was not defined for MPR requirements. Later, this 1RE configuration was added for MPR in CR [15]. However, current 36.521-4 has defined for edge single tone verification (configuration #3, #4) as BPSK in Table 3.

This is a contradiction that need to be solved. Although the SEM is not likely have large difference for BPSK/QPSK, still, if QPSK is confirmed to be used in the requirements, maybe more evaluation is needed for QPSK case and testing spec may need to be changed or supplemented at least for HPUE.

Observation 1-6: Further consideration needed for BPSK/QPSK for 1RE configuration.

Spurious Emission & EVM

Spurious emission is not verified in this evaluation. The main reason is that spurious emission is usually controlled by means of filter, not the PA itself. This part would not be an implementation bottleneck, thus dedicated verification of this requirements are not necessary.

EVM is also not considered, since it is also not a bottleneck and large margin would apply as demonstrated in earlier verification for GSM PA.

Observation 1-7: Spurious emission and EVM would not be bottle neck.

Analysis Summary

Based on the observations, we believe a nominal MOP of 31dBm can be achieved with commercial NR PA for NB-Iot based Iot-NTN, with a reasonable MPR defined.

Observation 2: A nominal MOP of 31dBm can be achieved to satisfy the current SEM and other requirements by assuming a reasonable MPR in different conditions and a reasonable IL, with available commercial NR PA for NB-Iot based Iot-NTN. Better performance observed compared to previous GSM PA verification.

HPUE Impact for Tx Requirements

General

The current status is for requirements applicability is as following which was referenced from last meeting’s WF [14].

Table: Current status for requirements applicability for Iot-NTN

|  | Power Class | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- | --- |
| NB-Iot based | PC1 | 1Tx | FFS (non-handheld already agreed) |
|  | PC2 | 1Tx | Both |
| eMTC based with half duplex | PC1 | 1Tx | FFS (non-handheld already agreed) |
|  | PC2 | 1Tx | Both |
| eMTC based with full duplex | PC1 | 1Tx | Non-handheld |
|  | PC2 | 1Tx | Both |

It can be seen that only PC1 for NB-Iot based and eMTC based with half duplex has not been confirmed yet.

For NB-Iot based Iot-NTN, three architectures were summarized in last meeting in [12]. In this meeting, further evaluation based on NR PA was done, and an analysis has been provided in previous section. Based on these observations, the following proposal is proposed:

Proposal 1: Apply PC1 requirements to handheld UE for NB-Iot based Iot-NTN.

For eMTC based with half duplex, currently not much evaluation has been done yet on PC1 applicability. Since no duplexer needed, the main constraint is also PA which is similar to NB-Iot. However, since the signal bandwidth is considerably larger compared to NB-Iot, and the SEM is significantly different, the current evaluation based on NB-Iot can not be reused. If no more input can be gathered, it might be a safter solution to not apply PC1 requirements for handheld UE for eMTC based with half duplex Iot-NTN.

Proposal 2: Not apply PC1 requirements to handheld UE for eMTC based with half duplex Iot-NTN if no more input on this issue.

NB-Iot & eMTC based

MOP Tolerance

In last meeting, the MOP tolerance were discussed but not concluded, the WF is as following:

Issue 1-1-8: The Basic MOP tolerance (Including eMTC and NB-Iot based)

- Proposals

  - For PC2:

    - Option 1: +2 / -3dB

    - Option 2: +2 / -2dB

  - For PC1:

    - Option 1:  +2 / -3dB

- WF

FFS

The support view of option 1 (+2/-3)is try to align the requirements with NR TN and NR NTN. Since LTE HPUE is not well commercialized, reusing NR High power components is likely.

However, option 2 (+2/-2) also receive support, in that this is being used in LTE TN spec. In addition, the current PA implementation seems have margin in this parameter, and even slight stricter option (+2/-2) would not be too challenge.

Observation 3-1: For PC2, there may be benefit to align Iot-NTN MOP tolerance with NR.

Observation 3-2: Based on current state of the art PA technology, both +2/-3 and +2/-2 dB are feasible for typical implementation.

Based on the observation, there is the following proposal:

Proposal 3a: Using either +2/-3dB or +2/-2dB as basic MOP tolerance for Iot-NTN for PC2.

For power class 1, although the implementation might be more complicated, the tolerance currently for both LTE and NR is +2/-3dB. It is natural to using this for Iot-NTN.

Proposal 3b: Using +2/-3dB as MOP tolerance for Iot-NTN for PC1.

NB-Iot Based

MPR

Since ACLR/SEM has been confirmed last meeting for NB-Iot based Iot-NTN, the study of MPR can be readily started.

In last meeting, some assumptions were agreed for NB-Iot based Iot-NTN:

Issue 1-1-5: MPR/A-MPR assumptions for NB-Iot based Iot-NTN for PC1/2

Agreement:

- For MPR/A-MPR assumptions for NB-Iot based Iot-NTN for PC1/2

  - For NB-IoT PC2, for A-MPR evaluations assume 25 dBc IQ-image and LO-leakage and 60 dBc CIM3

  - For NB-IoT PC1, for MPR and A-MPR evaluations assume [28~34]dBc IQ-image and LO-leakage and 60 dBc CIM3.

  - For NB-IoT PC2, as a starting point consider re-use of PC3 MPR

    - Editor’s Note: In RAN4#113 there was also a related agreement: “As the starting point, consider the center single tone allocation as MPR=0 reference point, and introduce MPR for edge single tone allocation.”

In addition, For NB-Iot based Iot NTN, both GAM PA and NR PA were agreed to be considered.

For PC2, since it is expected that the margin should be sufficient, there is already a starting point of reusing PC3 MPR. However, for PC1, more uncertainties would involve. It is proposed to consider the testing results from previous evaluation. For example, if considering the evaluation based on NR PA from previous section, the following MPR can be considered:

- Centre single tone: MPR = 0dB (Pout = 33dBm)

- Edge single tone: MPR = 1dB (Pout = 32dBm)

- Full tones: MPR = 4dB (Pout = 29dBm)

Proposal 4a. For PC1 NB-Iot based Iot-NTN, MPR requirements need consider measurement evaluation. E.g. the following backoff need in evaluation for NR PA:

- Centre single tone: MPR = 0dB (Pout = 33dBm)

- Edge single tone: MPR = 1dB (Pout = 32dBm)

- Full tones: MPR = 4dB (Pout = 29dBm)

In addition, it has been mentioned in clause 2.4 that currently 36.521-4 has defined for edge single tone verification points (configuration #3, #4) as BPSK in Table 3 without the single RE MPR requirements. But recently new single RE MPR requirements was added in maintenance CR [15] as QPSK. This misalignment should be addressed.

Proposal 4b: Discuss how to address the misalignment of single RE MPR requirements between 36.102 and 38.521-4 (BPSK and QPSK).

P-MPR

In last meeting, one tentative P-MPR proposal was raised for NB-Iot, but no conclusions made yet.

Issue 1-1-7: P-MPR definition for NB-Iot based Iot-NTN.

- Proposals

  - For NB-Iot based Iot-NTN, discuss the proposal, confirm it is applicable for all the power classes, and discuss whether it can be defined from Rel-18.

  - Discuss whether P-MPR should be applied to handheld UEs only and whether this needs to be specified or not.

- WF

  - Further discuss P-MPR definition

In this meeting, we would like to re-submit this proposal.

Proposal 5: Further discuss the P-MPR proposal for NB-Iot based Iot-NTN from last meeting. This is preferred from Rel-18, and for all power classes not differentiate UE types.

eMTC-Based

MPR

For ACLR, eMTC still not confirm the ACLR yet. And this also has an impact on the MPR.

However, as discussed in [16], using lower ACLR compared to TN PC2/1 not necessarily means that MPR evaluation should be redo and reset. MPR is after all a lower bound, and the flexibility of performance improvement can be implemented if necessary if lower but sufficient ACLR can be used.

Proposal 6: More relaxed ACLR is preferred for eMTC-based Iot-NTN, while reusing current MPR requirements from TN can also be considered.

SAR and Duty cycle

In last meeting, a complete background and detailed analysis of SAR was introduced in [12]. A comprehensive discussion was also taken place during the meeting.

However, the current available scheme both have problems and seems difficult to converge. Here we still think the proposal we made last meeting may be one viable way and re-submitted them for further discussion.

Observation 4-1 (Re-submit): The two basic ways to control SAR is by either reduce the power, or reduce transmission time.

Observation 4-2 (Re-submit): UE voluntarily dropping some uplink transmissions based on self-assessment can reduce SAR.

Proposal 7 (Re-submit): Consider to introduce a scheme that UE voluntarily dropping uplink transmissions by self-assessment to control the SAR. This may be simple enough, can be controlled by network and still useful.

Conclusion

Evaluation of NR PA for NB-Iot Iot NTN

Observation 1: From the evaluation of NR PA for NB-Iot NTN:

- The testing behaviour in 1643MHz and 1995MHz are quite aligned, meaning existing implementation for middle band NR PA is quite adaptable for band 256/255/254.

- An output power of 33dBm is achievable for NR PA evaluation. Under a 2dB front end IL assumption, a 31dBm nominal MOP can be achieved. (MPR not considered here yet.)

- UTRA ACLR for NR PA has quite large implementation margin compared to the current requirements and would not be a bottleneck.

- The SEM were passed in all edge single tone configurations for Pout = 32dBm, and in full allocation configurations for Pout = 29dBm.

- Though not verified this time, it is highly likely that the centre single tone configuration would pass SEM test in case of Pout = 33dBm.

- Further consideration needed for BPSK/QPSK for 1RE configuration.

- Spurious emission and EVM would not be bottle neck.

Observation 2: A nominal MOP of 31dBm can be achieved to satisfy the current SEM and other requirements by assuming a reasonable MPR in different conditions and a reasonable IL, with available commercial NR PA for NB-Iot based Iot-NTN. Better performance observed compared to previous GSM PA verification.

HPUE Impact for Tx Requirements

General:

Proposal 1: Apply PC1 requirements to handheld UE for NB-Iot based Iot-NTN.

Proposal 2: Not apply PC1 requirements to handheld UE for eMTC based with half duplex Iot-NTN if no more input on this issue.

NB-Iot & eMTC based:

MOP tolerance:

Observation 3:

- For PC2, there may be benefit to align Iot-NTN MOP tolerance with NR.

- Based on current state of the art PA technology, both +2/-3 and +2/-2 dB are feasible for typical implementation.

Proposal 3a: Using either +2/-3dB or +2/-2dB as basic MOP tolerance for Iot-NTN for PC2.

Proposal 3b: Using +2/-3dB as MOP tolerance for Iot-NTN for PC1.

NB-Iot based:

MPR:

Proposal 4a. For PC1 NB-Iot based Iot-NTN, MPR requirements need consider measurement evaluation. E.g. the following backoff need in evaluation for NR PA:

- Centre single tone: MPR = 0dB (Pout = 33dBm)

- Edge single tone: MPR = 1dB (Pout = 32dBm)

- Full tones: MPR = 4dB (Pout = 29dBm)

Proposal 4b: Discuss how to address the misalignment of single RE MPR requirements between 36.102 and 38.521-4 (BPSK and QPSK).

P-MPR:

Proposal 5: Further discuss the P-MPR proposal for NB-Iot based Iot-NTN from last meeting. This is preferred from Rel-18, and for all power classes not differentiate UE types.

eMTC based:

MPR:

Proposal 6: More relaxed ACLR is preferred for eMTC-based Iot-NTN, while reusing current MPR requirements from TN can also be considered.

SAR and duty cycle

Observation 4-1 (Re-submit): The two basic ways to control SAR is by either reduce the power, or reduce transmission time.

Observation 4-2 (Re-submit): UE voluntarily dropping some uplink transmissions based on self-assessment can reduce SAR.

Proposal 7 (Re-submit): Consider to introduce a scheme that UE voluntarily dropping uplink transmissions by self-assessment to control the SAR. This may be simple enough, can be controlled by network and still useful.

- References

[1] RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, RAN#103

[2] R4-2412829, Topic summary for [112][127] NR_IoT_NTN_HPUE , Samsung, RAN4#112

[3] R4-2414452, WF on UE RF requirements for NTN HPUE, Samsung, RAN4#112

[4] R4-2412099, Feasibility study and Initial analysis for NTN HPUE RF Tx Requirements, vivo, RAN4#112

[5] R4-2415790, Feasibility study and analysis for Iot NTN HPUE Tx Requirements, vivo, RAN4#112bis

[6] R4-2415229, Topic summary for [112bis][125] NR_IoT_NTN_HPUE_part2,vivo, RAN4#112bis

[7] R4-2417205, WF on HPUE for IoT NTN, vivo, RAN4#112bis

[8] R4-2418202, Iot NTN HPUE Tx Requirements and feasibility analysis, vivo, RAN4#113

[9] R4-2418149, Topic summary for [113][127] NR_IoT_NTN_HPUE_part2, Moderator (vivo), RAN4#113

[10] R4-2420404, WF on IoT NTN HPUE, vivo, RAN4#113

[11] R4- 2500766, CR on Remove GSM_ACLR for NB-Iot based Iot-NTN, vivo, Huawei, MediaTek, RAN4#114

[12] R4-2500764, Iot NTN HPUE Tx Requirements and feasibility analysis, vivo, RAN4#114

[13] R4-2500681, Topic summary for [114][129] NR_IoT_NTN_HPUE_part2, Moderator(Vivo), RAN4#114

[14] R4-2503006, WF on HPUE UE requirement for IoT-NTN, vivo, RAN4#114

[15] R4-2414414, Correct the MPR requirements for NTN Category NB1 and NB2, vivo, RAN4#112

[16] R4-2503840, NR NTN HPUE Tx Requirements and feasibility analysis, vivo, RAN4#114bis

- Annex. SEM results for NR PA

- 1643MHz

- 1@0, 4.0V

- Pout = 32dBm

- Pout = 33dBm

- 1@11, 4.0V

- Pout = 32dBm

- Pout = 33dBm

- 12@0, 3.4V

- Pout = 29dBm

- Pout = 30dBm

- 1995MHz

- 1@0, 4.0V

- Pout = 32dBm

- Pout = 33dBm

- 1@11, 4.0V

- Pout = 33dBm

- 12@0, 3.4V

- Pout = 29dBm

- Pout = 30dBm