3GPP TSG-RAN WG4 Meeting # 114bis                              R4-2503852

# Wuhan, Hubei, China, 07th – 11th April, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on impacts for measurement event prediction

Agenda Item:	7.21.2.2

Document for:	Approval

- Introduction

In RAN plenary #105 meeting, a revised SID[1] for Rel-19 was approved to study the AI/ML for mobility in NR. The objectives for RAN4 of this SID are listed as follows.

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation - NOTE 1: RAN1/3 work can be triggered via LS NOTE 2: To avoid duplicate study with “AI/ML for NG-RAN” led by RAN3 NOTE 3: Two-sided model is not included

In this document, we will provide some views on requirements impacts based on RAN2 assumptions of AI/ML for mobility from the RAN4 perspective.

- Discussion

In previous meeting, we discussed the priorities of different use cases and have reached consensus that RAN4 to start the discussion with RRM measurement prediction use-case. For measurement event prediction use-case and RLF use-case, more progress is needed from RAN2, and RAN4 to only start the discussion of Measurement event prediction use-case and RLF use-case if sufficient progress in RAN2. In the last meeting, we have an agreement that RAN4 will start the discussion on measurement event prediction use case in this meeting.

Issue 1-1-1: Measurement event prediction use case Agreement: - RAN4 will start the discussion on measurement event prediction use case in RAN4#114bis meeting.

RAN2 has discussed measurement event prediction use-case many times and reached some agreements. To our knowledge, RAN2 focused on discussing simulation assumptions based on measurement event prediction in previous meetings and plans to provide simulation results in this meeting. The progress of RAN2 is replicated as follows.

RAN2#127bis Agreements - Measurement event prediction simulations will at least focus on intra-frequency FR2, case A, and second study goal (i.e. HO KPI improvement). FFS what is KPI. - Companies can bring simulation results for intra-frequency measurement reduction for FR1 and report what they are doing. Focus on temporal case B. - Companies will prioritize simulations on indirect method. Companies can bring simulations on direct method and should report what method is being used. - Measurement event prediction results are expected in RAN2#129 Agreements on simulation assumptions - The Simulation assumption of RRM measurement prediction can be reused unless otherwise specified. - Companies can pick and report what they are using for filtering options (similar to RRM prediction) - Companies will focus on sub-case 2 for measurement event prediction. Companies can simulate other sub-cases if they wish and report what they are using. - Leave the simulation parameter discussion for email discussion. Pick only one value for A3. - It is up to company implementation how to model UE behavior after A3 event is trigger. Focus on intermediate KPIs for this exercise. System level KPI is FFFs - For measurement event prediction, traffic is not simulated. Agreements on inputs/outputs and KPIs - For indirect measurement event prediction, the intermediate output (i.e., the output of RRM prediction model) is RSRP of serving/neighboring cells. The final output is the expected occurrence time of a certain measurement event (ex. event A3). - For direct measurement event prediction, the model output is the probability of event occurrence within a time window. - A3 event prediction should follow legacy rules (i.e. the “predicted” conditions have to persist for the duration of TTT). - As baseline, we will use RLF event prediction KPI: - for indirect: F1 score. the following can be reported: RSRP difference, missed event detection, false event detection. FFS how to define F1 score. - time difference of true time event reporting triggered and predicted time event reporting triggered, true event prediction. - for direct: F1 score. The following can be reported: missed event detection, false event detection, - Continue discussion over email discussion to see if there is a difference. 2 The input of model for RRM measurement prediction can be reused as baseline for corresponding direct measurement event prediction. Additional input(s) is also allowed. 3 For intra-frequency temporal domain case B indirect measurement event prediction is taken as baseline. Direct prediction is optional.

In RAN2, indirect measurement event prediction and direct measurement event prediction are both within the scope of their research. Indirect measurement event prediction means that intermediate results are needed. That is to say, the prediction of RRM measurement is first performed to obtain the predicted RSRP, and then based on these prediction results, the calculation results of measurement events are further obtained. Correspondingly, direct measurement event prediction does not require to obtain the prediction results of RSRP.

Indirect measurement event prediction for temporal domain case A:

In indirect measurement event prediction, future measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case A at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model.

Indirect measurement event prediction for temporal domain case B:

In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case B at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model.

Indirect measurement event prediction for frequency domain:

In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in frequency domain at first. Afterwards, predicted and optional actual historical measurement result(s) of serving cell are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model.

Indirect measurement event prediction for spatial domain:

In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in spatial domain at first. Afterwards, predicted measurement result(s) and actual measurement results are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model

Based on RAN2’s progress, we perfer RAN4 to discuss both indirect measurement event and direct measurement event prediction. Meanwhile, RAN2 has an agreement that the simulation assumption of RRM measurement prediction can be reused unless otherwise specified, so we can apply the analysis results of the simulation parameters for RRM measurement prediction to measurement event prediction.

Proposal 1: RAN4 should discuss the RRM impact on indirect measurement event prediction and direct measurement event prediction.

Proposal 2: Measurement error should be considered into measurement event prediction, which is similar as RRM measurement prediction.

- Indirect prediction: Case A Most of the results show that the F1 score for prediction of measurement event is very good. For handover decision option 3 and 2 AI/ML performs better than baseline (legacy) in terms of HO failure rate. - Indirect prediction: Case B Measurement event case B can have very good F1 score depending on filtering approach/PW F1 score is good for the PW window for case B (i.e. low PW) F1 score decreases with an increasing MRRT value We will focus on finding cases that bring benefits to the system rather than trying to compare With indirect measurement event prediction based on temporal domain Case B (MRRT=50%), the AI-based HO has a minor/no system-level performance (i.e., HOF rate and HO number) decrease compared with the legacy HO mechanism. - Other Inter-frequency measurement event prediction will be considered for the specification impact study but no explicit simulations will be required. - Direct prediction F1 score for direct measurement is very good based on the current simulation results.

Based on the current discussion of RAN2, we suggest that similar to RRM measurement prediction, we first discuss the impact of temproal domain and frequency domain. As for spatial domain, if time permits or work needs, we can further discuss.

Proposal 3: For measurement event prediction, RAN4 should at least consider scenarios2(FR1 to FR1 intra-frequency temporal domain caseB), scenario 3 (FR1 to FR1 inter-frequency (frequency domain)), and scenario 4 (FR2 to FR2 intra-frequency temporal domain case A).

for indirect event measurement prediction, measurement result(s) is predicted by a RRM measurement prediction model first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Therefore, the RRM impact on RSRP difference prediction accuracy in RRM measurement prediction use case can be directly reused for indirect event measurement prediction.

Proposal 4: Discussions on RRM impact for RRM measurement prediction use case can be reused for indirect event measurement prediction.

- Conclusion

In this contribution, we have the following observations and proposals for the AI/ML monility:

Proposal 1: RAN4 should discuss the RRM impact on indirect measurement event prediction and direct measurement event prediction.

Proposal 2: Measurement error should be considered into measurement event prediction, which is similar as RRM measurement prediction.

Proposal 3: For measurement event prediction, RAN4 should at least consider scenarios2(FR1 to FR1 intra-frequency temporal domain caseB), scenario 3 (FR1 to FR1 inter-frequency (frequency domain)), and scenario 4 (FR2 to FR2 intra-frequency temporal domain case A).

Proposal 4: Discussions on RRM impact for RRM measurement prediction use case can be reused for indirect event measurement prediction.

- Reference

- RP-242393, Revised SID on AIML for mobility in NR , OPPO (rapporteur)

- R4-2502594, WF on FS_NR_AIML_Mob, Nokia